Appendix  D13

Attachment 2: Estimated Emission Reduction Calculations to Support Additional Control Measures and Benefits Not Included in the Regional Modeling

New Jersey is working to propose rules that will produce additional emission reductions that were not included in the regional modeling. These measures include federal, regional initiatives, State initiatives, and additional benefits not included in earlier calculations of benefits. The estimated additional benefits from these measures are listed in Table D13.3 and the measures and more detailed calculations are presented in the following section. These additional benefits provide further confidence that New Jersey will attain the 8-hour ozone NAAQS by 2009.

1.	Regional - New Refinery Rules

New Jersey is preparing to propose rules to reduce emissions from refineries that will cover catalytic and thermal cracking units, leaks detection and repair (LDAR), flares and process heaters. The largest categories of NOx emissions from refineries are from boilers and process heaters. Boilers and process heaters will see a reduction of approximately 40 percent (%) in NOx emissions using Ultra-low NOx burners or selective catalytic reduction.  The 2002 inventory for this category is approximately 8.2 tons per day and a 40% reduction will yield an emission benefit of about 3.3 tons per day of NOx.

New Jersey has two major point source facilities that will be subject to the proposed rule, one in the Northern New Jersey/New York/Connecticut nonattainment area and one in the Southern New Jersey/Philadelphia nonattainment area. The other refineries in the State are subject to similar control measures as a result of Administrative Consent Orders (ACOs).  The Table 1 below shows the proposed control measures, the 2002 actual emissions, and the estimated reduction for VOC and NOx by 2009.

Table 1: Estimated Reductions from Refineries (tons per day)

| Proposed Control Measure | % Reduction | NOx 2002 emissions | EstimatedNOx 2009 reductions | VOC 2002 emissions | Estimated VOC 2009 reductions |
| --- | --- | --- | --- | --- | --- |
| Catalytic & thermal cracking units | 60% | 1.36 | 0.54 | - | - |
| Leak detection & repair | 50% | - | - | 0.06 | 0.03 |
| Flares | 53% | - | - | 0.25 | 0.01 |
| Boilers & process heaters | 40% | 8.2 | 3.3 | - | - |

2.	Regional - High Electrical Demand Day Initiative

As part of the High Electrical Demand Day (HEDD) Initiative, New Jersey has committed to reduce NOx emissions in those electrical generating units that primarily operate on high electrical demand days by 19.8 tons per day by 2009. As of June 13, 2007, New Jersey joins the following OTC States in signing the Memorandum of Understanding regarding this initiative: Connecticut, Delaware, Maryland, Maine, Massachusetts, New Hampshire, New Jersey, Rhode Island, and Vermont. A copy of the Memorandum of Understanding can be found in Appendix C1.

In order to determine the emission reductions for New Jersey, emissions comparisons were made between an average day in June 2005, and a very high electric demand day in July 2005 that showed a 145% increase in New Jersey electric generator NOx emissions and five 8-hour ozone exceedances.  Estimated emissions from the 33 HEDD combustion turbines which operated on the high demand day without water injection were reduced by 40%.  Forty percent reduction is what would be generally achievable by the installation of water injection.  Emissions from the five (5) HEDD boilers that were operating without selective non-catalytic reduction (SNCR) were reduced by 30%.  A thirty percent reduction is what would be generally achievable by the installation of SNCR.  Note that this level of control provides an estimate of the benefits; not all units will be able to meet such reductions. For 2009, it is expected that the generators will be submitting plans to achieve the emission reductions to the State for case-by-case reviews.

The difference between the emissions as originally reported and the emissions as reduced by the above calculations is 19.8 tons for New Jersey. These HEDD emission reductions were estimated to be 13.8 tons in the Northern New Jersey/New York/Connecticut nonattainment area and 6.1 tons in the Southern New Jersey/Philadelphia nonattainment area based on the location of the HEDD units. Emission reduction estimates for the States that share a multi-state nonattainment area with New Jersey are listed Table D13.3.

3.	State - New Jersey Diesel Idling Rule Changes Calculations:

Changes to Subchapter 14, Control and Prohibition of Air Pollution from Diesel-Powered Motor Vehicles, were proposed in 2006 and adopted in May 2007. Emission reductions will be realized through implementation of these rule changes. These changes included clarifying the language on some exemptions; eliminating other exemptions, such as sun setting the sleeper berth exemption in 2010; and other associated changes such as enforcement, education, outreach, and legislative changes. There are both NOx and PM benefits for this rule. The NOx benefits calculations are included in this Appendix.

Methodology:

Emission benefits were estimated by starting with a published value for the percent of heavy duty truck emissions that result from idling, applying an assumed idling reduction percent and applying this to the New Jersey 2009 inventory emissions for heavy duty highway trucks.

- Idling emissions from on-road (class 8) heavy duty highway trucks represent approximately 7.8% of total emissions.

- Due to the inclusion of other heavy duty trucks in Subchapter 14 (classes 3-7, in addition to class 8 mentioned above), the amount of emissions attributable to idling was increased from 7.8% to 10%.

- Assuming that New Jersey’s actions will reduce idling by 50%, overall emissions from heavy duty highway trucks will be reduced to 5% (50% of 10%).

This 50% reduction in idling from Class 3-8 vehicles is estimated to be achieved from the following sources:

  - Subchapter 14 rule revision (which includes eliminating / tightening many exemptions, the phasing-in of idling alternates such as auxiliary power units and sun setting the sleeper berth exemption in 2010),

  - Increased enforcement through a legislative language change (N.J.S.A. 39:3-70.2), which allows State and local police to clearly enforce this rule,

  - Education and outreach,

  - Idling sweeps by the Compliance and Enforcement group,

  - Sales of “No Idling” signs, yielding increased awareness of the regulation

# The 2009 Emissions Inventory (for all on-road diesel heavy duty vehicles) is 111 tons per summer day NOx.  A 5 % reduction yields approximately 6 tons per summer day decrease in NOx. The emission benefit was allocated as 3 tpsd NOx for the Northern New Jersey/New York/Connecticut nonattainment area and 3 tpsd NOx for the Southern New Jersey/Philadelphia nonattainment area based on vehicles miles traveled for heavy duty highway trucks.

4.	State - New Jersey Diesel Cutpoint Rule Changes:

The NJDEP is working toward proposing and adopting in accordance with the New Jersey Administrative Procedures Act and the New Jersey Air Pollution Control Act changes to establish more stringent test standards (cutpoints) for the existing diesel I/M program. The stricter cutpoints will result in additional emission reductions of volatile organic compounds (VOCs) and oxides of nitrogen (NOx).  Emission reduction estimates are shown in tons of pollutants per day.

Table 2: 2009 Emission Benefits from Proposed Stricter Diesel I/M Cutpoint Program (Tons/Day)

| 2009 Emission Benefits from Proposed Stricter Diesel I/M Cutpoint Program (Tons/Day) |  |
| --- | --- |
| VOC | 0.6 |
| NOx | 0.2 |

Following is a summary of the methodology used to estimate these emission benefits:

- The California Air Resource Board (CARB) developed a series of tables of HDDV failure rates and pollutant-specific mal-performance factors. In general, emission benefits for the proposed cutpoint changes to the New Jersey diesel I/M program were estimated by combining the CARB mal-performance factors with: New Jersey HDDV VMT, emission factors (grams per mile) calculated using the USEPA MOBILE6 model and the failure/repair rates from the New Jersey diesel I/M program.

- Proposed cutpoint changes are expected to have a lower impact per vehicle than that of the original diesel I/M program implementation. The impact of the cutpoint reduction was estimated by analyzing historical data from the New Jersey diesel I/M program. The impact of the cutpoint change was estimated by considering the fraction of tested vehicles with emission levels between the existing and proposed cutpoints.

- The emission benefits were estimated as the product of the MOBILE6 calculated emission factor and the sum of the HDDV VMT attributed to repair and deterrence. The HDDV VMT attributed to repair was estimated using: the projected annual inspection failure rate increases, the repair rates, the weighted fleet % by model year ranges, the % of opacity changes and the pollutant mal-performance factors. The HDDV VMT attributed to deterrence was estimated using the previously calculated repair VMT and the annual inspection failure rate to roadside failure rate ratio.

5.	State - Municipal Waste Combustors

The NJDEP is considering proposing a NOx standard in the range of 100 to 150 ppm for municipal solid waste combustors (MSW), based upon the ability of selective non-catalytic reduction (SNCR) to reduce emissions more than are now being achieved. New Jersey has five resource recovery facilities located in the following counties: Essex, Union, Warren (Northern New Jersey/New York/Connecticut nonattainment area), Camden, and Gloucester (Southern New Jersey/Philadelphia nonattainment area).  There are 13 municipal waste combustors (MWC) at these five facilities. The NJDEP anticipates that a NOx emission reduction of 63 tons per year or 0.17 tons per day may be possible with this measure.

Table 3: Estimated Reductions from Municipal Waster Combustors

| Municipal Solid Waste Plant | Air Flow Rates in ACFM | Air Flow Rates in DSCFM | Pounds per hour using 100 ppm NOx limit | Pounds per hour using 130 ppm NOx limit | Pounds per hour using 150 ppm NOx limit |
| --- | --- | --- | --- | --- | --- |
| Essex | 237,900 | 60867 | 44 | 57 | 65 |
| Essex | 220,000 | 56287 | 40 | 52 | 60 |
| Essex | 229,000 | 58590 | 42 | 55 | 63 |
| Average ACFM | 228,967 | 58581 |  |  |  |
| Camden | 97,409 | 24922 | 18 | 23 | 27 |
| Camden | 99,064 | 25346 | 18 | 24 | 27 |
| Camden | 101,495 | 25967 | 19 | 24 | 28 |
| Average ACFM | 99,323 | 25412 |  |  |  |
| Union | 128,029 | 32756 | 23 | 30 | 35 |
| Union | 131,191 | 33565 | 24 | 31 | 36 |
| Union | 130,767 | 33457 | 24 | 31 | 36 |
| Average ACFM | 129,996 | 33259 |  |  |  |
| Warren | 50,005 | 12794 | 9 | 12 | 14 |
| Warren | 55,702 | 14251 | 10 | 13 | 15 |
| Average ACFM | 52,854 | 13523 |  | 13 |  |
|  |  | 0 | 0 |  |  |
| Gloucester | 62,820 | 16072 | 12 | 15 | 17 |
| Gloucester | 72,130 | 18454 | 13 | 17 | 20 |
| Average ACFM | 67,475 | 17263 |  |  |  |
| Total pounds per hour = |  |  | 296 | 397 | 444 |
| Total tons per year = |  |  | 1296 | 1740 | 1944 |
| Assumption: 15% oxygen in the stack and 300 degrees F stack temperature |  |  |  |  |  |
| Actual 2002 NOx in tons per year = | 1,803 | Expected reductions = | 507 | 63 |  |
| Permitted NOx = | 3,541 |  |  |  |  |

# The calculations of the estimated emission reductions, using the 130 ppm NOx limit as an example:

- Estimate reductions from each municipal solid waste plant in pounds per hour.

- Convert pounds per hour to tons per year:

(lbs/hr)(8760 hours/year)/2000 lbs/ton = tons/year

(397 lbs/hr)(8760 hours/year)/2000 lbs/ton = 1740 tons/year

- Calculate estimated emission reductions:

Expected reductions in emissions from MSW plants in tons/year =

(Actual 2002 NOx emissions from MSW plants in tons/year) – (Emission reductions from the MSW plants in tons/year)

= (1803 tons/year) – (1740 tons/year)

= 63 tons/year

6.	State - Petroleum Storage Tanks

New Jersey is working on proposing control measures for storage tanks which concentrates on the following storage operations; external floating roof tanks with slotted guide poles, degassing and interior tank cleaning operations, floating roof landing losses, and gasoline external floating roof tanks without domes.  These storage operations do not have any VOC emission controls but the NJDEP is considering proposing controls that will achieve reductions of 99%, 95%, 67%, and 63% respectfully, by 2009. The control measures and benefits under consideration are summarized below.

Table 4: Estimated Benefits from Petroleum Storage Tank Control Measures

| Control Measure | % Reduction | Estimated 2009 Benefit (tons per day) |
| --- | --- | --- |
| External floating roof tanks with slotted guidepole sleeves & covers | 99% | 1.02 |
| Tank degassing & cleaning | 95% | 0.13 |
| Floating roof landing losses | 67% | 4.38 |
| Gasoline external floating roof tanks without domes | 63% | 0.70 |
| TOTAL |  | 6.23 |

Slotted guidepole sleeves and covers would be required for tanks storing VOC and/or refinery products with a vapor pressure of less than 11.0 psia on external floating roof tanks which amounts to at least 236 tanks for New Jersey.  The base inventory for 2002 is 1.04 tpd.  Calculating growth and control from this measure would reduce this category to 0.02 tpd with a 99% reduction in VOC for 2009. The emission reductions of approximately 1.02 tpd were allocated to each nonattainment area based on the ratio of storage tanks located in New Jersey in the Northern New Jersey/New York/Connecticut area (80%) and in the Southern New Jersey/Philadelphia nonattainment area (20%).

The control measure that New Jersey is considering for tank degassing and interior tank cleaning will have an approximate efficiency of 95%.  New Jersey has approximately 1000 floating roof tanks that store VOC and/or refinery products with a vapor pressure less than 11.0 psia.  This control measure is based on the number of tanks cleaned once every 10 years.  The base 2002 inventory is 0.14 tpd of VOC emissions. Calculating growth and control from this measure would reduce this category to 0.01 tpd with a 95% reduction in VOC for 2009. Estimated emission reductions of  0.13 tpd were allocated to each nonattainment area based on the ratio of storage tanks located in New Jersey in the Northern New Jersey/New York/Connecticut area (80%) and in the Southern New Jersey/Philadelphia nonattainment area (20%).

The control measure for roof landing losses proposal is expected to require a retrofit with a six-inch lander height or cabled roof retrofit. Lowering the average roof leg height of 3.5 ft to 0.5 ft will yield a 67% VOC emission reduction.  New Jersey has approximately 1000 tanks with 8.23 tpd of VOC emissions in 2002. Applying growth and control measures would show a reduction of 67% reduction or an emission benefit of approximately 4.38 tpd of VOC emissions for 2009. The emission reductions of approximately 3.85 tpd were allocated to each nonattainment area based on the ratio of floating roof storage tanks in New Jersey in the Northern New Jersey/New York/Connecticut area (97%) and in the Southern New Jersey/Philadelphia nonattainment area (3%).

The proposed external floating roof tank measure entails retrofitting floating roof tanks that store gasoline with a dome.  New Jersey has approximately 53 floating roof tanks in the gasoline service at major facilities with VOC emissions of 1.1 tpd for 2002. Retrofitting the tanks with a dome would yield an average of 63% reduction in VOC emissions with an emission benefit of approximately 0.70 tpd of VOC emissions for 2009. The emission reductions of approximately 0.70 tpd were allocated to each nonattainment area based on the ratio of storage tanks located in New Jersey in the Northern New Jersey/New York/Connecticut area (80%) and in the Southern New Jersey/Philadelphia nonattainment area (20%).

7.	Federal - National Aerosol Coatings Rule

The USEPA is required to regulate VOC emissions from certain consumer and commercial products, according to 42 U.S.C. §7511b (Section 183(e)) of the Clean Air Act.  At the time of this proposed SIP revision, the USEPA was working to propose three national consumer and commercial product rules to establish or amend VOC limits for aerosol coatings (new rule), architectural and industrial maintenance (AIM) coatings (amendments), and household and institutional consumer products (amendments).  The USEPA issued guidance for states to take credit for these national rules.  According to the USEPA memorandum issued May 30, 2007, “OAQPS is working to finalize these national consumer and commercial products rules as soon as possible so that fully creditable emission reductions will be achieved by January 1, 2009.”  This means that VOC reductions will be realized for the 2009 ozone season.  However, USEPA further explains that states claiming credit for a national rule would have to develop measures to make up for any shortfalls if the national rule does not provide the anticipated emission reductions. Existing New Jersey regulations are expected to be as stringent as the soon to be proposed AIMs and consumer products rules, therefore New Jersey has not estimated additional benefits for these rules.

New Jersey calculated the additional credit from the new aerosol coatings rule using the USEPA guidance of 0.114 pounds per capita provided for previously unregulated areas.  The USEPA estimated that national aerosol coatings rule will achieve a 19 percent reduction in mass VOC emissions from the 1990 baseline.

The new percent reduction factor was calculated based on the example provided by MACTEC:

| OTC 2001 Model Rule Emission Factor | = | 6.06 lbs/capita |
| --- | --- | --- |
| Benefit from CARB 2005 Amendments | = | 0.122 lbs/capita |
| Benefit from USEPA Aerosol Coatings Rule | = | 0.114 lbs/capita |

The per capita factor after the implementation of the OTC 2001 model rule is 6.06 lbs/capita, as calculated in the 2001 Pechan Report, “Control Measure Development Support Analysis of Ozone Transport Commission Model Rules,” prepared by E.H. Pechan & Associates, March 31, 2001.  As discussed in more detail in the MANE-VU projection inventory report, since the OTC’s 2006 control measure is very similar to the CARB July 2005 amendments (with the exclusion of the anti-static products and shaving gel 2008/2009 limits), the per capita emission reductions are expected to be the same in the Ozone Transport Region, i.e., 0.122 lbs/capita.

| Percent Reduction | = | 100% *(1-((6.06-0.122-0.114) /6.06)) |
| --- | --- | --- |
|  | = | 3.89% |

# The 3.89 percent reduction was applied to the 2009 projected VOC emissions inventory for commercial and consumer solvent use, SCC 2465000000. The same equation was used to apply the new control measure to the inventory as discussed in Appendix E, pg. 12.  Since the category was previously controlled with an incremental control efficiency factor (CE) of 0.02 (see Appendix E, pg. 13), the benefit for the national rule was derived by taking the difference between the realized benefits before the rule and with the rule as follows:

Before the new national rule:

Control efficiency (CE) factor of 0.02

2002-2009 benefits:

Northern New Jersey/New York/Connecticut nonattainment area = 7.63 tpd

Southern New Jersey/Philadelphia nonattainment area = 3.45 tpd

Statewide benefits of 11.08 tpd

With the new national rule:

CE of 0.0389

Additional 2009 benefits

Northern New Jersey/New York/Connecticut nonattainment area = 8.44 tpd

Southern New Jersey/Philadelphia nonattainment area = 3.81 tpd

# Statewide benefits = 12.25 tpd

Incremental benefits for the new national rule:

Northern New Jersey/New York/Connecticut nonattainment area = 0.81 tpd

Southern New Jersey/Philadelphia nonattainment area = 0.36 tpd

Statewide benefits = 1.17 tpd

The equation for the benefit calculations is provided in Appendix E, Attachment 2.

- NOx Rule (2005)

The 2005 NOx rule was included in the modeling, but additional emission reductions were estimated in addition to those estimated for the modeling. Estimated emission reductions from the New Jersey 2005 amendments to Subchapter 19 “Additional NOx controls” were calculated in 2001 by E.H. Pechan.  However, as discussed in the New Jersey rule proposal dated September 20, 2004, additional reductions were estimated from annual tune-ups for boilers. The benefits of the NOx RACT rule (2005) included in the regional modeling were 7 tpsd (point and area sources) (see Appendix E); the benefits included in the New Jersey 2005 rule proposal were 13.3 tpsd. The additional emission reductions, for point sources, are approximately 4.7 tpsd; these additional emission reductions were allocated to each nonattainment area based on location such that 3.1 tpsd of benefits are in the Northern New Jersey/New York/Connecticut area (66%) and 1.6 tpsd are in the Southern New Jersey/Philadelphia nonattainment area (34%).

9.	Portable Fuel Container Rules

New Jersey’s existing Portable Fuel Container rule, which was one of the six OTC 1-hour ozone shortfall measures discussed in Chapter 4, and the anticipated amendments to that rule discussed as a BOTW measure in Chapter 4, were included in the 2009 BOTW modeling. However, additional emission reductions are anticipated beyond those estimated for the modeling.

Emissions from Portable Fuel Containers come from both area and non-road sources.  However, for New Jersey, only the estimated area source emission reductions for the 2005 Portable Fuel Container rule and the anticipated BOTW PFC rule amendments were included in the 2009 BOTW modeling.  For further details regarding the 2005 Portable Fuel Container rule calculations and additional details regarding the anticipated BOTW Portable Fuel Container amendment calculations see Appendix E. The NJDEP has calculated the following additional reductions for the non-road sector:

- approximately 2.1 tpsd of VOC in 2009 for the 2005 PFC rule, and;

- approximately 0.3 tpsd per summer day of VOC in 2009 for the anticipated BOTW PFC amendments.

The emissions were allocated to each nonattainment area based on the New Jersey nonattainment area ratios calculated for the 2005 PFC rule; this equates to approximately 1.7 tons per summer day VOC in the Northern New Jersey/New York/Connecticut nonattainment area and 0.7 tons per summer day in the Southern New Jersey/Philadelphia nonattainment area.

10.	ICI Boiler Rule

The BOTW ICI boiler rule, discussed in detail in Chapter 4, was included in the 2009 BOTW modeling.  However, additional emission reductions are anticipated beyond the 0.1 tpd that was estimated for the modeling.  Estimated emission reductions from the anticipated ICI boiler rule were calculated as discussed in Appendix E. Based on these calculations, New Jersey estimates an additional 6.8 tpd NOx reductions beyond those included in the 2009 BOTW modeling – 4.8 tpd in the Northern New Jersey/New York/Connecticut nonattainment area and 2 tpd in the Southern New Jersey/Philadelphia nonattainment area. The emission benefits were allocated to each nonattainment area based on the percent of point source emissions in each nonattainment area.

Memorandum of Understanding Among the States of the Ozone Transport Commission Concerning the Incorporation of High Electrical Demand Day Emission Reduction Strategies into Ozone Attainment State Implementation Planning, March 2, 2007.

http://www.otcair.org/

# NESCAUM Report: Final Report: Heavy-Duty Engine Emissions in the Northeast, May 1997.

# NESCAUM Report: Final Report: Heavy-Duty Engine Emissions in the Northeast, May 1997.

http://www.arb.ca.gov/msei/onroad/downloads/tsd/HDT_Emissions_New.pdf

USEPA Memorandum from Stephan D. Page, Director, Office of Air Quality Planning and Standards, to Region Air Division Directors, “Emission Reduction Credit for Three Federal Rules for Categories of Consumer and Commercial Products under Section 183(e) of the Clean Air Act,” May 30, 2007.

Calculation methodology modified from:  MACTEC.  Development of Emission Projections for 2009, 2012, and 2018 for NonEGU Point, Area, and Nonroad Sources in the MANE-VU Region, Final Technical Support Document.  Prepared for Mid-Atlantic Regional Air Management Association (MARAMA) by MACTEC Federal Programs, Inc., February 28, 2007.

ibid.

E.H. Pechan.  Control Measure Development Support Analysis of Ozone Transport Commission Model Rules.  Prepared for the Ozone Transport Commission by E.H. Pechan & Associates, Springfield, VA, 01.02.001/9408.000, March 31, 2001.

New Jersey Register 36 N.J.R. 4228(a); September 20, 2004.

MACTEC.  OTC Identification and Evaluation of Candidate Control Measures, Final Technical Support Document.  Prepared for the Ozone Transport Commission by MACTEC Federal Programs, Inc., Herdon, VA, February 28, 2007.

MACTEC.  Development of Emission Projections for 2009, 2012, and 2018 for NonEGU Point, Area, and Nonroad Sources in the MANE-VU Region, Final Technical Support Document.  Prepared for Mid-Atlantic Regional Air Management Association (MARAMA) by MACTEC Federal Programs, Inc., February 28, 2007.