Appendix D13

Attachment 2: Estimated Emission Reduction Calculations to Support Additional Control Measures and Benefits Not Included in the Regional Modeling

New Jersey is working to propose rules that will produce additional emission reductions that were not included in the regional modeling.  These measures include federal, regional initiatives, State initiatives, and additional benefits not included in earlier calculations of benefits.  The estimated additional benefits from these measures are listed in Table D13.3 and the measures and more detailed calculations are presented in the following section.  These additional benefits provide further confidence that New Jersey will attain the 8-hour ozone National Ambient Air Quality Standard (NAAQS) by 2009.

1.	Regional - New Refinery Rules

New Jersey is preparing to propose rules to reduce emissions from refineries from fluid catalytic cracking units (FCCUs), leaks detection and repair (LDAR), flares, and process heaters and boilers.  The largest categories of oxides of nitrogen (NOx) emissions from refineries are from boilers and process heaters.  Controlling boilers and process heaters will result in approximately 40 percent (%) in NOx emissions using Ultra-low NOx burners (ULNB) or selective catalytic reduction (SCR).  The 2002 inventory for this category is approximately 8.2 tons per day (tpd) and a 40 percent reduction will yield an emission benefit of about 3.15 tpd of NOx.  In addition, controlling VOC emissions from flares will result in the largest reduction of VOC emissions for refineries.  The 2002 VOC emissions inventory for flares is 1.4 tpd and installing Flare Gas Recovery (FGR) systems will result in an emission benefit of approximately 0.78 tpd.  In total, the control measures for refineries will result in an emission benefit of 3.28 tpd of NOx and 1.55 tpd of VOC.

New Jersey has two major point source facilities that will be subject to the proposed rule, one in the Northern New Jersey/New York/Connecticut nonattainment area and one in the Southern New Jersey/Philadelphia nonattainment area.  The other refineries in the State are subject to similar control measures as a result of Administrative Consent Orders (ACOs).  The Table 1 below shows the proposed control measures, the 2002 actual emissions, and the estimated reduction for volatile organic compounds (VOC) and NOx by 2009.

Table 1: Estimated Reductions from Refineries (tons per day)

| *Percent reductions are based on the information provided in the NJDEP white papers referenced in this section. The NJDEP white papers calculated emission benefits in tons per year. In this SIP, those emission benefits were converted to tons per day. Proposed Control Measure | % NOx Reduction* | NOx 2002 Emissions (tpd) | Estimated NOx 2009 Benefits (tpd) | % VOC Reduction* | VOC 2002 Emissions (tpd) | Estimated VOC 2009 Benefits (tpd) |
| --- | --- | --- | --- | --- | --- | --- |
| Fluid Catalytic Cracking Unit (FCCU) | 40% | 4.6 | -** | 85% | 0.37 | 0.31 |
| Leak Detection & Repair (LDAR) | - | - | - | 50% | 1.1 | 0.47 |
| Flares | 36% | 0.37 | 0.13 | 55% | 1.4 | 0.78 |
| Boilers & Process Heaters | 40% | 8.2 | 3.15 | - | - | - |
| Total | - | 13.17 | 3.28 | - | 2.87 | 1.55 |

**NOx reductions are expected post-2009.

2.	Regional - High Electrical Demand Day Initiative

As part of the High Electrical Demand Day (HEDD) Initiative, New Jersey has committed to reduce NOx emissions in those electrical generating units that primarily operate on high electrical demand days by 19.8 tons per day by 2009.  As of June 13, 2007, New Jersey joins the following OTC States in signing the Memorandum of Understanding regarding this initiative: Connecticut, Delaware, Maryland, Maine, Massachusetts, New Hampshire, New Jersey, Rhode Island, and Vermont.  A copy of the Memorandum of Understanding can be found in Appendix C1.

In order to determine the emission reductions for New Jersey, emissions comparisons were made between an average day in June 2005, and a very high electric demand day in July 2005 that showed a 145% increase in New Jersey electric generator NOx emissions and five 8-hour ozone exceedances.  Estimated emissions from the 33 HEDD combustion turbines which operated on the high demand day without water injection were reduced by 40%.  Forty percent reduction is what would be generally achievable by the installation of water injection.  Emissions from the five (5) HEDD boilers that were operating without selective non-catalytic reduction (SNCR) were reduced by 30%.  A thirty percent reduction is what would be generally achievable by the installation of SNCR.  Note that this level of control provides an estimate of the benefits; not all units will be able to meet such reductions.  For 2009, it is expected that the generators will be submitting plans to achieve the emission reductions to the State for case-by-case reviews.

The difference between the emissions as originally reported and the emissions as reduced by the above calculations is 19.8 tons for New Jersey.  These HEDD emission reductions were estimated to be 13.8 tons in the Northern New Jersey/New York/Connecticut nonattainment area and 6.1 tons in the Southern New Jersey/Philadelphia nonattainment area based on the location of the HEDD units.  Emission reduction estimates for the States that share a multi-state nonattainment area with New Jersey are listed Table D13.3.

3.	State - New Jersey Diesel Idling Rule Changes Calculations

Changes to Subchapter 14, Control and Prohibition of Air Pollution from Diesel-Powered Motor Vehicles, were proposed in 2006 and adopted in May 2007.  Emission reductions will be realized through implementation of these rule changes.  These changes included clarifying the language on some exemptions; eliminating other exemptions, such as sun setting the sleeper berth exemption in 2010; and other associated changes such as enforcement, education, outreach, and legislative changes.  There are both NOx and PM benefits for this rule.  The NOx benefits calculations are included in this Appendix.

Methodology:

Emission benefits were estimated by starting with a published value for the percent of heavy duty truck emissions that result from idling, applying an assumed idling reduction percent and applying this to the New Jersey 2009 inventory emissions for heavy duty highway trucks.

- Idling emissions from on-road (class 8) heavy duty highway trucks represent approximately 7.8% of total emissions.

- Due to the inclusion of other heavy duty trucks in Subchapter 14 (classes 3-7, in addition to class 8 mentioned above), the amount of emissions attributable to idling was increased from 7.8% to 10%.

- Assuming that New Jersey’s actions will reduce idling by 50%, overall emissions from heavy duty highway trucks will be reduced to 5% (50% of 10%).

This 50% reduction in idling from Class 3-8 vehicles is estimated to be achieved from the following sources:

  - Subchapter 14 rule revision (which includes eliminating / tightening many exemptions, the phasing-in of idling alternates such as auxiliary power units and sun setting the sleeper berth exemption in 2010),

  - Increased enforcement through a legislative language change (N.J.S.A. 39:3-70.2), which allows State and local police to clearly enforce this rule,

  - Education and outreach,

  - Idling sweeps by the Compliance and Enforcement group,

  - Sales of “No Idling” signs, yielding increased awareness of the regulation

# The 2009 Emissions Inventory (for all on-road diesel heavy duty vehicles) is 111 tons per summer day NOx.  A 5 % reduction yields approximately 6 tons per summer day decrease in NOx.  The emission benefit was allocated as 3 tpsd NOx for the Northern New Jersey/New York/Connecticut nonattainment area and 3 tpsd NOx for the Southern New Jersey/Philadelphia nonattainment area based on vehicles miles traveled for heavy duty highway trucks.

4.	State - New Jersey Diesel Cutpoint Rule Changes

The New Jersey Department of Environmental Protection (NJDEP) is working toward proposing and adopting in accordance with the New Jersey Administrative Procedures Act and the New Jersey Air Pollution Control Act changes to establish more stringent test standards (cutpoints) for the existing diesel inspection and maintenance (I/M) program.  The stricter cutpoints will result in additional emission reductions of VOCs and NOx.  Emission reduction estimates are shown in tons of pollutants per day.

Table 2: 2009 Statewide Emission Benefits from Proposed Stricter Diesel I/M Cutpoint Program (Tons/Day)

| 2009 Emission Benefits from Proposed Stricter Diesel I/M Cutpoint Program (Tons/Day) |  |
| --- | --- |
| VOC | 0.45 |
| NOx | 0.13 |

The methodology for the estimated benefits of the expected rule is described in this section.

Estimating inventory reductions due to diesel I/M using the USEPA MOBILE6 model directly is not possible because MOBILE6 has no diesel I/M model representation and assumes almost no deterioration of diesel vehicles.  The USEPA may assume additional in-use deterioration and mal-performance corrections of diesel vehicles in their future on-road emission model called Motor Vehicle Emission Simulator (MOVES).

The NJDEP has developed the following preliminary methodology to estimate emission benefits for stricter diesel I/M cutpoints.  The California Air Resources Board (CARB) has documented a series of tables of Heavy-Duty Diesel Vehicle (HDDV) emission I/M failure rates and pollutant specific mal-performance factors.  A mal-performance factor is the average fraction that emissions increase for vehicles that fail inspection.  The mal-performance factors used for the New Jersey methodology were derived from the CARB tables of mal-performance frequency and effects by model year and vehicle weight class.  The factors were weighted by model year and vehicle type.  Each criteria pollutant has its own mal-performance factor.  The preliminary New Jersey methodology uses: CARB mal-performance factors, New Jersey Heavy-Duty Diesel Vehicle (HDDV) vehicle miles traveled (VMT), MOBILE6 HDDV emission factors in grams per mile (GPM) for each criteria pollutant, and the failure/repair rates from the New Jersey diesel I/M program.

Emission benefits for the existing I/M program are estimated as the sum of the deterrent benefit and the repair benefit.  It is necessary to use data collected at the beginning of the New Jersey diesel I/M program and compare it to data collected after the initial start-up period.  The deterrent benefit is the product of: the difference between the pre-enforcement and post-enforcement I/M failure rates, the HDDV VMT, the mal-performance factors, and the average MOBILE6 HDDV emission factors.  The repair benefit is the product of: the Diesel Emissions Inspection Center (DEIC) failure rate, the repair rate, the HDDV VMT, the mal-performance factors, and the average MOBILE6 HDDV emission factors.  The repair rate is estimated by adding together: the number of vehicles repaired during the inspection, the number of vehicles that passed a violation reinspection, and the number of vehicles that passed a later inspection.  This sum is divided by the total number of initial inspection failures (pre-repair fails + violation reinspections) to obtain the repair rate.

The next step is to estimate the emission benefits of reducing the I/M diesel cutpoints, i.e., the “secondary implementation.”  When opacity cutpoints are reduced, the impact (mal-performance factors) of the secondary implementation must be adjusted from the impact of the initial I/M program implementation to include only the lower end of the mal-performance range (marginal emitters).  There will also be a deterrent benefit associated with the cutpoint change that can be initially estimated by comparing the roadside inspection data to the annual DEIC inspection data. Vehicles that arrive for their annual inspection tend to have necessary maintenance performed prior to inspection.  Vehicles pulled over at roadside are more likely to be in normal operational mode; that is, possibly in need of repairs or maintenance.  The ratio of roadside failure rates to annual DEIC rates can provide an estimate of the deterrent factor involved in a secondary implementation.  The actual impact of deterrence will be measurable once failure rates stabilize a year or two after introduction of the new cutpoints.

The emission benefits of the cutpoint change are estimated as the sum of the repair benefits and the deterrent benefits.  The repair benefits are estimated as the product of: the projected increase in the annual DEIC failure rate, the repair rate, the secondary implementation mal-performance factor, the HDDV VMT, and the average MOBILE6 HDDV emission factors.  The secondary implementation mal-performance factors were derived by adjusting the mal-performance factors by the fractional change in cutpoints for each model year range.  The deterrent benefits were estimated by multiplying the repair benefits by the ratio of the roadside failure rate to annual DEIC failure rates for the cutpoint changes.

The following is a summary of the equations used for the New Jersey methodology.

- Estimation of the Emission Benefits for the Current Diesel I/M Program

Diesel I/M Benefits = Diesel I/M Repair Benefit (DRB) + Diesel I/M Initial Deterrent Benefit (DDB)

Diesel I/M Repair Benefit (DRB) Estimation:

DRB  =  Failure Rate *  Repair Rate * NJ Average Daily HDDV VMT  *  NJ HDDV Average Emission Factor From Mobile 6  *  Mal-performance Factor

Failure Rate = DEIC initial fail rates within a calendar year. (CY 2002)

Repair Rate = (RV + V1 + RIP) / IVF

where:

RV = number of repaired vehicles (pre-repair fail/post-repair pass during the initial inspection)

V1 = number of violation reinspections (pre-repair pass only)

RIP = number of reinspection passes (initially failed vehicles, tracked to a post-repair pass on a later inspection)

IVF = number of Initial Inspection Vehicle Fails (Pre repair fails + Violation reinspections*)

*Pre-repair pass only – A portion of violations came in repaired prior to inspection, but after the violation.

The data used to derive the fail and repair rates is from the DEIC periodic inspection database.

The Mal-performance Factor is the average fraction that emissions increase for vehicles that fail inspection.  The mal-performance factors used for the New Jersey interim methodology were derived from the CARB tables of mal-performance frequency and effects by model year and vehicle weight class.  The factors were weighted by model year and vehicle type.  Each criteria pollutant has its own mal-performance factor.

Diesel I/M Initial Deterrent Benefit (DDB) Estimation:

DDB = FRD * DRB

where:

FRD = Failure Rate Differential (ERF1 – ERF2)

ERF1 = pre-enforcement fail rate (actual tested vehicles)

ERF2 = post-enforcement fail rate (actual tested vehicles)

These fail rates are for roadside inspection data, pre and post enforcement of the I/M program.  Pre-enforcement roadside failure rates are based on roadside team data from April 1996 to January 1998.

- Estimation of Emission Benefits for the Proposed Cutpoint Change

Cutpoint Change Benefit = Repair Benefits Due to Cutpoint Change + Deterrent Benefits Due to Cutpoint Change

Both the repair and deterrent benefits can be expressed as a HDDV mileage rate that when multiplied by the NJ HDDV Average Emission Factor from MOBILE6 results in annual emission benefits.  These mileage rates are called “Secondary repair mileage” and “Secondary deterrent mileage” so that:

Cutpoint Change Benefit = (Secondary Repair Mileage + Secondary Deterrent Mileage) * NJ HDDV Average Emission Factor from MOBILE6

Secondary Repair Mileage = Projected Annual Inspection Failure Rate Increase * Repair Rate * Secondary Implementation Mal-performance Factor * NJ Average Daily HDDV VMT

Projected Annual Inspection Failure Rate and Repair Rate are calculated using data from the DEIC periodic inspection database.  The fail rates due to the cutpoint changes are based on an analysis of the opacity data.

Secondary Implementation Mal-performance Factor = Weighted Fleet Fraction by Model Year Range * Fraction of Opacity Change * Mal-performance Factor

Fraction of Opacity Change = (Old Cutpoint – New Cutpoint) / Old Cutpoint

Weighted Fleet Fraction by Model Year Range = Fraction of Fleet summed from beginning to end of cutpoint range.

Secondary deterrent mileage = Secondary repair mileage * (Ratio of Annual Inspection failure rate due to the cutpoint changes to Roadside Failure rate due to the cutpoint change)

5.	State - Municipal Waste Combustors

The NJDEP is considering proposing a NOx standard in the range of 100 to 150 ppm for municipal solid waste combustors (MSW), based upon the ability of selective non-catalytic reduction (SNCR) to reduce emissions more than are now being achieved.  New Jersey has five resource recovery facilities located in the following counties:  Essex, Union, Warren (Northern New Jersey/New York/Connecticut nonattainment area), Camden, and Gloucester (Southern New Jersey/Philadelphia nonattainment area).  There are 13 municipal waste combustors (MWC) at these five facilities.  The NJDEP anticipates that a NOx emission reduction of greater than 100 tons per year (67 tpy for May 1, 2009 through Dec 31, 2009) or 0.27 tons per day may be possible with this measure.

Table 3: Estimated Reductions from Municipal Waster Combustors

| Municipal Solid Waste Plant | Air Flow Rates in Actual Cubic Feet per Minute (ACFM) | Air Flow Rates in Dry Standard Cubic Feet per Minute (DSCFM) | Pounds per hour using 100 ppm NOx limit |  | Pounds per hour using 130 ppm NOx limit | Pounds per hour using 150 ppm NOx limit |
| --- | --- | --- | --- | --- | --- | --- |
| Essex | 237,900 | 60867 | 44 |  | 57 | 65 |
| Essex | 220,000 | 56287 | 40 |  | 52 | 60 |
| Essex | 229,000 | 58590 | 42 |  | 55 | 63 |
| Average ACFM | 228,967 | 58581 |  |  |  |  |
| Camden | 97,409 | 24922 | 18 |  | 23 | 27 |
| Camden | 99,064 | 25346 | 18 |  | 24 | 27 |
| Camden | 101,495 | 25967 | 19 |  | 24 | 28 |
| Average ACFM | 99,323 | 25412 |  |  |  |  |
| Union | 128,029 | 32756 | 23 |  | 30 | 35 |
| Union | 131,191 | 33565 | 24 |  | 31 | 36 |
| Union | 130,767 | 33457 | 24 |  | 31 | 36 |
| Average ACFM | 129,996 | 33259 |  |  |  |  |
|  |  | 0 | 0 |  | 0 | 0 |
| Warren | 50,005 | 12794 | 9 |  | 12 | 14 |
| Warren | 55,702 | 14251 | 10 |  | 13 | 15 |
| Average ACFM | 52,854 | 13523 |  |  |  |  |
|  |  | 0 | 0 |  |  |  |
| Gloucester | 62,820 | 16072 | 12 |  | 15 | 17 |
| Gloucester | 72,130 | 18454 | 13 |  | 17 | 20 |
| Average ACFM | 67,475 | 17263 |  |  |  |  |
| Total pounds per hour = |  |  | 296 |  | 385 | 444 |
| Total tons per year = |  |  | 1296 |  | 1685 | 1944 |
| Assumption: 15% oxygen in the stack and 300 degrees F stack temperature |  |  |  |  |  |  |
| Actual 2002 NOx in tons per year = | 1,803 | Expected reductions = |  | 507 | 118 |  |
| Permitted NOx = | 3,541 |  |  |  |  |  |

# The calculations of the estimated emission reductions, using the 130 ppm NOx limit as an example:

- Estimate emissions from each municipal solid waste plant in pounds per hour.

- Convert pounds per hour to tons per year:

(lbs/hr)(8760 hours/year)/2000 lbs/ton = tons/year

(385 lbs/hr)(8760 hours/year)/2000 lbs/ton = 1685 tons/year

- Calculate estimated emission reductions:

Expected reductions in emissions from MSW plants in tons/year =

(Actual 2002 NOx emissions from MSW plants in tons/year) – (Emission emissions from the MSW plants in tons/year)

= (1803 tons/year) – (1685 tons/year)

= 118 tons/year

The NJDEP estimated these benefits based upon permitted airflow rates at each facility.  These emissions are higher than the actual emissions reported by the facility because, in general, the facilities operate at air flow rates below the permitted level.  Also, the NJDEP anticipates that the facilities will decrease their emissions due to optimizing their existing NOx control systems (i.e., either injecting more ammonia or adding more nozzles).  The NJDEP also expects that Camden County Resource Recovery facility will need to install a new NOx control to comply with the proposed rules.  Therefore, the NJDEP expects that the estimated benefits will be even greater than those calculated.

6.	State - Petroleum Storage Tanks

New Jersey is working on proposing control measures for storage tanks which concentrates on the following storage operations; external floating roof tanks with slotted guide poles, degassing and interior tank cleaning operations, floating roof landing losses, and gasoline external floating roof tanks without domes.  These storage operations do not have any VOC emission controls.  The controls that will achieve reductions by 2009 include the external floating roof tanks with slotted guide poles and degassing and interior tank cleaning operations.  Anticipated emission reductions from controls for floating roof landing losses and gasoline external floating roof tanks without domes are not included in this SIP revision as they will occur post-2009 as the tanks are taken out of service.  The control measures and benefits under consideration for 2009 are summarized in Table 4.  For 2009, the total reductions are 2.25 tons per day (tpd) during the ozone season, and 464 tons per year (tpy), and they will come from the cleaning/degassing provisions of the rule (1.74 tpd) and the deck fitting retrofits for external floating roof tanks (0.51 tpd).

Table 4: Estimated 2009 VOC Benefits from Petroleum Storage Tank Control Measures

| Control Measure | Estimated 2009 Benefit (tons per day) |
| --- | --- |
| External floating roof tanks with slotted guidepole sleeves & covers | 0.51 |
| Tank degassing & cleaning | 1.74 |
| TOTAL | 2.25 |

Slotted guidepole sleeves and covers would be required for tanks storing VOC and/or refinery products with a vapor pressure of less than 11.0 psia on external floating roof tanks which amounts to about 118 tanks for New Jersey.  The base inventory for 2002 is 0.52 tpd (189 tpy).  Calculating growth and control from this measure would reduce this category to 0.01 tpd (2 tpy) for 2009 with a 99% reduction in VOC from controlling 50% of the 236 external floating roof tanks in New Jersey.  The emission reductions of approximately 0.51 tpd were allocated to each nonattainment area based on the ratio of storage tanks located in New Jersey in the Northern New Jersey/New York/Connecticut area (80%) and in the Southern New Jersey/Philadelphia nonattainment area (20%).

The control measure that New Jersey is considering for tank degassing and interior tank cleaning will have an approximate efficiency of 95%.  New Jersey has approximately 1000 floating roof tanks that store VOC and/or refinery products with a vapor pressure less than 11.0 psia.  This control measure is based on the number of tanks cleaned during the ozone season once every 10 years (or 100 tanks per year).  The base inventory for 2002 is 292 tpy (calculated over 159 days, because the requirements only apply during ozone season).  Calculating growth and control from this measure would reduce this category to 0.094 tpd (15 tpy) with a 95% reduction in VOC for 2009 (calculated for 159 days, because requirements only apply during ozone season).  Estimated emission reductions of 1.74 tpd were allocated to each nonattainment area based on the ratio of storage tanks located in New Jersey in the Northern New Jersey/New York/Connecticut area (80%) and in the Southern New Jersey/Philadelphia nonattainment area (20%).

- NOx Rule (2005)

The 2005 NOx rule was included in the modeling, but additional emission reductions were estimated in addition to those estimated for the modeling.  Estimated emission reductions from the New Jersey 2005 amendments to Subchapter 19 “Additional NOx controls” were calculated in 2001 by E.H. Pechan.  However, as discussed in the New Jersey rule proposal dated September 20, 2004, additional reductions were estimated from annual tune-ups for boilers.  The benefits of the NOx RACT rule (2005) included in the regional modeling were 7 tpsd (point and area sources) (see Appendix E); the benefits included in the New Jersey 2005 rule proposal were 13.3 tpsd.  The additional emission reductions, for point sources, are approximately 4.7 tpsd; these additional emission reductions were allocated to each nonattainment area based on location such that 3.1 tpsd of benefits are in the Northern New Jersey/New York/Connecticut area (66%) and 1.6 tpsd are in the Southern New Jersey/Philadelphia nonattainment area (34%).

8.	Portable Fuel Container Rules

New Jersey’s existing Portable Fuel Container rule, which was one of the six OTC 1-hour ozone shortfall measures discussed in Chapter 4, and the anticipated amendments to that rule discussed as a BOTW measure in Chapter 4, were included in the 2009 BOTW modeling.  However, additional emission reductions are anticipated beyond those estimated for the modeling.

Emissions from Portable Fuel Containers come from both area and non-road sources.  However, for New Jersey, only the estimated area source emission reductions for the 2005 Portable Fuel Container rule and the anticipated BOTW PFC rule amendments were included in the 2009 BOTW modeling.  For further details regarding the 2005 Portable Fuel Container rule calculations and additional details regarding the anticipated BOTW Portable Fuel Container amendment calculations see Appendix E.  The NJDEP has calculated the following additional reductions for the non-road sector:

- approximately 2.1 tpsd of VOC in 2009 for the 2005 PFC rule, and;

- approximately 0.3 tpd per summer day of VOC in 2009 for the anticipated BOTW PFC amendments.

The emissions were allocated to each nonattainment area based on the New Jersey nonattainment area ratios calculated for the 2005 PFC rule; this equates to approximately 1.7 tons per summer day VOC in the Northern New Jersey/New York/Connecticut nonattainment area and 0.7 tons per summer day in the Southern New Jersey/Philadelphia nonattainment area.

9.	ICI Boiler Rule

The BOTW ICI boiler rule, discussed in detail in Chapter 4, was included in the 2009 BOTW modeling.  However, additional emission reductions are anticipated beyond the 0.1 tpd that was estimated for the modeling.  Estimated emission reductions from the anticipated ICI boiler rule were calculated as discussed in Appendix E.  Based on these calculations, New Jersey estimates an additional 2 tpd NOx reductions beyond those included in the 2009 BOTW modeling – 1.4 tpd in the Northern New Jersey/New York/Connecticut nonattainment area and 0.6 tpd in the Southern New Jersey/Philadelphia nonattainment area.  The emission benefits were allocated to each nonattainment area based on the percent of point source emissions in each nonattainment area.

Ahmed, S.  SCS004C – Fluid Catalytic Cracking Unit (FCCU) in a Petroleum Refinery.  New Jersey Department of Environmental Protection, Stationary Combustion Sources Workgroup, April 11, 2007.

Ahmed, S.  SCS004D – Equipment Leaks at Petroleum Refinery.  New Jersey Department of Environmental Protection, Stationary Combustion Sources Workgroup, February 22, 2007.

Ahmed, S.  SCS004B – Flares in a Petroleum Refinery.  New Jersey Department of Environmental Protection, Stationary Combustion Sources Workgroup, February 22, 2007.

Ahmed, S.  SCS004A – Process Heaters & Boilers in a Petroleum Refinery.  New Jersey Department of Environmental Protection, Stationary Combustion Sources Workgroup, February 22, 2007.

Memorandum of Understanding Among the States of the Ozone Transport Commission Concerning the Incorporation of High Electrical Demand Day Emission Reduction Strategies into Ozone Attainment State Implementation Planning, March 2, 2007.

http://www.otcair.org/

NESCAUM.  Final Report: Heavy-Duty Engine Emissions in the Northeast, May 1997.

NESCAUM.  Final Report: Heavy-Duty Engine Emissions in the Northeast, May 1997.

http://www.arb.ca.gov/msei/on-road/downloads/tsd/HDT_Emissions_New.pdf

E.H. Pechan.  Control Measure Development Support Analysis of Ozone Transport Commission Model Rules.  Prepared for the Ozone Transport Commission by E.H. Pechan & Associates, Springfield, VA, 01.02.001/9408.000, March 31, 2001.

New Jersey Register 36 N.J.R. 4228(a); September 20, 2004.

MACTEC.  OTC Identification and Evaluation of Candidate Control Measures, Final Technical Support Document.  Prepared for the Ozone Transport Commission by MACTEC Federal Programs, Inc., Herdon, VA, February 28, 2007.

MACTEC.  Development of Emission Projections for 2009, 2012, and 2018 for NonEGU Point, Area, and Nonroad Sources in the MANE-VU Region, Final Technical Support Document.  Prepared for Mid-Atlantic Regional Air Management Association (MARAMA) by MACTEC Federal Programs, Inc., February 28, 2007.