Appendix D13

Additional Control Measures and Benefits Not Included in the Regional Modeling

1.	Introduction

Some of the control measures that New Jersey is planning to propose have emission benefits that were not included in the regional modeling effort.  There are several reasons why these benefits were not included: 1) the control measures could not be quantified at the time that regional modeling began and were thus not included; 2) the control measure is only being considered by New Jersey and therefore was not appropriate to include in regional modeling effort; 3) the emission benefits for the control measure included in the regional modeling were recalculated, and more emission benefits should have been included; and 4) the emission benefits do not take effect until later than 2009.

In an effort to quantify the air quality benefits that may result from these additional measures and additional emission reductions, New Jersey modified an existing USEPA methodology in order to help reduce the uncertainty that New Jersey will reach attainment by June 15, 2010.  Also included in this methodology are some additional emission benefits from rules that were already adopted.

The remainder of this section:

- outlines a methodology for making the conversion from emission reductions to air quality benefits;

- discusses each category of expected additional emission reductions;

- provides the total air quality benefits (in ppb) that New Jersey estimates from the implementation of these additional measures, or refinements to the modeled measures.

#### 2.	Methodology for Converting Emission Reductions to Air Quality Benefits

In order to relate emission reductions, as discussed in the previous section, to improvements in air quality, the emission reductions were converted to air quality benefits, i.e., ozone concentrations in ppb.

Methodology for quantifying the estimated improvement in air quality:

In calculating the 1999 shortfalls for 1-hour ozone SIPs, the USEPA established a simple method to estimate a change in air quality (ozone concentration) due to an emission reduction in each nonattainment area.  The relationship between emission reductions and the air quality improvement exists for an entire nonattainment area (it is not State specific).  New Jersey used essentially the same methodology used by the USEPA in 1999 to calculate the relationship between emission reductions and future 8-hour ozone concentrations.

The updated methodology involves using the 2002 and 2006 (interpolated) VOC and NOx emission inventories and actual monitoring data from the controlling monitoring site in each nonattainment area to calculate the ratio of the change in percent (%) ozone precursor concentration that would result in a change in the ozone concentration (ppb).  The method was updated by using new emission inventories and 8-hour instead of 1-hour ozone concentrations.  The NJDEP converted 2002 and 2009 regional emission inventory data expressed as tons per year (tpy) from the regional inventory compiled by MANE-VU to tpsd emissions to perform these calculations.  Worksheets that show the details of the calculations can be found in Attachment 1 to this Appendix.  Table D13.1 contains a summary of the methodology with a sample calculation.  The emission reduction ratios in the Northern New Jersey/New York/Connecticut nonattainment area are 29.0 tpd/ppb for VOC and 55.7 tpd/ppb for NOx.  The emission reduction ratios in the Southern New Jersey/Philadelphia nonattainment area are 8.7 tpd/ppb for VOC and 15.9 tpd/ppb for NOx.

    - Control Measures and Benefits

##### Additional control measures and additional benefits from existing control measures for New Jersey are listed in Table D13.2. The benefits of these measures will be seen by 2009 and are listed by nonattainment area.  Emission reductions in other States in New Jersey’s multi-state nonattainment areas are listed in Table D13.3.

##### 4.	Conversion of Benefits to Air Quality Improvements

##### Table D13.3 includes New Jersey emission benefits from Table D13.2, as well as emission benefits from the other States that we share a nonattainment area with for the High Electric Demand Day initiative.

Table D13.1: Methodology for Converting Estimated Emission Reductions

To Air Quality Benefits

| Methodology Description | Sample Calculation for Converting VOC Emission Reductions to Air Quality Benefits |
| --- | --- |
| a. The ratio of New Jersey’s 2002 inventory (in tpsd) to New Jersey’s 2002 inventory (in tpy) was calculated (tpsd/tpy ratio). | 979tpsd / 378,877tpy = 0.0026 |
| b. The 2002, 2006 (estimated by interpolation) and 2009 emission inventories were converted from tpy to tons per summer day (tpsd) by multiplying the 2002, 2006 and 2009 emission inventories from the portion of each state in our nonattainment areas, by the New Jersey tpsd/tpy ratio. | For New York: 318,692tpy X 0.0026 = 822tpsd |
| c. The estimated emission inventories (in tpsd) for 2002, 2006 and 2009 from each State or portion of each state in our nonattainment areas were totaled. The percentage difference between the 2002 emissions inventory and estimated 2006 emissions inventory (by nonattainment area) was calculated for each nonattainment area. | For Northern New Jersey/New York/Connecticut nonattainment area: (1,649tpsd – 1,360tpsd) / 1,649tpsd X 100% = 17.55% |
| d. The difference between the 2002 and the 2006 design values for 8-hour ozone at the controlling monitors for each nonattainment area was calculated for each nonattainment area. (10 ppb in the Northern New Jersey/New York/Connecticut nonattainment area and 13 ppb in the Southern New Jersey/Philadelphia nonattainment area). | 98ppb – 88ppb = 10ppb |
| e. An “emission reduction factor” was calculated for each nonattainment area by dividing the percentage difference between the 2002 emissions and the estimated 2006 emissions (c.), by the change in 8-hour ozone monitoring data at the controlling monitor (d.). The “emission reduction factor” is expressed as percent divided by ppb (%/ppb). | 17.55% / 10ppb = 1.76 |
| f. The emission reductions that caused a reduction in 8-hour ozone for each nonattainment area were calculated by multiplying the “emission reduction factor” (e.) calculated for each nonattainment area by the 2002 emission inventory for the entire nonattainment area, in tpsd. (divided by 100). | 1.76 X 1,649tpsd / 100% = 28.9tpsd |

Table D13.2: Estimated Calculated Benefits from Control Measures and Other Additional Benefits not Included in the Regional Modeling

|  |  |  | NY NAA |  |  | PHL NAA |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Measure |  |  | VOC | NOx |  | VOC | NOx |
| Regional Control Measures |  |  |  |  |  |  |  |
| New Jersey RACT – refinery efforts |  |  | 0.8 | 1.6 |  | 0.8 | 1.6 |
| High Electrical Demand Day Initiative (HEDD) |  |  | 0 | 14 |  | 0 | 6.1 |
| State Control Measures |  |  |  |  |  |  |  |
| Diesel idling rule changes |  |  | 0 | 3 |  | 0 | 3 |
| Diesel cutpoint rule changes |  |  | 0.2 | 0.1 |  | 0.2 | 0.1 |
| Case by case VOC and NOx emission limit determinations (1) |  |  |  |  |  |  |  |
| Municipal Waste Combustors |  |  | 0 | 0 |  | 0 | 0.8 |
| New Jersey RACT – storage tank |  |  | 1.7 | 0 |  | 0.4 | 0 |
| USEPA CTGs: Flexible Packaging Printing Materials |  |  | 0.05 | 0 |  | 0.05 | 0 |
| USEPA CTGs: Offset Lithographic and Letterpress Printing Materials |  |  | 0.2 | 0 |  | 0.2 | 0 |
| USEPA CTGs: Flat Wood Paneling Coatings |  |  | 0 | 0 |  | 0 | 0 |
| Federal Control Measures |  |  |  |  |  |  |  |
| USEPA Small Nonroad Engine Rule (1) |  |  |  |  |  |  |  |
| Additional Benefits from modeled measures |  |  |  |  |  |  |  |
| NOx RACT (2005) (2) |  |  | 0 | 3.1 |  | 0 | 1.6 |
| PFCs (2005 rule) |  |  | 1.5 | 0 |  | 0.6 | 0 |
| PFCs (amendments) |  |  | 0.2 | 0 |  | 0.1 | 0 |
| New ICI Boiler Rule |  |  | 0 | 1.4 |  | 0 | 0.6 |
| New Source Review (1) |  |  |  |  |  |  |  |
| TOTAL |  |  | 4.8 | 22.9 |  | 2.4 | 13.8 |

- The benefits of this control strategy cannot be quantified at this time.

- Additional emission reductions not included in the modeling.

Table D13.3: Estimated Additional Reductions and Control Measures

Converted to Air Quality Improvements (tons per day)

|  | Northern New Jersey/New York/Connecticut Nonattainment Area |  | Southern New Jersey/Philadelphia Nonattainment Area |  |
| --- | --- | --- | --- | --- |
| Control Measures | VOC | NOx | VOC | NOx |
| New Jersey: HEDD | 0.0 | 13.7 | 0.0 | 6.1 |
| Other New Jersey Emission Reductions | 4.8 | 9.2 | 2.4 | 7.7 |
| New York: HEDD | 0 | 51 | - | - |
| Connecticut: HEDD | 0 | 12 | - | - |
| Pennsylvania: HEDD | - | - | 0 | 21.8 |
| Delaware: HEDD | - | - | 0 | 7 |
| Maryland: HEDD | - | - | 0 | 24 |
| Nonattainment Area Totals | 4.8 | 85.9 | 2.4 | 66.6 |
| Tons per day Reductions for a 1 ppb O3 Reduction (tpd/ppb) | 29.0 | 55.7 | 8.7 | 15.9 |
| O3 Reductions from the measures listed in this table (ppb) | 0.2 | 1.5 | 0.3 | 4.2 |
| NJDEP, Air Quality Planning |  |  |  |  |

    - Description of Control Measures and Benefits

a.	Regional Control Measures

The control measures identified under the Regional heading in Table D13.2 are described in Chapter 4.  New Jersey, in conjunction with the regional organizations - OTC for the High Electrical Demand Days (HEDD) effort and MARAMA for the Refinery Rules - calculated the benefits from these efforts for each New Jersey associated nonattainment area. The refinery rule benefits are included for New Jersey only; the HEDD benefits include the other States that share a nonattainment area with New Jersey and are listed in Table 13.3.  Detailed discussion of the benefit calculations for refineries and for the HEDD initiative is found in Attachment 2 of this Appendix.

b.	State Control Measures

Each of the New Jersey control measures identified in Table D13.2 is described in Chapter 4.  Detailed discussion of the benefit calculations for diesel idling rule changes, diesel cutpoint rule changes, municipal waste combustors and petroleum storage tank rule can be found in this Attachment 2 of this Appendix.

In New Jersey, facilities that have sources with the potential-to-emit NOx or VOC above Reasonably Available Control Technology (RACT) specified thresholds, and for which no 8-hour ozone RACT source limit has been established, will be required to develop facility specific emission limits (FSELs) in accordance with regulations that New Jersey is considering proposing.  Control technologies have advanced sufficiently over the last several years to warrant the reevaluation of these case-by-case, or source-specific, determinations.  However, the NJDEP staff were not able to estimate additional specific emission benefits for this control strategy in time to include in this SIP revision.

Subsequent to the proposed SIP revision, for the new CTGs, the NJDEP staff has evaluated current sources of emissions to determine if New Jersey has applicable sources, and whether or not New Jersey’s existing rules are more stringent than the new CTGs.  The NJDEP staff calculated a specific emission benefit for these control strategies but they are not being relied upon in this SIP revision.  These reductions are listed in Table 13.2.  However, these measures are expected to provide measurable emission reductions by 2009, although they must go through the rulemaking process.

c.	Federal Control Measures

The Federal control measure identified in Table D13.2 is described in Chapter 4.  On April 17, 2007, the USEPA proposed a new nonroad engine rule that contains compliance dates that would result in some emission benefits in 2009.  However, like the State case by case VOC and NOx emission limit determinations, and the new Control Technique Guidance (CTG) efforts, the USEPA and the State are not able to estimate a specific emission benefit for this control strategy in time to include in this SIP revision.

d.	Additional Benefits from Modeled Measures

For several of the control measures that were included in the 2009 BOTW modeling (either as a OTB measure or a BOTW measure), New Jersey revisited the benefit calculations, either as part of its Reasonable Further Progress (Chapter 6) exercise or in support of a rulemaking proposal effort.  In all of these cases, New Jersey determined that the emission reductions are greater than those that were included in the 2009 BOTW modeling.  Attachment 2 of this Appendix describes these calculation refinements.

Using this modified conversion methodology, the VOC emission reductions expected from the control measures and refinements discussed in this section result in an additional 0.2 ppb reduction in ozone in the Northern New Jersey/New York/Connecticut nonattainment area and an additional 0.3 ppb reduction in the Southern New Jersey/Philadelphia nonattainment area.  The NOx emission reductions expected from the control measures and refinements discussed in this section result in an additional 2 ppb and 4 ppb reduction in 8-hour ozone in the Northern New Jersey/New York/Connecticut nonattainment area and the Southern New Jersey/Philadelphia nonattainment area, respectively.  In total, New Jersey could experience a 0.2 – 2 ppb decrease in ozone concentrations in the Northern part of the State, and a 0.3 – 4 ppb decrease in ozone concentrations in the Southern part of the State.  Worksheets that contain the details for performing these conversions can be found in Attachment 1 to this Appendix.

Appendix  D13

Attachment 1: Methodology for Quantifying the Estimated Improvement in Air Quality

Worksheets that show the details of the calculations for quantifying the estimated improvement in air quality are available electronically upon request.

USEPA Region II. Technical Support Document: Modeling for the Trenton, New Jersey Portion of the Philadelphia Ozone Nonattainment Area. United States Environmental Protection Agency (USEPA), Region II, December 14, 1999

OTC.  Memorandum of Understanding Among the States of the Ozone Transport Commission Concerning the Incorporation of High Electrical Demand Day Emission Reduction Strategies into Ozone Attainment State Implementation Planning, March 2, 2007.