CONTROL MEASURE:

CVISN (Commercial Vehicle Information Systems and Networks)

DESCRIPTION

Intelligent Transportation Systems (ITS) are transportation systems, which utilize information, communication, sensor, and control technologies to achieve improved levels of performance. One of the elements of the National ITS Program Plan includes the ITS technologies which uniquely support Commercial Vehicle Operations (CVO). The term commercial vehicle information systems and networks (CVISN, pronounced see’ vision) refers to the ITS information system elements that support CVO. CVISN includes information systems owned and operated by governments, carriers, and other stakeholders. The goal of the CVISN program is to improve the safety and efficiency of commercial vehicle operations by creating a way for existing and newly designed systems to exchange information through the use of standards and available communications infrastructure.

The Federal Motor Carrier Safety Administration (FMCSA) is using Commercial Vehicle Information Systems and Networks (CVISN) “levels” to allow definition of a specific set of capabilities that can be deployed incrementally by a state and its motor carriers. The level definitions include capabilities a state would deploy, capabilities motor carriers in a state would deploy, and capabilities for several critical national systems, referred to as core infrastructure systems.

The current, primary objective of the CVISN Program is to develop and deploy information systems that will support new capabilities in three areas:

- Safety Information Exchange:

The CVISN Safety Information Exchange capability area is intended to provide improved electronic exchange of Motor Carrier Management Information System (MCMIS) and other safety information among roadside and desk side, state and federal systems.

- Credentials Administration:

The central concept for this capability area is to allow motor carriers to apply for, pay for, and receive credentials electronically.

- Electronic Screening:

The electronic screening aspect of CVISN is to automatically screen vehicles as they approach weigh stations using WIM scales and allow those that are safe and legal to bypass without slowing down or stopping.

IMPLEMENTATION

Truck traffic has grown dramatically over the past 20-30 years. Currently, many weigh stations do not have the capacity to handle existing levels of truck traffic. As truck arrivals exceed the operational capacity of weigh stations, queues develop and drivers are delayed. Often the backups require stations to close to avoid safety risks.

CVISN enables the electronic transmission of data between commercial vehicles and weigh stations. These systems employ trucks equipped with transponders that electronically communicate with Automated Vehicle Identification (AVI) readers located near the weigh stations. The AVI readers identify the transponder-equipped trucks, verifying their size, weight, and credentials. When the information is read and verified, the trucks receive a signal in the cab, either visual, audible or both. The signal directs the driver to either continue past the weigh station or to enter the station (for a random inspection or credential verification).

Electronic Screening of commercial vehicles on the mainline highway permits compliant vehicles to bypass the weigh station, therefore increasing operational efficiency and capacity.

ANALYSIS ASSUMPTIONS AND METHODOLOGY

Analysis was performed for 10,000 vehicles with CVISN capability per weigh station per county. Weigh stations located along an interstate highway with high truck traffic volume.  The following assumptions were made for the analysis:

- Assume that 100% of truck screening is done via electronic screening.

- Assume one lane truck screening

- Length of service lane = 1017ft and Length of upstream roadway = 1017ft

Benefits were analyzed for the truck idling reduced and for the change in speeds at for the section of the road affected by electronic screening.  NJAQONE Ver 3.0 was used to calculate benefits from reduction of truck idling.  Truck emission factors were calculated for the before and after average speeds at the roadway section.

CONCULSION AND RESULTS

One of the benefits of implementing CVISN in a state is that it would reduce delays and congestion on highways due to CVO.  Improving safety, administration, screening and enforcement of commercial vehicle operations will reduce the number of accidents on the highways whereby reducing vehicle delay and congestion. This can amount to a notable impact on the on-road vehicle emissions, although, it is a very difficult task to estimate this impact due to the broadness of the program and its distinctive effectiveness in each state.  The emission benefits resulting from electronic screening are as below:

Emission Reductions (Statewide):

| Pollutant Type | ## Analysis Year 2010 |  |
| --- | --- | --- |
|  | Tons/Day | Tons/Year |
| # VOC | 0.0464 | 8.799 |
| NOx | 0.9953 | 335.067 |
| PM2.5 | 0.0301 | 8.702 |
| SO2 | 0.0002 | 0.094 |

*Note: Negative means increase in emissions

Ozone Analysis – Daily Benefits in New Jersey

| Benefits (Reductions) | ## Ozone Non-Attainment Area |  |
| --- | --- | --- |
|  | New York-N. New Jersey-Long Island, NY-NJ-CT | Philadelphia-Wilmington-Atlantic City PA-NJ-MD-DE |
| Idling (Hrs/Day) | 4,496 | 3,372 |
| VOC (Tons/Day) | 0.0261 | 0.0204 |
| NOx (Tons/Day) | 0.5688 | 0.4266 |

*Note: Negative means increase in emissions

PM2.5 Analysis – Annual Benefits in New Jersey

| Benefits (Reductions) | ## PM2.5 Non-Attainment Area |  |
| --- | --- | --- |
|  | New York-N. New Jersey-Long Island, NY-NJ-CT | Philadelphia-Wilmington PA-NJ-DE |
| # Idling (Hrs/Year) | 39,030 | 11,709 |
| NOx (Tons/Year) | 160.0886 | 47.5207 |
| PM2.5 (Tons/Year) | 4.1573 | 1.2342 |
| SO2 (Tons/Year) | 0.045 | 0.0135 |

*Note: Negative means increase in emissions

COST/BENEFIT ANALYSIS

Electronic screening equipment at the roadside typically consists of automatic vehicle identification (AVI), weigh-in-motion (WIM) scales, signage, workstations, and roadside telecommunications. As per the FHWA’s study in Kentucky (EDL Call #13677) CVISN electronic screening at initial site will cost approximately $522,525 and the annual O&M cost per CVISN electronic screening site is $217,646.

Total costs were estimated as follows:

Capital Cost = $522,525 over 15 years

Annual Operating/Maintenance Cost = $217,646/Year

Total Annualized Cost = $252,481

Total cost for the various non-attainment areas are as follows:

| County | Estimated Cost For Each Non-Attainment Area |  |  |  |
| --- | --- | --- | --- | --- |
|  | Ozone |  | PM 2.5 |  |
|  | NY-NJ-CT | PA-NJ-DE | NY-NJ-CT | PA-NJ-DE |
| Atlantic | - | $252,481 | - | - |
| Bergen | $252,481 | - | $252,481 | - |
| Burlington | - | $252,481 | - | $252,481 |
| Camden | - | $252,481 | - | $252,481 |
| Cape May | - | $252,481 | - | - |
| Cumberland | - | $252,481 | - | - |
| Essex | $252,481 | - | $252,481 | - |
| Gloucester | - | $252,481 | - | $252,481 |
| Hudson | $252,481 | - | $252,481 | - |
| Hunterdon | $252,481 | - | - | - |
| Mercer | - | $252,481 | $252,481 | - |
| Middlesex | $252,481 | - | $252,481 | - |
| Monmouth | $252,481 | - | $252,481 | - |
| Morris | $252,481 | - | $252,481 | - |
| Ocean | - | $252,481 | - | - |
| Passaic | $252,481 | - | $252,481 | - |
| Salem | - | $252,481 | - | - |
| Somerset | $252,481 | - | $252,481 | - |
| Sussex | $252,481 | - | - | - |
| Union | $252,481 | - | $252,481 | - |
| Warren | $252,481 | - | - | - |
| Total | $3,029,772 | $2,272,329 | $2,524,810 | $757,443 |

Cost/Benefit Results

| ## Pollutant | ## Cost/Benefit for Each Pollutant |  |  |  |
| --- | --- | --- | --- | --- |
|  | Ozone |  | PM 2.5 |  |
|  | NY-NJ-CT | PA-NJ-DE-MD | NY-NJ-CT | PA-NJ-DE |
| VOC ($/Ton/Day) | $464,555 | $446,445 | - | - |
| NOx ($/Ton/Day) | $21,308 | $21308 | - | - |
| NOx ($/Ton/Year) | - | - | $15,771 | $15,939 |
| PM2.5 ($/Ton/Year) | - | - | $607,321 | $613,721 |
| SO2 ($/Ton/Year) | - | - | $56,138,949 | $56,138,949 |

* Daily Costs used in calculation of cost/benefit ratio are Annual costs divided by 250