Control Measure:

Encourage the use of transit by not increasing fares despite inflationary pressures.

Description:

Transit ridership is dependant on many factors including out-of-pocket costs such as transit fares.  One way to achieve greater ridership would be to limit or eliminate fare increases such that the real cost to transit riders goes down relative to inflation and other modes.  This would encourage more people to switch modes resulting in air quality benefits.

Methodology:

The modeling tools and procedures used to predict the interaction between transit fares and ridership generally do not reflect the impact of inflation.  Rather than modeling no fare increase directly, a fare decrease equal to inflation was applied to transit fares while all other transportation costs were held constant.  Inflation was assumed to be

Each of the MPO/Ozone Non-Attainment areas was analyzed using available tools, data and assumptions.  While using the regional transportation models would have been optimal, time and data requirements made this option impractical for the purposes of this study.  Instead, the following models and procedures were applied:

NJTPA Region

In this region a simplified, spreadsheet version of the NJ Transit Regional Transportation Demand Model was used to estimate the change in ridership.  This highly automated spreadsheet know as Quick-Share was recently updated to reflect the assumptions, constants and coefficients found in the most recent version of the full NJ Transit regional model.  Quick-Share works by looking at individual transit markets and then estimating the change and market share based on changes in the input values such as transit headways, available modes and transit pricing.  The entire northern New Jersey ozone non-attainment area was divided into 9 different markets blanketing the entire region.  The definition and the size of these markets were supplied by NJ Transit and correspond to existing market definitions used by the agency.  Samples of the Quick-Share inputs and results can be found at the end of this document.  Overall results were as follows:

Total Trips Reduced Region wide : 7,475

Total Vehicle Trips Reduced Region wide: 6,399

Total Vehicle Miles of Travel Reduced Region Wide: 133,407

DVRPC and South Jersey Regions

In order to calculate the impact of a no-fare-increase policy in these areas, a simple price elasticity was used.  Current ridership for various markets in the southern New Jersey were obtained from NJ Transit.  A fare elasticity of -0.25 used for both the DVRPC and South Jersey regions as suggested by NJ Transit.  The resulting increase in transit trips was divided by the average vehicle occupancy in the region to get the total number of trips reduced, and the average trip length for trips in the region was used to calculate the total vehicle miles of travel eliminated.  A summary of these calculations can be found in Table 1.  The VT and VMT reductions attributed to DVRPC and SJTPO region are shown in Table 2.

Distribution of VT/VMT Reductions to Counties and Emissions Analysis

Once the VT and VMT reductions by MPO had been established, these reductions were further distributed to the county level.  This distribution was developed from a weighted average combining the transit mode share and population in each county.  Table 3 summarizes the results of this redistribution.  Finally, the VT and VMT reductions were used as input into the Emissions Only analysis found in the NJ-AQONE software package.  Peak summer day and annual emission reductions were calculated by the software for every county, and the county data was aggregated into non-attainment areas.  The results are shown in the following Tables:

Results: Statewide Summer Day/Annual Benefits

Total VT Reduced = 8,222 trips/day Statewide

Total VMT Reduced = 148,263 miles/day Statewide

Emission Reductions (Statewide):

| Pollutant Type | ## Analysis Year 2009 |  |
| --- | --- | --- |
|  | Tons/Day | Tons/Year |
| # VOC | 0.07 | - |
| NOx | 0.08 | 19.15 |
| PM2.5 | - | 0.54 |
| SO2 | - | 0.33 |

Ozone Analysis – Daily Benefits in New Jersey

| Benefits (Reductions) | ## Ozone Non-Attainment Area |  |
| --- | --- | --- |
|  | New York-N. New Jersey-Long Island, NY-NJ-CT | Philadelphia-Wilmington-Atlantic City PA-NJ-MD-DE |
| # VT | 6,399 | 1,283 |
| # VMT | 133,406 | 10,464 |
| # VOC (Tons/Day) | 0.066 | 0.0057 |
| NOx (Tons/Day) | 0.071 | 0.0055 |

PM2.5 Analysis – Annual Benefits in New Jersey

| Benefits (Reductions) | ## PM2.5 Non-Attainment Area |  |
| --- | --- | --- |
|  | New York-N. New Jersey-Long Island, NY-NJ-CT | Philadelphia-Wilmington PA-NJ-DE |
| # VT | 1,607,750 | 115,750 |
| # VMT | 32,949,750 | 944,250 |
| NOx (Tons/Year) | 17.55 | 0.538 |
| PM2.5 (Tons/Year) | 0.49 | 0.017 |
| SO2 (Tons/Year) | 0.30 | 0.010 |

Cost/Benefit Analysis

Total costs of the program were as follows:

Lost revenue	= 11.5% x Current Total farebox Reveneues

= 11.5% x $530.6 million

= $61.02 Million

Average Fare Paid	= Total Farebox Revenues / Ridership

= $530.6 Million / $237.4 Million Riders

= $2.24 / Rider Currently

-11.5% (to account for no fare increase)

= $1.98/New Rider

New Riders: 	NJTPA: 7,475

DVRPC & SJTPO: 2,497

Total: 9,972 Additional Riders

Total Additional Revenue = 9,972 * $1.98 = $19,744.56

Net Cost = $61.02 Million - $19,745 = $61.0 Million

Total costs of the program would be incurred across the state, even in areas not part of a particular non-attainment area.  Total costs were proportioned by the expected vehicle trip reductions within the respective non-attainment areas:

For Ozone

83.3% to the NY-NJ-CT non-attainment area or $50.8 Million

16.7% to the PA-NJ-DE-MD non-attainment area $10.2 Million

For PM2.5

93.3% to the NY-NJ-CT non-attainment area or $56.9 Million

6.7% to the PA-NJ-DE non-attainment area or $4.1Million

Cost/Benefit Results

| Pollutant | Cost/Benefit for Each Pollutant |  |  |  |
| --- | --- | --- | --- | --- |
|  | Ozone |  | PM 2.5* |  |
|  | NY-NJ-CT | PA-NJ-DE-MD | NY-NJ-CT | PA-NJ-DE |
| VOC ($/Ton/Day) | $764,862,689 | $1,786,818,327 | - | - |
| NOx ($/Ton/Day) | $717,569,977 | $1,857,653,422 | - | - |
| NOx ($/Ton/Year) | - | - | $810,349,284 | $1,904,820,725 |
| PM2.5 ($/Ton/Year) | - | - | $28,932,807,672 | $58,922,210,876 |
| SO2 ($/Ton/Year) | - | - | $47,752,781,465 | $98,846,259,998 |

*Annual Costs used in calculation of cost/benefit ratio are daily costs multiplied by 250

Table 1: Summary of Fare Elasticity Calculation for the DVRPC and SJTPO Regions

| Market | Riders | Fair Change | Elasticity | New Ridership | Avg. Veh. Occupancy | Veh. Trips Reduced | Average Trip Length | VMT Reduced |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SJ Local Bus | 39,200 | -11.5% | -0.25 | 40,327 | 1.37 | 823 | 8.15 | 6,704 |
| SJ Commuter Bus | 15,100 | -11.5% | -0.25 | 15,534 | 1.37 | 317 | 8.15 | 2,583 |
| Seashore Bus | 700 | -11.5% | -0.25 | 720 | 1.37 | 15 | 8.15 | 120 |
| Phil Commuter Bus | 27,900 | -11.5% | -0.25 | 28,702 | 1.37 | 585 | 8.15 | 4,772 |
| AC Rail (Summer) | 3,960 | -11.5% | -0.25 | 4,074 | 1.37 | 83 | 8.15 | 677 |

Table 2: Distribution of VT/VMT Benefits to MPO Regions

| For NJTPA | Veh Trips Reduced | VMT Reduced |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 6,399 | 133,407 |  |  |  |  |  |  |  |
| For DVRPC and SJTPO |  |  |  |  |  |  |  |  |  |
| Market | New Riders | Veh Trips Reduced | VMT Reduced | DVRPC |  |  | SJTPO |  |  |
|  |  |  |  | Percent in MPO | Veh Trips Reduced | VMT Reduced | Percent in MPO | Veh Trips Reduced | VMT Reduced |
| SJ Local Bus | 1,127 | 823 | 6,704 | 70% | 576 | 4,693 | 30% | 247 | 2,011 |
| SJ Commuter Bus | 434 | 317 | 2,583 | 30% | 95 | 775 | 70% | 222 | 1,808 |
| Seashore Bus | 20 | 15 | 120 | 0% | 0 | 0 | 100% | 15 | 120 |
| Phil Commuter Bus | 802 | 585 | 4,772 | 80% | 468 | 3,817 | 20% | 117 | 954 |
| AC Rail (Summer) | 114 | 83 | 677 | 50% | 42 | 339 | 50% | 42 | 339 |
|  |  |  |  | Total: | 1,181 | 9,624 | Total: | 642 | 5,232 |

Table 3: Distribution of VT/VMT Reductions By County

| County | Percent of Total MPO Transit Trips | VT Reduction | VMT Reduction |
| --- | --- | --- | --- |
| Bergen | 14.79% | 946 | 19,725 |
| Essex | 21.18% | 1,355 | 28,253 |
| Hudson | 28.33% | 1,813 | 37,800 |
| Hunterdon | 0.26% | 17 | 349 |
| Middlesex | 9.62% | 616 | 12,839 |
| Monmouth | 7.42% | 475 | 9,897 |
| Morris | 2.57% | 165 | 3,432 |
| Ocean | 1.54% | 98 | 2,052 |
| Passaic | 5.13% | 328 | 6,842 |
| Somerset | 1.88% | 121 | 2,514 |
| Sussex | 0.35% | 22 | 466 |
| Union | 6.78% | 434 | 9,042 |
| Warren | 0.15% | 9 | 195 |
| NJTPA | 100.00% | 6,399 | 133,407 |
| Atlantic | 53.87% | 346 | 2,819 |
| Cape May | 19.13% | 123 | 1,001 |
| Cumberland | 21.49% | 138 | 1,124 |
| Salem | 5.51% | 35 | 288 |
| SJTPO | 100.00% | 642 | 5,232 |
| Burlington | 14.45% | 93 | 756 |
| Camden | 50.21% | 322 | 2,627 |
| Gloucester | 7.54% | 48 | 394 |
| Mercer | 27.81% | 178 | 1,455 |
| DVRPC (NJ) | 100.00% | 1,181 | 9,624 |

Sample Input Screen from Quickshare