Control Measure:

Retrofit Switch Yard Locomotives (SYL)

Description:

Both 2-stroke and 4-stroke switch yard locomotives in a total of 10 switch yards throughout the ozone nonattainment area and 9 yards in the PM 2.5 nonattainment area will utilize retrofit and idle reduction equipment to reduce emissions from 5 engines per yard.

Documentation:

- Current operable switch yard locations found at: www.conrail.com/freight.htm

- U.S. Environmental Protection Agency, “Guidance for Quantifying and Using Long Duration Switch Yard Locomotive Idling Emission Reductions in State Implementation Plans,” Office of Transportation and Air Quality, January, 2004.

- U.S. Environmental Protection Agency, “Case Study: Chicago Locomotive Idle Reduction Project,” Office of Air and Radiation, March, 2004.

- Department of Energy, Energy Information Administration (EIA), Retail Gasoline Prices:

http://www.eia.doe.gov/oil_gas/petroleum/data_publications/wrgp/mogas_home_page.html

Inputs/Assumptions:

- SYL operates 90% of the days out of the year, or 329 days.  The industry average is 92%

- SYL idles 10 hours per day, 3,000 hours annually.

- All idle hours can be impacted by idle reduction technology.

- There are 10 counties in the ozone nonattainment area with at least one yard.

- There are 9 counties in the PM2.5 nonattainment area with at least one yard.

- Each yard will retrofit 5 engines: 3, 4-stroke and 2, 2-stroke.

- The current documentation does not include any guidance on calculating VOC or SO2 emissions or benefits.

- The cost of the idle reduction technology is spread over the assumed life of 6 years.

- Retrofit technology for each SYL is assumed to cost $30,000.

- Gasoline cost = $2.40/gallon

- The average SYL consumes 0.8 gallons of fuel per hour of idling.

- Emission factors:

|  | 2-Stroke (g/hr) | 4-Stroke (g/hr) | APU (g/hr) |
| --- | --- | --- | --- |
| NOx | 800 | 620 | 71 |
| PM2.5 | 26 | 32 | 5.7 |

Results:

Ozone Analysis – Daily Benefits (tpd) in New Jersey:

| Benefits (Reductions) | ## Ozone Non-Attainment Area |  |
| --- | --- | --- |
|  | NY-NJ-CT | PA-NJ-MD-DE |
| # VOC (Tons/Day) | NA | NA |
| NOx (Tons/Day) | 0.17 | 0.17 |

PM2.5 Analysis – Annual Benefits (tpy) in New Jersey:

| Benefits (Reductions) | ## PM2.5 Non-Attainment Area |  |
| --- | --- | --- |
|  | NY-NJ-CT | PA-NJ-DE |
| NOx (Tons/Year) | 61.61 | 30.80 |
| PM2.5 (Tons/Year) | 2.37 | 1.19 |
| SO2 (Tons/Year) | NA | NA |

Cost Benefit Results:

| Nonattainment Area | Ozone ($/tpd) |  |
| --- | --- | --- |
|  | VOC | NOx |
| NY-NJ-CT | NA | $ (585) |
| PA-NJ-MD-DE | NA | $ (585) |
| Statewide | NA | $ (585) |

| Nonattainment Area | PM2.5 ($/tpy) |  |  |
| --- | --- | --- | --- |
|  | NOx | PM2.5 | SO2 |
| NY-NJ-CT | $ (370) | $ (9,616) | NA |
| PA-NJ-DE | $ (370) | $ (9,616) | NA |
| Statewide | $ (370) | $ (9,616) | NA |

* Parentheses denote a cost savings.