Control Measure:

Transit Bus Retrofit

Description:

Older, higher-emitting buses will be fitted with emissions reducing retrofit technology.

Documentation:

N/A

Methodology:

All model year 1992-2002 buses in the fleet will be equipped with retrofit technology. Retrofitting model year 1992-2002 buses is assumed to have the greatest potential to reduce emissions due to existing regulations and fleet age.

Inputs/Assumptions:

- A Lubrizol Engine Control Systems, Purifilter Diesel Particulate Filter would be used to retrofit the buses.

  - This filter is a verified EPA retrofit technology and is estimated to reduce VOC emissions by 85% and PM emissions by 90%.

  - The filter has no affect on NOx emissions.

- All model year 1992-2002 buses in the fleet will be equipped with retrofit technology.

- Retrofitting model year 1992-2002 buses is assumed to have the greatest potential to reduce emissions due to existing regulations and fleet age.

- 34.4% of all transit bus VMT (both daily and annual) can be attributed to model year 1992-2002 buses.

- Because no estimates could be found on the affect of retrofit technology on SO2 emissions, a zero benefit was assumed.

- MOBILE6.2 was used to generate emission factors and VMT.

Results:

Total Affected VMT:

Daily

Model Years 1992-2002 = 38,654 miles/day

Annual

Model Years 1992-2002 = 14,453,822 miles/year

Benefits (Emission Reductions):

| Pollutant Type | ## Analysis Year 2009 |  |
| --- | --- | --- |
|  | tons/day | tons/year |
| # VOC | 0.02 | 1.54 |
| NOx | 0.00 | 0.00 |
| PM2.5 | 0.00 | 4.93 |
| SO2 | NA | 0.00 |