Control Measure:

This project covers the expansion of existing bicycle facilities and the creation of entirely new facilities throughout the state.  This will encourage bicycle use for work and other trip purposes, replacing some motorized trips and leading to a reduction in emissions

Description:

Counties throughout the state of New Jersey have a number of programmed (funded) and proposed bicycle facilities.  These new and expanded facilities will help to attract shorter distant trips away from autos and will have a corresponding reduction in emissions.  Both commuter trips and recreational trips will be attracted to these new facilities.

Methodology:

The NJ-AQONE software package, specifically the “Bicycle Improvements” project type, was used in the evaluation of the impact of the proposed bike trails.  Separate runs were done for each county to reflect local conditions.

One of the major inputs into the analysis was the length of the existing bicycle facilities and the total length of new facilities to be created.  The GIS based inventory data supplied with the NJDOT Bicycle/Pedestrian Master Planning Phase Two CD ROM was used for this Information.  These bicycle facility inventory databases contain the follow information for both on and off road facilities:

- A list of know, existing facilities

- A list of programmed (funded) future facilities

- A list of proposed (unfunded) future facilities

For the purposes of this analysis, the total amount of existing bike trails was set equal to the total on-road and off-road recorded in the database.  For the length of the future (2009) bicycle trails, all of the programmed and 10% of the proposed trails were included.  The service area captured by the proposed bike trails was assumed to be a 1 mile wide band along the total length of new the trails.

Inputs/Assumptions:

- The length of the existing and future bicycle trails was obtained from the NJDOT Bicycle/Pedestrian Master Plan Phase 2 Facility Inventory and is summarized in Table 1

- Project impacts were estimated using the NJ-AQONE Bicycle Improvements methodology.  This methodology estimates the total number of commuter and recreational trips that the proposed facility would attract currently using auto

- The length of the additional trails was assumed to include all programmed (funded) facilities and 10% of proposed facilities.  The relatively low 10% figure was based on the assumption that most of the proposed trails, if built at all, would not be completed within the next 4-5 year (the timeframe of the RACM analysis)

- For the remaining inputs, the default values supplied by the NJAQONE software were used

- The service area (an input required by the software) was approximated by multiplying the total length of the new facilities by 1 mile, yielding an estimate of the total service area

- Each county was analyzed separately and results totaled statewide and by non-attainment area (both Ozone and PM 2.5).  Output from the software is found at the end of this documentation.

| Table 1: Inventory of Existing, Programmed and Proposed Bike Trails in New Jersey |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| County | Existing (Miles) | Programmed (Miles) | Proposed (Miles) | 10% of Proposed (Miles) | Total Effective Future (Miles) | Estimated Service Area (Miles) |
| Atlantic | 38.78 | 14.90 | 191.44 | 19.14 | 34.05 | 36.05 |
| Bergen | 15.22 | 0.00 | 0.00 | 0.00 | 0.00 | 2.00 |
| Burlington | 88.95 | 0.00 | 158.00 | 15.80 | 15.80 | 17.80 |
| Camden | 26.66 | 10.14 | 35.94 | 3.59 | 13.73 | 15.73 |
| Cape May | 23.05 | 12.22 | 2.73 | 0.27 | 12.49 | 14.49 |
| Cumberland | 0.67 | 0.00 | 80.08 | 8.01 | 8.01 | 10.01 |
| Essex | 40.46 | 20.48 | 5.53 | 0.55 | 21.04 | 23.04 |
| Gloucester | 4.43 | 3.65 | 9.30 | 0.93 | 4.58 | 6.58 |
| Hudson | 3.22 | 2.68 | 2.31 | 0.23 | 2.92 | 4.92 |
| Hunterdon | 120.99 | 3.63 | 182.03 | 18.20 | 21.84 | 23.84 |
| Mercer | 78.34 | 2.21 | 3.68 | 0.37 | 2.58 | 4.58 |
| Middlesex | 30.81 | 21.47 | 205.09 | 20.51 | 41.98 | 43.98 |
| Monmouth | 118.82 | 8.62 | 33.36 | 3.34 | 11.95 | 13.95 |
| Morris | 292.95 | 6.52 | 263.95 | 26.39 | 32.92 | 34.92 |
| Ocean | 21.98 | 4.61 | 21.90 | 2.19 | 6.80 | 8.80 |
| Passaic | 40.16 | 11.14 | 1.76 | 0.18 | 11.32 | 13.32 |
| Salem | 8.04 | 0.00 | 105.71 | 10.57 | 10.57 | 12.57 |
| Somerset | 52.38 | 23.28 | 0.00 | 0.00 | 23.28 | 25.28 |
| Sussex | 64.87 | 5.70 | 41.35 | 4.13 | 9.84 | 11.84 |
| Union | 14.15 | 0.00 | 16.41 | 1.64 | 1.64 | 3.64 |
| Warren | 5.94 | 1.57 | 1.32 | 0.13 | 1.70 | 3.70 |

Results: Statewide Summer Day/Annual Benefits

Total VT Reduced = 992 trips/day Statewide

Total VMT Reduced = 2,591 miles/day Statewide

Emission Reductions (Statewide):

| Pollutant Type | ## Analysis Year 2009 |  |
| --- | --- | --- |
|  | Tons/Day | Tons/Year |
| # VOC | 0.0027 | - |
| NOx | 0.0018 | 0.47 |
| PM2.5 | - | 0.02 |
| SO2 | - | 0.01 |

Ozone Analysis – Daily Benefits in New Jersey

| Benefits (Reductions) | ## Ozone Non-Attainment Area |  |
| --- | --- | --- |
|  | New York-N. New Jersey-Long Island, NY-NJ-CT | Philadelphia-Wilmington-Atlantic City PA-NJ-MD-DE |
| # VT | 839 | 153 |
| # VMT | 2,193 | 398 |
| # VOC (Tons/Day) | 0.0023 | 0.0043 |
| NOx (Tons/Day) | 0.0015 | 0.00029 |

PM2.5 Analysis – Annual Benefits in New Jersey

| Benefits (Reductions) | ## PM2.5 Non-Attainment Area |  |
| --- | --- | --- |
|  | New York-N. New Jersey-Long Island, NY-NJ-CT | Philadelphia-Wilmington PA-NJ-DE |
| # VT | 204,500 | 534,000 |
| # VMT | 19,500 | 50,750 |
| NOx (Tons/Year) | 0.38 | 0.041 |
| PM2.5 (Tons/Year) | 0.017 | 0.0016 |
| SO2 (Tons/Year) | 0.010 | 0.0010 |

Cost/Benefit Analysis

Total costs were estimated as follows:

For Off-Road Facilities:

Capital Cost: $150,000/Mile over 15 Years

Operating/Maintenance: $6,000/Mile/Year

Total Annual cost: $16,000/Mile/Year

For On-Road Facilities

$4,500/Mile over 5 years = $900/Mile/Year

Total cost for the various non-attainment areas are as follows:

| County | Estimated Cost For Each AQ Region |  |  |  |
| --- | --- | --- | --- | --- |
|  | Ozone |  | PM 2.5 |  |
|  | NY-NJ-CT | PA-NJ-DE-MA | NY-NJ-CT | PA-NJ-DE |
| Atlantic | $0 | $30,642 | $0 | $0 |
| Bergen | $0 | $0 | $0 | $0 |
| Burlington | $0 | $151,138 | $0 | $151,138 |
| Camden | $0 | $74,948 | $0 | $74,948 |
| Cape May | $0 | $148,908 | $0 | $0 |
| Cumberland | $0 | $24,809 | $0 | $0 |
| Essex | $59,084 | $0 | $59,084 | $0 |
| Gloucester | $0 | $49,182 | $0 | $49,182 |
| Hudson | $2,624 | $0 | $2,624 | $0 |
| Hunterdon | $24,157 | $0 | $0 | $0 |
| Mercer | $0 | $41,237 | $41,237 | $0 |
| Middlesex | $348,920 | $0 | $348,920 | $0 |
| Monmouth | $111,905 | $0 | $111,905 | $0 |
| Morris | $432,186 | $0 | $432,186 | $0 |
| Ocean | $0 | $103,530 | $0 | $0 |
| Passaic | $104,893 | $0 | $104,893 | $0 |
| Salem | $0 | $9,514 | $0 | $0 |
| Somerset | $110,063 | $0 | $110,063 | $0 |
| Sussex | $145,873 | $0 | $0 | $0 |
| Union | $1,477 | $0 | $1,477 | $0 |
| Warren | $3,531 | $0 | $0 | $0 |
| Total | $1,344,712 | $633,907 | $1,212,388 | $275,268 |

Cost/Benefit Results

| Pollutant | Cost/Benefit for Each Pollutant |  |  |  |
| --- | --- | --- | --- | --- |
|  | Ozone* |  | PM 2.5 |  |
|  | NY-NJ-CT | PA-NJ-DE-MD | NY-NJ-CT | PA-NJ-DE |
| VOC ($/Ton/Day) | $2,345,969 | $5,898,172 | - | - |
| NOx ($/Ton/Day) | $3,485,440 | $8,847,258 | - | - |
| NOx ($/Ton/Year) | - | - | $3,190,496 | $6,660,958 |
| PM2.5 ($/Ton/Year) | - | - | $69,788,164 | $171,040,616 |
| SO2 ($/Ton/Year) | - | - | $116,387,456 | $287,033,677 |

*Daily Costs used in calculation of cost/benefit ratio are Annual costs divided by 250