Control Measure:

Reduce the delays and idling emissions associated with toll plazas by adding Express EZ Pass lanes that allow users to bypass booths and proceed through the toll collection area at close to usual highway speeds

Description:

At toll plazas throughout the state, significant emissions are associated with the queuing to pay tolls and the excessive delays associated with toll plazas during peak periods.  To alleviate these delays, Express EZ-Pass higher speed toll collection facilities that allow traffic to pass safely at close to mainline speeds either have been or are in the process of being installed at a number of major toll plazas.  Three additional locations along the Garden State Parkway, namely the Union, Essex and Barnegat Toll Plazas, are scheduled to have new high speed lanes in operation prior to 2009/2010.

Methodology:

The NJ-AQONE software package contains a methodology for estimating the air quality impacts at toll plazas using a deterministic, queuing model approach.  This approach based largely on estimating the queue lengths generated under various operating conditions, along with the average speeds associated with each segment as a vehicle travels trough the toll facility.  Information specific to each plaza, including the current and future configurations of the plazas, percent of users with an EZ-Pass at each plaza, and total traffic volume estimates were obtained from the New Jersey Turnpike Authority.  A summary of the data is provided below:

Essex Toll Plaza (Essex County)

Current Configuration:

2 Booths accepting no EZ Pass, 4 Accepting all Payment types, 5 EZ Pass only booths

Future Configuration:

2 Booths accepting no EZ Pass, 4 Accepting all Payment types, 1 EZ Pass only Booth and 3 Express EZ Pass Lanes

Traffic Volume: 65, 267

% Vehicles with EZ-Pass: 59%

% of EZ-Pass Vehicles using High Speed Lanes: 90%

Barnegat Toll Plaza (Ocean County)

Current Configuration:

3 Booths accepting no EZ Pass, 2 accepting all Payment types, 2 EZ Pass only booths

Future Configuration:

2 Booths accepting no EZ Pass, 2 accepting all Payment types, No EZ Pass only Booth and 2 Express EZ Pass Lanes

Daily Traffic Volume: 22,484

% Vehicles with EZ-Pass: 67%

% of EZ-Pass Vehicles using High Speed Lanes: 90%

Union Toll Plaza (Union County)

Current Configuration:

3 Booths accepting no EZ Pass, 6 accepting all Payment types, 6 EZ Pass only booths

Future Configuration:

3 Booths accepting no EZ Pass, 4 accepting all Payment types, 3 EZ Pass only Booth and 3 Express EZ Pass Lanes

Daily Traffic Volume: 79,662

% Vehicles with EZ-Pass: 58%

% of EZ-Pass Vehicles using High Speed Lanes: 60%*

* Due to a ramp configuration, not all traffic will be able to access the High Speed lanes and a lower % was assumed accordingly

Note that all the toll plazas analyzed have already had tolls removed in one direction to help improve traffic flow.  However, since this improvement is already in place, only the benefits due to the installation of the Express EZ-Pass lanes were considered.

Emission Reductions (Statewide Benefits):

| Pollutant Type | ## Analysis Year 2009 |  |
| --- | --- | --- |
|  | Tons/Day | Tons/Year |
| # VOC | 0.024 | - |
| NOx | 0.0045 | 0.88 |
| PM2.5 | - | 0.00032 |
| SO2 | - | 0.000033 |

Ozone Analysis – Daily Benefits in New Jersey

| Benefits (Reductions) | ## Ozone Non-Attainment Area |  |
| --- | --- | --- |
|  | New York-N. New Jersey-Long Island, NY-NJ-CT | Philadelphia-Wilmington-Atlantic City PA-NJ-MD-DE |
| # VOC (Tons/Day) | 0.024 | 0.000022 |
| NOx (Tons/Day) | 0.0045 | Negl. |

PM2.5 Analysis – Annual Benefits in New Jersey

| Benefits (Reductions) | ## PM2.5 Non-Attainment Area |  |
| --- | --- | --- |
|  | New York-N. New Jersey-Long Island, NY-NJ-CT | Philadelphia-Wilmington PA-NJ-DE |
| NOx (Tons/Year) | 0.88 | - |
| PM2.5 (Tons/Year) | 0.00032 | - |
| SO2 (Tons/Year) | 0.000033 | - |

Cost/Benefit Analysis

Total costs were estimated using published estimates of average costs of the Express EZ-Pass locations in NJ of $8-12 Million per location.  The Union and Essex location were assumed to be at the upper end of that range, the Barnegat Plaza at the lower end.  While some operating cost savings may be expected due to reduced staffing needs, estimates of this were unavailable and not taken into consideration.

Estimated Total Cost: $32 Million

Approximate Annual cost over 15 years:	$2,133,333/Annually

Cost/Benefit Results

| Pollutant | Cost/Benefit for Each Pollutant |  |  |  |
| --- | --- | --- | --- | --- |
|  | Ozone* |  | PM 2.5 |  |
|  | NY-NJ-CT | PA-NJ-DE-MD | NY-NJ-CT | PA-NJ-DE |
| VOC ($/Ton/Day) | $355,556 | $387,878,727 | - | - |
| NOx ($/Ton/Day) | $1,896,296 | NA** | - | - |
| NOx ($/Ton/Year) | - | - | $1,657,542 | - |
| PM2.5 ($/Ton/Year) | - | - | N/A** | - |
| SO2 ($/Ton/Year) | - | - | N/A** | - |

*Daily Costs used in calculation of cost/benefit ratio are Annual costs divided by 250

**No significant Air Quality Benefit