Control Measure:

School Bus Retrofit

Description:

Older, higher-emitting buses will be fitted with emissions reducing retrofit technology.

Documentation:

- Environmental Protection Agency, Clean School Bus USA: http://www.epa.gov/cleanschoolbus/index.htm

- School Bus Transportation News, K-12 Enrollment / Transportation Data, 2002-03 School Year: http://www.stnonline.com/stn/data_statistics/2002-03_schoolyear.htm

Methodology:

All model year 1992-2002 buses in the fleet will be equipped with retrofit technology.

Retrofitting model year 1992-2002 buses is assumed to have the greatest potential to reduce emissions due to existing regulations and fleet age.

Inputs/Assumptions:

- A Lubrizol Engine Control Systems, Purifilter Diesel Particulate Filter would be used to retrofit the buses.

  - This filter is a verified EPA retrofit technology and is estimated to reduce VOC emissions by 85% and PM emissions by 90%.

  - The filter has no affect on NOx emissions.

- All model year 1992-2002 buses in the fleet will be equipped with retrofit technology.

- Retrofitting model year 1992-2002 buses is assumed to have the greatest potential to reduce emissions due to existing regulations and fleet age.

- 58.1% of all school bus VMT can be attributed to model year 1992-2002 buses.

- Because no estimates could be found on the affect of retrofit technology on SO2 emissions, a zero benefit was assumed.

- MOBILE6.2 was used to generate emission factors and VMT.

- The expected useful life of retrofit equipment is 6 years.

- School buses operate 180 days per year.

- It costs an average of $7,500 to retrofit one school bus.

- There are 21,228 buses operating in the state of NJ.

- 58.1% of the operating school buses are assumed to be MY 1992-2002.

Results:

Total Affected VMT:

Daily

Model Years 1992-2002 = 76,659 miles/day

Annual

Model Years 1992-2002 = 28,881,989 miles/year

Statewide Benefits (Emission Reductions):

| Pollutant Type | ## Analysis Year 2009 |  |
| --- | --- | --- |
|  | tons/day | tons/year |
| # VOC | 0.03 | NA |
| NOx | 0.00 | 0.00 |
| PM2.5 | NA | 10.50 |
| SO2 | NA | 0.00 |

*Statewide emission benefits include counties not included below

Ozone Analysis – Daily Benefits (tpd) in New Jersey:

| Benefits (Reductions) | ## Ozone Non-Attainment Area |  |
| --- | --- | --- |
|  | NY-NJ-CT | PA-NJ-MD-DE |
| # VMT | 37,432 | 28,515 |
| # VOC (Tons/Day) | 0.02 | 0.01 |
| NOx (Tons/Day) | NA | NA |

PM2.5 Analysis – Annual Benefits (tpy) in New Jersey:

| Benefits (Reductions) | ## PM2.5 Non-Attainment Area |  |
| --- | --- | --- |
|  | NY-NJ-CT | PA-NJ-DE |
| # VMT | 14,639,546 | 11,060,764 |
| NOx (Tons/Year) | NA | NA |
| PM2.5 (Tons/Year) | 5.32 | 4.02 |
| SO2 (Tons/Year) | NA | NA |

Cost Benefit Results:

| Nonattainment Area | Ozone ($/tpd) |  |
| --- | --- | --- |
|  | VOC | NOx |
| NY-NJ-CT | $ 3,049,989 | NA |
| PA-NJ-MD-DE | $ 3,050,055 | NA |
| Statewide | $ 3,050,018 | NA |

| Nonattainment Area | PM2.5 ($/tpy) |  |  |
| --- | --- | --- | --- |
|  | NOx | PM2.5 | SO2 |
| NY-NJ-CT | NA | $ 2,840,463 | NA |
| PA-NJ-DE | NA | $ 2,840,465 | NA |
| Statewide | NA | $ 2,840,464 | NA |