Control Measure:

Enforce posted speed limits on freeways.

Description:

By traveling in excess of the posted speed limit, vehicles on the road emit pollutants as a far different rate then they would should they travel at the limit.  This analysis estimates the impact of lowering average travel speed on all freeways in the state through an expanded enforcement presence in the form of increased police patrols.

Vehicles emit higher emissions at speeds greater than 65mph. NOx emissions are significantly lower at 55mph compared to 65mph. Hence, by enforcing speed limits to 55mph NOx emissions can be reduced. The downside to this is that the VOC emissions are higher at 55mph than at 65mph. Enforcement of the 65 MPH speed limit will have less of an effect as average speeds are only slightly higher.  Since PM2.5 and SO2 emissions are not impacted by speed changes, this project would have no impact on those pollutants.

Methodology:

Total Vehicle Miles of Travel (VMT) were available for every county, and the New Jersey Congestion Management System (NJCMS) was used to estimate the percentage of total travel in each county on 55 MPH and 65MPH freeways.  The estimated percentages are shown in Table 1

The basic assumption is that the average travel speed on all freeways in NJ is 67 MPH, a values obtained from the “18 Month Study Report on 65 MPH Speed Limit in New Jersey” found on the NJDOT website.  It is assumed that the enforcement program would reduce average speeds to the posted limit.  Emission rates were calculated for a composite vehicle traveling at 65mph and at 55mph using MOBILE6.2 and setups for each of the New Jersey counties for each season (summer and winter.)  Since MOBILE 6.2 does not calculate specific emission rates for speeds in excess of 65MPH, a straight line projection using the 55MPH and 65MPH emission rates was used to obtain the 67MPH rates.  The difference between the emission rates was used along with the total freeway VMT to calculate emissions benefits.  Summaries of these calculations are found at the end of this project description.

Inputs/Assumptions:

- The average travel speed on New Jersey freeways is 67 MPH

- This enforcement program would reduce average speeds to the posted limit

- Estimates of total VMT were readily available, the NJCCMS was used to estimate the percentage of travel on 55 MPH and 65 MPH freeways in each county.

| County | Percent of Total VMT On Freeways (From the NJCMS) | Estimated % on 55 MPH Freeways | Estimated % on 65 MPH Freeways |
| --- | --- | --- | --- |
| Atlantic | 42.2% | 11.07% | 31.18% |
| Bergen | 44.5% | 11.67% | 32.87% |
| Burlington | 51.1% | 13.39% | 37.73% |
| Camden | 41.6% | 10.89% | 30.68% |
| Cape May | 41.2% | 10.79% | 30.40% |
| Cumberland | 33.1% | 8.67% | 24.41% |
| Essex | 56.4% | 14.79% | 41.65% |
| Gloucester | 53.2% | 13.94% | 39.27% |
| Hudson | 51.0% | 13.36% | 37.62% |
| Hunterdon | 46.8% | 12.26% | 34.52% |
| Mercer | 45.3% | 11.87% | 33.44% |
| Middlesex | 35.2% | 9.23% | 26.00% |
| Monmouth | 42.2% | 11.05% | 31.12% |
| Morris | 59.2% | 15.51% | 43.69% |
| Ocean | 47.3% | 12.39% | 34.90% |
| Passaic | 33.7% | 8.83% | 24.86% |
| Salem | 56.0% | 14.67% | 41.32% |
| Somerset | 50.2% | 13.16% | 37.07% |
| Sussex | 2.7% | 0.00% | 2.70% |
| Union | 68.0% | 17.81% | 50.18% |
| Warren | 59.5% | 15.58% | 43.90% |

Table 1: Estimated Percentage of Total VMT on Freeways from NJCMS

Results:

Emission Reductions (Statewide):

| Pollutant Type | ## Analysis Year 2010 |  |
| --- | --- | --- |
|  | Tons/Day | Tons/Year |
| VOC | -0.35 | - |
| NOx | 4.88 | 1,845.58 |
| PM2.5 | - | 0.00 |
| SO2 | - | 0.00 |

*Note: Negative means increase in emissions

Ozone Analysis – Daily Benefits in New Jersey

| Benefits (Reductions) | ## Ozone Non-Attainment Area |  |
| --- | --- | --- |
|  | New York-N. New Jersey-Long Island, NY-NJ-CT | Philadelphia-Wilmington-Atlantic City PA-NJ-MD-DE |
| # VOC (Tons/Day) | -0.23 | -0.13 |
| NOx (Tons/Day) | 3.16 | 1.71 |

*Note: Negative means increase in emissions

PM2.5 Analysis – Annual Benefits in New Jersey

| Benefits (Reductions) | ## PM2.5 Non-Attainment Area |  |
| --- | --- | --- |
|  | New York-N. New Jersey-Long Island, NY-NJ-CT | Philadelphia-Wilmington PA-NJ-DE |
| NOx (Tons/Year) | 1,206.63 | 295.66 |
| PM2.5 (Tons/Year) | 0.00 | 0.00 |
| SO2 (Tons/Year) | 0.00 | 0.00 |

*Note: Negative means increase in emissions

COST/BENEFIT ANALYSIS

Assume approximately 5 officers are needed to patrol 20 miles of freeway 24 hrs a day.

Capital Cost = $33,000/officer over 7 years (vehicle, equipment, etc)

Annual Maintenance Cost = $10,000/officer (maintenance cost)

Annual Operating Cost = $52,000/officer (Salary, overhead)

Annualized Cost = $66,714/officer/year

Annual Cost/Mile of Freeway = $16, 678.50

Total Statewide Cost = 757.1 Miles x 16,678.50 = $12,627,292/Year

Total cost for the various non-attainment areas are as follows:

| County | Miles of Freeway | Patrol Officers Required | Estimated Cost For Each Non-Attainment Area |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Ozone |  | PM 2.5 |  |
|  |  |  | NY-NJ-CT | PA-NJ-DE-MD | NY-NJ-CT | PA-NJ-DE |
| Atlantic | 48.7 | 12 | - | $812,476 | - | - |
| Bergen | 54.9 | 14 | $915,804 | - | $915,804 | - |
| Burlington | 55.5 | 14 | - | $926,088 | - | $926,088 |
| Camden | 38.7 | 10 | - | $645,514 | - | $645,514 |
| Cape May | 26.7 | 7 | - | $445,127 | - | - |
| Cumberland | 17.7 | 5 | - | $294,394 | - | - |
| Essex | 33.7 | 9 | $562,113 | - | $562,113 | - |
| Gloucester | 51.8 | 13 | - | $863,417 | - | $863,417 |
| Hudson | 29.3 | 7 | $489,318 | - | $489,318 | - |
| Hunterdon | 19.0 | 5 | $316,088 | - | - | - |
| Mercer | 38.6 | 10 | - | $643,586 | $643,586 | - |
| Middlesex | 57.2 | 14 | $953,246 | - | $953,246 | - |
| Monmouth | 40.9 | 10 | $682,635 | - | $682,635 | - |
| Morris | 57.9 | 15 | $965,619 | - | $965,619 | - |
| Ocean | 45.6 | 12 | - | $760,250 | - | - |
| Passaic | 15.1 | 4 | $251,488 | - | $251,488 | - |
| Salem | 21.2 | 5 | - | $353,691 | - | - |
| Somerset | 34.7 | 9 | $578,022 | - | $578,022 | - |
| Sussex | 0.8 | 0 | $13,177 | - | - | - |
| Union | 40.9 | 10 | $682,313 | - | $682,313 | - |
| Warren | 28.4 | 7 | $472,927 | - | - | - |
| Total | 757.1 | 192.0 | $6,882,749 | $5,744,543 | $6,724,143 | $2,435,019 |

Cost/Benefit Results

| Pollutant | Cost/Benefit for Each Pollutant |  |  |  |
| --- | --- | --- | --- | --- |
|  | Ozone |  | PM 2.5 |  |
|  | NY-NJ-CT | PA-NJ-DE-MD | NY-NJ-CT | PA-NJ-DE |
| VOC ($/Ton/Day) | Increase in VOC | Increase in VOC | - | - |
| NOx ($/Ton/Day) | $101,338 | $140,451 | - | - |
| NOx ($/Ton/Year) | - | - | $55,290 | $67,693 |
| PM2.5 ($/Ton/Year) | - | - | No Change | No Change |
| SO2 ($/Ton/Year) | - | - | No Change | No Change |

* Daily Costs used in calculation of cost/benefit ratio are Annual costs divided by 250