Control Measure:

Limit the use of recreational watercraft

Description:

Based on registration data, all marine pleasure craft with registration numbers ending with odd numbers would be restricted to use on odd days of the month, while pleasure craft with registration numbers ending with even numbers would be restricted to use on even days of the month only.

Documentation:

- Department of Energy, Energy Information Administration (EIA), Retail Diesel Prices: http://tonto.eia.doe.gov/oog/info/wohdp/diesel_detail_report.asp

- Department of Energy, Energy Information Administration (EIA), Retail Gasoline Prices:

http://www.eia.doe.gov/oil_gas/petroleum/data_publications/wrgp/mogas_home_page.html

Methodology:

The new NONROAD 2005 model, released in December 2005, was used to estimate 2009 VOC, NOx, PM2.5 and SO2 emissions.  Both daily and annual emissions were modeled.  This measure was assumed to result in a 1% reduction of each pollutant.  VOC benefits were only taken from the current exhaust emission totals, this control measure was assumed to have no impact on diurnal, crankcase, displacement or spillage emissions. The NONROAD model was used to estimate the gallons of gasoline and diesel fuel used on a daily and annual basis.

Inputs/Assumptions:

- The percent reduction resulting from the implementation of restrictions on use are assumed to only have an impact on exhaust emissions.

- It is assumed that the restrictions would result in approximately a 1% reduction in use as the restrictions would be both costly and difficult to police.

- Cost benefits were calculated based on fuel consumption estimates generated using NONROAD 2005.

- Cost estimates to not include costs of enforcing the regulations.

- Diesel fuel cost = $2.62/gallon.

- Gasoline cost = $2.40/gallon.

NONROAD inputs:

|  | NY-NJ-CT |  | PA-NJ-DE | PA-NJ-MD-DE |
| --- | --- | --- | --- | --- |
| Inputs | Annual | Sum. Day | Annual | Sum. Day |
| Min | 19.1 | 66.8 | 24.2 | 65.1 |
| Max | 83.9 | 82.9 | 85.3 | 78.1 |
| Avg. | 51.0 | 75.0 | 54.4 | 75.0 |
| RVP | 15.0 | 15.0 | 15.0 | 15.0 |
| O2 Weight | 2.1 | 2.1 | 2.1 | 2.1 |
| Gas Sulfur | 0.0030 | 0.0030 | 0.0030 | 0.0030 |
| Diesel Sulfur | 0.0015 | 0.0015 | 0.0015 | 0.0015 |
| Stage II | 0.0 | 0.0 | 0.0 | 0.0 |

Results:

Ozone Analysis – Daily Benefits (tpd) in New Jersey:

| Benefits (Reductions) | ## Ozone Non-Attainment Area |  |
| --- | --- | --- |
|  | NY-NJ-CT | PA-NJ-MD-DE |
| # VOC (Tons/Day) | 0.48 | 2.32 |
| NOx (Tons/Day) | 0.12 | 0.50 |

PM2.5 Analysis – Annual Benefits (tpy) in New Jersey:

| Benefits (Reductions) | ## PM2.5 Non-Attainment Area |  |
| --- | --- | --- |
|  | NY-NJ-CT | PA-NJ-DE |
| NOx (Tons/Year) | 9.97 | 2.12 |
| PM2.5 (Tons/Year) | 0.92 | 0.28 |
| SO2 (Tons/Year) | 0.46 | 0.08 |

Cost Benefit Results:

| Nonattainment Area | Ozone ($/tpd) |  |
| --- | --- | --- |
|  | VOC | NOx |
| NY-NJ-CT | $ (13,196) | $ (51,949) |
| PA-NJ-MD-DE | $ (7,870) | $ (36,613) |
| Statewide | $ (8,788) | $ (39,643) |

| Nonattainment Area | PM2.5 ($/tpy) |  |  |
| --- | --- | --- | --- |
|  | NOx | PM2.5 | SO2 |
| NY-NJ-CT | $ (35,174) | $ (379,657) | $ (769,192) |
| PA-NJ-DE | $ (43,263) | $ (322,915) | $ (1,153,048) |
| Statewide | $ (36,594) | $ (366,299) | $ (826,278) |

* Parentheses denote a cost savings.