Control Measure:

Encourage the use of transit through the completion of significant fixed guideway/rail projects.

Description:

New Jersey Transit is in the process of completing a number of fixed guideway/rail projects, each having a significant impact on ridership and air quality.  For the purposes of this investigation, five major projects were identified for analysis:

- The Hudson-Bergen LRT (HBLRT) “MOS-II” extension

This project is an extension of the existing HBLRT system to Weehawken/Port Imperial and Tonnelle Avenue

- Pascack Valley Line Sidings

The addition of new sidings on the Pascack Valley line will allow for more off-peak and weekend service and shorter service headways

- Newark-Elizabeth Rail Link (NERL) Phase 1

The first phase of a new LRT line connecting Newark Penn Station to the City of Elizabeth

- Broad Street Newark Station Platform Project

The rehabilitation of the existing station platforms and the addition of a new platform serving the third active track

- Meadowlands Sports Complex Spur

The building of a spur line and station to serve the Meadowlands sports complex and the subsequent development scheduled for the site

Methodology:

All projects in this analysis have previously been studied by NJ Transit and the traffic (VMT/VT) impacts already calculated.  A summary of the transportation impacts of each project are provided in Tables 1a, b and c, and the total overall impact shown in table 1d.  These VT and VMT impacts were used as inputs into the “emissions only” analysis in the NJ-AQONE software in order to calculate the air quality benefits for each project.  A summary of the Air Quality analysis is provided below:

Results: Statewide Summer Day/Annual Benefits

Total VT Reduced = 20,319 trips/day Statewide

Total VMT Reduced = 239,373 miles/day Statewide

Emission Reductions (Statewide):

| Pollutant Type | ## Analysis Year 2009 |  |
| --- | --- | --- |
|  | Tons/Day | Tons/Year |
| # VOC | 0.12 | - |
| NOx | 0.12 | 30.29 |
| PM2.5 | - | 0.94 |
| SO2 | - | 0.57 |

Ozone Analysis – Daily Benefits in New Jersey

| Benefits (Reductions) | ## Ozone Non-Attainment Area |  |
| --- | --- | --- |
|  | New York-N. New Jersey-Long Island, NY-NJ-CT | Philadelphia-Wilmington-Atlantic City PA-NJ-MD-DE |
| # VT | 20,528 | 16 |
| # VMT | 219,502 | 2,248 |
| # VOC (Tons/Day) | 0.12 | 0.0009 |
| NOx (Tons/Day) | 0.12 | 0.0011 |

PM2.5 Analysis – Annual Benefits in New Jersey

| Benefits (Reductions) | ## PM2.5 Non-Attainment Area |  |
| --- | --- | --- |
|  | New York-N. New Jersey-Long Island, NY-NJ-CT | Philadelphia-Wilmington PA-NJ-DE |
| # VT | 5,117,000 | 2,000 |
| # VMT | 54,167,000 | 284,500 |
| NOx (Tons/Year) | 30.15 | 0.1400 |
| PM2.5 (Tons/Year) | 0.94 | 0.0036 |
| SO2 (Tons/Year) | 0.57 | 0.0021 |

Cost/Benefit Analysis

Total Capital costs were provided directly by NJ Transit or from values listed in the NJTPA Transportation Improvement Plan:

HBLRT MOS-II:	$1.1 Billion

Pascack Valley Sidings:	$25 Million

NERL Phase 1:	$210 Million

Broad Street Platforms:	$72.8 Million

Meadowlands Spur:	$350 Million

Total:	$1.76 Billion

Over a 30 year lifespan, the annual capital costs of these projects would be $58,593,333/Year

Unfortunately, operating costs for these projects were unavailable and are not included in the cost/benefit analysis.  Were these costs to be included, the cost per ton would be significantly higher that reported what is reported here.

Cost/Benefit Results

| Pollutant | Cost/Benefit for Each Pollutant |  |  |  |
| --- | --- | --- | --- | --- |
|  | Ozone* |  | PM 2.5 |  |
|  | NY-NJ-CT | PA-NJ-DE-MD | NY-NJ-CT | PA-NJ-DE |
| VOC ($/Ton/Day) | $1,953,111 | NA** | - | - |
| NOx ($/Ton/Day) | $1,953,111 | NA** | - | - |
| NOx ($/Ton/Year) | - | - | $1,943,394 | NA** |
| PM2.5 ($/Ton/Year) | - | - | $62,333,333 | NA** |
| SO2 ($/Ton/Year) | - | - | $102,795,321 | NA** |

*Daily Costs used in calculation of cost/benefit ratio are Annual costs divided by 250

** No projects are actually being built in the southern non-attainment areas and the benefits to these regions minor; therefore no cost/benefit ratio was calculated