Control Measure:

Replace high polluting heavy-duty diesel vehicles older than 15 years.

Description:

This control measure analyzes emission impacts of replacing heavy-duty diesel vehicles older than 15 years in all the counties in New Jersey.

Methodology:

Emission factors were calculated for each county for a Typical Summer Day as well as Annual.  The emission rates were calculated using MOBILE6.2 and the setups for each county of the respective MPO.

Inputs/Assumptions:

- 2009 HDV8b seasonal VMT was calculated for all three MPOs (NJTPA, SJTPO & DVRPC) based on their conformity setups.

- It was assumed that 22% of the HDV8b vehicles are MY ’85-’91.

- It was assumed that 30% of these vehicles are replaced in each county.

- Total days in Summer = 183 and Winter = 182.

- Assume each HDV8b type of truck replaced travels 500 miles/day

- Assume replacement cost per truck = $90,000 for the project.

Results:

Emission Reductions (Statewide):

| Pollutant Type | ## Analysis Year 2010 |  |
| --- | --- | --- |
|  | Tons/Day | Tons/Year |
| VOC | 0.1971 | 104.62 |
| NOx | 8.2252 | 2,912.02 |
| PM2.5 | 0.4982 | 172.82 |
| SO2 | 0.4982 | 0.2189 |

*Note: Negative means increase in emissions

Ozone Analysis – Daily Benefits in New Jersey

| Benefits (Reductions) | ## Ozone Non-Attainment Area |  |
| --- | --- | --- |
|  | New York-N. New Jersey-Long Island, NY-NJ-CT | Philadelphia-Wilmington-Atlantic City PA-NJ-MD-DE |
| # HDV VMT Reduced (VMT/Day) | 174,487 | 94,385 |
| # VOC (Tons/Day) | 0.1279 | 0.0692 |
| NOx (Tons/Day) | 5.3378 | 2.8874 |

*Note: Negative means increase in emissions

PM2.5 Analysis – Annual Benefits in New Jersey

| Benefits (Reductions) | ## PM2.5 Non-Attainment Area |  |
| --- | --- | --- |
|  | New York-N. New Jersey-Long Island, NY-NJ-CT | Philadelphia-Wilmington PA-NJ-DE |
| # HDV VMT Reduced (VMT/Year) | 58,302,183 | 26,447,919 |
| NOx (Tons/Year) | 1600.322 | 725.96 |
| PM2.5 (Tons/Year) | 94.976 | 43.084 |
| SO2 (Tons/Year) | 0.12 | 0.055 |

*Note: Negative means increase in emissions

COST/BENEFIT ANALYSIS

Cost of replacing each truck was assumed to be approximately $90,000 per truck.  This cost would be the capital cost. The O&M cost will not be considered for this study.

Total costs were estimated as follows:

Capital Cost = $90,000/truck over 7 years

Annual Operating/Maintenance Cost = N/A

Total Annualized Cost = $12,857/truck/year

Total cost for the various non-attainment areas are as follows:

| County | Estimated Cost For Each Non-Attainment Area |  |  |  |
| --- | --- | --- | --- | --- |
|  | Ozone |  | PM 2.5 |  |
|  | NY-NJ-CT | PA-NJ-DE | NY-NJ-CT | PA-NJ-DE |
| Atlantic | - | $244,283 | - | - |
| Bergen | $655,707 | - | $655,707 | - |
| Burlington | - | $1,105,702 | - | $1,105,702 |
| Camden | - | $951,418 | - | $951,418 |
| Cape May | - | $89,999 | - | - |
| Cumberland | - | $154,284 | - | - |
| Essex | $449,995 | - | $449,995 | - |
| Gloucester | - | $668,564 | - | $668,564 |
| Hudson | $347,139 | - | $347,139 | - |
| Hunterdon | $411,424 | - | - | - |
| Mercer | - | $809,991 | $809,991 | - |
| Middlesex | $1,362,842 | - | $1,362,842 | - |
| Monmouth | $578,565 | - | $578,565 | - |
| Morris | $642,850 | - | $642,850 | - |
| Ocean | - | $321,425 | - | - |
| Passaic | $308,568 | - | $308,568 | - |
| Salem | - | $257,140 | - | - |
| Somerset | $385,710 | - | $385,710 | - |
| Sussex | $231,426 | - | - | - |
| Union | $449,995 | - | $449,995 | - |
| Warren | $501,423 | - | - | - |
| Total | $6,325,644 | $4,602,806 | $5,991,362 | $2,725,684 |

Cost/Benefit Results

| Pollutant | Cost/Benefit for Each Pollutant |  |  |  |
| --- | --- | --- | --- | --- |
|  | Ozone |  | PM 2.5 |  |
|  | NY-NJ-CT | PA-NJ-DE-MD | NY-NJ-CT | PA-NJ-DE |
| VOC ($/Ton/Day) | $140,749 | $139,763 | - | - |
| NOx ($/Ton/Day) | $3,372 | $3,349 | - | - |
| NOx ($/Ton/Year) | - | - | $3,744 | $3,755 |
| PM2.5 ($/Ton/Year) | - | - | $63,083 | $63,264 |
| SO2 ($/Ton/Year) | - | - | $49,815,355 | $49,958,165 |

* Daily Costs used in calculation of cost/benefit ratio are Annual costs divided by 250