Control Measure:

Transit Bus Replacement

Description:

All model year 2002 and older buses will be replaced with model year 2007 diesel buses.

Documentation:

- National Transit Database (NTD), 2003 Database: http://www.ntdprogram.com/NTD/ntdhome.nsf/Docs/NTDData?OpenDocument

- Average cost of new diesel buses: www.hybridcenter.org

Methodology:

All model year 2002 and older buses will be replaced with model year 2007 diesel buses.  Both daily and annual benefits were calculated.  Annual emissions were calculated using a two-season approach. Costs were calculated using data from the National Transit Database (NTD).  According to the NTD, 3,573 transit buses were in operation as of 2003.  It was assumed that all buses, model years 2002 and older, will be replaced in 2009 at a cost of $300,000 per bus. The costs were split over the assumed life of the bus or 10 years.  Transit buses are assumed to operate 250 days per year.

Inputs/Assumptions:

- 34.4% of all transit bus VMT (both daily and annual) can be attributed to model year 1992-2002 buses.

- 6.5% of all transit bus VMT (both daily and annual) can be attributed to model year 1991 and older buses.

- MOBILE6.2 was used to generate emission factors and VMT.

- The 2003 National Transit Database was used to discern how many transit buses, model years 2002 and older, are operating in New Jersey.

- 3,573 transit buses in NJ.

- Expected life of a transit bus is 10 years.

- Transit buses operate 250 days per year.

Results:

Total Affected VMT:

Daily

Model Years 1992-2002 = 38,654 miles/day

Model Years 1991 & Older = 7,312 miles/day

Annual

Model Years 1992-2002 = 14,453,822 miles/year

Model Years 1991 & Older = 2,739,503 miles/year

Benefits (Emission Reductions):

| Pollutant Type | ## Analysis Year 2009 |  |
| --- | --- | --- |
|  | tons/day | tons/year |
| # VOC | 0.02 | NA |
| NOx | 0.47 | 282.09 |
| PM2.5 | NA | 12.52 |
| SO2 | NA | 0.01 |

*Statewide emission benefits include counties not included below

Ozone Analysis – Daily Benefits (tpd) in New Jersey:

| Benefits (Reductions) | ## Ozone Non-Attainment Area |  |
| --- | --- | --- |
|  | NY-NJ-CT | PA-NJ-MD-DE |
| # VMT | 31,145 | 9,604 |
| # VOC (Tons/Day) | 0.01 | 0.004 |
| NOx (Tons/Day) | 0.32 | 0.10 |

PM2.5 Analysis – Annual Benefits (tpy) in New Jersey:

| Benefits (Reductions) | ## PM2.5 Non-Attainment Area |  |
| --- | --- | --- |
|  | NY-NJ-CT | PA-NJ-DE |
| # VMT | 10,638,922 | 3,892,631 |
| NOx (Tons/Year) | 183.87 | 50.83 |
| PM2.5 (Tons/Year) | 7.75 | 2.83 |
| SO2 (Tons/Year) | 0.01 | 0.002 |

Cost Benefit Results:

| Nonattainment Area | Ozone ($/tpd) |  |
| --- | --- | --- |
|  | VOC | NOx |
| NY-NJ-CT | $ 23,058,147 | $ 10,178,644 |
| PA-NJ-MD-DE | $ 23,225,764 | $ 1,025,768 |
| Statewide | $ 23,097,663 | $ 1,019,727 |

| Nonattainment Area | PM2.5 ($/tpy) |  |  |
| --- | --- | --- | --- |
|  | NOx | PM2.5 | SO2 |
| NY-NJ-CT | $ 444,770 | $ 10,558,190 | $ 13,340,128,816 |
| PA-NJ-DE | $ 499,924 | $ 8,966,055 | $ 11,328,487,748 |
| Statewide | $ 456,714 | $ 10,131,698 | $ 12,801,261,650 |