Control Measure:

The project evaluates the impact of placing electric vehicles at transit stations to provide commuters with a way to travel from the stations to job locations nearby.  The electric vehicle would also be used by the commuter to make midday trips before retuning the vehicle to the station at the end of the day.

Description:

While NJ Transit has a number of suburban rail stations, employment tends to be some distance from these rail stations.  Unable to easily complete this “last mile” of their journey, potential transit users instead use private vehicles to make their entire journey.

One proposed solution to this “last mile” issue is to place electric vehicles at transit stations, allowing commuters to use them to travel the last leg of their journey to work.  The advantage of this approach is that not only is the short distance trip from the station to the workplace replaced with a low-polluting mode, but the commuter’s entire journey from home is now a combination of transit and electric vehicle, displacing a more significant amount of VMT than the mileage of the electric vehicles would indicate.  Also, since the commuter would be in possession of the vehicle during the day, any midday trips for lunch or for shopping would be by the electric vehicle rather than a traditionally fueled vehicle.  This would further enhance the benefits of the program.

Methodology:

The impacts of this program were estimated using assumptions about the trips to the region and usage of the vehicle during the day. The NJAQONE “Emissions Only” methodology was used to calculate the air quality impacts of the vehicles.  Calculations were done as though all vehicles were located in Middlesex County.

Traffic impacts are based on the assumption that each vehicle in the program replaces a single auto commuter trip.  The transit portion of the new trip is assumed to use existing services such that no additional transit emissions would occur.  The transportation/traffic impacts of the program were calculated as follows:

| Average Work Trip Length | 12.15 |  |
| --- | --- | --- |
| Return Work Trip | 12.15 |  |
| Midday Return Trip | 4 | (For Lunch, Shopping, Etc) |
|  | 28.3 | VMT reduced |
| Number of vehicles in the program: | X 10 |  |
|  | 283 | VMT reduced (total) |
| Number of trips reduced (2 x 10) | 20 | Vehicle Trips reduced/day |

These transportation impacts were then entered into the NJAQONE software in order to calculate the emissions impacts of the program.  Note, the impacts of this project were so small that the project was entered into NJ-AQONE as 10,000 vehicles and the results divided by 1,000 to yield the actual impact.

Results: Typical Summer Day/Annual Emissions

Total VT Reduced = 20 trips/day Statewide

Total VMT Reduced = 283 miles/day Statewide

Statewide Benefits (Emission Reductions Due to Reduced Queuing):

| Pollutant Type | ## Analysis Year 2009 |  |
| --- | --- | --- |
|  | Tons/Day | Tons/Year |
| # VOC | 0.00015 | - |
| NOx | 0.00015 | 0.0398 |
| PM2.5 | - | 0.00113 |
| SO2 | - | 0.00069 |

Ozone Analysis – Daily Benefits in New Jersey

| Benefits (Reductions) | ## Ozone Non-Attainment Area |  |
| --- | --- | --- |
|  | New York-N. New Jersey-Long Island, NY-NJ-CT | Philadelphia-Wilmington-Atlantic City PA-NJ-MD-DE |
| # VT | 20 | - |
| # VMT | 283 | - |
| # VOC (Tons/Day) | 0.00015 | - |
| NOx (Tons/Day) | 0.00015 | - |

PM2.5 Analysis – Annual Benefits in New Jersey

| Benefits (Reductions) | ## PM2.5 Non-Attainment Area |  |
| --- | --- | --- |
|  | New York-N. New Jersey-Long Island, NY-NJ-CT | Philadelphia-Wilmington PA-NJ-DE |
| # VT | 5,000 | - |
| # VMT | 70,750 | - |
| NOx (Tons/Year) | 0.0398 | - |
| PM2.5 (Tons/Year) | 0.00113 | - |
| SO2 (Tons/Year) | 0.00069 | - |

Cost/Benefit Analysis

Total costs were estimated as follows:

Vehicle cost estimated for $16,000 for a base, sub-compact vehicle + $16,000 to convert to electric.

$32,000*/Car – spread over 4 years = $8,000/year

$7,500/Car/Year Operating Expenses

Cost Per Year:

$15,500 x 10 vehicles = $155,000/year

Annual costs were divided by 250 to obtain the daily cost used in calculating the Ozone cost/benefit ratios

Cost/Benefit Results

| Pollutant Benefits | Cost/Benefit for Each Pollutant |  |  |  |
| --- | --- | --- | --- | --- |
|  | Ozone |  | PM 2.5 |  |
|  | NY-NJ-CT | PA-NJ-DE-MD | NY-NJ-CT | PA-NJ-DE |
| VOC ($/Ton, Daily) | $4,133,333 | - | - | - |
| NOx ($/Ton, Daily) | $4,133,333 | - | - | - |
| NOx ($/Ton, Annual) | - | - | $3,894,472 | - |
| PM2.5 ($/Ton, Annual) | - | - | $137,168,142 | - |
| SO2 ($/Ton, Annual) | - | - | $224,637,681 | - |