Control Measure:

Retrofit Construction Equipment

Description:

Twenty percent of the construction equipment used on state projects must use a combination of retrofit technology and ULSD.

Documentation:

Effectiveness of retrofit technology: http://www.lubrizol.com/EngineControl/products/

Methodology:

The NONROAD 2005 model was used to estimate 2009 VOC, NOx, PM2.5 and SO2 emissions.  Both daily and annual emissions were modeled.  This measure was assumed to result in a 2% reduction of each pollutant.  VOC benefits were only taken from the current exhaust emission totals, this control measure was assumed to have no impact on diurnal, crankcase, displacement or spillage emissions.

Inputs/Assumptions:

- The percent reduction resulting from retrofitting construction equipment used on state projects is assumed to only have an impact on exhaust emissions.

- It is assumed that 10% of the 2009 inventory of the following equipment will be used on state contracts: airport equipment, commercial equipment, construction and mining equipment, industrial equipment and railroad equipment.

- It is assumed that 20% of the vehicles used on state contracts will be required to use retrofit technology.

- Based on information published by Lubrizol Engine Control Systems, retrofit technology is expected to reduce PM emissions by 91% and VOC emissions by 96%.

- The NONROAD 2005 model was used to estimate 2009 emissions.

- The average cost of retrofit equipment is $7,500.

- The expected useful life of the retrofit equipment is 6 years.

- The construction equipment operates on an average of 250 days per year.

NONROAD inputs:

|  | NY-NJ-CT |  | PA-NJ-DE | PA-NJ-MD-DE |
| --- | --- | --- | --- | --- |
| Inputs | Annual | Sum. Day | Annual | Sum. Day |
| Min | 19.1 | 66.8 | 24.2 | 65.1 |
| Max | 83.9 | 82.9 | 85.3 | 78.1 |
| Avg. | 51.0 | 75.0 | 54.4 | 75.0 |
| RVP | 15.0 | 15.0 | 15.0 | 15.0 |
| O2 Weight | 2.1 | 2.1 | 2.1 | 2.1 |
| Gas Sulfur | 0.0030 | 0.0030 | 0.0030 | 0.0030 |
| Diesel Sulfur | 0.0015 | 0.0015 | 0.0015 | 0.0015 |
| Stage II | 0.0 | 0.0 | 0.0 | 0.0 |

Results:

Ozone Analysis – Daily Benefits (tpd) in New Jersey:

| Benefits (Reductions) | ## Ozone Non-Attainment Area |  |
| --- | --- | --- |
|  | NY-NJ-CT | PA-NJ-MD-DE |
| # VOC (Tons/Day) | 0.27 | 0.10 |
| NOx (Tons/Day) | NA | NA |

PM2.5 Analysis – Annual Benefits (tpy) in New Jersey:

| Benefits (Reductions) | ## PM2.5 Non-Attainment Area |  |
| --- | --- | --- |
|  | NY-NJ-CT | PA-NJ-DE |
| NOx (Tons/Year) | NA | NA |
| PM2.5 (Tons/Year) | 21.94 | 3.70 |
| SO2 (Tons/Year) | NA | NA |

Cost Benefit Results:

| Nonattainment Area | Ozone ($/tpd) |  |
| --- | --- | --- |
|  | VOC | NOx |
| NY-NJ-CT | $ 133,392 | NA |
| PA-NJ-MD-DE | $ 111,839 | NA |
| Statewide | $ 127,778 | NA |

| Nonattainment Area | PM2.5 ($/tpy) |  |  |
| --- | --- | --- | --- |
|  | NOx | PM2.5 | SO2 |
| NY-NJ-CT | NA | $ 16,969 | NA |
| PA-NJ-DE | NA | $ 16,891 | NA |
| Statewide | NA | $ 16,958 | NA |