Control Measure:

Replace old fleet vehicles with cleaner vehicles such as Hybrids.

Description:

This measure looks at benefits from replacing 100 old fleet vehicles in each county and replacing them by new Hybrid vehicles such as Toyota Prius.

Methodology:

Emission rates were calculated for an average fleet of 9 years old using MOBILE6.2.  For hybrid vehicle, emission rates of Toyota Prius were used for the calculations.  Assuming that 100 fleet vehicles would be replaced in each county, emission benefits were calculated.

# Documentation:

Hybrid Electric Vehicles in Government Fleets, Air Pollution Control Division, Colorado Department of Public Health and Environment, Dec 2004

Inputs/Assumptions:

- Annual average miles traveled by a fleet vehicle = 12,500 miles/yr

- Fleet vehicles would be used on an average of 250 days/yr.

- Retired fleet vehicles are 9 yrs or older.

- Average speed = 40mph

Results:

Emission Reductions (Statewide):

| Pollutant Type | ## Analysis Year 2010 |  |
| --- | --- | --- |
|  | Tons/Day | Tons/Year |
| VOC | 0.0335 | 9.193 |
| NOx | 0.0574 | 14.86 |
| PM2.5 | 0.0012 | 0.304 |
| SO2 | 0.0008 | 0.191 |

*Note: Negative means increase in emissions

Ozone Analysis – Daily Benefits in New Jersey

| Benefits (Reductions) | ## Ozone Non-Attainment Area |  |
| --- | --- | --- |
|  | New York-N. New Jersey-Long Island, NY-NJ-CT | Philadelphia-Wilmington-Atlantic City PA-NJ-MD-DE |
| # Total Vehicles Replaced | 1,200 | 900 |
| # VOC (Tons/Day) | 0.0191 | 0.0144 |
| NOx (Tons/Day) | 0.0328 | 0.0246 |

*Note: Negative means increase in emissions

PM2.5 Analysis – Annual Benefits in New Jersey

| Benefits (Reductions) | ## PM2.5 Non-Attainment Area |  |
| --- | --- | --- |
|  | New York-N. New Jersey-Long Island, NY-NJ-CT | Philadelphia-Wilmington PA-NJ-DE |
| # Total Vehicles Replaced | 1,000 | 300 |
| NOx (Tons/Year) | 7.076 | 2.123 |
| PM2.5 (Tons/Year) | 0.145 | 0.043 |
| SO2 (Tons/Year) | 0.091 | 0.027 |

*Note: Negative means increase in emissions

COST/BENEFIT ANALYSIS

Total costs were estimated as follows:

|  | Hybrid Sedan | Gasoline Sedan | Difference (Hybrid vs. Gasoline) |
| --- | --- | --- | --- |
| Purchase Price | $20,810 | $19,390 | $1,420 |
| Residual Value | $6,841 | $5,685 | $1,156 |
| Net Price | $13,969 | $13,705 | $264 |

* Based on Colorado Department of Public Health and Environment, Dec 2004 report

Calculations reflect values over the life of the vehicle. The life cycle for all vehicles in the analysis is assumed to be 8 years/100,000 miles in accordance with Colorado State Fleet Management desired vehicle replacement schedule.  Residual Value based on a 5 year, 60-month projection.  Assume that maintenance costs would be the same for the Hybrid and Gasoline Vehicles.  There will be a big savings in fuel costs over the life of the vehicles but that cost has not been considered for this analysis.

Total Annualized Cost 	= $ 264 over 8 years for 100 vehicles/county

= $3,300/county/year

Total cost for the various non-attainment areas are as follows:

| County | Estimated Cost For Each Non-Attainment Area |  |  |  |
| --- | --- | --- | --- | --- |
|  | Ozone |  | PM 2.5 |  |
|  | NY-NJ-CT | PA-NJ-DE | NY-NJ-CT | PA-NJ-DE |
| Atlantic | - | $3,300 | - | - |
| Bergen | $3,300 | - | $3,300 | - |
| Burlington | - | $3,300 | - | $3,300 |
| Camden | - | $3,300 | - | $3,300 |
| Cape May | - | $3,300 | - | - |
| Cumberland | - | $3,300 | - | - |
| Essex | $3,300 | - | $3,300 | - |
| Gloucester | - | $3,300 | - | $3,300 |
| Hudson | $3,300 | - | $3,300 | - |
| Hunterdon | $3,300 | - | - | - |
| Mercer | - | $3,300 | $3,300 | - |
| Middlesex | $3,300 | - | $3,300 | - |
| Monmouth | $3,300 | - | $3,300 | - |
| Morris | $3,300 | - | $3,300 | - |
| Ocean | - | $3,300 | - | - |
| Passaic | $3,300 | - | $3,300 | - |
| Salem | - | $3,300 | - | - |
| Somerset | $3,300 | - | $3,300 | - |
| Sussex | $3,300 | - | - | - |
| Union | $3,300 | - | $3,300 | - |
| Warren | $3,300 | - | - | - |
| Total | $39,600 | $29,700 | $33,000 | $9,900 |

Cost/Benefit Results

| Pollutant | Cost/Benefit for Each Pollutant |  |  |  |
| --- | --- | --- | --- | --- |
|  | Ozone |  | PM 2.5 |  |
|  | NY-NJ-CT | PA-NJ-DE-MD | NY-NJ-CT | PA-NJ-DE |
| VOC ($/Ton/Day) | $8,278 | $8,278 | - | - |
| NOx ($/Ton/Day) | $4,829 | $4,829 | - | - |
| NOx ($/Ton/Year) | - | - | $4,664 | $4,664 |
| PM2.5 ($/Ton/Year) | - | - | $228,093 | $228,093 |
| SO2 ($/Ton/Year) | - | - | $362,875 | $362,875 |

* Daily Costs used in calculation of cost/benefit ratio are Annual costs divided by 250