Control Measure:

Limit the use of lawn and garden equipment

Description:

Create an incentive (perhaps, a form of exchange program) to encourage residential lawnmower users to switch to electric mowers.

Documentation:

- Department of Energy, Energy Information Administration (EIA), Retail Gasoline Prices:

http://www.eia.doe.gov/oil_gas/petroleum/data_publications/wrgp/mogas_home_page.html

Methodology:

The NONROAD 2005 model was used to estimate 2009 VOC, NOx, PM2.5 and SO2 emissions.  Both daily and annual emissions were modeled.  This measure was assumed to result in a 1% reduction of each pollutant.  VOC benefits were only taken from the current exhaust emission totals, this control measure was assumed to have no impact on diurnal, crankcase, displacement or spillage emissions. The NONROAD model was used to estimate the gallons of gasoline used on a daily and annual basis for lawnmower use. The NONROAD model assumes residential diesel mowers will not be in use in 2009.

Inputs/Assumptions:

- The percent reduction resulting from the implementation of restrictions on use are assumed to only have an impact on exhaust emissions.

- It is assumed that the restrictions would result in approximately a 1% reduction in use as the restrictions would be both costly and difficult to police.

- Cost benefits were calculated based on fuel consumption estimates generated using NONROAD 2005.

- Cost estimates to not include costs of enforcing the regulations.

- Gasoline cost = $2.40/gallon.

NONROAD inputs:

|  | NY-NJ-CT |  | PA-NJ-DE | PA-NJ-MD-DE |
| --- | --- | --- | --- | --- |
| Inputs | Annual | Sum. Day | Annual | Sum. Day |
| Min | 19.1 | 66.8 | 24.2 | 65.1 |
| Max | 83.9 | 82.9 | 85.3 | 78.1 |
| Avg. | 51.0 | 75.0 | 54.4 | 75.0 |
| RVP | 15.0 | 15.0 | 15.0 | 15.0 |
| O2 Weight | 2.1 | 2.1 | 2.1 | 2.1 |
| Gas Sulfur | 0.0030 | 0.0030 | 0.0030 | 0.0030 |
| Diesel Sulfur | 0.0015 | 0.0015 | 0.0015 | 0.0015 |
| Stage II | 0.0 | 0.0 | 0.0 | 0.0 |

Results:

Ozone Analysis – Daily Benefits (tpd) in New Jersey:

| Benefits (Reductions) | ## Ozone Non-Attainment Area |  |
| --- | --- | --- |
|  | NY-NJ-CT | PA-NJ-MD-DE |
| # VOC (Tons/Day) | 0.05 | 0.03 |
| NOx (Tons/Day) | 0.01 | 0.004 |

PM2.5 Analysis – Annual Benefits (tpy) in New Jersey:

| Benefits (Reductions) | ## PM2.5 Non-Attainment Area |  |
| --- | --- | --- |
|  | NY-NJ-CT | PA-NJ-DE |
| NOx (Tons/Year) | 1.28 | 0.26 |
| PM2.5 (Tons/Year) | 0.06 | 0.01 |
| SO2 (Tons/Year) | 0.006 | 0.001 |

Cost Benefit Results:

| Nonattainment Area | Ozone ($/tpd) |  |
| --- | --- | --- |
|  | VOC | NOx |
| NY-NJ-CT | $(11,343) | $ (84,461) |
| PA-NJ-MD-DE | $(11,343) | $ (84,464) |
| Statewide | $(11,343) | $ (84,462) |

| Nonattainment Area | PM2.5 ($/tpy) |  |  |
| --- | --- | --- | --- |
|  | NOx | PM2.5 | SO2 |
| NY-NJ-CT | $(68,185) | $ (1,382,401) | $ (14,587,642) |
| PA-NJ-DE | $(70,284) | $ (1,382,401) | $ (14,587,642) |
| Statewide | $(68,541) | $ (1,382,401) | $ (14,587,642) |

* Parentheses denote a cost savings.