VDEQ Conceptual Description for the Inclusion of Distributed Generation Units in Base Year and Future Year Modeling Efforts

Issue:

VDEQ has identified the need to examine more closely in modeling studies the effects on ozone air quality of distributed generation units.

Background:

Distributed generation units are mainly internal combustion engines that participate in some type of demand response program.  These are small units, each usually no more than 2 or 3 megawatts in capacity, and they generally run on distillate fuel oil  Most are permitted for well under 100 tons of NOx emissions annually and do not run frequently.

Annual emissions of NOX are usually not very large from these units.  However, ozone season daily emissions estimates from previous ozone SIPs show that facilities that have one or more of these types of units can be quite significant NOX emitters when examined on an ozone season daily basis.  For instance, in 2002 in the Northern Virginia portion of the metro DC ozone nonattainment area, the inventory estimate for point sources includes about 60 tons/day of NOX.  EGU’s account for 40 tons/day of NOX.  Distributed generation units account for just over 10 tons/day NOX.  In 2002 for the Northern Virginia region, distributed generation units were the second largest point source NOX-emitting category, and many new facilities of this type have been permitted since 2002.

In Northern Virginia especially, but across the Commonwealth, demand response programs are gaining in popularity, and utilities are actively recruiting participants.  Such demand response programs are very helpful in maintaining the integrity of the power grid during high demand days or times when power capacity in small areas with transmission constraints is being taxed.  However, older units can have very high emissions, and control strategies do exist to significantly reduce NOX from these units.

Previous modeling efforts minimized the effects of these units on air quality because the annual emission rates were not very large (less than 100 tons/year NOX) and because the default profile of 365 days of operation annually was applied to the annual emission rate in the modeling inventories.  This calculation methodology greatly minimized the impacts from these units.

VDEQ would like to facilitate the review of these units for potential future control strategies by creating a profile for distributed generation that more accurately reflects operating conditions.  Such a profile will limit operations of such units to a smaller, more representative number of days during the year and to a limited number of hours during each day, as dictated by actual operating data.  NOx emissions on a daily basis will therefore increase as compared to the daily NOX emissions based on the default profile.  NOX emissions will also vary from day to day, depending on the number of hours of operation for that day.  Once these units are more accurately represented in base year emissions inventories and modeling platforms, control strategy runs may be performed to determine if controlling these units will improve ozone air quality in the Northern Virginia/metro DC area.

Methodology:

To more accurately characterize emissions from distributed generation, VDEQ intends to follow these steps in the inventory and modeling process.

1.	Inclusion of DG units in the Emissions Inventory:  The first step is to ensure that as many distributed generation units are included in the base year inventory as possible.  Often agencies use 10 tons/year of any criteria pollutant as a cutoff for inclusion of point sources in the point source modeling inventory.  VDEQ used a 10 ton/year level to ensure that as many units were captured as possible, while screening out sources with emissions that are too small to be of any consequence.  However, VDEQ also included stack/point/segment combinations identified by the regional office or by EIA data as distributed generation units, regardless of emissions rates. These line items were identified by county plant identifiers as well as stack, point, segment identifiers.  2007 data were retrieved from CEDS on these line items, and this data was provided to the regional planning organization (MARAMA) as well as the base year emissions inventory contractor (MACTEC).  A listing of the identified DG units in the 2007 base year inventory is contained in Table 5.  The table identifies if the unit was originally part of the 2007 inventory or if it was added to the inventory as a DG unit.

2.	Identification of DG units:  VDEQ also intends to maintain a listing of stack/point/segment combinations that represent DG units both statewide and in the Northern Virginia area.  The emissions profile created from base year (assumed to be 2007) operations data will be applied to all pollutant emissions for each unit identified as a DG unit.  This listing will also be helpful when applying a NOX control strategy reduction factor.

At least one regional office, the Northern Regional Office (NRO), has a detailed listing of registration numbers and units that are or may be distributed generation units.  Additionally, EIA data can be used to identify other units.  Analysis of the EIA data for Virginia units has already been performed to ensure that these units are in the Virginia database.  CEDS data regarding hours of operation were examined in creating the listing in Table 5.  If a unit’s hours of operation as listed in CEDs indicated significantly more operational time than would normally be expected by a distributed generation unit, then that unit was removed from the listing and will not have the distributed generation profile applied in the 2007 base year regional air quality modeling.  Figure 1 below shows the locations of these facilities in the Commonwealth.

Figure 1:  Location of DG Units in the Commonwealth

3.	Inventory Review:  After a listing of potential DG units was created from EIA and regional office input, the 2007 inventory data for all line items were examined and modified as follows:

3.a. Stack Parameters:  Stack data for each stack/point/segment combination were examined to ensure all necessary modeling parameters exist in the database.  A point to note is that many IC engines have horizontal stacks and are equipped with some type of sound muffler.  Newer units are more often equipped with vertical stacks.  For any stack with an obstructed or non-vertical discharge, the exit gas velocity was changed to 0.001 meters/second (0.003 feet/second).   For any stack with no stack data due to the stack being labeled as a fugitive emissions source, the height was changed to 15 feet, the diameter was changed to 1.5 feet, the stack temperature was changed to 900 deg F, the stack configuration was changed to a horizontal discharge, and the exit gas velocity was changed to 0.001 meters/second (0.003 feet/second).   A few line items did not have latitude and/or longitude data in the correct decimal format.  That information was provided from the UTM converter in CEDS.

3.b. Non DG operations:  Care was taken to remove from the inventory any engines that may be associated with rock processing plants, asphalt plants, or concrete plants.   Also, for facilities that were beneath the 10 tons/year cut off and therefore would not normally be included in the 2007 point source modeling inventory, any line item in that facility’s inventory that was not related to DG operations was deleted and not added to the 2007 point source modeling inventory.

3.c. Non-operational units:  For facilities that were beneath the 10 tons/year threshold and therefore not normally included in the 2007 point source modeling inventory, any distributed generator line item with zero emissions was deleted, even if the line item was considered to be planned for operations.  Also deleted from the DG inventory was any line item identified as DG and normally in the 2007 point source modeling inventory but that had zero 2007 emissions.

3.d. Emissions inventory:  The 2007 CEDS data only included the filterable portion of the particulate emissions from all units.  Additionally, some DG units were coded with both PM10 and PM2.5, and some were coded with either PM10 or PM2.5.  Some were also coded without any mention of PM10 and PM2.5.  The main thrust of this inventory and modeling exercise is to examine DG units’ effect on ozone concentrations.  However, since augmented data to help fill in data gaps were available from MACTEC, the augmented data containing filterable and condensable PM emissions fractions as well as data for PM10 and PM2.5 were included in the final DG inventory.

4.	Creation of an emissions profile:  VDEQ used 2007 operations data obtained from Dominion Virginia Power for their demand response programs (including the Manassas VMEA program) as well as data obtained from AOL, NOVEC, and duPont Fabros to create a 2007 profile for when these units generally operated.  To simplify the creation of the profile, units are assumed to be operating at full capacity whenever they are being used.  The generic profile will be applied to all units identified as DG units in the 2007 emissions inventory for Virginia.

5.	Application of the emissions profile:  VDEQ developed a generic profile in a SMOKE readable format for each pollutant listed in the emissions inventory.  This profile simulates a step function, turning on and off the identified generation units depending on their operational mode.  VDEQ passed on this profile to the emissions inventory contractors and requested that it be included in both base and future inventories for SMOKE modeling.

Figure 2 below graphically presents the application of the chosen profile during May of 2007 on an example unit.  Units are assumed to be on or off, and when on, the units are assumed to be operating at the same load and emissions rates.

Figure 2:  May 2007 DG Operations for 51-047-00044

6.	Future year control strategies:  In future years, control strategies that assume up to 90% -95% NOX reduction can be applied to the distributed generation units that are not already controlled.

7.	Growth Factors:  In previous emissions inventory work, growth factors for these types of units were created using E-GAS, which underestimated the growth of this source sector between 2002 and 2009.  One issue for future consideration is the creation of a better estimate of growth between base and future years for this source sector.

Development of the Emissions Profile:

Table 4 below summarizes the information collected from a variety of sources in the distributed generation sector.  Dominion provided 2007 data for its stand-by generation (SG) program.  The program is also referred to as a curtailable service and stand-by generation program.  The program requests that facilities reduce load such as shutting down a production line or requests that existing generation be brought on line to peak shave or sell power back to the grid.  Dominion also provided 2007 operations data for the VMEA, which is a co-op with the City of Manassas.  These units can operate as stand-by generation or peak shavers.

The NRO staff provided data gathered during the execution of several enforcement actions against distributed generation units.  Data from an America On-Line (AOL) data center site, a duPont Fabros (DFT) data center site, and Northern Virginia Electric Cooperative (NOVEC) are listed.  These facilities have large amounts of generation on site, sometimes totaling more than 25 MW of capacity.  The purpose of the generation at data centers is to provide back up power to ensure that data center operations maintain a high level of up time, often required in contracts to be better than 99.9% reliable.  These facilities also have lucrative contracts to use this generation in a distributed generation program.  It should be noted that as a result of the enforcement actions brought by NRO, many of these units have been retrofitted with either open or closed loop SCR to reduce NOX emission by 80%-90%.

Table 4 demonstrates that in 2007, a significant amount of overlap existed for days and times when these units were in operation.  Also, four days (grey highlighted rows) contain two separate time periods when units were called to participate in various programs.  To create a generic profile, Table 4 examines two methods for analyzing the data.

The first method (option 1) includes every day that any program operated according to the known data.  For any day that a program operated, the data for the day with the most hours of operation was included in the generic profile.  These columns are labeled “Generic Profile, Any Run Day” in Table 1.  When including all days for which data exist, the generic profile for 2007 would contain 514 hours of operation over 111 days during the year.  When compared to the maximum that any one particular program operated (93 days, and 314 hours), this methodology tends to overestimate the amount of time any unit is on line. However, as shown in the examples below, the results are quite a bit higher than the application of the default profile.

The second method (option 2) includes every day that any two programs operated according to the known data.  For any day that two programs operated, the data for the day with the most hours of operation was included in the generic profile.  These columns are labeled “Generic Profile, Days with Two or More Data Points.”  When including these parameters in the make-up of the generic profile, the profile for 2007 would contain 381 hours and 92 day of operation.  This methodology seems to better represent the maximum program times and days although it may still overestimate the number of hours of operation in the stand-by generation program operated by Dominion.  Overestimation of the number of hours results in underestimation of daily and hourly emission rates for NOX.

As an example of how each profile may be applied, assume a facility has three units, each 2 MW.  Two units reported emitting 10 tons of NOx in 2007, each.  One unit emitted 5 tons of NOx in 2007.

If the default, 365 day, 8760 hrs/year operational profile was assumed, the units would have the following emission rates in the modeling analysis:

Table 1:  Example Emissions from a Hypothetical DG Facility, using 365 days/year

| Unit | Annual NOx | Daily NOx |
| --- | --- | --- |
| Unit 1 | 10 tons NOx/year | 0.027 tons NOx/day |
| Unit 2 | 10 tons NOx/year | 0.027 tons NOx/day |
| Unit 3 | 5 tons NOx/year | 0.014 tons NOx/day |
| Total NOx in 2007 | 25 tons NOx | 0.068 tons NOx/day |

The calculation is as follows:

10 tons NOx/yr * 1 yr/8,760 hrs * 24 hrs/day = 0.027 tons NOx/day

Each day and each hour would have exactly the same amount of emissions.

If the first generic profile is used (option 1), 514 hours are divided up over 111 days.  The maximum number of hours of operation in any day is 16, which occurs in December.  Twelve hours is the maximum for the ozone season and occurs August 8th from 11:00 to 23:00.  For this ozone season example, 12 hours will be used:

Table 2:  Example Emissions from a Hypothetical DG Facility, using 111 days/year

| Unit | Annual NOx | Daily Ozone Season NOx |
| --- | --- | --- |
| Unit 1 | 10 tons NOx/year | 0.23 tons NOx/day |
| Unit 2 | 10 tons NOx/year | 0.23 tons NOx/day |
| Unit 3 | 5 tons NOx/year | 0.12 tons NOx/day |
| Total NOx in 2007 | 25 tons NOx | 0.58 tons NOx/day |

10 tons NOx/yr * 1 yr/514 hr * 12 hours/day = 0.23 tons NOx/day

In the model, these hourly and daily emissions for August 8th would be assigned to 11:00 through 23:00.  Each day would vary, as noted in Table 4.

If the second generic profile is used (option 2), 381 hours will be divided over 92 days.  Again, August 8th provides the summer day with the most hours of operation (12), so this day will be used for comparison:

Table 3:  Example Emissions from a Hypothetical DG Facility, using 92 days/year

| Unit | Annual NOx | Daily Ozone Season NOx |
| --- | --- | --- |
| Unit 1 | 10 tons NOx/year | 0.31 tons NOx/day |
| Unit 2 | 10 tons NOx/year | 0.31 tons NOx/day |
| Unit 3 | 5 tons NOx/year | 0.16 tons NOx/day |
| Total NOx in 2007 | 25 tons NOx | 0.78 tons NOx/day |

10 tons NOx/yr * 1 yr/381 hours * 12 hours/day = 0.31 tons NOx/day

This methodology is not nearly as accurate a procedure for assigning emissions as is the CAMD emissions data from CEMS.  However, at the current time, it may be the best methodology available for better representing the emissions from the distributed generation category in the base year emissions inventory.

After review of the data, option 2 above was chosen to create the profile for the DG units.  This approach omits days from the profile where only one system was running, which reduces the time operated by 19 days.  It spreads the emissions over 381 hours in the year rather than 514 hours.  This total hourly value is higher than any of the hourly operations of any of the systems as shown in Table 4 so that the estimates on an hourly basis from each system should be conservative, with little overestimation of hourly emission rates.

| Table 4: Operational Data from Selected VA Companies Engaged in Distributed Generation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DATE | NOVEC |  |  | AOL |  |  | DFT | Dominion-SG |  |  | Dominion-VMEA |  |  | Generic Profile, Any Run Day (option 1) |  |  | Generic Profile, Days with 2 or more data points (option 2) |  |  |
|  | Hrs | Start Time | Stop Time | Hrs | Start Time | Stop Time | Hrs | Hrs | Start Time | Stop Time | Hrs | Start Time | Stop Time | Hrs | Start Time | Stop Time | Hrs | Start Time | Stop Time |
| 1/3/2007 | 2 | 7:00 | 9:00 | 2 | 0700 | 0900 | 2.5 |  |  |  |  |  |  | 2 | 0700 | 0900 | 2 | 0700 | 0900 |
| 1/10/2007 | 1.75 | 7:00 | 8:40 | 1.5 | 0700 | 0830 | 2 |  |  |  |  |  |  | 1.75 | 7:00 | 8:40 | 1.75 | 7:00 | 8:40 |
| 1/10/2007 | 3 | 18:00 | 21:00 | 3 | 1800 | 2100 | 3.6 |  |  |  |  |  |  | 3 | 18:00 | 21:00 | 3 | 18:00 | 21:00 |
| 1/11/2007 | 1.5 | 7:00 | 8:30 | 1.5 | 0700 | 0830 | 2 |  |  |  |  |  |  | 1.5 | 7:00 | 8:30 | 1.5 | 7:00 | 8:30 |
| 1/17/2007 | 1.25 | 7:00 | 8:15 | 1 | 0700 | 0800 | 1.7 |  |  |  |  |  |  | 1.25 | 7:00 | 8:15 | 1.25 | 7:00 | 8:15 |
| 1/18/2007 | 2 | 7:00 | 9:00 | 2 | 0700 | 0900 | 2.6 |  |  |  |  |  |  | 2 | 7:00 | 9:00 | 2 | 7:00 | 9:00 |
| 1/26/2007 | 2 | 7:00 | 9:00 | 1.5 | 0700 | 0830 | 2.1 |  |  |  |  |  |  | 2 | 7:00 | 9:00 | 2 | 7:00 | 9:00 |
| 1/30/2007 | 1.25 | 7:00 | 8:15 | 1 | 0700 | 0800 | 1.7 |  |  |  |  |  |  | 1.25 | 7:00 | 8:15 | 1.25 | 7:00 | 8:15 |
| 2/1/2007 | 2 | 7:00 | 9:00 | 2 | 0700 | 0900 | 2.6 |  |  |  |  |  |  | 2 | 7:00 | 9:00 | 2 | 7:00 | 9:00 |
| 2/1/2007 | 2 | 18:00 | 20:00 | 2 | 1800 | 2000 | 2.6 |  |  |  |  |  |  | 2 | 18:00 | 20:00 | 2 | 18:00 | 20:00 |
| 2/5/2007 | 3 | 7:00 | 10:00 | 2 | 0700 | 0900 | 2.6 | 5 | 600 | 1100 | 4 | 500 | 900 | 5 | 600 | 1100 | 5 | 600 | 1100 |
| 2/5/2007 | 3 | 18:00 | 21:00 | 3 | 1800 | 2100 | 3.5 |  |  |  |  |  |  | 3 | 18:00 | 21:00 | 3 | 18:00 | 21:00 |
| 2/6/2007 | 3.25 | 6:00 | 9:15 | 3 | 0600 | 0900 | 3.6 | 5 | 600 | 1100 | 5 | 500 | 1000 | 5 | 500 | 1000 | 5 | 500 | 1000 |
| 2/16/2007 | 1.25 | 7:00 | 8:15 | 2 | 0700 | 0900 | 1.7 | 5 | 1700 | 2200 |  |  |  | 5 | 1700 | 2200 | 5 | 1700 | 2200 |
| 3/1/2007 | 3 | 6:00 | 9:00 | 3 | 0600 | 0900 | 3.4 |  |  |  |  |  |  | 3 | 6:00 | 9:00 | 3 | 6:00 | 9:00 |
| 3/5/2007 | 2.5 | 6:00 | 8:30 | 2.5 | 0600 | 0830 | 3 |  |  |  |  |  |  | 2.5 | 6:00 | 8:30 | 2.5 | 6:00 | 8:30 |
| 3/5/2007 | 3 | 18:00 | 21:00 | 3 | 1800 | 2100 | 3.6 |  |  |  |  |  |  | 3 | 18:00 | 21:00 | 3 | 18:00 | 21:00 |
| 3/6/2007 | 3 | 6:00 | 9:00 | 3 | 0600 | 0900 | 3.8 |  |  |  |  |  |  | 3 | 6:00 | 9:00 | 3 | 6:00 | 9:00 |
| 3/6/2007 | 2 | 19:00 | 21:00 | 2 | 1900 | 2100 | 2.5 |  |  |  |  |  |  | 2 | 19:00 | 21:00 | 2 | 19:00 | 21:00 |
| 3/7/2007 | 2.75 | 6:00 | 8:40 | 2.5 | 0600 | 0900 | 3 |  |  |  |  |  |  | 2.75 | 6:00 | 8:40 | 2.75 | 6:00 | 8:40 |
| 3/7/2007 | 2 | 19:00 | 21:00 | 2 | 1900 | 2100 | 2.3 |  |  |  |  |  |  | 2 | 19:00 | 21:00 | 2 | 19:00 | 21:00 |
| 3/8/2007 | 2.75 | 6:00 | 8:45 | 2.5 | 0600 | 0830 | 3.3 |  |  |  |  |  |  | 2.75 | 6:00 | 8:45 | 2.75 | 6:00 | 8:45 |
| 3/9/2007 |  |  |  | 2.25 | 0600 | 0800 |  |  |  |  |  |  |  | 2.25 | 0600 | 0800 |  |  |  |
| 4/6/2007 | 3 | 7:00 | 10:00 | 3 | 0700 | 1000 | 3.7 |  |  |  |  |  |  | 3 | 7:00 | 10:00 | 3 | 7:00 | 10:00 |
| 4/7/2007 | 3 | 7:00 | 10:00 | 3 | 0700 | 1000 | 3.7 |  |  |  |  |  |  | 3 | 7:00 | 10:00 | 3 | 7:00 | 10:00 |
| 4/7/2007 | 1 | 20:00 | 21:00 | 1 | 2000 | 2100 | 1.6 |  |  |  |  |  |  | 1 | 20:00 | 21:00 | 1 | 20:00 | 21:00 |
| 4/8/2007 | 3 | 7:00 | 10:00 | 3 | 0700 | 1000 | 3.7 |  |  |  |  |  |  | 3 | 7:00 | 10:00 | 3 | 7:00 | 10:00 |
| 4/9/2007 | 2 | 7:00 | 9:00 | 2 | 0700 | 0900 | 2.6 |  |  |  |  |  |  | 2 | 7:00 | 9:00 | 2 | 7:00 | 9:00 |
| 4/10/2007 | 2 | 7:00 | 9:00 | 2 | 0700 | 0900 | 2.5 |  |  |  |  |  |  | 2 | 7:00 | 9:00 | 2 | 7:00 | 9:00 |
| 4/11/2007 | 1.5 | 7:00 | 8:20 | 2 | 0700 | 0900 | 1.7 |  |  |  |  |  |  | 2 | 0700 | 0900 | 2 | 0700 | 0900 |
| 4/25/2007 | 5 | 16:00 | 21:00 | 5 | 1600 | 2100 | 5.7 |  |  |  |  |  |  | 5 | 1600 | 2100 | 5 | 1600 | 2100 |
| 5/1/2007 | 5.5 | 16:00 | 21:35 | 5 | 1600 | 2100 | 5.5 |  |  |  |  |  |  | 5.5 | 16:00 | 21:35 | 5.5 | 16:00 | 21:35 |
| 5/2/2007 | 4 | 15:00 | 19:00 | 4 | 1500 | 1900 | 4.5 |  |  |  |  |  |  | 4 | 15:00 | 19:00 | 4 | 15:00 | 19:00 |
| 5/11/2007 | 3 | 15:00 | 18:00 | 3 | 1500 | 1800 | 3.5 |  |  |  |  |  |  | 3 | 15:00 | 18:00 | 3 | 15:00 | 18:00 |
| 5/25/2007 | 3 | 15:00 | 18:00 | 3 | 1500 | 1800 | 3.4 |  |  |  |  |  |  | 3 | 15:00 | 18:00 | 3 | 15:00 | 18:00 |
| 5/26/2007 | 3 | 15:00 | 18:00 | 3 | 1500 | 1800 | 3.5 |  |  |  |  |  |  | 3 | 15:00 | 18:00 | 3 | 15:00 | 18:00 |
| 5/27/2007 | 3 | 15:00 | 18:00 | 3.75 | 1500 | 1800 | 3.6 |  |  |  |  |  |  | 3.75 | 1500 | 1800 | 3.75 | 1500 | 1800 |
| 5/28/2007 | 3 | 15:00 | 18:00 | 3 | 1500 | 1800 | 3.3 |  |  |  |  |  |  | 3 | 15:00 | 18:00 | 3 | 15:00 | 18:00 |
| 5/29/2007 | 2 | 16:00 | 18:00 | 2 | 1600 | 1800 | 2.5 |  |  |  |  |  |  | 2 | 16:00 | 18:00 | 2 | 16:00 | 18:00 |
| 5/30/2007 | 2 | 16:00 | 18:00 | 2 | 1600 | 1800 | 2.4 |  |  |  |  |  |  | 2 | 16:00 | 18:00 | 2 | 16:00 | 18:00 |
| 5/31/2007 | 5 | 14:00 | 19:00 | 4 | 1400 | 1800 | 4.4 |  |  |  |  |  |  | 5 | 14:00 | 19:00 | 5 | 14:00 | 19:00 |
| 6/1/2007 | 2 | 16:00 | 18:00 | 2 | 1600 | 1800 | 2.4 |  |  |  |  |  |  | 2 | 16:00 | 18:00 | 2 | 16:00 | 18:00 |
| 6/8/2007 | 5 | 15:00 | 20:00 | 3 | 1500 | 1800 | 3.5 | 7 | 1400 | 2100 | 9 | 1200 | 2100 | 5 | 15:00 | 20:00 | 5 | 15:00 | 20:00 |
| 6/13/2007 |  |  |  | 3.75 | 1630 | 2015 |  |  |  |  |  |  |  | 3.75 | 1630 | 2015 |  |  |  |
| 6/18/2007 | 4 | 16:00 | 20:00 | 2 | 1600 | 1800 | 2.4 | 7 | 1400 | 2100 | 9 | 1200 | 2100 | 9 | 1200 | 2100 | 9 | 1200 | 2100 |
| 6/19/2007 | 5 | 15:00 | 20:00 | 3 | 1500 | 1800 | 3.4 | 7 | 1400 | 2100 | 9 | 1200 | 2100 | 9 | 1200 | 2100 | 9 | 1200 | 2100 |
| 6/26/2007 | 4 | 15:00 | 19:00 | 2 | 1500 | 1700 | 2.5 | 7 | 1400 | 2100 | 8 | 1200 | 2000 | 8 | 1200 | 2000 | 8 | 1200 | 2000 |
| 6/27/2007 | 5 | 15:00 | 20:00 | 3 | 1500 | 1800 | 3.5 | 7 | 1400 | 2100 | 8 | 1200 | 2000 | 8 | 1200 | 2000 | 8 | 1200 | 2000 |
| 6/28/2007 | 1 | 15:00 | 16:00 | 1 | 1500 | 1600 | 1.5 | 7 | 1400 | 2100 | 7 | 1300 | 2000 | 7 | 1300 | 2000 | 7 | 1300 | 2000 |
| 7/8/2007 | 4 | 16:00 | 20:00 | 2 | 1600 | 1800 | 2.4 |  |  |  |  |  |  | 4 | 16:00 | 20:00 | 4 | 16:00 | 20:00 |
| 7/9/2007 | 6 | 14:00 | 20:00 | 3 | 1500 | 1800 | 3.5 | 7 | 1400 | 2100 | 8 | 1100 | 1900 | 8 | 1100 | 1900 | 8 | 1100 | 1900 |
| 7/10/2007 | 5 | 15:00 | 20:00 | 4 | 1500 | 1900 | 4.5 | 7 | 1400 | 2100 | 7 | 1300 | 2000 | 7 | 1400 | 2100 | 7 | 1400 | 2100 |
| 7/16/2007 |  |  |  |  |  |  |  | 7 | 1400 | 2100 | 7 | 1300 | 2000 | 7 | 1400 | 2100 | 7 | 1400 | 2100 |
| 7/17/2007 |  |  |  |  |  |  |  | 7 | 1400 | 2100 |  |  |  | 7 | 1400 | 2100 |  |  |  |
| 7/18/2007 |  |  |  |  |  |  |  |  |  |  | 8 | 1200 | 2000 | 8 | 1200 | 2000 |  |  |  |
| 7/19/2007 |  |  |  |  |  |  |  | 7 | 1400 | 2100 |  |  |  | 7 | 1400 | 2100 |  |  |  |
| 7/29/2007 |  |  |  | 3.25 | 1600 | 1915 |  |  |  |  |  |  |  | 3.25 | 1600 | 1915 |  |  |  |
| 8/1/2007 | 5 | 15:00 | 20:00 | 4 | 1500 | 1900 | 4.7 | 7 | 1400 | 2100 |  |  |  | 7 | 1400 | 2100 | 7 | 1400 | 2100 |
| 8/2/2007 | 5 | 15:00 | 20:00 | 4 | 1500 | 1900 | 4.5 | 7 | 1400 | 2100 |  |  |  | 7 | 1400 | 2100 | 7 | 1400 | 2100 |
| 8/3/2007 | 5 | 15:00 | 20:00 | 4 | 1500 | 1900 | 4.5 | 7 | 1400 | 2100 | 8 | 1300 | 2100 | 8 | 1300 | 2100 | 8 | 1300 | 2100 |
| 8/4/2007 | 5 | 15:00 | 20:00 | 4 | 1500 | 1900 | 4.5 |  |  |  |  |  |  | 5 | 15:00 | 20:00 | 5 | 15:00 | 20:00 |
| 8/6/2007 | 5 | 15:00 | 20:00 | 4 | 1500 | 1900 | 4.5 | 7 | 1400 | 2100 | 8 | 1300 | 2100 | 8 | 1300 | 2100 | 8 | 1300 | 2100 |
| 8/7/2007 | 5 | 15:00 | 20:00 | 4 | 1500 | 1900 | 4.5 | 7 | 1400 | 2100 |  |  |  | 7 | 1400 | 2100 | 7 | 1400 | 2100 |
| 8/8/2007 | 7 | 14:00 | 20:00 | 5 | 1400 | 1900 | 5.5 | 7 | 1400 | 2100 | 12 | 1100 | 2300 | 12 | 1100 | 2300 | 12 | 1100 | 2300 |
| 8/9/2007 | 1.5 | 14:00 | 15:30 | 2 | 1400 | 1600 | 2.8 | 7 | 1400 | 2100 | 9 | 1200 | 2100 | 9 | 1200 | 2100 | 9 | 1200 | 2100 |
| 8/15/2007 |  |  |  |  |  |  |  |  |  |  | 9 | 1200 | 2100 | 9 | 1200 | 2100 |  |  |  |
| 8/16/2007 |  |  |  |  |  |  |  |  |  |  | 10 | 1200 | 2200 | 10 | 1200 | 2200 |  |  |  |
| 8/25/2007 | 3.25 | 16:40 | 20:00 |  |  |  |  |  |  |  |  |  |  | 3.25 | 16:40 | 20:00 |  |  |  |
| 8/29/2007 |  |  |  |  |  |  |  |  |  |  | 11 | 1100 | 2200 | 11 | 1100 | 2200 |  |  |  |
| 8/30/2007 |  |  |  |  |  |  |  |  |  |  | 9 | 1200 | 2100 | 9 | 1200 | 2100 |  |  |  |
| 9/3/2007 | 3 | 16:00 | 19:00 | 2 | 1600 | 1800 |  |  |  |  |  |  |  | 3 | 16:00 | 19:00 | 3 | 16:00 | 19:00 |
| 9/4/2007 | 4 | 15:00 | 19:00 | 3 | 1500 | 1800 |  |  |  |  |  |  |  | 4 | 15:00 | 19:00 | 4 | 15:00 | 19:00 |
| 9/5/2007 | 4 | 15:00 | 19:00 | 3 | 1500 | 1800 |  |  |  |  |  |  |  | 4 | 15:00 | 19:00 | 4 | 15:00 | 19:00 |
| 9/6/2007 | 4 | 15:00 | 19:00 | 3 | 1500 | 1800 |  |  |  |  |  |  |  | 4 | 15:00 | 19:00 | 4 | 15:00 | 19:00 |
| 9/7/2007 | 4 | 15:00 | 19:00 | 3 | 1500 | 1800 |  |  |  |  |  |  |  | 4 | 15:00 | 19:00 | 4 | 15:00 | 19:00 |
| 9/8/2007 | 4 | 15:00 | 19:00 |  |  |  |  |  |  |  |  |  |  | 4 | 15:00 | 19:00 |  |  |  |
| 9/10/2007 | 4 | 15:00 | 19:00 | 3 | 1500 | 1800 |  |  |  |  | 9 | 1200 | 2100 | 9 | 1200 | 2100 | 9 | 1200 | 2100 |
| 9/10/2007 |  |  |  |  |  |  |  |  |  |  | 5 | 1600 | 2100 | 5 | 1600 | 2100 |  |  |  |
| 9/25/2007 |  |  |  |  |  |  |  |  |  |  | 9 | 1200 | 2100 | 9 | 1200 | 2100 |  |  |  |
| 9/26/2007 |  |  |  |  |  |  |  |  |  |  | 10 | 1100 | 2100 | 10 | 1100 | 2100 |  |  |  |
| 10/1/2007 | 6 | 15:00 | 21:00 | 6 | 1500 | 2100 |  |  |  |  |  |  |  | 6 | 15:00 | 21:00 | 6 | 15:00 | 21:00 |
| 10/2/2007 | 6 | 15:00 | 21:00 | 6 | 1500 | 2100 |  |  |  |  |  |  |  | 6 | 15:00 | 21:00 | 6 | 15:00 | 21:00 |
| 10/3/2007 | 6 | 15:00 | 21:00 | 6 | 1500 | 2100 |  |  |  |  |  |  |  | 6 | 15:00 | 21:00 | 6 | 15:00 | 21:00 |
| 10/4/2007 | 5 | 16:00 | 21:00 | 5 | 1600 | 2100 |  |  |  |  |  |  |  | 5 | 16:00 | 21:00 | 5 | 16:00 | 21:00 |
| 10/6/2007 | 6 | 15:00 | 21:00 | 5 | 1500 | 2000 |  |  |  |  |  |  |  | 6 | 15:00 | 21:00 | 6 | 15:00 | 21:00 |
| 10/7/2007 | 6 | 15:00 | 21:00 | 5 | 1500 | 2000 |  |  |  |  |  |  |  | 6 | 15:00 | 21:00 | 6 | 15:00 | 21:00 |
| 10/8/2007 | 6 | 15:00 | 21:00 | 6 | 1500 | 2100 |  |  |  |  |  |  |  | 6 | 15:00 | 21:00 | 6 | 15:00 | 21:00 |
| 10/9/2007 | 6 | 15:00 | 21:00 | 6 | 1500 | 2100 |  |  |  |  |  |  |  | 6 | 15:00 | 21:00 | 6 | 15:00 | 21:00 |
| 11/2/2007 | 1 | 7:00 | 8:00 | 1 | 0700 | 0800 |  |  |  |  |  |  |  | 1 | 7:00 | 8:00 | 1 | 7:00 | 8:00 |
| 11/7/2007 | 2 | 7:00 | 9:00 | 2 | 0700 | 0900 |  |  |  |  |  |  |  | 2 | 7:00 | 9:00 | 2 | 7:00 | 9:00 |
| 11/8/2007 | 3 | 6:00 | 9:00 | 3 | 0600 | 0900 |  |  |  |  |  |  |  | 3 | 6:00 | 9:00 | 3 | 6:00 | 9:00 |
| 11/9/2007 | 2 | 6:00 | 8:00 | 2 | 0600 | 0800 |  |  |  |  |  |  |  | 2 | 6:00 | 8:00 | 2 | 6:00 | 8:00 |
| 11/16/2007 | 2 | 18:00 | 20:00 | 2 | 1800 | 2000 |  |  |  |  |  |  |  | 2 | 18:00 | 20:00 | 2 | 18:00 | 20:00 |
| 11/19/2007 | 2 | 6:00 | 8:00 | 2 | 0600 | 0800 |  |  |  |  |  |  |  | 2 | 6:00 | 8:00 | 2 | 6:00 | 8:00 |
| 11/19/2007 | 2 | 18:00 | 20:00 | 2 | 1800 | 2000 |  |  |  |  |  |  |  | 2 | 18:00 | 20:00 | 2 | 18:00 | 20:00 |
| 11/28/2007 | 3 | 6:00 | 9:00 | 2 | 0700 | 0900 |  |  |  |  |  |  |  | 3 | 6:00 | 9:00 | 3 | 6:00 | 9:00 |
| 11/28/2007 | 2 | 18:00 | 20:00 | 2 | 1800 | 2000 |  |  |  |  |  |  |  | 2 | 18:00 | 20:00 | 2 | 18:00 | 20:00 |
| 11/29/2007 | 1.75 | 7:00 | 8:45 | 1.5 | 0700 | 0830 |  |  |  |  |  |  |  | 1.75 | 7:00 | 8:45 | 1.75 | 7:00 | 8:45 |
| 11/29/2007 | 3 | 18:00 | 21:00 | 3 | 1800 | 2100 |  |  |  |  |  |  |  | 3 | 18:00 | 21:00 | 3 | 18:00 | 21:00 |
| 11/30/2007 | 2 | 7:00 | 9:00 | 2 | 0700 | 0900 |  |  |  |  |  |  |  | 2 | 7:00 | 9:00 | 2 | 7:00 | 9:00 |
| 11/30/2007 | 3 | 18:00 | 21:00 | 3 | 1800 | 2100 |  |  |  |  |  |  |  | 3 | 18:00 | 21:00 | 3 | 18:00 | 21:00 |
| 12/4/2007 | 3 | 18:00 | 21:00 | 3 | 1800 | 2100 |  |  |  |  |  |  |  | 3 | 18:00 | 21:00 | 3 | 18:00 | 21:00 |
| 12/5/2007 | 2 | 7:00 | 9:00 | 2 | 0700 | 0900 |  |  |  |  |  |  |  | 2 | 7:00 | 9:00 | 2 | 7:00 | 9:00 |
| 12/5/2007 | 3 | 17:00 | 20:00 | 3 | 1700 | 2000 |  |  |  |  |  |  |  | 3 | 17:00 | 20:00 | 3 | 17:00 | 20:00 |
| 12/6/2007 | 2 | 7:00 | 9:00 | 2 | 0700 | 900 |  |  |  |  |  |  |  | 2 | 7:00 | 9:00 | 2 | 7:00 | 9:00 |
| 12/6/2007 | 3 | 18:00 | 21:00 | 3 | 1800 | 2100 |  |  |  |  |  |  |  | 3 | 18:00 | 21:00 | 3 | 18:00 | 21:00 |
| 12/7/2007 | 2 | 7:00 | 9:00 | 2 | 0700 | 0900 |  |  |  |  | 5 | 1700 | 2200 | 5 | 1700 | 2200 | 5 | 1700 | 2200 |
| 12/7/2007 | 3 | 18:00 | 21:00 | 3 | 1800 | 2100 |  |  |  |  |  |  |  | 3 | 18:00 | 21:00 | 3 | 18:00 | 21:00 |
| 12/17/2007 | 1.25 | 7:00 | 8:15 | 1.5 | 0700 | 0830 |  |  |  |  |  |  |  | 1.25 | 7:00 | 8:15 | 1.25 | 7:00 | 8:15 |
| 12/17/2007 | 3 | 18:00 | 21:00 | 3 | 1800 | 2100 |  |  |  |  |  |  |  | 3 | 18:00 | 21:00 | 3 | 18:00 | 21:00 |
| 12/18/2007 | 2 | 7:00 | 9:00 | 2 | 0700 | 0900 |  |  |  |  |  |  |  | 2 | 7:00 | 9:00 | 2 | 7:00 | 9:00 |
| 12/19/2007 |  |  |  |  |  |  |  |  |  |  | 12 | 1000 | 2200 | 12 | 1000 | 2200 |  |  |  |
| 12/20/2007 |  |  |  |  |  |  |  |  |  |  | 16 | 600 | 2200 | 16 | 600 | 2200 |  |  |  |
| 12/26/2007 |  |  |  |  |  |  |  |  |  |  | 3 | 1700 | 2000 | 3 | 1700 | 2000 |  |  |  |
| Total Hours | 314.0 |  |  | 281.0 |  |  | 186.6 | 141.0 |  |  | 234.0 |  |  | 513.5 |  |  | 381.0 |  |  |
| Total Days | 93 |  |  | 94 |  |  | 57 | 21 |  |  | 28 |  |  | 111 |  |  | 92 |  |  |

| Table 5: Line items in the DG inventory |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Year | Plant Name | Reg # | FIPS | Plant ID | Stack # | Point # | Seg # | Segment Description | SCC | NO2 TPY | Submitted with initial 2007 inventory? |
| 2007 | US Army - Fort Myer | 71714 | 013 | 00009 | 13 | 13 | 1 | COMBINED DIESEL GENERATOR | 20300101 | 2.12 | Y |
| 2007 | Pentagon Reservation | 70030 | 013 | 00010 | 20 | 20 | 1 | RDF 1-9 Cat. 3516B Diesel Gensets | 20200102 | 4.25 | Y |
| 2007 | Pentagon Reservation | 70030 | 013 | 00010 | 21 | 21 | 1 | Cummins QSK7B Diesel Genset | 20200102 | 0.17 | Y |
| 2007 | Fort AP Hill | 40306 | 033 | 00017 | 4 | 4 | 1 | 28 DIESEL GENERATORS | 39999992 | 1.16 | Y |
| 2007 | Merillat - Culpeper | 40728 | 047 | 00032 | 1 | 1 | 1 | Caterpillar G3412 diesel generator | 20200401 | 3.99 | Y |
| 2007 | The Rochester Corporation | 40369 | 047 | 00044 | 4 | 1 | 1 | Caterpillar gen 725kW - diesel | 20200102 | 29.38 | Y |
| 2007 | Town of Culpeper / Electric Power Plant | 40927 | 047 | 00047 | 4 | 4 | 1 | 1275 hp-engine #7(FUEL OIL) | 20100107 | 0.09 | Y |
| 2007 | Wythe Park Power, LLC - Petersburg | 51007 | 053 | 00077 | 1 | 1 | 1 | #1 Engine, liquid fuel thruput | 20200102 | 7.72 | Y |
| 2007 | Wythe Park Power, LLC - Petersburg | 51007 | 053 | 00077 | 1 | 2 | 1 | #2 Engine, liquid fuel thruput | 20200102 | 7.37 | Y |
| 2007 | Wythe Park Power, LLC - Petersburg | 51007 | 053 | 00077 | 1 | 3 | 1 | #3 Engine, liquid fuel thruput | 20200102 | 4.89 | Y |
| 2007 | US Army - Fort Belvoir | 70550 | 059 | 00018 | 19 | 25 | 1 | DCEETA*10 CAT. 3516 GENSET | 39999992 | 2.41 | Y |
| 2007 | US Army - Fort Belvoir | 70550 | 059 | 00018 | 19 | 25 | 2 | DCEETA*4 CAT.3516B GENSET | 39999992 | 0.65 | Y |
| 2007 | US Army - Fort Belvoir | 70550 | 059 | 00018 | 26 | 26 | 3 | B2444*Three Diesel Generators | 20200401 | 5.49 | Y |
| 2007 | US Army - Fort Belvoir | 70550 | 059 | 00018 | 26 | 26 | 5 | B2462*Four CAT.SR-4 Genset | 39999992 | 4.98 | Y |
| 2007 | US Army - Fort Belvoir | 70550 | 059 | 00018 | 26 | 26 | 2 | B2310*Three Diesel Generators | 39999992 | 2.33 | Y |
| 2007 | US Army - Fort Belvoir | 70550 | 059 | 00018 | 26 | 26 | 1 | B193*Cummins VT Diesel Gen. | 20200401 | 0.17 | Y |
| 2007 | US Army - Fort Belvoir | 70550 | 059 | 00018 | 27 | 27 | 2 | AMC*Gen.#2, Generac SD0750 | 39999992 | 0.22 | Y |
| 2007 | US Army - Fort Belvoir | 70550 | 059 | 00018 | 27 | 27 | 1 | AMC*Gen.#1, Generac SD0750 | 39999992 | 0.19 | Y |
| 2007 | US Army - Fort Belvoir | 70550 | 059 | 00018 | 27 | 27 | 3 | AMC*Gen.#3, Generac SD0600 | 39999992 | 0.12 | Y |
| 2007 | US Army - Fort Belvoir | 70550 | 059 | 00018 | 28 | 28 | 1 | DTRA*Caterpillar SR-4 Genset | 39999992 | 1.25 | Y |
| 2007 | US Army - Fort Belvoir | 70550 | 059 | 00018 | 29 | 29 | 1 | INSCOM*Two Cummins 750 Gen. | 39999992 | 0.97 | Y |
| 2007 | US Army - Fort Belvoir | 70550 | 059 | 00018 | 30 | 30 | 1 | PPS*12 Gen. Cummins, Cat., EMD | 20200401 | 4.41 | Y |
| 2007 | US Army - Fort Belvoir | 70550 | 059 | 00018 | 45 | 45 | 1 | Diesel Generators | 20200401 | 0.01 | Y |
| 2007 | US Army - Fort Belvoir | 70550 | 059 | 00018 | 46 | 46 | 1 | Diesel Generator | 20200401 | 0.17 | Y |
| 2007 | US Army - Fort Belvoir | 70550 | 059 | 00018 | 47 | 47 | 1 | Diesel Generator | 20200401 | 0.54 | Y |
| 2007 | US Army - Fort Belvoir | 70550 | 059 | 00018 | 48 | 48 | 1 | BLDG 2953 Generator (800 kW) | 20200401 | 0.11 | Y |
| 2007 | Inova Fairfax Hospital | 70021 | 059 | 00022 | 4 | 4 | 1 | Generators-Distillate Fuel | 20200401 | 0.45 | Y |
| 2007 | Inova Fairfax Hospital | 70021 | 059 | 00022 | 20 | 20 | 1 | EG7&8*Caterpillar 3516B Genset | 20300101 | 0.59 | Y |
| 2007 | Washington Gas Light Company | 70151 | 059 | 00056 | 1 | 1 | 1 | 1*CATERPILLER G399/GAS | 20200202 | 15.27 | Y |
| 2007 | Washington Gas Light Company | 70151 | 059 | 00056 | 1 | 2 | 1 | 2*CATERPILLER G399/GAS | 20200202 | 10.18 | Y |
| 2007 | Washington Gas Light Company | 70151 | 059 | 00056 | 2 | 4 | 1 | 3*CATERPILLER G399/GAS | 20200202 | 10.25 | Y |
| 2007 | Washington Gas Light Company | 70151 | 059 | 00056 | 2 | 5 | 1 | 4*CATERPILLER G399/GAS | 20200202 | 9.57 | Y |
| 2007 | Washington Gas Light Company | 70151 | 059 | 00056 | 3 | 8 | 1 | 6*CATERPILLER G399/GAS | 20200202 | 16.25 | Y |
| 2007 | Washington Gas Light Company | 70151 | 059 | 00056 | 3 | 7 | 1 | 5*CATERPILLER G399/GAS | 20200202 | 9.52 | Y |
| 2007 | Washington Gas Light Company | 70151 | 059 | 00056 | 6 | 12 | 1 | 12*CUMMINS GENER./#2 OIL | 20300101 | 0.02 | Y |
| 2007 | Washington Gas Light Company | 70151 | 059 | 00056 | 8 | 14 | 1 | Volvo 500 RD Diesel Generator | 20200401 | 0.27 | Y |
| 2007 | George Mason University | 70691 | 059 | 00277 | 10 | 10 | 1 | 24 SMALL EMER. GEN. (diesel) <447kW kw | 20300101 | 1.78 | Y |
| 2007 | George Mason University | 70691 | 059 | 00277 | 11 | 11 | 1 | 2 LARGE EMER. GEN. (diesel) >447 kW | 20300101 | 0.67 | Y |
| 2007 | Science Applications International Corp. | 71804 | 059 | 00402 | 1 | 1 | 1 | 1-4 CUMMINS GEN KTA2300GS | 20200401 | 0.44 | Y |
| 2007 | Science Applications International Corp. | 71804 | 059 | 00402 | 2 | 2 | 1 | 6-8 WAKESHA GEN VHP667ODS | 20200401 | 0.67 | Y |
| 2007 | ExxonMobil Global Services Company | 71847 | 059 | 00413 | 2 | 2 | 1 | Generators G1-G4 750 | 20200102 | 4.93 | Y |
| 2007 | ExxonMobil Global Services Company | 71847 | 059 | 00413 | 3 | 3 | 1 | Generators GC1 - GC2 1100 | 20200102 | 2.91 | Y |
| 2007 | Bush Center for Intelligence (CIA) | 71757 | 059 | 00421 | 7 | 7 | 1 | 7 ALLISON TVRBINES/#2 OIL | 20200101 | 18.93 | Y |
| 2007 | Bush Center for Intelligence (CIA) | 71757 | 059 | 00421 | 8 | 8 | 1 | EMERG.DIESEL ENG.GENERATR | 20200102 | 0.02 | Y |
| 2007 | National Reconnaissance Office | 71988 | 059 | 00585 | 1 | 1 | 1 | GS1-5*CATERPILLAR 3516 | 20200401 | 4.73 | Y |
| 2007 | CRP 12100 SVD | 72296 | 059 | 00733 | 1 | 1 | 1 | #1-4 Caterpillar 3516B Generators rated at 2200 kW | 20200102 | 3.52 | Y |
| 2007 | Dulles Discovery - Phase 1 | 73340 | 059 | 73340 | 1 | 1 | 1 | Engine Generator A | 39999992 | 3.14 | Y |
| 2007 | Dulles Discovery - Phase 1 | 73340 | 059 | 73340 | 2 | 2 | 1 | Engine Generator B | 39999992 | 2.92 | Y |
| 2007 | Dulles Discovery - Phase 1 | 73340 | 059 | 73340 | 3 | 3 | 1 | Engine Generator C | 39999992 | 2.81 | Y |
| 2007 | Dulles Discovery - Phase 1 | 73340 | 059 | 73340 | 4 | 4 | 1 | Engine Generator D | 39999992 | 2.43 | Y |
| 2007 | Naval Support Facility | 40307 | 099 | 00001 | 1 | 1 | 1 | 3- 2500 KW Diesel Generators | 20200401 | 14.94 | Y |
| 2007 | Naval Support Facility | 40307 | 099 | 00001 | 9 | 5 | 1 | (32) DIESEL GEN UP TO 448 KW | 20300101 | 1.91 | Y |
| 2007 | Naval Support Facility | 40307 | 099 | 00001 | 10 | 10 | 1 | (3) DIESEL GEN > 448 KW | 20200401 | 4.35 | Y |
| 2007 | Naval Support Facility | 40307 | 099 | 00001 | 11 | 11 | 2 | 3B-NO 2 OIL GENERATORS (6) | 20200401 | 5.42 | Y |
| 2007 | America Online Incorporated | 72346 | 107 | 00134 | 1 | 1 | 1 | DIESL CAT#3516 EMRG GEN w/o Selective Reduction Systems | 20200102 | 0.92 | Y |
| 2007 | America Online Incorporated | 72346 | 107 | 00134 | 4 | 4 | 1 | 8 SMALL DIESL EMERG GEN | 20200102 | 1.77 | Y |
| 2007 | America Online Incorporated | 72346 | 107 | 00134 | 5 | 5 | 1 | 7 DIESL CAT#3516 EMRG GEN w/ Selective Reduction Systems | 20200102 | 0.61 | Y |
| 2007 | MCI-Verizon | 72367 | 107 | 00141 | 1 | 1 | 1 | 1-8 CAT GENERATORS/#2 OIL | 20300101 | 5.51 | Y |
| 2007 | MCI-Verizon | 72367 | 107 | 00141 | 4 | 4 | 1 | 1-8 Onan generators/#2oil | 20300101 | 13.85 | Y |
| 2007 | Conectiv Delmarva Generation Inc - Bayview | 40602 | 131 | 00008 | 1 | 1 | 1 | 1 E M D 16-645-ED4 | 20100102 | 23.39 | Y |
| 2007 | Conectiv Delmarva Generation Inc - Bayview | 40602 | 131 | 00008 | 2 | 2 | 1 | 2 E M D 16-645-ED4 | 20100102 | 20.68 | Y |
| 2007 | Conectiv Delmarva Generation Inc - Bayview | 40602 | 131 | 00008 | 3 | 3 | 1 | 3 E M D 16-645-ED4 | 20100102 | 22.51 | Y |
| 2007 | Conectiv Delmarva Generation Inc - Bayview | 40602 | 131 | 00008 | 4 | 4 | 1 | 4 E M D 16-645-ED4 | 20100102 | 16.28 | Y |
| 2007 | Conectiv Delmarva Generation Inc - Bayview | 40602 | 131 | 00008 | 5 | 5 | 1 | 5 E M D 16-645-ED4 | 20100102 | 21.28 | Y |
| 2007 | Conectiv Delmarva Generation Inc - Bayview | 40602 | 131 | 00008 | 6 | 6 | 1 | 6 E M D 16-645-ED4 | 20100102 | 21.73 | Y |
| 2007 | US Marine Corps - Quantico | 70267 | 153 | 00010 | 23 | 23 | 1 | (Bldg 3300) Caterpillar 3512 Diesel-fired Emergency Gen | 39999992 | 1.69 | Y |
| 2007 | US Marine Corps - Quantico | 70267 | 153 | 00010 | 34 | 58 | 1 | Fueling Station CNG Compressor Engine - NG fired | 40600401 | 0.08 | Y |
| 2007 | US Marine Corps - Quantico | 70267 | 153 | 00010 | 35 | 59 | 1 | Hochmuth Hall Building 2033 - Emergency Generator 3 | 39999992 | 0.63 | Y |
| 2007 | US Marine Corps - Quantico | 70267 | 153 | 00010 | 36 | 60 | 1 | Diamond Hall Blg 3098 Emergency Generator 750 hp | 39999992 | 0.10 | Y |
| 2007 | US Marine Corps - Quantico | 70267 | 153 | 00010 | 37 | 61 | 1 | Newlin Hall Building 3255 Emergency Generator #1 | 39999992 | 0.14 | Y |
| 2007 | US Marine Corps - Quantico | 70267 | 153 | 00010 | 38 | 62 | 1 | Newlin Hall Building 3255 Emergency Generator #2 | 39999992 | 0.11 | Y |
| 2007 | US Marine Corps - Quantico | 70267 | 153 | 00010 | 39 | 63 | 1 | Newlin Hall Building 3255 Emergency Generator #3 | 39999992 | 1.07 | Y |
| 2007 | US Marine Corps - Quantico | 70267 | 153 | 00010 | 40 | 64 | 1 | NOC - 1 Network Operations Center Emergency Generator | 39999992 | 1.10 | Y |
| 2007 | US Marine Corps - Quantico | 70267 | 153 | 00010 | 41 | 65 | 1 | Marsh Center Building 3280 Emergency Generator | 39999992 | 0.70 | Y |
| 2007 | US Marine Corps - Quantico | 70267 | 153 | 00010 | 42 | 66 | 1 | Water Plant Building 1303 Emergency Generator | 39999992 | 1.74 | Y |
| 2007 | US Marine Corps - Quantico | 70267 | 153 | 00010 | 43 | 67 | 1 | Sewage Plant Building 660 Emergency Generator | 39999992 | 0.84 | Y |
| 2007 | US Marine Corps - Quantico | 70267 | 153 | 00010 | 44 | 68 | 1 | EG #2 Sewage Plant Building 660 | 39999992 | 0.53 | Y |
| 2007 | US Marine Corps - Quantico | 70267 | 153 | 00010 | 46 | 70 | 1 | Whiteside Hanger Emergency Generator | 39999992 | 0.45 | Y |
| 2007 | US Marine Corps - Quantico | 70267 | 153 | 00010 | 47 | 71 | 1 | Greenside Hanger Emergency Generator | 39999992 | 0.34 | Y |
| 2007 | US Marine Corps - Quantico | 70267 | 153 | 00010 | 48 | 72 | 1 | Emer. Gen. - Harry Lee Hall Bldg. #17 | 39999992 | 0.03 | Y |
| 2007 | Lockheed Martin Manassas | 70261 | 153 | 00021 | 11 | 11 | 1 | (EGEN 001) Cummins NTA400 Emergency Generator, 230 kW (400 hp engine) | 20200102 | 0.05 | Y |
| 2007 | Lockheed Martin Manassas | 70261 | 153 | 00021 | 12 | 12 | 1 | (EGEN 101) Katolight Emergency Generator, gas-fired, 60 kW (80.5 hp engine) | 20200202 | 0.01 | Y |
| 2007 | Lockheed Martin Manassas | 70261 | 153 | 00021 | 13 | 13 | 1 | (EGEN 105) Cummins NTA355 no.2 oil-fired, emergency generator, 200 kW (355 hp engine) | 20200102 | 0.04 | Y |
| 2007 | Lockheed Martin Manassas | 70261 | 153 | 00021 | 14 | 14 | 1 | (EGEN 106) Cummins VT1710, no. 2 oil-fired emergency generator, 450 kW (685 hp engine) | 20200401 | 0.05 | Y |
| 2007 | Lockheed Martin Manassas | 70261 | 153 | 00021 | 15 | 15 | 1 | (EGEN 110-1) Cummins NTA355 no.2 oil-fired, emergency generator, 200 kW (355 hp | 20200102 | 0.04 | Y |
| 2007 | Lockheed Martin Manassas | 70261 | 153 | 00021 | 16 | 16 | 1 | (EGEN 110-2) Caterpillar 3412, no. 2 oil-fired emergency generator, 500 kW (747 hp engine) | 20200401 | 0.06 | Y |
| 2007 | Lockheed Martin Manassas | 70261 | 153 | 00021 | 17 | 17 | 1 | (EGEN 120) Caterpillar 3306PC, no. 2 oil-fired, emergency generator 155 kW (275 hp engine) | 20200102 | 0.04 | Y |
| 2007 | Lockheed Martin Manassas | 70261 | 153 | 00021 | 18 | 18 | 1 | (EGEN 200) Caterpillar 3412, no.2 oil-fired, emergency generator 500 kW (747 hp engine) | 20200401 | 0.06 | Y |
| 2007 | Lockheed Martin Manassas | 70261 | 153 | 00021 | 20 | 20 | 1 | (EGEN400) Cummins HS240, no.2 oil-fired, emergency generator, 125 kW (240 hp engine) | 20200401 | 0.02 | Y |
| 2007 | America Online Inc | 72368 | 153 | 00143 | 1 | 1 | 1 | 4 CAT Generators without SCR systems & 3 Rotary power systems | 20200102 | 1.79 | Y |
| 2007 | America Online Inc | 72368 | 153 | 00143 | 3 | 3 | 2 | 6 Cat 3516B generators with Selective Catalytic Reduction Systems - Uncontrolled | 20200102 | 4.77 | Y |
| 2007 | America Online Inc | 72368 | 153 | 00143 | 3 | 3 | 1 | 6 Cat 3516B generators with Selective Catalytic Reduction Systems - Controlled | 20200102 | 1.43 | Y |
| 2007 | Dupont Fabros Technology - Porpoise Ventures LLC | 73180 | 153 | 00812 | 1 | 1 | 2 | Generators SCR (Ref #1, 2, 5, 6, 9, 10) When NOx reduction is less than 90% | 39999992 | 9.52 | Y |
| 2007 | Dupont Fabros Technology - Porpoise Ventures LLC | 73180 | 153 | 00812 | 1 | 1 | 5 | Non-SCR Units (Ref# EG3, EG4, EG7, EG8, RPU 1, RPU 2, RPU3) 51-75 % Load | 39999992 | 3.26 | Y |
| 2007 | Dupont Fabros Technology - Porpoise Ventures LLC | 73180 | 153 | 00812 | 1 | 1 | 1 | Generators SCR (Ref # 1, 2, 5, 6, 9, 10) When NOx is 90% reduced | 39999992 | 2.00 | Y |
| 2007 | Dupont Fabros Technology - Porpoise Ventures LLC | 73180 | 153 | 00812 | 1 | 1 | 4 | Non-SCR Units (Ref# EG3, EG4, EG7, EG8, RPU 1, RPU 2, RPU3) 26-50 % Load | 39999992 | 1.46 | Y |
| 2007 | Dupont Fabros Technology - Porpoise Ventures LLC | 73180 | 153 | 00812 | 1 | 1 | 3 | Non-SCR Units (Ref# EG3, EG4, EG7, EG8, RPU 1, RPU 2, RPU3) | 39999992 | 0.19 | Y |
| 2007 | Dupont Fabros Technology - Porpoise Ventures LLC | 73180 | 153 | 00812 | 1 | 1 | 6 | Non-SCR Units (Ref# EG3, EG4, EG7, EG8, RPU 1, RPU 2, RPU3) 76-100% Load | 39999992 | 0.12 | Y |
| 2007 | FBI Academy | 40368 | 179 | 00020 | 7 | 7 | 1 | (6) 1500 kW Emer. Gen-N6G1 thru N6G6 | 20200401 | 1.34 | Y |
| 2007 | FBI Academy | 40368 | 179 | 00020 | 8 | 8 | 1 | (2) 1500 kW Emer gen-19G1 & 19G2 | 20200401 | 1.23 | Y |
| 2007 | FBI Academy | 40368 | 179 | 00020 | 9 | 9 | 1 | (1) 800 kW Emer Gen-9-C-G1 | 20200401 | 0.29 | Y |
| 2007 | FBI Academy | 40368 | 179 | 00020 | 10 | 10 | 1 | (1) 300 kW Emer Gen-N7-G1 | 20300101 | 0.03 | Y |
| 2007 | FBI Academy | 40368 | 179 | 00020 | 11 | 11 | 1 | (1) 425 kW Emer Gen-16G1 | 20200401 | 0.08 | Y |
| 2007 | FBI Academy | 40368 | 179 | 00020 | 12 | 12 | 1 | (7) Gen, diesel-ELSG1&2, 10G1, 12G1, 6G1, HRLSG1, 17G1 | 20300101 | 0.82 | Y |
| 2007 | City of Manassas/VMEA | 71977 | 683 | 00090 | 1 | 1 | 1 | V3-10 CAT 3516TA-ENGINES | 39999991 | 49.59 | Y |
| 2007 | City of Manassas/VMEA | 71977 | 683 | 00090 | 2 | 2 | 1 | V1-2, C7-10 Cat 3516TA Engines | 39999991 | 21.35 | Y |
| 2007 | City of Manassas/VMEA | 71977 | 683 | 00090 | 3 | 3 | 1 | V11-12 CAT3516TA-ENG | 39999991 | 7.01 | Y |
| 2007 | Manassas City - Church Street Power Generation Fac | 72004 | 683 | 00094 | 1 | 1 | 1 | 1#DETROIT DIESEL9163-7305 | 39999991 | 2.54 | Y |
| 2007 | Manassas City - Church Street Power Generation Fac | 72004 | 683 | 00094 | 2 | 2 | 1 | 2*DETROIT DIESEL9163-7305 | 39999991 | 2.26 | Y |
| 2007 | Manassas City - Church Street Power Generation Fac | 72004 | 683 | 00094 | 3 | 3 | 1 | 3*CATERPILLAR 3512 DITA 6 | 39999991 | 2.41 | Y |
| 2007 | Manassas City - Church Street Power Generation Fac | 72004 | 683 | 00094 | 4 | 4 | 1 | 4*DETROIT DIESEL9163-7305 | 39999991 | 2.11 | Y |
| 2007 | Manassas City - Church Street Power Generation Fac | 72004 | 683 | 00094 | 5 | 5 | 1 | 5*CATERPILLAR 3516 DITA | 39999991 | 4.07 | Y |
| 2007 | Manassas City - Church Street Power Generation Fac | 72004 | 683 | 00094 | 6 | 6 | 1 | 6*CATERPILLAR 3516 DITA | 39999991 | 4.06 | Y |
| 2007 | Micron Technology Incorporated | 72299 | 683 | 00131 | 4 | 4 | 1 | EG -1 Emergency generator - CAT 3508 600 kW | 39999992 | 0.07 | Y |
| 2007 | Micron Technology Incorporated | 72299 | 683 | 00131 | 5 | 5 | 1 | EG2 Emergency Generator - CAT 3508 720 kW | 39999992 | 0.24 | Y |
| 2007 | Micron Technology Incorporated | 72299 | 683 | 00131 | 6 | 6 | 1 | EG-3 Emergency Generator CAT 3412 470kW | 39999992 | 0.03 | Y |
| 2007 | Micron Technology Incorporated | 72299 | 683 | 00131 | 7 | 7 | 1 | EG-4 Emergency Generator CAT 3406DI 250 kW 519 bhp | 39999992 | 0.03 | Y |
| 2007 | Micron Technology Incorporated | 72299 | 683 | 00131 | 8 | 8 | 1 | EG5, EG6, EG7 Emergency Generators CAT 3512, 1250 kW, 1786 bhp | 39999992 | 0.41 | Y |
| 2007 | Micron Technology Incorporated | 72299 | 683 | 00131 | 9 | 9 | 1 | EG9 Emergency Generators CAT 3512, 1250 kW, 1786 bhp | 39999992 | 0.10 | Y |
| 2007 | Micron Technology Incorporated | 72299 | 683 | 00131 | 10 | 10 | 1 | EG8, CAT 3516, 1040kW, Nat. Gas fired | 20100202 | 0.05 | Y |
| 2007 | Manassas City of - Central Park Plant | 72327 | 683 | 00134 | 1 | 1 | 1 | 4 Caterpillar Gensets - 3516B | 39999991 | 7.32 | Y |
| 2007 | Manassas City of - Central Park Plant | 72327 | 683 | 00134 | 2 | 2 | 1 | 1 R.R. AVON 1533 TURBINE | 39999991 | 0.62 | Y |
| 2007 | Wythe Park Power, LLC Richmond Plant | 51055 | 760 | 00405 | 1 | 1 | 2 | Dual Fuel emissions for all three (3) KTA-50 engines | 20100201 | 5.00 | Y |
| 2007 | Wythe Park Power, LLC Richmond Plant | 51055 | 760 | 00405 | 1 | 1 | 1 | Single Fuel emissions for all three (3) KTA-50 engines | 20100201 | 0.04 | Y |
| 2007 | A and N Electric Cooperative - Tangier Island | 40564 | 001 | 00012 | 1 | 1 | 1 | 1-4 Generators/#2 oil | 20100102 | 1.63 | N |
| 2007 | Old Dominion Electric Cooperative - Unit 7 | 61410 | 001 | 61410 | 1 | 1 | 1 | Diesel Generators (UNIT 7) | 20200401 | 1.63 | N |
| 2007 | Old Dominion Electric Cooperative - Unit 8 | 61411 | 001 | 61411 | 1 | 1 | 1 | Diesel Generator (UNIT 8) | 20200401 | 1.33 | N |
| 2007 | Old Dominion Electric Cooperative - UNIT 5 | 61412 | 001 | 61412 | 1 | 1 | 1 | Diesel Generator (UNIT 5) | 20200401 | 1.61 | N |
| 2007 | Old Dominion Electric Cooperative - Unit 6 | 61413 | 001 | 61413 | 1 | 1 | 1 | Diesel Generator (UNIT 6) | 20200401 | 1.50 | N |
| 2007 | Old Dominion Electric Cooperative - UNIT 9 | 61414 | 001 | 61414 | 1 | 1 | 1 | Diesel Generator (Unit 9) | 20200401 | 1.47 | N |
| 2007 | Old Dominion Electric Cooperative - UNIT 10 | 61415 | 001 | 61415 | 1 | 1 | 1 | Diesel Generator (Unit 10) | 20200401 | 1.72 | N |
| 2007 | Pointon Substation-Amelia County-Unit 4 | 51934 | 007 | 51934 | 1 | 1 | 1 | Caterpillar diesel engine | 39999992 | 1.08 | N |
| 2007 | MWAA - Ronald Reagan National Airport | 70005 | 013 | 00015 | 15 | 15 | 1 | EM.GEN.&PUMP | 20300101 | 1.60 | N |
| 2007 | Virginia Hospital Center - Arlington | 70226 | 013 | 00163 | 2 | 2 | 1 | G1-8*DIESEL GENERATORS | 20200102 | 1.51 | N |
| 2007 | Verizon Virginia Incorporated - North Irving St | 73302 | 013 | 00812 | 1 | 2 | 2 | Spectrum Gen Set - 1600¦DS60 | 39999992 | 0.26 | N |
| 2007 | Verizon Virginia Incorporated - North Irving St | 73302 | 013 | 00812 | 1 | 1 | 1 | Spectrum Gen Set - 1600¦DS60 | 39999992 | 0.27 | N |
| 2007 | Dominion - Bath County Power Station | 80834 | 017 | 00003 | 1 | 1 | 1 | 1 EMERG DZL GEN 9163-7305 | 20100102 | 0.22 | N |
| 2007 | Dominion - Bath County Power Station | 80834 | 017 | 00003 | 2 | 2 | 1 | 2 EMERG DZL GEN 9163-7305 | 20100102 | 0.17 | N |
| 2007 | Dominion - Bath County Power Station | 80834 | 017 | 00003 | 3 | 3 | 1 | 3 EMERG DZL GEN 1043-7305 | 20100102 | 0.03 | N |
| 2007 | Culpeper Regional Hospital | 40265 | 047 | 00017 | 1 | 1 | 1 | Caterpillar Generator Model 3412 | 20300101 | 0.66 | N |
| 2007 | Culpeper Regional Hospital | 40265 | 047 | 00017 | 1 | 1 | 2 | Kohler Generator Model 230RO7271 | 20300101 | 0.15 | N |
| 2007 | US Army-Warrenton Training Center Station D | 40901 | 047 | 00042 | 1 | 1 | 1 | (4) Diesel Emergency Generators | 20200102 | 0.60 | N |
| 2007 | Culpeper Wastewater Treatment Plant | 41019 | 047 | 00055 | 1 | 1 | 1 | 750 kW Emergency Generator | 20300101 | 0.13 | N |
| 2007 | Culpeper Wastewater Treatment Plant | 41019 | 047 | 00055 | 2 | 2 | 1 | 1825kW Emergency Generator | 20300101 | 1.17 | N |
| 2007 | Town of Culpeper Light & Power | 41024 | 047 | 00057 | 1 | 1 | 1 | #1 Cat 3516B Diesel Eng | 20200401 | 3.22 | N |
| 2007 | Town of Culpeper Light & Power | 41024 | 047 | 00057 | 2 | 2 | 1 | #2 Cat 3516B Diesel Eng | 20200401 | 3.22 | N |
| 2007 | Town of Culpeper Light & Power | 41024 | 047 | 00057 | 3 | 3 | 1 | #3 Cat 3516B Diesel Eng | 20200401 | 2.77 | N |
| 2007 | National Audio Visual Conservation Center | 41039 | 047 | 00061 | 3 | 3 | 2 | G1 Genset Diesel Fuel | 20200401 |  | N |
| 2007 | National Audio Visual Conservation Center | 41039 | 047 | 00061 | 3 | 3 | 1 | G1 Genset Hours Operated | 39999992 | 0.28 | N |
| 2007 | National Audio Visual Conservation Center | 41039 | 047 | 00061 | 4 | 4 | 2 | G2 Genset Diesel Fuel | 20200401 |  | N |
| 2007 | National Audio Visual Conservation Center | 41039 | 047 | 00061 | 4 | 4 | 1 | G2 Genset Hours Operated | 39999992 | 0.23 | N |
| 2007 | National Audio Visual Conservation Center | 41039 | 047 | 00061 | 5 | 5 | 2 | G3 Genset Diesel Fuel | 20200401 |  | N |
| 2007 | National Audio Visual Conservation Center | 41039 | 047 | 00061 | 5 | 5 | 1 | G3 Genset Hours Operated | 39999992 | 0.24 | N |
| 2007 | National Audio Visual Conservation Center | 41039 | 047 | 00061 | 6 | 6 | 2 | G4 Genset Diesel Fuel | 20200401 |  | N |
| 2007 | National Audio Visual Conservation Center | 41039 | 047 | 00061 | 6 | 6 | 1 | G4 Genset Hours Operated | 39999992 | 0.24 | N |
| 2007 | US Geological Survey | 71723 | 059 | 00343 | 8 | 8 | 2 | G4*Caterpillar 3306 genset/oil | 39999992 | 0.08 | N |
| 2007 | US Geological Survey | 71723 | 059 | 00343 | 8 | 8 | 1 | G2, G3*Cummins AP-800 gen./oil | 39999992 | 0.35 | N |
| 2007 | US Geological Survey | 71723 | 059 | 00343 | 9 | 9 | 1 | G1*Cummins GTA28 genset/gas | 39999992 | 0.11 | N |
| 2007 | US Geological Survey | 71723 | 059 | 00343 | 9 | 9 | 2 | G5*Cummins GTA-50 genset/gas | 39999992 | 0.37 | N |
| 2007 | GANNETT OFFSET-SPRINGFIELD | 71883 | 059 | 00439 | 3 | 31 | 1 | G1 KATOLIGHT D2000F Y4 DIESEL GENSET | 20300101 | 0.30 | N |
| 2007 | Reston Hospital Center | 71918 | 059 | 00558 | 2 | 40 | 1 | Two Emerg. Generators | 20200102 | 0.38 | N |
| 2007 | CENTRAL INTELLIGENCE AGENCY | 72289 | 059 | 00729 | 1 | 1 | 1 | 1*CATERPILLAR 200KW DIEGN | 20200102 | 0.05 | N |
| 2007 | CENTRAL INTELLIGENCE AGENCY | 72291 | 059 | 00731 | 1 | 1 | 1 | 1*CATERPILLAR 900KW DIEGN | 20200102 | 0.12 | N |
| 2007 | Fannie Mae | 72292 | 059 | 00732 | 1 | 1 | 1 | 1*CATERPILLER 1500KW DGEN | 20200102 | 1.51 | N |
| 2007 | Fannie Mae | 72292 | 059 | 00732 | 2 | 2 | 1 | 2*CATERPILLAR 1500KW DGEN | 20200102 | 1.51 | N |
| 2007 | Fannie Mae | 72292 | 059 | 00732 | 3 | 3 | 1 | 3*CATERPILLAR 1500KW DGEN | 20200102 | 1.21 | N |
| 2007 | Fannie Mae | 72292 | 059 | 00732 | 4 | 4 | 1 | 4*CATERPILLAR 1500KW DGEN | 20200102 | 0.60 | N |
| 2007 | Fannie Mae | 72292 | 059 | 00732 | 5 | 5 | 1 | 5*CATERPILLAR 500KW DGEN | 20300101 | 0.04 | N |
| 2007 | Quest Diagnostics | 72342 | 059 | 00745 | 2 | 2 | 1 | 1-2*CATPLER GEN / DIESEL | 20200102 | 0.39 | N |
| 2007 | Washington International Teleport | 72349 | 059 | 00749 | 1 | 1 | 1 | A,B*ONAN450 DFECGENERATOR | 20300101 | 0.68 | N |
| 2007 | Washington International Teleport | 72349 | 059 | 00749 | 2 | 2 | 1 | 3,4*ONAN250 DFBE/CAT.SR4F | 20300101 | 0.47 | N |
| 2007 | Inova Fair Oaks Hospital | 72353 | 059 | 00753 | 2 | 2 | 1 | 1-2 Magna Diesel Generators | 20200102 | 0.30 | N |
| 2007 | Inova Fair Oaks Hospital | 72353 | 059 | 00753 | 4 | 4 | 1 | 3* Kohler Emergency Diesel Gen. | 20200102 | 0.13 | N |
| 2007 | Mainrock II Chantilly LLC | 72375 | 059 | 00761 | 1 | 1 | 1 | Generator 1 | 39999992 | 0.06 | N |
| 2007 | Mainrock II Chantilly LLC | 72375 | 059 | 00761 | 2 | 2 | 1 | Generator 2 | 39999992 | 0.06 | N |
| 2007 | Mainrock II Chantilly LLC | 72375 | 059 | 00761 | 3 | 8 | 1 | Generator 3 | 39999992 | 0.06 | N |
| 2007 | Freddi Mac Corporation | 73105 | 059 | 00763 | 1 | 1 | 1 | 4 diesl engine gen sets 2000 KW | 39999992 | 0.90 | N |
| 2007 | Lemur Properties Limited Liability Corporation | 73162 | 059 | 00817 | 1 | 01 | 1 | 1-18 Caterpillar 3516B Dsl-Gen. | 20200102 | 2.33 | N |
| 2007 | Time Warner Cable | 73170 | 059 | 00819 | 1 | 1 | 1 | 1-2*Caterpillar model 3508B gen | 39999992 | 1.21 | N |
| 2007 | Time Warner Cable | 73170 | 059 | 00819 | 2 | 2 | 1 | 1-2*Caterpillar model 3508B gen | 39999992 | 1.15 | N |
| 2007 | National Air & Space Museum | 73220 | 059 | 00829 | 4 | 4 | 1 | 900 kW Kohler Emergency Generator | 39999992 | 0.44 | N |
| 2007 | National Air & Space Museum | 73220 | 059 | 00829 | 5 | 5 | 1 | 400 kW Kohler Emergency Generator | 39999992 | 0.20 | N |
| 2007 | Gannett Satellite Information Network Incorporated | 73230 | 059 | 00832 | 1 | 1 | 1 | CAT 3508B Engine Generator #1 | 39999992 | 0.01 | N |
| 2007 | Gannett Satellite Information Network Incorporated | 73230 | 059 | 00832 | 2 | 2 | 1 | CAT 3508B Engine Generator #2 | 39999992 | 0.01 | N |
| 2007 | Gannett Satellite Information Network Incorporated | 73230 | 059 | 00832 | 3 | 3 | 1 | CAT 3508B Engine - Generator #3 | 39999992 | 0.01 | N |
| 2007 | Gannett Satellite Information Network Incorporated | 73230 | 059 | 00832 | 4 | 4 | 1 | CAT 3508B Engine - Generator #4 | 39999992 | 0.01 | N |
| 2007 | Unisys Corporation | 73279 | 059 | 00840 | 1 | 1 | 1 | Cummins generator sets (ref. 1 & 2) | 20100102 | 0.29 | N |
| 2007 | UOSA - Cub Run | 73282 | 059 | 00841 | 1 | 1 | 1 | Kohler Diesel Genset (GEN 1) | 39999992 | 0.24 | N |
| 2007 | UOSA - Cub Run | 73282 | 059 | 00841 | 2 | 2 | 1 | Kohler Diesel Genset (GEN 2) | 39999992 | 0.21 | N |
| 2007 | Nextel Communications of the Mid Atlantic, Inc. | 73287 | 059 | 00842 | 1 | 1 | 1 | Diesel engine exhaust | 39999992 | 0.28 | N |
| 2007 | Nextel Communications of the Mid Atlantic, Inc. | 73288 | 059 | 00843 | 1 | 1 | 1 | Diesel engine/generator exhaust | 39999992 | 0.22 | N |
| 2007 | Nextel Communications of the Mid Atlantic, Inc. | 73289 | 059 | 00844 | 1 | 1 | 1 | Diesel engine/generator exhaust stack | 39999992 | 0.20 | N |
| 2007 | Verizon Virginia Incorporated - Moore Road | 73303 | 059 | 00849 | 1 | 1 | 1 | Diesel Driven Engine 1700 hp | 39999992 | 0.37 | N |
| 2007 | Nextel NOC MSO Tech Lab PKT Data | 73342 | 059 | 00864 | 1 | 1 | 1 | Onan Gen - DFED/K 5005243A | 20100102 | 0.38 | N |
| 2007 | Nextel NOC MSO Tech Lab PKT Data | 73342 | 059 | 00864 | 1 | 4 | 1 | Onan Gen - DFED/K 404657SE | 20100102 | 0.38 | N |
| 2007 | Nextel NOC MSO Tech Lab PKT Data | 73342 | 059 | 00864 | 1 | 2 | 1 | Onan Gen - DFFD-1715 | 20100102 | 0.38 | N |
| 2007 | Nextel NOC MSO Tech Lab PKT Data | 73342 | 059 | 00864 | 1 | 3 | 1 | Onan Gen - DFD-3387771 | 20100102 | 0.38 | N |
| 2007 | Nextel NOC MSO Tech Lab PKT Data | 73342 | 059 | 00864 | 1 | 5 | 1 | Kohler Generator | 20100102 | 0.27 | N |
| 2007 | Clearwire | 73343 | 059 | 00865 | 1 | 1 | 1 | Caterpillar Generator | 20100102 | 0.25 | N |
| 2007 | Nextel National Headquarters | 73344 | 059 | 00866 | 1 | 1 | 1 | Caterpillar Generator | 20100102 | 0.58 | N |
| 2007 | Mitre Corporation | 73345 | 059 | 00867 | 2 | 5 | 1 | diesel fired generator (G1) | 39999992 | 0.46 | N |
| 2007 | Mitre Corporation | 73345 | 059 | 00867 | 3 | 6 | 1 | diesel fired generator (G2) | 39999992 | 0.12 | N |
| 2007 | Mitre Corporation | 73345 | 059 | 00867 | 4 | 7 | 1 | diesel fired generator (G3) | 39999992 | 0.39 | N |
| 2007 | Mitre Corporation | 73345 | 059 | 00867 | 5 | 8 | 1 | diesel fired generator (G4) | 39999992 | 0.09 | N |
| 2007 | Mitre Corporation | 73345 | 059 | 00867 | 6 | 9 | 1 | diesel fired generator (G5) | 39999992 | 0.41 | N |
| 2007 | Mitre Corporation | 73345 | 059 | 00867 | 7 | 10 | 1 | diesel fired generator (G6) | 39999992 | 0.16 | N |
| 2007 | Enersol Technologies Incorporated | 72359 | 059 | 72359 | 2 | 21 | 3 | DIESEL GEN. KILOWATT-HRS | 39999991 | 0.25 | N |
| 2007 | Enersol Technologies Incorporated | 72359 | 059 | 72359 | 2 | 21 | 2 | DIESEL GENERATORS- HOURS | 39999992 | 0.51 | N |
| 2007 | Sprint United Management Company | 73515 | 059 | 73515 | 1 | 1 | 1 | Cat 3512 diesel generator | 20100102 | 0.63 | N |
| 2007 | Sprint United Management Company | 73515 | 059 | 73515 | 2 | 2 | 1 | Cat 3512 diesel generator | 20100102 | 0.63 | N |
| 2007 | Sprint United Management Company | 73515 | 059 | 73515 | 3 | 3 | 1 | Cat 3512 diesel generator | 20100102 | 0.63 | N |
| 2007 | Sprint United Management Company | 73515 | 059 | 73515 | 4 | 4 | 1 | Cat 3512 diesel generator | 20100102 | 0.63 | N |
| 2007 | Sprint United Management Company | 73515 | 059 | 73515 | 5 | 5 | 1 | Cat 3512 diesel generator | 20100102 | 0.14 | N |
| 2007 | Sprint United Management Company | 73515 | 059 | 73515 | 6 | 6 | 1 | Cat 3512 diesel generator | 20100102 | 0.29 | N |
| 2007 | Sprint United Management Company | 73515 | 059 | 73515 | 7 | 7 | 1 | Cat 3516 B diesel generator | 20100102 | 0.56 | N |
| 2007 | Sprint United Management Company | 73515 | 059 | 73515 | 8 | 8 | 1 | Cummins DQKAB - 2919 BHP | 20100102 | 0.05 | N |
| 2007 | U. S. Army-Warrenton Training Station A | 40384 | 061 | 00022 | 2 | 2 | 2 | (3) Diesel Emergency Generators | 20200102 | 2.02 | N |
| 2007 | Warrenton Training Center Station C | 40900 | 061 | 00055 | 1 | 1 | 1 | (2) Diesel Emergency Generators | 20200102 | 0.10 | N |
| 2007 | Warrenton Training Station B | 40902 | 061 | 00056 | 1 | 1 | 1 | (19) diesel generators <400 kW ea | 20200102 | 0.33 | N |
| 2007 | Warrenton Training Station B | 40902 | 061 | 00056 | 3 | 3 | 1 | (10) Cat 3516B Diesel Generators | 20200401 | 4.97 | N |
| 2007 | Potomac Consolidated Tracon (FAA) | 41047 | 061 | 00068 | 1 | 1 | 1 | Engine Generator #1 | 39999992 | 0.24 | N |
| 2007 | Potomac Consolidated Tracon (FAA) | 41047 | 061 | 00068 | 2 | 2 | 1 | Engine Generator #2 | 39999992 | 0.31 | N |
| 2007 | Potomac Consolidated Tracon (FAA) | 41047 | 061 | 00068 | 3 | 3 | 1 | Engine Generator #3 | 39999992 | 0.37 | N |
| 2007 | Potomac Consolidated Tracon (FAA) | 41047 | 061 | 00068 | 4 | 4 | 1 | Engine Generator #4 | 39999992 | 0.32 | N |
| 2007 | Air BP | 71734 | 107 | 00064 | 7 | 7 | 1 | FF-EG1 Caterpillar diesel generator | 39999992 | 0.27 | N |
| 2007 | MWAA - Washington Dulles International Airport | 70003 | 107 | 00074 | 16 | 16 | 1 | ALL OTHER DIESEL GENRATRS | 20200102 | 0.02 | N |
| 2007 | Dominion - Loudoun Compressor Station | 71809 | 107 | 00075 | 1 | 2 | 2 | Caterpillar Model G3616 | 20200202 | 0.16 | N |
| 2007 | Dominion - Loudoun Compressor Station | 71809 | 107 | 00075 | 1 | 1 | 1 | Caterpillar Model G3616 Compressor Engines | 20200202 | 0.14 | N |
| 2007 | Dominion - Loudoun Compressor Station | 71809 | 107 | 00075 | 2 | 3 | 1 | Internal combustion compressor engine | 20200202 | 0.61 | N |
| 2007 | Town of Leesburg/Water Poll Control Div | 72260 | 107 | 00124 | 2 | 2 | 1 | CATERPILLAR 3512 GENSET (G1) | 20200102 | 0.30 | N |
| 2007 | Town of Leesburg/Water Poll Control Div | 72260 | 107 | 00124 | 3 | 3 | 1 | CUMMINS 1500DFMB GENSET (G2) | 20200102 | 0.54 | N |
| 2007 | Iridium Satellite, L.L.C | 72302 | 107 | 00128 | 1 | 1 | 1 | Emerg. Generator, Cat. 3516TA | 20300101 | 0.51 | N |
| 2007 | Inova Loudoun Hospital | 72307 | 107 | 00130 | 3 | 21 | 1 | E-1,2,3*CATERPILLAR 3412 | 20200102 | 0.11 | N |
| 2007 | TOWN OF LEESBURG-WATER SUPPLY DIVISION | 72328 | 107 | 00133 | 1 | 1 | 1 | 1 Caterpillar 1000 KW GEN.3512 | 20300101 | 0.30 | N |
| 2007 | TOWN OF LEESBURG-WATER SUPPLY DIVISION | 72328 | 107 | 00133 | 3 | 3 | 1 | 1 Caterpillar 1000 KW GEN.3512 | 20300101 | 0.18 | N |
| 2007 | NTTA Communications | 72376 | 107 | 00144 | 1 | 1 | 1 | Diesel engine exhaust EGU 1 | 39999992 | 0.67 | N |
| 2007 | NTTA Communications | 72376 | 107 | 00144 | 2 | 2 | 1 | Diesel engine exhaust (EGU2) | 39999992 | 0.15 | N |
| 2007 | NTTA Communications | 72376 | 107 | 00144 | 3 | 3 | 1 | Diesel engine exhaust (EGU3) | 39999992 | 0.69 | N |
| 2007 | NTTA Communications | 72376 | 107 | 00144 | 4 | 4 | 1 | Diesel engine exhaust (EGU 5) | 39999992 | 0.34 | N |
| 2007 | NTTA Communications | 72376 | 107 | 00144 | 5 | 5 | 1 | Diesel engine exhaust (EGU 6) | 39999992 | 0.71 | N |
| 2007 | Qwest Communications Corporation - Sterling | 73158 | 107 | 00814 | 1 | 1 | 1 | 1-3*Caterpillar 3516B Genset | 20200102 | 0.57 | N |
| 2007 | Qwest Communications Corporation - Sterling | 73158 | 107 | 00814 | 2 | 2 | 1 | 1-10*Cummins Onan Genset | 20200102 | 1.24 | N |
| 2007 | Equinix Incorporated | 73199 | 107 | 00820 | 1 | 1 | 1 | DC4-1,1-7*Caterpillar 3516B Gen | 20100101 | 3.91 | N |
| 2007 | Equinix Incorporated - 21711 Filigree Court | 73233 | 107 | 00823 | 1 | 1 | 2 | C1-3* Caterpillar 3412B Genset | 39999992 | 0.43 | N |
| 2007 | Equinix Incorporated - 21711 Filigree Court | 73233 | 107 | 00823 | 2 | 2 | 2 | F1-6* Caterpillar 3516B Genset | 39999992 | 4.81 | N |
| 2007 | Equinix Incorporated - 21711 Filigree Court | 73233 | 107 | 00823 | 3 | 3 | 2 | G1-7* Caterpillar 3516C genset | 39999992 | 0.72 | N |
| 2007 | AT&T Web Hosting Facility | 73240 | 107 | 00824 | 1 | 1 | 1 | 1-6 GenSets #2 Oil | 20200102 | 0.92 | N |
| 2007 | AT&T Web Hosting Facility | 73240 | 107 | 00824 | 2 | 2 | 1 | G7 - CAT 3516C GenSet, 2000 kW, Diesel Fueled | 39999992 | 0.64 | N |
| 2007 | AT&T Web Hosting Facility | 73240 | 107 | 00824 | 3 | 3 | 1 | G8 - CAT 3516C GenSet, 2000 kW, Diesel Fueled | 39999992 | 0.64 | N |
| 2007 | AT&T Web Hosting Facility | 73240 | 107 | 00824 | 4 | 4 | 1 | G9 - CAT 3516C GenSet, 2000 kW, Diesel Fueled | 39999992 | 0.64 | N |
| 2007 | AT&T Web Hosting Facility | 73240 | 107 | 00824 | 5 | 5 | 1 | G10 - CAT 3516C GenSet, 2000 kW, Diesel Fueled | 39999992 | 0.64 | N |
| 2007 | AT&T Web Hosting Facility | 73240 | 107 | 00824 | 6 | 6 | 1 | G11 - CAT 3516C GenSet, 2000 kW, Diesel Fueled | 39999992 | 0.64 | N |
| 2007 | AT&T Web Hosting Facility | 73240 | 107 | 00824 | 7 | 7 | 1 | G12 - CAT 3516C GenSet, 2000 kW, Diesel Fueled | 39999992 | 0.64 | N |
| 2007 | LCSA - Broad Run Water Reclamation | 73268 | 107 | 00831 | 1 | 1 | 2 | G1-3*Cummins 2000DQKC, kW-hrs | 39999991 | 0.01 | N |
| 2007 | LCSA - Broad Run Water Reclamation | 73268 | 107 | 00831 | 1 | 1 | 1 | G1-3*Cummins 2000DQKC Gen. | 39999992 | 0.16 | N |
| 2007 | Nextel Sterling Lab VA-9 | 73286 | 107 | 00838 | 1 | 1 | 1 | Diesel engine exhaust | 39999992 | 0.17 | N |
| 2007 | VeriSign Incorporated Broad Run Facility2 | 73341 | 107 | 00843 | 1 | 1 | 1 | Desiel fuel firing | 39999992 | 0.72 | N |
| 2007 | VeriSign Incorporated Broad Run Facility2 | 73341 | 107 | 00843 | 2 | 2 | 1 | Desiel fuel firing | 39999992 | 0.72 | N |
| 2007 | VeriSign Incorporated Broad Run Facility2 | 73341 | 107 | 00843 | 3 | 3 | 1 | Desiel fuel firing | 39999992 | 0.72 | N |
| 2007 | HB Mellott Estate Incorporated - Bull Run Plant | 73356 | 107 | 00847 | 2 | 2 | 1 | diesel fuel firing | 39999992 | 0.42 | N |
| 2007 | HB Mellott Estate Incorporated - Bull Run Plant | 73356 | 107 | 00847 | 3 | 3 | 1 | diesel fuel firing | 39999992 | 0.44 | N |
| 2007 | HB Mellott Estate Incorporated - Bull Run Plant | 73356 | 107 | 00847 | 4 | 4 | 1 | diesel fuel firing | 39999992 | 1.23 | N |
| 2007 | HB Mellott Estate Incorporated - Bull Run Plant | 73356 | 107 | 00847 | 5 | 5 | 1 | diesel fuel firing | 39999992 | 1.82 | N |
| 2007 | Savvis Communications Corporation-DC Four Facility | 73641 | 107 | 00885 | 1 | 1 | 1 | Cummins Diesel GenSet, EG R (1) | 39999992 | 0.16 | N |
| 2007 | Savvis Communications Corporation-DC Four Facility | 73641 | 107 | 00885 | 2 | 2 | 1 | Cummins Diesel GenSet, EG A (2) | 39999992 | 0.20 | N |
| 2007 | Savvis Communications Corporation-DC Four Facility | 73641 | 107 | 00885 | 3 | 3 | 1 | Cummins Diesel GenSet, EG B (3) | 39999992 | 0.20 | N |
| 2007 | Savvis Communications Corporation-DC Four Facility | 73641 | 107 | 00885 | 4 | 4 | 1 | Cummins Diesel GenSet, EG C (4) | 39999992 | 0.16 | N |
| 2007 | Savvis Communications Corporation-DC Four Facility | 73641 | 107 | 00885 | 5 | 5 | 1 | Cummins Diesel GenSet, EG D (5) | 39999992 | 0.15 | N |
| 2007 | Savvis Communications Corporation-DC Four Facility | 73641 | 107 | 00885 | 6 | 6 | 1 | Cummins Diesel GenSet, EG E (6) | 39999992 | 0.16 | N |
| 2007 | Savvis Communications | 73160 | 107 | 73160 | 1 | 1 | 1 | 1-5*Caterpillar model 3512 gen. #2 oil | 39999992 | 0.92 | N |
| 2007 | Savvis Communications | 73160 | 107 | 73160 | 2 | 2 | 1 | 1-6 2000 kw Emergency generators #2 oil | 39999992 | 0.80 | N |
| 2007 | Savvis Communications | 73160 | 107 | 73160 | 3 | 3 | 1 | Caterpillar model 3512-C 1250 kW GenSet. Diesel Fuel | 39999992 | 0.09 | N |
| 2007 | Blackstone Town | 30881 | 135 | 00033 | 1 | 1 | 1 | DSL GEN hours of operation | 28888803 | 1.21 | N |
| 2007 | American Tower Corporation | 40876 | 137 | 00031 | 1 | 1 | 1 | Segment 1 Description | 20200102 | 0.12 | N |
| 2007 | Danville City - Department of Utilities | 32039 | 143 | 00157 | 1 | 1 | 1 | Generator - distillate oil | 20200401 | 0.43 | N |
| 2007 | Prince William Hospital Corporation | 70006 | 153 | 00011 | 5 | 5 | 1 | EG-1 through EG-5 Generators | 20300101 | 2.02 | N |
| 2007 | Potomac Hospital | 70295 | 153 | 00047 | 3 | 9 | 1 | HG-4 - 1500 kW Caterpillar 3512B emergency generator | 20200102 | 0.43 | N |
| 2007 | Potomac Hospital | 70295 | 153 | 00047 | 3 | 3 | 1 | HG-3 - 1500 kW Caterpillar 3512B emergency generator | 20200102 | 0.64 | N |
| 2007 | Comcast Cablevision of Virginia Incorporated | 72374 | 153 | 00144 | 1 | 1 | 1 | Two diesel-driven emergency generators | 39999992 | 0.27 | N |
| 2007 | Verizon Business | 73200 | 153 | 00147 | 1 | 1 | 1 | Emergency Generators (Ref #s 1-7) | 39999992 | 3.60 | N |
| 2007 | City of Manassas WTP | 73229 | 153 | 00817 | 1 | 1 | 2 | Caterpillar 3516B diesel generator - kilowatt/hours | 39999991 | 1.16 | N |
| 2007 | Architect of the Capitol | 73264 | 153 | 00889 | 2 | 2 | 1 | Cummins generator, Model KTA5063, 1250 kw, EG 250-1 | 20100102 | 1.09 | N |
| 2007 | Architect of the Capitol | 73264 | 153 | 00889 | 3 | 3 | 1 | Cummins generator, Model VT1710, 450 kW, EG 250-2 | 20100102 | 0.39 | N |
| 2007 | Architect of the Capitol | 73264 | 153 | 00889 | 4 | 4 | 1 | #1 Cummins generator, QSK60-G6, rated at 2000kW | 20100102 | 0.44 | N |
| 2007 | Architect of the Capitol | 73264 | 153 | 00889 | 7 | 5 | 1 | #2 Cummins generator, QSK60-G6, rated at 2000kW | 20100102 | 0.88 | N |
| 2007 | Old Dominion Electric Cooperative - Portable 61416 | 61416 | 175 | 61416 | 1 | 1 | 1 | 2000 kW Diesel Generator-Unit 1 23636 Meherrin Rd, Courtland VA 23837 | 20200401 | 0.89 | N |
| 2007 | Old Dominion Electric Cooperative - Portable 61417 | 61417 | 175 | 61417 | 1 | 1 | 1 | 2000 kW Diesel Generator, 23636 Meherrin Rd, Courtland VA 23837 | 20200401 | 0.94 | N |
| 2007 | GM Powertrain Fredericksburg | 40684 | 177 | 00043 | 5 | 6 | 1 | Emergency Generator (540bhp) | 20200102 | 0.15 | N |
| 2007 | GM Powertrain Fredericksburg | 40684 | 177 | 00043 | 6 | 7 | 1 | Fire Pump - 255bhp | 20200102 | 0.05 | N |
| 2007 | Spotsylvania Co Water Treatment-Ni River | 40887 | 177 | 00079 | 1 | 1 | 1 | Cummins Emergency Generator-No. 2 fuel oil | 20200401 | 0.12 | N |
| 2007 | Spotsylvania Co Water Treatment-Ni River | 40887 | 177 | 00079 | 1 | 2 | 1 | Emergency Generator - 400 kW | 20300101 | 0.14 | N |
| 2007 | Rappahannock Electric Coop Thornburg Middle Sch | 40980 | 177 | 00099 | 1 | 1 | 1 | 2000 kW Caterpillar Genset | 20100102 | 7.13 | N |
| 2007 | FMC Waste Water Treatment Plant | 41009 | 177 | 00105 | 1 | 3 | 1 | 100 kW emerg. genrator, falls under emerg. generator rule | 20200102 | 0.07 | N |
| 2007 | FMC Waste Water Treatment Plant | 41009 | 177 | 00105 | 1 | 1 | 1 | Caterpillar DP100P4, 750 kW | 20200401 | 0.08 | N |
| 2007 | FMC Waste Water Treatment Plant | 41009 | 177 | 00105 | 1 | 2 | 1 | Emerg. Generator - 625 kW | 20200401 | 0.07 | N |
| 2007 | Spotsylvania County Utilities Dept. Motts Run WTP | 41010 | 177 | 00106 | 1 | 1 | 1 | Emerg. Generator | 20200102 | 0.19 | N |
| 2007 | STAFFORD CORRECTIONAL UNIT 21 | 40569 | 179 | 00013 | 1 | 1 | 1 | CUMMINGS DIESEL 176 BHP | 20300101 | 4.53 | N |
| 2007 | Inova Alexandria Hospital | 70268 | 510 | 00035 | 1 | 4 | 1 | CATERPLR #1 GENERATOR | 20200102 | 0.85 | N |
| 2007 | Inova Alexandria Hospital | 70268 | 510 | 00035 | 1 | 4 | 2 | CATERPLR #2 GENERATOR | 20200102 | 0.85 | N |
| 2007 | Surface Deployment Distribution Command | 73362 | 510 | 00367 | 1 | 1 | 2 | G-2 Cummins Onan diesel genset | 39999992 | 0.20 | N |
| 2007 | Surface Deployment Distribution Command | 73362 | 510 | 00367 | 1 | 1 | 1 | G-1 Cummins Onan diesel genset | 39999992 | 0.84 | N |
| 2007 | Danville City - Department of Utilities | 32037 | 590 | 00152 | 1 | 1 | 1 | Generator - distillate oil | 20200401 | 0.45 | N |
| 2007 | Danville City - Department of Utilities | 32038 | 590 | 00153 | 1 | 1 | 1 | Generator - distillate oil | 20200401 | 0.38 | N |
| 2007 | Verizon Virginia Incorporated - Lee Highway | 73301 | 600 | 00848 | 1 | 1 | 1 | Two 1000 kW Generators | 39999992 | 0.52 | N |
| 2007 | Franklin City - Electric Dept - Mechanic Street | 60182 | 620 | 00011 | 1 | 1 | 1 | 2 DIESEL GENERATORS | 20100102 | 5.82 | N |
| 2007 | Franklin City - Pretlow Power Plant | 61419 | 620 | 00056 | 1 | 1 | 1 | 2 - PEAK SHAVING GENERATORS | 20200401 | 0.04 | N |
| 2007 | The Harrisonburg Electric Commission | 81272 | 660 | 00146 | 1 | 1 | 1 | RROYCEAVON1533 OIL TURBI | 20100101 | 4.63 | N |
| 2007 | The Harrisonburg Electric Commission | 81272 | 660 | 00146 | 2 | 2 | 1 | Caterpillar model 3516B compression ignition engine driven electri generator | 20100102 | 0.83 | N |
| 2007 | The Harrisonburg Electric Commission | 81272 | 660 | 00146 | 3 | 3 | 1 | Caterpillar Model 3516B D-6 generator | 20200102 | 1.01 | N |
| 2007 | Harrisonburg Electric Commission - Mt Clinton Pike | 81342 | 660 | 00155 | 1 | 1 | 1 | Stationary Gas Turbine #2 | 20100101 | 2.92 | N |
| 2007 | Harrisonburg Electric Commission - Mt Clinton Pike | 81342 | 660 | 00155 | 2 | 2 | 1 | Caterpillar model 3516B-D2 compression-ignition engine driven generator | 20100102 | 1.03 | N |
| 2007 | Harrisonburg Electric Commission - Mt Clinton Pike | 81342 | 660 | 00155 | 3 | 3 | 1 | Caterpillar model 3516B -D3 compression-ignition engine driven generator | 20100102 | 0.94 | N |
| 2007 | Harrisonburg Electric Commission - Mt Clinton Pike | 81342 | 660 | 00155 | 4 | 4 | 1 | Caterpillar model 3516B -D5 compression-ignition engine driven generator | 20100102 | 0.95 | N |
| 2007 | City of Manassas - Gateway Generation | 73642 | 683 | 00155 | 1 | 1 | 1 | EU01*1825 kW Caterpillar/ kW-hrs | 39999991 | 2.80 | N |
| 2007 | City of Manassas - Portable Genset 1 | 73569 | 683 | 73569 | 1 | 1 | 1 | PG-1*Caterpillar 3516B- kW-Hours | 39999991 | 2.32 | N |
| 2007 | City of Manassas - Portable Genset 2 | 73570 | 683 | 73570 | 2 | 2 | 1 | PG-2*Caterpillar 3516B- kW-hours | 39999991 | 2.09 | N |
| 2007 | Salem Water Treatment Plant | 21403 | 775 | 00252 | 1 | 1 | 1 | Emergency Diesel Generator | 39999992 | 4.90 | N |
|  |  |  |  |  |  | Total of NOx for units submitted with initial 2007 inventory: |  |  |  | 530.12 |  |
|  |  |  |  |  |  | Total of NOx for units not submitted with initial 2007 inventory: |  |  |  | 155.88 |  |
|  |  |  |  |  |  | Total NOx, all DG units: |  |  |  | 686.00 |  |