The State of New Jersey

Department of Environmental Protection

# Proposed

State Implementation Plan (SIP) Revisions for

the Attainment and Maintenance of the

8-Hour Carbon Monoxide National Ambient Air Quality Standard, 1-Hour Ozone National Ambient Air Quality Standard, and

Fine Particulate Matter National Ambient Air Quality Standard; and the 2002 Periodic Emission Inventory

February 2006

Preface

New Jersey is proposing revisions to several State Implementation Plans (SIP) that cover the attainment and maintenance of the 8-Hour Carbon Monoxide National Ambient Air Quality Standard, the 1-Hour Ozone National Ambient Air Quality Standard, and the Annual Fine Particulate Matter National Ambient Air Quality Standard.  In addition, the 2002 Periodic Emission Inventory is being submitted.

Specifically, proposed SIP revisions include updated carbon monoxide and ozone budgets for northern New Jersey for transportation conformity purposes that incorporate updated planning assumptions; a revised general conformity emission budget for McGuire Air Force Base to ensure that increases in activity at the McGuire Air Force Base conform with the ozone SIP; a fine particulate matter, PM2.5, transportation conformity budget for northern New Jersey for the interim period prior to the PM2.5 attainment demonstration SIP; a carbon monoxide limited maintenance plan for Camden County and the nine not-classified areas for the second ten-year compliance period; and the 2002 periodic emission inventory.

### Acknowledgments

The New Jersey Department of Environmental Protection acknowledges the efforts and assistance of the many agencies and individuals whose contributions were instrumental in the preparation of this State Implementation Plan Revision.  In particular, the New Jersey Department of Environmental Protection wishes to acknowledge the many individuals within the New Jersey Department of Transportation, the United States Environmental Protection Agency Region 2, and the North Jersey Transportation Planning Authority, the Delaware Valley Regional Planning Commission, and as well as staff within the New Jersey Department of Environmental Protection for their assistance and guidance.

Table of Contents

## Preface	i

## Acknowledgements	i

## Table of Contents	ii

## List of Tables	iv

## List of Figures	v

## List of Appendices and Attachments	vi

##### Acronyms and Abbreviations	vii

## Executive Summary	viii

## Introduction	1

## Conformity Emission Budgets	1

## Background	2

## Revised Budgets	3

###### 1-Hour Ozone and a Carbon Monoxide Budget for Northern New Jersey	3

- Effect of Updated Transportation Conformity Emission Budgets on the Carbon Monoxide Maintenance Plans	9

##### Effect of Updated Transportation Conformity Emission Budgets on the Ozone Attainment Demonstration	9

###### Amendment to the McGuire Air Force Base Conformity Budget	12

## New Budget – Particulate Matter Budget for Northern New Jersey	15

- PM2.5 General Background	16

- 2.   	Establishment of the PM2.5 Standard	16

3.	Designation of PM2.5 Nonattainment Areas in New Jersey	17

- 4.        Transportation Conformity for PM2.5	18

- Early PM2.5 Transportation Conformity Emission Budgets	20

- PM2.5 and Annual NOx Inventories for 2002 and 2009	20

##### Early Transportation Conformity Emission Budgets for PM2.5 and Annual NOx	24

## 8-Hour Carbon Monoxide Limited Maintenance Plan for Southern New Jersey and the Nine Not-Classified Areas 	26

- Background	26

- Air Quality Update	28

- The USEPA Guidance and Requirements for Limited Maintenance Plans	30

8-Hour Carbon Monoxide Maintenance Plan	32

- Attainment Inventory	32

- Maintenance Demonstration	34

- Monitoring Network	34

- Verification of Continued Attainment	34

- Contingency Plan	36

- Transportation Conformity	37

## 2002 Periodic Emission Inventory	38

##### Background	38

##### Statutory and Regulatory Background	38

- Emission Inventory Overview	38

- Emission Inventory Summary	41

- Emissions Comparison Summary	50

## Public Participation	50

### List of Tables

Table ES1	Updated Carbon Monoxide and 1-Hour Ozone Transportation Conformity Emission Budgets	ix

Table ES2  	Comparison of the Updated Carbon Monoxide and 1-Hour Ozone Transportation Conformity Emission Budgets with Prior Budgets	ix

Table ES3	Emission Budgets for McGuire Air Force Base	xi

Table ES4	New PM2.5 Transportation Conformity Emission Budgets	xii

Table I 	Updated Transportation Conformity Emission Budgets	 5

Table II 	Comparison of the Updated Transportation Conformity Emission Budgets with Prior Budgets	 5

Table III	Maintenance Plan Inventories for the New Jersey Portion of the New York City/Northern New Jersey/Long Island Carbon Monoxide Nonattainment Area	 6

Table IV	2007 Margin of Safety Calculation	 7

Table  V	2014 Margin of Safety Calculation	 8

Table VI	Comparison of the Onroad Previous Inventories to the Updated Onroad Inventories (Tons per Ozone Day Unless Designated Otherwise)	11

Table VII	Emission Budgets for McGuire Air Force Base	15

Table VIIII	Direct PM2.5 Emission Inventories for 2002 and 2009 for the New Jersey Portion of the New York /New Jersey/Long Island/Connecticut PM2.5 Nonattainment Area	21

Table IX	Direct PM2.5: Calculation of the Percent Reduction in Projected 2009 Emissions From the 2002 Emissions by County	22

Table X	Annual NOx Emission Inventories for 2002 and 2009 for the New Jersey Portion of the New York /New Jersey/Long Island/Connecticut PM2.5 Nonattainment Area	23

Table XI	Annual NOx: Calculation of the Percent Reduction in Projected 2009 Emissions from the 2002 Emissions by County	24

Table XII	Transportation Conformity Emission Budgets	25

Table XIII	Design Values for Carbon Monoxide in New Jersey (8-hour standard)	30

Table XIV	New Jersey Carbon Monoxide Maintenance Plan and Contingency Measure History	32

Table XV	New Jersey Wintertime Carbon Monoxide Emissions (tons per winter day) for 2002	33

Table XVI	2002 Inventories Prepared	41

Table XVII	2002 Statewide Emission Inventory by Source Sector and Pollutant	42

Table XVIII	2002 Statewide Emission Inventory by County and Pollutant	43

Table XIX	2002 Statewide Emission Inventory by County and Source Sector	45

Table XX	1996 and 2002 Statewide Emission Inventory by Source Sector and Pollutant	50

### List of Figures

Figure I	Metropolitan Planning Organizations in New Jersey	4

Figure II	Carbon Monoxide Air Quality, 1975-2004, 2nd Highest 8-Hour Average	7

Figure III	USEPA Designations of Nonattainment Areas for PM2.5 in New Jersey	19

Figure IV	New Jersey Carbon Monoxide Maintenance Areas	29

Figure V	New Jersey Carbon Monoxide Monitoring Network (2004)	35

Figure VI	New Jersey 8-Hour Ozone Nonattainment Areas	39

Figure VII	New Jersey Fine Particulate Matter Nonattainment Areas	40

### List of Appendices

##### Appendix A	History of New Jersey’s Carbon Monoxide State Implementation Plans

Appendix B	History of New Jersey’s Ozone State Implementation Plans

*Appendix C	Transportation Conformity Emission Budget Back-up Files

##### Appendix D	2002 Periodic Emissions Inventory Document

# Attachment 1	Inventory Top 15 by SCC Graphs

# Attachment 2	Fugitive Dust Inventory Discussion and Summary

# Attachment 3	Point Source: Creating the 2002 Point Source Inventory

# Attachment 4*	Point Source VOC Inventory

# Attachment 5*	Point Source NOx Inventory

# Attachment 6*	Point Source CO Inventory

# Attachment 7*	Point Source PM10 Inventory

# Attachment 8*	Point Source PM2.5 Inventory

# Attachment 9*	Point Source SO2 Inventory

# Attachment 10*	Point Source NH3 Inventory

# Attachment 11*	Area Source Calculation Sheets

# Attachment 12*	Area Source Inventories

# Attachment 13*	Onroad Source DVMT

# Attachment 14*	Onroad Source PPSUITE Files

# Attachment 15*	Onroad Source North Jersey Transportation Planning Authority Calculation Files

# Attachment 16*	Onroad Source South Jersey Transportation Planning Organization Calculation Files

# Attachment 17*	Onroad Source Delaware Valley Regional Planning Commission Calculation Files

# Attachment 18*	Onroad Source Inventory

# Attachment 19	Onroad Sources Refueling Emissions by County

# Attachment 20*	Nonroad Calculation Sheets for Orphan Categories

# Attachment 21*	Nonroad Source Inventory

# Attachment 22*	Biogenic Source NH3 Inventory

# Attachment 23	Point Source Quality Assurance Documentation

# Attachment 24 	Point Source Detailed Information

# Attachment 25*	Area Source Inventory Comparisons

* Note: These appendices and attachments are only available electronically.

### Acronyms and Abbreviations

AADF	Annual Activity Day Factors

CFR	Code of Federal Regulations

CNG	Compressed Natural Gas

DVMT	Daily Vehicle Miles Traveled

EDMS	Emissions and Dispersion Modeling System

EWR	Newark Liberty International Airport

FMVCP	Federal Motor Vehicle Control Program

FR	Federal Register

GSE	Ground Support Equipment

HDDV	Heavy Duty Diesel Vehicles

I/M	Inspection and Maintenance

LDGT	Light Duty Gasoline Trucks

LDGV	Light Duty Gasoline Vehicle

LPG	Liquefied Petroleum Gas

LTO	Landing and Take-off Operations

NAAQS	National Ambient Air Quality Standards

NEI	National Emissions Inventory

NH3	Ammonia

NJDEP	New Jersey Department of Environmental Protection

NJR	New Jersey Register

NLEV	National Low Emission Vehicle

NNEM	NONROAD Emission Model

NOx	Oxides of Nitrogen

PM2.5	Fine Particulate Matter (particles with an aerodynamic diameter less than or equal to a nominal 2.5 micrometers)

PM10	Particles with an aerodynamic diameter less than or equal to a nominal 10 micrometers

PPSUITE	Performance Evaluation and Emissions Analysis

P.L.	Public Law

RACT	Reasonably Available Control Technology

RFG	Reformulated Gasoline

RVP	Reid Vapor Pressure

SCC	Source Classification Code

SIP	State Implementation Plan

SMOKE	Sparse Matrix Operator Kernel Emissions

SO2	Sulfur Dioxide

SOx	Oxides of Sulfur

U.S.C.	United States Code

USEPA	United States Environmental Protection Agency

VOCs	Volatile Organic Compounds

Executive Summary

The State of New Jersey is proposing several revisions to its State Implementation Plans (SIPs) governing the National Ambient Air Quality Standards (NAAQS) for carbon monoxide, ozone, and fine particulate matter.  Specifically, New Jersey is proposing the following:

- Revised transportation conformity emission budgets consisting of a 1-Hour Ozone and Carbon Monoxide Budget for Northern New Jersey and an amendment to the McGuire Air Force Base General Conformity Budget;

- New transportation conformity emission budgets consisting of a Fine Particulate Matter Budget for Northern New Jersey;

- The second ten-year Carbon Monoxide Limited Maintenance Plan for Camden County and the nine not classified maintenance areas; and,

- New Jersey’s 2002 periodic emission inventory.

1-Hour Ozone 2005/2007 Budgets and Carbon Monoxide 2007/2014 Budgets for Northern New Jersey:

New Jersey is proposing updated volatile organic compounds (VOCs) and oxides of nitrogen (NOx) transportation conformity emission budgets that incorporate new data that will be required for use in future conformity determinations for New Jersey.  The proposed updates to the budgets are being proposed because of significant changes in planning assumptions involving vehicle miles traveled predictions, vehicle age distributions, and the latest allocation of vehicle miles traveled to the various vehicle types in the North Jersey Transportation Planning Authority area. An analysis was performed that compares these updated budgets to the budgets that represented the attainment demonstration for the 1-hour ozone standard.  This analysis demonstrates that the updated budgets continue to support predicted achievement of rate of progress and projected attainment of the 1-hour ozone NAAQS for the New Jersey portion of the New York City/Northern New Jersey/Long Island nonattainment area by the attainment date of 2007.

All 21 New Jersey counties are designated as moderate nonattainment for the 8-hour ozone NAAQS by the United States Environmental Protection Agency (USEPA). The 1-hour ozone NAAQS was revoked on June 15, 2005.  The attainment demonstration SIP for the 8-hour ozone NAAQS is not due until June 2007.  The USEPA has established that prior to the establishment of 8-hour budgets, the use of 1-hour budgets ensures that air quality progress to date is maintained. The USEPA also contends that even after the 1-hour NAAQS was revoked, the 1-hour budgets continue to be part of a nonattainment area’s SIP and are therefore appropriate to use as proxies for the 8-hour standard.

The updated transportation conformity emission budgets are provided in Table ES1.  Table ES2 contains a comparison of the updated transportation conformity emission budgets with prior budgets.  The updated budgets supersede the prior budgets and must be used for future transportation conformity determinations by the North Jersey Transportation Planning Authority Metropolitan Planning Organization once approved by the USEPA.

Table ES1

Proposed Updated Carbon Monoxide and 1-Hour Ozone

Transportation Conformity Emission Budgets

| Transportation Planning Area | Carbon Monoxide Emissions(1) (tons per winter day) |  |  | VOC Emissions(2) (tons per ozone day) |  | NOx Emissions(2) (tons per ozone day) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1997 | 2007 | 2014 | 2005 | 2007 | 2005 | 2007 |
| North Jersey Transportation Planning Authority | 1,550.74 | 1,150.99 | 899.01 | 146.33 | 122.53 | 327.83 | 256.58 |

NOTES:

- For Passaic, Bergen, Essex, Hudson and Union counties.

- For all counties within the Metropolitan Planning Organization.

Table ES2

Comparison of the Proposed Updated Carbon Monoxide and 1-Hour Ozone

Transportation Conformity Emission Budgets with Prior Budgets

| Transportation Planning Area | Prior or Updated | Carbon Monoxide Emissions(1) (tons per winter day) |  |  | VOC Emissions(2) (tons per ozone day) |  | NOx Emissions(2) (tons per ozone day) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1997 | 2007 | 2014 | 2005 | 2007 | 2005 | 2007 |
| North Jersey Transportation Planning Authority | Prior | 1,550.74 | 783.39 | 605.63 | 148.27 | 125.82 | 253.05 | 198.34 |
|  | Updated | Not Updated | 1,150.99 | 899.01 | 146.33 | 122.53 | 327.83 | 256.58 |

NOTES:

- For Passaic, Bergen, Essex, Hudson and Union counties.

- For all counties within the Metropolitan Planning Organization.

Transportation conformity emission budgets for carbon monoxide are updated to include a safety margin but were not recalculated to include the latest planning assumptions.  This proposed SIP revision does not affect any of the planned or implemented control measures for carbon monoxide, VOCs, and NOx.  In addition, the updated budgets do not result in backsliding on New Jersey’s plans to address the 1-hour ozone NAAQS and maintain attainment of the carbon monoxide standard.

The carbon monoxide budgets were updated to incorporate the “margin of safety” provisions of the USEPA’s transportation conformity rule.  The safety margin is the difference between the attainment year total emissions and future year total emissions.  This safety margin was applied to the existing North Jersey Transportation Planning Authority carbon monoxide budgets for 2007 and 2014.

Actual measured carbon monoxide concentrations have been steadily decreasing over the last thirty years as a likely consequence of improvements in motor vehicle emission technology.  Onroad mobile sources have historically contributed the largest portion of the carbon monoxide inventory in New Jersey relative to the other source sectors.  The last time the carbon monoxide NAAQS was exceeded in New Jersey was in January of 1995.  The monitored carbon monoxide levels have trended downward so that the maximum measured values are currently about one-half of the NAAQS level.

The use of the updated budgets for transportation conformity determinations will continue to achieve the goal of the maintenance of the NAAQS for carbon monoxide.  The updates to the carbon monoxide budgets do not affect the conclusions of the carbon monoxide maintenance plan for the New Jersey portion of the New York City/Northern New Jersey/Long Island area.

McGuire Air Force Base General Conformity Budget Amendment:

McGuire Air Force Base currently has a 1-hour ozone general conformity budget.  As noted previously, the USEPA revoked the 1-hour ozone standard for the nation on June 15, 2005, as the newly established 8-hour ozone standard superseded it.  McGuire Air Force Base has agreed to live within its 1-hour ozone budget until such time as a new budget is established under the 8-hour ozone standard, with one condition.  McGuire Air Force Base requests that the State allow it to reapportion additional VOC reductions from its VOC budget to its NOx budget to accommodate anticipated mission changes for the McGuire Air Force Base.  Specifically, McGuire Air Force Base proposes to increase its existing NOx budget by 450 tons per year by decreasing its VOC budget by 468 tons per year.  Previously, USEPA approved a similar reapportionment at McGuire Air Force Base in 2003.  The VOC/NOx emission ratio (1.04) was derived from the 1-hour ozone attainment demonstration for the area.  Table ES3 contains updated emission budget information for McGuire Air Force Base.

Fine Particulate Matter Transportation Conformity Budget for the New Jersey Portion of the New York/New Jersey/Long Island/Connecticut Nonattainment Area:

New Jersey is proposing to establish early transportation conformity emission budgets for directly emitted fine particulate matter (direct PM2.5) and annual NOx (a PM2.5 precursor) for the New Jersey portion of the New York/New Jersey/Long Island/Connecticut PM2.5 nonattainment area.  The USEPA is allowing areas to establish early budgets as an alternative to other interim conformity tests prior to submittal of the PM2.5 attainment demonstration in April 2008.  To establish an early budget, an area must meet certain criteria defined by the USEPA.  The New Jersey portion of the New York/New Jersey/Long Island/Connecticut PM2.5 nonattainment area meets the criteria.  The use at the proposed budget is expected to be more air quality constraining than the choice of using either of the two other interim conformity tests.  Once found to be adequate, these early PM2.5 budgets must be used by the Metropolitan Planning Organizations during the interim period prior to the establishment of PM2.5 budgets with the PM2.5 attainment demonstration SIP.

Table ES3

Emission Budgets for McGuire Air Force Base

|  | #### Prior Budget |  | Updated Budget to Accommodate Additional Aircraft |  |
| --- | --- | --- | --- | --- |
|  | VOC (Tons/Year) | NOx (Tons/Year) | VOC (Tons/Year) | NOx (Tons/Year) |
| #### 1990 Baseline | 1,112 | 1,038 | 1,112 | 1,038 |
| 1996 | 1,186 | 1,107 | 1,186 | 1,107 |
| 1999 | 1,223 | 1,142 | 1,223 | 1,142 |
| 2002 | 1,405 | 875 | 1,405 | 875 |
| 2005(1) | 1,198 | 1,084 | 730 | 1,534 |

NOTES:

(1) Budgets updated such that the increase in NOx is offset by a decrease in VOC. Updated 2005 budgets apply to 2005 and all future years until new budgets are established for the 8- hour ozone attainment demonstration

The early transportation conformity emission budgets are provided in Table ES4.  The early budgets must be used for future transportation conformity determinations by the North Jersey Transportation Planning Authority and the Delaware Valley Regional Planning Commission Metropolitan Planning Organization once found to be adequate by the USEPA.

Table ES4

New PM2.5 Transportation Conformity Emission Budgets

| Transportation Planning Area | Direct PM2.5 Emissions(1) (tons per year) | Annual NOx Emissions (tons per year) |
| --- | --- | --- |
|  | 2009 | 2009 |
| North Jersey Transportation Planning Authority(2) | 1,207 | 61,676 |
| Delaware Valley Regional Planning Commission(3) | 89 | 4,328 |

NOTES:

- Direct PM2.5 consists of the sum of: SO4, Organic Carbon, Elemental Carbon, particulate Matter from gasoline vehicles, lead, brake particles, and tire particles.

- For Bergen, Essex, Hudson, Middlesex, Monmouth, Morris, Passaic, Somerset and Union Counties.

- For Mercer County.

Carbon Monoxide Limited Maintenance Plan:

According to 42 U.S.C. §7505a(b) New Jersey is proposing the second ten-year maintenance plans for ten of its eleven carbon monoxide maintenance areas whose initial maintenance plans expire in 2007.  The ten maintenance areas are:

- The Camden County Carbon Monoxide Maintenance Area – covering all of Camden County; and,

- The Nine Not-Classified Carbon Monoxide Maintenance Areas – covering the City of Atlantic City (in Atlantic County), the City of Burlington (in Burlington County), the Borough of Freehold (in Monmouth County), the Town of Morristown (in Morris County), the Borough of Penns Grove (in Salem County), the City of Perth Amboy (in Middlesex County), the Borough of Somerville (in Somerset County), the Toms River Area (in Ocean County), and the City of Trenton (in Mercer County).

In an effort to consolidate the preparation of these second ten-year maintenance plans, the State is proposing a consolidated Limited Maintenance Plan encompassing all ten (10) areas, covering the second ten-year maintenance period.  New Jersey’s eleventh carbon monoxide maintenance area, encompassing four counties and part of a fifth county in the Northeastern portion of the State, has an initial maintenance plan that does not expire until 2014.  As such, this area is not required and not able to submit a second ten-year plan until around 2012.

This proposed carbon monoxide Limited Maintenance Plan demonstrates that the New Jersey carbon monoxide maintenance areas continue to be in attainment of the health-based NAAQS for carbon monoxide and will continue to be in attainment for another ten (10) years.  In general, monitoring data for these areas show a trend of decreasing carbon monoxide concentrations in the air over the past decade.  These improvements are due to permanent and enforceable measures that the state and federal government have implemented, and not necessarily attributable to favorable meteorology or other factors.  Despite the growth in economic activity, vehicle miles traveled, and population that have and are expected to occur, the maintenance areas are expected to meet the health-based carbon monoxide NAAQS through the remainder of the their first ten-tear maintenance plans and all of the second ten-year maintenance plans.

Finally, in compliance with the USEPA’s Limited Maintenance Plan Guidance, the emission budgets in these Limited Maintenance Plan areas are now treated as not constraining for purposes of transportation conformity.  This is true for the length of the maintenance periods because it is unreasonable to expect that the area would experience so much growth in that period that a violation of the carbon monoxide NAAQS would result.  As such, once a Limited Maintenance Plan has been approved for these areas, it will no longer be necessary for the Metropolitan Planning Organizations to perform numerical regional analyses to demonstrate transportation conformity for Transportation Plans and Transportation Improvement Programs.  However, project-level carbon monoxide evaluation of transportation projects (project-level conformity) still needs to be performed in areas with approved Limited Maintenance Plans.

2002 Periodic Emission Inventory:

42 U.S.C. §7410(a)(2)(F) requires the submission by states to the USEPA of periodic reports on the nature and amounts of emissions and emissions related data.  In 2002, the USEPA promulgated the Consolidated Emission Reporting Rule at 40 CFR 51, Subpart A.  As indicated by its name, this rule “consolidated” the various emissions reporting requirements that already had established for existing NAAQS (e.g., ozone (VOC and NOx), carbon monoxide, etc.) into one location within the Code of Federal Regulations.  In addition, the Consolidated Emission Reporting Rule:

- Established new reporting requirements related to PM2.5, its potential precursors (ammonia (NH3), oxides of sulfur (SOx), NOx and VOC), and regional haze;

- Established new requirements for the statewide reporting of area source and mobile source emissions; and,

- Required two types of inventories (annual inventories and three year cycle inventories).

New Jersey’s proposed 2002 Periodic Emission Inventory meets the new Consolidated Emission Reporting Rule requirements and establishes 2002 as the base year for future attainment planning purposes with respect to 8-hour ozone, PM2.5 and regional haze, as required by the USEPA.  This proposed inventory is a compilation of the emissions from sources of biogenic (natural) and anthropogenic (human-made) VOC, NOx, carbon monoxide, particulate matter less than 10 micrometers in diameter (PM10), PM2.5, sulfur dioxide (SO2) and ammonia (NH3) in the outdoor air.  The sources are divided into five sectors and each making up one component of the inventory: point sources, area (nonpoint) sources, onroad sources, nonroad sources, and biogenic sources.

The pie charts on the following pages show the major air contaminants estimated within the 2002 Emissions Inventory.  As can be seen from these charts, VOC emissions are occurring from all sectors of the inventory with no sector having a much more predominant role than the others. On-Road mobile sources predominate the NOx and carbon monoxide emissions and point sources predominate the sulfur dioxide emissions. Area sources predominate the releases of ammonia and particulate matter (PM2.5 and PM10).

- Introduction

The Clean Air Act provides for states to submit revisions to their State Implementation Plans (SIP) whenever states’ programs are modified from the existing SIP. This document proposes the following revisions to the existing New Jersey SIP:

- Revised transportation conformity emission budgets for ozone and carbon monoxide for northern New Jersey;

- Revised general conformity budget for McGuire Air Force Base;

- New transportation conformity budget for fine particulate matter, PM2.5, for northern New Jersey;

- Limited Maintenance Plan for carbon monoxide for Camden County and the nine not-classified areas;

- New Jersey’s 2002 Periodic Emission Inventory.

The USEPA requires the 2002 Periodic Emission Inventory to be submitted as a SIP revision because it is the foundation for the planning to attain the 8-hour ozone and fine particulate matter health standards, and for the regional haze SIP.

Summaries of the history of New Jersey’s SIPs for carbon monoxide and ozone are presented in Appendix A and Appendix B, respectively.

- Conformity Emission Budgets

New Jersey is proposing updating the existing transportation conformity VOC, NOx and CO emission budgets and proposing early transportation conformity emission budgets for direct PM2.5 and annual NOx (a PM2.5 precursor) for the New Jersey portion of the New York/New Jersey/Long Island/Connecticut PM2.5 nonattainment Area. New Jersey is also proposing updates to the existing general conformity budgets for McGuire Air Force Base.

The updates to the transportation conformity emission budgets for VOCs and NOx are being proposed to reflect the most recent predictions of vehicle miles traveled, vehicle age distributions, and the latest allocation of vehicles miles traveled to the various vehicle types.  Updates to the transportation conformity emission budgets for carbon monoxide are being proposed to include a safety margin.  These proposed updated budgets do not affect any of the planned or implemented control measures for carbon monoxide, VOCs, or NOx.  In addition, the updated budgets do not result in backsliding on New Jersey’s plans to address the 1-hour ozone NAAQS and maintain attainment of the carbon monoxide standard.

Early budgets to address the PM2.5 NAAQS are provided for direct PM2.5 and annual NOx for the New Jersey portion of the New York City/Northern New Jersey/Long Island/Connecticut nonattainment area. The attainment demonstration SIP for the PM2.5 NAAQS is not due until April 2008.  Once found to be adequate, these early PM2.5 budgets must be used by the Metropolitan Planning Organizations during the interim period prior to the establishment of PM2.5 budgets with the PM2.5 attainment demonstration SIP. The USEPA is allowing areas to establish early budgets as an alternative to other interim conformity tests prior to submittal of the PM2.5 attainment demonstration in April 2008.

New Jersey is proposing changes in the general conformity budget for McGuire Air Force Base based on projected future increases NOx budget needs. The current and projected VOC emissions at McGuire Air Force Base are well below budget levels.  Therefore, this request for an increase in the NOx budget will be offset by a decrease in their VOC budget.

- Background

42 U.S.C. §7506 states “no department, agency, or instrumentality of the federal government shall engage in, support in any way, or provide financial assistance for, or approve, any activity which does not conform to an approved or promulgated state implementation plan.  No Metropolitan Planning Organization designated under section 134 of Title 23, shall give its approval to any project, program, or plan which does not conform to an approved or promulgated state implementation plan.”  These requirements are interpreted by the USEPA to apply to maintenance as well as nonattainment areas.

“Conformity to an implementation plan” means conforming to the implementation plan’s purpose of eliminating or reducing the severity and number of violations of the health-based NAAQS and achieving expeditious attainment of such standards.  In order for a proposed transportation activity to conform to the SIP, the Clean Air Act specifies that such activity will not:

- Cause or contribute to any new violation of any standard in any area;

- Increase the frequency or severity of any existing violation of any standard in any area; or,

- Delay timely attainment of any standard or any required interim emission reductions or any other milestones in any area.

The Federal Transportation Conformity Rule (40 CFR 93.100-160) provides the process by which the air quality impact of transportation plans, transportation improvement programs, and projects will be analyzed.  The agency preparing plans (twenty or more years), transportation improvement programs (at least four years), or approving a transportation project must analyze the emissions expected from such a proposal in accordance with the Transportation Conformity Rule.

For the purposes of transportation conformity, the emission budget is that portion of the total allowable emissions in the SIP emission inventory that is allocated to onroad vehicles.  The projected emissions from a plan, transportation improvement program, or project, estimated in accordance with the Transportation Conformity Rule, may not exceed the motor vehicle emissions budget contained in the appropriate SIP.  Emissions in years for which no motor vehicle emissions budgets are specifically established must be less than or equal to the motor vehicle emissions budget established for the most recent prior year.

B.	Revised Budgets

This section discusses the proposed updates to the existing transportation conformity budgets for certain areas within the North Jersey Transportation Planning Authority and the proposed updates to the existing general conformity budgets for McGuire Air Force Base.

- 1-Hour Ozone and a Carbon Monoxide Budgets for Northern New Jersey

The Metropolitan Planning Organizations are required to use the latest planning assumptions pursuant to the USEPA Transportation Conformity Rule (40 CFR 93.100-160).  There are three Metropolitan Planning Organizations in New Jersey, Figure I.  The proposed budgets described in this section are for the North Jersey Transportation Planning Authority.

The proposed transportation conformity emission budgets for VOCs and NOx are updated to reflect the most recent Highway Performance Monitoring System reconciliation of vehicle miles traveled updated (2005), vehicle age distributions, and the latest allocation of vehicle miles traveled to the various vehicle types.  The proposed transportation conformity emission budgets for carbon monoxide are updated to include a safety margin and are not updated to include the latest planning assumptions.  The proposed budgets do not affect any of the planned or implemented control measures for carbon monoxide, VOCs, and NOx.  In addition, the updated budgets do not indicate a need for any additional control measures for New Jersey to maintain attainment of the carbon monoxide NAAQS or reach attainment of the 1-hour ozone NAAQS.  Please note: The attainment demonstration SIP for the 8-hour ozone NAAQS is not due until June 2007.  Prior to approval of a SIP for the 8-hour ozone NAAQS, compliance with the 1-hour ozone NAAQS SIP continues to be required with regard to Transportation Conformity.

The updated transportation conformity emission budgets are provided in Table I.  Table II contains a comparison of the updated transportation conformity emission budgets with prior budgets. The updated budgets supersede the prior budgets and should be used for future transportation conformity determinations by the North Jersey Transportation Planning Authority once approved by the USEPA.

Figure I

Metropolitan Planning Organizations in New Jersey

Table I

Proposed Updated Transportation Conformity Emission Budgets

| Transportation Planning Area | Carbon Monoxide Emissions(1) (tons per winter day) |  |  | VOC Emissions(2) (tons per ozone day) |  | NOx Emissions(2) (tons per ozone day) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1997 | 2007 | 2014 | 2005 | 2007 | 2005 | 2007 |
| North Jersey Transportation Planning Authority | 1,550.74 | 1,150 | 899 | 146.33 | 122.53 | 327.83 | 256.58 |

NOTES:

- For Passaic, Bergen, Essex, Hudson and Union counties.

- For all counties within the Metropolitan Planning Organization.

Table II

Comparison of the Proposed Updated Transportation Conformity Emission Budgets

with Prior Budgets

| Transportation Planning Area | Prior or Updated | Carbon Monoxide Emissions(1) (tons per winter day) |  |  | VOC Emissions(2) (tons per ozone day) |  | NOx Emissions(2) (tons per ozone day) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1997 | 2007 | 2014 | 2005 | 2007 | 2005 | 2007 |
| North Jersey Transportation Planning Authority | Prior | 1,550.74 | 783.39 | 605.63 | 148.27 | 125.82 | 253.05 | 198.34 |
|  | Updated(3) | Not Updated | 1,150 | 899 | 146.33 | 122.53 | 327.83 | 256.58 |

NOTES:

- For Passaic, Bergen, Essex, Hudson and Union counties.

- For all counties within the Metropolitan Planning Organization.

- Even though a number of the updated budgets are greater that the prior budgets, the maintenance SIP for carbon monoxide continues to demonstrate maintenance of the NAAQS and the attainment SIP for ozone continues to demonstrate attainment of the NAAQS as discussed in the subsequent subsections of the text.

Carbon Monoxide Budgets

The carbon monoxide budgets for the five New Jersey counties in the New York City/Northern New Jersey/Long Island carbon monoxide maintenance area are proposed to be updated to incorporate the “margin of safety” provisions of the USEPA’s transportation conformity rule. The safety margin is the difference between the attainment year total emissions and future year total emissions. The rule indicates that when the projected emissions from all sources are less than the total emissions that would be consistent with attainment, the SIP may explicitly quantify a safety margin and include some or all of it in the motor vehicle emission budget for purposes of conformity. Table III shows the 1996 attainment emission inventory and future projection emission inventories.  Tables I and II include the margins of safety.

Note that even though only the municipalities of Clifton, Passaic, and Paterson in Passaic County are in the maintenance area, the entire Passaic County is included in the area budget because this is how prior budgets were established.  These inventories are from the 2002 USEPA approved maintenance plan.

Table IV and Table V show the calculation of the updated 2007 and 2014 budgets, respectively. It should be noted that 1996 statewide levels of carbon monoxide were measured to be only about 75 percent (maximum second highest 8-hour average) of the carbon monoxide 8-hour NAAQS monitored in the area as shown in Figure II.  Also, for monitors within the New Jersey portion of the New York/New Jersey/Long Island carbon monoxide maintenance area, the maximum second highest 8-hour average carbon monoxide levels were all below 7.0 parts per million (NAAQS is 9.0 parts per million). Therefore, the 1996 inventory represents carbon monoxide emission levels that are likely significantly lower than the minimum levels required to achieve attainment of the NAAQS resulting in conservatively lower carbon monoxide budgets.  Additional information concerning historical measurements of carbon monoxide levels in New Jersey are provided in the carbon monoxide Limited Maintenance Plan section of this SIP revision, Section III.

Table III

Maintenance Plan Inventories for the New Jersey Portion of the

New York City/Northern New Jersey/Long Island Carbon Monoxide Nonattainment Area

| #### County | New Jersey Portion of the New York City/Northern New Jersey/Long Island Carbon Monoxide Area - Carbon Monoxide Emissions in Tons Per Winter Season Day |  |  |
| --- | --- | --- | --- |
|  | 1996 (Attainment Year) | 2007 | 2014 |
| Bergen | 455.29 | 350.56 | 381.01 |
| Essex | 287.10 | 209.08 | 222.18 |
| Hudson | 183.64 | 118.20 | 130.76 |
| Passaic | 220.17 | 163.46 | 171.53 |
| Union | 219.11 | 156.41 | 166.45 |
| Totals | 1,365.31 | 997.71 | 1,071.93 |

2007 Safety Margin Calculation

1365.31 tons per day (1996 total emissions) – 997.71 tons per day (2007 total emissions) = 367.60 tons per day (2007 margin of safety)

367.60 tons per day + 783.39 tons per day (existing 2007 carbon monoxide budget) = 1150.99 tons per day (onroad emission budget for 2007 including the margin of safety)

Table IV

2007 Margin of Safety Calculation

| Year | Total Emissions 1996 | Total Emissions 2007 | 2007 Margin of Safety | Prior 2007 budget | Updated 2007 budget |
| --- | --- | --- | --- | --- | --- |
| Budget (tons/day) | 1365.31 | 997.71 | 367.60 | 783.39 | 1150.99 |

2014 Safety Margin Calculation

1365.31 tons per day (1996 total emissions) – 1071.93 tons per day (2014 total emissions) = 293.38 tons per day (2014 margin of safety)

293.38 tons per day + 605.63 tons per day (existing 2014 carbon monoxide budget) = 899.01 tons per day (onroad emission budget for 2014 including the margin of safety)

Table V

2014 Margin of Safety Calculation

| Year | Total Emissions 1996 | Total Emissions 2014 | 2014 Margin of Safety | Prior 2014 budget | Updated 2014 budget |
| --- | --- | --- | --- | --- | --- |
| Budget (tons/day) | 1365.31 | 1071.93 | 293.38 | 605.63 | 899.01 |

Ozone Budgets – VOC/NOx

The United States Department of Transportation (USDOT) and the USEPA guidance strongly recommend five year updates to planning assumptions.  The new planning assumptions involve updates to the vehicle miles traveled adjustment files and the vehicle miles traveled fractions assigned to the various vehicle types planning assumptions.  These vehicle miles traveled adjustment files are used to modify travel demand model predicted vehicle miles traveled to match the vehicle miles traveled reported in the Highway Performance Monitoring System.  The use of updated predicted levels of vehicle miles traveled and an updated allocation of vehicle miles traveled between the various vehicle types for the North Jersey Transportation Planning Authority region caused VOC and NOx emission predictions to increase by amounts greater than can be reduced by the Metropolitan Planning Organization changing transportation projects.  An additional change in planning assumptions was an update of the vehicle age distribution for light duty gasoline cars and trucks that had a net effect of generally reducing the age of these vehicle classes.  This caused significant reductions in the forecasting of both VOC and NOx emissions because vehicle emission standards become more stringent over time.

The overall result of all planning assumption changes was a reduction in the VOC budgets but an increase in the NOx budgets.  The changes in the budgets are less than the amounts that would cause New Jersey emissions to no longer support predicted achievement of projected attainment of the 1-hour ozone NAAQS by the attainment date as shown in Section II.B.1.i.b.  Even though the 1-hour ozone standard is revoked, the USEPA requires the 1-hour ozone budgets to be used until the 8-hour ozone standard budgets are established as part of the attainment demonstration due in 2007.  The computer files used to generate the updated VOC and NOx budgets are contained in Appendix C.

The budgets for Ocean County are provided in Appendix C where breakouts by county are reported.  The North Jersey Transportation Planning Authority needs to breakout Ocean County in its conformity determinations because it is part of the Philadelphia/Wilmington/Atlantic City 8-hour ozone nonattainment area while the other North Jersey Transportation Planning Authority counties are part of the New York City/Northern New Jersey/Long Island 8-hour ozone nonattainment area.

- Effect of Updated Transportation Conformity Emission Budgets on the Carbon Monoxide Maintenance Plans

Actual measured carbon monoxide concentrations have been steadily decreasing over the last thirty years as a likely consequence of improvements in motor vehicle emission technology.  Onroad mobile sources have historically contributed the largest portion of the carbon monoxide inventory in New Jersey relative to the other source sectors.  The last time the carbon monoxide NAAQS was exceeded in New Jersey was in January of 1995.  The monitored carbon monoxide levels have trended downward so that the maximum measured values are currently about one-half of the NAAQS level as shown in Figure II.

Prior and proposed updated carbon monoxide budgets for the New Jersey portion of the New York City/Northern New Jersey/Long Island carbon monoxide maintenance area were provided in Table II.  The only revision performed to the prior carbon monoxide budgets was the inclusion of safety margins calculated from the USEPA approved maintenance plan.  Safety margins were incorporated so that the updated carbon monoxide transportation conformity emission budgets represent constraints on emissions from onroad sources that will maintain attainment of the carbon monoxide NAAQS without being overly restrictive of the transportation project planning process.  The updates to the carbon monoxide budgets do not affect the conclusions of the carbon monoxide maintenance plan for the New York City/Northern New Jersey/Long Island area because they were performed using the inventory estimates from that plan.

- Effect of Updated Transportation Conformity Emission Budgets on the Ozone Attainment Demonstration

It is necessary to assess the impacts on the 1-hour ozone SIP when the 1-hour budgets are updated to ensure that air quality progress is maintained.  In order to perform this comparison, the State's attainment demonstration and the USEPA's subsequent re-analyses of the attainment demonstration were examined in order to extract mobile onroad inventories which best represent conditions in both the base year and the attainment year.  Inventories for both of these years are needed because the weight of evidence method was used to demonstrate attainment.  A detailed description of the weight of evidence method is available in the 2003 MOBILE6 SIP revision.  The determination of whether or not attainment is still demonstrated depends on the relative reduction of the ozone precursors between the base year and the attainment year.  If these relative reductions with the updated inventories (consistent with the updated budgets) are equal to or greater than the relative reductions with the previous inventories (representative of the attainment demonstrations), then attainment continues to be demonstrated.

Inventories from the recent Rate of Progress SIP were used to determine the required percent reduction in ozone precursors in order to achieve attainment by the attainment date of 2007.  The onroad mobile source inventories from the Rate of Progress SIP are used for this attainment analysis because they are the most recent SIP-quality inventories prepared that include essentially all of the control measures anticipated for the areas to achieve attainment.  In addition, the Rate of Progress SIP inventories were prepared for the 1996 base year as well as the attainment year for the nonattainment area.  These inventories are the best available representations of the base year and attainment year inventories that formed the basis for the 1-hour ozone attainment demonstration so they are the appropriate inventories to use for this attainment analysis.

The results of the comparisons between the previous inventories (from the Rate of Progress SIP) and the updated inventories are summarized in Table VI, which presents the relative reductions (expressed as percent reductions) in onroad mobile source ozone precursor inventories between the base year and the attainment year.  The differences in percent reductions are shown between the previous inventories and the updated inventories.  The updated inventories for 1996 were the same as those established in the 2003 MOBILE6 SIP except that the updated vehicle miles traveled-by-vehicle-type file was used.   The updated vehicle miles traveled-by–vehicle-type data was used for both the base year and projection year runs because this change in planning assumptions represents an improvement in the methodology of estimating local conditions as opposed to a change in the actual conditions.  This contrasts with the update to the vehicle miles traveled adjustment and vehicle age distribution data that only applies to projection year runs and is not used for the 1996 run.

As a result of the use of the weight of evidence method for demonstration of attainment, increases in percent reductions mean that the updated inventories predict lower ozone precursor emissions in the attainment year relative to the base year.  Similarly, decreases in percent reductions mean that the updated inventories predict higher ozone precursor emissions in the attainment year relative to the base year.  In Table VI, the magnitude that the ozone precursor emissions are lower or higher are represented by the calculated "increase" or "decrease", respectively.

Table VI

Comparison of the Onroad Previous Inventories to the Updated Onroad Inventories

(Tons Per Ozone Day Unless Designated Otherwise)

|  | New Jersey Portion of the New York City/Northern New Jersey/Long Island Area(1) - 2007 Attainment Year - |  |
| --- | --- | --- |
|  | VOC | NOx |
| (1) Previous Inventory - 1996 (2) Previous Inventory - Attainment Year | 206.52 89.82 | 302.92 165.11 |
| (3) Previous Reductions [(1) – (2)] (4) Previous % Reductions [(3) / (1) x 100%] | 116.70 56.51% | 137.81 45.49% |
| (5) Updated - 1996 (6) Updated - Attainment Year | 306.50 119.29 | 439.88 245.87 |
| (7) Updated - Reductions [(5) – (6)] (8) Updated - % Reductions [(7) / (5) x 100%] | 187.21 61.08% | 194.01 44.11% |
| (9) Difference in % Reductions [(8) – (4)] Note that positive values represent increases in percent reductions. | +4.57% | -138% |
| (10) Emission reduction increase (+) or decrease (-) in tons per day(2) [((9) / 100%) x (5)] | +14.01 | -6.09 |

NOTES:

- This area includes all of the North Jersey Transportation Planning Authority counties except for Warren County.  These emissions differ from the transportation conformity budgets because the budgets include all of the North Jersey Transportation Planning Authority counties including Warren County.

- The "increase" or "decrease" was calculated by multiplying the differences in percent reductions by the 1996 updated inventories.  These "increases" and "decreases" are calculated only for the purpose of demonstrating if the updated inventories continue to meet the objectives of the attainment demonstration.

The results summarized in Table VI indicate that the updated inventories predict that VOC emissions in the attainment year relative to the base year are lower, i.e., an “increase” of 14.01 tons per day. However, the updated inventories predict that NOx emissions in the attainment year relative to the base year are higher, i.e., a “decrease” of 6.09 tons per day.  In order to evaluate the net effect of these changes, a means of substitution of VOC reductions with NOx reductions is needed. The following substitution methodology was developed based on the methodology developed in a prior SIP revision by New Jersey that was approved by the USEPA.

42 U.S.C. §7511a. allows for the substitution of VOC emission reductions with NOx emission reductions. Consistent with the USEPA guidance on NOx substitution, for substitution to be acceptable it must be demonstrated that such substitution yields equivalent ozone results. The air quality modeling in New Jersey’s Phase I Ozone SIP provided such an equivalency demonstration for the nonattainment area. In order to make the substitution in this case, a VOC to NOx emissions ratio for the entire Northern New Jersey/New York City/Long Island Nonattainment Area was calculated. This calculation is presented in Appendix C.  First, the VOC and NOx emission inventories for the counties from the various states in the nonattainment area were totaled.  A VOC to NOx ratio of 1.29 was derived from these data, i.e., 1 ton of NOx emission reduction is equivalent to 1.29 tons of VOC in terms of ozone reduction.

Since 1 ton of NOx emission reduction is equivalent to 1.29 tons of VOC in terms of ozone reduction, the 6.09 tons per day of NOx can be offset by 7.86 tons per day of the VOC ((6.09 tons per day NOx ) X (1.29) = 7.86 tons per day VOC). This leaves a net VOC reduction increase of 6.15 tons per day (14.01 tons per day – 7.86 tons per day = 6.15 tons per day). The remaining VOC emissions indicate that New Jersey’s attainment demonstration for the nonattainment area remains valid when the current updates to the on-road modeled emission projections are completed.

The Rate of Progress demonstrations for the New Jersey portion of the New York City/Northern New Jersey/Long Island nonattainment area also remain valid because the projected increases in the onroad inventories due to the change in planning assumptions are orders of magnitude below the amounts that would result in failure to meet the Rate of Progress targets.  According to the Rate of Progress SIP, VOC emissions would have to be higher by 354.06 tons per day in 2005 or 314.05 tons per day in 2007 before Rate of Progress targets are missed.

Based on New Jersey's update of its proposed transportation conformity emission budgets, the New Jersey portion of the New York City/Northern New Jersey/Long Island nonattainment area is still predicted to achieve Rate of Progress targets and attainment of the 1-hour ozone standard by the attainment date of 2007. It should be noted that these increases in percent emission reductions could not be reallocated to cover potential emission shortfalls in other areas without a more rigorous reassessments.

- Amendment to the McGuire Air Force Base Conformity Budget

An emission budget was established for McGuire Air Force Base under the General Conformity Rule in order to ensure that any increases in activity at McGuire Air Force Base conform to the 1-hour ozone SIP.  Emission budgets for VOC and NOx were established for 1990, 1996 and 1999 in cooperation with the United States Air Force.,  In 2001, the general conformity emission budgets for McGuire Air Force Base were extended to 2002 and 2005.  In 2003, the general conformity emission budgets for McGuire Air Force Base were updated to accommodate additional aircraft activity.

McGuire Air Force Base has had success over the years in reducing emissions through the implementation of pollution prevention measures.  Their efforts have been more successful in reducing VOC emissions than in reducing NOx emissions.  McGuire Air Force Base has utilized aircraft retirement, mission changes, fuel replacement and maintenance measures to reduce VOCs.  McGuire Air Force Base continues to enhance its pollution prevention measures.  McGuire Air Force Base uses the Hazardous Material Pharmacy, which enhances the management of VOC containing materials used on base.  McGuire Air Force Base provides education and training on the proper use of materials, conducts audits, and utilizes product substitution and consolidation to reduce VOCs.  In addition, the anticipated retirement of additional aircraft will further reduce VOC emissions.

McGuire Air Force Base is anticipating expansion of their overall mission, including anticipated base realignment and closure actions and other mission changes.  In addition, it is critical for McGuire Air Force Base to have operational flexibility in order to meet its mission as well as the future missions of the Department of Defense. Since McGuire Air Force Base has calculated that they will need an increased NOx budget to be in such a position, McGuire Air Force Base has requested a change in their general conformity budgets.  The current and projected VOC emissions at McGuire Air Force Base are well below budget levels.  Therefore, this request for an increase in the NOx budget will be offset by a decrease in their VOC budget.

Such a change in McGuire Air Force Base’s general conformity budgets is an acceptable air quality solution, as this proposed change is consistent with the USEPA policy of substitution of ozone precursor emission reductions. In a previous budget revision for McGuire Air Force Base, a substitution ratio of 1.04 tons of VOC for every ton of NOx was established.  Air quality modeling conducted for the Philadelphia airshed was used to develop the substitution ratio for the McGuire area.  This approach was approved by the USEPA.

McGuire Air Force Base holds a vital status in the national defense.  Mission responsibilities include the movement of troops, passengers, military equipment, cargo and mail, and aerial refueling.  The mission of McGuire Air Force Base carries its aircrews and aircraft throughout more than fifty countries around the globe on an around-the-clock basis.  With peacetime taskings serving as training for wartime requirements, the base is continually postured in a state of preparedness.  Approval of the SIP emission budgets change would enhance the base’s ability to meets its overall mission.

The State has agreed to propose McGuire Air Force Base’s request for a general conformity budget change.  The NOx budgets are being increased by 450 tons per year and the VOC budgets are being decreased by 468 tons per year (Table VII).

Table VII

Emission Budgets for McGuire Air Force Base

|  | #### Prior Budget |  | Proposed Updated Budget |  |
| --- | --- | --- | --- | --- |
|  | VOC (Tons/Year) | NOx (Tons/Year) | VOC (Tons/Year) | NOx (Tons/Year) |
| #### 1990 Baseline | 1,112 | 1,038 | 1,112 | 1,038 |
| 1996 | 1,186 | 1,107 | 1,186 | 1,107 |
| 1999 | 1,223 | 1,142 | 1,223 | 1,142 |
| 2002 | 1,405 | 875 | 1,405 | 875 |
| 2005(1) | 1,198 | 1,084 | 730 | 1,534 |

NOTES:

- Budgets updated such that the increase in NOx is offset by a decrease in VOC such that there is no expected net increase in ozone formation.  Updated 2005 budgets apply to 2005 and all future years until new budgets are established for the 8- hour ozone attainment demonstration.

- New Budget – Particulate Matter Budget for Northern New Jersey

The establishment of early PM2.5 budgets is to require their use by the Metropolitan Planning Organizations in their transportation conformity determinations during the interim period prior to the PM2.5 attainment demonstration SIP budgets.  Establishment of these early transportation conformity emission budgets will enable the affected Metropolitan Planning Organizations to demonstrate transportation conformity by the preferred method of remaining below area specific budgets instead of using one of the interim emissions tests.  The early budgets are based on direct PM2.5 and annual NOx inventories that demonstrate at least a five to ten percent reduction between 2002 and 2009 for sources in the New Jersey portion of the New York /New Jersey/Long Island/Connecticut nonattainment area.

The early budgets must be used for future transportation conformity determinations by the North Jersey Transportation Planning Authority and the Delaware Valley Regional Planning Commission once approved by the USEPA.  The North Jersey Transportation Planning Authority and the Delaware Valley Regional Planning Commission are two of the three Metropolitan Planning Organizations that cover the State of New Jersey (see Figure I).

- PM2.5 General Background

Particulate matter is the term for particles found in the air, including dust, dirt, soot, smoke, and liquid droplets.  Many manmade and natural sources emit particulate matter directly or emit other pollutants that react in the atmosphere to form particulate matter.  Sources of fine particles include all types of combustion activities (motor vehicles, power plants, wood burning, etc.) and certain industrial processes. Other particles may be indirectly formed when gases from burning fuels react with sunlight and water vapor.

Particles less than 10 micrometers in diameter (PM10) pose a health concern because they can be inhaled into and accumulate in the respiratory system.  Particles less than 2.5 micrometers in diameter, PM2.5, are referred to as "fine" particles and are believed to pose the greatest health risks.  Because of their small size (approximately 1/30th the average width of a human hair), fine particles can lodge deeply into the lungs.  Particles with diameters between 2.5 and 10 micrometers are referred to as "coarse."

Health studies have shown a significant association between exposure to fine particles and premature death.  Other important effects include aggravation of respiratory and cardiovascular disease (as indicated by increased hospital admissions, emergency room visits, absences from school or work, and restricted activity days), lung disease, decreased lung function, asthma attacks, and certain cardiovascular problems such as heart attacks and irregular heart beat.  Individuals particularly sensitive to fine particle exposure include older adults, people with heart and lung disease, and children.  Roughly one out of every three people in the United States is at a higher risk of experiencing PM2.5 related health effects: active children because they often spend a lot of time playing outdoors and their bodies are still developing and oftentimes the elderly population is at risk.

- Establishment of the PM2.5 Standard

42 U.S.C §7409 requires the USEPA to set national ambient air quality standards (NAAQS) for widespread pollutants from numerous and diverse sources that are considered harmful to public health and the environment.  These standards are established to protect the most sensitive individuals.  The Clean Air Actestablished two types of NAAQS.  Primary standards set limits to protect public health, including the health of "sensitive" populations such as asthmatics, children, and the elderly.  Secondary standards set limits to protect public welfare, including protection against visibility impairment and damage to animals, crops, vegetation, and buildings.  The Clean Air Act requires periodic review of the science upon which the standards are based and the standards themselves.  The USEPA has set NAAQS for six criteria pollutant. These criteria pollutants are carbon monoxide, lead, nitrogen dioxide, particulate matter, ozone, and sulfur oxides.

In July of 1997, the USEPA revised the primary (health-based) particulate matter standards by promulgating two PM2.5 NAAQS: a 24-hour PM2.5 standard set at 65 micrograms per cubic meter (µg/m3) (24-hour average) and an annual PM2.5 standard set at 15 µg/m3 (annual arithmetic mean).  The USEPA added these new PM2.5 standards while retaining the existing annual PM10 standard of 50 µg/m3 and adjusting the PM10 24-hour standard of 150 µg/m3 by changing the form of the standard.  The ambient air quality in New Jersey continues to meet the PM10 NAAQS and is designated as attainment.

After the USEPA promulgated the PM2.5 and 8-hour ozone standards in July 1997, several industry organizations and state governments challenged USEPA's action in the U.S. Court of Appeals for the District of Columbia Circuit (the D.C. Circuit). After several years of legal challenges, some of which were heard by the United States Supreme Court, the D.C. Circuit issued its decision on March 26, 2002 rejecting the claim that the USEPA had acted arbitrarily and capriciously in setting the levels of the standards.  This last decision by the D.C. Circuit gave the USEPA a clear path to move forward with implementation of the PM2.5 standards.

The Clean Air Act (42 U.S.C. §7409(d)) requires the USEPA to periodically review air quality standards to ensure they provide adequate health and environmental protection and to update those standards if necessary.  The USEPA has proposed revisions to strengthen the current PM2.5 standards as well as proposed a standard for reducing inhalable coarse particles (particles between 2.5 and 10 micrometers).  The USEPA has established a tentative timeline for this process that includes November of 2006 as the effective date of the revised/new standards, July of 2013 as the effective date of designations, and July of 2018 as the prospective attainment date.

- Designation of PM2.5 Nonattainment Areas in New Jersey

The USEPA promulgated the NAAQS for PM2.5 in July of 1997.  According to the Clean Air Act, states and tribes are required to submit recommendations for designations no later than one year after a NAAQS has been revised or newly promulgated.  The USEPA is then required to designate areas across the country within two years following the promulgation of the NAAQS.  USEPA may extend the time period for making designations by up to one additional year if the USEPA lacks sufficient information to make the designations.

The USEPA issued final area designations for PM2.5 on December 17, 2004.  The affected New Jersey counties are shown in Figure III.  Designations became effective on April 5, 2005.  Transportation conformity for PM2.5 becomes effective on April 5, 2006.  This is because there is a one-year grace period from the effective date of designations before transportation conformity applies for that standard.

States with designated PM2.5 nonattainment areas are required to develop a SIP and submit it to the USEPA within three years of designation or April of 2008.  This plan must include enforceable measures for reducing air pollutant emissions leading to the formation of fine particles in the atmosphere.  The plan must also provide steps for the area to attain the PM2.5 standard as quickly as possible.  The USEPA has recently issued a proposed PM2.5 implementation rule to provide further guidance on what should be included in PM2.5 plans.  State plans need to demonstrate that the nonattainment area will attain the standards "as expeditiously as practicable."

- Transportation Conformity for PM2.5

The Transportation Conformity Rules that established the criteria and procedures relating to transportation conformity for PM2.5 were promulgated by the USEPA on July 1, 2004.  Before a SIP budget is available, either through an adequacy finding or approval by the USEPA, conformity of the transportation plan, transportation improvement program, or project not from a conforming plan is demonstrated with the interim emissions tests. The interim emissions tests for PM2.5 are either the baseline year test or the build/no-greater-than-no-build test.

The baseline year test is passed when the emissions from the proposed transportation system are either less than or no greater than 2002 motor vehicle emissions in a given nonattainment area.  Conformity is demonstrated with the build/no-greater-than-no-build test if emissions from the proposed transportation system (“build” or “action” scenario) are less than or equal to the emissions in the same future analysis year from the existing transportation system (“no-build” or “baseline” scenario).  The Metropolitan Planning Organizations in PM2.5 nonattainment areas must utilize either the baseline year test or the build/no-greater-than-no-build test until the budgets are found adequate by the USEPA. Regardless of the test performed, conformity analyses are subject to final approval by the United States Department of Transportation.

Four transportation related PM2.5 precursors – NOx, VOCs, SOx, and NH3 – must be considered in the conformity process in PM2.5 nonattainment areas.  The USEPA requirements for the consideration of PM2.5 precursors are:

- Regional emissions analysis must include NOx as a PM2.5 precursor in all PM2.5 nonattainment areas, unless the head of the state air agency and the USEPA Regional Administrator make a finding that NOx is not a significant contributor to the PM2.5 air quality problem in a given area.

- Regional emissions analyses are not required for VOC, SOx or NH3 before an approved SIP budget for such precursors is established, unless the head of the state air agency or the USEPA Regional Administrator makes a finding that onroad emissions of any of these precursors is a significant contributor.

Figure III

The following criteria are considered in making significance or insignificance findings for PM2.5 precursors:

- The contribution of onroad emissions of the precursor to the total 2002 baseline SIP inventory;

- The current state of air quality for the area;

- The results of speciation monitoring for the area;

- The likelihood that future motor vehicle control measures will be implemented for a given precursor; and,

- Projections of future onroad emissions of the precursor.

The proposed early budgets for PM2.5 precursors include the establishment of an annual NOx budget. The State will study the significance of the other PM2.5 precursors in the attainment demonstration SIP, due to be submitted in April 2008. Studying the significance of these precursors with the attainment demonstration will enable the State to utilize the best available information, monitoring data, inventory data and modeling data in its determinations.

- Early PM2.5 Transportation Conformity Emission Budgets

- PM2.5 and Annual NOx Inventories for 2002 and 2009

To establish an early PM2.5 budget, the USEPA requires that:

In reference to the voluntary SIP that includes early budgets, the preamble states:

"To be approvable, such a SIP would have to include inventories for all source sectors and meet other SIP requirements.  While these early SIPs would have to show some progress toward attainment, it is not a requirement that all of the reductions would come from onroad mobile vehicles."

The term "some progress toward attainment" has been interpreted by the USEPA to mean that the total 2009 inventories for direct PM2.5 and annual NOx to be less than the 2002 values by at least five to ten percent.  Annual NOx is the only PM2.5 precursor for which a budget is being established at this time.  The additional information and data on PM2.5 precursors that will be available during the preparation of the PM2.5 attainment demonstration may result in the establishment of budgets for additional or different specific precursors. The five percent to ten percent criteria is  met for the New Jersey portion of the New York/New Jersey/Long Island /Connecticut PM2.5 nonattainment area but is not met for the New Jersey portion of the Philadelphia/Wilmington PM2.5 nonattainment area. Preliminary results show a small increase in overall direct PM2.5 emissions between 2002 and 2009 for the New Jersey portion of the Philadelphia/Wilmington PM2.5 nonattainment area. Therefore, early PM2.5 budgets are being established for the New Jersey portion of the New York/New Jersey/Long Island /Connecticut PM2.5 nonattainment area only.

Table VIII shows the results of the 2002 and 2009 direct PM2.5 inventory (with anticipated controls in place) by source type for the New Jersey counties in the New York /New Jersey/Long Island/Connecticut PM2.5 nonattainment area.  Emissions from controlled stationary and area sources are projected to increase by nine and three percent respectively, for a total increase of 438 tons per year.  The increase is projected to be more than offset by projected decreases in PM2.5 emissions from onroad and nonroad mobile sources, by a total of 1,343 tons per year, for an overall decrease of 905 tons per year by 2009; this represent an overall 6.5% reduction in direct PM2.5 emissions, see Table IX. Thus the area meets the USEPA’s criteria for “progress toward attainment” for direct PM2.5 emissions.

Table VIII

Direct PM2.5 Emission Inventories for 2002 and 2009 for the New Jersey Portion of the

New York /New Jersey/Long Island/Connecticut PM2.5 Nonattainment Area

|  | DIRECT ANNUAL PM2.5 CONTROLLED EMISSIONS (TONS PER YEAR) (1) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AREA |  |  | NONROAD |  |  | STATIONARY |  |  | ONROAD |  |  |
| COUNTY | 2002 | 2009 | (2009-2002) | 2002 | 2009 | (2009-2002) | 2002 | 2009 | (2009-2002) | 2002 | 2009 | (2009-2002) |
| BERGEN | 537 | 569 | +32 | 478 | 419 | -59 | 149 | 183 | +34 | 376 | 214 | -162 |
| ESSEX | 411 | 436 | +25 | 393 | 341 | -51 | 185 | 222 | +37 | 291 | 163 | -128 |
| HUDSON | 269 | 286 | +16 | 345 | 299 | -45 | 1,077 | 1,085 | +7 | 134 | 76 | -58 |
| MERCER | 530 | 548 | +18 | 203 | 177 | -26 | 188 | 212 | +24 | 141 | 89 | -52 |
| MIDDLESEX | 467 | 497 | +30 | 346 | 299 | -47 | 483 | 553 | +70 | 347 | 207 | -140 |
| MONMOUTH | 981 | 1,002 | +21 | 501 | 426 | -75 | 55 | 66 | +10 | 244 | 145 | -100 |
| MORRIS | 1,284 | 1,297 | +13 | 280 | 251 | -29 | 39 | 45 | +6 | 209 | 126 | -83 |
| PASSAIC | 543 | 554 | +11 | 178 | 151 | -27 | 19 | 22 | +3 | 141 | 81 | -60 |
| SOMERSET | 441 | 452 | +11 | 149 | 131 | -19 | 55 | 60 | +4 | 152 | 88 | -64 |
| UNION | 272 | 289 | +17 | 333 | 291 | -42 | 540 | 589 | +49 | 185 | 108 | -78 |
| TOTAL for New Jersey portion of the NY/NJ/ LI/CT Area | 5,736 | 5,930 | +193 (+3%) | 3,206 | 2,788 | -419 (-13%) | 2,790 | 3,035 | +245 (+9%) | 2,220 | 1,296 | -924 (-42%) |

NOTES:

- In order for the calculated inventory values to more closely match the actual measured levels in New Jersey air quality monitors, the fugitive dust emissions were multiplied by a dust adjustment factor of 20%.  Fugitive dusts are directly released air contaminants that do not pass through an exhaust pipe, stack, flue, vent or chimney.  The main sources of fugitive dusts are dust from paved and unpaved roadways, stock/storage piles, landfill activity, quarry/mining activity, raw material handling, construction and agricultural tilling.

Table IX

Direct PM2.5: Calculation of the Percent Reduction in Projected 2009 Emissions

from the 2002 Emissions by County

|  | % EMISSION REDUCTION | CONTROLLED EMISSIONS ANNUAL (TONS PER YEAR)(1) |  |  |
| --- | --- | --- | --- | --- |
|  |  | TOTAL OF ALL SECTORS |  |  |
| COUNTY | (2009-2002)/2002 | 2002 | 2009 | 2009-2002 |
| BERGEN | -10.0% | 1,540 | 1,385 | -155 |
| ESSEX | -9.2% | 1,280 | 1,162 | -117 |
| HUDSON | -4.4% | 1,825 | 1,746 | -80 |
| MERCER | -3.4% | 1,062 | 1,026 | -36 |
| MIDDLESEX | -5.3% | 1,643 | 1,556 | -87 |
| MONMOUTH | -8.1% | 1,781 | 1,639 | -144 |
| MORRIS | -5.1% | 1,812 | 1,719 | -93 |
| PASSAIC | -8.3% | 881 | 808 | -73 |
| SOMERSET | -8.6% | 797 | 731 | -68 |
| UNION | -4.0% | 1,330 | 1,277 | -54 |
| TOTAL for New Jersey portion of the NY/NJ/ LI/CT Area | -6.5% | 13,952 | 13,049 | -905 |

NOTES:

- In order for the calculated inventory values to more closely match the actual measured levels in New Jersey air quality monitors, the fugitive dust emissions were multiplied by a dust adjustment factor of 20%.  Fugitive dusts are directly released air contaminants that do not pass through an exhaust pipe, stack, flue, vent or chimney.  The main sources of fugitive dusts are dust from paved and unpaved roadways, stock/storage piles, landfill activity, quarry/mining activity, raw material handling, construction and agricultural tilling.

Table X shows the results of the 2002 and projected 2009 NOx inventories by source type for the New Jersey counties in the New York /New Jersey/Long Island/Connecticut PM2.5 nonattainment area.  Emissions from stationary and area sources are projected to increase by seven percent in both categories, for a total increase of 3,698 tons per year.  The increase is projected to be more than offset by projected decreases in NOx emissions from onroad and nonroad mobile sources, by a total of 79,959 tons per year, for an overall decrease of 76,261 tons per year, or about thirty-two percent (32%), by 2009.  Thus the area meets the USEPA’s criteria for “progress toward attainment” for direct NOx emissions.

Table X

Annual NOx Emission Inventories for 2002 and 2009 for the New Jersey Portion

of the New York /New Jersey/Long Island/Connecticut PM2.5 Nonattainment Area

|  | CONTROLLED EMISSIONS ANNUAL (TONS PER YEAR) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SOURCE CATEGORY | AREA |  |  | NONROAD |  |  | STATIONARY |  |  | ONROAD |  |  |
| COUNTY | 2002 | 2009 | (2009-2002) | 2002 | 2009 | (2009-2002) | 2002 | 2009 | (2009-2002) | 2002 | 2009 | (2009-2002) |
| BERGEN | 2,815 | 3,019 | +204 | 6,707 | 5,178 | -1,530 | 988 | 1,189 | +201 | 23,917 | 11,198 | -12,719 |
| ESSEX | 2,436 | 2,621 | +185 | 8,137 | 7,048 | -1,090 | 2,441 | 3,081 | +640 | 16,537 | 7,979 | -8,558 |
| HUDSON | 1,735 | 1,864 | +129 | 5,976 | 5,291 | -685 | 9,674 | 9,970 | +296 | 7,853 | 3,873 | -3,980 |
| MERCER | 1,257 | 1,354 | +97 | 2,427 | 1,898 | -529 | 13,034 | 13,201 | +167 | 8,505 | 4,328 | -4,177 |
| MIDDLESEX | 2,343 | 2,512 | +169 | 4,849 | 3,745 | -1,104 | 3,567 | 4,164 | +597 | 22,147 | 10,871 | -11,276 |
| MONMOUTH | 1,806 | 1,934 | +128 | 4,316 | 3,846 | -470 | 240 | 272 | +31 | 14,860 | 6,973 | -7,887 |
| MORRIS | 1,752 | 1,879 | +127 | 3,151 | 2,417 | -735 | 284 | 337 | +53 | 13,748 | 6,398 | -7,350 |
| PASSAIC | 1,361 | 1,452 | +91 | 2,413 | 1,800 | -613 | 122 | 144 | +22 | 8,748 | 4,164 | -4,584 |
| SOMERSET | 1,048 | 1,121 | +74 | 2,097 | 1,570 | -527 | 313 | 370 | +57 | 9,090 | 4,376 | -4,715 |
| UNION | 1,621 | 1,732 | +111 | 5,883 | 4,903 | -980 | 3,757 | 4,077 | +320 | 12,294 | 5,844 | -6,451 |
| TOTAL for New Jersey portion of the NY/NJ/ LI/CT Area | 18,173 | 19,488 | +1,314 (+7%) | 45,957 | 37,694 | -8,262 (-18%) | 34,420 | 36,804 | +2,384 (+7%) | 137,701 | 66,004 | -71,697 (-52%) |

Table XI is a comparison of total NOx emissions for 2002 and 2009 by source sector for the New Jersey counties in the New York /New Jersey/Long Island/Connecticut PM2.5 nonattainment area. Annual NOx emissions are projected to be lower in each county and across the entire New Jersey portion of the New York /New Jersey/Long Island/Connecticut PM2.5 nonattainment area by 17.6 percent to 41.7 percent with an average of over 30 percent.  Based on these annual NOx inventories, the New Jersey portion of the New York /New Jersey/Long Island/Connecticut area meets the USEPA criteria of a reduction of at least five to ten percent to allow the area to be considered for the establishment of early PM2.5 budgets in a voluntary SIP.

Table XI

Annual NOx: Calculation of the Percent Reduction in Projected 2009 Emissions

from the 2002 Emissions by County

| SOURCE CATEGORY | % EMISSION REDUCTION | CONTROLLED EMISSIONS ANNUAL (TONS PER YEAR) |  |  |
| --- | --- | --- | --- | --- |
|  |  | TOTAL OF ALL SECTORS |  |  |
| COUNTY | (2009-2002)/2002 | 2002 | 2009 | 2009-2002 |
| BERGEN | -40.2% | 34,427 | 20,584 | -13,843 |
| ESSEX | -29.9% | 29,551 | 20,729 | -8,822 |
| HUDSON | -16.8% | 25,238 | 20,998 | -4,240 |
| MERCER | -17.6% | 25,223 | 20,781 | -4,442 |
| MIDDLESEX | -35.3% | 32,906 | 21,292 | -11,614 |
| MONMOUTH | -38.6% | 21,222 | 13,025 | -8,197 |
| MORRIS | -41.7% | 18,935 | 11,031 | -7,904 |
| PASSAIC | -40.2% | 12,644 | 7,560 | -5,084 |
| SOMERSET | -40.7% | 12,548 | 7,437 | -5,112 |
| UNION | -29.7% | 23,555 | 16,556 | -7,000 |
| TOTAL for the New Jersey portion of the NY/NJ/ LI/CT Area | -32.3% | 236,251 | 159,990 | -76,261 |

The New Jersey portion of the New York /New Jersey/Long Island/Connecticut PM2.5 nonattainment area meets the USEPA criterion for progress towards attainment and is eligible for establishing an early PM2.5 transportation budget. The calculation methodologies used for the emissions are found in Appendix C.

- Early Transportation Conformity Emission Budgets for PM2.5 and Annual NOx

The early direct PM2.5 and annual NOx transportation conformity emission budgets are provided in Table XII.  These budgets must be used for future transportation conformity determinations by the Metropolitan Planning Organizations once the USEPA finds them adequate or approves them.

Table XII

Transportation Conformity Emission Budgets

| Transportation ### Planning Area | Direct PM2.5 Emissions(1) (tons per year) | Annual NOx Emissions (tons per year) |
| --- | --- | --- |
|  | 2009 | 2009 |
| North Jersey Transportation Planning Authority(2) | 1,207 | 61,676 |
| Delaware Valley Regional Planning Commission(3) | 89 | 4,328 |

NOTES:

- Direct PM2.5 consists of the sum of: SO4, Organic Carbon, Elemental Carbon, particulate matter from Gasoline Vehicles, Lead, Brake particles and Tire particles.

- For Bergen, Essex, Hudson, Middlesex, Monmouth, Morris, Passaic, Somerset and Union counties.

- For Mercer County.

- Carbon Monoxide Limited Maintenance Plan for Camden County and the Nine Not-Classified Areas

In 1995, the State demonstrated to the USEPA that Camden County and the nine not-classified areas were in attainment of the carbon monoxide health standard by submitting an Attainment Demonstration and Maintenance Plan.  The USEPA subsequently approved the State’s Plan. This SIP revision proposes the second ten (10) year maintenance plan for these areas.

Attainment and maintenance of the carbon monoxide health standard represents a significant health benefit to the citizens of New Jersey.  Carbon monoxide has significant health effects when present in levels above the standard.  An odorless, colorless gas, carbon monoxide is readily absorbed by the body through the lungs and can reduce the amount of oxygen that reaches the heart, brain, and other tissues.  Exposure to elevated carbon monoxide levels has been linked to adverse health effects and can be especially harmful to children, people with heart disease, and pregnant women.  At moderate levels, carbon monoxide exposure has been linked to symptoms such as dizziness, nausea, fatigue, poor vision and concentration, headaches, and heart pains.  Exposure to high levels of carbon monoxide may result in unconsciousness and death.

This SIP revision proposes a consolidated Maintenance Plan for ten of New Jersey’s eleven existing carbon monoxide maintenance areas that demonstrates continued compliance with the carbon monoxide health-based standard, describes how the State will continue to maintain the carbon monoxide NAAQS until the year 2017 in those areas, and provides a contingency plan that would be implemented should the State ever again violate the carbon monoxide NAAQS in those areas. The history of New Jersey’s previous Carbon Monoxide SIP revisions is included as Appendix A.

- Background

The Clean Air Act, 42 U.S.C. §7401 et seq. requires all areas of the nation to attain and maintain compliance with the NAAQS.  These NAAQS are designed to protect public health and welfare from specific pollutants.  For carbon monoxide, there are two primary NAAQS: an average 1-hour standard of 35 parts per million and a non-overlapping average 8-hour standard of 9 parts per million.

Carbon monoxide concentrations in New Jersey have not exceeded the 1-hour standard since the late 1970s.  Typical 1-hour maximum concentrations in New Jersey in recent years have been less than 7 parts per million, well below the 35 parts per million level.  The last exceedance of the 8-hour carbon monoxide NAAQS was in 1995.  Typical 8-hour carbon monoxide levels are less than five parts per million. New Jersey’s noncompliance with the 8-hour carbon monoxide NAAQS prior to 1996 was due primarily to highway sources and was limited to specific areas during stagnating meteorological conditions.  A monitoring site is in violation of the 8-hour standard if it experiences two or more exceedances of the 9 parts per million standard within any calendar year.

Based on prior violations of the 8-hour carbon monoxide standard, New Jersey had eleven nonattainment areas, all of which have since been redesignated to attainment and are currently considered maintenance areas.  New Jersey’s three 8-hour carbon monoxide maintenance plans cover the following areas of the State:

- Camden County – All of Camden County

- Nine Not-Classified Areas - the City of Atlantic City (in Atlantic County), the City of Burlington (in Burlington County), the Borough of Freehold (in Monmouth County), the Town of Morristown (in Morris County), the Borough of Penns Grove (in Salem County), the City of Perth Amboy (in Middlesex County), the Borough of Somerville (in Somerset County), the Toms River Area (in Ocean County), and the City of Trenton (in Mercer County)

- Northeastern New Jersey - Hudson, Essex, Bergen and Union Counties, and the municipalities of Clifton, Passaic and Paterson in Passaic County. This area is part of the New York City/Northern New Jersey/Long Island carbon monoxide maintenance area.

New Jersey’s 8-hour carbon monoxide maintenance areas are shown in Figure IV.

The Camden County area's classification as a moderate nonattainment area reflected its 1989 design value of 9.7 parts per million.  The 1994 8-hour average design value for Camden County was 6.9 parts per million, well below the standard of 9 parts per million.  No violation of the 8-hour average carbon monoxide standard has occurred in any of the nine not-classified areas since 1986.  The Camden County area and the Nine Not-Classified areas were redesignated by the USEPA as attainment areas in 1996.

New Jersey’s first ten-year maintenance plans (which included contingency measures) for the Camden County area and the Nine Not-Classified areas covered a 12-year period (1995–2007).

New Jersey’s eleventh carbon monoxide nonattainment area was redesignated by the USEPA as an attainment area in 2002.  New Jersey’s attainment demonstration submittal of August 7, 1998, for the Northern area showed that 8-hour average concentrations of carbon monoxide at New Jersey’s monitoring sites in the Northern New Jersey carbon monoxide nonattainment area fell below the standard beginning in 1996.  The first ten-year maintenance plan and contingency measures for this area covered the 12-year period, 2002-2014.

This consolidated Limited Maintenance Plan covers the second follow-on ten-year maintenance plans for the Camden County and Nine Not-Classified Carbon Monoxide Maintenance Areas.  The Northern New Jersey Carbon Monoxide Maintenance Area is not addressed in this Limited Maintenance Plan.  A second ten-year maintenance plan that will cover the years 2015-2024 for the northeastern New Jersey Carbon Monoxide Maintenance Area is expected to be proposed in 2012.  Elsewhere in this proposed SIP revision, the carbon monoxide budgets for the northern New Jersey Carbon Monoxide Maintenance Area are being updated.

- Air Quality Update

Carbon monoxide levels have improved dramatically in New Jersey over the past thirty years and are currently about one-half that of the standard, Figure II.  The last time the carbon monoxide 8-hour NAAQS was exceeded in New Jersey was in January of 1995.  Figure II shows the second highest 8-hour value recorded throughout the monitoring network during each year.

A design value is based on monitored readings used by the USEPA to determine an area's air quality status.  New Jersey’s carbon monoxide design values for the years 2002-2003 (see Table XIII) are all well below 7.65 parts per million, an eligibility requirement to qualify for a Limited Maintenance Plan.

- New Jersey Carbon Monoxide Maintenance Areas

Nine Not-Classified Areas

- Atlantic City

- Burlington City

- Freehold Boro

- Morristown

- Penns Grove

- Perth Amboy

- Somerville (in

- Toms River Area

- Trenton

Figure IV

Table XIII

Design Values for Carbon Monoxide in New Jersey

(8-hour standard - 9 parts per million)

| Monitoring Location | 2002-2003 Design Value (parts per million) |
| --- | --- |
| Ancora S.H. | 0.8 |
| Burlington | 2.5 |
| Camden Lab(1) | 2.1 |
| East Orange | 4.2 |
| Elizabeth | 4.4 |
| Elizabeth Lab | 3.1 |
| Fort Lee(2) | 2.6 |
| Freehold | 2.2 |
| Hackensack | 3.4 |
| Jersey City | 2.9 |
| Morristown | 2.4 |
| Newark Lab(3) | 2.9 |
| Perth Amboy | 2.5 |

Notes:

- Data not available October-December 2003

- Data not available July-August 2002

- Data not available July-December 2003

- The USEPA Guidance and Requirements for Limited Maintenance Plans

42 U.S.C. §7505a requires that, eight years after redesignation of any area as an attainment area, states submit an additional revision of the SIP for maintaining the NAAQS for ten years beyond the initial ten-year maintenance period.  Two of New Jersey’s maintenance plans (the Camden County area and the Nine Not-Classified areas) expire in 2007.  New Jersey is proposing a consolidated Limited Maintenance Plan that would cover all of the second ten-year period for both areas.

The USEPA issued guidance in 1995 describing the eligibility criteria and planning requirements for limited maintenance plans.  The USEPA Region II provided supplemental information specific to New Jersey in correspondence dated January 27, 2005.  USEPA Region II provided additional guidance.

Areas are eligible for limited maintenance plans only if current carbon monoxide design values are at or below 7.65 parts per million (i.e., set at 85 percent of the 8-hour NAAQS of 9 parts per million).  Table XIII demonstrates that this is the case for all of the sites located in the three New Jersey’s carbon monoxide maintenance areas.  In addition to an analysis of monitoring data to demonstrate eligibility, approvable Limited Maintenance Plans must contain the following planning elements:

- Attainment Inventory: New Jersey is required to submit an attainment inventory (summary of wintertime carbon monoxide emissions data by county and sector) to the USEPA that coincides with a year where monitoring data show attainment.  Emission projections for the maintenance period are not required.

- Maintenance Demonstration: The following elements provide adequate assurance of maintenance:

- Continued applicability of Prevention of Significant Deterioration requirements;

- Any control measures already in the SIP.

- SIP Commitments: New Jersey must commit to:

- Maintain a monitoring network to verify attainment through the maintenance period;

- Continue to perform project-level transportation conformity reviews (area wide emission “budget tests” are not required for limited maintenance plans); and,

- Submittal of a full maintenance plan if future design values in an area exceed 7.65 parts per million.

- Contingency Plan: New Jersey must document the measures that will be promptly adopted and implemented if a violation (or exceedance) of the NAAQS occurs during the maintenance period.

- Conformity Determinations: According to the 1995 USEPA guidance document for Limited Maintenance Plans, the Transportation Conformity Rule and the General Conformity Rule apply to nonattainment areas and maintenance areas.,  The guidance document states that emissions budgets in Limited Maintenance Plan areas may be treated as not constraining for purposes of conformity.  This is true for the length of the maintenance periods because it is unreasonable to expect that the area would experience so much growth in that period that a violation of the carbon monoxide NAAQS would result.

Once a Limited Maintenance Plan has been approved for these areas, it will no longer be	 necessary for the Metropolitan Planning Organizations to perform numerical regional analyses to demonstrate transportation conformity for Transportation Plans and Transportation Improvement Programs.  The State is aware that project-level carbon monoxide evaluation of transportation projects (project-level conformity) still needs to be performed in areas with approved Limited Maintenance Plans.  As stated previously, a transportation conformity budget will still be required for the northeastern New Jersey carbon monoxide maintenance area, and as such, a revised budget is proposed as part of this submittal.

- 8-Hour Carbon Monoxide Maintenance Plan

The first ten-year maintenance plans and contingency measures for the Camden County and the nine not-classified carbon monoxide maintenance areas are summarized in Table XIV.

Table XIV

New Jersey Carbon Monoxide Maintenance Plan and Contingency Measure History

| Camden County | Nine Not-Classified Areas |
| --- | --- |
| #### Maintenance Plan Federal Motor Vehicle Control Program Reformulated Gasoline Basic Inspection and Maintenance Program | #### Maintenance Plan Federal Motor Vehicle Control Program Reformulated Gasoline Basic Inspection and Maintenance Program |
| Contingency Measure Enhanced Inspection and Maintenance Program | Contingency Measure Enhanced Inspection and Maintenance Program |
| Triggering Mechanism for Contingency Measure Contingency measure to be implemented as quickly as practicable | Triggering Mechanism for Contingency Measure Contingency measure to be implemented as quickly as practicable |

Each of these elements has been included in New Jersey’s Second Ten-Year Carbon Monoxide Limited Maintenance Plan for Camden County and the nine not-classified areas. This Limited Maintenance Plan consolidates Maintenance Plans for these 10 maintenance areas.

- Attainment Inventory

The USEPA’s Limited Maintenance Plan guidance requires states to develop an attainment emission inventory identifying a level of emissions sufficient to attain the NAAQS.  The inventory should represent “typical winter day” emissions during a time period coincident with monitored data showing attainment.  The USEPA required New Jersey to provide an attainment inventory (point, area, and mobile) for the periodic inventory year 2002.  Table XV provides a summary of the 2002 wintertime summary carbon monoxide emissions data for each sector for each county included in one of the ten maintenance areas.

New Jersey’s carbon monoxide attainment inventory was submitted to the USEPA on September 29, 1995 (1990 base year carbon monoxide emission inventory).  Given the amount of time that has passed since the submittal of New Jersey’s carbon monoxide attainment inventory, New Jersey thought it more appropriate to submit our most recent carbon monoxide inventory values (proposed 2002 inventory) for the purposes of the Limited Maintenance Plan. Since 2002 is a calendar year that had monitoring data that demonstrates attainment, no projection inventories are required over the years of the maintenance period.  A summary of the wintertime carbon monoxide emissions, as required by the USEPA, has been provided in this submittal.  In addition, the State’s entire 2002 base year inventory, which outlines the method and calculation used to develop the carbon monoxide wintertime inventory, is proposed in this submittal.

County level data was used because:

- Estimating emissions from areas smaller than counties would not be statistically significant.

- Much of the activity data upon which the area source and nonroad mobile source inventory estimates are based were developed at the county-level.  Therefore, some proportional adjustment factor would need to be applied to the county-level total emissions for those sectors to represent the selected municipalities.

- The vehicle miles traveled estimates used to develop the onroad mobile source inventory would require similar adjustments.  These adjustments would necessarily be based on population or economic statistical data and, as such, would simply represent a proportion of the county-level estimate rather than specific municipality data.

- All growth and control factors applied to develop the future year inventories are estimated on a county-wide basis.  Therefore, the same proportional change in emissions would be applied to estimate the projected future year inventories regardless of the assumptions used to represent the attainment year base case.

The maintenance areas are then totaled (partial counties are included as full counties for summary purposes).  Please note that these numbers are part of the New Jersey’s 2002 Periodic Emission Inventory, which is proposed elsewhere as a part of this SIP revision.  Details on how the wintertime carbon monoxide numbers for each sector were developed are outlined in the State’s proposed inventory submittal.
                                                                    Table XV

New Jersey Wintertime Carbon Monoxide Emissions for 2002

(tons per winter day)

- Maintenance Demonstration

The 1995 USEPA guidance document states that the maintenance demonstration requirement is considered to be satisfied if the monitoring data show that the area is meeting the air quality criteria for limited maintenance areas (7.65 parts per million or 85 percent of the carbon monoxide NAAQS).  Emission projections for the maintenance period are not required.  According to the guidance document, the USEPA believes that if an area begins a maintenance period at or below 85 percent of the exceedance levels, then the continued applicability of Prevention of Significant Deterioration  requirements, any control measure already in the SIP, and federal measures should provide adequate assurance of maintenance over the second ten-year maintenance period.

Table XIII shows that all of the design values are well below 7.65 parts per million.  As such, New Jersey is not required to include emission projections in the Limited Maintenance Plan.  New Jersey commits to continued implementation of its Prevention of Significant Deterioration program and all other federal and state measures already implemented as part of the carbon monoxide SIP.

- Monitoring Network

To verify that carbon monoxide maintenance areas remain in attainment over the maintenance period, New Jersey will continue to operate an appropriate air monitoring network.  New Jersey’s current carbon monoxide monitoring network is shown in Figure V.  The air monitoring results will detect any changes in the ambient air quality, as well as assist the State in determining whether or not it is necessary to implement any contingency measures.  The State will continue to work with the USEPA through the air monitoring network review process, as required by 40 CFR Part 58, to determine:

- The adequacy of the carbon monoxide monitoring network;

- If additional monitoring is needed; and,

- When monitoring can be discontinued.

These determinations must be consistent with the section 105 air grant process.  Air monitoring data will continue to be quality assured according to the requirements in the USEPA regulations.

- Verification of Continued Attainment

To track the progress of the maintenance plan, the State will review the carbon monoxide concentrations from its monitoring sites each year.  If design values in any maintenance area exceed 7.65 parts per million (the eligibility requirement for Limited Maintenance Plan areas),

Figure V

New Jersey Carbon Monoxide Monitoring Network (2004)

the NJDEP will coordinate with USEPA Region II to:

- Verify the validity of the data;

- Evaluate whether the data should be excluded based on an “exceptional event”; and,

- If warranted based on the data review, develop a full maintenance plan for the affected maintenance area(s), if deemed necessary.

- Contingency Plan

42 U.S.C. §7505a(d) requires that maintenance plans include contingency provisions.  The purpose of the contingency provisions is to assure that any violations of the NAAQS that occur after the redesignation of an area to attainment will be corrected promptly.  The USEPA issued guidance describing the contents of the contingency plan.  This guidance specifies that the contingency plan should clearly identify the measure(s) to be adopted, a schedule and procedure for adoption and implementation, and a specific time limit for action by the State.  The USEPA has also recommended that the State specify triggers that will be used to determine when the contingency measure(s) need to be implemented.  The triggers specified in the previous Maintenance Plan are included in this proposed Limited Maintenance Plan.

Contingency Measure Triggers

If air quality monitoring data indicate that either of the carbon monoxide NAAQS were exceeded, New Jersey will first analyze available data regarding the air quality, meteorology, and related activities in the area to determine the cause of the violation.  After this analysis is complete, if it is determined that the violation was caused by non-local motor vehicle usage (i.e., not due to a local traffic problem, a special event, or stationary sources, and not occurring during the same meteorological episode as the first exceedance), then the State will institute the contingency measures described in this SIP revision.

Contingency Measures and Timeframes

42 U.S.C. §7505a(d) requires that, at a minimum, a contingency plan include reinstatement of all measures that were contained in the SIP before redesignation of the area as an attainment area.  Table XIV outlines the contingency measures from the original maintenance plan for the area.   The plans included implementation of an enhanced I/M program.  This program is fully operational and the State commits to continue to meet the performance standard for an enhanced I/M program in an effort to maintain the carbon monoxide NAAQS.

The State continues to commit to implementing a program to reduce truck idling emissions.  If it becomes necessary to reduce carbon monoxide levels in the future, New Jersey will work with the local Metropolitan Planning Organizations to implement transportation control measures such as Transportation Demand Management measures, arterial and signal improvement projects, bicycle projects, and various transit related projects.  Since the implementation of potential contingency measures would not be expected to take place until well in the future, providing the specific details of the measures is not practicable.  The most appropriate contingency measures may be significantly different from the measures mentioned above due to technological, societal, economic, and political factors that are impossible to predict.

- Transportation Conformity

According to the 1995 USEPA guidance document for Limited Maintenance Plans, the Transportation Conformity Rule and the General Conformity Rule apply to nonattainment areas and maintenance areas operating under maintenance plans.,  The guidance document also states that emission budgets in Limited Maintenance Plan areas may be treated as not constraining for the length of the initial maintenance period because it is unreasonable to expect that such an area would experience so much growth in that period that a violation of the carbon monoxide NAAQS would result.  New Jersey’s consolidated Limited Maintenance Plan covers the second maintenance periods for two of New Jersey’s carbon monoxide maintenance areas.  According to correspondence from USEPA Region II, a budget test (outlined in the Transportation Conformity Rule) is not required for limited maintenance plans.  The NJDEP will comply with the requirement to conduct a project-level carbon monoxide evaluation of transportation projects (project-level conformity).

Consistent with the discussion, the NJDEP will use the interagency consultation process to:

- Inform the New Jersey Department of Transportation and Metropolitan Planning Organizations that, upon approval of the limited maintenance plans, carbon monoxide emission budgets will no longer be constraining for transportation conformity because of the low levels of emissions and expected growth rates during the duration of the limited maintenance periods.  Once the Limited Maintenance Plan is approved, regional transportation conformity is presumed to be satisfied, with no need for quantitative comparisons to budgets for the second ten-year maintenance periods.

- Ensure that project-level carbon monoxide evaluations of transportation projects (i.e., project-level conformity, as described in 40 CFR 93.116) are carried out in each area as part of environmental reviews.

A transportation conformity budget is still required for the Northeastern New Jersey carbon monoxide maintenance area, and as such, a revised budget is being proposed as part of this submittal.

## 2002 Periodic Emission Inventory

##### Background

##### Statutory and Regulatory Background

42 U.S.C. §7410 (a)(2)(F) requires the submission by states to the USEPA of periodic reports on the nature and amounts of emissions and emissions related data. For example, 42 U.S.C. §7511a.(a)(3)(A) required states to submit an emission inventory every three years for 1-hour ozone nonattainment areas beginning in 1993.  The inventories are required to include all ozone precursors including VOCs, NOx, and carbon monoxide.  Similarly, 42 U.S.C. §7512a.(a)(5) required States to submit an inventory every three years for carbon monoxide nonattainment areas for the same source classes as ozone, except biogenic sources. As part of the NOx  SIP Call Rule (40 CFR 51.121), the USEPA established emissions reporting requirements to be included in the SIPs submitted by the affected states.

In 2002, the USEPA promulgated the Consolidated Emission Reporting Rule, 40 CFR Part 51, Subpart A, that

- Consolidated the various emissions reporting requirements that already existed;

- Established new reporting requirements related to PM2.5, its precursors (NH3, SOx, NOx, and VOC) and regional haze;

- Established new requirements for the statewide reporting of area source and mobile source emissions; and,

- Required two types of inventories – annual inventories and three year cycle inventories.

Figures VI and VII represent New Jersey’s nonattainment areas for 8-hour ozone and PM2.5, respectively, and Figure IV represents the maintenance areas for carbon monoxide. The 2002 periodic emission inventory is based on the 8-hour ozone standard (0.08 parts per million) as the 1-hour ozone standard (0.12 parts per million) was revoked by the USEPA on June 15, 2005.

- Emission Inventory Overview

The 2002 Periodic Emission Inventory is a compilation of the emissions from sources of biogenic (natural) and anthropogenic (human-made) volatile organic chemical (VOC), oxides of nitrogen (NOx), carbon monoxide (CO), particulate matter less than 10 micrometers in diameter (PM10), particulate matter five micrometers or less in diameter (PM2.5), sulfur dioxide (SO2), and ammonia (NH3) in the outdoor air.  The sources are divided into five sectors and each making up one component of the inventory: point sources, area (nonpoint) sources, onroad sources, nonroad sources, and biogenic sources.

Figure VI

Figure VII

This report includes the 2002 periodic emission inventory for the parameters listed in Table XVI. Appendix D contains a description of the 2002 periodic emission inventory and the methodologies used for collecting and calculating emission data for New Jersey for 2002.  This appendix also contains twenty-five (25) attachments that provide supplemental information for evaluating the inventory.

### Table XVI

### 2002 Inventories Prepared

|  | Summer Day(1) | Winter Day(2) | Annual Emissions |
| --- | --- | --- | --- |
| VOC | √ |  | √ |
| NOx | √ |  | √ |
| CO | √ | √ | √ |
| PM10 |  | √ | √ |
| PM2.5 |  | √ | √ |
| SO2 |  |  | √ |
| NH3 |  |  | √ |

NOTES:

(1) Average daily emissions for a typical summer work week day.

(2) Average daily emissions for a typical winter work week day.

- Emission Inventory Summary

A summary of the 2002 Periodic Emission Inventory for New Jersey is presented in Table XVII by pollutant and source sector.  Table XVIII presents the inventory data by pollutant and county.  Table XIX presents the inventory data by pollutant, source sector, and county.  A series of figures showing the top fifteen pollutants by Source Classification Code (SCC) for each pollutant and source sector can be found in Appendix D, Attachment 1.

Note, the summary tables, graphs, and the detailed county source sector tables found in the attachments to this report contain adjusted values for fugitive dust.  Discussion on the fugitive dust inventory and the adjustment can be found in Appendix D, Attachment 2.

### Table XVII

### 2002 Statewide Emission Inventory by Source Sector and Pollutant

|  | VOC |  |  | NOx |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Source Sector | Tons per Summer Day | Tons per Year | % of Total Annual Inventory | Tons per Summer Day | Tons per Year | % of Total Annual Inventory |
| Point | 113.15 | 30,169 | 6.41% | 280.36 | 52,121 | 14.77% |
| Area | 369.83 | 127,673 | 27.12% | 35.92 | 26,742 | 7.58% |
| Onroad | 274.74 | 106,589 | 22.65% | 558.66 | 206,280 | 58.44% |
| Nonroad | 220.60 | 70,407 | 14.96% | 231.56 | 66,443 | 18.82% |
| Biogenic | 371.95 | 135,851 | 28.86% | 3.78 | 1,382 | 0.39% |
| Total in State | 1,350.27 | 470,689 |  | 1,110.28 | 352,968 |  |

|  | Carbon Monoxide |  |  | PM10(1) |  |
| --- | --- | --- | --- | --- | --- |
| Source Sector | Tons per Summer Day | Tons per Year | % of Total Annual Inventory | Tons per Year | % of Total Annual Inventory |
| Point | 89.35 | 13,254 | 0.60% | 5,555 | 13.37% |
| Area | 66.45 | 94,067 | 4.26% | 24,760 | 59.61% |
| Onroad | 2,856.37 | 1,421,004 | 64.40% | 4,718 | 11.36% |
| Nonroad | 2,497.80 | 665,944 | 30.18% | 6,505 | 15.66% |
| Biogenic | 34.09 | 12,450 | 0.56% | 0 | 0.00% |
| Total in State | 5,544.06 | 2,206,719 |  | 41,538 |  |

|  | PM2.5(1) |  | SO2 |  | NH3 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Source Sector | Tons per Year | % of Total Inventory | Tons per Year | % of Total Inventory | Tons per Year | % of Total Inventory |
| Point | 4,868 | 16.02% | 61,231 | 64.68% | 38 | 0.15% |
| Area | 16,230 | 53.42% | 10,876 | 11.49% | 8,005 | 31.38% |
| Onroad | 3,361 | 11.07% | 5,793 | 6.12% | 7,469 | 29.27% |
| Nonroad | 5,922 | 19.49% | 16,772 | 17.71% | 970 | 3.80% |
| Biogenic | 0 | 0.00% | 0 | 0.00% | 9,032 | 35.40% |
| Total in State | 30,381 |  | 94,672 |  | 25,514 |  |

NOTE:

- These totals include adjusted emissions from fugitive dust categories. See Attachment 2 for further discussion.

##### Table XVIII

##### 2002 Statewide Emission Inventory by County and Pollutant

|  | VOC |  | NOx |  | Carbon Monoxide |  |
| --- | --- | --- | --- | --- | --- | --- |
| County | Tons per Summer Day | Tons per Year | Tons per Summer Day | Tons per Year | Tons per Summer Day | Tons per Year |
| Atlantic | 74.67 | 27,426 | 33.81 | 9,706 | 231.77 | 85,555 |
| Bergen | 105.32 | 34,106 | 94.16 | 34,452 | 687.72 | 261,862 |
| Burlington | 87.21 | 31,814 | 58.36 | 18,214 | 297.03 | 126,456 |
| Camden | 65 | 22,629 | 41.44 | 15,119 | 270.08 | 107,408 |
| Cape May | 52.34 | 19,481 | 34.5 | 8,636 | 138.02 | 50,041 |
| Cumberland | 54.2 | 19,375 | 30.04 | 7,829 | 112.23 | 40,088 |
| Essex | 68.06 | 22,580 | 89.32 | 29,578 | 377.37 | 152,468 |
| Gloucester | 84.24 | 30,076 | 42.19 | 14,615 | 183.71 | 74,719 |
| Hudson | 46.01 | 15,111 | 95.68 | 25,367 | 167.29 | 67,897 |
| Hunterdon | 28.22 | 10,636 | 32.07 | 8,651 | 122.31 | 50,996 |
| Mercer | 46.45 | 16,068 | 81.81 | 25,295 | 231.18 | 90,194 |
| Middlesex | 104.31 | 34,222 | 123.5 | 33,048 | 554.28 | 212,020 |
| Monmouth | 91.54 | 32,769 | 57.2 | 21,301 | 444.87 | 180,921 |
| Morris | 69.79 | 25,153 | 50.34 | 18,978 | 443.06 | 174,989 |
| Ocean | 103.91 | 37,326 | 38.56 | 13,676 | 314.69 | 125,240 |
| Passaic | 49.71 | 17,014 | 34.46 | 12,682 | 206.71 | 85,648 |
| Salem | 34.63 | 11,726 | 31.01 | 7,727 | 74.94 | 26,533 |
| Somerset | 42.74 | 14,963 | 36.61 | 12,602 | 228.79 | 88,817 |
| Sussex | 34.9 | 14,544 | 10.86 | 4,140 | 84.05 | 43,747 |
| Union | 77.8 | 22,468 | 73.69 | 23,906 | 286.09 | 117,897 |
| Warren | 29.22 | 11,204 | 20.7 | 7,451 | 87.9 | 43,226 |
| Total in State | 1350.27 | 470,689 | 1110.28 | 352,968 | 5544.06 | 2,206,719 |

Table XVIII (cont.)

2002 Statewide Emission Inventory by County and Pollutant

| County | PM10(1) Tons per Year | PM2.5(1) Tons per Year | SO2 Tons per Year | NH3 Tons per Year |
| --- | --- | --- | --- | --- |
| Atlantic | 2,282 | 1,889 | 886 | 823 |
| Bergen | 2,164 | 1,540 | 2155 | 2390 |
| Burlington | 3,209 | 2,362 | 3568 | 1535 |
| Camden | 1,823 | 1,382 | 2038 | 1238 |
| Cape May | 1,468 | 1,254 | 13409 | 334 |
| Cumberland | 1,467 | 1,201 | 3281 | 652 |
| Essex | 1,682 | 1,280 | 4597 | 1934 |
| Gloucester | 2,103 | 1,514 | 7275 | 1006 |
| Hudson | 2,690 | 1,825 | 21653 | 1325 |
| Hunterdon | 1,426 | 908 | 695 | 934 |
| Mercer | 1,613 | 1,062 | 15594 | 1032 |
| Middlesex | 2,561 | 1,643 | 2395 | 2122 |
| Monmouth | 2,520 | 1,781 | 1947 | 1725 |
| Morris | 2,473 | 1,812 | 1529 | 1464 |
| Ocean | 3,091 | 2,341 | 1196 | 1291 |
| Passaic | 1,242 | 881 | 974 | 1126 |
| Salem | 1,234 | 927 | 5504 | 657 |
| Somerset | 1,435 | 797 | 744 | 1092 |
| Sussex | 1,849 | 1,449 | 733 | 674 |
| Union | 1,569 | 1,330 | 3856 | 1467 |
| Warren | 1,629 | 1,205 | 643 | 688 |
| Total in State | 41,538 | 30,381 | 94672 | 25514 |

NOTES:

- These totals include adjusted emissions from fugitive dust categories. See Attachment 2 of this report for further discussion.

### Table XIX

### 2002 Statewide Emission Inventory by County and Source Sector

| County | VOC Tons per Summer Day |  |  |  |  | VOC Tons per Year |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Point Sources | Area Sources | Onroad Sources | Nonroad Sources | Biogenic | Point Sources | Area Sources | Onroad Sources | Nonroad Sources | Biogenic |
| Atlantic | 0.15 | 11.04 | 12.85 | 10.25 | 40.38 | 52 | 5,492 | 3,613 | 3,521 | 14,748 |
| Bergen | 5.72 | 36.86 | 36.09 | 22.05 | 4.60 | 773 | 11,243 | 14,048 | 6,361 | 1,681 |
| Burlington | 4.02 | 17.54 | 15.80 | 10.01 | 39.84 | 927 | 7,057 | 6,278 | 3,000 | 14,552 |
| Camden | 1.23 | 22.68 | 13.80 | 7.23 | 20.06 | 453 | 7,228 | 5,512 | 2,110 | 7,326 |
| Cape May | 0.20 | 5.26 | 4.72 | 22.61 | 19.55 | 39 | 2,474 | 1,348 | 8,480 | 7,140 |
| Cumberland | 0.46 | 8.93 | 5.37 | 11.03 | 28.41 | 102 | 3,208 | 1,492 | 4,196 | 10,377 |
| Essex | 2.95 | 31.53 | 18.26 | 11.92 | 3.40 | 791 | 9,568 | 7,238 | 3,739 | 1,244 |
| Gloucester | 32.01 | 20.39 | 9.10 | 5.91 | 16.83 | 11,560 | 7,032 | 3,650 | 1,686 | 6,148 |
| Hudson | 7.33 | 21.09 | 9.10 | 5.22 | 3.27 | 2,104 | 6,628 | 3,567 | 1,617 | 1,195 |
| Hunterdon | 0.64 | 5.49 | 5.99 | 3.66 | 12.44 | 144 | 2,468 | 2,441 | 1,038 | 4,545 |
| Mercer | 2.13 | 13.06 | 11.60 | 7.01 | 12.65 | 446 | 4,445 | 4,636 | 1,922 | 4,619 |
| Middlesex | 16.08 | 34.87 | 26.00 | 14.58 | 12.78 | 4,366 | 10,594 | 10,478 | 4,115 | 4,669 |
| Monmouth | 1.37 | 24.65 | 22.26 | 21.26 | 22.00 | 287 | 8,477 | 8,973 | 6,996 | 8,036 |
| Morris | 1.27 | 20.81 | 18.87 | 15.09 | 13.75 | 309 | 7,947 | 7,662 | 4,211 | 5,024 |
| Ocean | 0.26 | 24.01 | 14.30 | 21.54 | 43.80 | 76 | 7,746 | 5,792 | 7,714 | 15,998 |
| Passaic | 1.99 | 19.84 | 10.22 | 6.62 | 11.04 | 253 | 6,537 | 4,109 | 2,081 | 4,034 |
| Salem | 4.92 | 3.47 | 4.23 | 3.37 | 18.64 | 1,034 | 1,516 | 1,205 | 1,162 | 6,809 |
| Somerset | 0.73 | 12.29 | 10.65 | 6.87 | 12.20 | 224 | 4,075 | 4,311 | 1,898 | 4,455 |
| Sussex | 0.25 | 5.69 | 4.62 | 3.86 | 20.48 | 38 | 3,656 | 1,881 | 1,490 | 7,479 |
| Union | 26.56 | 25.26 | 15.92 | 7.75 | 2.31 | 5,382 | 7,652 | 6,354 | 2,237 | 843 |
| Warren | 2.88 | 5.07 | 4.99 | 2.78 | 13.50 | 809 | 2,631 | 2,001 | 832 | 4,931 |
| Total in State | 113.15 | 369.83 | 274.74 | 220.60 | 371.95 | 30,169 | 127,673 | 106,589 | 70,407 | 135,851 |

### Table XIX (cont.) – 2002 Statewide Emission Inventory by County and Source Sector

| County | NOx Tons per Summer Day |  |  |  |  | NOx Tons per Year |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Point Sources | Area Sources | Onroad Sources | Nonroad Sources | Biogenic |  | Point Sources | Area Sources | Onroad Sources | Nonroad Sources | Biogenic |  |
| Atlantic | 1.67 | 1.17 | 24.50 | 6.26 | 0.21 |  | 129 | 964 | 6,764 | 1,771 | 78 |  |
| Bergen | 3.64 | 3.83 | 63.24 | 23.38 | 0.07 |  | 988 | 2,815 | 23,917 | 6,707 | 25 |  |
| Burlington | 12.35 | 1.77 | 31.10 | 12.88 | 0.26 |  | 1,273 | 1,424 | 11,644 | 3,776 | 97 |  |
| Camden | 2.69 | 2.10 | 27.00 | 9.44 | 0.21 |  | 776 | 1,523 | 10,074 | 2,669 | 77 |  |
| Cape May | 19.15 | 0.42 | 8.82 | 5.92 | 0.19 |  | 3,819 | 357 | 2,433 | 1,959 | 68 |  |
| Cumberland | 10.50 | 0.65 | 10.61 | 7.94 | 0.34 |  | 1,778 | 469 | 2,883 | 2,574 | 125 |  |
| Essex | 16.18 | 3.31 | 44.06 | 25.70 | 0.07 |  | 2,441 | 2,436 | 16,537 | 8,137 | 27 |  |
| Gloucester | 14.48 | 1.01 | 18.50 | 8.01 | 0.19 |  | 4,645 | 800 | 6,899 | 2,200 | 71 |  |
| Hudson | 51.61 | 2.24 | 21.05 | 20.71 | 0.07 |  | 9,776 | 1,735 | 7,853 | 5,976 | 27 |  |
| Hunterdon | 9.47 | 0.54 | 17.17 | 4.70 | 0.19 |  | 491 | 424 | 6,444 | 1,223 | 69 |  |
| Mercer | 47.87 | 1.72 | 22.70 | 9.32 | 0.20 |  | 13,034 | 1,257 | 8,505 | 2,427 | 72 |  |
| Middlesex | 44.47 | 3.33 | 58.00 | 17.54 | 0.16 |  | 3,651 | 2,343 | 22,147 | 4,849 | 58 |  |
| Monmouth | 0.86 | 2.23 | 38.15 | 15.74 | 0.22 |  | 240 | 1,806 | 14,860 | 4,316 | 79 |  |
| Morris | 1.18 | 2.40 | 35.06 | 11.58 | 0.12 |  | 284 | 1,752 | 13,748 | 3,151 | 43 |  |
| Ocean | 3.68 | 2.39 | 24.65 | 7.57 | 0.27 |  | 395 | 1,507 | 9,538 | 2,138 | 98 |  |
| Passaic | 0.68 | 1.79 | 23.01 | 8.88 | 0.10 |  | 122 | 1,361 | 8,748 | 2,413 | 38 |  |
| Salem | 15.26 | 0.31 | 11.91 | 3.21 | 0.32 |  | 3,267 | 227 | 3,185 | 932 | 116 |  |
| Somerset | 3.60 | 1.44 | 23.85 | 7.57 | 0.15 |  | 313 | 1,048 | 9,090 | 2,097 | 54 |  |
| Sussex | 0.21 | 0.57 | 7.47 | 2.46 | 0.15 |  | 39 | 495 | 2,936 | 615 | 55 |  |
| Union | 18.88 | 2.26 | 32.22 | 20.25 | 0.08 |  | 4,080 | 1,621 | 12,294 | 5,883 | 28 |  |
| Warren | 1.93 | 0.47 | 15.60 | 2.48 | 0.22 |  | 580 | 379 | 5,782 | 631 | 79 |  |
| Total in State | 280.36 | 35.92 | 558.66 | 231.56 | 3.78 |  | 52,121 | 26,742 | 206,280 | 66,443 | 1,382 |  |

### Table XIX (cont.) – 2002 Statewide Emission Inventory by County and Source Sector

| County | Carbon Monoxide Tons per Summer Day |  |  |  |  | Carbon Monoxide Tons per Year |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Point Sources | Area Sources | Onroad Sources | Nonroad Sources | Biogenic | Point Sources | Area Sources | Onroad Sources | Nonroad Sources | Biogenic |
| Atlantic | 0.36 | 2.66 | 155.53 | 70.26 | 2.96 | 66 | 10,726 | 53,885 | 19,798 | 1,080 |
| Bergen | 2.36 | 2.07 | 324.50 | 358.25 | 0.54 | 619 | 1,453 | 166,589 | 93,002 | 199 |
| Burlington | 1.48 | 1.97 | 168.90 | 121.35 | 3.33 | 413 | 9,709 | 83,768 | 31,350 | 1,216 |
| Camden | 3.28 | 6.89 | 145.90 | 112.44 | 1.57 | 1,154 | 3,789 | 72,489 | 29,402 | 574 |
| Cape May | 2.18 | 0.66 | 53.58 | 80.06 | 1.54 | 311 | 4,145 | 18,758 | 26,265 | 562 |
| Cumberland | 1.56 | 1.13 | 56.91 | 50.35 | 2.28 | 126 | 3,196 | 19,994 | 15,941 | 831 |
| Essex | 3.61 | 2.40 | 187.93 | 182.98 | 0.45 | 624 | 1,306 | 96,967 | 53,407 | 164 |
| Gloucester | 3.27 | 1.54 | 99.80 | 77.69 | 1.41 | 1,029 | 4,513 | 49,458 | 19,203 | 516 |
| Hudson | 9.42 | 1.22 | 87.49 | 68.72 | 0.44 | 2,058 | 896 | 44,767 | 20,015 | 161 |
| Hunterdon | 6.43 | 1.03 | 64.94 | 48.31 | 1.60 | 259 | 3,973 | 34,283 | 11,896 | 585 |
| Mercer | 1.51 | 1.37 | 122.70 | 104.18 | 1.42 | 323 | 2,567 | 61,101 | 25,685 | 518 |
| Middlesex | 34.20 | 2.54 | 287.54 | 228.84 | 1.16 | 3,034 | 1,309 | 149,288 | 57,965 | 424 |
| Monmouth | 1.28 | 1.79 | 227.22 | 212.60 | 1.98 | 381 | 5,252 | 118,952 | 55,614 | 722 |
| Morris | 2.24 | 2.35 | 209.14 | 227.91 | 1.42 | 266 | 8,121 | 109,947 | 56,136 | 519 |
| Ocean | 1.21 | 29.78 | 135.96 | 143.85 | 3.89 | 271 | 10,563 | 72,072 | 40,914 | 1,420 |
| Passaic | 0.40 | 1.23 | 105.86 | 98.09 | 1.13 | 68 | 2,985 | 55,414 | 26,769 | 412 |
| Salem | 2.28 | 0.57 | 49.04 | 21.42 | 1.63 | 487 | 2,389 | 17,071 | 5,991 | 595 |
| Somerset | 5.96 | 1.16 | 112.52 | 107.75 | 1.40 | 226 | 2,079 | 59,270 | 26,731 | 511 |
| Sussex | 0.33 | 1.80 | 42.35 | 37.57 | 2.00 | 83 | 8,995 | 23,055 | 10,883 | 731 |
| Union | 3.87 | 1.11 | 162.44 | 118.31 | 0.36 | 1,012 | 794 | 84,178 | 31,780 | 133 |
| Warren | 2.12 | 1.19 | 56.12 | 26.89 | 1.58 | 444 | 5,306 | 29,700 | 7,198 | 578 |
| Total in State | 89.35 | 66.45 | 2,856.37 | 2,497.80 | 34.09 | 13,254 | 94,067 | 1,421,004 | 665,944 | 12,451 |

### Table XIX (cont.) – 2002 Statewide Emission Inventory by County and Source Sector

| County | PM10(1) Tons per Year |  |  |  |  | PM2.5(1) Tons per Year |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Point Sources | Area Sources | Onroad Sources | Nonroad Sources | Biogenic | Point Sources | Area Sources | Onroad Sources | Nonroad Sources | Biogenic |
| Atlantic | 17 | 1,863 | 154 | 248 | NA | 19 | 1,541 | 104 | 225 | NA |
| Bergen | 135 | 981 | 524 | 524 | NA | 149 | 537 | 376 | 478 | NA |
| Burlington | 318 | 2,145 | 275 | 471 | NA | 308 | 1,448 | 193 | 413 | NA |
| Camden | 126 | 1,210 | 238 | 249 | NA | 233 | 754 | 167 | 228 | NA |
| Cape May | 102 | 799 | 58 | 509 | NA | 109 | 637 | 40 | 468 | NA |
| Cumberland | 266 | 721 | 73 | 407 | NA | 280 | 495 | 52 | 374 | NA |
| Essex | 203 | 646 | 389 | 444 | NA | 185 | 411 | 291 | 393 | NA |
| Gloucester | 531 | 1,169 | 161 | 242 | NA | 426 | 754 | 112 | 222 | NA |
| Hudson | 1,705 | 431 | 179 | 375 | NA | 1,077 | 269 | 134 | 345 | NA |
| Hunterdon | 50 | 1,115 | 148 | 113 | NA | 50 | 644 | 111 | 103 | NA |
| Mercer | 221 | 967 | 201 | 224 | NA | 188 | 530 | 141 | 203 | NA |
| Middlesex | 537 | 1,162 | 486 | 376 | NA | 483 | 467 | 347 | 346 | NA |
| Monmouth | 48 | 1,575 | 352 | 545 | NA | 55 | 981 | 244 | 501 | NA |
| Morris | 46 | 1,813 | 305 | 309 | NA | 39 | 1,284 | 209 | 280 | NA |
| Ocean | 39 | 2,377 | 229 | 446 | NA | 38 | 1,734 | 160 | 409 | NA |
| Passaic | 18 | 835 | 195 | 194 | NA | 19 | 543 | 141 | 178 | NA |
| Salem | 435 | 590 | 77 | 132 | NA | 371 | 377 | 57 | 122 | NA |
| Somerset | 76 | 984 | 211 | 164 | NA | 55 | 441 | 152 | 149 | NA |
| Sussex | 6 | 1,667 | 77 | 99 | NA | 5 | 1,301 | 54 | 89 | NA |
| Union | 434 | 512 | 261 | 362 | NA | 540 | 272 | 185 | 333 | NA |
| Warren | 240 | 1,195 | 123 | 71 | NA | 240 | 809 | 92 | 64 | NA |
| Total in State | 5,555 | 24,760 | 4,718 | 6,505 | NA | 4,868 | 16,230 | 3,361 | 5,922 | NA |

NOTES:

- These totals include adjusted emissions from fugitive dust categories. See Attachment 2 of this report for further discussion.

### Table XIX (cont.) – 2002 Statewide Emission Inventory by County and Source Sector

| County | SO2 #### Tons per Year |  |  |  |  | NH3 Tons per Year |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Point Sources | Area Sources | Onroad Sources | Nonroad Sources | Biogenic | Point Sources | Area Sources | Onroad Sources | Nonroad Sources | Biogenic |
| Atlantic | 10 | 498 | 202 | 176 | NA | 0 | 184 | 297 | 13 | 329 |
| Bergen | 82 | 819 | 634 | 620 | NA | 0 | 543 | 821 | 163 | 863 |
| Burlington | 286 | 459 | 361 | 2,462 | NA | 0 | 522 | 454 | 39 | 520 |
| Camden | 162 | 506 | 313 | 1,057 | NA | 0 | 281 | 393 | 46 | 518 |
| Cape May | 12,178 | 163 | 75 | 993 | NA | 5 | 86 | 107 | 6 | 130 |
| Cumberland | 665 | 412 | 89 | 2,115 | NA | 1 | 310 | 118 | 20 | 203 |
| Essex | 2,110 | 1,078 | 429 | 980 | NA | 0 | 598 | 492 | 82 | 762 |
| Gloucester | 5,431 | 390 | 211 | 1,243 | NA | 0 | 445 | 265 | 22 | 274 |
| Hudson | 19,250 | 625 | 196 | 1,582 | NA | 14 | 461 | 222 | 56 | 572 |
| Hunterdon | 18 | 391 | 163 | 123 | NA | 0 | 569 | 187 | 14 | 164 |
| Mercer | 14,379 | 450 | 264 | 501 | NA | 3 | 310 | 331 | 41 | 347 |
| Middlesex | 504 | 689 | 590 | 612 | NA | 11 | 492 | 765 | 108 | 746 |
| Monmouth | 55 | 510 | 453 | 929 | NA | 0 | 399 | 628 | 47 | 651 |
| Morris | 52 | 798 | 403 | 276 | NA | 0 | 273 | 572 | 75 | 544 |
| Ocean | 38 | 652 | 290 | 216 | NA | 0 | 258 | 396 | 21 | 616 |
| Passaic | 26 | 494 | 231 | 223 | NA | 0 | 264 | 292 | 65 | 505 |
| Salem | 4,590 | 156 | 85 | 673 | NA | 1 | 463 | 97 | 7 | 89 |
| Somerset | 41 | 273 | 250 | 180 | NA | 0 | 423 | 317 | 43 | 309 |
| Sussex | 0 | 566 | 98 | 69 | NA | 0 | 296 | 135 | 8 | 235 |
| Union | 1,253 | 602 | 321 | 1,680 | NA | 3 | 456 | 425 | 82 | 501 |
| Warren | 101 | 345 | 134 | 63 | NA | 0 | 371 | 152 | 12 | 153 |
| Total in State | 61,231 | 10,876 | 5,793 | 16,772 | NA | 38 | 8,005 | 7,469 | 970 | 9,032 |

4.	Emissions Comparison Summary

A comparison of the 1996 man-made emission inventory to the 2002 man-made emission inventory for New Jersey is presented in Table XX, by pollutant and source sector.  The 1996 inventory was chosen for comparison, because the 1999 area source inventory and portions of the 1999 nonroad inventory were projections of the 1996 base year inventory, therefore are also a reflection of the growth factors chosen, in addition to the methodologies used to calculated emissions.  This comparison shows the following:

- Total man-made VOC, summer tons per day: Overall slight decrease, decreases in point and onroad, increases in area and nonroad;

- Total man-made NOx summer tons per day: Overall slight increase, slight decreases in point, area and nonroad, increase in onroad; and,

- Total man-made CO summer tons per day: Overall increase, increases in point, area, nonroad and onroad.

More detailed discussions of increases and decreases in emissions are included in the individual sector comparisons in Appendix D.  Decreases are due primarily to federal and state rules that control emissions from industries such as the NOx Budget Program; revisions in federal engine standards; reformulated gasoline; and basic inspection and maintenance programs.  Increases are due primarily to population growth and increases in vehicle miles traveled; changes in calculation and model methodologies and inputs; and the addition of new emission sources not previously included in the inventory.

Table XX

1996 and 2002 Statewide Emission Inventory by Source Sector and Pollutant

|  | VOC |  | ###### NOx |  | CO |  |
| --- | --- | --- | --- | --- | --- | --- |
| Source Sector | 1996 Tons per Summer Day | 2002 Tons per Summer Day | 1996 Tons per Summer Day | 2002 Tons per Summer Day | 1996 Tons per Summer Day | 2002 Tons per Summer Day |
| Point | 173.22 | 113.15 | 291.05 | 280.36 | 78.45 | 89.35 |
| Area | 304.98 | 369.83 | 39.66 | 35.92 | 26.89 | 66.45 |
| Onroad | 309.01 | 274.74 | 453.82 | 558.66 | 2,182.99 | 2,856.37 |
| Nonroad | 203.73 | 220.60 | 269.24 | 231.56 | 2,152.25 | 2,497.80 |
| Total in State | 990.94 | 978.32 | 1,053.77 | 1106.50 | 4,440.58 | 5,509.97 |

- Public Participation

New Jersey is planning on holding a public hearing on the proposed SIP revisions.  Notice of the hearing will be published in several newspapers throughout the State before the date of the public hearing.  A complete description of the public hearing process, the comments received and New Jersey’s response to those comments will be included in the final SIP document.

Appendix A

History of New Jersey’s Carbon Monoxide State Implementation Plan

This appendix provides a history of the previous updates to New Jersey's carbon monoxide SIP.

1982 Carbon Monoxide SIP

The 1982 Carbon Monoxide SIP identified two State measures and one federal measure to bring New Jersey’s nonattainment areas into compliance with the NAAQS.  The state measures identified were the pre-1990 modifications to the State’s basic motor vehicle inspection and maintenance (I/M) program (not to be confused with the enhanced I/M program described in the 1990 Clean Air Act) and local transportation control measures.  The federal measure was the Federal Motor Vehicle Control Program.

The USEPA approved the pre-1990 modifications to the basic I/M program for inclusion in the SIP.  The USEPA also found that New Jersey had implemented all of the transportation control measures committed to in the 1982 SIP revision.  The Federal Motor Vehicle Control Program was implemented nationally and was subsequently revised by the Clean Air Act Amendments of 1990, which contained new programs to further reduce emissions from motor vehicles.  These programs continue to produce emission reductions as newer motor vehicles constantly replace older vehicles, a phenomenon commonly referred to as vehicle fleet turnover.

1992 Carbon Monoxide SIP Revisions

On November 15, 1992, New Jersey submitted to the USEPA revisions to the carbon monoxide SIP required by the 1990 Clean Air Act.  These revisions included:

- Submission of a 1990 emission inventory;

- Commitment to perform periodic emission inventories;

- Commitment to demonstrate attainment of the carbon monoxide NAAQS using modeling;

- Commitment to submit annual vehicle miles traveled tracking reports;

- Requirement for the sale of oxygenated gasoline;

- Adoption of contingency measures for failure to attain the standard;

- Adoption of contingency measures for exceedance of the vehicle miles traveled forecast;

- Commitment to adopt an enhanced I/M program;

- Adoption of a new source review program; and,

- Commitment to perform conformity determinations.

The USEPA approved New Jersey’s emission inventory and contingency measures on December 7, 1995, at 60 FR 62741. The State has since complied with all of the commitments made in its 1992 carbon monoxide SIP and has implemented the necessary measures. Many of the commitments included in the 1992 carbon monoxide SIP have been approved by the USEPA, as outlined in the next few paragraphs.

The USEPA adopted a limited approval of New Jersey’s oxygenated fuels rule on February 12, 1996, at 61 FR 5299.  It should be noted that the 1992 SIP revision contained a wintertime oxygenated fuels rule that outlined a program designed for both the Camden County (southern) carbon monoxide nonattainment area and the New Jersey portion of the New York City/Northern New Jersey/Long Island (northeastern) carbon monoxide nonattainment area.  However, at the time of the USEPA’s approval of the State’s wintertime oxygenated fuels program, New Jersey was in attainment in the Camden County area and the USEPA’s direct final rule redesignating that area to attainment was in effect.  Consequently, the USEPA’s SIP approval for New Jersey’s wintertime oxygenated fuels program applied only to the northeastern carbon monoxide nonattainment area.  New Jersey tried several times to end the program due to concern regarding methyl tertiary-butyl ether (or MTBE), and subsequently adopted regulations that ended the wintertime oxygenated fuels program in the southern portion of the State.  At New Jersey’s request, the USEPA approved the removal of New Jersey’s oxygenated gasoline program from its SIP on November 22, 1999.

The USEPA proposed both a limited approval and a limited disapproval of the State’s carbon monoxide New Source Review rule and a disapproval of the State’s carbon monoxide attainment demonstration for the northeastern part of the State on November 10, 1994, at 59 FR 56019.  The USEPA’s proposed disapproval of the State’s carbon monoxide attainment demonstration was predicated on the fact that the demonstration relied on the implementation of an enhanced I/M program that had not been fully developed or implemented by the State.  On July 25, 1996, at 61 FR 38591, the USEPA adopted its limited approval of the State’s New Source Review regulation, as well as adopting its proposed approvals of New Jersey’s vehicle miles traveled forecast and its multi-state coordination commitment.  As part of its July 25, 1996, promulgation, the USEPA committed to taking future action on New Jersey’s attainment demonstration and enhanced I/M program in separate Federal Registers.  The USEPA has granted conditional interim approval of New Jersey’s enhanced I/M program and has proposed a full approval.,  The USEPA determined on November 22, 1999, that the entire northeastern nonattainment area had met the NAAQS for carbon monoxide.

1994 Carbon Monoxide SIP Revisions

On November 17, 1994, New Jersey revised its carbon monoxide SIP for the northeastern carbon monoxide nonattainment area to incorporate the results for the most recent planning tools available to the NJDEP.  The updated planning tools included:

- The USEPA Mobile Source Emission Factor Model, MOBILE5a;

- The latest version of the line-source dispersion model, CAL3QHC version 2.0; and,

- The travel demand model for the northern part of the State.

These latest planning tools were used for the 1994 carbon monoxide SIP revision, in part, to ensure that the methodologies and assumptions used to calculate emission reductions for SIP purposes were consistent with those used to calculate emission reductions for Transportation Improvement Program conformity purposes.

The combined application of these updated tools resulted in an increase in the emission inventory, increased benefits for the control programs, a lower vehicle miles traveled growth rate, and a higher predicted concentration at each intersection examined in the attainment demonstration.  The 1994 attainment demonstration also included the effects of the State’s wintertime oxygenated fuel and enhanced I/M programs.  However, the conclusion remained the same as in the 1992 SIP revision; that is, the carbon monoxide NAAQS would be attained by December 31, 1995.

1995 Carbon Monoxide SIP Revision

In 1995, the State of New Jersey applied to the USEPA for redesignation of both the Camden County carbon monoxide nonattainment area and the nine not-classified carbon monoxide areas to attainment of the carbon monoxide NAAQS.  The USEPA approved these redesignation requests in a Federal Register notice published on December 7, 1995, that became effective on February 7, 1996.  This approval was re-affirmed by the USEPA in a Federal Register notice published on June 28, 1996 that incorporated the USEPA’s responses to comments received during the public comment period.

1996 Request for an Extension of the Attainment Date

It was not possible to demonstrate two years of non-violating air quality data by the December 31, 1995 attainment deadline due to carbon monoxide NAAQS violations in 1994.  Therefore, the State had the choice of either allowing the area to be reclassified to the higher classification of serious carbon monoxide nonattainment or applying to the USEPA for an extension of the attainment date as allowed by the Clean Air Act.  On April 24, 1996, the State submitted a request to the USEPA for a 1-year extension of the attainment date to December 31, 1996.  New York and Connecticut, the two other states that comprise the New York City/Northern New Jersey/Long Island carbon monoxide nonattainment area, subsequently submitted letters to the USEPA on July 31, 1996, and June 27, 1996, respectively, concurring with New Jersey’s request for an attainment date extension.  The northeastern carbon monoxide nonattainment area met the Clean Air Act’s requirements and the USEPA’s criteria for obtaining an extension of an attainment date for a moderate nonattainment area in that it:

- Had complied with all the requirements and commitments pertaining to the area in the applicable implementation plan; and,

- Had no more than one exceedance of the carbon monoxide NAAQS at any monitoring site in the year preceding the extension year, that is, 1995.,

The USEPA approved New Jersey’s and the other states’ 1-year attainment date extension requests on November 5, 1996, at 61 FR 56897.

1998 Carbon Monoxide SIP Revision

On July 21, 1997, the State proposed regulatory amendments to repeal its wintertime oxygenated fuel requirements for Northern New Jersey in the New Jersey Register (29 NJR 3222(a)).  In addition to this proposed rulemaking, the NJDEP also prepared a proposed carbon monoxide SIP revision that, in part:

- Demonstrated that the New Jersey portion of the New York City/Northern New Jersey/Long Island carbon monoxide nonattainment area had attained the carbon monoxide NAAQS;

- Requested that, based on this attainment demonstration and a comprehensive plan to maintain the standard for at least the next ten years, the New Jersey portion of the multi-state nonattainment area be redesignated to attainment; and,

- Removed the State’s wintertime oxygenated fuel program from New Jersey’s carbon monoxide SIP.

A hearing to take public comment on both the rulemaking proposal and the SIP revision was held on August 11, 1997, and written comments were accepted until close of business, August 20, 1997.  Based upon comments received during the comment period and subsequent conversations with the USEPA and the other states in the multi-state nonattainment area, the State decided, on August 7, 1998, to submit only portions of the proposed carbon monoxide SIP revision to the USEPA.  The State subsequently submitted the entire proposal except for:

- The maintenance plan (which demonstrated that New Jersey would continue to maintain the carbon monoxide NAAQS until the year 2009 and discussed the contingency measure(s) that would be implemented should New Jersey again violate the NAAQS); and,

- The request that the USEPA redesignate the northeastern nonattainment area to attainment (the “redesignation request”).

In that submittal, the State also committed to revise its transportation conformity emission budget once the USEPA took action on the SIP revision.

On August 17, 1998, the NJDEP adopted its regulatory proposal calling for the removal of the wintertime oxygenated fuel program in Northern New Jersey.  On November 22, 1999, the USEPA determined that the New York City/Northern New Jersey/Long Island carbon monoxide area had attained the carbon monoxide NAAQS.  The USEPA also approved the State’s request to remove New Jersey’s oxygenated gasoline program from its SIP.

2002 Redesignation Request and Maintenance Plan for the New Jersey Portion of the New York City/Northern New Jersey/Long Island Carbon Monoxide Nonattainment Area

In 2002, New Jersey submitted a request to the USEPA to redesignate the northern carbon monoxide nonattainment area to attainment.  This SIP revision contained:

- Updated air quality monitoring data that demonstrated that measured carbon monoxide levels continued to remain below standards;

- A maintenance plan that included control measures, transportation conformity emission budgets, and a contingency plan; and,

- Other information that supported the Request for Redesignation.

The air quality monitoring data showed attainment with the health-based carbon monoxide NAAQS since 1996, while the carbon monoxide inventory projections for the years 2007 and 2014 that were included in the maintenance plan showed reductions in emissions relative to the emissions estimated for 1996.

The USEPA approved New Jersey's redesignation request and maintenance plan on August 23, 2002 at 67 FR 54574.  Included in the approval were transportation conformity emission budgets for 1997, 2007, and 2014.

2004 New Jersey Revised Motor Vehicle Transportation Conformity Emission Budgets Using the MOBILE6 Model

In 2004, New Jersey submitted a request to the USEPA for a SIP revision to establish updated transportation conformity emission budgets that incorporated new data and the use of the new USEPA motor vehicle emissions model, MOBILE6, that was required for use in future conformity determinations for New Jersey.  Transportation conformity emission budgets for carbon monoxide, VOCs, and NOx were updated to reflect the most recent version of the USEPA’s emission factor prediction model and the latest vehicle registration data.  The latest vehicle registration data were used to establish new estimates of the age distribution of New Jersey’s onroad motor vehicle fleet.  This SIP revision did not affect any of the planned or implemented control measures for carbon monoxide, VOCs, and NOx.  In addition, the updated budgets did not indicate a need for any additional control measures for New Jersey to maintain attainment of the carbon monoxide NAAQS or reach attainment of the 1-hour ozone NAAQS.

The USEPA approved New Jersey's SIP revision request on August 30, 2004 at 69 FR 52834-52836.

Appendix B

History of New Jersey’s Ozone State Implementation Plan

This appendix provides a brief history of the previous revisions to New Jersey's ozone SIP.

Attainment Demonstration SIP History

On August 31, 1998, New Jersey submitted to the USEPA a SIP revision containing a demonstration of attainment of the 1-hour ozone NAAQS for the New York City/Northern New Jersey/Long Island and Philadelphia/Wilmington/Trenton nonattainment areas.  This original attainment demonstration submittal is hereafter referred to as the State’s Phase II Ozone SIP.  The Phase II Ozone SIP submittal provided for an attainment demonstration as required by 42 U.S.C. §7511a(c)(2)(A), §182(c)(2)(A) of the Clean Air Act and addressed the USEPA’s subsequent requirements regarding attainment demonstration for the 1-hour NAAQS for ozone.,

New Jersey used a "weight of evidence" to determine the emission reductions needed to attain the ozone standard.  A weight of evidence analysis combines results from advanced photochemical grid models and the most recent air quality data to improve the estimate of emission reductions needed to attain.  The method used by New Jersey predicts future ozone concentrations from a baseline of actual historic air quality data and the ozone improvement predicted by the photochemical grid model.  The improvement is the model-predicted base year concentration divided by the model-predicted future attainment year concentration.  This method takes advantage of the fact that air quality models may be more accurate at calculating relative improvement in air quality as opposed to predicting an absolute concentration at a particular geographic site.

In addition to including a demonstration of attainment of the 1-hour NAAQS for ozone for the New York City/Northern New Jersey/Long Island and Philadelphia/Wilmington/Trenton nonattainment areas and a list of the control measures adopted by the State to date, the Phase II Ozone SIP committed the State to:

- Submit post-1999 Rate of Progress Plans and any adopted regulations needed to achieve the post-1999 emission reductions by December 31, 2000;

- Implement the New Jersey portion of the USEPA regional NOx cap (NOx SIP Call);

- Undertake a midcourse review and submit a report to the USEPA by December 31, 2002;

- Evaluate additional control measures which are not currently implemented for potential future implementation; and,

- Propose such reasonable and necessary control measures needed to address any shortfall identified in the midcourse review which are necessary for attainment.

In reviewing the attainment demonstrations submitted by New Jersey, as well as other states’ submittals (such as New York, Pennsylvania and Maryland), the USEPA performed its own analyses (also using the weight of evidence method but with a different base year and different modeling results) and determined that further emission reductions were necessary to insure attainment by the applicable dates.  For New Jersey, the USEPA’s analyses results were reasonably similar to the uncertainty analysis results New Jersey presented in its Phase II Ozone SIP to quantify the uncertainties incorporated its air quality projections.  Therefore, considering both the USEPA and the prior state analyses, the State revised its attainment demonstration to include a commitment to a process designed to secure New Jersey’s fair share of the additional emission reductions identified by the USEPA.

On September 12, 2001, New Jersey submitted a SIP revision containing an update to meeting the requirements of the alternative ozone attainment demonstration policy (Control Measures SIP).  Specifically, this SIP revision provided:

- An enforceable commitment by New Jersey to adopt sufficient measures to address its fair share of the level of additional emission reductions identified by the USEPA, and to revise its Attainment Demonstration accordingly to reflect those measures;

- A revised transportation conformity emission budget that included the Tier 2 Motor Vehicle Standard/Low Sulfur Gasoline Program benefits;

- An enforceable commitment to revise the New Jersey Ozone Attainment Demonstration to recalculate the transportation conformity emission budgets to reflect any additional measures adopted (beyond the Tier 2 Motor Vehicle Standard/Low Sulfur Program) pertaining to motor vehicles;

- An enforceable commitment to revise the New Jersey Ozone Attainment Demonstration to recalculate the transportation conformity emission budgets, within one year after the MOBILE6 model is released and required for use in the development of SIPs;

- A list of possible additional control measures from which a suite of measures can be drawn that would be expected to meet New Jersey's fair share of the USEPA – identified emission reduction shortfall; and,

- An enforceable commitment to perform a midcourse review by December of 2003 that was subsequently changed to December of 2004.

The control measure SIP included the following additional rules that implemented control measures: NOx rule, consumer products rule, portable fuel containers rule, architectural and industrial maintenance coatings rule, mobile equipment refinishing rule, and solvent cleaning operations rule.  The projected emission benefits from these rules just covered the USEPA identified shortfall for the Philadelphia/Wilmington/Trenton nonattainment area and more than covered the USEPA identified shortfall for the New York City/Northern New Jersey/Long Island nonattainment area.

Rate of Progress SIP History

The State submitted its original 1996 15-percent Rate of Progress plans to the USEPA on November 15, 1993.  Subsequently, on December 31, 1996, New Jersey submitted to the USEPA, as part of its Phase I Ozone SIP submittal, a revision which updated its 1993 15-percent Rate of Progress plans and included its 1999 24-percent Rate of Progress plans to the USEPA.  The USEPA granted conditional interim approval to New Jersey’s Phase I Ozone SIP submittal on June 30, 1997.  The USEPA’s approval of New Jersey’s Phase I Ozone SIP was conditional based on the modeling contained in the 15-percent and 24-percent Rate of Progress Plans.  On December 12, 1997, the USEPA disapproved the 15-percent Rate of Progress Plans’ portion of New Jersey’s Phase I Ozone SIP due to the realization that the benefits claimed in these plans for the State’s enhanced I/M program would not be obtained.

On February 5, 1999, the State submitted revised 15-percent Rate of Progress and 24-percent Rate of Progress plans that no longer relied on the benefits anticipated from the enhanced I/M program.  These revised plans were approved by the USEPA on April 23, 1999.  On December 13, 1999, the State began implementation of its enhanced I/M program.

On March 31, 2001, New Jersey submitted a SIP revision containing the actual 1996 inventory and Rate of Progress plans for 2002, 2005 and 2007.  The Rate of Progress SIP contained the remaining Rate of Progress plans for each milestone year up to and including the attainment years for each applicable nonattainment area.  Using control measures consistent with those in the State's demonstration of attainment of the 1-hour ozone standard, it was shown that the Rate of Progress targets were achieved.  In addition, the State agreed to find further emission reductions, identified by the USEPA, and is currently working with other Ozone Transport Region states in this regard.  Once these measures are adopted, projected controlled emission levels would decrease further.  The Rate of Progress SIP also contained revised transportation conformity emission budgets.

The purpose of the Rate of Progress submittals was to demonstrate steady incremental progress (three percent of the 1990 VOC baseline emission level averaged over each consecutive three year period beginning in 1991) leading towards the ultimate goal of attainment.  The purpose of the attainment demonstration, however, was to assess the overall emission reductions necessary to actually achieve attainment, which could be greater than or less than the Rate of Progress incremental reductions.  If the attainment demonstration shows that a state needs less than three percent over each consecutive three year period to reach attainment, it can petition the USEPA to reduce the Rate of Progress requirement for their particular state.  In New Jersey’s case, however, attaining the standard requires emission reductions that exceed Rate of Progress requirements.  By way of illustration, the control measures in the attainment demonstration were incorporated in the Rate of Progress SIP, and the resulting controlled emission levels indicate that the inventories for New Jersey portion of the New York City/Northern New Jersey/ Long Island and Philadelphia/Wilmington/Trenton nonattainment areas are well below the targets derived from the three percent reduction over each consecutive three year period.  For example, for the New York City/Northern New Jersey/Long Island nonattainment area for 2007, the sum of the New Jersey VOC and NOx percentage emission reduction was 83.5 percent as compared to a 48 percent Rate of Progress test requirement.  Therefore, for New Jersey, the emission reductions needed to attain the ozone standard significantly exceed the three percent per year Rate of Progress requirements.

2003 New Jersey Revised Motor Vehicle Emission Inventories and Transportation Conformity Emission Budgets Using the MOBILE6 Model

In 2003, New Jersey submitted a SIP revision to fulfill its commitment to revise 2005 and 2007 onroad motor vehicle emission budgets for the Philadelphia/Wilmington/Trenton nonattainment area and New York City/Northern New Jersey/Long Island nonattainment area using the new MOBILE6 model.  In addition, this SIP revision showed that the new levels of onroad motor vehicle emissions calculated using MOBILE6 continue to support predicted achievement of rate of progress requirements and projected attainment of the 1-hour ozone NAAQS by the attainment dates for each nonattainment area.  The MOBILE6 generated inventories were also used to establish transportation conformity emission budgets for the appropriate Metropolitan Planning Organizations (MPOs) in New Jersey.

The USEPA approved the revised emission inventories and transportation conformity emission budgets using MOBILE6 on May 5, 2003 at 68 FR 23662.

2004 New Jersey Revised Motor Vehicle Transportation Conformity Emission Budgets Using the MOBILE6 Model

In 2004, New Jersey submitted a request to the USEPA for a SIP revision to establish updated transportation conformity emission budgets that incorporated new data and the use of the new USEPA motor vehicle emissions model (MOBILE6) that was required for use in future conformity determinations for New Jersey.  Transportation conformity emission budgets for carbon monoxide, VOCs, and NOx were updated to reflect the most recent version of the USEPA’s emission factor prediction model and the latest vehicle registration data.  The latest vehicle registration data were used to establish new estimates of the age distribution of New Jersey’s onroad motor vehicle fleet.  This SIP revision did not affect any of the planned or implemented control measures for carbon monoxide, VOCs, and NOx.  In addition, the updated budgets did not indicate a need for any additional control measures for New Jersey to maintain attainment of the carbon monoxide NAAQS or reach attainment of the 1-hour ozone NAAQS.

The USEPA approved New Jersey's SIP revision request on August 30, 2004 at 69 FR 52834-52836.

Transportation Conformity Rule Amendments for the New 8-Hour Ozone and PM2.5 National Ambient Air Quality Standards and Miscellaneous Revisions for Existing Areas; Transportation Conformity Rule Amendments: Response to Court Decision and Additional Rule Changes; Final Rule, 69 Fed. Reg., 40026-40027 (July 1, 2004).

40 CFR  93.124

68 Fed. Reg., 43462-43465 (July 23, 2003).

USEPA, Memorandum from Lydia Wegman (OAQPS) to EPA Regional Air Directors, “Limited Maintenance Plan Option for Moderate PM10 Nonattainment Areas,” August 21, 2001.

SO2 has been reported in the inventory instead of SOx as required in the Consolidated Emissions Reporting Rule because the USEPA MOBILE and NON-ROAD models and the majority of USEPA guidance on emission factors is based on SO2, not SOx.  In addition, the USEPA National Emissions Inventory reports SO2.

42 U.S.C. §7410

42 U.S.C. §7401

USEPA, Memorandum from John Calcagni, Director, Air Quality Management Division, to Regional Air Directors, “Procedures for Processing Requests to Redesignate Areas to Attainment,” September 4, 1992, page 6.

42 U.S.C. §7506(c)(1)

For New Jersey such plans are prepared by three Metropolitan Planning Organizations (North Jersey Transportation Planning Authority, South Jersey Transportation Planning Organization and Delaware Valley Regional Planning Commission).

Transportation Conformity Rule Amendments for the New 8-Hour Ozone and PM2.5 National Ambient Air Quality Standards and Miscellaneous Revisions for Existing Areas; Transportation Conformity Rule Amendments: Response to Court Decision and Additional Rule Changes; Final Rule, 69 Fed. Reg., 40026-40027 (July 1, 2004).

40 CFR 93.124

Op cit, note 12.

67 Fed. Reg., 54577 (August 23, 2002).

USEPA and U.S. Department of Transportation, Memorandum on “Use of Latest Planning Assumptions in Conformity Determinations,” January 18, 2001.

NJDEP, SIP Revision for the Attainment and Maintenance of the Ozone NAAQS, New Jersey Revised Motor Vehicle Emission Inventories and Transportation Conformity Budgets Using the MOBILE6 Model, April 4, 2003.

NJDEP, State Implementation Plan revision for the Attainment and Maintenance of the Ozone National Ambient Air Quality Standard, New Jersey 1996 Actual Emission Inventory and Rate of Progress (ROP) Plans for 2002, 2005 and 2007, March 31, 2001.

NJDEP.  Final State Implementation Plan Revision for the Attainment and Maintenance of the Ozone National Ambient Air Quality Standard (NAAQS), New Jersey Revised Motor Vehicle Emission Inventories and Transportation Conformity Budgets Using MOBILE6 Model, 2003.

NJDEP. Final State Implementation Plan Revision for the Attainment and Maintenance of the Ozone National Ambient Air Quality Standard (NAAQS), New Jersey Revised Motor Vehicle Emission Inventories and Transportation Conformity Budgets Using MOBILE6 Model, 2003.

USEPA approval: 68 Fed. Reg.  43462.

USEPA, Office of Air Quality Planning and Standards, NOx Substitution Guidance, December 1993.

NJDEP, “State Implementation Plan Revision for the Attainment and Maintenance of the Ozone National Ambient Air Quality Standards, Meeting the Requirements of the Alternative Ozone Attainment Demonstration Policy, Phase I Ozone SIP Submittal”, December 31, 1996.

NJDEP, State Implementation Plan Revision for the Attainment and Maintenance of the Ozone National Ambient Air Quality Standards, New Jersey 1996 Actual Emission Inventory and Rate of Progress Plans for 2002, 2005 and 2007, 2001, Table 29 on page 65.

McGuire Air Force Base Conformity Determination.  July, 1995.

NJDEP, State Implementation Plan Revision for the Attainment and Maintenance of the Ozone National Ambient Air Quality Standards, Phase I Ozone SIP Submittal, 1996, p. 123.

NJDEP, State Implementation Plan Revision for the Attainment and Maintenance of the Ozone National Ambient Air Quality Standards, New Jersey 1996 Actual Emission Inventory and Rate of Progress Plans for 2002, 2005 and 2007, 2001, p. 71.

Approval and Promulgation of Implementation Plans; New Jersey; Revised Motor Vehicle Emissions Inventories for 1996, 2005, and 2007 and Motor Vehicle Emissions Budgets for 2005 and 2007 Using MOBILE6, Final Rule, 68 Fed. Reg., 43463 (July 23, 2003).

U.S. Dept. of the Air Force, Letter from John Hoertz, Program Manager, Air Force Center for Environmental Excellence, Atlanta, Georgia to Sandra Krietzman, Chief, NJDEP, Bureau of Air Quality Planning, Requesting Conformity Budget Change, October 11, 2005.

USEPA, “NOx Substitution Guidance,” 1993.

NJDEP. Final State Implementation Plan Revision for the Attainment and Maintenance of the Ozone National Ambient Air Quality Standard (NAAQS), New Jersey Revised Motor Vehicle Emission Inventories and Transportation Conformity Budgets Using MOBILE6 Model, 2003.

68 Fed. Reg., 43462-43465 (July 23, 2003).

The attainment demonstration SIP for the PM2.5 NAAQS is not due until April 2008.

Personal communication with USEPA II, October 2005.

Additional background information can be found at the USEPA website (http://www.epa.gov/pmdesignations/faq.htm%230).

42 U.S.C. §7409(a).

42 U.S.C §7409(d).

http://www.epa.gov/pmdesignations

Transportation Conformity Rule Amendments for the New 8-hour Ozone and PM2.5 National Ambient Air Quality Standards and Miscellaneous Revisions for Existing Areas; Transportation Conformity Rule Amendments: Response to Court Decision and Additional Rule Changes; Final Rule, 69 Fed. Reg., 40004-40081 (July 1, 2004).

40 CFR 93.119.

Transportation Conformity Rule Amendments for the New PM2.5 National Ambient Air Quality Standard: PM2.5 Precursors, Final Rule, 70 Fed. Reg., 24280 (May 6, 2005).

69 Fed. Reg., 40030.

USEPA Region 2 and NJDEP, conference call, October 4, 2005.

New Jersey Carbon Monoxide State Implementation Plan, Redesignation And Maintenance Plan For Camden County, New Jersey Department of Environmental Protection, September 29, 1995, and

New Jersey Carbon Monoxide State Implementation Plan, Redesignation and Maintenance Plan for the Nine Not-Classified Areas, New Jersey Department of Environmental Protection, September 29, 1995.

SIP Revision for the Attainment and Maintenance of the Carbon Monoxide National Ambient Air Quality Standards, Redesignation Request and Maintenance Plan for the New Jersey Portion of the New York-Northern New Jersey-Long Island Carbon Monoxide Nonattainment Area, New Jersey Department of Environmental Protection, January 15, 2002.

New Jersey Carbon Monoxide State Implementation Plan, Redesignation And Maintenance Plan For Camden County, New Jersey Department of Environmental Protection, September 29, 1995, and

New Jersey Carbon Monoxide State Implementation Plan, Redesignation and Maintenance Plan for the Nine Not-Classified Areas, New Jersey Department of Environmental Protection, September 29, 1995.

SIP Revision for the Attainment and Maintenance of the Carbon Monoxide National Ambient Air Quality Standards, Attainment Demonstration for the New Jersey Portion of the New York-Northern New Jersey-Long Island Carbon Monoxide Nonattainment Area, New Jersey Department of Environmental Protection, August 7, 1998.

USEPA, Memorandum from Joseph Paisie (OAQPS) to Regional Air Branch Chiefs, “Limited Maintenance Plan Option for Non-Classifiable CO Nonattainment Areas,” 10/6/1995.

Op cit, note 43.

USEPA, Electronic mail from Henry Feingersh of Region II to Christine Schell, NJDEP, Bureau of Air Quality Planning, January 27, 2005, 8:45 am.

NJDEP and USEPA, Region II, Conference call, September 23, 2005.

Ibid.

58 Fed. Reg., 62188 (November 24, 1993).

58 Fed. Reg., 63214 (November 30, 1993).

New Jersey Carbon Monoxide State Implementation Plan, Redesignation and Maintenance Plan for the Nine Not-Classified Areas, New Jersey Department of Environmental Protection, September 29, 1995.

Ibid

42 §U.S.C. 7405.

40 CFR 58.

42 U.S.C. §7505a(d).

USEPA, Memorandum from John Calcagni, Director, Air Quality Management Division, to Regional Air Directors, “Procedures for Processing Requests to Redesignate Areas to Attainment,” September 4, 1992, page 12.

USEPA, Memorandum from Joseph Paisie (OAQPS) to Regional Air Branch Chiefs, “Limited Maintenance Plan Option for Non-Classifiable CO Nonattainment Areas,” October 6, 1995.

58 Fed. Reg., 62188 (November 24, 1993).

58 Fed. Reg. 63214 (November 30, 1993).

Environmental review documents are prepared when required by the National Environmental Policy Act.

70 Fed. Reg. 44470 (August 3, 2005).

SO2 has been reported in the inventory instead of SOx as required in the Consolidated Emissions Reporting Rule because the USEPA MOBILE and NONROAD models and the majority of USEPA guidance on emission factors is based on SO2, not SOx.  In addition, the USEPA National Emissions Inventory (NEI) reports SO2.

40 CFR 52.1570 et seq.

USEPA, Letter from USEPA Region II to Anthony McMahon, dated August 29, 1989.

61 Fed. Reg. 33678 (June 28, 1996) and 60 Fed. Reg. 62741 (December 7, 1997).

27 NJ Reg.,. 4731 (November 20, 1995) and 28 NJ Reg. 851 (February 5, 1996).

62 Fed. Reg. 26401 (May 14, 1997).

66 Fed. Reg. 47130.

64 Fed. Reg. 48970.

60 Fed. Reg. 62741.

61 Fed. Reg. 33678.

42 U.S.C. 7512(a)(4).

NJDEP, Letter from Robert C. Shinn, Commissioner to Jeanne M. Fox, Regional Administrator, USEPA, Region II, dated April 24, 1996.

USEPA memorandum dated October 23, 1995, entitled Criteria for Granting Attainment Date Extensions, Making Attainment Determinations, and Determinations of Failure to Attain the NAAQS for Moderate Carbon Monoxide Nonattainment Areas, from Sally L. Shaver, Director, Air Quality Strategies and Standards Division, to Regional Air Office Directors.

42 U.S.C. 7512(a).

30 NJ Reg., 3025.

64 Fed. Reg., 48970.

64 Fed. Reg. 63690.

NJDEP, SIP Revision for the Attainment and Maintenance of the Carbon Monoxide NAAQS, Redesignation Request and Maintenance Plan for the New Jersey Portion of the New York/Northern New Jersey/Long Island Carbon Monoxide Nonattainment Area, January 15, 2002.

NJ SIP Revision, Meeting the Requirements of the Alternative Ozone Attainment Demonstration Policy-Phase II Ozone Submittal, August 31, 1998.

USEPA, Memorandum from Mary D. Nichols, Assistant Administrator for Air and Radiation to the Regional Administrators, Region I-X.  This Policy is commonly referred as "The March 2nd Policy," March 2, 1995.

USEPA, Memorandum from Richard D. Wilson, Acting Assistant Administrator for the Office of Air and Radiation to the Regional Administrators, Regions I-X,  “Guidance for Implementing the 1-Hour Ozone and Pre-Existing PM10 NAAQS,” December 29, 1997.

USEPA, Guidance for Improving Weight of Evidence Through Identification of Additional Emission Reductions Not Modeled, November, 1999.

New Jersey Department of Environmental Protection, State Implementation Plan (SIP) Revision for the Attainment and Maintenance of the One-Hour Ozone National Ambient Air Quality Standard, Update to Meeting the Requirements of the Alternative Ozone Attainment Demonstration Policy-Additional Emission Reduction Commitment and Transportation Conformity Budgets, April 26, 2000.

64 Fed. Reg. 70380, (December 16, 1999).

New Jersey Department of Environmental Protection and Energy, State Implementation Plan (SIP) Revision for the Attainment and Maintenance of the Ozone National Ambient Air Quality Standards, Meeting the Federal Clean Air Act Requirements, November 15, 1993.

The State of New Jersey, Department of Environmental Protection, State Implementation Plan (SIP) Revision for the Attainment and Maintenance of the Ozone National Ambient Air Quality Standards, Meeting the Requirements of the Alternative Ozone Attainment Demonstration Policy, Phase I Ozone SIP submittal, December 31, 1996.

62 Fed. Reg. 35100 (June 30, 1997).

In a letter dated May 29, 1997, New Jersey committed to perform the modeling necessary to estimate the emissions reductions that would result from the enhanced I/M program, as implemented, within 12 months from the effective date of the USEPA’s approval action (that is, by July 30, 1998).

Letter dated December 12, 1997 to New Jersey Governor Christine Todd Whitman from Regional Administrator Muszynski, and a similar but more detailed letter dated December 12, 1997 to Commissioner Robert C. Shinn, Jr., NJDEP and Commissioner John J. Haley, Jr., New Jersey Department of Transportation, from Deputy Regional Administrator William J. Muszynski, P.E., USEPA, Region II.  This action was later formalized by the USEPA at 63 Fed. Reg. 45399 (August 26, 1998).

64 Fed. Reg. 19913 (April 23, 1999).

NJDEP, State Implementation Plan revision for the Attainment and Maintenance of the Ozone National Ambient Air Quality Standard, New Jersey 1996 Actual Emission Inventory and Rate of Progress (ROP) Plans for 2002, 2005 and 2007, March 31, 2001.

42 U.S.C. §7511a(c)(2)(B)(ii).

NJDEP, SIP Revision for the Attainment and Maintenance of the Ozone NAAQS, New Jersey Revised Motor Vehicle Emission Inventories and Transportation Conformity Budgets Using the MOBILE6 Model, April 4, 2003.