The State of New Jersey

## Department of Environmental Protection

2002 Periodic Emission Inventory

Attachment 2:

Fugitive Dust Inventory

Discussion and Summary

May 2006

## Fugitive Dust Inventory Discussion and Summary

- Point Sources

Point source facilities are able to report fugitive and insignificant sources directly through the emission statement program.  From the emission statements the fugitive and insignificant sources were evaluated to determine which of the sources are classified as fugitive dust.  The emission statement program defines fugitive emissions as any emissions of an air contaminant released directly or indirectly into the atmosphere that does not pass through a stack, flue, vent, or chimney.  An insignificant source is any piece of equipment that is below the thresholds for a significant source which is equipment or control apparatus that requires a permit and certificate under N.J.A.C. 7:27-8.2.  All sources that contained PM10 or PM2.5 emissions that were classified, as fugitive dust became half of the point source fugitive dust inventory.  An evaluation of the rest of the PM emissions from the inventory was conducted in comparison with the emission unit’s description.  Based on a detailed visual sort, all applicable sources were marked and became the other half of the fugitive dust inventory.  The two data sorts were appended and became the final fugitive dust inventory.  The main categories were dust from paved and unpaved roadways, dust from stock/storage piles, landfill activity, quarry activity, and raw material handling.

# Area Sources

Area source fugitive dust categories include the following categories in accordance with USEPA Guidance (Procedures Document for National Emission Inventory, Criteria Air Pollutants 1985-1999):  paved roads, unpaved roads, construction (residential, commercial, road and other misc. heavy), mining and quarrying and agricultural tilling.  A detailed discussion regarding how these emissions were calculated is included in the Area Source Calculation Sheets.

C. Adjusted Inventory for Dust/Natural Particles

The carbonaceous combustion related PM2.5 and dust related PM2.5 emissions were calculated using prescribed USEPA Guidance factors.  However, it is believed that the dust-related PM2.5 emissions are overstated with respect to their actual contribution at New Jersey's monitoring sites.  Calculated combustion-related carbonaceous PM2.5 emissions are about 25,000 tons per year.  Dust-related PM2.5 emissions are about 5,443 tons per year, about 22% of the carbonaceous emissions.  However, at New Jersey's monitoring sites (Table 2) with species capability, the ratio of the natural dust component to the  carbonaceous component is only 4.2%.  Therefore, to have the calculated emissions match the monitored observations and their true proportional impacts to human health, a change in the calculated dust emissions of 4.2% / 22% or about 19% of their calculated value is appropriate.

This reduction is also supported by a study of fine particle matter near urban roadways which found that emissions of resuspended particulate matter near urban roads calculated using the prescribed guidance would result in fine particle levels 9 to 20 times higher than those observed.  Further, the USEPA and other regional air quality modeling work have found it necessary to reduce calculated dust-related emissions by 75-90% in order to have the modeling results match monitored PM2.5 speciated data.

Therefore, the fugitive dust emissions were multiplied by a dust adjustment factor (DAF) of 20%.

## Table 1:  2002 Unadjusted Statewide Fugitive Dust Emission Inventory by County, Source Sector and Pollutant

| County |  |  | PM10 Tons per Year |  |  |  |  | PM2.5 Tons per Year |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Point Sources |  |  |  | Area Sources |  | Point Sources |  | Area Sources |  |
| Atlantic | 2.99 |  |  |  | 1,644.46 |  | 3.55 |  | 110.47 |  |
| Bergen | 0.25 |  |  |  | 2,343.86 |  | 0.25 |  | 165.52 |  |
| Burlington | 102.74 |  |  |  | 3,847.27 |  | 27.51 |  | 510.84 |  |
| Camden | 27.13 |  |  |  | 2,474.26 |  | 6.68 |  | 282.39 |  |
| Cape May | 11.04 |  |  |  | 875.89 |  | 7.79 |  | 82.44 |  |
| Cumberland | 24.10 |  |  |  | 1,250.91 |  | 6.30 |  | 140.87 |  |
| Essex | 0.04 |  |  |  | 1,219.49 |  | 0.01 |  | 95.96 |  |
| Gloucester | 17.31 |  |  |  | 2,372.73 |  | 4.68 |  | 315.42 |  |
| Hudson | 9.99 |  |  |  | 840.30 |  | 4.01 |  | 53.73 |  |
| Hunterdon | 1.75 |  |  |  | 2,764.08 |  | 1.57 |  | 420.16 |  |
| Mercer | 2.46 |  |  |  | 2,440.47 |  | 2.28 |  | 301.27 |  |
| Middlesex | 88.54 |  |  |  | 3,803.42 |  | 16.07 |  | 369.80 |  |
| Monmouth | 38.30 |  |  |  | 3,167.40 |  | 39.54 |  | 254.51 |  |
| Morris | 10.12 |  |  |  | 2,779.92 |  | 4.84 |  | 232.08 |  |
| Ocean | 16.81 |  |  |  | 3,068.25 |  | 8.22 |  | 309.80 |  |
| Passic | 13.04 |  |  |  | 1,545.12 |  | 8.88 |  | 152.16 |  |
| Salem | 9.02 |  |  |  | 1,253.64 |  | 7.10 |  | 193.63 |  |
| Somerset | 246.80 |  |  |  | 3,028.07 |  | 169.98 |  | 328.39 |  |
| Sussex | 27.45 |  |  |  | 2,125.63 |  | 20.94 |  | 314.63 |  |
| Union | 2.04 |  |  |  | 1,271.73 |  | 0.86 |  | 98.69 |  |
| Warren | 4.56 |  |  |  | 2,288.13 |  | 2.28 |  | 366.64 |  |
| ## Total in State | 656.48 |  |  |  | 46,405.06 |  | 343.34 |  | 5,099.40 |  |

Table 2: New Jersey Speciated PM2.5 Data for 2002

| Site | % Carbonaceous | % Natural (Dust) | Ratio (Natural to Carbonaceous) % |
| --- | --- | --- | --- |
| Camden Chester New Brunswick New Brunswick Elizabeth | 38.99 36.69 41.78 41.61 46.92 | 1.49 1.87 1.72 1.83 1.64 | 3.82 5.09 4.11 4.40 3.50 |
| Average | 41.2 | 1.71 | 4.15 |

1993, Morgan Balogh, Timothy Larson, and Fred Mannering, “Analysis of  Fine Particle Matter near Urban Roadways”, Transportation Research Record 1416