The State of New Jersey

## Department of Environmental Protection

2002 Periodic Emission Inventory

- Attachment 11:

- 2002 Area Source Calculation Methodology Sheets

VOC, NOx, CO, SO2, PM2.5, PM10

May 2006

TABLE OF CONTENTS

Fuel Combustion

Industrial Anthracite Coal Combustion	1

Industrial Subbituminous/Bituminous/Lignite Coal Combustion	3

Industrial Distillate Oil Combustion	6

Industrial Residual Oil Combustion	8

Industrial Natural Gas Combustion	10

Industrial Liquified Petroleum Gas Combustion	12

Industrial Kerosene Combustion	14

Commercial/Institutional Anthracite Coal Combustion	16

Commercial/Institutional Bituminous/Subbituminous/Lignite Coal Combustion	18

Commercial/Institutional Distillate Oil Combustion	21

Commercial/Institutional Residual Oil Combustion	23

Commercial/Institutional Natural Gas Combustion	25

Commercial/Institutional Liquified Petroleum Gas Combustion	27

Commercial/Institutional Kerosene Combustion	29

Residential Anthracite Coal Combustion	31

Residential Distillate Oil Combustion	33

Residential Natural Gas Combustion	35

Residential Liquified Petroleum Gas Combustion	37

Residential Wood Combustion	39

Residential Kerosene Oil Combustion	41

Aircraft Refueling

Aircraft Refueling Aviation Gasoline	43

Aircraft Refueling Jet Kerosene	45

Bioprocess Emission Sources

Bakeries	47

Breweries	49

Wineries	51

Distilleries	53

Surface Coating

Architectural Surface Coating	55

Auto Refinishing Surface Coating	57

Architectural Surface Coating,Traffic Paints	59

Industrial Surface Coating Factory Finished Wood	61

Industrial Surface Coating Wood And Metal Furniture & Fixtures	63

Industrial Surface Coating Metal Containers	65

Industrial Surface Coating Sheet, Strip, & Coil	67

Industrial Surface Coating Machinery & Equipment	69

Industrial Surface Coating Appliances	71

Industrial Surface Coating Electrical Insulation	73

Industrial Surface Coating New Automobile	75

Industrial Surface Coating Other Transportational Equipment	77

Industrial Surface Coating Marine Coatings	79

Industrial Surface Coating Other Product Surface Coating	81

Architectural and Industrial Surface Coating, High Performance Maintenance	83

Architectural and Industrial Surface Coating, Other Special Purpose Surface Coating	85

Solvent Utilization

Degreasing	87

Commercial/Industrial Dry Cleaning	89

Graphic Arts (Offset Lithography/Letterpress)	91

Graphic Arts (Rotogravure/Flexography)	93

Industrial Adhesives	95

Asphalt Application Paving Asphalt	96

Asphalt Application Cutback Asphalt	98

Asphalt Application Emulsified Asphalt	100

Asphalt Application Roofing Asphalt	102

Pesticide Application, Agricultural	104

Pesticide Application, Golf Courses	106

Pesticide Application, Lawn And Garden	108

Pesticide Application, Mosquito Control	110

Pesticide Application, Right-Of-Way	112

Consumer Products	114

Fuel Storage, Transport Loading and Unloading

Portable Fuel Containers	116

Gasoline Handling & Marketing, Tank Truck Unloading Loss (Balanced Submerged Filling Or Stage I)	120

Gasoline Handling & Marketing, Tank Breathing	122

Gasoline Handling & Marketing, Transport By Rail Tank Car And Truck	124

Petroleum Transport, Marine Vessel-Crude Oil Loading	126

Petroleum Transport, Marine Vessel-Residual Oil Loading	129

Petroleum Transport, Marine Vessel-Distillate Oil Loading	132

Petroleum Transport, Marine Vessel-Gasoline Loading	135

Petroleum Transport, Marine Vessel-Jet Naphtha Loading	138

Petroleum Transport, Marine Vessel-Kerosene Loading	141

Petroleum Transport Marine Vessel-Crude Oil Transit	144

Petroleum Transport, Marine Vessel-Residual Oil Transit	147

Petroleum Transport, Marine Vessel-Distillate Oil Transit	150

Petroleum Transport, Marine Vessel-Gasoline Transit	153

Petroleum Transport, Marine Vessel-Jet Naphtha Transit	156

Petroleum Transport, Marine Vessel-Kerosene Transit	159

Petroleum Transport, Marine Vessel-Crude Oil Ballasting	162

Petroleum Transport, Marine Vessel-Gasoline Ballasting	165

Waste Disposal, Treatment and Recovery

On-Site Incineration, Industrial	168

On-Site Incineration, Apartment-Solid Waste Incinerator	170

On-Site Incineration, Pathological Incinerator	172

On-Site Incineration, Sewage Sludge	174

Open Burning	176

Landfills	178

Industrial Treatment Works (ITWs)	181

Public Owned Treatment Works (POTWs)	183

Leaking Underground Storage Tanks	186

Miscellaneous Combustion

Agricultural Field Burning, Land Clearing	188

Agricultural Field Burning, Herbaceous	190

Agricultural Field Burning, Infested	192

Agricultural Field Burning, Orchard	194

Forest Wildfires	196

Managed Burning	198

Restaurant Operations	200

Structural Fires	202

Vehicle Fires	204

Cigarette Smoking	206

# Fugitive Dust

Paved Roads	208

Unpaved Roads	211

Construction -  Residential	213

Construction - Commercial	216

Construction – Heavy Miscellaneous	218

Construction -  Roadway	221

Mining And Quarrying – Sand And Gravel	223

Mining And Quarrying - Stone	225

Agricultural Tilling	228

Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Industrial Anthracite Coal Combustion

SCC:  2102001000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC), oxides of nitrogen (NOx), carbon monoxide (CO), sulfur dioxide (SO2), particulate matter less than or equal to 10 microns (PM10), particulate matter less than or equal to 2.5 microns (PM2.5) and lead (Pb).  The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation methodology sheet.

Calculation Methodology:

The following calculation methodologies are recommended for this source category: 1

1.  Surveying individual facilities

2.  Fuel consumption analysis

The fuel consumption analysis/methodology is selected because the input data elements are more readily available.

Emissions from the combustion of anthracite coal in industrial boilers are calculated using statewide tons of coal burned, allocated to the county level by industrial employees.

- Required Input Parameters:

- The following input data is required to calculate emissions for this source category.

1.  Total statewide industrial anthracite coal use, COAL 2 (tons)

2.  County and statewide employment totals for SIC 10-3999, 461, 4939 or NAICS 21- 339999, 4861, 4869, CEMP, NJEMP 3

3.  Emission Factors, EF, (lbs/ton of coal burned)

Emission Factor							Reference

VOC = 		0.07						4 SCC 10100102 stoker-fired boiler

NOx = 		9.0						5 Table 1.2-1 stoker fired boiler

CO = 		0.60						5 Table 1.2-2

PM10 =		4.75						5  Figure 1.2-1 total particulate

PM2.5 =	2.4 						5  Figure 1.2-1 total particulate

SO2 = 39*(wt% sulfur, S)*0.95(weight fraction of SO2 in SOx)	5 Table 1.2-1 and 1.1-3

Pb  = 		0.0089					 	4

4.  Weekly activity factor, WAF=6 days/week 6

5.  Seasonal adjustment factors, SAF 2

Summer Season Adjustment Factor 		0.97

Fall Season Adjustment Factor 			1.02

Winter Season Adjustment Factor 		1.02

Spring Season Adjustment Factor 		0.99

6.  Wt% sulfur, S = 0.8 7

- Process:

The following equations are used to calculate the emissions without control for this source category.

Epa	= COAL*(EF/CF)*(CEMP/NJEMP)

Eps	= Epa*SAF/AADF

Epw	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

Epw	= (tons/day) for a typical winter day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1.  Employment data is representative of the industrial establishments surveyed by Department of Energy in the preparation of the State Energy Data Report. 8

2.  All industrial anthracite coal burners are uncontrolled overfeed stokers.

3.  All industrial coal consumption included in the State Energy Data Report is bituminous coal.

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:

New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 10, N.J.A.C. 7:27-10, Sulfur in Solid Fuels, New Jersey State Department of Environmental Protection

- Double Counting:

Emissions for this source category have also been reported in the point source inventory.  The following methodology is used to adjust the area source emissions for this category to remove the emissions already accounted for in the point source inventory:

Epadj	= (COAL-PT)*(EF/CF)*(CEMP/NJEMP)

Where:

Epadj	= (tons/yr) for an annual emission of pollutant by county excluding double  counting

PT	= Industrial point sources' anthracite coal use (tons) 9

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

NOx		tons/yr		tons/day		tons/day

CO		tons/yr		tons/day		tons/day

SO2         	tons/yr   	tons/day		tons/day

PM10		tons/yr   	tons/day		tons/day

PM2.5		tons/yr   	tons/day		tons/day

- References:

1. Procedures for Emission Inventory Preparation Vol III:  Area Sources, September 1981, United States Environmental Protection Agency, Office of  Air Quality Planning and Standards, Research Triangle Park, NC, EPA-450/4-81-026

2. State Energy Data 2000 Consumption Tables (formerly the State Energy Data Report), United States Department of Energy, Energy Information Administration, Office of Energy Markets and End Use, Washington, DC

3. Total 2002 employment by 6 digit NAICS code and by county, New Jersey  Department of Labor.

4.  Factor Information Retrieval (FIRE) system, United States Environmental   Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

5.  Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

6. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction,  Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c

7. New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 10, N.J.A.C. 7:27-10, Sulfur in Solid Fuels, New Jersey State Department of Environmental Protection

8. Memo to file concerning discussion on fuel use sale data with Department of Energy officials, June 1,1999

9.  NJ Point Source Emission Inventory for 2002, New Jersey Department of Environmental Protection

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Industrial Subbituminous/Bituminous/Lignite Coal Combustion

- SCC:  2102001000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC), oxides of nitrogen (NOx), carbon monoxide (CO), sulfur dioxide (SO2), particulate matter less than or equal to 10 microns (PM10), particulate matter less than or equal to 2.5 microns (PM2.5) and lead (Pb).  The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

Calculation Methodology:

The following calculation methodologies are recommended for this source category:1

1.  Surveying individual facilities

2.  Fuel consumption analysis

The fuel consumption analysis/methodology is selected because the input data elements are more readily available.

Emissions from the combustion of bituminous coal in industrial boilers are calculated using statewide tons of coal burned, allocated to the county level by industrial employees.

- Required Input Parameters:

- The following input data is required to calculate emissions for this source category.

1.  Total statewide industrial bituminous/subbituminous/lignite coal use, COAL (tons)2

2. County and statewide employment totals for SIC 10-3999, 461, 4939 or NAICS 21- 339999, 4861, 4869, CEMP, NJEMP 3

3.  Emission Factors, EF (lbs/ton of coal burned)4

VOC = (0.11 + 0.05 + 0.05 + 1.3 + 0.07 + 0.03 + 0.03) / 7=0.23

which is the average of uncontrolled emission factors for bituminous coal (AP-42 Table 1.1-19) and lignite (AP-42 Table 1.7-1) for the following furnaces: cyclone furnaces, spreader stoker, overfeed stokers, and underfeed stokers

NOx = (33 + 17 + 11 + 8.8 + 7.5 + 9.5 + 15 + 5.8) / 8 = 13.45

CO = (0.5 + 0.5 + 5 + 5 + 6 + 11) / 6 = 4.67

which is the average of uncontrolled emission factors for bituminous coal (AP-42 Table 1.1-3) and lignite (AP-42 Table 1.7-1) for the following furnaces: cyclone furnaces, spreader stoker, overfeed stokers, and underfeed stokers

SO2 = 35*(wt% sulfur, S)*0.95(weight fraction of SO2 in SOx)

PM10 = (13.2 + 6.0 + 6.2) / 3+ (0.04lb/MMBTU*23 MMBTU/ton)  = 9.39

PM2.5 = (4.6 + 2.2 + 3.8) / 3 + (0.04lb/MMBTU*23 MMBTU/ton)  = 4.45

which is the average of uncontrolled emission factors for bituminous coal for the following furnaces: spreader stoker (AP-42 Table 1.1-9), overfeed stokers (AP-42 Table 1.1-10), and underfeed stokers (AP-42 Table 1.1-11) for filterable particulates plus the condensible particulates in Table 1.1-5.

Pb = 0.0133

4.  Weekly activity, WAF=6 days/week5

5.  Seasonal adjustment factor, SAF 2

Summer Season Adjustment Factor		0.97

Fall Season Adjustment Factor			1.02

Winter Season Adjustment Factor		1.02

Spring Season Adjustment Factor			0.99

6.  Wt% sulfur, S= 6	1.0:	Atlantic, Cape May, Cumberland, Ocean Counties,

Hunterdon, Sussex, and Warren Counties 			 		  		0.2:	Burlington, Camden, Gloucester, Mercer, Salem,

Bergen, Essex, Hudson, Middlesex, Monmouth, Morris, 						  	Passaic, Somerset, and Union Counties

- Process:

The following equations are used to calculate the emissions without control for this source category.

Epa	=  COAL*(EF/CF)*(CEMP/NJEMP)

Eps	=  Epa*SAF/AADF

Epw	=  Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

Epw	= (tons/day) for a typical winter day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1.  Employment data is representative of the industrial establishments surveyed by Department of Energy in the preparation of the State Energy Data Report. 7

2.  All industrial coal consumption included in the State Energy Data Report is bituminous coal.

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:

New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 10, N.J.A.C. 7:27-10, Sulfur in Solid Fuels, New Jersey State Department of Environmental Protection

- Double Counting:

Emissions for this source category have also been reported in the point source inventory.  The following methodology is used to adjust the area source emissions for this category to remove the emissions already accounted for in the point source inventory.

Epadj	= (COAL-PT)*(EF/CF)*(CEMP/NJEMP)

Where:

Epadj	= (tons/yr) for an annual emission of pollutant by county excluding double counting

PT	= Industrial point sources' subbituminous/bituminous/lignite coal use (tons) 8

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

NOx		tons/yr		tons/day		tons/day

CO		tons/yr		tons/day		tons/day

SO2         	tons/yr   	tons/day		tons/day

PM10		tons/yr   	tons/day		tons/day

PM2.5		tons/yr   	tons/day		tons/day

- References:

1.  Procedures for Emission Inventory Preparation Vol III:  Area Sources, September 1981, United States Environmental Protection Agency, Office of  Air Quality Planning and Standards, Research Triangle Park, NC,

- EPA-450/4-81-026

2. State Energy Data 2000 Consumption Tables (formerly the State Energy Data Report), United States Department of Energy, Energy Information Administration, Office of Energy Markets and End Use, Washington, DC

3. Total 2002 employment by 6 digit NAICS code and by county, New Jersey  Department of Labor.

4.  Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

5. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction,  Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c

6. New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 10, N.J.A.C. 7:27-10, Sulfur in Solid Fuels, New Jersey State Department of Environmental Protection

7. Memo to file concerning discussion on fuel use sale data with Department of Energy officials, June 1,1999

8.  NJ Point Source Emission Inventory for 2002, New Jersey Department of Environmental Protection

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Industrial Distillate Oil Combustion

- SCC:  2102004000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC), oxides of nitrogen (NOx), carbon monoxide (CO), sulfur dioxide (SO2), particulate matter less than or equal to 10 microns (PM10), particulate matter less than or equal to 2.5 microns (PM2.5) and lead (Pb).  The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

Calculation Methodology:

The following calculation methodologies are recommended for this source category:1

1.  Surveying individual facilities

2.  Fuel consumption analysis

The fuel consumption analysis/methodology is selected because the input data elements are more readily available.

Emissions from the combustion of distillate oil coal in industrial boilers are calculated using statewide gallons of fuel burned, allocated to the county level by industrial employees.

- Required Input Parameters:

- The following input data is required to calculate emissions for this source category.

1.  Total statewide industrial distillate oil use, FUEL2 (103 gallons)

2. County and statewide employment totals for SIC 10-3999, 461, 4939 or NAICS 21- 339999, 4861, 4869, CEMP, NJEMP 3

3.  Emission Factors, EF (lbs/103 gallons burned)

Emission Factor				Reference

VOC = 		0.2			4 Table 1.3-3 industrial boilers distilate oil fired

NOx = 		20			4 Table 1.3-1 boilers<100 million btu/hr, distillate oil fired

CO = 		5			4 Table 1.3-1 boilers<100 million btu/hr, distillate oil fired

PM10 =	 (1.0+1.3)=2.3			4 Table 1.3-6 and 1.3-2 filterable plus condensible

PM2.5 = (0.25+1.3)=1.55 		4 Table 1.3-6 and 1.3-2 filterable plus condensible

SO2 = 142*(wt% sulfur, S)		4 Table 1.3-1 boilers<100 million btu/hr, distillate oil fired

Pb=  (8.9 lb/1012 BTU) * (1.42 *108  BTU/103 gals) = 0.001264

4.  Weekly activity, WAF=6 days/week5

5. Seasonal adjustment factor, SAF 2

Summer Season Adjustment Factor		0.98

Fall Season Adjustment Factor			0.99

Winter Season Adjustment Factor		1.02

Spring Season Adjustment Factor			1.01

6.  Wt% sulfur content, S = 6	0.3:	Atlantic, Cape May, Cumberland, Ocean Counties,

Hunterdon, Sussex, and Warren Counties 			 		     		0.2:	Burlington, Camden, Gloucester, Mercer, 						                  	Salem,Bergen, Essex, Hudson, Middlesex,Monmouth 					                	Morris,Passaic,Somerset,and Union Counties

- Process:

The following equations are used to calculate the emissions without control  for this source category.

Epa = OIL*(EF/CF)*(CEMP/NJEMP)

Eps = Epa/AADF*SAF

Epw = Epa/AADF*SAF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

Epw	= (tons/day) for a typical winter day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

SAF	= Seasonal adjustment factor

Assumptions:

1.  Employment data is representative of the industrial establishments surveyed by Department of Energy in the preparation of the State Energy Data Report.7

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:

New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 9, N.J.A.C. 7:27-9, Sulfur in Fuels, New Jersey State Department of Environmental Protection

- Double Counting:

Emissions for this source category have also been reported in the point source inventory.  The following methodology is used to adjust the area source emissions for this category to remove the emissions already accounted for in the point source inventory.

Epadj	= (OIL-PT)*(EF/CF)*(CEMP/NJEMP)

Where:

Epadj	= (tons/yr) for an annual emission of pollutant by county excluding double counting

PT	= Industrial point sources' distillate oil use8 (103 gallons)

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

NOx		tons/yr		tons/day		tons/day

CO		tons/yr		tons/day		tons/day

SO2         	tons/yr   	tons/day		tons/day

PM10		tons/yr   	tons/day		tons/day

PM2.5		tons/yr   	tons/day		tons/day

- References:

1.  Procedures for Emission Inventory Preparation Vol III:  Area Sources, September 1981, United States Environmental       Protection Agency, Office of  Air Quality Planning and Standards, Research Triangle Park, NC,

- EPA-450/4-81-026

2. State Energy Data 2000 Consumption Tables (formerly the State Energy Data Report), United States Department of Energy, Energy Information Administration, Office of Energy Markets and End Use, Washington, DC

3. Total 2002 employment by 6 digit NAICS code and by county, New Jersey  Department of Labor.

4. Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, , United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

5. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction,  Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c

6. New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 9, N.J.A.C. 7:27-9, Sulfur in Fuels, New Jersey State Department of Environmental Protection

7. Memo to file concerning discussion on fuel use sale data with Department of Energy officials, June 1,1999

8.  NJ Point Source Emission Inventory for 2002, New Jersey Department of Environmental Protection

Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Industrial Residual Oil Combustion

- SCC:  2102005000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC), oxides of nitrogen (NOx), carbon monoxide (CO), sulfur dioxide (SO2), particulate matter less than or equal to 10 microns (PM10), particulate matter less than or equal to 2.5 microns (PM2.5) and lead (Pb).  The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

Calculation Methodology:

The following calculation methodologies are recommended for this source category:1

1.  Surveying individual facilities

2.  Fuel consumption analysis

The fuel consumption analysis/methodology is selected because the input data elements are more readily available.

Emissions from the combustion of residual oil in industrial boilers are calculated using statewide gallons of fuel burned, allocated to the county level by industrial employees.

- Required Input Parameters:

1.  Total statewide industrial residual oil use, OIL2 (103 gallons)

2. County and statewide employment totals for SIC 10-3999, 461, 4939 or NAICS 21- 339999, 4861, 4869, CEMP, NJEMP 3

3.  Emission Factors, EF (lbs/103 gallons burned)

Emission Factor					Reference

VOC = 		0.28				4 Table 1.3-3 industrial  no 6 oil fired

NOx = 		55				4 Table 1.3-1 boilers<100 million btu/hr, no 6 oil fired

CO = 		5				4 Table 1.3-1 boilers<100 million btu/hr, no 6 oil fired

PM10=(7.17*(1.12*Wt%sulfur, S)+0.37))+1.5	4 Table 1.3-2 and 5 filterable plus condensible

PM2.5=(4.67*(1.12*Wt%sulfur, S)+0.37))+1.5	4 Table 1.3-2 and 5 filterable plus condensible

SO2 = 157*(Wt% sulfur, S)			4 Table 1.3-1 boilers<100 million btu/hr, no 6 oil fired

Pb=( (28+194)/2 lb/1012 BTU)*(1.51/108 BTU/103 gals)= 111*1.51*10-4lb /103 gals = 0.01676 lb/103 gals5

4.  Weekly activity factor, WAF=6 days/week5

5. Seasonal adjustment factor, SAF 2

Summer Season Adjustment Factor		0.98

Fall Season Adjustment Factor			0.99

Winter Season Adjustment Factor		1.02

Spring Season Adjustment Factor			1.01

6.  Wt% sulfur content, S= 6	2.0:	Atlantic, Cape May, Cumberland, and Ocean Counties

1.0:	Hunterdon, Sussex, Warren, and Salem Counties

0.5:	Burlington County, Camden, Gloucester, and Mercer Counties except those municipalities included below in Zone 6

0.3:	The municipalities of Bass River, Shamong, Southampton, Tabernacle, 				  	Washington, Woodland Townships in Burlington County, and Waterford 				  	Township in Camden County, Bergen, Essex, Hudson, Middlesex,               				  	Monmouth,Morris, Passaic, Somerset and Union Counties.

- Process:

The following equations are used to calculate the emissions without control for this source category.

Epa	= OIL*(EF/CF)*(CEMP/NJEMP)

Eps	= Epa*SAF/AADF

Epw	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

Epw	= (tons/day) for a typical winter day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1.  Employment data is representative of the industrial establishments surveyed by Department of Energy in the preparation of the State Energy Data Report.7

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:

New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 9, N.J.A.C. 7:27-9, Sulfur in Fuels, New Jersey State Department of Environmental Protection

- Double Counting:

Emissions for this source category have also been reported in the point source inventory.  The following methodology is used to adjust the area source emissions for this category to remove the emissions already accounted for in the point source inventory.

Epadj	= (OIL-PT)*(EF/CF)*(CEMP/NJEMP)

Where:

Epadj	= (tons/yr) for an annual emission of pollutant by county excluding double counting

PT	= Industrial point sources' residual oil use8 (103 gallons)

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

NOx		tons/yr		tons/day		tons/day

CO		tons/yr		tons/day		tons/day

SO2         	tons/yr   	tons/day		tons/day

PM10		tons/yr   	tons/day		tons/day

PM2.5		tons/yr   	tons/day		tons/day

- References:

1.  Procedures for Emission Inventory Preparation Vol III:  Area Sources, September 1981, United States Environmental Protection Agency, Office of  Air Quality Planning and Standards, Research Triangle Park, NC,

- EPA-450/4-81-026

2. State Energy Data 2000 Consumption Tables (formerly the State Energy Data Report), United States Department of Energy, Energy Information Administration, Office of Energy Markets and End Use, Washington, DC

3. Total 2002 employment by 6 digit NAICS code and by county, New Jersey  Department of Labor.

4. Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

5. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction,  Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c

6. New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 9, N.J.A.C. 7:27-9, Sulfur in Fuels, New Jersey State Department of Environmental Protection

7. Memo to file concerning discussion on fuel use sale data with Department of Energy officials, June 1,1999

8.  NJ Point Source Emission Inventory for 2002, New Jersey Department of Environmental Protection

Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Industrial Natural Gas Combustion

- SCC:  2102006000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC), oxides of nitrogen (NOx), carbon monoxide (CO), sulfur dioxide (SO2), particulate matter less than or equal to 10 microns (PM10), particulate matter less than or equal to 2.5 microns (PM2.5) and lead (Pb).  The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

Calculation Methodology:

The following calculation methodologies are recommended for this source category:1

1.  Surveying individual facilities

2.  Fuel consumption analysis

The fuel consumption analysis/methodology is selected because the input data elements are more readily available.

Emissions from the combustion of industrial natural gas are calculated using statewide amount of fuel burned, allocated to the county level by industrial employees.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  Total statewide industrial natural gas use, GAS2 (106 cubic feet)

2. County and statewide employment totals for SIC 10-3999, 461, 4939 or NAICS 21- 339999, 4861, 4869, CEMP, NJEMP 3

3.  Emission Factors, EF (lbs/106 cubic feet burned)

Emission Factor				Reference

VOC = 		5.5			4 Table 1.4-2

NOx = 		100			4 Table 1.4-1 boilers<100 million btu/hr

CO = 		84			4 Table 1.4-1 boilers<100 million btu/hr

PM10 =		7.6			4 Table 1.4-2

PM2.5 =	7.6			4 Table 1.4-2

SO2 = 		0.6			4 Table 1.4-2

Pb = 		0.0005

4.  Weekly activity, WAF=6 days/week4

5. Seasonal adjustment factor, SAF 2

Summer Season Adjustment Factor		0.92

Fall Season Adjustment Factor			0.93

Winter Season Adjustment Factor		1.17

Spring Season Adjustment Factor			0.99

- Process:

The following equations are used to calculate the emissions without control for this source category.

Epa	= GAS*(EF/CF)*(CEMP/NJEMP)

Eps	= Epa*SAF/AADF

Epw	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

Epw	= (tons/day) for a typical winter day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1.  Employment data is representative of the industrial establishments surveyed by Department of Energy in the preparation of the State Energy Data Report.5

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:   None

- Double Counting:

Emissions for this source category have also been reported in the point source inventory.  The following methodology is used to adjust the area source emissions for this category to remove the emissions already accounted for in the point source inventory.

Epadj	= (GAS-PT)*(EF/CF)*(CEMP/NJEMP)

Where:

Epadj	= (tons/yr) for an annual emission of pollutant by county excluding double counting

PT	= Industrial point sources' natural gas use (106 cubic feet)6

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

NOx		tons/yr		tons/day		tons/day

CO		tons/yr		tons/day		tons/day

SO2         	tons/yr   	tons/day		tons/day

PM10		tons/yr   	tons/day		tons/day

PM2.5		tons/yr   	tons/day		tons/day

- References:

1.  Procedures for Emission Inventory Preparation Vol III:  Area Sources, September 1981, United States Environmental       Protection Agency, Office of  Air Quality Planning and Standards, Research Triangle Park, NC,

- EPA-450/4-81-026

2. State Energy Data 2000 Consumption Tables (formerly the State Energy Data Report), United States Department of Energy, Energy Information Administration, Office of Energy Markets and End Use, Washington, DC

3.  Total 2002 employment by 6 digit NAICS code and by county, New Jersey  Department of Labor.

4. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction,  Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c

5. Memo to file concerning discussion on fuel use sale data with Department of Energy officials, June 1,1999

6. NJ Point Source Emission Inventory for 2002, New Jersey Department of Environmental Protection

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Industrial Liquified Petroleum Gas Combustion

- SCC:  2102007000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC), oxides of nitrogen (NOx), carbon monoxide (CO), sulfur dioxide (SO2), particulate matter less than or equal to 10 microns (PM10) and particulate matter less than or equal to 2.5 microns (PM2.5).  The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

Calculation Methodology:

The following calculation methodologies are recommended for this source category:1

1.  Surveying individual facilities

2.  Fuel consumption analysis

The fuel consumption analysis/methodology is selected because the input data elements are more readily available.

Emissions from the combustion of industrial liquid petroleum gas are calculated using statewide amount of fuel burned, allocated to the county level by industrial employees.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  Total statewide industrial liquid petroleum gas use, LPG (103 gallons)2

2. County and statewide employment totals for SIC 10-3999, 461, 4939 or NAICS 21- 339999, 4861, 4869, CEMP, NJEMP 3

3.  Emission Factors, EF, (lbs/103 gallons burned)

Emission Factor				Reference

VOC=(0.6-0.2)+(0.5-0.2)/2=0.35		4 Table 1.5-1, industrial boilers

NOx=(21+19)/2=20			4 Table 1.5-1, industrial boilers

CO =(3.6+3.2)/2=3.4			4 Table 1.5-1, industrial boilers

PM10=(0.6+0.6)/2=0.6			4 Table 1.5-1, industrial boilers, filterable particulate

PM2.5=(0.6+0.6)/2=0.6			4 Table 1.5-1, industrial boilers, filterable particulate

SO2=(0.09S+0.10S)/2=0.23		4 Table 1.5-1, industrial boilers

4.  Weekly activity, WAF=6 days/week5

5. Seasonal adjustment factor, SAF 2

Summer Season Adjustment Factor		0.98

Fall Season Adjustment Factor			0.99

Winter Season Adjustment Factor		1.02

Spring Season Adjustment Factor			1.01

6.  Wt% sulfur content, S=2.456 gr/100cf  6

- Process:

The following equations are used to calculate the emissions without control  for this source category.

Epa	= LPG*(EF/CF)*(CEMP/NJEMP)

Eps	= Epa*SAF/AADF

Epw	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

Epw	= (tons/day) for a typical winter day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1.  Employment data is representative of the industrial establishments surveyed by Department of Energy in the preparation of the State Energy Data Report.7

2.  Assumed 50/50 mixture of butane and propane in the development of the emission factors.

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:   None

- Double Counting:

Emissions for this source category have also been reported in the point source inventory.  The following methodology is used to adjust the area source emissions for this category to remove the emissions already accounted for in the point source inventory.

Epadj	= (LPG-PT)*(EF/CF)*(CEMP/NJEMP)

Where:

Epadj	= (tons/yr) for an annual emission of pollutant by county excluding double counting

PT	= Industrial point sources' liquified petroleum gas use (103 gallons)8

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

NOx		tons/yr		tons/day		tons/day

CO		tons/yr		tons/day		tons/day

SO2         	tons/yr   	tons/day		tons/day

PM10		tons/yr   	tons/day		tons/day

PM2.5		tons/yr   	tons/day		tons/day

- References:

1.  Procedures for Emission Inventory Preparation Vol III:  Area Sources, September 1981, United States

Environmental Protection Agency, Office of  Air Quality Planning and Standards, Research Triangle Park, NC,

EPA-450/4-81-026

2. State Energy Data 2000 Consumption Tables (formerly the State Energy Data Report), United States Department of Energy, Energy Information Administration, Office of Energy Markets and End Use, Washington, DC

3.  Total 2002 employment by 6 digit NAICS code and by county, New Jersey  Department of Labor.

4.  Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

5. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction,  Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c

6.  Nonroad Source Inventory Development for Nonroad Engines presentation,

http://www.epa.gov/ttn/chief/eidocs/partisec4.pdf

7. Memo to file concerning discussion on fuel use sale data with Department of Energy officials, June 1,1999

8.  NJ Point Source Emission Inventory for 2002, New Jersey Department of Environmental Protection

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Industrial Kerosene Combustion

- SCC:  2102011000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC), oxides of nitrogen (NOx), carbon monoxide (CO), sulfur dioxide (SO2), particulate matter less than or equal to 10 microns (PM10), particulate matter less than or equal to 2.5 microns (PM2.5) and lead (Pb).  The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

Calculation Methodology:

The following calculation methodologies are recommended for this source category:1

1.  Surveying individual facilities

2.  Fuel consumption analysis

The fuel consumption analysis/methodology is selected because the input data elements are more readily available.

Emissions from the combustion of industrial kerosene are calculated using statewide amount of fuel burned, allocated to the county level by industrial employees.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  Total statewide industrial kerosene oil use, OIL2 (103 gallons)

2. County and statewide employment totals for SIC 10-3999, 461, 4939 or NAICS 21- 339999, 4861, 4869, CEMP, NJEMP 3

3.  Emission Factors, EF (lbs/103 gallons burned)

Emission Factor				Reference

VOC = 		0.2			4 Table 1.3-3 industrial boilers distilate oil fired

NOx = 		20			4 Table 1.3-1 boilers<100 million btu/hr, distillate oil fired

CO = 		5			4 Table 1.3-1 boilers<100 million btu/hr, distillate oil fired

PM10 =	 (1.0+1.3)=2.3			4 Table 1.3-6 and 1.3-2 filterable plus condensible

PM2.5 = (0.25+1.3)=1.55 		4 Table 1.3-6 and 1.3-2 filterable plus condensible

SO2 = 142*(wt% sulfur, S)		4 Table 1.3-1 boilers<100 million btu/hr, distillate oil fired

Pb = ( (28+194)/2 lb/1012 BTU)*(1.51/108  BTU/103 gals) = 111*1.51*10-4lb /103 gals =0.01676 lb/103 gals5

4.  Weekly activity, WAF=6 days/week5

5. Seasonal adjustment factor, SAF 2

Summer Season Adjustment Factor		0.98

Fall Season Adjustment Factor			0.99

Winter Season Adjustment Factor		1.02

Spring Season Adjustment Factor			1.01

6.  Wt% sulfur content, S= 6	0.3:	Atlantic, Cape May, Cumberland, Ocean,

Hunterdon, Sussex, and Warren Counties

0.2: 	Burlington, Camden, Gloucester, Mercer, Salem, 							Bergen,Essex,Hudson,Middlesex,Monmouth, 						                	Morris,Passaic, Somerset, and Union Counties

- Process:

The following equations are used to calculate the emissions without control for this source category.

Epa	= OIL*(EF/CF)*(CEMP/NJEMP)

Eps	= Epa*SAF/AADF

Epw	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

Epw	= (tons/day) for a typical winter day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1.  Employment data is representative of the industrial establishments surveyed by Department of Energy in the preparation of the State Energy Data Report.7

2.  Assume emission factors for kerosene are equivalent to distillate oil emission factors.

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:

New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 9, N.J.A.C. 7:27-9, Sulfur in Fuels, New Jersey State Department of Environmental Protection

- Double Counting:

Emissions for this source category have also been reported in the point source inventory.  The following methodology is used to adjust the area source emissions for this category to remove the emissions already accounted for in the point source inventory.

Epadj	= (OIL-PT)*(EF/CF)*(CEMP/NJEMP)

Where:

Epadj	= (tons/yr) for an annual emission of pollutant by county excluding double counting

PT	= Industrial point sources' kerosene oil use8 (103 gallons)

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

NOx		tons/yr		tons/day		tons/day

CO		tons/yr		tons/day		tons/day

SO2         	tons/yr   	tons/day		tons/day

PM10		tons/yr   	tons/day		tons/day

PM2.5		tons/yr   	tons/day		tons/day

- References:

1.  Procedures for Emission Inventory Preparation Vol III:  Area Sources, September 1981, United States Environmental Protection Agency, Office of  Air Quality Planning and Standards, Research Triangle Park, NC,

- EPA-450/4-81-026

2. State Energy Data 2000 Consumption Tables (formerly the State Energy Data Report), United States Department of Energy, Energy Information Administration, Office of Energy Markets and End Use, Washington, DC

3.  Total 2002 employment by 6 digit NAICS code and by county, New Jersey  Department of Labor.

4.  Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

5. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction,  Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c

6. New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 9, N.J.A.C. 7:27-9, Sulfur in Fuels, New Jersey State Department of Environmental Protection

7. Memo to file concerning discussion on fuel use sale data with Department of Energy officials, June 1,1999

8.  NJ Point Source Emission Inventory for 2002, New Jersey Department of Environmental Protection

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Commercial/Institutional Anthracite Coal Combustion

- SCC:  2103001000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC), oxides of nitrogen (NOx), carbon monoxide (CO), sulfur dioxide (SO2), particulate matter less than or equal to 10 microns (PM10), particulate matter less than or equal to 2.5 microns (PM2.5) and lead (Pb).  The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

Calculation Methodology:

The following calculation methodologies are recommended for this source category:1

1.  Surveying individual facilities

2.  Fuel consumption analysis

The fuel consumption analysis/methodology is selected because the input data elements are more readily available.

Emissions from the combustion of commercial anthracite coal are calculated using statewide amount of fuel burned, allocated to the county level by commercial employees.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  Total statewide commercial/institutional bituminous/subbituminous coal use, COAL2 (tons)

2.  County and statewide employment totals for SIC's 0850, 415,417,422-23,43,458,47-8,494-96,50-97, not including 881, or NAICS 1153, 2213, 42, 44 - 5, 48 - 9, 51 - 6, 61 - 2, 71 - 2, 81, not including 8141, 92, CEMP, NJEMP 3

3.  Emission Factors, EF, (lbs/ton of coal burned)

Emission Factor							Reference

VOC = 		0.07						4 SCC 10300102 commercial

stoker-fired boiler

NOx = 		9.0						5 Table 1.2-1 stoker fired boiler

CO = 		0.60						5 Table 1.2-2

PM10 =		4.75						5  Figure 1.2-1 total particulate

PM2.5 =	2.4 						5  Figure 1.2-1 total particulate

SO2 = 39*(wt% sulfur, S)*0.95(weight fraction of SO2 in SOx)	5 Table 1.2-1 and 1.1-3

Pb  = 		0.0089					 	4

4.  Weekly activity factor, WAF=6 days/week 6

5.  Seasonal adjustment factors, SAF

Summer Season Adjustment Factor		0.82

Fall Season Adjustment Factor			0.87

Winter Season Adjustment Factor		1.36

Spring Season Adjustment Factor		0.94

6.  Wt% sulfur, S = 0.8 7

- Process:

The following equations are used to calculate the emissions without control  for this source category.

Epa	= COAL*(EF/CF)*(CEMP/NJEMP)

Eps	= Epa*SAF /AADF

Epw	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

Epw	= (tons/day) for a typical winter day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1.  Employment data is representative of the commercial establishments surveyed by Department of Energy in the preparation of the State Energy Data Report.8

2.  All commercial  anthracite coal burners are uncontrolled overfeed stokers.

3.  All commercial coal consumption included in the State Energy Data Report is anthracite coal.

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:

New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 10, N.J.A.C. 7:27-10, Sulfur in Solid Fuels, New Jersey State Department of Environmental Protection

- Double Counting:

Emissions for this source category have also been reported in the point source inventory.  The following methodology is used to adjust the area source emissions for this category to remove the emissions already accounted for in the point source inventory.

Epadj	= (COAL-PT)*(EF/CF)*(CEMP/NJEMP)

Where:

Epadj	= (tons/yr) for an annual emission of pollutant by county excluding double counting

PT	= Commercial/Institutional point sources' anthracite coal use9 (tons)

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

NOx		tons/yr		tons/day		tons/day

CO		tons/yr		tons/day		tons/day

SO2         	tons/yr   	tons/day		tons/day

PM10		tons/yr   	tons/day		tons/day

PM2.5		tons/yr   	tons/day		tons/day

- References:

1. Procedures for Emission Inventory Preparation Vol III:  Area Sources, September 1981, United States        Environmental Protection Agency, Office of  Air Quality Planning and Standards, Research Triangle Park, NC,

EPA-450/4-81-026

2. State Energy Data 2000 Consumption Tables (formerly the State Energy Data Report), United States Department of Energy, Energy Information Administration, Office of Energy Markets and End Use, Washington, DC

3.  Total 2002 employment by 6 digit NAICS code and by county, New Jersey  Department of Labor.

4.  Factor Information Retrieval (FIRE) system, United States Environmental   Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

5. Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, , United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

6. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction,  Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c

7. New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 10, N.J.A.C. 7:27-10, Sulfur in Solid Fuels, New Jersey State Department of Environmental Protection

8. Memo to file concerning discussion on fuel use sale data with Department of Energy officials, June 1,1999

9.  NJ Point Source Emission Inventory for 2002, New Jersey Department of Environmental Protection

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Commercial/Institutional Bituminous/Subbituminous/Lignite Coal Combustion

- SCC:  2103002000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC), oxides of nitrogen (NOx), carbon monoxide (CO), sulfur dioxide (SO2), particulate matter less than or equal to 10 microns (PM10), particulate matter less than or equal to 2.5 microns (PM2.5) and lead (Pb).  The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

Calculation Methodology:

The following calculation methodologies are recommended for this source category:1

1.  Surveying individual facilities

2.  Fuel consumption analysis

The fuel consumption analysis/methodology is selected because the input data elements are more readily available.

Emissions from the combustion of commercial bituminous coal are calculated using statewide amount of fuel burned, allocated to the county level by commercial employees.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  Total statewide commercial/institutional bituminous/lignite coal use, COAL2 (tons)

2.  County and statewide employment totals for SIC's 0850, 415,417,422-23,43,458,47-8,494-96,50-97, not including 881, or NAICS 1153, 2213, 42, 44 - 5, 48 - 9, 51 - 6, 61 - 2, 71 - 2, 81, not including 8141, 92, CEMP, NJEMP 3

3.  Emission Factors, EF (lbs/tons burned)

VOC = (0.11 + 0.05 + 0.05 + 1.3 + 0.07 + 0.03 + 0.03) / 7=0.23

which is the average of uncontrolled emission factors for bituminous coal (AP-42 Table 1.1-19) and lignite (AP-42 Table 1.7-1) for the following furnaces: cyclone furnaces, spreader stoker, overfeed stokers, and underfeed stokers

NOx = (33 + 17 + 11 + 8.8 + 7.5 + 9.5 + 15 + 5.8) / 8 = 13.45

CO = (0.5 + 0.5 + 5 + 5 + 6 + 11) / 6 = 4.67

which is the average of uncontrolled emission factors for bituminous coal (AP-42 Table 1.1-3) and lignite (AP-42 Table 1.7-1) for the following furnaces: cyclone furnaces, spreader stoker, overfeed stokers, and underfeed stokers

SO2 = 35*(wt% sulfur, S)*0.95(weight fraction of SO2 in SOx)

PM10 = (13.2 + 6.0 + 6.2) / 3 + (0.04lb/MMBTU*23 MMBTU/ton)  = 9.39

PM2.5 = (4.6 + 2.2 + 3.8) / 3 + (0.04lb/MMBTU*23 MMBTU/ton)  = 4.45

which is the average of uncontrolled emission factors for bituminous coal for the following furnaces: spreader stoker (AP-42 Table 1.1-9), overfeed stokers (AP-42 Table 1.1-10), and underfeed stokers (AP-42 Table 1.1-11) for filterable particulates plus the condensible particulates in Table 1.1-5.

Pb = 0.0133 4

4.  Weekly activity, WAF=6 days/week5

5. Seasonal adjustment factor, SAF 2

Summer Season Adjustment Factor		0.82

Fall Season Adjustment Factor			0.87

Winter Season Adjustment Factor		1.36

Spring Season Adjustment Factor			0.94

6.  Wt% sulfur content, S = 6	1.0	Atlantic, Cape May, Cumberland, Ocean Counties,

Hunterdon, Sussex, and Warren Counties

0.2	Burlington, Camden, Gloucester, Mercer,Salem, Bergen, Essex, Hudson, Middlesex, Monmouth, Morris, Passaic, Somerset, and Union Counties

- Process:

The following equations are used to calculate the emissions without control for this source category.

Epa	= COAL*(EF/CF)*(CEMP/NJEMP)

Eps	= Epa*SAF /AADF

Epw	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

Epw	= (tons/day) for a typical winter day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1.  Employment data is representative of the commercial establishments surveyed by Department of Energy in the preparation of the State Energy Data Report.7

3.  All commercial coal consumption included in the State Energy Data Report is anthracite coal.

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:

New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 10, N.J.A.C. 7:27-10, Sulfur in Solid Fuels, New Jersey State Department of Environmental Protection

- Double Counting:

Emissions for this source category have also been reported in the point source inventory.  The following methodology is used to adjust the area source emissions for this category to remove the emissions already accounted for in the point source inventory.

Epadj	= (COAL-PT)*(EF/CF)*(CEMP/NJEMP)

Where:

Epadj	= (tons/yr) for an annual emission of pollutant by county excluding double  counting

PT	= Commercial/Institutional point sources' bituminous/subbituminous/lignite coal use (tons)8

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

NOx		tons/yr		tons/day		tons/day

CO		tons/yr		tons/day		tons/day

SO2         	tons/yr   	tons/day		tons/day

PM10		tons/yr   	tons/day		tons/day

PM2.5		tons/yr   	tons/day		tons/day

- References:

1. Procedures for Emission Inventory Preparation Vol III:  Area Sources, September 1981, United States        Environmental Protection Agency, Office of  Air Quality Planning and Standards, Research Triangle Park, NC,

EPA-450/4-81-026

2. State Energy Data 2000 Consumption Tables (formerly the State Energy Data Report), United States Department of Energy, Energy Information Administration, Office of Energy Markets and End Use, Washington, DC

3.  Total 2002 employment by 6 digit NAICS code and by county, New Jersey  Department of Labor.

4.  Factor Information Retrieval (FIRE) system, United States Environmental   Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

5. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction,  Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c

6. New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 10, N.J.A.C. 7:27-10, Sulfur in Solid Fuels, New Jersey State Department of Environmental Protection

7. Memo to file concerning discussion on fuel use sale data with Department of Energy officials, June 1,1999

8.  NJ Point Source Emission Inventory for 2002, New Jersey Department of Environmental Protection

Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Commercial/Institutional Distillate Oil Combustion

- SCC:  2103004000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC), oxides of nitrogen (NOx), carbon monoxide (CO), sulfur dioxide (SO2), particulate matter less than or equal to 10 microns (PM10), particulate matter less than or equal to 2.5 microns (PM2.5) and lead (Pb).  The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

Calculation Methodology:

The following calculation methodologies are recommended for this source category:1

1.  Surveying individual facilities

2.  Fuel consumption analysis

The fuel consumption analysis/methodology is selected because the input data elements are more readily available.

Emissions from the combustion of commercial distillate oil are calculated using statewide amount of fuel burned, allocated to the county level by commercial employees.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  Total statewide commercial/institutional distillate oil use, OIL (103 gallons)2,

2.  County and statewide employment totals for SIC's 0850, 415,417,422-23,43,458,47-8,494-96,50-97, not including 881, or NAICS 1153, 2213, 42, 44 - 5, 48 - 9, 51 - 6, 61 - 2, 71 - 2, 81, not including 8141, 92, CEMP, NJEMP 3

3.  Emission Factors, EF (lbs/103 gallons burned)

Emission Factor				Reference

VOC = 		0.34			4 Table 1.3-3 commercial boilers distilate oil fired

NOx = 		20			4 Table 1.3-1 boilers<100 million btu/hr, distillate oil fired

CO = 		5			4 Table 1.3-1 boilers<100 million btu/hr, distillate oil fired

PM10 =	 (1.08+1.3)=2.38		4 Table 1.3-7 and 1.3-2 filterable plus condensible

PM2.5 = (0.83+1.3)=2.13 		4 Table 1.3-7 and 1.3-2 filterable plus condensible

SO2 = 142*(wt% sulfur, S)		4 Table 1.3-1 boilers<100 million btu/hr, distillate oil fired

Pb=  (8.9 lb/1012 BTU) *(1.42*108  BTU/103 gals)=0.0012644

4.  Weekly activity, WAF=6 days/week5

5 Seasonal adjustment factor, SAF 2

Summer Season Adjustment Factor		0.60

Fall Season Adjustment Factor			0.83

Winter Season Adjustment Factor		1.52

Spring Season Adjustment Factor			1.04

6.  Wt% sulfur content, S= 6	0.3:	Atlantic, Cape May, Cumberland, Ocean Counties,

Hunterdon, Sussex, and Warren Counties 			   	0.2:	Burlington, Camden, Gloucester, Mercer, Salem, Bergen, Essex,

Hudson, Middlesex, Monmouth, Morris,Passaic, Somerset, and

Union Counties

- Process:

The following equations are used to calculate the emissions without control for this source category.

Epa	= OIL*(EF/CF)*(CEMP/NJEMP)

Eps	= Epa*SAF/AADF

Epw	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

Epw	= (tons/day) for a typical winter day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1.  Employment data is representative of the commercial establishments surveyed by Department of Energy in the preparation of the State Energy Data Report.7

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:

New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 9, N.J.A.C. 7:27-9, Sulfur in Fuels, New Jersey State Department of Environmental Protection

- Double Counting:

Emissions for this source category have also been reported in the point source inventory.  The following methodology is used to adjust the area source emissions for this category to remove the emissions already accounted for in the point source inventory.

Epadj	= (OIL-PT)*(EF/CF)*(CEMP/NJEMP)

Where:

Epadj	= (tons/yr) for an annual emission of pollutant by county excluding counting

PT	= Commercial/Institutional point sources' distillate oil use (103 gallons)8

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

NOx		tons/yr		tons/day		tons/day

CO		tons/yr		tons/day		tons/day

SO2         	tons/yr   	tons/day		tons/day

PM10		tons/yr   	tons/day		tons/day

PM2.5		tons/yr   	tons/day		tons/day

- References:

1. Procedures for Emission Inventory Preparation Vol III:  Area Sources, September 1981, United States        Environmental Protection Agency, Office of  Air Quality Planning and Standards, Research Triangle Park, NC,

EPA-450/4-81-026

2. State Energy Data 2000 Consumption Tables (formerly the State Energy Data Report), United States Department of Energy, Energy Information Administration, Office of Energy Markets and End Use, Washington, DC

3.  Total 2002 employment by 6 digit NAICS code and by county, New Jersey  Department of Labor.

4.  Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, , United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

5. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction,  Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c

6. New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 9, N.J.A.C. 7:27-9, Sulfur in Fuels, New Jersey State Department of Environmental Protection

7. Memo to file concerning discussion on fuel use sale data with Department of Energy officials, June 1,1999

8.  NJ Point Source Emission Inventory for 2002, New Jersey Department of Environmental Protection

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Commercial/Institutional Residual Oil Combustion

- SCC:  2103005000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC), oxides of nitrogen (NOx), carbon monoxide (CO), sulfur dioxide (SO2), particulate matter less than or equal to 10 microns (PM10), particulate matter less than or equal to 2.5 microns (PM2.5) and lead (Pb).  The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

Calculation Methodology:

The following calculation methodologies are recommended for this source category:1

1.  Surveying individual facilities

2.  Fuel consumption analysis

The fuel consumption analysis/methodology is selected because the input data elements are more readily available.

Emissions from the combustion of commercial residual oil are calculated using statewide amount of fuel burned, allocated to the county level by commercial employees.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  Total statewide commercial/institutional residual oil use, OIL (103 gallons) 2

2.  County and statewide employment totals for SIC's 0850, 415,417,422-23,43,458,47-8,494-96,50-97, not including 881, or NAICS 1153, 2213, 42, 44 - 5, 48 - 9, 51 - 6, 61 - 2, 71 - 2, 81, not including 8141, 92, CEMP, NJEMP 3

3.  Emission Factors, EF, (lbs/103 gallons burned)

Emission Factor					Reference

VOC = 		1.13				4 Table 1.3-3 commercial boiler  no 6 oil fired

NOx = 		55				4 Table 1.3-1 boilers<100 million btu/hr, no 6 oil fired

CO = 		5				4 Table 1.3-1 boilers<100 million btu/hr, no 6 oil fired

PM10=(5.17*(1.12*Wt%sulfur, S)+0.37))+1.5	4 Table 1.3-2 and 7 filterable plus condensible

PM2.5=(1.92*(1.12*Wt%sulfur, S)+0.37))+1.5	4 Table 1.3-2 and 7 filterable plus condensible

SO2 = 157*(Wt% sulfur, S)			4 Table 1.3-1 boilers<100 million btu/hr, no 6 oil fired

Pb = ((28+194)/2 lb)/1012 BTU*1.51/108  BTU/103 gals= 111*1.51*10-4lb /103 gals =0.01676 lb/103 gals4

4.  Weekly activity, WAF=6 days/week5

5. Seasonal adjustment factor, SAF 2

Summer Season Adjustment Factor		0.60

Fall Season Adjustment Factor			0.83

Winter Season Adjustment Factor		1.52

Spring Season Adjustment Factor			1.04

- 6.  Wt% sulfur content, S= 6	2.0:	Atlantic, Cape May, Cumberland, and Ocean Counties

1.0: 	Hunterdon, Sussex, Warren, and Salem Counties

0.5:	Burlington County, Camden, Gloucester, and 							  	Mercer Counties except those municipalities included below in Zone 6

0.3: 	The municipalities of Bass River, Shamong, Southampton, Tabernacle,

Washington, Woodland Townships in Burlington County, and Waterford

Township in Camden County, Bergen, Essex, Hudson, Middlesex,

Monmouth, Morris, Passaic, Somerset and Union Counties.

- Process:

The following equations are used to calculate the emissions without control  for this source category.

Epa	= OIL*(EF/CF)*(CEMP/NJEMP)

Eps	= Epa*SAF/AADF

Epw	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

Epw	= (tons/day) for a typical winter day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1.  Employment data is representative of the commercial establishments surveyed by Department of Energy in the preparation of the State Energy Data Report.7

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:

New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 9, N.J.A.C. 7:27-9, Sulfur in Fuels, New Jersey State Department of Environmental Protection

- Double Counting:

Emissions for this source category have also been reported in the point source inventory.  The following methodology is used to adjust the area source emissions for this category to remove the emissions already accounted for in the point source inventory.

Epadj	= (OIL-PT)*(EF/CF)*(CEMP/NJEMP)

Where:

Epadj	= (tons/yr) for an annual emission of pollutant by county excluding double counting

PT	= Commercial/Institutional point sources' residual oil use (103 gallons)8

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

NOx		tons/yr		tons/day		tons/day

CO		tons/yr		tons/day		tons/day

SO2         	tons/yr   	tons/day		tons/day

PM10		tons/yr   	tons/day		tons/day

PM2.5		tons/yr   	tons/day		tons/day

- References:

1.  Procedures for Emission Inventory Preparation Vol III:  Area Sources, September 1981, United States        Environmental Protection Agency, Office of  Air Quality Planning and Standards, Research Triangle Park, NC,

EPA-450/4-81-026

2. State Energy Data 2000 Consumption Tables (formerly the State Energy Data Report), United States Department of Energy, Energy Information Administration, Office of Energy Markets and End Use, Washington, DC

3.  Total 2002 employment by 6 digit NAICS code and by county, New Jersey  Department of Labor.

4.  Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, , United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

5. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction,  Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c

6. New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 9, N.J.A.C. 7:27-9, Sulfur in Fuels, New Jersey State Department of Environmental Protection

7. Memo to file concerning discussion on fuel use sale data with Department of Energy officials, June 1,1999

8.  NJ Point Source Emission Inventory for 2002, New Jersey Department of Environmental Protection

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Commercial/Institutional Natural Gas Combustion

- SCC:  2103006000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC), oxides of nitrogen (NOx), carbon monoxide (CO), sulfur dioxide (SO2), particulate matter less than or equal to 10 microns (PM10), particulate matter less than or equal to 2.5 microns (PM2.5) and lead (Pb).  The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

Calculation Methodology:

The following calculation methodologies are recommended for this source category:1

1.  Surveying individual facilities

2.  Fuel consumption analysis

The fuel consumption analysis/methodology is selected because the input data elements are more readily available.

Emissions from the combustion of commercial natural gas are calculated using statewide amount of fuel burned, allocated to the county level by commercial employees.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  Total statewide commercial/institutional natural gas use, GAS (106 cubic feet)2

2.  County and statewide employment totals for SIC's 0850, 415,417,422-23,43,458,47-8,494-96,50-97, not including 881, or NAICS 1153, 2213, 42, 44 - 5, 48 - 9, 51 - 6, 61 - 2, 71 - 2, 81, not including 8141, 92, CEMP, NJEMP 3

3.  Emission Factors, EF, (lbs/106 cubic feet burned)

Emission Factor				Reference

VOC = 		5.5			4 Table 1.4-2

NOx = 		100			4 Table 1.4-1 boilers<100 million btu/hr

CO = 		84			4 Table 1.4-1 boilers<100 million btu/hr

PM10 =		7.6			4 Table 1.4-2

PM2.5 =	7.6			4 Table 1.4-2

SO2 = 		0.6			4 Table 1.4-2

Pb=0.0005

4.  Weekly activity, WAF=6 days/week 5

5. Seasonal adjustment factor, SAF 2

Summer Season Adjustment Factor		0.50

Fall Season Adjustment Factor			0.83

Winter Season Adjustment Factor		1.67

Spring Season Adjustment Factor			1.00

- Process:

The following equations are used to calculate the emissions (without rule effectiveness) for this source category.

Epa	= GAS*(EF/CF)*(CEMP/NJEMP)

Eps	= Epa*SAF /AADF

Epw	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

Epw	= (tons/day) for a typical winter day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1.  Employment data is representative of the commercial establishments surveyed by Department of Energy in the preparation of the State Energy Data Report. 6

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:  None

- Double Counting:

Emissions for this source category have also been reported in the point source inventory.  The following methodology is used to adjust the area source emissions for this category to remove the emissions already accounted for in the point source inventory.

Epadj	= (GAS-PT)*(EF/CF)*(CEMP/NJEMP)

Where:

Epadj	= (tons/yr) for an annual emission of pollutant by county excluding double  counting

PT	= Commercial/Institutional point sources' natural gas use (106 cubic feet)7

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

NOx		tons/yr		tons/day		tons/day

CO		tons/yr		tons/day		tons/day

SO2         	tons/yr   	tons/day		tons/day

PM10		tons/yr   	tons/day		tons/day

PM2.5		tons/yr   	tons/day		tons/day

- References:

1.  Procedures for Emission Inventory Preparation Vol III:  Area Sources, September 1981, United States Environmental Protection Agency, Office of  Air Quality Planning and Standards, Research Triangle Park, NC, EPA-450/4-81-026

2. State Energy Data 2000 Consumption Tables (formerly the State Energy Data Report), United States Department of Energy, Energy Information Administration, Office of Energy Markets and End Use, Washington, DC

3.  Total 2002 employment by 6 digit NAICS code and by county, New Jersey  Department of Labor.

4.  Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

5. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction,  Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c

6. Memo to file concerning discussion on fuel use sale data with Department of Energy officials, June 1,1999

7.  NJ Point Source Emission Inventory for 2002, New Jersey Department of Environmental Protection

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Commercial/Institutional Liquified Petroleum Gas Combustion

- SCC:  2103007000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC), oxides of nitrogen (NOx), carbon monoxide (CO), sulfur dioxide (SO2), particulate matter less than or equal to 10 microns (PM10), particulate matter less than or equal to 2.5 microns (PM2.5) and lead (Pb).  The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

Calculation Methodology:

The following calculation methodologies are recommended for this source category:1

1.  Surveying individual facilities

2.  Fuel consumption analysis

The fuel consumption analysis/methodology is selected because the input data elements are more readily available.

Emissions from the combustion of commercial liquid petroleum gas are calculated using statewide amount of fuel burned, allocated to the county level by commercial employees.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  Total statewide commercial/institutional liquified petroleum gas use, LPG (103 gallon)2

2.  County and statewide employment totals for SIC's 0850, 415,417,422-23,43,458,47-8,494-96,50-97, not including 881, or NAICS 1153, 2213, 42, 44 - 5, 48 - 9, 51 - 6, 61 - 2, 71 - 2, 81, not including 8141, 92, CEMP, NJEMP 3

3.  Emission Factors, EF (lbs/103 gallon burned)

Emission Factor				Reference

VOC=(0.6-0.2)+(0.5-0.2)/2=0.35		4 Table 1.5-1, commercial boilers

NOx=(15+14)/2=14.5			4 Table 1.5-1, commercial boilers

CO =(2.0+1.9)/2=2			4 Table 1.5-1, commercial boilers

PM10=(0.5+0.4)/2=0.45			4 Table 1.5-1, commercial boilers, filterable particulate

PM2.5=(0.5+0.4)/2=0.45		4 Table 1.5-1, commercial boilers, filterable particulate

SO2=(0.09S+0.10S)/2=0.23		4 Table 1.5-1, commercial boilers

4.  Weekly activity, WAF=6 days/week 5

5. Seasonal adjustment factor, SAF 2

Summer Season Adjustment Factor		0.60

Fall Season Adjustment Factor			0.83

Winter Season Adjustment Factor		1.52

Spring Season Adjustment Factor			1.04

6. Wt% sulfur content, S=2.456 gr/100cf  6

- Process:

The following equations are used to calculate the emissions without control for this source category.

Epa	= LPG*(EF/CF)*(CEMP/NJEMP)

Eps	= Epa*SAF/AADF

Epw	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

Epw	= (tons/day) for a typical winter day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1.  Employment data is representative of the commercial establishments surveyed by Department of Energy in the preparation of the State Energy Data Report. 7

2. Assumed 50/50 mixture of butane and propane in the development of the

emission factors

3. All commercial/institution combustion liquid petroleum gas burned in New Jersey is included as liquid petroleum gas  in the State Energy Data Report 2.

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:  None

- Double Counting:

Emissions for this source category have also been reported in the point source inventory.  The following methodology is used to adjust the area source emissions for this category to remove the emissions already accounted for in the point source inventory.

Epadj	= (LPG-PT)*(EF/CF)*(CEMP/NJEMP)

Where:

Epadj	= (tons/yr) for an annual emission of pollutant by county excluding double counting

PT	= Commercial/Institutional point sources' liquified petroleum gas use (103 gallons) 8

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

NOx		tons/yr		tons/day		tons/day

CO		tons/yr		tons/day		tons/day

SO2         	tons/yr   	tons/day		tons/day

PM10		tons/yr   	tons/day		tons/day

PM2.5		tons/yr   	tons/day		tons/day

- References:

1. Procedures for Emission Inventory Preparation Vol III:  Area Sources, September 1981, United States

Environmental Protection Agency, Office of  Air Quality Planning and Standards, Research Triangle Park, NC,

EPA-450/4-81-026

2. State Energy Data 2000 Consumption Tables (formerly the State Energy Data Report), United States Department of Energy, Energy Information Administration, Office of Energy Markets and End Use, Washington, DC

3.  Total 2002 employment by 6 digit NAICS code and by county, New Jersey  Department of Labor.

4.  Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

5. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction,  Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c

6.  Nonroad Source Inventory Development for Nonroad Engines presentation,

http://www.epa.gov/ttn/chief/eidocs/partisec4.pdf

7.  Air Pollution Control Regulations, N.J.S.A. 7:27-10.2, Sulphur Contents Standards, Office of Administrative Law, CN 301, Trenton, New Jersey

8. Memo to file concerning discussion on fuel use sale data with Department of Energy officials, June 1,1999

9. NJ Point Source Emission Inventory for 2002, New Jersey Department of Environmental Protection

Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Commercial/Institutional Kerosene Combustion

- SCC:  2103011000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC), oxides of nitrogen (NOx), carbon monoxide (CO), sulfur dioxide (SO2), particulate matter less than or equal to 10 microns (PM10), particulate matter less than or equal to 2.5 microns (PM2.5) and lead (Pb).  The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

Calculation Methodology:

The following calculation methodologies are recommended for this source category:1

1.  Surveying individual facilities

2.  Fuel consumption analysis

The fuel consumption analysis/methodology is selected because the input data elements are more readily available.

Emissions from the combustion of commercial kerosene are calculated using statewide amount of fuel burned, allocated to the county level by commercial employees.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  Total statewide commercial/institutional kerosene oil use, OIL2 (103 gallons)

2.  County and statewide employment totals for SIC's 0850, 415,417,422-23,43,458,47-8,494-96,50-97, not including 881, or NAICS 1153, 2213, 42, 44 - 5, 48 - 9, 51 - 6, 61 - 2, 71 - 2, 81, not including 8141, 92, CEMP, NJEMP 3

3.  Emission Factors, EF, (lbs/103 gallons burned)

Emission Factor				Reference

VOC = 		0.34			4 Table 1.3-3 commercial boilers distilate oil fired

NOx = 		20			4 Table 1.3-1 boilers<100 million btu/hr, distillate oil fired

CO = 		5			4 Table 1.3-1 boilers<100 million btu/hr, distillate oil fired

PM10 =	 (1.08+1.3)=2.38		4 Table 1.3-7 and 1.3-2 filterable plus condensible

PM2.5 = (0.83+1.3)=2.13 		4 Table 1.3-7 and 1.3-2 filterable plus condensible

SO2 = 142*(wt% sulfur, S)		4 Table 1.3-1 boilers<100 million btu/hr, distillate oil fired

Pb=   8.9 lb/1012 BTU *1.42 *108  BTU/103 gals=0.0012644 4

4.  Weekly activity, WAF=6 days/week 5

5. Seasonal adjustment factor, SAF 2

Summer Season Adjustment Factor		0.60

Fall Season Adjustment Factor			0.83

Winter Season Adjustment Factor		1.52

Spring Season Adjustment Factor			1.04

6.  Wt% sulfur content, S= 6	0.3:	Atlantic, Cape May, Cumberland, Ocean Counties,

Hunterdon, Sussex, and Warren Counties

0.2:	Burlington, Camden, Gloucester, Mercer, Salem, 						             Bergen,Essex,Hudson,Middlesex,Monmouth, 							  	Morris,Passaic, Somerset, and Union Counties

- Process:

The following equations are used to calculate the emissions without control  for this source category.

Epa	= OIL*(EF/CF)*(CEMP/NJEMP)

Eps	= Epa*SAF/AADF

Epw	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

Epw	= (tons/day) for a typical winter day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1.  Employment data is representative of the commercial establishments surveyed by Department of Energy in the preparation of the State Energy Data Report.7

2.  Assume emission factors for kerosene are equivalent to distillate oil  emission factors.

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:

New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 9, N.J.A.C. 7:27-9, Sulfur in Fuels, New Jersey State Department of Environmental Protection

- Double Counting:

Emissions for this source category have also been reported in the point source inventory.  The following methodology is used to adjust the area source emissions for this category to remove the emissions already accounted for in the point source inventory.

Epadj	= (OIL-PT)*(EF/CF)*(CEMP/NJEMP)

Where:

Epadj	= (tons/yr) for an annual emission of pollutant by county excluding double counting

PT	= Commercial/Institutional point sources' kerosene oil use 8 (103   gallons)

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

NOx		tons/yr		tons/day		tons/day

CO		tons/yr		tons/day		tons/day

SO2         	tons/yr   	tons/day		tons/day

PM10		tons/yr   	tons/day		tons/day

PM2.5		tons/yr   	tons/day		tons/day

- References:

1. Procedures for Emission Inventory Preparation Vol III:  Area Sources, September 1981, United States        Environmental Protection Agency, Office of  Air Quality Planning and Standards, Research Triangle Park, NC,EPA-450/4-81-026

2. State Energy Data 2000 Consumption Tables (formerly the State Energy Data Report), United States Department of Energy, Energy Information Administration, Office of Energy Markets and End Use, Washington, DC

3.  Total 2002 employment by 6 digit NAICS code and by county, New Jersey  Department of Labor.

4.  Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, , United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

5. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction,  Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c

6. New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 9, N.J.A.C. 7:27-9, Sulfur in Fuels, New Jersey State Department of Environmental Protection

7. Memo to file concerning discussion on fuel use sale data with Department of Energy officials, June 1,1999

8.  NJ Point Source Emission Inventory for 2002, New Jersey Department of Environmental Protection

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Residential Anthracite Coal Combustion

- SCC:  2104001000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC), oxides of nitrogen (NOx), carbon monoxide (CO), sulfur dioxide (SO2), particulate matter less than or equal to 10 microns (PM10), particulate matter less than or equal to 2.5 microns (PM2.5) and lead (Pb).  The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

Calculation Methodology:

The following calculation methodologies are recommended for this source category:1

1.  Surveying individual facilities

2.  Fuel consumption analysis

The fuel consumption analysis/methodology is selected because the input data elements are more readily available.

Emissions from the combustion of residential anthracite coal are calculated using statewide amount of coal burned, allocated to the county level by the number of housing units using coal for primary heat source.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  Total statewide residential anthracite coal use, COAL (tons)2

2.  County and state totals of housing units heated by anthracite coal burners, CHEAT, SHEAT3

3.  Emission Factors, EF (lbs/ton of coal burned)

Emission Factor							Reference

VOC = 		0.07						4 SCC 10300102 commercial

stoker-fired boiler

NOx = 		3.0						4 SCC 2104001000

CO = 		0.6						5 Table 1.2-2

PM10 =		4.75						5  Figure 1.2-1 total particulate

PM2.5 =	2.38 						5  Figure 1.2-1 total particulate

SO2 = 39*(wt% sulfur, S)*0.95(weight fraction of SO2 in SOx)	6  Table 1.2-1 and 1.1-3

Pb  = 		0.0089					 	5

4.  Weekly activity factor, WAF=7 days/week 6

5.  Seasonal adjustment factor, SAF 2

Summer Season Adjustment Factor		0.83

Fall Season Adjustment Factor			0.87

Winter Season Adjustment Factor		1.37

Spring Season Adjustment Factor		0.94

6.  Wt% sulfur, S = 0.8 7

- Process:

The following equations are used to calculate the emissions (without rule effectiveness) for this source category.

Epa	= COAL*(CHEAT/SHEAT)*(EF/CF)

Eps	= Epa*SAF/AADF

= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

Epw	= (tons/day) for a typical winter day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1. All residential anthracite coal burners are of the handfed design except for determination of sulfur dioxide emissions which represents space heaters for commercial/institutional operations.

2.  All residential coal consumption included in the State Energy Data Report is anthracite coal.

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:

New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 10, N.J.A.C. 7:27-10, Sulfur in Solid Fuels, New Jersey State Department of Environmental Protection

- Double Counting:

Emissions for this source category have not been reported in the point source inventory, therefore no adjustment of the area source emissions was required.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

NOx		tons/yr		tons/day		tons/day

CO		tons/yr		tons/day		tons/day

SO2         	tons/yr   	tons/day		tons/day

PM10		tons/yr   	tons/day		tons/day

PM2.5		tons/yr   	tons/day		tons/day

- References:

1. Procedures for Emission Inventory Preparation Vol III:  Area Sources, September 1981, United States        Environmental Protection Agency, Office of  Air Quality Planning and Standards, Research Triangle Park, NC,

EPA-450/4-81-026

2. State Energy Data 2000 Consumption Tables (formerly the State Energy Data Report), United States Department of Energy, Energy Information Administration, Office of Energy Markets and End Use, Washington, DC

3. Census of Housing General and Detailed Characteristics, United States Department of Commerce, Bureau of the Census, Washington, DC

4.  Factor Information Retrieval (FIRE) system, United States Environmental   Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC, 1995

5. Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

6. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction,  Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c

7. New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 10, N.J.A.C. 7:27-10, Sulfur in Solid Fuels, New Jersey State Department of Environmental Protection

Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Residential Distillate Oil Combustion

- SCC:  2104004000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC), oxides of nitrogen (NOx), carbon monoxide (CO), sulfur dioxide (SO2), particulate matter less than or equal to 10 microns (PM10), particulate matter less than or equal to 2.5 microns (PM2.5) and lead (Pb).  The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

Calculation Methodology:

The following calculation methodologies are recommended for this source category:1

1.  Surveying individual facilities

2.  Fuel consumption analysis

The fuel consumption analysis/methodology is selected because the input data elements are more readily available.

Emissions from the combustion of residential distillate oil are calculated using statewide amount of fuel burned, allocated to the county level by the number of housing units using distillate oil for primary heat source.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  Total statewide residential distillate oil use, OIL (103 gallons)2

2.  County and state totals of housing units heated by distillate oil  burners, CHEAT, SHEAT3

3.  Emission Factors, EF (lbs/103 gallons burned)

Emission Factor				Reference

VOC = 		0.71			4 Table 1.3-3 residential furnace, distillate oil fired

NOx = 		18			4 Table 1.3-1 residential furnace

CO = 		5			4 Table 1.3-1 residential furnace

PM10 = (0.22+1.3)=1.52		4 Table1.3-1, 2 and 7 res. furnace, filterable plus condensible

PM2.5 = (0.17+1.3)=1.47		4 Table1.3-1, 2 and 7 res. furnace, filterable plus condensible

SO2 = 142*(wt% sulfur, S)		4 Table 1.3-1 residential furnace, distillate oil fired

Pb=  (8.9 lb/1012 BTU) *(1.42*108  BTU/103 gals)=0.0012644

4.  Weekly activity, WAF=7 days/week5

5.  Seasonal adjustment factor, SAF 2

Summer Season Adjustment Factor		0.45

Fall Season Adjustment Factor			0.76

Winter Season Adjustment Factor		1.73

Spring Season Adjustment Factor			1.06

- 6.  Wt% sulfur content, S= 6	0.3:	Atlantic, Cape May, Cumberland, Ocean Counties,

Hunterdon, Sussex, and Warren Counties

- 0.2:	Burlington, Camden, Gloucester, Mercer, Salem,

Bergen, Essex, Hudson, Middlesex, Monmouth, Morris, 						   	Passaic, Somerset, and Union Counties

- Process:

The following equations are used to calculate the emissions without control for this source category.

Epa	= OIL*(CHEAT/SHEAT)(EF/CF)

Eps	= Epa*SAF/AADF

Epw	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

Epw	= (tons/day) for a typical winter day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1. All residential distillate oil  burned in New Jersey is included as residential distillate oil in the State Energy Data Report.

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:

New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 9, N.J.A.C. 7:27-9, Sulfur in Fuels, New Jersey State Department of Environmental Protection

- Double Counting:

Emissions for this source category have not been reported in the point source inventory, therefore no adjustment of the area source emissions was required.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

NOx		tons/yr		tons/day		tons/day

CO		tons/yr		tons/day		tons/day

SO2         	tons/yr   	tons/day		tons/day

PM10		tons/yr   	tons/day		tons/day

PM2.5		tons/yr   	tons/day		tons/day

- References:

1. Procedures for Emission Inventory Preparation Vol III:  Area Sources, September 1981, United States        Environmental Protection Agency, Office of  Air Quality Planning and Standards, Research Triangle Park, NC,

EPA-450/4-81-026

2. State Energy Data 2000 Consumption Tables (formerly the State Energy Data Report), United States Department of Energy, Energy Information Administration, Office of Energy Markets and End Use, Washington, DC

3. Census of Housing General and Detailed Characteristics, United States Department of Commerce, Bureau of the Census, Washington, DC

4.  Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, , United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

5. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction,  Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c

6. New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 9, N.J.A.C. 7:27-9, Sulfur in Fuels, New Jersey State Department of Environmental Protection

Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Residential Natural Gas Combustion

- SCC:  2104006000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC), oxides of nitrogen (NOx), carbon monoxide (CO), sulfur dioxide (SO2), particulate matter less than or equal to 10 microns (PM10), particulate matter less than or equal to 2.5 microns (PM2.5) and lead (Pb).  The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

Calculation Methodology:

The following calculation methodologies are recommended for this source category:1

1.  Surveying individual facilities

2.  Fuel consumption analysis

The fuel consumption analysis/methodology is selected because the input data elements are more readily available.

Emissions from the combustion of residential natural gas are calculated using statewide amount of fuel burned, allocated to the county level by the number of housing units using natural gas for primary heat source.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  Total residential natural gas use, GAS2 (106 cubic feet)

2.  County  and state totals of housing units heated by natural gas burners, CHEAT, SHEAT3

3.  Emission Factors, EF (lbs/106 cubic feet burned)

Emission Factor				Reference

VOC = 		5.5			4 Table 1.4-2

NOx = 		94			4 Table 1.4-1 residential furnaces

CO = 		40			4 Table 1.4-1 residential furnaces

PM10 =		7.6			4 Table 1.4-2

PM2.5 =	7.6			4 Table 1.4-2

SO2 = 		0.6			4 Table 1.4-2

Pb=0.0005

5.  Weekly activity, WAF=7 days/week5

6. Seasonal adjustment factor, SAF 2

Summer Season Adjustment Factor		0.3

Fall Season Adjustment Factor			0.71

Winter Season Adjustment Factor		1.94

Spring Season Adjustment Factor			1.05

- Process:

The following equations are used to calculate the emissions without control for this source category.

Epa	= GAS*(CHEAT/SHEAT)*(EF/CF)

Eps	= Epa*SAF/AADF

Epw	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

Epw	= (tons/day) for a typical winter day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1. All emission rates based on residential operations except for lead emissions which is based on Commercial/Institutional operations of <10MMBTU/hr.

2. All residential distillate oil  burned in New Jersey is included as residential distillate oil in the State Energy Data Report1.

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:  None

- Double Counting:

Emissions for this source category have not been reported in the point source inventory, therefore no adjustment of the area source emissions was required.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

NOx		tons/yr		tons/day		tons/day

CO		tons/yr		tons/day		tons/day

SO2         	tons/yr   	tons/day		tons/day

PM10		tons/yr   	tons/day		tons/day

PM2.5		tons/yr   	tons/day		tons/day

- References:

1.  Procedures for Emission Inventory Preparation Vol III:  Area Sources, September 1981, United States Environmental Protection Agency, Office of  Air Quality Planning and Standards, Research Triangle Park, NC, EPA-450/4-81-026

2. State Energy Data 2000 Consumption Tables (formerly the State Energy Data Report), United States Department of Energy, Energy Information Administration, Office of Energy Markets and End Use, Washington, DC

3.Census of Housing General and Detailed Characteristics, United States Department of Commerce, Bureau of the Census, Washington, DC

4.  Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, , United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

5. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction,  Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Residential Liquified Petroleum Gas Combustion

- SCC:  2104007000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC), oxides of nitrogen (NOx), carbon monoxide (CO), sulfur dioxide (SO2), particulate matter less than or equal to 10 microns (PM10), particulate matter less than or equal to 2.5 microns (PM2.5) and lead (Pb).  The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

Calculation Methodology:

The following calculation methodologies are recommended for this source category:1

1.  Surveying individual facilities

2.  Fuel consumption analysis

The fuel consumption analysis/methodology is selected because the input data elements are more readily available.

Emissions from the combustion of residential liquid petroleum gas are calculated using statewide amount of fuel burned, allocated to the county level by the number of housing units using liquid petroleum gas for primary heat source.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  Total statewide residential liquified petroleum gas use, LPG (103 gallons)2

2.  County and state totals of housing units heated by liquified petroleum gas  burners, CHEAT, SHEAT3

3.  Emission Factors, EF, (lbs/103 gallon burned)

Emission Factor				Reference

VOC=(0.6-0.2)+(0.5-0.2)/2=0.35		4 Table 1.5-1, commercial boilers

NOx=(15+14)/2=14.5			4 Table 1.5-1, commercial boilers

CO =(2.0+1.9)/2=2			4 Table 1.5-1, commercial boilers

PM10=(0.5+0.4)/2=0.45			4 Table 1.5-1, commercial boilers, filterable particulate

PM2.5=(0.5+0.4)/2=0.45		4 Table 1.5-1, commercial boilers, filterable particulate

SO2=(0.09S+0.10S)/2=0.23		4 Table 1.5-1, commercial boilers

4.  Weekly activity, WAF=7 days/week 5

5. Seasonal adjustment factor, SAF 2

Summer Season Adjustment Factor		0.45

Fall Season Adjustment Factor			0.76

Winter Season Adjustment Factor		1.73

Spring Season Adjustment Factor			1.06

6. Wt% sulfur content, S=2.456 gr/100cf  6

- Process:

The following equations are used to calculate the emissions without control for this source category.

Epa	= LPG*(CHEAT/SHEAT)*(EF/CF)

Eps	= Epa*SAF/AADF

Epw	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

Epw	= (tons/day) for a typical winter day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1.Assume residential liquid petroleum gas combustion has same emission factors as commercial/institution liquid petroleum gas combustion.

2. Assumed 50/50 mixture of butane and propane in the development of the emission factors.

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:  None

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

NOx		tons/yr		tons/day		tons/day

CO		tons/yr		tons/day		tons/day

SO2         	tons/yr   	tons/day		tons/day

PM10		tons/yr   	tons/day		tons/day

PM2.5		tons/yr   	tons/day		tons/day

- Double Counting:

Emissions for this source category have not been reported in the point source inventory, therefore no adjustment of the area source emissions was required.

- References:

1. Procedures for Emission Inventory Preparation Vol III:  Area Sources, September 1981, United States Environmental Protection Agency, Office of  Air Quality Planning and Standards, Research Triangle Park, NC,EPA-450/4-81-026

2. State Energy Data 2000 Consumption Tables (formerly the State Energy Data Report), United States Department of Energy, Energy Information Administration, Office of Energy Markets and End Use, Washington, DC

3. Census of Housing General and Detailed Characteristics, United States Department of Commerce, Bureau of the Census, Washington, DC

4.  Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

5. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction,  Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c

6.  Nonroad Source Inventory Development for Nonroad Engines presentation,

http://www.epa.gov/ttn/chief/eidocs/partisec4.pdf

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Residential Wood Combustion

- SCC: 2104008000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC), oxides of nitrogen (NOx), carbon monoxide (CO), sulfur dioxide (SO2), particulate matter less than or equal to 10 microns (PM10) and particulate matter less than or equal to 2.5 microns (PM2.5).  The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

Calculation Methodology:

The following calculation methodologies are recommended for this source category:1

1.  Surveying individual facilities

2.  Fuel consumption analysis

The fuel consumption analysis/methodology is selected because the input data elements are more readily available.

Emissions from the combustion of residential wood are calculated using statewide amount of wood burned, allocated to the county level by the number of housing units using wood for primary heat source.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  Residential wood use, statewide, WOOD (cords) 2

2.  County and state totals of housing units heated by wood, CHEAT, SHEAT 3

3.  Heating degree days, by season, HDD 4

4.  Emission Factors, EF (lbs/ton burned) 5

VOC =  	53	SOx = 	0.4

NOx =    	2.8  	PM10 =	30.6

CO =  	230.8	PM2.5=	30.6

5.   Seasonal adjustment factor, SAF 4

- SAF=(Seasonal HDD/Total annual HDD)*4

- Summer Season Adjustment Factor		0.02

- Fall Season Adjustment Factor			0.82

- Winter Season Adjustment Factor		2.14

- Spring Season Adjustment Factor			1.02

- Process:

The following equations are used to calculate the emissions without control for this source category.

Epa	= WOOD*CF2*CF3*(CHEAT/SHEAT)*(EF/CF)

Eps	= Epa*SAF/AADF

Epw	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

Epw	= (tons/day) for a typical winter day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

CF2	= Conversion factor =79 cf of wood/ a cord of wood 5

CF3	= Conversion factor = 35.78 lb of wood/cf of wood 5

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1. Assume all residential wood combustion represents noncatalytic conventional wood stove, which represents an average value of emissions factors for all controlled and uncontrolled wood stove and fireplaces use.

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:    None

- Double Counting:

Emissions for this source category have not been reported in the point source inventory, therefore no adjustment of the area source emissions was required.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

NOx		tons/yr		tons/day		tons/day

CO		tons/yr		tons/day		tons/day

SO2         	tons/yr   	tons/day		tons/day

PM10		tons/yr   	tons/day		tons/day

PM2.5		tons/yr   	tons/day		tons/day

- References:

1. Procedures for Emission Inventory Preparation Vol III:  Area Sources,  September 1981, United States Environmental Protection Agency, Office of  Air Quality Planning and Standards, Research Triangle Park, NC, EPA-450/4-81-026

2. State Energy Data 2000 Consumption Tables (formerly the State Energy Data Report), United States Department of Energy, Energy Information Administration, Office of Energy Markets and End Use, Washington, DC

3. Census of Housing General and Detailed Characteristics, United States  Department of Commerce, Bureau of the Census, Washington, DC

4. Local Climatological Data: 2002 Annual Summary with Comparative Data, United   States Department of Commerce, Washington, DC

5. EIIP Volume III, Area Sources Preferred and Alternative Methods, Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Residential Kerosene Oil Combustion

- SCC:  2104011000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC), oxides of nitrogen (NOx), carbon monoxide (CO), sulfur dioxide (SO2), particulate matter less than or equal to 10 microns (PM10), particulate matter less than or equal to 2.5 microns (PM2.5) and lead (Pb).  The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

Calculation Methodology:

The following calculation methodologies are recommended for this source category:1

1.  Surveying individual facilities

2.  Fuel consumption analysis

The fuel consumption analysis/methodology is selected because the input data elements are more readily available.

Emissions from the combustion of residential kerosene are calculated using statewide amount of fuel burned, allocated to the county level by the number of housing units using kerosend for primary heat source.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  Total statewide residential kerosene oil use, OIL (103 gallons)2

2.  County and state totals of housing units heated by kerosene oil  burners, CHEAT, SHEAT3

3.  Emission Factors, EF, (lbs/103 gallons burned)

Emission Factor				Reference

VOC = 		0.71			4 Table 1.3-3 residential furnace, distillate oil fired

NOx = 		18			4 Table 1.3-1 residential furnace

CO = 		5			4 Table 1.3-1 residential furnace

PM10 = (0.22+1.3)=1.52		4 Table1.3-1, 2 and 7 res. furnace, filterable plus condensible

PM2.5 = (0.17+1.3)=1.47		4 Table1.3-1, 2 and 7 res. furnace, filterable plus condensible

SO2 = 142*(wt% sulfur, S)		4 Table 1.3-1 residential furnace, distillate oil fired

Pb=  (8.9 lb/1012 BTU) *(1.42*108  BTU/103 gals)=0.0012644

4.  Weekly activity, WAF=7 days/week5

5. Seasonal adjustment factor, SAF 2

Summer Season Adjustment Factor		0.45

Fall Season Adjustment Factor			0.76

Winter Season Adjustment Factor		1.73

Spring Season Adjustment Factor			1.06

- 6.  Wt% sulfur content, S= 6	0.3:	Atlantic, Cape May, Cumberland, Ocean Counties,

Hunterdon, Sussex, and Warren Counties

- 0.2: 	Burlington, Camden, Gloucester, Mercer, Salem,

Bergen, Essex, Hudson, Middlesex, Monmouth, Morris, 					                 	Passaic, Somerset, and Union Counties

- Process:

The following equations are used to calculate the emissions without control for this source category.

Epa	= OIL*(CHEAT/SHEAT)*(EF/CF)

Eps	= Epa*SAF/AADF

Epw	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

Epw	= (tons/day) for a typical winter day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1. All residential kerosene oil  burned in New Jersey is included as residential kerosene oil in the State Energy Data Report.

2.  Assume emission factors for kerosene are equivalent to distillate oil  emission factors.

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:

New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 9, N.J.A.C. 7:27-9, Sulfur in Fuels, New Jersey State Department of Environmental Protection

- Double Counting:

Emissions for this source category have not been reported in the point source inventory, therefore no adjustment of the area source emissions was required.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

NOx		tons/yr		tons/day		tons/day

CO		tons/yr		tons/day		tons/day

SO2         	tons/yr   	tons/day		tons/day

PM10		tons/yr   	tons/day		tons/day

PM2.5		tons/yr   	tons/day		tons/day

- References:

1. Procedures for Emission Inventory Preparation Vol III:  Area Sources, September 1981, United States        Environmental Protection Agency, Office of  Air Quality Planning and Standards, Research Triangle Park, NC,

EPA-450/4-81-026

2. State Energy Data 2000 Consumption Tables (formerly the State Energy Data Report), United States Department of Energy, Energy Information Administration, Office of Energy Markets and End Use, Washington, DC

3. Census of Housing General and Detailed Characteristics, United States Department of Commerce, Bureau of the Census, Washington, DC

4.  Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

5. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction, Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c

6. New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 9, N.J.A.C. 7:27-9, Sulfur in Fuels, New Jersey State Department of Environmental Protection

Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Aircraft Refueling Aviation Gasoline

- SCC:  2275900101

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category: 1

1.  Surveying individual airports

2.  Aviation fuel sales analysis

The aviation fuel sales analysis is selected because the input data elements are readily available.

Emissions from the refueling of aircraft using aviation gasoline are calculated using statewide amount of fuel burned, allocated to the county level by the number of landing and takeoff cycles at each airport.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  Total statewide Aviation Gasoline use, SAG (103 gallons) 2

2.  The number of landing and takeoff cycles at each airport and statewide, ALTO, SLTO 3, 4, 5

3.  Emission factor, EF (lbs/103 gallons of jet kerosene fuel) 6

- VOC = (12.46*S*P*M)/T

VOC = (12.46*1.45*3.496*72.25)/519.67

- VOC = 8.78

Where:

S = saturation factor for splash loading with dedicated normal service = 1.45

P = fuel true vapor pressure in psia at 60 degrees F. = 3.496 7

M = vapor molecular weight, lb/lb-mole  = 72.25 7

T = bulk liquid temperature at 60 degrees = 519.67 Rankin

4.  Weekly activity, WAF=7 days/week

5.  Seasonal adjustment factor, SAF=1 8

- Process:

The following equations are used to calculate the emissions without control  for this source category:

AAG = SAG*(ALTO/SLTO)

CAG = AAG summed by county

Epa = CAG*EF/CF

Eps = Epa*SAF/AADF

Where:

AAG	= Aviation Gasoline use at each airport (103 gallons)

CAG	= Aviation Gasoline use in  each county (103 gallons)

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1.  Assume average fuel temperature of 60 oF

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:    None

- Double Counting:

Emissions for this source category have not been reported in the point source inventory, therefore no adjustment of the area source emissions was required.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

- References:

1. Procedures for the Preparation of Emission Inventories for Carbon Monoxideand Precursors of Ozone, Vol. I:  General Guidance for Stationary Sources, May 1991, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC, EPA-450/4-91-016

2. State Energy Data 2000 Consumption Tables (formerly the State Energy Data Report), United States Department of Energy, Energy Information Administration, Office of Energy Markets and End Use, Washington, DC

3. APO TERMINAL AREA FORECAST DETAIL REPORT FOR FISCAL YEAR 2002, Federal Aviation

Administration, Office of Aviation Policy, Plans and Management Analysis (www.apo.data.faa.gov).

4.  June 5, 2003 email submittal from Kimberlee McDonald of McQuire AFB to NJDEP

5.  May 5, 2003 email submittal from Mike Figura of Naval Air Engineering Station in Lakehurst

6.  Compilation of Air Pollutant Emission Factors, AP-42, Fifth Edition, United States Environmental Protection Agency, Office of  Air Quality Planning and Standards, Research Triangle Park, NC, Section 5.2 and Table 5.2-1 and Table 7.1-2.

7.  Radian Corporation report prepared for J.F. Durham, USEPA, AUgust 10, 1993

8.  Seasonal and Monthly Activity Allocation Fractions for Nonroad Engine Emissions Modeling, EPA 420-P-02-010, Table 4, Mid-Atlantic Region.

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Aircraft Refueling Jet Kerosene

- SCC:  2275900101

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category: 1

1.  Surveying individual airports

2.  Jet fuel sales analysis

The jet fuel sales analysis is selected because the input data elements are readily available.

Emissions from the refueling of aircraft using jet kerosene are calculated using statewide amount of fuel burned, allocated to the county level by the number of landing and takeoff cycles at each airport.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  Total statewide Jet Kerosene use, SJET (103 gallons) 2

2.  The number of landing and takeoff cycles at each airport and statewide, ALTO, SLTO 3, 4, 5

3.  Emission factor, EF (lbs/103 gallons of jet kerosene fuel) 6

- VOC = (12.46*S*P*M)/T

VOC = (12.46*1.45*0.0085*130)/519.67

- VOC = 0.038

Where:

S = saturation factor for splash loading with dedicated normal service = 1.45

P = fuel true vapor pressure in psia at 60 degrees F. = .0085

M = vapor molecular weight, lb/lb-mole  = 130

T = bulk liquid temperature at 60 degrees = 519.67 Rankin

4.  Weekly activity, WAF=7 days/week

5.  Seasonal adjustment factor, SAF=1 7

- Process:

The following equations are used to calculate the emissions without control  for this source category:

AJET = SJET*(ALTO/SLTO)

CJET = AJET summed by county

Epa = CJET*EF/CF

Eps = Epa*SAF/AADF

Where:

AJET	= Jet Kerosene use at each airport (103 gallons)

CJET	= Jet Kerosene use in  each county (103 gallons)

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1.  Assume average fuel temperature of 60 oF

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:    None

- Double Counting:

Emissions for this source category have not been reported in the point source inventory, therefore no adjustment of the area source emissions was required.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

- References:

1. Procedures for the Preparation of Emission Inventories for Carbon Monoxideand Precursors of Ozone, Vol. I:  General Guidance for Stationary Sources, May 1991, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC, EPA-450/4-91-016

2. State Energy Data 2000 Consumption Tables (formerly the State Energy Data Report), United States Department of Energy, Energy Information Administration, Office of Energy Markets and End Use, Washington, DC

3. APO TERMINAL AREA FORECAST DETAIL REPORT FOR FISCAL YEAR 2002, Federal Aviation

Administration, Office of Aviation Policy, Plans and Management Analysis (www.apo.data.faa.gov).

4.  June 5, 2003 email submittal from Kimberlee McDonald of McQuire AFB to NJDEP

5.  May 5, 2003 email submittal from Mike Figura of Naval Air Engineering Station in Lakehurst

6.  Compilation of Air Pollutant Emission Factors, AP-42, Fifth Edition, United States Environmental Protection Agency, Office of  Air Quality Planning and Standards, Research Triangle Park, NC, Section 5.2 and Table 5.2-1 and Table 7.1-2.

7.  Seasonal and Monthly Activity Allocation Fractions for Nonroad Engine Emissions Modeling, EPA 420-P-02-010, Table 4, Mid-Atlantic Region.

Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Bakeries

- SCC:  23020500

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category:1

1.  Surveying individual bakeries

2.  Employment based analysis

3.  Population based analysis

The employment based analysis/methodology is selected because (1) the input data elements are more readily available and (2) it is more accurate than population.

Emissions from bakeries are calculated using county level employment.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  County employment for SIC 2051 and 546 or NAICS 31181-3, 445291, 722213, CEMP 2

2.  Emission Factors, EF (tons/employee/year) 1    VOC=220.0

3.  Weekly activity, WAF=7 days/week

4.  Seasonal adjustment factors, SAF = 1

- Process:

The following equations are used to calculate the emissions without control for this source category.

Epa	= CEMP*EF

Eps	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1.  Assume a weekly activity factor of 7 days per week

2.  Assume a seasonal adjustment factor of 1

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:    None

- Double Counting:

Emissions for this source category have also been reported in the point source inventory.  The following methodology is used to adjust the area source emissions for this category to remove the emissions already accounted for in the point source inventory.

Epadj	=  EF* (CEMP - PT)

Where:

Epadj	= (tons/yr) for an annual emission of pollutant by county excluding double counting

PT	= Point source employment for SIC  2051 and 546 3

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

- References:

1. Memorandum from RADIAN Corp to SIP Inventory Preparers and EPA regions concerning VOC Emissions from Bakeries, April 24, 1992

2. Total 2002 employment by 6 digit NAICS code and by county, New Jersey  Department of Labor.

3. NJ Point Source Emission Inventory for 2002, New Jersey Department of Environmental Protection

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Breweries

- SCC:  2302070001

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category: 1

1.  Surveying individual breweries

2.  Beer production analysis

The beer production analysis/methodology is selected because the input data elements are more readily available.

Emissions from breweries are calculated using statewide beer production allocated to the county level using employment.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  Statewide beer production, BEER (103 barrels of 42 gallons)

=(1995 production of malt beverages in barrels of 31 gallons) 2 * (0.74 barrels of 42 gallons/barrel of 31 gallons)*(1.0251, growth factor 1996-1999) 3 * (1.0377, growth factor 1999-2002) 3

2.  County and state employment for SIC 2082 and 5181 or NAICS 312120, CEMP, NJEMP 4

3.  Emission Factors, EF, (lbs/103 barrels) 5    VOC=56.80

4.  Weekly activity, WAF=5 days/week

5.  Seasonal adjustment factor, SAF=1

- Process:

The following equations are used to calculate the emissions without control  for this source category.

Epa	= BEER*(CEMP/NJEMP)*EF/CF

Eps	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1.  The emission factor for California's breweries are appropriate for NJ breweries

2.  Assume a weekly activity factor of 5 days per week

3.  Assume a seasonal adjustment factor of 1

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:   None

- Double Counting:

Emissions for this source category have also been reported in the point source inventory.   The Anheuser-Busch facility in Newark is the only facility which has reported emission in the point source inventory. 6   The number of employees that were reported for this facility were subtracted from the Essex county employment total.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

- References:

1. Procedures for the Preparation of Emission Inventories for Carbon Monoxide and Precursors of Ozone, Vol. I:  General Guidance for Stationary Sources, May 1991, United States Environmental Protection Agency, Office of Air  Quality Planning and Standards, Research Triangle Park, NC,   EPA-450/4-91-016

2. Beer Insitutute Brewer's Almanac 1998

3.  The State of New Jersey Department of Environmental Protection, State Implementation Plan (SIP) Revision for the Attainment and Maintenance of the Ozone National Ambient Air Quality Standard (NAAQS) New Jersey 1996 Actual Emission Inventory and Rate of Progress (ROP) Plans for 2002, 2005 and 2007, Appendix II, March 31, 2001

4. Total 2002 employment by 6 digit NAICS code and by county, New Jersey  Department of Labor.

5. Memorandum from RADIAN Corp to SIP Inventory Preparers and EPA regions concerning VOC Emissions from Breweries, February 5, 1992

6. NJ Point Source Emission Inventory for 2002, New Jersey Department of Environmental Protection

Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Wineries

- SCC:  2302070005

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category:1

1.  Surveying individual wineries

2.  Wine production analysis

Survey individual wineries is selected because the input data elements are more readily available.

Emissions from wineries are calculated using facility specific estimated wine production production.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  Facility specific wine production, WINE (103 gallons) 2,3,4

= (1993 wine production from 14 wineries) * (growth factors 1996-1999) 5 * (growth factors 1999-2002) 6 + (estimated wine production from an additional 7 wineries in 2002 based on an average production of the 14 wineries in 1993)

1996-1999	1999-2002

Growth factors, Atlantic City Air Quality Control Region		1.019		1.0357

Growth factors, Northern New Jersey Air Quality Control Region	1.0307		1.0412

Growth factors, Southern New Jersey Air Quality Control Region	1.0255		1.0362

2.  Emission Factors, EF, (lbs/103 gallons)6

- VOC=(1.76+5.52)/2= 3.64

3.  Weekly activity, WAF=5 days/week

4.  Seasonal adjustment factors, SAF=1

- Process:

The following equations are used to calculate the emissions without control  for this source category.

Epa          = WINE*EF/CF

Eps      = Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1.  Assume 50/50 mix of red and white wines

2.  Assume all white wine fermented at 52oF

3.  Assume all red wine fermented at 80oF

4.  Assume a weekly activity factor of 5 days per week

5.  Assume a seasonal adjustment factor of 1

6.  Assume that the survey conducted for 1993 approximates 1996 acitivity.

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:  None

- Double Counting:

Emissions for this source category have not been reported in the point source inventory, therefore no adjustment of the area source emissions was required.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

- References:

1. Procedures for the Preparation of Emission Inventories for Carbon Monoxideand Precursors of Ozone, Vol. I:  General Guidance for Stationary Sources, May 1991, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,   EPA-450/4-91-016

2. List of Active and Plenary Winery Licenses, Division of Alcoholic Beverage Control,  Department of Law and Public Safety, Division of Alcoholic Beverage Control, 1993

3. Winery Survey, Division of Air Quality Management, Bureau of Air Quality Planning, April 1995

4. Garden State Wine Growers Association-Wineries, www.newjerseywines.com/wineries.html, 3/15/04

5. The State of New Jersey Department of Environmental Protection, State Implementation Plan (SIP) Revision for the Attainment and Maintenance of the Ozone National Ambient Air Quality Standard (NAAQS) New Jersey 1996 Actual Emission Inventory and Rate of Progress (ROP) Plans for 2002, 2005 and 2007, Appendix II, March 31, 2001

6. Memorandum from RADIAN Corp to SIP Inventory Preparers and EPA regions concerning VOC Emissions from Wineries, March 10, 1992

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Distilleries

- SCC:  2302070010

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category:1

1.  Surveying individual distilleries

2.  Distillery production analysis

Survey individual distilleries is selected because the input data elements are more readily available.

Emissions from distilleries are calculated using facility specific estimated emissions.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1. Facility calculated annual VOC emissions (lb/year)

Black Prince Distillery 1995 Emissions (BPEM):	7,9202

Laird Distillery  1996 Emissions (LDEM):            	9,3203

William Grant & Sons 1995 Emissions (WGEM):	15,0404

2. Weekly activity, WAF=5 days/week

3. Seasonal adjustment factors, SAF=1

- Process:

The following equations are used to calculate the emissions without control for this source category.

Epa = (BPEM + LDEM + WGEM)*( 1.0255, growth factor 1996-1999) 5 * (1.0362, growth factor 1999-2002) 5/CF

Eps	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1. Assume that Black Prince, Laird and William Grant distilleries represent the only distillery operations in New Jersey.

2.  Assume that the 1996 emissions for  Black Prince blending operations approximates those emissions provided by  this facility in its 1995 emission statement.

3.  Assume that 1996 emissions for Laird & Company approximates the 1995 emissions provided by this facility. 4.  Assume a weekly activity factor of 5 days per week.

5.  Assume a seasonal adjustment factor of 1.

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:  None

- Double Counting:

Emissions for this source category have not been reported in the point source inventory, therefore no adjustment of the area source emissions was required.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

- References:

1. Procedures for the Preparation of Emission Inventories for Carbon Monoxide and Precursors of Ozone, Vol. I:  General Guidance for Stationary Sources,      May 1991, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,   EPA-450/4-91-016

2. 1995 Emission Statement for Black Prince Distillery, Inc., New Jersey Department of Environmental Program, Bureau of Air Quality Planning

3. January 10, 1997 Cover Letter with enclosed VOC Emission Calculations for 1995 Distillery Operations from William Grant & Sons, Inc.

4. February 21, 1997 Cover Letter with enclosed VOC Emission Calculations for 1996 Distillery Operations from Laird & Company

5. The State of New Jersey Department of Environmental Protection, State Implementation Plan (SIP) Revision for the Attainment and Maintenance of the Ozone National Ambient Air Quality Standard (NAAQS) New Jersey 1996 Actual Emission Inventory and Rate of Progress (ROP) Plans for 2002, 2005 and 2007, Appendix II, March 31, 2001

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Architectural Surface Coating

- SCC:  2401001000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category:1

1.  Surveying wholesale & retail suppliers of architectural coatings

2.  Population based analysis

The population based analysis/methodology is selected because the input data elements are more readily available.

Emissions from architectural surface coatings application are calculated using county level population.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  County population, CPOP 2

2.  Emission Factors, EF, (lbs/year/capita) 3

- VOC= 3.63

3. Control Efficiency, CE = 12.57% 4

4. Rule Effectiveness, RE = 80 % 1

5. Rule Penetration, RP = 100 %

6. Weekly activity, WAF= 7 days/week

7. Seasonal adjustment factors, SAF 5

Summer Season Adjustment Factor		1.32

Fall Season Adjustment Factor			0.89

Winter Season Adjustment Factor		0.89

Spring Season Adjustment Factor			0.89

- Process:

The following equations are used to calculate the emissions with control efficiency for this source category.

Epa	= CPOP*EF*( 1- (CE*RP*RE) )/CF

Eps	= Epa*SAF//ADDF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

- Rule Effectiveness:

The emissions from this source category are regulated by the following rule:

1.  New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 23, Architectural Coatings, February 21, 1989

2. National Volatile Organic Compound Emission Standards for Consumer and Commercial Products, CFR 40, Chapter 1, Subchapter C, Part 59, Subpart D National Volatile Organic Compound Standards for Architectural Coatings, operative September 13, 1999.

- Double Counting:

Emissions for this source category have not been reported in the point source inventory, therefore no adjustment of the area source emissions was required.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

- References:

1. Procedures for the Preparation of Emission Inventories for Carbon Monoxide and Precursors of Ozone Vol I:  General Guidance for Stationary Sources,  United States Environmental Protection Agency, Office of Air   Quality Planning and Standards, Research Triangle Park, NC,    EPA-450/4-91-016, May 1991

2. Estimate of  2002 Total Resident Population by County, NJDOT, July 17, 2003.

3. Emission Inventory Improvement Program, Volume III, Area Sources Preferred  and Alternative Methods, Chapter 3, Architectural Surface Coating, Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC November 8, 1995

4.  The State of New Jersey Department of Environmental Protection, State Implementation Plan (SIP) Revision for the Attainment and Maintenance of the Ozone National Ambient Air Quality Standard (NAAQS) New Jersey 1996 Actual Emission Inventory and Rate of Progress (ROP) Plans for 2002, 2005 and 2007, Appendix I, Attachment I-G, March 31, 2001.

5. Emission Inventory Improvement Program Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction, Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Auto Refinishing Surface Coating

- SCC:  2401005000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category:1

1.  Surveying automotive refinishing shops

2.  Employment based analysis

3.  Population based analysis

The employment based analysis/methodology is selected because the (1) input data elements are more readily available and (2) it is more accurate than population.

Emissions from auto refinishing operations are calculated using county level employment.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  County employment for SIC code 7532 or NAICS 811121, CEMP2

2.  Emission Factors, EF, (lbs/employee-year)

- Coating VOC=  653.99  see calculations below

- Cleaning VOC = 411.72 see calculations below

3.  Control Efficiency, CE = 37% 3,4

4.  Rule Effectiveness, RE = 100 % 5

5.  Rule Penetration, RP = 100 %

6.  Weekly activity, WAF= 5 days/week 6

7.  Seasonal adjustment factors, SAF=1 6

Emission Factor Calculations:

1997 National  employment for SIC code 7532, Automotive paint or body repair shops,  NEMP = 192,8537

1997-98 National annual VOC Emissions from coating applications  after implementation of  USEPA VOC content       rule, NATCOAT   = 39,728.94 tpy4

1997-98 National annual VOC Emissions from equipment cleaning with no rule, NATCLEAN = 39,700.65 tpy8

Conversion factor, CE = 2000 lb/ton

National control efficiency applied after 1996 to develop the national auto refinishing coating  inventory, CE = 37 %.5,6

National Rule Effectiveness (RE) factor applied to National CE, 100 %5

National Rule Penetration (RP) factor applied to National CE, 100 %

- Coating Application EF

- Calculate EF for coating application prior to EPA coating rule:

VOCCOATw/o epa rule  	=  (CF*(NATCOAT/NEMP))/(1-CE*RE*RP)

=  (2000 lb/ton *(39,728.94 ton/yr/192,853 employees))/(1-.37*1.00*1.00)

=   653.99 lb/employees

Cleaning Application EF

Calculate EF for cleaning application with no EPA rule (the EPA voc content rule did not include any equipment cleaning requirements):

- VOCCLEAN  		=  CF*(NATCLEAN/NEMP)

=  2000 lb/ton*(39,700.65 ton/yr/192,853)

=  411.72 lb/employees

- Process:

The following equations are used to calculate the emissions with control for this source category.

Epcoata     = (EFCOATw/o epa rule ) * (CEMP/NJEMP)( 1-CE*RE*RP)/CF

Epcoats     = Epcoata*SAF/AADF

Epcleana     = (EFCLEAN)*(CEMP/NJEMP)* (1-CE*RE*RP)/CF

Epcleans   =  Epcleana*SAF*/AADF

Where:

Epcoata, Epcleana  = (tons/yr) for an annual emission of pollutant by county

Epcoats,  Epcleans  = (tons/day) for a typical summer day emission of pollutant

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:

- National Volatile Organic Compound Emission Standards for Consumer and Commercial Products, CFR 40, Chapter 1, Subchapter C, Part 59, Subpart B-National Volatile Organic Compound Emission Standards for Automobile Refinish Coatings, operative January 11, 1999.

- Double Counting:

Emissions for this source category have not been reported in the point source inventory, therefore no adjustment of the area source emissions was required.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

- References:

1. Compilation of Air Pollutant Emission Factors Volume I:Stationary Point and  Area Sources, Fourth Edition,  United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC 27711, September 1991

2. Total 2002 employment by 6 digit NAICS code and by county, New Jersey  Department of Labor.

3. EIIP Volume III, Chapter 13, Auto Body Refinishing, External Review Draft, Table 13.2-1.Estimated VOC Emission Reductions for Automotive Refinishing Coatings, January 2000, Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  January 2000

4. April 27, 2000 E-mail from Lucy Adams on Auto Body Refinishing

5. Revision to the State Implementation Plan (SIP) for the Attainment and Maintenance of the Ozone National Ambient Air Quality Standards, Revision to the New Jersey 15 Percent Rate of Progress Plan, Appendix III:Emission Benefit Calculations, Section 4: Autobody Refinishing Standards, 1999 Projection Year Ozone Season Emission Inventory with Controls Calculations, the State of New Jersey, Department of Environmental Protection, February 8, 1999

6. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction,  Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c

7. U.S. Census Bureau, 1997 Economic Census,  Other Services (Public Administration)-Geog. Area Series, United States, Summary Statistics for Firms Subject to Federal  Income tax for the United States: 1997, November 30, 1999

8. EIIP Volume III, Chapter 13, Auto Body Refinishing, External Review Draft, Table 13.4-1.National VOC Emissions by Body Shop Size, Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  January 2000

Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Architectural Surface Coating, Traffic Paints

- SCC:  2401008000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category:1

1.  Surveying highway maintenance departments & contractors engaged in new road construction.

2.  Lane-mile painted analysis

3.  Population based analysis

The lane-mile painted analysis/methodology is selected because (1) the input data elements are more readily available and (2) it is more accurate than population.

Emissions from	traffic paints surface coating applications are calculated using county specific lane mileage.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  Total lane mileage within county, TOTLANE 2

= (1996 estimated lane miles) * (county growth factors 1996-2002) 2a

2.  Total lane miles maintained by State of New Jersey, NJDOT 2a

3.  Emission Factors, EF, (lbs/lane-mile/year) 1

- VOC = 69

4. Control Efficiency, CE

The following process is used to calculate CE for this source category:

- STEP ONE: Establish voc content (lbs/gal) prior to rule implementation

voc contentno rule	= (EF lbs/year)/(coating usage factor) (gal/year)

voc contentno rule	= (69 lbs/lane-mile1)/(22 gal/year1)

voc contentno rule	=  3.14 lbs/gal*(453.593 grams/lb/3.78544 liter/gallon)

voc contentno rule      =  375.82 grams/liter

- STEP TWO: Establish rule voc content

voc contentrule   =   150 grams/liter 5

- STEP THREE: Calculate CE

CE =  (voc contentno rule -  voc contentrule )/(voc contentno rule)

CE =  (375.82 grams/liter   -    150 grams/liter)/(375.82 grams/liter) * 100 %

CE =   60  %

5. Rule Effectiveness, RE = 80 % 1

6. Rule Penetration, RP = 100 %

7. Weekly activity, WAF=5 days/week 6

8. Seasonal adjustment factors, SAF  6

Summer Season Adjustment Factor		1.32

Fall Season Adjustment Factor			0.89

Winter Season Adjustment Factor		0.89

Spring Season Adjustment Factor			0.89

- Process:

The following equations are used to calculate the emissions with control for this source category.

Epa	= (TOTLANE - NJDOT)*EF*( 1 -  (CE*RP*RE) )/CF

Eps	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1.  Assume that a pre-formed 100 % solid field-reacted epoxy based traffic paint is applied in  3 year intervals on all NJDOT maintained roadways. and that the application of this epoxy generates  negligible VOC emissions. 7

- Rule Effectiveness:

The emissions from this source category are regulated by the following rule:

National Volatile Organic Compound Emission Standards for Consumer and Commercial Products, CFR 40, Chapter 1, Subchapter C, Part 59, Subpart D National Volatile Organic Compound Standards for Architectural Coatings, operative September 13, 1999.

- Double Counting:

Emissions for this source category have not been reported in the point source inventory, therefore no adjustment of the area source emissions was required.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

- References:

1.  Procedures for the Preparation of Emission Inventories for Carbon Monoxideand Precursors of Ozone Vol I:  General Guidance for Stationary Sources,  United States Environmental Protection Agency, Office of Air  Quality Planning and Standards, Research Triangle Park, NC,  EPA-450/4-91-016, May 1991

2. New Jersey's Public Road Mileage by Functional Classification, Year Ending 1996, New Jersey Department of Transportation (NJDOT), Bureau of Transportation Data Development, Faxed by Jim Carl on February 19, 1998

2a. New Jersey's Public Road Mileage by Jurisdiction, Years Ending 1996 and 2002, New Jersey Department of Transportation (NJDOT), Bureau of Transportation Data Development

3.  NJDOT Specification for Latex (water based traffic paint and 100 % solid epoxy traffic paint,Section 912.34 Traffic Paint, NJDOT, Bureau of  Materials, faxed by Fred Lovett on May 9, 1998

4. Memo to file on traffic paint application on New Jersey roadways, May 9, 1998

- 5. National Volatile Organic Compound Emission Standards for Consumer and Commercial Products, CFR 40, Chapter 1, Subchapter C, Part 59, Subpart B-National Volatile Organic Compound Emission Standards for Automobile Refinish Coatings.

6. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction,  Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c

7.  Emission Inventory Improvement Program, Volume III, Area Sources Preferred  and Alternative Methods, Chapter 14, Traffic Markings,  Table 14.2-2, Advantages and Disadvantages of Alternative Traffic Marking Materials  (EPA, 1988), Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC, September 26, 1997

8.  Emission Inventory Improvement Program, Volume III, Area Sources Preferred  and Alternative Methods, Chapter 14, Traffic Markings,  Table 14.2-1, Comparison of Estimated VOC Emissions (EPA, 1988).

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Industrial Surface Coating Factory Finished Wood

- SCC:  2401015000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category:1

1.  Surveying individual factory finished wood facilities

2.  Employment based analysis

3.  Population based analysis

The employment based analysis/methodology is selected because the (1) input data elements are more readily available and (2) it is more accurate than population.

Emissions from factory finished wood surface coating applications are calculated using county level employment.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  County employment for SIC 2426-9,243-245,2492,2499 or NAICS 321113, 321211-4, 3219, 333414, 337215, 339999, CEMP 2

2.  Emission Factors, EF, (lbs/employee/year) 3

- VOC= 131

3.  Weekly activity, WAF=5 days/week4

4.  Seasonal adjustment factor, SAF=14

- Process:

The following equations are used to calculate the emissions without control for this source category.

Epa	= CEMP*EF/CF

Eps	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:  None

- Double Counting:

Emissions for this source category have also been reported in the point source inventory. The following methodology is used to adjust the area source emissions for this category to remove the emissions already accounted for in the point source inventory.

Epadj      = (CEMP - PTCEMP)*EF/CF

where:

Epadj    = (tons/yr) for an annual emission of pollutant by county excluding double counting

PTCEMP  = Point source employment in SIC code 2426-9, 243-245,2493,2499 5

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

- References:

1. Procedures for the Preparation of Emission Inventories for Carbon Monoxide and Precursors of Ozone Vol I:  General Guidance for Stationary Sources,    United States Environmental Protection Agency, Office of Air  Quality Planning and Standards, Research Triangle Park, NC,  EPA-450/4-91-016, May 1991

2. Total 2002 employment by 6 digit NAICS code and by county, New Jersey  Department of Labor.

3.  Emission Inventory Improvement Program, Volume III, Area Sources Preferred  and Alternative Methods, Chapter 8, Industrial Surface Coating, September 26, 1997

4. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction,   Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c

5. NJ Point Source Emission Inventory for 2002, New Jersey Department of Environmental Protection

Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Industrial Surface Coating Wood and Metal Furniture & Fixtures

- SCC:  2401025000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category:1

1.  Surveying furniture & fixtures facilities

2.  Employment based analysis

3.  Population based analysis

The employment based analysis/methodology is selected because the (1) input data elements are more readily available and (2) it is more accurate than population.

Emissions from wood and metal furniture and fixtures surface coating applications are calculated using county level employment.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  County employment for SIC 25 or NAICS 3371-2, 339111, 339942, CEMP 2

2.  Emission Factors, EF, (lbs/employee/year) 1

- VOC= 944

3. Control Efficiency, CE

The following equations are used to calculate the control efficiency for this source category:

- STEP ONE: Establish voc content (lbs/gal) prior to rule implementation

voc contentno rule	= (EF lbs/year)/(coating usage factor) (gal/year)

voc contentno rule	= (944 lbs/year 1)/(175 gal/year 1)

voc contentno rule	=  5.39 lbs/gal

- STEP TWO: Establish rule voc content

voc contentrule   =   3.0 lbs/gal 3

- STEP THREE: Calculate CE

CE = (voc contentno rule -  voc contentrule )/(voc contentno rule)

CE = (    5.3943           -        3  )/(5.3943) * 100 %

CE =    44.39 %

4.	Rule Effectiveness, RE = 80 % 1

5.	Rule Penetration, RP = 100 %

6. 	Weekly Activity, WAF = 5 days/week 4

7.	Seasonal adjustment factor, SAF  = 14

- Process:

The following equations are used to calculate the emissions with control for this source category.

Epa	= CEMP*EF*(1 -  (CE*RE*RP) )/CF

Eps	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

- Rule Effectiveness:

The emissions from this source category are regulated by the following rule:

New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 16, 7:27-16.7 Table 7B, Miscellaneous Surface Coating Operations Control Criteria and Compliance Dates

- Double Counting:

Emissions for this source category have also been reported in the point source inventory. The following methodology is used to adjust the area source emissions for this category to remove the emissions already accounted for in the point source inventory.

Epadj      = (CEMP - PTCEMP)*EF/CF

where:

Epadj    = (tons/yr) for an annual emission of pollutant by county excluding double counting

PTCEMP  = Point source employment in SIC code 25 5

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

- References:

1. Procedures for the Preparation of Emission Inventories for Carbon Monoxideand Precursors of Ozone Vol I:  General Guidance for Stationary Sources, May 1991, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC, EPA-450/4-91-016

2. Total 2002 employment by 6 digit NAICS code and by county, New Jersey  Department of Labor.

3.  New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 16, 7:27-16.7 Table 7B, Miscellaneous Surface Coating Operations Control Criteria and Compliance Dates

4. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction,  Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c

5. NJ Point Source Emission Inventory for 2002, New Jersey Department of Environmental Protection

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Industrial Surface Coating Metal Containers

- SCC:  2401040000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category:1

1.  Surveying individual metal container facilities

2.  Employment based analysis

3.  Population based analysis

The employment based analysis/methodology is selected because the (1) input data elements are more readily available and (2) it is more accurate than population.

Emissions from metal container surface coating applications are calculated using county level employment.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  County employment for SIC 341 or NAICS 33243, CEMP 2

2.  Emission Factors, EF, (lbs/employee/year) 3

- VOC= 6029

3.  Weekly activity, WAF=5 days/week 4

4.  Seasonal adjustment factor, SAF=14

- Process:

The following equations are used to calculate the emissions without control for this source category.

Epa	= CEMP*EF/CF

Eps	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:  None

- Double Counting:

Emissions for this source category have also been reported in the point source inventory. The following methodology is used to adjust the area source emissions for this category to remove the emissions already accounted for in the point source inventory.

Epadj      = (CEMP - PTCEMP)*EF/CF

where:

Epadj    = (tons/yr) for an annual emission of pollutant by county excluding double counting

PTCEMP  = Point source employment in SIC code 3415

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

- References:

1. Procedures for the Preparation of Emission Inventories for Carbon Monoxideand Precursors of Ozone Vol I:  General Guidance for Stationary Sources,   United States Environmental Protection Agency, Office of Air  Quality Planning and Standards, Research Triangle Park, NC, EPA-450/4-91-016, May 1991

2. Total 2002 employment by 6 digit NAICS code and by county, New Jersey  Department of Labor.

3.  Emission Inventory Improvement Program, Volume III, Area Sources Preferred  and Alternative Methods, Chapter 8, Industrial Surface Coating, Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC, September 26, 1997

4. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction,  Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c

5. NJ Point Source Emission Inventory for 2002, New Jersey Department of Environmental Protection

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Industrial Surface Coating Sheet, Strip, & Coil

- SCC:  2401050000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category:1

1.  Surveying individual sheet, strip, & coil facilities

2.  Employment based analysis

3.  Population based analysis

The employment based analysis/methodology is selected because the (1) input data elements are more readily available and (2) it is more accurate than population.

Emissions from sheet, stirp and coil surface coating applications are calculated using county level employment.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  County employment for SIC 3479 or NAICS 332812, 339911-2, 339914, CEMP 2

2.  Emission Factors, EF, (lbs/employee/year) 3

- VOC= 2877

3. Control Efficiency, CE = 58 % 4

4. Rule Effectiveness, RE = 80 % 1

5. Rule Penetration, RP = 100 %

6.  Weekly activity, WAF=5 days/week 5

7.  Seasonal adjustment factor, SAF=1 5

- Process:

The following equations are used to calculate the emissions without control for this source category.

Epa	= CEMP*EF*( 1- (CE*RE*RP) )/CF

Eps	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

- Rule Effectiveness:

The emissions from this source category are regulated by the following rule:

New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 16, 7:27-16.7 Table 7B, Miscellaneous 	Surface Coating Operations Control Criteria and Compliance Dates

- Double Counting:

Emissions for this source category have also been reported in the point source inventory. The following methodology is used to adjust the area source emissions for this category to remove the emissions already accounted for in the point source inventory.

Epadj      = (CEMP - PTCEMP)*EF/CF

where:

Epadj    = (tons/yr) for an annual emission of pollutant by county excluding double counting

PTCEMP  = Point source employment in SIC code 3479 6

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

- References:

1. Procedures for the Preparation of Emission Inventories for Carbon Monoxideand Precursors of Ozone Vol I:  General Guidance for Stationary Sources,  United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,   EPA-450/4-91-016, May 1991

2. Total 2002 employment by 6 digit NAICS code and by county, New Jersey  Department of Labor.

3. Emission Inventory Improvement Program, Volume III, Area Sources Preferred  and Alternative Methods, Chapter 8, Industrial Surface Coating, Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC, September 26, 1997

4. State Implementation Plan (SIP) Revision for the Attainment and Maintenance of the Ozone National Ambient Air Quality Standards, Meeting the Requirements of the Alternative Ozone Attainment Demonstration Policy, Appendix VI: 1990 Base Year Emission Inventory, Table 7:Minor Point Source Emission Calculation, The State of New Jersey, Department of Environmental Protection, December 31, 1996

5. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction, Emission Inventory

Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c

6. NJ Point Source Emission Inventory for 2002, New Jersey Department of Environmental Protection

Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Industrial Surface Coating Machinery & Equipment

- SCC:  2401055000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category: 1

1.  Surveying individual machinery & equipment facilities

2.  Employment based analysis

3.  Population based analysis

The employment based analysis/methodology is selected because the (1) input data elements are more readily available and (2) it is more accurate than population.

Emissions from machinery and equipment surface coating applications are calculated using county level employment.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  County employment for SIC 35 or NAICS 314999, 332212, 332323, 332439, 332991, 332997, 332999, 33311, 33313, 33329, 333311-3, 333319, 3334-41, 334518, 334613, 335311, 336391, 336399, 339942, CEMP 2

2.  Emission Factors, EF, (lbs/employee/year) 1

- VOC= 77

3. Control Efficiency, CE

The following equations are used to calculate the control efficiency for this source category:

- STEP ONE: Establish voc content (lbs/gal) prior to rule implementation

voc contentno rule	= (EF lbs/year)/(coating usage factor) (gal/year)

voc contentno rule	= (77 lbs/year1)/(17 gal/year1)

voc contentno rule	=  4.5294 lbs/gal

- STEP TWO: Establish average rule voc content from miscellaneous metal parts and products

coating operations voc content included in reference 3 as shown below in Table # 1:

- Table # 1

- Miscellaneous Metal Parts and Products

| Type of Operation | Voc Content (lbs/gallon) |
| --- | --- |
| clear coating | 4.3 |
| Air-dried coating | 3.5 |
| Extreme perf coating | 3.5 |
| All other coatings | 3.0 |

voc contentrule   =   (4.3  +  3.5 + 3.5 + 3.0 lbs/gal)/4 = 3.575

- STEP THREE: Calculate CE

CE = (voc contentno rule -  voc contentrule )/(voc contentno rule)

CE = (    4.5294     -   3.575  )/(4.5294) * 100 %

CE =         21.07  %

4. Rule Effectiveness, RE;  RE = 80 % 1

5. Rule Penetration, RP;  RP = 100 %

6. Weekly Activity, WAF = 5 days/week 4

7. Seasonal adjustment factor, SAF  = 1 4

- Process:

The following equations are used to calculate the emissions with control for this source category.

Epa	= CEMP*EF*(1 -  (CE*RE*RP) )/CF

Eps	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1. Assume a rule penetration factor of:  100 %

2. Assume that the  4  types of  miscellaneous metal parts and products voc coating operations

included in Table # 1 represent equivalent portions of the coating applied to miscellaneous

metal parts and products.

- Rule Effectiveness:

The emissions from this source category are regulated by the following rule:

New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 16, 7:27-16.7 Table 7B, Miscellaneous 	Surface Coating Operations Control Criteria and Compliance Dates

- Double Counting:

Emissions for this source category have also been reported in the point source inventory. The following methodology is used to adjust the area source emissions for this category to remove the emissions already accounted for in the point source inventory.

Epadj      = (CEMP - PTCEMP)*EF/CF

Epadj    = (tons/yr) for an annual emission of pollutant by county excluding double counting

Where:

PTCEMP  = Point source employment in SIC code 35 5

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

- References:

1.Procedures for the Preparation of Emission Inventories for Carbon Monoxide  and Precursors of Ozone Vol I:  General Guidance for Stationary Sources,  May 1991, United States Environmental Protection Agency, Office of Air   Quality Planning and Standards, Research Triangle Park, NC,  EPA-450/4-91-016

2. Total 2002 employment by 6 digit NAICS code and by county, New Jersey  Department of Labor.

3. New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 16, 7:27-16.7,Table 7B, Miscellaneous Surface Coating Operations Control Criteria and Compliance Dates

4. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction,  Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c

5. NJ Point Source Emission Inventory for 2002, New Jersey Department of Environmental Protection

Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Industrial Surface Coating Appliances

- SCC:  2401060000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category:1

1.  Surveying individual appliances facilities

2.  Employment based analysis

3.  Population based analysis

The employment based analysis/methodology is selected because the (1) input data elements are more readily available and (2) it is more accurate than population.

Emissions from appliances surface coating applications are calculated using county level employment.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  County employment for SIC 363 or NAICS 333298, 333414, 3352, 339999, CEMP 2

2.  Emission Factors, EF, (lbs/employee/year)1

- VOC= 463

3.  Weekly activity, WAF=5 days/week 3

4.  Seasonal adjustment factor, SAF=1 3

- Process:

The following equations are used to calculate the emissions without control for this source category.

Epa	= CEMP*EF/CF

Eps	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:  None

- Double Counting:

Emissions for this source category have not been reported in the point source inventory, therefore no adjustment of the area source emissions was required.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

- References:

1. Procedures for the Preparation of Emission Inventories for Carbon Monoxide  and Precursors of Ozone Vol I:  General Guidance for Stationary Sources,  May 1991, United States Environmental Protection Agency, Office of Air  Quality Planning and Standards, Research Triangle Park, NC,   EPA-450/4-91-016

2. Total 2002 employment by 6 digit NAICS code and by county, New Jersey  Department of Labor.

3. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction,  Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c

Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Industrial Surface Coating Electrical Insulation

- SCC:  2401065000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category:1

1.  Surveying individual electrical insulation facilities

2.  Employment based analysis

3.  Population based analysis

The employment based analysis/methodology is selected because the (1) input data elements are more readily available and (2) it is more accurate than population.

Emissions from electrial insulation surface coating applications are calculated using county level employment.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  County employment for SIC 3357,3612 or NAICS 331319, 331422, 331491, 335311, 33592, CEMP 2

2.  Emission Factors, EF, (lbs/employee/year) 3

- VOC= 290

3.  Weekly activity, WAF=5 days/week 4

4.  Seasonal adjustment factor, SAF=1 4

- Process:

The following equations are used to calculate the emissions without control  for this source category.

Epa	= CEMP*EF/CF

Eps	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:  None

- Double Counting:

Emissions for this source category have not been reported in the point source inventory.  If they were, the following methodology is used to adjust the area source emissions for this category to remove the emissions already accounted for in the point source inventory.

Epadj      = (CEMP - PTCEMP)*EF/CF

Epadj    = (tons/yr) for an annual emission of pollutant by county excluding double counting

Where:

PTCEMP  = Point source employment in SIC code 3357 and 3612 5

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

- References:

1. Procedures for the Preparation of Emission Inventories for Carbon Monoxideand Precursors of Ozone Vol I:  General Guidance for Stationary Sources,  United States Environmental Protection Agency, Office of Air  Quality  Planning and Standards, Research Triangle Park, NC, EPA-450/4-91-016, May 1991

2. Total 2002 employment by 6 digit NAICS code and by county, New Jersey  Department of Labor.

3. Emission Inventory Improvement Program, Volume III, Area Sources Preferred  and Alternative Methods, Chapter 8, Industrial Surface Coating, Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC, September 26, 1997

4. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction, Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c

5. NJ Point Source Emission Inventory for 2002, New Jersey Department of Environmental Protection

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Industrial Surface Coating New Automobile

- SCC:  2401070000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category:1

1.  Surveying individual new automobile facilities

2.  Employment based analysis

3.  Population based analysis

The employment based analysis/methodology is selected because the (1) input data elements are more readily available and (2) it is more accurate than population.

Emissions from new automobile surface coating applications are calculated using county level employment.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  County employment for SIC 3711 or NAICS 33611, 336211, 336992, CEMP 2

2.  Emission Factors, EF, (lbs/employee/year)

- VOC= 794 3

3. Control Efficiency, CE = 54 % 4

4. Rule Effectiveness, RE = 80 % 1

5. Rule Penetration, RP = 100 %

6.  Weekly activity, WAF=5 days/week 5

7.  Seasonal adjustment factor, SAF=1 5

- Process:

The following equations are used to calculate the emissions with control for this source category.

Epa	= CEMP*EF*( 1 - (CE*RE*RP) )/CF

Eps	= Epa*SAF./AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

- Rule Effectiveness:

The emissions from this source category are regulated by the following rule:

New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 16, 7:27-16.7 Table 7A, Automobile and 	Light Duty Truck Surface Coating Operations Control Criteria and Compliance Dates

- Double Counting:

Emissions for this source category have also been reported in the point source inventory. The following methodology is used to adjust the area source emissions for this category to remove the emissions already accounted for in the point source inventory.

Epadj      = (CEMP - PTCEMP)*EF/CF

Epadj    = (tons/yr) for an annual emission of pollutant by county excluding double  counting

Where:

PTCEMP  = Point source employment in SIC code 3711 6

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

- References:

1. Procedures for the Preparation of Emission Inventories for Carbon Monoxideand Precursors of Ozone Vol I:  General Guidance for Stationary Sources, United States Environmental Protection Agency, Office of Air  Quality Planning and Standards, Research Triangle Park, NC, EPA-450/4-91-016,  May 1991

2. Total 2002 employment by 6 digit NAICS code and by county, New Jersey  Department of Labor.

3. Emission Inventory Improvement Program, Volume III, Area Sources Preferred  and Alternative Methods, Chapter 8, Industrial Surface Coating, Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC, September 26, 1997

4. State Implementation Plan (SIP) Revision for the Attainment and Maintenance of the Ozone National Ambient Air Quality Standards, Meeting the Requirements of the Alternative Ozone Attainment Demonstration Policy, Appendix VI: 1990 Base Year Emission Inventory, Table 7:  Minor Point Source Emission Calculation, The State of New Jersey, Department of Environmental Protection, December 31, 1996

5. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction,  , Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park  NC,  EPA-454/R-97-004c

6. NJ Point Source Emission Inventory for 2002, New Jersey Department of Environmental Protection

Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Industrial Surface Coating Other Transportational Equipment

- SCC:  2401075000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category:1

1.  Surveying individual marine coating facilities

2.  Employment based analysis

3.  Population based analysis

The employment based analysis/methodology is selected because the (1) input data elements are more readily available and (2) it is more accurate than population.

Emissions from other transportational equipment surface coating applications are calculated using county level employment.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  County employment for SIC 37 except 3711 & 373, or NAICS 332912, 333911, 333924, 3362, 336312, 336322, 336399, 3364, 3369, CEMP 2

2.  Emission Factors, EF, (lbs/employee/year 3

- VOC= 35

3.  Weekly activity, WAF=5 days/week 4

4.  Seasonal adjustment factor, SAF=14

- Process:

The following equations are used to calculate the emissions without control  for this source category.

Epa	= CEMP*EF/CF

Eps	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:  None

- Double Counting:

Emissions for this source category have also been reported in the point source inventory. The following methodology is used to adjust the area source emissions for this category to remove the emissions already accounted for in the point source inventory.

Epadj      = (CEMP - PTCEMP)*EF/CF

Epadj    = (tons/yr) for an annual emission of pollutant by county excluding double counting

Where:

PTCEMP  = Point source employment in SIC code 37, except 3711 and 373 5

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

- References:

1. Procedures for the Preparation of Emission Inventories for Carbon Monoxide and Precursors of Ozone Vol I:  General Guidance for Stationary Sources, May 1991, United States Environmental Protection Agency, Office of Air  Quality Planning and Standards, Research Triangle Park, NC,    EPA-450/4-91-016

2. Total 2002 employment by 6 digit NAICS code and by county, New Jersey  Department of Labor.

3. Emission Inventory Improvement Program, Volume III, Area Sources Preferred  and Alternative Methods, Chapter 8, Industrial Surface Coating, September 26, 1997

4. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction,  Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c

5. NJ Point Source Emission Inventory for 2002, New Jersey Department of Environmental Protection

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Industrial Surface Coating Marine Coatings

- SCC:  2401080000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category:1

1.  Surveying individual marine coating facilities

2.  Employment based analysis

3.  Population based analysis

The employment based analysis/methodology is selected because the (1) input data elements are more readily available and (2) it is more accurate than population.

Emissions from marine surface coating applications are calculated using county level employment.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  County employment for SIC 373 or NAICS 3366, CEMP 2

2.  Emission Factors, EF, (lbs/employee/year) 3

- VOC= 308

3.  Weekly activity, WAF=5 days/week 4

4.  Seasonal adjustment factor, SAF=1 4

- Process:

The following equations are used to calculate the emissions without  control for this source category.

Epa	= CEMP*EF/CF

Eps	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:  None

- Double Counting:

Emissions for this source category have also been reported in the point source inventory. The following methodology is used to adjust the area source emissions for this category to remove the emissions already accounted for in the point source inventory.

Epadj      = (CEMP - PTCEMP)*EF/CF

Where:

Epadj    = (tons/yr) for an annual emission of pollutant by county excluding double counting

PTCEMP  = Point source employment in SIC code 373 5

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

- References:

1. Procedures for the Preparation of Emission Inventories for Carbon Monoxideand Precursors of Ozone Vol I:  General Guidance for Stationary Sources, May 1991, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC, EPA-450/4-91-016

2. Total 2002 employment by 6 digit NAICS code and by county, New Jersey  Department of Labor.

3. Emission Inventory Improvement Program, Volume III, Area Sources Preferred  and Alternative Methods, Chapter 8, Industrial Surface Coating, September 26, 1997

4. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction,  Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c

5. NJ Point Source Emission Inventory for 2002, New Jersey Department of Environmental Protection

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Industrial Surface Coating Other Product Surface Coating

- SCC:  2401090000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category:1

1.  Surveying wholesale & retail suppliers of architectural coatings

2.  Population based analysis

The population based analysis/methodology is selected because the input data elements are more readily available.

Emissions from other product surface coatings are calculated using county level population.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  County population, CPOP 2

2.  Emission Factors, EF, (lbs/year/capita) 3

- VOC= 0.6

3.  Weekly activity, WAF=5 days/week 4

4.  Seasonal adjustment factor, SAF=1 4

- Process:

The following equations are used to calculate the emissions without control for this source category.

Epa	= CPOP*EF/CF

Eps	= Epa*SAF//ADDF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

- Rule Effectiveness:

The emissions from this source category are regulated by the following rule:  None

- Double Counting:

Emissions for this source category have not been reported in the point source inventory, therefore no adjustment of the area source emissions was required.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

- References:

1. Procedures for the Preparation of Emission Inventories for Carbon Monoxide and Precursors of Ozone Vol I:  General Guidance for Stationary Sources, May 1991, United States Environmental Protection Agency, Office of Air   Quality Planning and Standards, Research Triangle Park, NC,    EPA-450/4-91-016

2. Estimate of  2002 Total Resident Population by County, NJDOT, July 17, 2003.

3. Emission Inventory Improvement Program, Volume III, Area Sources Preferred  and Alternative Methods, Chapter 8, Industrial  Surface Coating, Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC September 26, 1997.

4. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction,  Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Architectural and Industrial Surface Coating, High Performance Maintenance

- SCC:  2401100000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category: 1

1.  Surveying individual high performance maintenance facilities

2.  Population based analysis

The population based analysis/methodology is selected because the input data elements are more readily available.

Emissions from high maintenance surface coating application are calculated using county level population.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  County population, CPOP 2

2.  Emission Factors, EF, (lbs/capita/year) 3

- VOC= 0.8

3.  Control Efficiency, CE = 44 % 4

4.  Rule Effectiveness, RE = 80 % 1

5.  Rule Penetration, RP = 100 %

6.  Weekly activity, WAF=5 days/week 5

7.  Seasonal adjustment factor, SAF 5

Summer Season Adjustment Factor		1.32

Fall Season Adjustment Factor			0.89

Winter Season Adjustment Factor		0.89

Spring Season Adjustment Factor			0.89

- Process:

The following equations are used to calculate the emissions with control for this source category.

Epa	= CPOP*EF*( 1 - (CE*RP*RE) )/CF

Eps	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

- Rule Effectiveness:

The emissions from this source category are regulated by the following rule:

1.  New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 23, Architectural Coatings, February 21, 1989

2. National Volatile Organic Compound Emission Standards for Consumer and Commercial Products, CFR 40, Chapter 1, Subchapter C, Part 59, Subpart D National Volatile Organic Compound Standards for Architectural Coatings, operative September 13, 1999.

- Double Counting:

Emissions for this source category have not been reported in the point source inventory, therefore no adjustment of the area source emissions was required.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

- References:

1. Procedures for the Preparation of Emission Inventories for Carbon Monoxideand Precursors of Ozone Vol I:  General Guidance for Stationary Sources, May 1991, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-450/4-91-016

2. Estimate of  2002 Total Resident Population by County, NJDOT, July 17, 2003.

3.  Emission Inventory Improvement Program, Volume III, Area Sources Preferred  and Alternative Methods, Chapter 8, Industrial Surface Coating, September 26, 1997, Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

4. State Implementation Plan (SIP) Revision for the Attainment and Maintenance of the Ozone National Ambient Air Quality Standards, Meeting the Requirements of the Alternative Ozone Attainment Demonstration Policy, Appendix VI: 1990 Base Year Emission Inventory, Table 10: Area Source Emission Calculation Data, December 31, 1996, the State of New Jersey, Department of Environmental Protection

5. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction,  Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Architectural and Industrial Surface Coating, Other Special Purpose Surface Coating

- SCC:  2401200000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category: 1

1.  Surveying wholesale & retail suppliers of architectural coatings

2.  Population based analysis

The population based analysis/methodology is selected because the input data elements are more readily available.

Emissions from other special purpose coatings are calculated using county level population.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  County population, CPOP 2

2.  Emission Factors, EF, (lbs/year/capita) 3

- VOC= 0.8

3. Control Efficiencies, CE = 1.45% 4

4. Rule Effectiveness, RE = 80 % 1

5. Rule Penetration, RP = 100 %

6. Weekly activity, WAF=5 days/week 5

7. Seasonal adjustment factor, SAF 5

Summer Season Adjustment Factor		1.32

Fall Season Adjustment Factor			0.89

Winter Season Adjustment Factor		0.89

Spring Season Adjustment Factor			0.89

- Process:

The following equations are used to calculate the emissions with control for this source category.

Epa	= CPOP*EF*( 1 - (CE*RP*RE) )/CF

Eps	= Epa*SAF//ADDF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

- Rule Effectiveness:

The emissions from this source category are regulated by the following rule:

1.  New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 23, Architectural Coatings, February 21, 1989

2. National Volatile Organic Compound Emission Standards for Consumer and Commercial Products, CFR 40, Chapter 1, Subchapter C, Part 59, Subpart D National Volatile Organic Compound Standards for Architectural Coatings, operative September 13, 1999.

- Double Counting:

Emissions for this source category have not been reported in the point source inventory, therefore no adjustment of the area source emissions was required.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

- References:

1. Procedures for the Preparation of Emission Inventories for Carbon Monoxide and Precursors of Ozone Vol I:  General Guidance for Stationary Sources,  United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC, EPA-450/4-91-016, May 1991

2. Estimate of  2002 Total Resident Population by County, NJDOT, July 17, 2003.

3. Emission Inventory Improvement Program, Volume III, Area Sources Preferred  and Alternative Methods, Chapter 8, Industrial  Surface Coating,  Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  September 26, 1997

4.  The State of New Jersey Department of Environmental Protection, State Implementation Plan (SIP) Revision for the Attainment and Maintenance of the Ozone National Ambient Air Quality Standard (NAAQS) New Jersey 1996 Actual Emission Inventory and Rate of Progress (ROP) Plans for 2002, 2005 and 2007, Appendix I, Attachment I-G, March 31, 2001.

5. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction,  Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Degreasing

- SCC:  2415000000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category: 1

1.  Surveying individual degreasing facilities

2.  Employment based analysis

3.  Population based analysis

The employment based analysis/methodology is selected because (1) the input data elements are more readily available and (2) it is more accurate than population.

Emissions from degreasing operations are calculated using county level employment.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  County employment for SIC 25, 33-39, 417, 423, 551, 552, 554-556, 753 or NAICS 1521*,23511*,314999,31611*,322225,322291,324199,325992,325998,326192,326199,326212,331*,332*,333*,334*,335*,336*,337*,339*,441*,442299,4431*,447*,45111*,45299*,48839*,48841*,48849*,51222*,53211*,53212*,54171*,561622,56179*,562991,562998,71151*,811*,81293*, CEMP 2

2.  Emission Factors, EF, (lbs/employee/year)

- VOC= 87 3

3. Control Efficiencies, CE;   CE = 60 %4

4. Rule Effectiveness, RE;  RE = 80 % 1

5. Rule Penetration, RP;  RP = 100 %

6. Weekly activity, WAF=6 days/week 5

7.  Seasonal adjustment factor, SAF=1 5

- Process:

The following equations are used to calculate the emissions with control for this source category.

Epa	= CEMP*EF*( 1- (CE*RE*RP) )/CF

Eps	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

- Rule Effectiveness:

The emissions from this source category are regulated by the following rule:

New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 16, 7:27-16.6, Open Top Tanks and Surface Cleaners, September 22, 1986

- Double Counting:

Emissions for this source category have also been reported in the point source inventory. The following methodology is used to adjust the area source emissions for this category to remove the emissions already accounted for in the point source inventory.

Epadj      = (CEMP - PTCEMP)*EF/CF

Epadj    = (tons/yr) for an annual emission of pollutant by county excluding double counting

Where:

PTCEMP  = Point source employment in SIC code 25, 33-39, 417, 423, 551, 552, 554-556, 753 6

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

- References:

1. Procedures for the Preparation of Emission Inventories for Carbon Monoxideand Precursors of Ozone Vol I:  General Guidance for Stationary Sources, May 1991, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,    EPA-450/4-91-016

2. Total 2002 employment by 6 digit NAICS code and by county, New Jersey  Department of Labor.

3. Emission Inventory Improvement Program, Volume III, Area Sources Preferred  and Alternative Methods, Chapter 6, Solvent Cleaning, Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC, September  1997

4. State Implementation Plan (SIP) Revision for the Attainment and Maintenance of the Ozone National Ambient Air Quality Standards, Meeting the Requirements of the Alternative Ozone Attainment Demonstration Policy, Appendix VI: 1990 Base Year Emission Inventory, Table 7:Minor Point Source Emission Calculation, The State of New Jersey, Department of Environmental Protection, December 31, 1996

5. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction,

Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c

6. NJ Point Source Emission Inventory for 2002, New Jersey Department of Environmental Protection

Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Commercial/Industrial Dry Cleaning

- SCC:  2420010000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category: 1

1.  Surveying individual commercial/industrial cleaning facilities

2.  Employment based analysis

3.  Population based analysis

The employment based analysis/methodology is selected because the (1) input data elements are more readily available and (2) it is more accurate than population.

Emissions from dry cleaning operations are calculated using county level employment.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  County employment for SIC 7216 or NAICS 81231, 81233, CEMP 2

2.  Emission Factors, EF, (lbs/employee/year) 3

- VOC= 1800

3. Control Efficiencies, CE = 92 % 4

4. Rule Effectiveness, RE = 80 % 1

5. Rule Penetration, RP = 100 %

6.  Weekly activity, WAF=5 days/week 5

7.  Seasonal adjustment factor, SAF=1 5

- Process:

The following equations are used to calculate the emissions with control for this source category.

Epa	= CEMP*EF*( 1 - (CE*RE*RP) )/CF

Eps	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1. Assume a rule penetration factor of 100 % percent.

2.  Assume that the emission factor of 1800 lbs/year/employee  from table 4.5-1 of reference 3  represents the total  VOC emissions from all reactive solvents used in all dry cleaning operations.

3. Assume that all coin operated dry cleaning operations use only the nonreactive solvents which do not generate VOC emissions.

- Rule Effectiveness:

The emissions from this source category are regulated by the following rule:

New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 16, 7:27-16.20, Petroleum Solvent Dry 	   Cleaning Operations, September 22, 1986

- Double Counting:

Emissions for this source category have not been reported in the point source inventory, therefore no adjustment of the area source emissions was required.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

- References:

1. Procedures for the Preparation of Emission Inventories for Carbon Monoxideand Precursors of Ozone Vol I:  General Guidance for Stationary Sources,  United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,    EPA-450/4-91-016,  May 1991

2. Total 2002 employment by 6 digit NAICS code and by county, New Jersey  Department of Labor.

3. Emission Inventory Improvement Program, Volume III, Area Sources Preferred  and Alternative Methods, Chapter 4, Dry  Cleaning,  Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  May 17, 1996

4. State Implementation Plan (SIP) Revision for the Attainment and Maintenance of the Ozone National Ambient Air Quality Standards, Meeting the Requirements of the Alternative Ozone Attainment Demonstration Policy, Appendix VI: 1990 Base Year Emission Inventory, Table 7:Minor Point Source Emission Calculation, The State of New Jersey, Department of Environmental Protection, December 31, 1996

5. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction, Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c

Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Graphic Arts (Offset lithography/letterpress)

- SCC:  2425000000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category: 1

1.  Surveying individual graphic arts facilities

2.  Population based analysis

The population based analysis/methodology is selected because the input data elements are more readily available.

Emissions from graphic arts operations are calculated using county level population.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  County Population, CPOP 2

2. Percentage of  population representative of offset lithography/letterpress operations,

PERPOP = 28 + 18 = 46 % 3

3.  Emission Factors, EF, (lbs/person/year) 4

- VOC= 1.3

4.  Weekly activity, WAF=5 days/week 5

5.  Seasonal adjustment factor, SAF=1 5

- Process:

The following equations are used to calculate the emissions without control efficiency for this source category.

Epa	= CPOP*PERPOP*EF/CF

Eps	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1.  All solvent consumed is emitted; no accounting for waste solvent recycling and disposal

2.  The 46.0 % of  population that utilizes offset lithography/letterpress within the graphic arts inventory is presumed to be valid.

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:  None

- Double Counting:

Emissions for this source category have also been reported in the point source inventory.   The following methodology is used to adjust the area source emissions for this category to remove the emissions already accounted for in the point source inventory.

- Epadj = Epa - PT

Where:

Epadj  = (tons/yr) for an annual emission of pollutant by county excluding double counting

PT    = Point source emissions (tons/yr) 6

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

- References:

1. Procedures for the Preparation of Emission Inventories for Carbon Monoxideand Precursors of Ozone Vol I:  General Guidance for Stationary Sources, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC, EPA-450/4-91-016, May 1991

2. Estimate of  2002 Total Resident Population by County, NJDOT, July 17, 2003.

3. Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, Section 4.9.1.2,  , United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

4. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 7 - Graphic Arts,  November 18, 1996, Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c

5. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction,  Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c

6. NJ Point Source Emission Inventory for 2002, New Jersey Department of Environmental Protection

Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Graphic Arts (rotogravure/flexography)

- SCC:  2425000000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category: 1

1.  Surveying individual graphic arts facilities

2.  Population based analysis

The population based analysis/methodology is selected because the input data elements are more readily available.

Emissions from graphic arts operations are calculated using county level population.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  County Population, CPOP 2

2. Percentage of  population representative of rotogravure/flexography operations,

PERPOP = 41 + 13 = 54 % 3

3.  Emission Factors, EF, (lbs/person/year) 4

- VOC= 1.3

4. Control Efficiencies, CE

The following equations are used to calculate the control efficiency for this source category:

- STEP ONE: Establish voc content (lbs/gal) prior to rule implementation

voc contentno rule	=  75 % by  volume for both rotogravure 5 and flexography 6

- STEP TWO: Establish rule voc content

voc contentrule   =   25 % lbs/gal 7

- STEP THREE: Calculate CE

CE = (voc contentno rule -  voc contentrule )/(voc contentno rule)

- CE = ( 75 - 25)/(75) * 100 %

CE =    66.67  %

5. Rule Effectiveness, RE = 80 % 1

6. Rule Penetration, RP = 100 %

7. Weekly activity, WAF=6 days/week 8

8. Seasonal adjustment factor, SAF=1 8

- Process:

The following equations are used to calculate the emissions with control for this source category.

Epa	= CPOP*PERPOP*EF*( 1 - (CE*RE*RP) )/CF

Eps	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1.  All solvent consumed is emitted; no accounting for waste solvent recycling  and disposal.

2.  The 54.0 % of  population that utilizes rotogravure and flexography operations within the graphic arts inventory is presumed to be valid.

3.   Assume that the 75 % by  volume voc content for rotogravure and flexography operations corresponds to 0.75  lb/gal.

4. Assume that voc content limitation  of  25 % for graphic arts source operation  required by the rule cited by

reference 7, applies to all rotogravure and flexography operations conducted in New Jersey.

5. Assume a rule penetration factor of  100 %.

- Rule Effectiveness:

The emissions from this source category are regulated by the following rule:

New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 16, 7:27-16.7(4), Surface Coating and Graphic Arts Operations, Table 7D Graphic Arts Operations, Part B, Control Criteria for Graphic Arts Source Operations Except Screen Printing Operations,  December 31, 1981 and December 31, 1987 and May 31, 1995

- Double Counting:

Emissions for this source category have also been reported in the point source inventory.   The following methodology is used to adjust the area source emissions for this category to remove the emissions already accounted for in the point source inventory.

Epadj = Epa - PT

Where:

Epadj  = (tons/yr) for an annual emission of pollutant by county excluding double counting

PT    = Point source emissions (tons/yr) 9

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

- References:

1. Procedures for the Preparation of Emission Inventories for Carbon Monoxideand Precursors of Ozone Vol I:  General Guidance for Stationary Sources,  May 1991, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-450/4-91-016

2. Estimate of  2002 Total Resident Population by County, NJDOT, July 17, 2003.

3. Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, Section 4.9.1.2, page 4.9.1-6, , United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

4.  EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 7 - Graphic Arts,  November 18, 1996, Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c

5. Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, Section 4.9.1.1.3 , page 4.9.1-3

6. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 7 - Graphic Arts,  Section 2.2 Flexographic Printing, page 7.2-7

7. New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 16, 7:27-16.7(4), Surface Coating and Graphic Arts Operations, Table 7D Graphic Arts Operations, Part B, Control Criteria for Graphic Arts Source Operations Except Screen Printing Operations,  December 31, 1981 and December 31, 1987 and May 31, 1995

8. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction,  Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c

- 9. NJ Point Source Emission Inventory for 2002, New Jersey Department of Environmental Protection

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Industrial Adhesives

- SCC:  2440020000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodology is used for this source category:

1.  California Air Resources Board (CARB)/Population based analysis

Emissions from industrial adhesive use and application are calculated using the CARB calculations and allocating emissions to the county level using population.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  County population, CPOP 1

2.  Emission Factor, EF, (lbs/year/capita) 2

VOC = 45 tons/day * 365 days/year * 2000 lbs/ton / 29,760,021 capita where 45 tons/day

is the estimated state-wide emissions for industrial adhesives in California, 2000 lbs/ton is a conversion factor, and 29,760,021 capita is the 1990 population of California.

VOC = 1.1

3.  Weekly activity, WAF=7 days/week

4. Seasonal adjustment factors SAF = 1

- Process:

The following equations are used to calculate the emissions with out control for this source category.

Epa	= CPOP*EF/CF

Eps	= Epa*SAF//ADDF

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

- Rule Effectiveness:

The emissions from this source category are regulated by the following rule:   None

- Double Counting:

Emissions for this source category have not been reported in the point source inventory, therefore no adjustment of the area source emissions was required.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

- References:

1. Estimate of  2002 Total Resident Population by County, NJDOT, July 17, 2003

- 2.  Determination of Reasonably Available Control Technology and Best Available Retrofit Control Technology for Adhesives and Sealants, California Air Resources Board, December 1998.

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Asphalt Application Paving Asphalt

- SCC:  2461020000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category:1

1.  Surveying individual construction contractors

2.  Population based analysis

3.  Volume of asphalt used analysis

The Asphalt Institute supplied NJDEP with total asphalt use in NJ.  With this information, AP-42's volume of asphalt use methodology is selected because of the available data.

Emissions from asphalt paving applications are calculated using  statewide asphalt use, allocated to the county level using county lane mileage.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  NJ asphalt cement, AC = 922,685 tons/yr 2

2.  State and county lane mileage, SLM, CLM 3

= (1996 estimated lane miles) * (growth factors 1996-2002) 3a

3.  Emission Factors, EF, (lbs/ton asphalt cement) 4

VOC= 0.0006 lbs/ton asphalt cement

4.  Weekly activity, WAF=5 days/week

5.  Seasonal adjustment factor, SAF=1

- Process:

The following equations are used to calculate the emissions without control efficiency for this source category.

Epa	= AC*EF*(CLM/SLM)/CF

Eps	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1.  Assume that the asphalt cement emission factor calculation provided by reference 4 is valid.

2.  All asphalt cement was used for paving purposes.

3.  Assume a weekly activity factor of 5 days/week.

4. Assume a SAF of  1.0.

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:   None

- Double Counting:

Emissions for this source category have not been reported in the point source inventory, therefore no adjustment of the area source emissions was required.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

- References:

1. Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, Section 4.5, Asphalt Paving Operations, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

2. New Jersey Asphalt Usage 2001, Asphalt Institute, Carlos Rosenberger, crosenberger@asphaltinstitute.org.

3. New Jersey's Public Road Mileage by Jurisdiction, Years Ending 1996 and 2002, New Jersey Department of Transportation (NJDOT), Bureau of Transportation Data Development

4. Daily Amount of Organic Emissions from Hot Mix Paving Operations, Fax transmittal from Una Connolly of the National Asphalt Pavement Association, March 29, 2000

Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Asphalt Application Cutback Asphalt

- SCC:  2461021000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category: 1

1.  Surveying individual construction contractors

2.  Population based analysis

3.  Volume of asphalt used analysis

The Asphalt Institute supplied NJDEP with total asphalt use in NJ.  With this information, AP-42's volume of asphalt use methodology is selected because of the available data.

Emissions from cutback asphalt applications are calculated using  statewide cutback asphalt use, allocated to the county level using county lane mileage.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  NJ cutback asphalt use, CU= 5,313 tons/yr 2

2.  State and county lane mileage, SLM, CLM 3

= (1996 estimated lane miles) * (growth factors 1996-2002) 3

3.  Evaporative VOC emissions by weight content from medium cure cutback asphalt with 35 % of Diluent in Cutback, CONTENT = 20% 4, 5

4.  EF (lb/ton cutback asphalt)

VOC = CONTENT  *  2000 lb/ton cutback

VOC =     0.20       *   2000 lb/ton cutback

VOC =           400 lb/ton cutback

5.  Weekly activity, WAF=5 days/week

6.  Seasonal adjustment factor, SAF=1

- Process:

The following equations are used to calculate the emissions without  control  for this source category.

Epa	= CU*EF*(CLM/SLM)/CF

Eps	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1.  Average diluent content by volume is 35%

2.  Medium cure evaporative loss of 70% is assumed

3.  All cutback asphalt was used for paving purposes

4.  Assume a weekly activity factor of 5 days/week

5.  Assume a SAF of  1.0 and no control efficiency, regardless of  the New Jersey rule 5 prohibition on cutback usage during ozone season.  NJDEP does not have any enforcement mechanism for this rule.

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:

New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 16, N.J.A.C. 7:27-16.19, New Jersey State Department of Environmental Protection,  March 2, 1992

- Double Counting:

Emissions for this source category have not been reported in the point source inventory, therefore no adjustment of the area source emissions was required.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

- References:

1. Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, Section 4.5, Asphalt Paving Operations, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

2. New Jersey Asphalt Usage 2001, Asphalt Institute, Carlos Rosenberger, crosenberger@asphaltinstitute.org.

3. New Jersey's Public Road Mileage by Jurisdiction, Years Ending 1996 and 2002, New Jersey Department of Transportation (NJDOT), Bureau of Transportation Data Development

4. Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, Section 4.5, Table 4.5-1, EVAPORATIVE VOC EMISSIONS FROM CUTBACK ASPHALTS AS A FUNCTION OF DILUENT CONTENT AND CUTBACK ASPHALT TYPE

5. New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 16, N.J.A.C. 7:27-16.19,

New Jersey State Department of Environmental Protection,  March 2, 1992

Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Asphalt Application Emulsified Asphalt

- SCC:  2461022000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category:1

1.  Surveying individual construction contractors

2.  Population based analysis

3.  Volume of asphalt used analysis

The Asphalt Institute supplied NJDEP with total asphalt use in NJ.  With this information, AP-42's volume of asphalt use methodology is selected because of the available data.

Emissions from emulsified asphalt applications are calculated using statewide emulsified asphalt use, allocated to the county level using county lane mileage.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  NJ emulsified asphalt use, EU=21,021 tons/yr 2

2.  State and county lane mileage, SLM, CLM 3

= (1996 estimated lane miles) * (growth factors 1996-2002)3

3.  Evaporative VOC emissions by weight content from emulsified asphalt, CONTENT = 8% 4, 5

4.  Emission factor, EF, (lbs/ton of asphalt)

VOC = CONTENT  *  2000 lb/ton cutback

VOC =      0.08       *   2000 lb/ton cutback

VOC =          160 lb/ton emulsified asphalt applied

5.  Weekly activity, WAF=5 days/week

6.  Seasonal adjustment factor, SAF=1

- Process:

The following equations are used to calculate the emissions without control  for this source category.

Epa	= EU*EF*(CLM/SLM)/CF

Eps	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1.  Major use of emulsion asphalt is for road paving

2.  Assume 100 % evaporation of  the VOC portion of  the  emulsified asphalt.

3.  Assume percent VOC by volume percent is equivalent to percent VOC by weight

because emulsified asphalt has a density nearly equivalent to water.

4.  Assume a SAF of  1.0 and no control efficiency, regardless of  the New Jersey rule 5 prohibition on cutback usage during ozone season.  NJDEP does not have any enforcement mechanism for this rule.

5.  Assume a weekly activity factor of 5 days per week

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:

New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 16, N.J.A.C. 7:27-16.19, New Jersey State Department of Environmental Protection,  March 2, 1992

- Double Counting:

Emissions for this source category have not been reported in the point source inventory, therefore no adjustment of the area source emissions was required.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

- References:

1. Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, Section 4.5, Asphalt Paving Operations, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

2. New Jersey Asphalt Usage 2001, Asphalt Institute, Carlos Rosenberger, crosenberger@asphaltinstitute.org.

3. New Jersey's Public Road Mileage by Jurisdiction, Years Ending 1996 and 2002, New Jersey Department of Transportation (NJDOT), Bureau of Transportation Data Development

4. Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, Section 4.5, Table 4.5-1, EVAPORATIVE VOC EMISSIONS FROM CUTBACK ASPHALTS AS A FUNCTION OF DILUENT CONTENT AND CUTBACK ASPHALT TYPE

5. New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 16, N.J.A.C. 7:27-16.19, New Jersey State Department of Environmental Protection, March 2, 1992

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Asphalt Application Roofing Asphalt

- SCC:  2461023000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category:1

1.  Surveying individual construction contractors

2.  Population based analysis

3.  Volume of asphalt used analysis

The Asphalt Institute supplied NJDEP with total asphalt use in NJ.  With this information, AP-42's volume of asphalt use methodology suggested for paving operations is selected for roofing applications because of the available data.

Emissions from asphalt roofing applications are calculated using  statewide roofing asphalt use, allocated to the county level using employment.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  NJ roofing asphalt use, RU=71,776 tons 2

2.  Percentage of roofing asphalt used for direct application to roofs:PERCENT = 40 % 3

3.  State and county totals for employment in SIC 15 or NAICS 2362, NJEMP, CEMP 4

4.  Emission factor, EF, (lbs/ton of asphalt) 5

VOC = 6  lb/ton melted asphalt

5.  Weekly activity, WAF=5 days/week

6.  Seasonal adjustment factor, SAF=1

- Process:

The following equations are used to calculate the emissions without control for this source category.

Epa	= RU*PERCENT*EF*(CEMP/NJEMP)

Eps	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1.   Assume that the emission factor of  6 pounds VOC/ton of asphalt melted from reference 5 is representative of voc emissions from the direct application of  melted asphalt to roofs.

2.   Assume that only 40 % of roofing asphalt usage obtained from reference 2 constitutes direct application of melted asphalt to roofing structures.

3.  Building contractors are assumed to only do asphalt roofing in a localized  area

4.  Assume a weekly activity factor of 5 days per week

5.  Assume a seasonal adjustment factor of 1

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:    None

- Double Counting:

Emissions for this source category have not been reported in the point source inventory, therefore no adjustment of the area source emissions was required.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

- References:

1.  Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, Section 4.5, Asphalt Paving Operations, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

2. New Jersey Asphalt Usage 2001, Asphalt Institute, Carlos Rosenberger, crosenberger@asphaltinstitute.org.

3. April 6, 2000  E-mail from Tonalee Key concerning telephone conversation with Russ Synder of the Asphalt Roofing Manufacturers Assoc.

4. Total 2002 employment by 6 digit NAICS code and by county, New Jersey  Department of Labor.

5. "STRAWMAN" FOR AREA SOURCE CATEGORY ABSTRACT,  Asphalt Roofing Kettles (SIC 1761),Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition,  January 21, 1998, Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Pesticide Application, Agricultural

- SCC:  2461800000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category: 1

1.  Surveying individual areas for pesticide applications

This is the only recommended method and was used in developing the emission inventory.

Emissions from agricultural pesticide applications are calculated using county level agricultural pesticide use.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  2000 county totals of agricultural pesticide applications, AGRI (lbs) 2

2.  Uncontrolled Emission Factors for Pesticide Active Ingredients, EFA

VOC = 1160 lb/ton pest applied = 0.58 lb/lb pest applied 3

3.  Average VOC Content of Pesticide Inert Ingredient Portion, by Formulation Type, EFI

VOC = 66 % lb/lb pest applied 3

4.  Amount inert portion in relation to active portion: 1.45*AGRI 3

5.  Overall Emission factor in relation to AGRI, EF (lbs/lb pest applied) 3

- EF = EFA + 1.45*EFI

- EF = 0.58 + 1.45*0.66

EF = 1.537

6.  Weekly activity, WAF=6 days/week 4

7.  Seasonal adjustment factor, SAF 4

Summer Season Adjustment Factor		1.32

Fall Season Adjustment Factor			0.89

Winter Season Adjustment Factor		0.89

Spring Season Adjustment Factor			0.89

- Process:

The following equations are used to calculate the emissions without control for this source category.

Epa	= AGRI*EF/CF

Eps	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

CF   = Conversion factor for units = 2,000 lbs/ton

Assumptions:

1. Assume that  the inert portion of the pesticide applied is 1.45 times the active ingredient. 3

2. Assume that the average VOC content of the pesticide inert ingredient portion, by formulation type represents 66 % which represents the highest % weight of inert portion included in Table 9.4-3 from reference 3.

3. Assume that all pesticides are applied to the surface and have a vapor pressure of >1 x 10-4  as referenced in table 9.4-4 from reference 3.

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:    None

- Double Counting:

Emissions for this source category have not been reported in the point source inventory, therefore no adjustment of the area source emissions was required.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

- References:

1. Procedures for the Preparation of Emission Inventories for Carbon Monoxideand Precursors of Ozone, Vol. I:  General Guidance for Stationary Sources, May 1991, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC, EPA-450/4-91-016

2. Agricultural Pesticide Use in New Jersey: 2000 Survey, the Bureau of Pesticide Operations,  New Jersey Department of Environmental Protection, Pesticide Control Program, West Trenton

3. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 9 - Pesticides - Agricultural and Nonagricultural,  December 1997, Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

4. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction,  Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c4.

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Pesticide Application, Golf Courses

- SCC:  2461800000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category:1

1.  Surveying individual areas for pesticide applications

This is the only recommended method and was used in developing the emission inventory.

Emissions from golf course pesticide applications are calculated using county level golf course pesticide use.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  2002 County totals of golf course pesticide applications, AGRI (lbs) 2

2.  Uncontrolled Emission Factors for Pesticide Active Ingredients, EFA

VOC = 1160 lb/ton pest applied = 0.58 lb/lb pest applied 3

3.  Average VOC Content of Pesticide Inert Ingredient Portion, by Formulation Type, EFI

VOC = 66 % lb/lb pest applied 3

4.  Amount inert portion in relation to active portion: 1.45*AGRI 3

5.  Overall Emission factor in relation to AGRI, EF (lbs/lb pest applied) 3

- EF = EFA + 1.45*EFI

- EF = 0.58 + 1.45*0.66

EF = 1.537

6.  Weekly activity, WAF=6 days/week 4

7.  Seasonal adjustment factor, SAF 4

Summer Season Adjustment Factor		1.32

Fall Season Adjustment Factor			0.89

Winter Season Adjustment Factor		0.89

Spring Season Adjustment Factor			0.89

- Process:

The following equations are used to calculate the emissions without control for this source category.

Epa	= AGRI*EF/CF

Eps	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

CF   = Conversion factor for units = 2,000 lbs/ton

Assumptions:

1.  Assume that  the inert portion of the pesticide applied is 1.45 times the active ingredient.3

2.  Assume that the average VOC content of the pesticide inert ingredient portion, by formulation type represents 66 % which represents the highest % weight of inert portion included in Table 9.4-3 from reference 3.

3.  Assume that all pesticides are applied to the surface and have a vapor pressure of >1 x 10-4  as referenced in table 9.4-4 from reference 3.

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:   None

- Double Counting:

Emissions for this source category have not been reported in the point source inventory, therefore no adjustment of the area source emissions was required.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

- References:

1. Procedures for the Preparation of Emission Inventories for Carbon Monoxideand Precursors of Ozone, Vol. I:  General Guidance for Stationary Sources,  May 1991, United States Environmental Protection Agency, Office of Air  Quality Planning and Standards, Research Triangle Park, NC, EPA-450/4-91-016

2. New Jersey 2002 Golf Course Pesticide Use Survey, the Bureau of Pesticide Operations,  New Jersey Department of Environmental Protection, Pesticide Control Program, West Trenton

3. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 9 - Pesticides - Agricultural and Nonagricultural,  December 1997, Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

4. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction,  Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c4.

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Pesticide Application, Lawn and Garden

- SCC:  2461800000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category:1

1.  Surveying individual areas for pesticide applications

This is the only recommended method and was used in developing the emission inventory.

Emissions from lawn and garden pesticide applications are calculated using county level lawn and garden pesticide use.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  2001 County totals of lawn & garden pesticide applications, AGRI (lbs) 2

2.  Uncontrolled Emission Factors for Pesticide Active Ingredients, EFA

VOC = 1160 lb/ton pest applied = 0.58 lb/lb pest applied 3

3.  Average VOC Content of Pesticide Inert Ingredient Portion, by Formulation Type, EFI

VOC = 66 % lb/lb pest applied 3

4.  Amount inert portion in relation to active portion: 1.45*AGRI 3

5.  Overall Emission factor in relation to AGRI, EF (lbs/lb pest applied)3

- EF = EFA + 1.45*EFI

- EF = 0.58 + 1.45*0.66

EF = 1.537

6.  Weekly activity, WAF=6 days/week 4

7.  Seasonal adjustment factor, SAF 4

Summer Season Adjustment Factor		1.32

Fall Season Adjustment Factor			0.89

Winter Season Adjustment Factor		0.89

Spring Season Adjustment Factor			0.89

- Process:

The following equations are used to calculate the emissions without control for this source category.

Epa	= AGRI*EF/CF

Eps	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

CF   = Conversion factor for units = 2,000 lbs/ton

Assumptions:

1.  Assume that  the inert portion of the pesticide applied is 1.45 times the active ingredient. 3

2.  Assume that the average VOC content of the pesticide inert ingredient portion, by formulation type represents 66 % which represents the highest % weight of inert portion included in Table 9.4-3 from reference 3.

3. Assume that all pesticides are applied to the surface and have a vapor pressure of >1 x 10-4  as referenced in table 9.4-4 from reference 3.

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:    None

- Double Counting:

Emissions for this source category have not been reported in the point source inventory, therefore no adjustment of the area source emissions was required.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

- References:

1. Procedures for the Preparation of Emission Inventories for Carbon Monoxide and Precursors of Ozone, Vol. I:  General Guidance for Stationary Sources,    May 1991, United States Environmental Protection Agency, Office of Air  Quality Planning and Standards, Research Triangle Park, NC,   EPA-450/4-91-016

2. New Jersey 2001 Lawn Care Pesticide Use Survey the Bureau of Pesticide Operations,  New Jersey Department of Environmental Protection, Pesticide Control Program, West Trenton, NJ

3. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 9 - Pesticides - Agricultural and Nonagricultural,  December 1997, Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

4. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction,  Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c4.

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Pesticide Application, Mosquito Control

- SCC:  2461800000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category:1

1.  Surveying individual areas for pesticide applications

This is the only recommended method and was used in developing the emission inventory.

Emissions from mosquito control pesticide applications are calculated using county level mosquito control pesticide use.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  2001 County totals of mosquito control pesticide applications, AGRI (lbs) 2

2.  Uncontrolled Emission Factors for Pesticide Active Ingredients, EFA

VOC = 1160 lb/ton pest applied = 0.58 lb/lb pest applied 3

3.  Average VOC Content of Pesticide Inert Ingredient Portion, by Formulation Type, EFI

VOC = 66 % lb/lb pest applied 3

4.  Amount inert portion in relation to active portion: 1.45*AGRI 3

5.  Overall Emission factor in relation to AGRI, EF (lbs/lb pest applied)3

- EF = EFA + 1.45*EFI

- EF = 0.58 + 1.45*0.66

EF = 1.537

6.  Weekly activity, WAF=6 days/week 4

7.  Seasonal adjustment factor, SAF 4

Summer Season Adjustment Factor		1.32

Fall Season Adjustment Factor			0.89

Winter Season Adjustment Factor		0.89

Spring Season Adjustment Factor			0.89

- Process:

The following equations are used to calculate the emissions without control for this source category.

Epa	= AGRI*EF/CF

Eps	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

CF   = Conversion factor for units = 2,000 lbs/ton

Assumptions:

1.  Assume that  the inert portion of the pesticide applied is 1.45 times the active ingredient. 3

2.  Assume that the average VOC content of the pesticide inert ingredient portion, by formulation type represents 66 % which represents the highest % weight of inert portion included in Table 9.4-3 from reference 3.

3. Assume that all pesticides are applied to the surface and have a vapor pressure of >1 x 10-4  as referenced in table 9.4-4 from reference 3.

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:    None

- Double Counting:

Emissions for this source category have not been reported in the point source inventory, therefore no adjustment of the area source emissions was required.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

- References:

1. Procedures for the Preparation of Emission Inventories for Carbon Monoxide and Precursors of Ozone, Vol. I:  General Guidance for Stationary Sources,    May 1991, United States Environmental Protection Agency, Office of Air  Quality Planning and Standards, Research Triangle Park, NC,   EPA-450/4-91-016

2. New Jersey 2001 Mosquito Control Pesticide Use Survey the Bureau of Pesticide Operations,  New Jersey Department of Environmental Protection, Pesticide Control Program, West Trenton, NJ

3. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 9 - Pesticides - Agricultural and Nonagricultural,  December 1997, Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

4. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction,  Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c4.

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Pesticide Application, Right-of-Way

- SCC:  2461800000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category:1

1.  Surveying individual areas for pesticide applications

This is the only recommended method and was used in developing the emission inventory.

Emissions from right-of-way pesticide applications are calculated using statewide right-of-way pesticide use, allocated to the county level using population.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  2000 Statewide totals of right-of-way pesticide applications, AGRI (lbs) 2

2.  County and state population, CPOP, SPOP 3

2.  Uncontrolled Emission Factors for Pesticide Active Ingredients, EFA

VOC = 1160 lb/ton pest applied = 0.58 lb/lb pest applied 3

3.  Average VOC Content of Pesticide Inert Ingredient Portion, by Formulation Type, EFI

VOC = 66 % lb/lb pest applied 3

4.  Amount inert portion in relation to active portion: 1.45*AGRI 3

5.  Overall Emission factor in relation to AGRI, EF (lbs/lb pest applied) 4

- EF = EFA + 1.45*EFI

- EF = 0.58 + 1.45*0.66

EF = 1.537

6.  Weekly activity, WAF=6 days/week 4

7.  Seasonal adjustment factor, SAF 4

Summer Season Adjustment Factor		1.32

Fall Season Adjustment Factor			0.89

Winter Season Adjustment Factor		0.89

Spring Season Adjustment Factor			0.89

- Process:

The following equations are used to calculate the emissions without control for this source category.

Epa	= AGRI*EF*(CPOP/SPOP)/CF

Eps	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

CF   = Conversion factor for units = 2,000 lbs/ton

Assumptions:

1.  Assume that  the inert portion of the pesticide applied is 1.45 times the active ingredient. 3

2.  Assume that the average VOC content of the pesticide inert ingredient portion, by formulation type represents 66 % which represents the highest % weight of inert portion included in Table 9.4-3 from reference 3.

3. Assume that all pesticides are applied to the surface and have a vapor pressure of >1 x 10-4  as referenced in table 9.4-4 from reference 3.

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:    None

- Double Counting:

Emissions for this source category have not been reported in the point source inventory, therefore no adjustment of the area source emissions was required.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

- References:

1. Procedures for the Preparation of Emission Inventories for Carbon Monoxide and Precursors of Ozone, Vol. I:  General Guidance for Stationary Sources,    May 1991, United States Environmental Protection Agency, Office of Air  Quality Planning and Standards, Research Triangle Park, NC,   EPA-450/4-91-016

2. New Jersey 2000 Right-of-Way Pesticide Use Survey the Bureau of Pesticide Operations,  New Jersey Department of Environmental Protection, Pesticide Control Program, West Trenton, NJ

3. Estimate of  2002 Total Resident Population by County, NJDOT, July 17, 2003.

4. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 9 - Pesticides - Agricultural and Nonagricultural,  December 1997, Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

5. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction,  Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c4.

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Consumer Products

- SCC:  2465000000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category:1

1.  Surveying individual households

2.  Population based analysis

The population based analysis is selected because the input data elements are more readily available.

Emissions from consumer products are calculated using county level population.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  County population, CPOP 2

2.  Emission factor, EF, (lbs/capita/year)

VOC	  = 7.84 lbs/capita 3

3. Control Efficiency, CE

- STEP ONE: NJ Consumer Product Rule Emission Reduction  Benefit  (lbs/capita/year):

ERNJ =  0.75 lbs/capita/year 4

- STEP TWO:  Additional Emission Reduction Benefit from National rule limits for windshield washer fluids and charcoal lighter fluids, categories which are not in the NJ rule.

- ERNAT = 0.22 lbs/capita/year 5

- STEP THREE:  Determine new  EF after application of NJ and National Consumer Product Rules

EFrule   =  (EF - ERNJ - ERNAT)

=  (7.84 - 0.75 – 0.22)

=   6.87 lbs/capita/year

- STEP FOUR: Calculate CE

CE = ( EFno rule -  EFrule )/( EFno rule)

- CE = ( 7.84 –6.87)/(7.84) * 100 %

CE =    12.4 %

4. Rule Effectiveness, RE;   RE = 80 % 1

5. Rule Penetration, RP;   RP = 100 %

5.  Weekly activity, WAF=7 days/week 6

7.  Seasonal adjustment factor, SAF=1 6

- Process:

The following equations are used to calculate the emissions with control for this source category.

Epa	= CPOP*EF*( 1 - (CE*RE*RP) )/CF

Eps	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

- Rule Effectiveness:

The emissions from this source category are regulated by the following rules:

1.  New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 24, New Jersey StateDepartment of Environmental Protection, November 6, 1995

- 2.  National Volatile Organic Compound Emission Standards for Consumer and Commercial Products, CFR 40, Chapter 1, Subchapter C, Part 59, Subpart C-National Volatile Organic Compound Emission Standards for Consumer Products

- Double Counting:

Emissions for this source category have not been reported in the point source inventory, therefore no adjustment of the area source emissions was required.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

- References:

1. Procedures for the Preparation of Emission Inventories for Carbon Monoxideand Precursors of Ozone, Vol. I:  General Guidance for Stationary Sources, May 1991, United States Environmental Protection Agency, Office of Air  Quality Planning and Standards, Research Triangle Park, NC, EPA-450/4-91-016

2. Estimate of  2002 Total Resident Population by County, NJDOT, July 17, 2003.

3. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 5 - Consumer and Commercial Solvent Use, August 2, 1996, Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

4. State Implementation Plan (SIP) Revision for the Attainment and Maintenance of the Ozone National Ambient Air Quality Standards,  Meeting the Requirements of the Alternative Ozone Attainment Demonstration Policy,  Appendix V: Rate-of-Progress Plan Benefits Calculations, Section XX, New Jersey's Consumer Products Rule, page 24, The State of New Jersey, Department of Environment Protection, December 31, 1996

5. EPA  Report to Congress:  Study of Volatile Organic Compound Emissions From Consumer and Commercial Products, March 1995, Table 2-3.

6. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction,  Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c

Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Portable Fuel Containers

- SCC: 2501000120

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are used for this source category:

1.  CARB methodology and survey modified for the OTC 1, 2

Portable fuel container emission are calculated by accounting for emissions from five different components related to gas container use: permeation, diurnal, transport-spillage, refueling spillage and refueling vapor displacement emissions.  The permeation, diurnal emissions (associated with storage) and transport-spillage (associated with filling the can) emissions are included in the area source inventory.  The equipment refueling spillage and refueling vapor displacement emissions are calculated from the non-road model and are included in the non-road inventory.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  Number of residential housing units 2002 by county, N 3

2.  Number of commercial businesses 2002 expected to have at least one gas can by county, NAICS 111*, 112*, 113*,114*, 115*,  23*, 311119, 326212, 4411*, 447*, 452990, 488410, 5321*, 541320, 541620, 541690, 81111* 812930 4

3.  Weekly activity, WAF=7 days/week

4. Seasonal adjustment factors, SAF = 1

- Process:

The following equations are used to calculate the emissions without control for this source category 1, 2

The residential-gas-can population is calculated as follows:

PopR   =   (N)(A)(CountR)	 (Eq. 1)

where:	PopR 	=	Statewide Residential-Gas-Can Population

N	=	Number of Occupied-Housing Units in OTC State

A	=	Percentage of Households with Gas Cans (46%)

CountR	= 	Average Number of Residential-Gas Cans per Household

Statewide residential-gas-can-permeation emissions are computed as follows:

HCPR  =             (PopR)(S)(EFP)(BR)(SizeR)(Level)/CF	 (Eq. 2)

where:	HCPR 	=	Permeation Emissions in tons per day (tpd)

PopR 	=	Statewide Residential-Gas-Can Population

EFP	=	Appropriate Permeation-Emission Factor (g/gal-day)

S	=	Percentage of Gas Cans Stored with Fuel (70%)

BR	=	Percentage of Cans Stored in Closed Condition with respect to Material (Plastic 53%; Metal 13%)

SizeR	=	Weighted Average Capacity of Residential-Gas Cans (2.34 gal.)

Level	= 	Weighted Average Amount of Stored Fuel (49%)

CF	=	908,105 g/ton

Diurnal emissions from both open- and closed-system-residential-gas cans are calculated as follows:

HCDR  =  (PopR)(S)(EFD)(BR)(SizeR)(Level)	 (Eq. 3)

where:	HCDR 	=	Diurnal Emissions (tpd) for Residential-Gas Cans with respect to Storage Condition (Open or Closed) and Material (Plastic or Metal)

PopR 	=	Statewide Residential-Gas-Can Population

S	=	Percentage of Gas-Can Population Stored with Fuel (70%)

EFD	=	Appropriate Diurnal-Emission Factor with respect to Storage Condition and Material (g/gal-day or g/day)

BR	=	Percentage of Gas-Can Population with respect to Storage Condition and  Material

SizeR	=	Weighted Average Capacity of Residential-Gas Cans (2.34 gal.)

Level	= 	Weighted Average Amount of Stored Fuel (49%)

CF 	=	908,105 g/ton

Residential-transport-spillage emissions are determined as:

HCTR  =  (PopR)(S)(RefillR)(EFT)(BR)	(Eq. 4)

where:	HCTR 	=	Residential-Gas-Can-Transport-Spillage Emissions (tpd)

PopR 	=	Statewide Residential-Gas-Can Population

S	=	Percentage of Gas Cans Stored with Fuel (70%)

RefillR	= 	Average Number of Residential-Gas-Cans-Pump-Refills per Day per Can (refill/day from survey)

EFT	=	Transport-Emission Factor with respect to Storage Condition (g/refill)

BR	=	Percentage of Gas Cans with respect to Storage Condition and Material

CF 	=	908,105 g/ton

The commercial-gas-can population is calculated as follows:

PopC     =     (NC )( CountC)	(Eq. 5)

where:	PopC 	=	Statewide Commercial-Gas-Can Population

NC	=	Number of Occupied Businesses in State

CountC	= 	Average Number of Gas Cans per Business

Statewide commercial-gas-can-permeation emissions are computed as follows:

HCPC  =             (PopC)(S)(EFP)(BC)(SizeC)(Level) 	(Eq. 6)

where:	HCPC 	=	Permeation Emissions (tpd)

PopC 	=	Statewide Commercial-Gas-Can Population

EFP	=	Appropriate Permeation-Emission Factor (g/gal-day)

S	=	Percentage of Gas Cans Stored with Fuel (70% for Residential Survey)

BC	=	Percentage of Applicable Gas Cans Stored in Closed Condition

SizeC	=	Weighted Average Capacity of Commercial-Gas Cans (3.43 gal)

Level	= 	Weighted Average Amount of Stored Fuel (49% from Residential Survey)

CF 	=	908,105 g/ton

The amount of diurnal emissions from both open- and closed-system commercial-gas cans is calculated as follows:

HCDC  =  (PopC)(S)(EFD)(BC)(SizeC)(Level)	 (Eq. 7)

where:	HCDC 	=	Diurnal Emissions (tpd) for Commercial-Gas Cans with respect to Storage Condition (Open or Closed) and Material (Plastic or Metal)

PopC 	=	Statewide Commercial-Gas-Can Population

EFD	=	Appropriate Diurnal-Emission Factor with respect to Storage Condition and Material (g/gal-day or g/day)

S	=	Percentage of Gas Cans Stored with Fuel (70% from Residential Survey)

BC	=	Percentage of Gas Cans with respect to Storage Condition and Material

SizeC	=	Weighted Average Capacity of Commercial-Gas Cans (3.43 gal.)

Level	= 	Weighted Average Amount of Stored Fuel (49% from Residential Survey)

CF 	=	908,105 g/ton

The non-lawn-and-garden-equipment commercial-gas-can refills at the pump is derived as follows:

RefillC     =                ( Fuel)        		                     (Eq. 8)

(SizeC)(POPNON)(S)

where:	RefillC	= 	Average Number of Non-Lawn-and-Garden Equipment Commercial-Gas-Cans Pump Refills per Day per Can (refill/day)

Fuel	=		Non-Lawn-and-Garden Equipment Fuel Consumption (gal/day) for 2000

SizeC	=	Weighted Average Capacity of Commercial-Gas Cans (3.43 gal/can-refill)

POPNON 	=	Statewide Commercial-Gas-Can Population with respect to Non-Lawn-and-Garden Businesses

S	=	Percentage of Gas Cans Stored with Fuel (70% from Residential Survey)

CF 	=	908,105 g/ton

The commercial-transport-spillage emissions are determined as:

HCTC  =  (PopC)(S)(BC)(RefillC)(EFTC)			          (Eq. 9)

where:	HCTC 	=	Commercial-Gas-Can-Transport-Spillage Emissions (tpd)

PopC 	=	Statewide Commercial-Gas-Can Population

S	=	Percentage of Gas Cans Stored with Fuel (70% from Residential Survey)

BC	=	Percentage of Gas Cans with respect to Storage Condition and Material

RefillC 	= 	Average Number of Gas-Cans Pump Refills per

Day per Can

EFTC	=	Transport-Spillage Emission Factor (g/refill) with respect to Storage Condition

CF 	=	908,105 g/ton

The total area source portable fuel container emissions are summed as follows:

Epa	= HCPR+HCDR+HCTR+HCPC+HCDC+HCTC

Eps	= Epa*SAF//ADDF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

- Rule Effectiveness:

The emissions from this source category are regulated by the following rule:  None

- Double Counting:

Emissions for this source category have not been reported in the point source inventory, therefore no adjustment of the area source emissions was required.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

- References:

- 1.  ARB’s Mailout MSC 99-25, “Public Meeting to Consider Approval of CA’s Portable Gasoline-Container Emissions Inventory,” (ARB, 1999b)

- 2.  Control Measure Development Support Analysis of Ozone Transport Commission Model Rules, E.H. Pechan & Associates, Inc., 5528-B Hempstead Way, Springfield, VA 22151, March 31, 2001.

- 3.  Residential Housing Units 2002, US Census Bureau

- 4. Total 2002 employment and business establishments by 6 digit NAICS code and by county, New Jersey  Department of Labor.

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Gasoline Handling & Marketing, Tank Truck Unloading Loss (Balanced Submerged Filling or Stage I)

- SCC:  2501060050

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category:1

1.  Surveying individual gasoline service stations

2.  Gasoline throughput analysis

The gasoline throughput analysis is selected because the input data elements are more readily available.

Emissions from gasoline tank truck unloading are calculated using statewide gasoline consumption, allocated to the county level using vehicle miles travelled.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  NJ gasoline consumption, GAS (103 gallons) 2

2.  State and County Daily Vehicle Miles Travelled, SVMT, CVMT 3

3.  Emission factor, EF, (lbs/103 gallons) 4

- VOC=7.6

3. Control Efficiencies, CE = 90% 5

4. Rule Effectiveness, RE = 80 % 1

5. Rule Penetration, RP = 100 %

4.  Weekly activity, WAF=6 days/week 6

5.  Seasonal adjustment factor, SAF=1

- Process:

The following equations are used to calculate the emissions with control for this source category.

Epa	= GAS*EF*(CVMT/SVMT) *(1- (CE*RE*RP)/CF

Eps	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1.  Assume seasonal adjustment factor of 1

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:

New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 16, N.J.A.C. 7:27-16.3, New Jersey State Department of Environmental Protection, May 15, 2000

- Double Counting:

Emissions for this source category have not been reported in the point source inventory, therefore no adjustment of the area source emissions was required.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

- References:

1. Procedures for the Preparation of Emission Inventories for Carbon Monoxideand Precursors of Ozone, Vol. I:  General Guidance for Stationary Sources, May 1991, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-450/4-91-016

2. State Energy Data 2000 Consumption Tables (formerly the State Energy Data Report), United States Department of Energy, Energy Information Administration, Office of Energy Markets and End Use, Washington, DC

3. New Jersey Department of Environmental Protection 2002 Mobile Inventory

4. CARB Uncontrolled Vapor Emission Factor at Gasoline Dispensing Facilities, January 5, 2000

5. New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 16, N.J.A.C. 7:27-16.3, New Jersey State Department of Environmental Protection, May 15, 2000

6. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction,  Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c4.

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Gasoline Handling & Marketing, Tank Breathing

- SCC:  2501060201

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category: 1

1.  Surveying individual gasoline service stations

2.  Gasoline throughput analysis

The gasoline throughput analysis is selected because the input data elements are more readily available.

Emissions from gasoline tank breathing are calculated using statewide gasoline consumption, allocated to the county level using vehicle miles travelled.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  NJ gasoline consumption, GAS (103 gallons) 2

2.  State and County Vehicle Miles Travelled, SVMT, CVMT 3

3.  Emission factor, EF, (lbs/103 gallons) 4

- VOC=1.0

4.  Weekly activity, WAF=7 days/week 5

5.  Seasonal adjustment factor, SAF=1 5

- Process:

The following equations are used to calculate the emissions without control for this source category.

Epa	= GAS*EF*(CVMT/SVMT)/CF

Eps	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1.  Assume seasonal adjustment factor of 1.

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:

New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 16, N.J.A.C. 7:27-16, New Jersey State Department of Environmental Protection, May 15, 2000

- Double Counting:

Emissions for this source category have not been reported in the point source inventory, therefore no adjustment of the area source emissions was required.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

- References:

1. Procedures for the Preparation of Emission Inventories for Carbon Monoxideand Precursors of Ozone, Vol. I:  General Guidance for Stationary Sources, May 1991, United States Environmental Protection Agency, Office of Air  Quality Planning and Standards, Research Triangle Park, NC,    EPA-450/4-91-016

2. State Energy Data 2000 Consumption Tables (formerly the State Energy Data Report), United States Department of Energy, Energy Information Administration, Office of Energy Markets and End Use, Washington, DC

3. New Jersey Department of Environmental Protection 2002 Mobile Inventory

4. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 11 – Gasoline Marketing (Stage I and Stage II),  Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c

5. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction,  Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c

Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Gasoline Handling & Marketing, Transport by Rail Tank Car and Truck

- SCC:  2505030120

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category: 1

1.  Surveying the transport industry

2.  Gasoline throughput analysis

The gasoline throughput analysis is selected because the input data elements are more readily available.

Emissions from gasoline tank truck and rail car transport are calculated using statewide gasoline consumption, allocated to the county level using vehicle miles travelled.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  New Jersey gasoline consumption, GAS (103 gallons) 2

2.  State and County Daily Vehicle Miles Travelled, SVMT, CVMT 3

3.  A gasoline transportation adjustment factor (GTAF) of 1.25 is applied to account for gasoline resale 4

4.  Emission factor, EF, (lbs/103 gallons) 4

- VOC=0.06

5.Control Efficiencies, CE

CEprocedures   = 90.0 % 5

6.Rule Effectiveness, RE;   RE = 80 % 1

7.Rule Penetration, RP = 100 %

8.  Weekly activity, WAF=6 days/week 6

9.  Seasonal adjustment factor, SAF=1

- Process:

The following equations are used to calculate the emissions with control for this source category.

Epa	= 1.25*GAS*(CVMT/SVMT)*EF*(1- (CE*RE*RP) )/CF

Eps	= Epa/AADF*SAF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

2.  Railcar and truck transport were incorporated into one category because NJDEP was unable to distingusih the amount of gasoline carried by each.

3.  All gasoline transported to individual facilities are either taken by  truck or railcar.

4.  Assume a seasonal adjustment factor of 1

5.  Assume a rule penetration factor of  1

- Rule Effectiveness:

The emissions from this source category are regulated by the following rule:

New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 16, N.J.A.C. 7:27-16, New Jersey State Department of Environmental Protection, May 15, 2000

- Double Counting:

Emissions for this source category have not been reported in the point source inventory, therefore no adjustment of the area source emissions was required.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

- References:

1. Procedures for the Preparation of Emission Inventories for Carbon Monoxideand Precursors of Ozone, Vol. I:  General Guidance for Stationary Sources,  May 1991, United States Environmental Protection Agency, Office of Air   Quality Planning and Standards, Research Triangle Park, NC, EPA-450/4-91-016

2. State Energy Data 2000 Consumption Tables (formerly the State Energy Data Report), United States Department of Energy, Energy Information Administration, Office of Energy Markets and End Use, Washington, DC

3. New Jersey Department of Environmental Protection 2002 Mobile Inventory

4. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 11 – Gasoline Marketing (Stage I and Stage II),  Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c

5.  New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 16, N.J.A.C. 7:27-16, New Jersey State Department of Environmental Protection, May 15, 2000

6. EIIP Volume III, Chapter 1, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction,  Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Petroleum Transport, Marine Vessel-Crude Oil Loading

- SCC:  2505020030

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category: 1

1.  Surveying individual shipping companies

2.  Fuel transported analysis

The fuel transported analysis is selected because the input data elements are more readily available.

Emissions from crude oil loading onto marine vessels are calculated using tons of crude oil loaded at each port, allocated to the county level by the total fuel storage capacity of each docking facility.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  Crude oil exported from each port (see item 3 below in assumption section), [FUEL in 103 tons] 2

2.  Number of ships and barges at each port, [S, B]2

3.  Fuel oil storage capacity of all docks within municipality within port, [MUNSTOR in barrels] 3,4

4.  Total port storage capacity of fuel oil, [TOTSTOR in barrels] 3,4

5.  Emission factor, [EFS in lbs/103 gallons transported by ships] 5

[EFB in lbs/103 gallons transported by barges] 5

[PER = portion of EFS and EFB represents VOC emissions] 6

- VOCS=EFS*PER. = 0.61*0.85 = 0.5185

- VOCB=EFB*PER =  1.00*0.85 = 0.85

6.  Weekly activity, WAF=7 days/week

7.  Seasonal adjustment factor, SAF 6a

Summer Season Adjustment Factor		1.2

Fall Season Adjustment Factor			0.93

Winter Season Adjustment Factor		0.93

Spring Season Adjustment Factor			0.93

- Process:

The following equations are used to calculate the emissions without control  for this source category.

Epa = (FUEL*CF/CF1)*(MUNSTOR/TOTSTOR)*((B/(B+S))*EFB*PER + (S/(S+B))*EFS*PER)/CF

Eps	= Epa/AADF*SAF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

CF1    = Conversion factor for units =[7.1 lb/gal] 7

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1.  All crude oil exported from a port is loaded at that port.

2.  Barges are defined as non-self propelled vessels.

3. Foreign and Canadian exports, coastwise and lakewise shipments, internal shipments, internal outbound (Upbnd & Downbnd) internal intra (Upbnd & Downbnd), internal intraport and intra-territory shipments from port freight traffic sheets  included in reference 2 constitutes marine fuel loading  or export activity. 8

4. Crude oil marine vessel loading includes crude petroleum loading activities from port freight traffic sheets  included in reference 2. 9

5.  Assume that the number of ships (S) and barges (B) specified for all ports from trips and drafts of vessels  included in reference 2 includes both upbound and downbound shipping except as noted in item 10 below.

6.  Assume that  ship/ocean going vessel contribution to overall waterborne commerce (ships and barges) emissions  is correctly proportioned  by multiplication of  the ship/ocean going vessel  emission factor  (EFS) with the ratio of the  total number of ships divided by the total number of ships and barges, ie (Ships/(Ships+Barges))*EFS.  Similarly, the barge contribution to overall waterborne commerce is correctly proportioned by multiplication of the barge emission factor (EFB) with the ratio of the total number of  barges divided by the total number of ships and barges, ie (Ships/(Ships+Barges))*EFB.

7.  Assume that the municipal fuel oil storage capacity of all docks within a municipality within a port (MUNSTOR) divided by the total fuel oil storage capacity within the port (TOTSTOR) allocates port emissions from fuel oil activities to a municipal basis.

8. % emissions to NJ from shipping activity to Ports shared by NJ or New York are allocated as follows for each of these respective ports10 :

Upper Bay NY Harbor, NY & NJ                   :  54.3 % port emissions to NJ

Hudson River Channel, NY & NJ                   :  50.0 % port emissions to NJ

New York & New Jersey Channel,NY & NJ  : 52.6 % port emissions to NJ

9.  % emissions to NJ from shipping activity to Ports shared by Pennsylannia and New Jersey, ie Delaware River (Phila to sea), Delaware River (Phila to Trenton), Camden and Trenton is allocated by crediting all northbound (upbound or inbound) ships(S) and barges(B) to NJ.11  Assume this emission allocation is accomplished by including only upbound or inbound shipping in the top portion of the emission equation related to S & B loading but including both upbound and downbound shipping in its bottom portion for every port shared by Pennsylannia and New Jersey, ie upbd S/(downbd S & upbd S + downbd B & upbd B) or upbd B/(downbd S & upbd S + downbd B & upbd B).

11. Fuel and shipping (S & B) amounts for the Port of Camden are subtracted from those amounts obtained for the Port of Delaware River (Phila to the sea)   because this port includes Camden and loading data for Camden has been obtained separately without allocation from this larger port.

12.  Assume a weekly activity factor of 7 days per week.

13.  Assume a seasonal adjustment factor of 1.

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:    None

- Double Counting:

Emissions for this source category may have been reported in the point source inventory, however, based on the descriptions given by the point source facilities it cannot be determined.  Therefore, to be conservative, no adjustment of the area source emissions was conducted.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

- References:

1. Procedures for the Preparation of Emission Inventories for Carbon Monoxide and Precursors of Ozone, Vol. I:  General Guidance for Stationary Sources,  United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-450/4-91-016, May 1991

2. Waterborne Commerce of the United States, Part 1, Waterways and Harbors Atlantic Coast, Calendar Year 2001, US Army Corp of Engineers, Water  Resources Support Center.

3. The Ports of New York, NY and NJ and Ports on Long Island, NY, Port Series No. 5,  US Army Corps of Engineers, Revised 1988

4. The Ports of Philadelphia, Pa, Camden, NJ, Wilmington, De, and Ports on Delaware River, Port Series No. 8, US Army Corps of Engineers, Revised 1984

5. EIIP Volume III, Chapter 12 - Marine Vessel Loading, Ballasting, and Transit, Table 12.4-5, Uncontrolled VOC Emission Factors for Petroleum Carrying Marine Vessels , Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

6. Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, Section 5.2-14, Table 5.2-6. Total Organic Emission Factors for Petroleum Marine Vessel Sources, Footnote c, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

6a.  Based on monthly – temperature adjusted true vapor pressures obtained from USEPA Tanks 4.09b model for Newark, New Jersey.

7. Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, Section 7.1-83, Table 7.1-2. Properties (MV,WVC, PVA,WL) of Selected Petroleum Liquids

8. EIIP Volume III, Chapter 12 - Marine Vessel Loading, Ballasting, and Transit, Table 12.4-1, Emission Points for Petroleum Vessel Traffic Classifications

9. EIIP Volume III, Chapter 12 - Marine Vessel Loading, Ballasting, and Transit Table 12.4-1, Product Type

Classifications for Common Petroleum Vessel Commodities

10. Fax from Julie Tucker of Radian Corporation, Spatial Allocation of Waterways Shared by New York and New Jersey, November 13, 1992

11. Memorandum from Whitney Gadsby of the State of Delaware, Department of Natural Resources & Environmental Control, Division of Air & Waste Management, October 9, 1992

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Petroleum Transport, Marine Vessel-Residual Oil Loading

- SCC:  2505020060

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category: 1

1.  Surveying individual shipping companies

2.  Fuel transported analysis

The fuel transported analysis is selected because the input data elements are more readily available.

Emissions from residual oil loading onto marine vessels are calculated using tons of residual oil loaded at each port, allocated to the county level by the total fuel storage capacity of each docking facility.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  Residual oil exported from each port (see item 3 below in assumption section), [FUEL in 103 tons] 2

2.  Number of ships and barges at each port,[S, B] 2

3.  Fuel oil storage capacity of all docks within municipality within port, [MUNSTOR in barrels]3,4

4.  Total port storage capacity of fuel oil, [TOTSTOR in barrels] 3,4

5.  Emission factor,	[EFS in lbs/103 gallons transported by ships] 5

[EFB in lbs/103 gallons transported by barges] 5

- VOCS=0.00004

- VOCB=0.00009

6.  Weekly activity, WAF=7 days/week

7.  Seasonal adjustment factor, SAF 5a

Summer Season Adjustment Factor		1.41

Fall Season Adjustment Factor			0.86

Winter Season Adjustment Factor		0.86

Spring Season Adjustment Factor			0.86

- Process:

The following equations are used to calculate the emissions without control  for this source category.

Epa= (FUEL*CF/CF1)*(MUNSTOR/TOTSTOR)*((B/(B+S))*EFB + (S/(S+B))*EFS)/CF

Eps= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

CF1  	= Conversion factor for units =[ 7.9 lb/gal] 6

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1.  All residual oil exported from a port is loaded at that port.

2.  Barges are defined as non-self propelled vessels.

3. Foreign and Canadian exports, coastwise and lakewise shipments, internal shipments, internal outbound (Upbnd & Downbnd) internal intra (Upbnd & Downbnd), internal intraport and intra-territory shipments from port freight traffic sheets  included in reference 2 constitutes marine fuel loading or export activity. 7

4. Residual oil marine vessel loading includes residual fuel oil, asphalt, tar, and pitch, and petroleum coke loading activities from port freight traffic sheets  included in reference 2. 8

5.  Assume that the number of ships (S) and barges (B) specified for all ports from trips and drafts of vessels  included in reference 2 includes both upbound and downbound shipping except as noted in item 10 below.

6.  Assume that  ship/ocean going vessel contribution to overall waterborne commerce (ships and barges) emissions  is correctly proportioned  by multiplication of  the ship/ocean going vessel  emission factor  (EFS) with the ratio of the  total number of ships divided by the total number of ships and barges, ie (Ships/(Ships+Barges))*EFS.  Similarly, the barge contribution to overall waterborne commerce is correctly proportioned by multiplication of the barge emission factor (EFB) with the ratio of the total number of  barges divided by the total number of ships and barges, ie (Ships/(Ships+Barges))*EFB.

7.  Assume that the municipal fuel oil storage capacity of all docks within a municipality within a port (MUNSTOR) divided by the total fuel oil storage capacity within the port (TOTSTOR) allocates port emissions from fuel oil activities to a municipal basis.

8. % emissions to NJ from shipping activity to Ports shared by NJ or New York are allocated as follows for each of these respective ports:9

Upper Bay NY Harbor, NY & NJ                   :  54.3 % port emissions to NJ

Hudson River Channel, NY & NJ                   :  50.0 % port emissions to NJ

New York & New Jersey Channel,NY & NJ  :  52.6 % port emissions to NJ

9.  % emissions to NJ from shipping activity to Ports shared by Pennsylannia and New Jersey, ie Delaware River (Phila to sea), Delaware River (Phila to Trenton), Camden and Trenton is allocated by crediting all northbound (upbound or inbound) ships(S) and barges(B) to NJ.10  Assume this emission allocation is accomplished by including only upbound or inbound shipping in the top portion of the emission equation related to S & B loading but including both upbound and downbound shipping in its bottom portion for every port shared by Pennsylannia and New Jersey, ie upbd S/(downbd S & upbd S + downbd B & upbd B) or upbd B/(downbd S & upbd S + downbd B & upbd B).

10. Fuel and shipping (S & B) amounts for the Port of Camden are subtracted from those amounts obtained for the Port of Delaware River (Phila to the sea)   because this port includes Camden and loading data for Camden has been obtained separately without allocation from this larger port.

11.  Assume a weekly activity factor of 7 days per week.

12.  Assume a seasonal adjustment factor of 1.

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:        None

- Double Counting:

Emissions for this source category may have been reported in the point source inventory, however, based on the descriptions given by the point source facilities it cannot be determined.  Therefore, to be conservative, no adjustment of the area source emissions was conducted.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

- References:

1. Procedures for the Preparation of Emission Inventories for Carbon Monoxide and Precursors of Ozone, Vol. I:  General Guidance for Stationary Sources,  United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-450/4-91-016, May 1991

2. Waterborne Commerce of the United States, Part 1, Waterways and Harbors Atlantic Coast, Calendar Year 2001, US Army Corp of Engineers, Water  Resources Support Center.

3. The Ports of New York, NY and NJ and Ports on Long Island, NY, Port Series No. 5,  US Army Corps of Engineers, Revised 1988

4. The Ports of Philadelphia, Pa, Camden, NJ, Wilmington, De, and Ports on Delaware River, Port Series No. 8,  US Army Corps of Engineers, Revised 1984

5. EIIP Volume III, Chapter 12 - Marine Vessel Loading, Ballasting, and Transit, Table 12.4-5, Uncontrolled VOC Emission Factors for Petroleum Carrying Marine Vessels , Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

5a.  Based on monthly – temperature adjusted true vapor pressures obtained from USEPA Tanks 4.09b model for Newark, New Jersey.

6. Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, Section 7.1-83, Table 7.1-2. Properties (MV,WVC, PVA,WL) of Selected Petroleum Liquids,

7. EIIP Volume III, Chapter 12 - Marine Vessel Loading, Ballasting, and Transit, Table 12.4-1, Emission Points for Petroleum Vessel Traffic Classifications

8. EIIP Volume III, Chapter 12 - Marine Vessel Loading, Ballasting, and Transit Table 12.4-1, Product Type Classifications for Common Petroleum Vessel Commodities

9. Fax from Julie Tucker of Radian Corporation, Spatial Allocation of Waterways Shared by New York and New Jersey, November 13, 1992

10.  Memorandum from Whitney Gadsby of the State of Delaware, Department of Natural Resources & Environmental Control, Division of Air & Waste Management, October 9, 1992

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Petroleum Transport, Marine Vessel-Distillate Oil Loading

- SCC:  2505020090

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category: 1

1.  Surveying individual shipping companies

2.  Fuel transported analysis

The fuel transported analysis is selected because the input data elements are more readily available.

Emissions from distillate oil loading onto marine vessels are calculated using tons of distillate oil loaded at each port, allocated to the county level by the total fuel storage capacity of each docking facility.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  Distillate oil exported from each port (see item 3 below in assumption section), [FUEL in 103 tons] 2

2.  Number of ships and barges at each port, [S, B] 2

3.  Fuel oil storage capacity of all docks within municipality within port,[ MUNSTOR in barrels] 3,4

4.  Total port storage capacity of fuel oil (barrels), [TOTSTOR in barrels] 3,4

5.  Emission factor,	[EFS in lbs/103 gallons transported by ships] 5

[EFB in lbs/103 gallons transported by barges] 5

- VOCS=0.005

- VOCB=0.012

6.  Weekly activity, WAF=7 days/week

7.  Seasonal adjustment factor, SAF 5a

Summer Season Adjustment Factor		1.34

Fall Season Adjustment Factor			0.89

Winter Season Adjustment Factor		0.89

Spring Season Adjustment Factor			0.89

- Process:

The following equations are used to calculate the emissions without control for this source category.

Epa= (FUEL*CF/CF1)*(MUNSTOR/TOTSTOR)*((B/(B+S))*EFB + (S/(S+B))*EFS)/CF

Eps	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

CF1  	= Conversion factor for units = [7.1 lb/gal]6

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1.  All distillate oil exported from a port is loaded at that port.

2.  Barges are defined as non-self propelled vessels.

3. Foreign and Canadian exports, coastwise and lakewise shipments, internal shipments, internal outbound (Upbnd & Downbnd) internal intra (Upbnd & Downbnd), internal intraport and intra-territory shipments from port freight traffic sheets included in reference 2 constitutes marine fuel loading or export activity. 7

4. Distillate oil marine vessel loading includes distillate fuel oil, lube oil and greases, and petro, jelly and waxes  loading activities from port freight traffic sheets included in reference 2. 8

5.  Assume that the number of ships (S) and barges (B) specified for all ports from trips and drafts of vessels  included in reference 2 includes both upbound and downbound shipping except as noted in item 10 below.

6.  Assume that  ship/ocean going vessel contribution to overall waterborne commerce (ships and barges) emissions  is correctly proportioned  by multiplication of  the ship/ocean going vessel  emission factor  (EFS) with the ratio of the  total number of ships divided by the total number of ships and barges, ie (Ships/(Ships+Barges))*EFS.  Similarly, the barge contribution to overall waterborne commerce is correctly proportioned by multiplication of the barge emission factor (EFB) with the ratio of the total number of  barges divided by the total number of ships and barges, ie (Ships/(Ships+Barges))*EFB.

7.  Assume that the municipal fuel oil storage capacity of all docks within a municipality within a port (MUNSTOR) divided by the total fuel oil storage capacity within the port (TOTSTOR) allocates port emissions from fuel oil activities to a municipal basis.

8. % emissions to NJ from shipping activity to Ports shared by NJ or New York are allocated as follows for each of these respective ports:9

Upper Bay NY Harbor, NY & NJ                   :  54.3 % port emissions to NJ

Hudson River Channel, NY & NJ                   :  50.0 % port emissions to NJ

New York & New Jersey Channel,NY & NJ  :  52.6 % port emissions to NJ

9.  % emissions to NJ from shipping activity to Ports shared by Pennsylannia and New Jersey, ie Delaware River (Phila to sea), Delaware River (Phila to Trenton), Camden and Trenton is allocated by crediting all northbound (upbound or inbound) ships(S) and barges(B) to NJ10.   Assume this emission allocation is accomplished by including only upbound or inbound shipping in the top portion of the emission equation related to S & B loading but including both upbound and downbound shipping in its bottom portion for every port shared by Pennsylannia and New Jersey, ie upbd S/(downbd S & upbd S + downbd B & upbd B) or upbd B/(downbd S & upbd S + downbd B & upbd B).

10.	Fuel and shipping (S & B) amounts for the Port of Camden are subtracted from those amounts obtained for the Port of Delaware River (Phila to the sea)   because this port includes Camden and loading data for Camden has been obtained separately without allocation from this larger port.

11. Assume that all point source marine vessel loading operations included in the emission statement program constitutes gas loading operations where control is indicated.  While all point source marine vessel loading operations without controls  represents distillate fuel loading unless otherwise noted in the emission statement record.

12.  Assume a weekly activity factor of  7 days per week.

13.  Assume a seasonal adjustment factor of 1.

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:    None

- Double Counting:

Emissions for this source category may have been reported in the point source inventory, however, based on the descriptions given by the point source facilities it cannot be determined.  Therefore, to be conservative, no adjustment of the area source emissions was conducted.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

- References:

1. Procedures for the Preparation of Emission Inventories for Carbon Monoxide and Precursors of Ozone, Vol. I:  General Guidance for Stationary Sources,  United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-450/4-91-016, May 1991

2. Waterborne Commerce of the United States, Part 1, Waterways and Harbors Atlantic Coast, Calendar Year 2001, US Army Corp of Engineers, Water  Resources Support Center.

3. The Ports of New York, NY and NJ and Ports on Long Island, NY, Port Series No. 5,  US Army Corps of Engineers, Revised 1988

4. The Ports of Philadelphia, Pa, Camden, NJ, Wilmington, De, and Ports on Delaware River, Port Series No. 8,

US Army Corps of Engineers, Revised 1984

5. EIIP Volume III, Chapter 12 - Marine Vessel Loading, Ballasting, and Transit, Table 12.4-5, Uncontrolled VOC Emission Factors for Petroleum Carrying Marine Vessels , Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

5a.  Based on monthly – temperature adjusted true vapor pressures obtained from USEPA Tanks 4.09b model for Newark, New Jersey.

6. Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, Section 7.1-83, Table 7.1-2. Properties (MV,WVC, PVA,WL) of Selected Petroleum Liquids,  United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC, ,

7. EIIP Volume III, Chapter 12 - Marine Vessel Loading, Ballasting, and Transit, Table 12.4-1, Emission Points for Petroleum Vessel Traffic Classifications

8. EIIP Volume III, Chapter 12 - Marine Vessel Loading, Ballasting, and Transit Table 12.4-1, Product Type Classifications for Common Petroleum Vessel Commodities

9. Fax from Julie Tucker of Radian Corporation, Spatial Allocation of Waterways Shared by New York and New Jersey, November 13, 1992

10. Memorandum from Whitney Gadsby of the State of Delaware, Department of Natural Resources & Environmental Control, Division of Air & Waste Management, October 9, 1992

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Petroleum Transport, Marine Vessel-Gasoline Loading

- SCC:  2505020120

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category: 1

1.  Surveying individual shipping companies

2.  Fuel transported analysis

The fuel transported analysis is selected because the input data elements are more readily available.

Emissions from gasoline oil loading onto marine vessels are calculated using tons of gasoline oil loaded at each port, allocated to the county level by the total fuel storage capacity of each docking facility.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  Gasoline exported from each port (see item 3 below in assumption section), [FUEL  in 103 tons] 2

2.  Number of ships and barges at each port,[S, B] 2

3.  Fuel oil storage capacity of all docks within a specific municipality

within the designated port, [MUNSTOR in barrels] 3,4

4.  Total port storage capacity of fuel oil, [TOTSTOR in barrels] 3,4

5.  Emission factor, 	[EFS in lbs/103 gallons transported by ships] 5

[EFB in lbs/103 gallons transported by barges] 5

- VOCS=1.8

- VOCB=3.4

6. Control Efficiencies, CE 6   = 95.0 %

7. Rule Effectiveness, RE 3 = 80 %

8. Rule Penetration, RP = 100 %

9. Weekly activity, WAF=7 days/week

10. Seasonal adjustment factor, SAF 6a

Summer Season Adjustment Factor		0.82

Fall Season Adjustment Factor			1.06

Winter Season Adjustment Factor		1.06

Spring Season Adjustment Factor			1.06

- Process:

The following equations are used to calculate the emissions with control for this source category.

Epa= (FUEL*CF/CF1)*(MUNSTOR/TOTSTOR)*((B/(B+S))*EFB+(S/(S+B))*EFS)*( 1 - (RP*RE*CE) )/CF

Eps= Epa/AADF*SAF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

CF1  	= Conversion factor for units = [5.6 lb/gal]7

AADF = Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1.  All gasoline exported from a port is loaded at that port.

2.  Barges are defined as non-self propelled vessels included in reference 2.

3. Foreign and Canadian exports, coastwise and lakewise shipments, internal shipments, internal outbound (Upbnd & Downbnd) internal intra (Upbnd & Downbnd), internal intraport and intra-territory shipments from port freight traffic sheets included in reference 2 constitutes marine fuel loading or exporting  activity. 8

4. Gasoline marine vessel loading includes gasoline and liquid nautral gas loading activities from port freight traffic sheets included in reference 2. 9

5.  Assume that the number of ships (S) and barges (B) specified for all ports from trips and drafts of vessels  included in reference 2 includes both upbound and downbound shipping except as noted in item 10 below.

6.  Assume that  ship/ocean going vessel contribution to overall waterborne commerce (ships and barges) emissions  is correctly proportioned  by multiplication of  the ship/ocean going vessel  emission factor  (EFS) with the ratio of the  total number of ships divided by the total number of ships and barges, ie (Ships/(Ships+Barges))*EFS.  Similarly, the barge contribution to overall waterborne commerce is correctly proportioned by multiplication of the barge emission factor (EFB) with the ratio of the total number of  barges divided by the total number of ships and barges, ie (Ships/(Ships+Barges))*EFB.

7.  Assume that the municipal fuel oil storage capacity of all docks within a municipality within a port (MUNSTOR) divided by the total fuel oil storage capacity within the port (TOTSTOR) allocates port emissions from fuel oil activities to a municipal basis.

8. % emissions to NJ from shipping activity to Ports shared by NJ or New York are allocated as follows for each of these respective ports: 10

Upper Bay NY Harbor, NY & NJ                   :  54.3 % port emissions to NJ

Hudson River Channel, NY & NJ                   :  50.0 % port emissions to NJ

New York & New Jersey Channel,NY & NJ  :  52.6 % port emissions to NJ

9.  % emissions to NJ from shipping activity to Ports shared by Pennsylannia and New Jersey, ie Delaware River (Phila to sea), Delaware River (Phila to Trenton), Camden and Trenton is allocated by crediting all northbound (upbound or inbound) ships(S) and barges(B) to NJ11.   Assume this emission allocation is accomplished by including only upbound or inbound shipping in the top portion of the emission equation related to S & B loading but including both upbound and downbound shipping in its bottom portion for every port shared by Pennsylannia and New Jersey, ie upbd S/(downbd S & upbd S + downbd B & upbd B) or upbd B/(downbd S & upbd S + downbd B & upbd B).

10. Fuel and shipping (S & B) amounts for the Port of Camden are subtracted from those amounts obtained for the Port of Delaware River (Phila to the sea)   because this port includes Camden and loading data for Camden has been obtained separately without allocation from this larger port.

11.  Assume that every marine vessel gasoline loading and ballasting activity  installs and operates a control apparatus in accordance with N.J.A.C. 7:27-16.5, Marine tank vessel loading and ballasting operations.

12. Assume that all point source marine vessel fuel transfer operations included in the emission statement program constitutes gas loading operations where control is indicated.  While all point source marine vessel fuel transfer operations without controls  represents distillate fuel loading unless otherwise noted in the emission statement record.

13.  Assume a rule penetration factor of  100 %.

14.  Assume a weekly activity factor of  7 days per week.

15.  Assume a seasonal adjustment factor of 1.

- Rule Effectiveness:

The emissions from this source category are regulated by the following rule:

- New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 16, 7:27-16.5, Marine tank

vessel loading and ballasting operations, June 20, 1994

- Double Counting:

Emissions for this source category may have been reported in the point source inventory, however, based on the descriptions given by the point source facilities it cannot be determined.  Therefore, to be conservative, no adjustment of the area source emissions was conducted.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

- References:

1. Procedures for the Preparation of Emission Inventories for Carbon Monoxide and Precursors of Ozone, Vol. I:  General Guidance for Stationary Sources,  United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-450/4-91-016, May 1991

2. Waterborne Commerce of the United States, Part 1, Waterways and Harbors Atlantic Coast, Calendar Year 2001, US Army Corp of Engineers, Water  Resources Support Center

3. The Ports of New York, NY and NJ and Ports on Long Island, NY, Port Series No. 5,  US Army Corps of Engineers, Revised 1988

4. The Ports of Philadelphia, Pa, Camden, NJ, Wilmington, De, and Ports on Delaware River, Port Series No. 8,

US Army Corps of Engineers, Revised 1984

5. EIIP Volume III, Chapter 12 - Marine Vessel Loading, Ballasting, and Transit, Table 12.4-5, Uncontrolled VOC Emission Factors for Petroleum Carrying Marine Vessels , Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

6. State Implementation Plan (SIP) Revision for the Attainment and Maintenance of the Ozone National Ambient Air Quality Standards, Meeting the Requirements of the Alternative Ozone Attainment Demonstration Policy, Appendix V: Rate-of-Progress Plan Benefits Calculations, Section VIII. Marine Barge and Tanker Loading, The State of New Jersey, Department of Environmental Protection, December 31, 1996

6a.  Based on monthly – temperature adjusted true vapor pressures obtained from USEPA Tanks 4.09b model for Newark, New Jersey.

7. Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, Section 7.1-83, Table 7.1-2. Properties (MV,WVC, PVA,WL) of Selected Petroleum Liquids,  United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

8. EIIP Volume III, Chapter 12 - Marine Vessel Loading, Ballasting, and Transit, Table 12.4-1, Emission Points for Petroleum Vessel Traffic Classifications

9. EIIP Volume III, Chapter 12 - Marine Vessel Loading, Ballasting, and Transit Table 12.4-1, Product Type Classifications for Common Petroleum Vessel Commodities

10. Fax from Julie Tucker of Radian Corporation, Spatial Allocation of Waterways Shared by New York and New Jersey, November 13, 1992

11. Memorandum from Whitney Gadsby of the State of Delaware, Department of Natural Resources & Environmental Control, Division of Air & Waste Management, October 9, 1992

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Petroleum Transport, Marine Vessel-Jet Naphtha Loading

- SCC:  2505020150

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category: 1

1.  Surveying individual shipping companies

2.  Fuel transported analysis

The fuel transported analysis is selected because the input data elements are more readily available.

Emissions from jet naphtha loading onto marine vessels are calculated using tons of jet naphtha loaded at each port, allocated to the county level by the total fuel storage capacity of each docking facility.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  Jet naphtha exported from each port (see item 3 below in assumption section), [FUEL in 103 tons] 2

2.  Number of ships and barges at each port, [S, B] 2

3.  Fuel oil storage capacity of all docks within municipality within port, [MUNSTOR in barrels] 3,4

4.  Total port storage capacity of fuel oil, [TOTSTOR in barrels] 3,4

5.  Emission factor, 	[EFS in lbs/103 gallons transported by ships] 5

[EFB in lbs/103 gallons transported by barges] 5

- VOCS=0.50

- VOCB=1.20

6.  Weekly activity, WAF=7 days/week

7.  Seasonal adjustment factor, SAF 5a

Summer Season Adjustment Factor		1.22

Fall Season Adjustment Factor			0.93

Winter Season Adjustment Factor		0.93

Spring Season Adjustment Factor			0.93

- Process:

The following equations are used to calculate the emissions without control  for this source category.

Epa= (FUEL*CF/CF1)*(MUNSTOR/TOTSTOR)*((B/(B+S))*EFB + (S/(S+B))*EFS)/CF

Eps	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

CF1  = Conversion factor for units       = [6.4 lb/gal]6

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1.  All jet naptha exported from a port is loaded at that port.

2.  Barges are defined as non-self propelled vessels.

3. Foreign and Canadian exports, coastwise and lakewise shipments, internal shipments, internal outbound (Upbnd & Downbnd) internal intra (Upbnd & Downbnd), internal intraport and intra-territory shipments from port freight traffic sheets included in reference 2 constitutes marine fuel loading or export activity. 7

4. Jet Naptha marine vessel loading includes naptha and solvents and petroleum products not elsewhere classified activities from port freight traffic sheets included in reference 2. 8

5.  Assume that the number of ships (S) and barges (B) specified for all ports from trips and drafts of vessels  included in reference 2 includes both upbound and downbound shipping except as noted in item 10 below.

6.  Assume that  ship/ocean going vessel contribution to overall waterborne commerce (ships and barges) emissions  is correctly proportioned  by multiplication of  the ship/ocean going vessel  emission factor  (EFS) with the ratio of the  total number of ships divided by the total number of ships and barges, ie (Ships/(Ships+Barges))*EFS.  Similarly, the barge contribution to overall waterborne commerce is correctly proportioned by multiplication of the barge emission factor (EFB) with the ratio of the total number of  barges divided by the total number of ships and barges, ie (Ships/(Ships+Barges))*EFB.

7. Assume that the municipal fuel oil storage capacity of all docks within a municipality within a port (MUNSTOR) divided by the total fuel oil storage capacity within the port (TOTSTOR) allocates port emissions from fuel oil activities to a municipal basis.

8. % emissions to NJ from shipping activity to Ports shared by NJ or New York are allocated as follows for each of these respective ports: 9

Upper Bay NY Harbor, NY & NJ                   :  54.3 % port emissions to NJ

Hudson River Channel, NY & NJ                   :  50.0 % port emissions to NJ

New York & New Jersey Channel,NY & NJ  :  52.6 % port emissions to NJ

9.  % emissions to NJ from shipping activity to Ports shared by Pennsylannia and New Jersey, ie Delaware River (Phila to sea), Delaware River (Phila to Trenton), Camden and Trenton is allocated by crediting all northbound (upbound or inbound) ships(S) and barges(B) to NJ10.   Assume this emission allocation is accomplished by including only upbound or inbound shipping in the top portion of the emission equation related to S & B loading but including both upbound and downbound shipping in its bottom portion for every port shared by Pennsylannia and New Jersey, ie upbd S/(downbd S & upbd S + downbd B & upbd B) or upbd B/(downbd S & upbd S + downbd B & upbd B).

10. Fuel and shipping (S & B) amounts for the Port of Camden are subtracted from those amounts obtained for the Port of Delaware River (Phila to the sea)   because this port includes Camden and loading data for Camden has been obtained separately without allocation from this larger port.

11. Assume a weekly activity factor of  7 days per week.

12. Assume a seasonal adjustment factor of 1.

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:   None

- Double Counting:

Emissions for this source category may have been reported in the point source inventory, however, based on the descriptions given by the point source facilities it cannot be determined.  Therefore, to be conservative, no adjustment of the area source emissions was conducted.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

- References:

1. Procedures for the Preparation of Emission Inventories for Carbon Monoxide and Precursors of Ozone, Vol. I:  General Guidance for Stationary Sources,  United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-450/4-91-016, May 1991

2. Waterborne Commerce of the United States, Part 1, Waterways and Harbors Atlantic Coast, Calendar Year 2001, US Army Corp of Engineers, Water  Resources Support Center

3. The Ports of New York, NY and NJ and Ports on Long Island, NY, Port Series No. 5,  US Army Corps of Engineers, Revised 1988

4. The Ports of Philadelphia, Pa, Camden, NJ, Wilmington, De, and Ports on Delaware River, Port Series No. 8, US Army Corps of Engineers, Revised 1984

5. EIIP Volume III, Chapter 12 - Marine Vessel Loading, Ballasting, and Transit, Table 12.4-5, Uncontrolled VOC Emission Factors for Petroleum Carrying Marine Vessels , Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

5a.  Based on monthly – temperature adjusted true vapor pressures obtained from USEPA Tanks 4.09b model for Newark, New Jersey.

6. Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, Section 7.1-83, Table 7.1-2. Properties (MV,WVC, PVA,WL) of Selected Petroleum Liquids,  United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

7. EIIP Volume III, Chapter 12 - Marine Vessel Loading, Ballasting, and Transit, Table 12.4-1, Emission Points for Petroleum Vessel Traffic Classifications

8. EIIP Volume III, Chapter 12 - Marine Vessel Loading, Ballasting, and Transit Table 12.4-1, Product Type Classifications for Common Petroleum Vessel Commodities

9. Fax from Julie Tucker of Radian Corporation, Spatial Allocation of Waterways Shared by New York and New Jersey, November 13, 1992

10. Memorandum from Whitney Gadsby of the State of Delaware, Department of Natural Resources & Environmental Control, Division of Air & Waste Management, October 9, 1992

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Petroleum Transport, Marine Vessel-Kerosene Loading

- SCC:  2505020180

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category: 1

1.  Surveying individual shipping companies

2.  Fuel transported analysis

The fuel transported analysis is selected because the input data elements are more readily available.

Emissions from kerosene loading onto marine vessels are calculated using tons of kerosene loaded at each port, allocated to the county level by the total fuel storage capacity of each docking facility.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  Kerosene exported from each port (see item 3 below in assumption section), FUEL [103 tons] 2

2.  Number of ships and barges at each port, [S, B] 2

3.  Fuel oil storage capacity of all docks within municipality within port, [MUNSTOR in barrels] 3,4

4.  Total port storage capacity of fuel oil (barrels),[ TOTSTOR in barrels] 3,4

5.  Emission factor,	[EFS in lbs/103 gallons transported by ships] 5

[EFB in lbs/103 gallons transported by barges] 5

- VOCS=0.005

- VOCB=0.013

6.  Weekly activity, WAF=7 days/week

7.  Seasonal adjustment factor, SAF 5a

Summer Season Adjustment Factor		1.35

Fall Season Adjustment Factor			0.88

Winter Season Adjustment Factor		0.88

Spring Season Adjustment Factor			0.88

- Process:

The following equations are used to calculate the emissions without control  for this source category.

Epa= (FUEL*CF/CF1)*(MUNSTOR/TOTSTOR)*((B/(B+S))*EFB + (S/(S+B))*EFS)/CF

Eps= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

CF1        = Conversion factor for units = [7 lb/gal]6

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1.  All kerosene oil exported from a port is loaded at that port.

2.  Barges are defined as non-self propelled vessels.

3. Foreign and Canadian exports, coastwise and lakewise shipments, internal shipments, internal outbound (Upbnd & Downbnd) internal intra (Upbnd & Downbnd), internal intraport and intra-territory shipments from port freight traffic sheets included in reference 2 constitutes marine fuel loading or export activity. 7

4. kerosene oil marine vessel loading includes kerosene fuel oil loading activities from port freight traffic sheets included in reference 2. 8

5.  Assume that the number of ships (S) and barges (B) specified for all ports from trips and drafts of vessels  included in reference 2 includes both upbound and downbound shipping except as noted in item 10 below.

6.  Assume that  ship/ocean going vessel contribution to overall waterborne commerce (ships and barges) emissions  is correctly proportioned  by multiplication of  the ship/ocean going vessel  emission factor  (EFS) with the ratio of the  total number of ships divided by the total number of ships and barges, ie (Ships/(Ships+Barges))*EFS.  Similarly, the barge contribution to overall waterborne commerce is correctly proportioned by multiplication of the barge emission factor (EFB) with the ratio of the total number of  barges divided by the total number of ships and barges, ie (Ships/(Ships+Barges))*EFB.

7. Assume that the municipal fuel oil storage capacity of all docks within a municipality within a port (MUNSTOR) divided by the total fuel oil storage capacity within the port (TOTSTOR) allocates port emissions from fuel oil activities to a municipal basis.

8. % emissions to NJ from shipping activity to Ports shared by NJ or New York are allocated as follows for each of these respective ports:9

Upper Bay NY Harbor, NY & NJ                   :  54.3 % port emissions to NJ

Hudson River Channel, NY & NJ                   :  50.0 % port emissions to NJ

New York & New Jersey Channel,NY & NJ  :  52.6 % port emissions to NJ

9.  % emissions to NJ from shipping activity to Ports shared by Pennsylannia and New Jersey, ie Delaware River (Phila to sea), Delaware River (Phila to Trenton), Camden and Trenton is allocated by crediting all northbound (upbound or inbound) ships(S) and barges(B) to NJ10.   Assume this emission allocation is accomplished by including only upbound or inbound shipping in the top portion of the emission equation related to S & B loading but including both upbound and downbound shipping in its bottom portion for every port shared by Pennsylannia and New Jersey, ie upbd S/(downbd S & upbd S + downbd B & upbd B) or upbd B/(downbd S & upbd S + downbd B & upbd B).

10. Fuel and shipping (S & B) amounts for the Port of Camden are subtracted from those amounts obtained for the Port of Delaware River (Phila to the sea)   because this port includes Camden and loading data for Camden has been obtained separately without allocation from this larger port.

11. Assume that all point source marine vessel loading operations included in the emission statement program constitutes gas loading operations where control is indicated.  While all point source marine vessel loading operations without controls  represents distillate fuel loading unless otherwise noted in the emission statement record.

12.  Assume a weekly activity factor of  7 days per week.

13.  Assume a seasonal adjustment factor of 1.

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:      None

- Double Counting:

Emissions for this source category may have been reported in the point source inventory, however, based on the descriptions given by the point source facilities it cannot be determined.  Therefore, to be conservative, no adjustment of the area source emissions was conducted.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

- References:

1. Procedures for the Preparation of Emission Inventories for Carbon Monoxide and Precursors of Ozone, Vol. I:  General Guidance for Stationary Sources,  United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-450/4-91-016, May 1991

2. Waterborne Commerce of the United States, Part 1, Waterways and Harbors Atlantic Coast, Calendar Year 2001, US Army Corp of Engineers, Water  Resources Support Center

3. The Ports of New York, NY and NJ and Ports on Long Island, NY, Port Series No. 5,  US Army Corps of Engineers, Revised 1988

4. The Ports of Philadelphia, Pa, Camden, NJ, Wilmington, De, and Ports on Delaware River, Port Series No. 8,  US Army Corps of Engineers, Revised 1984

5. EIIP Volume III, Chapter 12 - Marine Vessel Loading, Ballasting, and Transit, Table 12.4-5, Uncontrolled VOC Emission Factors for Petroleum Carrying Marine Vessels , Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

5a.  Based on monthly – temperature adjusted true vapor pressures obtained from USEPA Tanks 4.09b model for Newark, New Jersey.

6. Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, Section 7.1-83, Table 7.1-2. Properties (MV,WVC, PVA,WL) of Selected Petroleum Liquids,  United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC, ,

7. EIIP Volume III, Chapter 12 - Marine Vessel Loading, Ballasting, and Transit, Table 12.4-1, Emission Points for Petroleum Vessel Traffic Classifications

8. EIIP Volume III, Chapter 12 - Marine Vessel Loading, Ballasting, and Transit Table 12.4-1, Product Type Classifications for Common Petroleum Vessel Commodities

9.  Fax from Julie Tucker of Radian Corporation, Spatial Allocation of Waterways Shared by New York and New

Jersey, November 13, 1992

10. Memorandum from Whitney Gadsby of the State of Delaware, Department of Natural Resources & Environmental Control, Division of Air & Waste Management, October 9, 1992

Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Petroleum Transport Marine Vessel-Crude Oil Transit

- SCC:  2505020030

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category: 1

1.  Surveying individual shipping companies

2.  Fuel transported analysis

The fuel transported analysis is selected because the input data elements are more readily available.

Emissions from marine vessel crude oil transit are calculated using tons of crude oil in transit at each port, allocated to the county level by the total fuel storage capacity of each docking facility and the mileage facing waterfront.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  Crude oil imported & exported from each port, [FUEL in 103 tons] 2

2. Water front length of a municipality downstream of advancing shipping

in port, [D in miles ]3,4,5

3.  Fuel oil storage capacity of individual docks within port, [DOCSTOR in barrels] 3,4

4.  Sum of the fuel oil storage capacity of all docks downstream of advancing shipping within port,

[DOCSTOR in barrels]3,4

5.  Total port storage capacity of fuel oil, [TOTSTOR in barrels] 3,4

6.  Time of transit in each port, [TIME=0.275*D  in hours] 6

7.  Emission factor, 	[EF in lbs/week-103 gallons] 7

[PER = portion of EF represents VOC emissions] 8

VOC=   EF*PER

VOC=  1.3*0.85

VOC=    1.105 lbs/week-103 gallons

8.  Weekly activity, WAF=7 days/week

9.  Seasonal adjustment factor, SAF 8a

Summer Season Adjustment Factor		1.40

Fall Season Adjustment Factor			0.86

Winter Season Adjustment Factor		0.86

Spring Season Adjustment Factor			0.86

- Process:

The following equations are used to calculate the emissions without control for this source category.

Epa=((FUEL*CF/CF2)*((TOTSTOR - DOCSTOR)/TOTSTOR)*(TIME/CF1)*((PER*EF)/CF)

Eps= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

CF1	= Conversion factor for units = 168 hrs/week

CF2        = Conversion factor for units = [7.1 lb/gal]9

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1.  Ships only enter and leave ports for import/export purposes.

2.  All marine vessel traffic from port freight traffic sheets (see reference 2) constitutes marine vessel import/export  fuel transit activity. 10

3. Crude oil marine vessel loading includes crude petroleum loading activities from port freight traffic sheets  included in reference 2. 11

4.  Assume that the average travel time in hours for vessels using the port is computed by the equation: t = 0.275*d. 6

5.  Assume a valid allocation of  port emissions from fuel oil activities to a municipal basis includes  the subtraction of the fuel oil storage capacity of all docks downstream of advancing shipping (DOCSTOR) from the total fuel oil storage capacity within the port (TOTSTOR) followed by division by TOTSTOR.  After which this amount is multiplied by the equation that converts distance traveled to time where distance is represented by the length in miles a municipality  faces on the waterfront route taken by ships/barges in transit.

6. % emissions to NJ from shipping activity to Ports shared by NJ or New York are allocated as follows for each of these respective ports: 12

Upper Bay NY Harbor, NY & NJ                 :  54.3 % port emissions to NJ

Hudson River Channel, NY & NJ                 :  50.0 % port emissions to NJ

New York & New Jersey Channel,NY & NJ:  52.6 % port emissions to NJ

7.  % emissions to NJ from shipping activity to Ports shared by Pennsylannia and New Jersey, ie Delaware River (Phila to sea) and Delaware River (Phila to Trenton), Camden and Trenton is allocated by crediting all northbound shipping, ie upbound or inbound, to NJ13.  Assume emissions allocation to NJ achieved from dividing upbound port S & B by total port S & B shipping, ie (upbd S or upbdB)/(downbd & upbd S + downbd & upbd B). The following allocation of emissions has been achieved for ports on the Delaware river.

Delaware River (Phila to the Sea)=(1313+4361)/11318 = 0.501325

Delaware River (Phila to Trenton)=(122+473)/1178 = 0.5051

Trenton= (1+6)/12 = 0.58333

8. Distinct NJ municipalities located on opposite sides of a waterway receive 50 % of the total emissions determined for that portion of a waterway that each town shares with another town. 6   Specifically, the ports with distinct NJ towns located on opposite sides of their waterway include the ports of: 1) Newark except for the last mile which is entirely within the boundary of the City of Newark 2) Hackensack River 3)Raritan River except for the 1st mile which is entirely within the boundaries of the City of South Amboy 3) Portions of Mantua Creek 4) Portions of Intracoastal Waterway.

9. Transit from Delaware River (Phila to the Sea) includes the Port of Camden. 2  Therefore no adjustment is made to make allowance for a separate Camden calculation based on shipping data to Camden.  Camden is just included as one more dock station on the Delaware River Port (Phila to the Sea).

10. Transit from Delaware River (Phila to Trenton) includes through traffic to Trenton but not the port of Trenton.  The port of Trenton is only a single dock which is the last stop on the River.  Which means it does not receive any through shipping from any other port or upstream docks but only shipping directed to the  Port of Trenton.  For this reason the fuel oil storage capacity of Trenton docks have not been included in the Delaware River (Phila to Trenton) to allocate port emissions to municipality level (see item 5 above).  Moreover, the small amount of ships and barges traveling just to the single dock in the port of Trenton have not been deducted from the total number of shipping indicated for the Delaware River (Phila to Trenton) port which is used to determine NJ and Penn shipping proportions.  Whereas the small amount of shipping traveling to Trenton from the second to last port on the Delaware River is used only to apportion  NJ/Penn traffic from the second to last port in Burlington Twp to the last port on the river in Trenton.  (see item 7 above)

11. Assume that all shipping traveling to ports upstream of other designated ports must first travel through the  waterways of the downstream port.  This requires that emissions caused by this through travel have to be included with the regular emissions associated with travel in the designated port.

12.  Assume a weekly activity factor of 7 days per week.

13.  Assume a seasonal adjustment factor of 1.

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:   None

- Double Counting:

Emissions for this source category may have been reported in the point source inventory, however, based on the descriptions given by the point source facilities it cannot be determined.  Therefore, to be conservative, no adjustment of the area source emissions was conducted.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

- References:

1. Procedures for the Preparation of Emission Inventories for Carbon Monoxide and Precursors of Ozone, Vol. I:  General Guidance for Stationary Sources,  United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-450/4-91-016, May 1991

2. Waterborne Commerce of the United States, Part 1, Waterways and Harbors Atlantic Coast, Calendar Year 2001, US Army Corp of Engineers, Water  Resources Support Center

3. The Ports of New York, NY and NJ and Ports on Long Island, NY, Port Series No. 5,  US Army Corps of Engineers, Revised 1988

4. The Ports of Philadelphia, Pa, Camden, NJ, Wilmington, De, and Ports on Delaware River, Port Series No. 8, US Army Corps of Engineers, Revised 1984

5. County Maps by Hagstrom or Alfred B. Patton.

6. Memorandum from James Labanowski of Radian Corporation, Spatial Allocation of Commercial Marine Vessel Emissions for SIP Base Year Inventories, June 9, 1993

7. EIIP Volume III, Chapter 12 - Marine Vessel Loading, Ballasting, and Transit, Table 12.4-5, Uncontrolled VOC Emission Factors for Petroleum Carrying Marine Vessels , Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

8. Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, Section 5.2-14, Table 5.2-6. Total Organic Emission Factors for Petroleum Marine Vessel Sources, Footnote c, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

8a.  Based on monthly – temperature adjusted true vapor pressures obtained from USEPA Tanks 4.09b model for Newark, New Jersey.

9. Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition,  United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC Section 7.1-83, Table 7.1-2. Properties (MV,WVC, PVA,WL) of Selected Petroleum Liquids

10. EIIP Volume III, Chapter 12 - Marine Vessel Loading, Ballasting, and Transit, Table 12.4-1, Emission Points for Petroleum Vessel Traffic Classifications

11. EIIP Volume III, Chapter 12 - Marine Vessel Loading, Ballasting, and Transit Table 12.4-1, Product Type Classifications for Common Petroleum Vessel Commodities

12. Fax from Julie Tucker of Radian Corporation, Spatial Allocation of Waterways Shared by New York and New Jersey, November 13, 1992

13. Memorandum from Whitney Gadsby of the State of Delaware, Department of Natural Resources & Environmental Control, Division of Air & Waste Management, October 9, 1992

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Petroleum Transport, Marine Vessel-Residual Oil Transit

- SCC:  2505020060

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category: 1

1.  Surveying individual shipping companies

2.  Fuel transported analysis

The fuel transported analysis is selected because the input data elements are more readily available.

Emissions from marine vessel residual oil transit are calculated using tons of residual oil in transit at each port, allocated to the county level by the total fuel storage capacity of each docking facility and the mileage facing waterfront.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  Residual oil imported & exported from each port, [FUEL  in 103 tons] 2

2.  Water front length of a municipality downstream of advancing shipping in port [D in miles] 3,4,5

3.  Fuel oil storage capacity of individual docks within port, [DOCSTOR in barrels] 3,4

4.  Sum of the fuel oil storage capacity of all docks downstream of advancing shipping within port DOCSTOR 3,4

5.  Total port storage capacity of fuel oil,[TOTSTOR in barrels] 3,4

6.  Time of transit in each port, [TIME=0.275*D in hours] 6

7.  Emission factor,[EF in lbs/week-103 gallons] 7

- VOC=0.00003

8.  Weekly activity, [WAF=7 days/week]

9.  Seasonal adjustment factor, SAF 7a

Summer Season Adjustment Factor		1.6

Fall Season Adjustment Factor			0.8

Winter Season Adjustment Factor		0.8

Spring Season Adjustment Factor			0.8

- Process:

The following equations are used to calculate the emissions without control for this source category.

Epa=((FUEL*CF/CF2)*((TOTSTOR - DOCSTOR)/TOTSTOR)*(TIME/CF1)*(EF/CF)

Eps= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

CF1	= Conversion factor for units = 168 hrs/week

CF2  = Conversion factor for units = [7.9 lb/gal]8

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1.  Ships only enter and leave ports for import/export purposes.

2.  All marine vessel traffic from port freight traffic sheets (see reference 2) constitutes marine vessel import/export  fuel transit activity. 9

3. Residual oil marine vessel loading includes residual fuel oil, asphalt, tar, and pitch, and petroleum coke loading activities from port freight traffic sheets included in reference 2. 10

4.  Assume that the average travel time in hours for vessels using the port is computed by the equation: t = 0.275*d. 6

5.  Assume a valid allocation of  port emissions from fuel oil activities to a municipal basis includes  the subtraction of the fuel oil storage capacity of all docks downstream of advancing shipping (DOCSTOR) from the total fuel oil storage capacity within the port (TOTSTOR) followed by division by TOTSTOR.  After which this amount is multiplied by the equation that converts distance traveled to time where distance is represented by the length in miles a municipality  faces on the waterfront route taken by ships/barges in transit.

6. % emissions to NJ from shipping activity to Ports shared by NJ or New York are allocated as follows for each of these respective ports:11

Upper Bay NY Harbor, NY & NJ                 :  54.3 % port emissions to NJ

Hudson River Channel, NY & NJ                 :  50.0 % port emissions to NJ

New York & New Jersey Channel,NY & NJ:  52.6 % port emissions to NJ

7.  % emissions to NJ from shipping activity to Ports shared by Pennsylannia and New Jersey, ie Delaware River (Phila to sea) and Delaware River (Phila to Trenton), Camden and Trenton is allocated by crediting all northbound shipping, ie upbound or inbound, to NJ12.  Assume emissions allocation to NJ achieved from dividing upbound port S & B by total port S & B shipping, ie (upbd S or upbdB)/(downbd & upbd S + downbd & upbd B). The following allocation of emissions has been achieved for ports on the Delaware river.

Delaware River (Phila to the Sea)=(1313+4361)/11318 = 0.501325

Delaware River (Phila to Trenton)=(122+473)/1178 = 0.5051

Trenton= (1+6)/12 = 0.58333

8. Distinct NJ municipalities located on opposite sides of a waterway receive 50 % of the total emissions determined for that portion of a waterway that each town shares with another town. 6   Specifically, the ports with distinct NJ towns located on opposite sides of their waterway include the ports of: 1) Newark except for the last mile which is entirely within the boundary of the City of Newark 2) Hackensack River 3)Raritan River except for the 1st mile which is entirely within the boundaries of the City of South Amboy 3) Portions of Mantua Creek 4) Portions of Intracoastal Waterway.

9. Transit from Delaware River (Phila to the Sea) includes the Port of Camden. 2  Therefore no adjustment is made to make allowance for a separate Camden calculation based on shipping data to Camden.  Camden is just included as one more dock station on the Delaware River Port (Phila to the Sea).

10. Transit from Delaware River (Phila to Trenton) includes through traffic to Trenton but not the port of Trenton.  The port of Trenton is only a single dock which is the last stop on the River.  Which means it does not receive any through shipping from any other port or upstream docks but only shipping directed to the  Port of Trenton.  For this reason the fuel oil storage capacity of Trenton docks have not been included in the Delaware River (Phila to Trenton) to allocate port emissions to municipality level (see item 5 above).  Moreover, the small amount of ships and barges traveling just to the single dock in the port of Trenton have not been deducted from the total number of shipping indicated for the Delaware River (Phila to Trenton) port which is used to determine NJ and Penn shipping proportions.  Whereas the small amount of shipping traveling to Trenton from the second to last port on the Delaware River is used only to apportion  NJ/Penn traffic from the second to last port in Burlington Twp to the last port on the river in Trenton.  (see item 7 above)

11. Assume that all shipping traveling to ports upstream of other designated ports must first travel through the waterways of the downstream port.  This requires that emissions caused by this through travel have to be included with the regular emissions associated with travel in the designated port.

12.  Assume a weekly activity factor of 7 days per week.

13.  Assume a seasonal adjustment factor of 1.

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:     None

- Double Counting:

Emissions for this source category may have been reported in the point source inventory, however, based on the descriptions given by the point source facilities it cannot be determined.  Therefore, to be conservative, no adjustment of the area source emissions was conducted.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

- References:

1. Procedures for the Preparation of Emission Inventories for Carbon Monoxide and Precursors of Ozone, Vol. I:  General Guidance for Stationary Sources,  United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-450/4-91-016, May 1991

2. Waterborne Commerce of the United States, Part 1, Waterways and Harbors Atlantic Coast, Calendar Year 2001, US Army Corp of Engineers, Water  Resources Support Center

3. The Ports of New York, NY and NJ and Ports on Long Island, NY, Port Series No. 5,  US Army Corps of Engineers, Revised 1988

4. The Ports of Philadelphia, Pa, Camden, NJ, Wilmington, De, and Ports on Delaware River, Port Series No. 8, US Army Corps of Engineers, Revised 1984

5. County Maps by Hagstrom or Alfred B. Patton.

6. Memorandum from James Labanowski of Radian Corporation, Spatial Allocation of Commercial Marine Vessel Emissions for SIP Base Year Inventories, June 9, 1993

7. EIIP Volume III, Chapter 12 - Marine Vessel Loading, Ballasting, and Transit, Table 12.4-5, Uncontrolled VOC Emission Factors for Petroleum Carrying Marine Vessels , Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

7a.  Based on monthly – temperature adjusted true vapor pressures obtained from USEPA Tanks 4.09b model for Newark, New Jersey.

8. Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, Section 5.2-14, Table 5.2-6. Total Organic Emission Factors for Petroleum Marine Vessel Sources, Section 7.1-83, Table 7.1-2. Properties (MV,WVC, PVA,WL) of Selected Petroleum Liquids, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

9. EIIP Volume III, Chapter 12 - Marine Vessel Loading, Ballasting, and Transit, Table 12.4-1, Emission Points for Petroleum Vessel Traffic Classifications

10. EIIP Volume III, Chapter 12 - Marine Vessel Loading, Ballasting, and Transit Table 12.4-1, Product Type Classifications for Common Petroleum Vessel Commodities

11. Fax from Julie Tucker of Radian Corporation, Spatial Allocation of Waterways Shared by New York and New Jersey, November 13, 1992

12. Memorandum from Whitney Gadsby of the State of Delaware, Department of Natural Resources & Environmental Control, Division of Air & Waste Management, October 9, 1992

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Petroleum Transport, Marine Vessel-Distillate Oil Transit

- SCC:  2505020090

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category: 1

1.  Surveying individual shipping companies

2.  Fuel transported analysis

The fuel transported analysis is selected because the input data elements are more readily available.

Emissions from marine vessel distillate oil transit are calculated using tons of distillate oil in transit at each port, allocated to the county level by the total fuel storage capacity of each docking facility and the mileage facing waterfront.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  Distillate oil imported & exported from each port,[ FUEL in 103 tons] 2

2.  Water front length of a municipality downstream of advancing shipping in port [D in miles] 3,4,5

3.  Fuel oil storage capacity of individual docks within port, [DOCSTOR in barrels] 3,4

4.  Sum of the fuel oil storage capacity of all docks downstream of advancing shipping within port DOCSTOR 3,4

5.  Total port storage capacity of fuel oil, [TOTSTOR in barrels] 3,4

6.  Time of transit in each port, [TIMEport=.275*D  in hours] 6

7.  Emission factor, [EF in lbs/week - 103 gallons] 7

- VOC=0.005

8.  Weekly activity, WAF=7 days/week

9.  Seasonal adjustment factor, SAF 7a

Summer Season Adjustment Factor		1.54

Fall Season Adjustment Factor			0.82

Winter Season Adjustment Factor		0.82

Spring Season Adjustment Factor			0.82

- Process:

The following equations are used to calculate the emissions without control for this source category.

Epa=((FUEL*CF/CF2)*((TOTSTOR - DOCSTOR)/TOTSTOR)*(TIME/CF1)*(EF/CF)

Eps= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

CF1	= Conversion factor for units = 168 hrs/week

CF2  	= Conversion factor for units = [7.1 lb/gal] 8

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1.  Ships only enter and leave ports for import/export purposes.

2.  All marine vessel traffic from port freight traffic sheets (see reference 2) constitutes marine vessel import/export  fuel transit activity. 9

3. Distillate oil marine vessel loading includes distillate fuel oil, lube oil and greases, and petro, jelly and waxes  loading activities from port freight traffic sheets included in reference 2. 10

4.  Assume that the average travel time in hours for vessels using the port is computed by the equation: t = 0.275*d. 6

5.  Assume a valid allocation of  port emissions from fuel oil activities to a municipal basis includes  the subtraction of the fuel oil storage capacity of all docks downstream of advancing shipping (DOCSTOR) from the total fuel oil storage capacity within the port (TOTSTOR) followed by division by TOTSTOR.  After which this amount is multiplied by the equation that converts distance traveled to time where distance is represented by the length in miles a municipality  faces on the waterfront route taken by ships/barges in transit.

6. % emissions to NJ from shipping activity to Ports shared by NJ or New York are allocated as follows for each of these respective ports:11

Upper Bay NY Harbor, NY & NJ                 :  54.3 % port emissions to NJ

Hudson River Channel, NY & NJ                 :  50.0 % port emissions to NJ

New York & New Jersey Channel,NY & NJ:  52.6 % port emissions to NJ

7.  % emissions to NJ from shipping activity to Ports shared by Pennsylannia and New Jersey, ie Delaware River (Phila to sea) and Delaware River (Phila to Trenton), Camden and Trenton is allocated by crediting all northbound shipping, ie upbound or inbound, to NJ12.  Assume emissions allocation to NJ achieved from dividing upbound port S & B by total port S & B shipping, ie (upbd S or upbdB)/(downbd & upbd S + downbd & upbd B). The following allocation of emissions has been achieved for ports on the Delaware river.

Delaware River (Phila to the Sea)=(1313+4361)/11318 = 0.501325

Delaware River (Phila to Trenton)=(122+473)/1178 = 0.5051

Trenton= (1+6)/12 = 0.58333

8. Distinct NJ municipalities located on opposite sides of a waterway receive 50 % of the total emissions determined for that portion of a waterway that each town shares with another town. 6   Specifically, the ports with distinct NJ towns located on opposite sides of their waterway include the ports of: 1) Newark except for the last mile which is entirely within the boundary of the City of Newark 2) Hackensack River 3)Raritan River except for the 1st mile which is entirely within the boundaries of the City of South Amboy 3) Portions of Mantua Creek 4) Portions of Intracoastal Waterway.

9. Transit from Delaware River (Phila to the Sea) includes the Port of Camden. 2  Therefore no adjustment is made to make allowance for a separate Camden calculation based on shipping data to Camden.  Camden is just included as one more dock station on the Delaware River Port (Phila to the Sea).

10. Transit from Delaware River (Phila to Trenton) includes through traffic to Trenton but not the port of Trenton.  The port of Trenton is only a single dock which is the last stop on the River.  Which means it does not receive any through shipping from any other port or upstream docks but only shipping directed to the  Port of Trenton.  For this reason the fuel oil storage capacity of Trenton docks have not been included in the Delaware River (Phila to Trenton) to allocate port emissions to municipality level (see item 5 above).  Moreover, the small amount of ships and barges traveling just to the single dock in the port of Trenton have not been deducted from the total number of shipping indicated for the Delaware River (Phila to Trenton) port which is used to determine NJ and Penn shipping proportions.  Whereas the small amount of shipping traveling to Trenton from the second to last port on the Delaware River is used only to apportion  NJ/Penn traffic from the second to last port in Burlington Twp to the last port on the river in Trenton.  (see item 7 above)

11. Assume that all shipping traveling to ports upstream of other designated ports must first travel through the waterways of the downstream port.  This requires that emissions caused by this through travel have to be included with the regular emissions associated with travel in the designated port.

12.  Assume a weekly activity factor of 7 days per week.

13.  Assume a seasonal adjustment factor of 1.

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:    None

- Double Counting:

Emissions for this source category may have been reported in the point source inventory, however, based on the descriptions given by the point source facilities it cannot be determined.  Therefore, to be conservative, no adjustment of the area source emissions was conducted.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

- References:

1. Procedures for the Preparation of Emission Inventories for Carbon Monoxide and Precursors of Ozone, Vol. I:  General Guidance for Stationary Sources,  United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-450/4-91-016, May 1991

2. Waterborne Commerce of the United States, Part 1, Waterways and Harbors Atlantic Coast, Calendar Year 2001, US Army Corp of Engineers, Water  Resources Support Center

3. The Ports of New York, NY and NJ and Ports on Long Island, NY, Port Series No. 5,  US Army Corps of Engineers, Revised 1988

4. The Ports of Philadelphia, Pa, Camden, NJ, Wilmington, De, and Ports on Delaware River, Port Series No. 8, US Army Corps of Engineers, Revised 1984

5.  County Maps by Hagstrom or Alfred B. Patton.

6.  Memorandum from James Labanowski of Radian Corporation, Spatial Allocation of Commercial Marine Vessel Emissions for SIP Base Year Inventories, June 9, 1993

7.  EIIP Volume III, Chapter 12 - Marine Vessel Loading, Ballasting, and Transit, Table 12.4-5, Uncontrolled VOC Emission Factors for Petroleum Carrying Marine Vessels , Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

7a.  Based on monthly – temperature adjusted true vapor pressures obtained from USEPA Tanks 4.09b model for Newark, New Jersey.

8. Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, Section 5.2-14, Table 5.2-6. Total Organic Emission Factors for Petroleum Marine Vessel Sources, Section 7.1-83, Table 7.1-2. Properties (MV,WVC, PVA,WL) of Selected Petroleum Liquids, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

9.  EIIP Volume III, Chapter 12 - Marine Vessel Loading, Ballasting, and Transit, Table 12.4-1, Emission Points for Petroleum Vessel Traffic Classifications

10.  EIIP Volume III, Chapter 12 - Marine Vessel Loading, Ballasting, and Transit Table 12.4-1, Product Type Classifications for Common Petroleum Vessel Commodities

11.  Fax from Julie Tucker of Radian Corporation, Spatial Allocation of Waterways Shared by New York and New Jersey, November 13, 1992

12. Memorandum from Whitney Gadsby of the State of Delaware, Department of Natural Resources & Environmental Control, Division of Air & Waste Management, October 9, 1992

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Petroleum Transport, Marine Vessel-Gasoline Transit

- SCC:  2505020120

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category: 1

1.  Surveying individual shipping companies

2.  Fuel transported analysis

The fuel transported analysis is selected because the input data elements are more readily available.

Emissions from marine vessel gasoline oil transit are calculated using tons of gasoline oil in transit at each port, allocated to the county level by the total fuel storage capacity of each docking facility and the mileage facing waterfront.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  Gasoline imported & exported from each port,[FUEL in 103 tons] 2

2.  Water front length of a municipality downstream of advancing shipping in port [D in miles] 3,4,5

3.  Fuel oil storage capacity of individual docks within port, [DOCSTOR in barrels] 3,4

4.  Sum of the fuel oil storage capacity of all docks downstream of advancing shipping within port

[DOCSTOR in barrels] 3,4

5.  Total port storage capacity of fuel oil, [TOTSTOR in barrels] 3,4

6.  Time of transit in each port, [TIME=.275*D  in hours] 6

7.  Emission factor, [EF in lbs/week - 103 gallons] 7

- VOC=2.7

8.  Weekly activity, WAF=7 days/week

9.  Seasonal adjustment factor, SAF 7a

Summer Season Adjustment Factor		0.90

Fall Season Adjustment Factor			1.03

Winter Season Adjustment Factor		1.03

Spring Season Adjustment Factor			1.03

- Process:

The following equations are used to calculate the emissions without control  for this source category.

Epa=((FUEL*CF/CF2)*((TOTSTOR - DOCSTOR)/TOTSTOR)*(TIME/CF1)*(EF/CF)

Eps	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

CF1	= Conversion factor for units = 168 hrs/week

CF2  = Conversion factor for units = [5.6 lb/gal]8

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1.  Ships only enter and leave ports for import/export purposes.

2.  All marine vessel traffic from port freight traffic sheets (see reference 2) constitutes marine vessel import/export  fuel transit activity. 9

3. Gasoline marine vessel loading includes gasoline and liquid nautral gas loading activities from port freight traffic sheets included in reference 2. 10

4.  Assume that the average travel time in hours for vessels using the port is computed by the equation: t = 0.275*d. 6

5.  Assume a valid allocation of  port emissions from fuel oil activities to a municipal basis includes  the subtraction of the fuel oil storage capacity of all docks downstream of advancing shipping (DOCSTOR) from the total fuel oil storage capacity within the port (TOTSTOR) followed by division by TOTSTOR.  After which this amount is multiplied by the equation that converts distance traveled to time where distance is represented by the length in miles a municipality  faces on the waterfront route taken by ships/barges in transit.

6. % emissions to NJ from shipping activity to Ports shared by NJ or New York are allocated as follows for each of these respective ports:11

Upper Bay NY Harbor, NY & NJ                 :  54.3 % port emissions to NJ

Hudson River Channel, NY & NJ                 :  50.0 % port emissions to NJ

New York & New Jersey Channel,NY & NJ:  52.6 % port emissions to NJ

7.  % emissions to NJ from shipping activity to Ports shared by Pennsylannia and New Jersey, ie Delaware River (Phila to sea) and Delaware River (Phila to Trenton), Camden and Trenton is allocated by crediting all northbound shipping, ie upbound or inbound, to NJ12.  Assume emissions allocation to NJ achieved from dividing upbound port S & B by total port S & B shipping, ie (upbd S or upbdB)/(downbd & upbd S + downbd & upbd B). The following allocation of emissions has been achieved for ports on the Delaware river.

Delaware River (Phila to the Sea)=(1313+4361)/11318 = 0.501325

Delaware River (Phila to Trenton)=(122+473)/1178 = 0.5051

Trenton= (1+6)/12 = 0.58333

8. Distinct NJ municipalities located on opposite sides of a waterway receive 50 % of the total emissions determined for that portion of a waterway that each town shares with another town. 6   Specifically, the ports with distinct

NJ towns located on opposite sides of their waterway include the ports of: 1) Newark except for the last mile which is entirely within the boundary of the City of Newark 2) Hackensack River 3)Raritan River except for the 1st mile which is entirely within the boundaries of the City of South Amboy 3) Portions of Mantua Creek 4) Portions of Intracoastal Waterway.

9. Transit from Delaware River (Phila to the Sea) includes the Port of Camden. 2  Therefore no adjustment is made to make allowance for a separate Camden calculation based on shipping data to Camden.  Camden is just included as one more dock station on the Delaware River Port (Phila to the Sea).

10. Transit from Delaware River (Phila to Trenton) includes through traffic to Trenton but not the port of Trenton.  The port of Trenton is only a single dock which is the last stop on the River.  Which means it does not receive any through shipping from any other port or upstream docks but only shipping directed to the  Port of Trenton.  For this reason the fuel oil storage capacity of Trenton docks have not been included in the Delaware River (Phila to Trenton) to allocate port emissions to municipality level (see item 5 above).  Moreover, the small amount of ships and barges traveling just to the single dock in the port of Trenton have not been deducted from the total number of shipping indicated for the Delaware River (Phila to Trenton) port which is used to determine NJ and Penn shipping proportions.  Whereas the small amount of shipping traveling to Trenton from the second to last port on the Delaware River is used only to apportion  NJ/Penn traffic from the second to last port in Burlington Twp to the last port on the river in Trenton.  (see item 7 above)

11. Assume that all shipping traveling to ports upstream of other designated ports must first travel through the waterways of the downstream port.  This requires that emissions caused by this through travel have to be included with the regular emissions associated with travel in the designated port.

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:   None

- Double Counting:

Emissions for this source category may have been reported in the point source inventory, however, based on the descriptions given by the point source facilities it cannot be determined.  Therefore, to be conservative, no adjustment of the area source emissions was conducted.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

- References:

1.  Procedures for the Preparation of Emission Inventories for Carbon Monoxide and Precursors of Ozone, Vol. I:  General Guidance for Stationary Sources,  United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-450/4-91-016, May 1991

2.  Waterborne Commerce of the United States, Part 1, Waterways and Harbors Atlantic Coast, Calendar Year 2001, US Army Corp of Engineers, Water  Resources Support Center

3. The Ports of New York, NY and NJ and Ports on Long Island, NY, Port Series No. 5,  US Army Corps of Engineers, Revised 1988

4. The Ports of Philadelphia, Pa, Camden, NJ, Wilmington, De, and Ports on Delaware River, Port Series No. 8, US Army Corps of Engineers, Revised 1984

5.  County Maps by Hagstrom or Alfred B. Patton.

6.  Memorandum from James Labanowski of Radian Corporation, Spatial Allocation of Commercial Marine Vessel Emissions for SIP Base Year Inventories, June 9, 1993

7.  EIIP Volume III, Chapter 12 - Marine Vessel Loading, Ballasting, and Transit, Table 12.4-5, Uncontrolled VOC Emission Factors for Petroleum Carrying Marine Vessels , Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

7a.  Based on monthly – temperature adjusted true vapor pressures obtained from USEPA Tanks 4.09b model for Newark, New Jersey.

8. Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, Section 5.2-14, Table 5.2-6. Total Organic Emission Factors for Petroleum Marine Vessel Sources, Section 7.1-83, Table 7.1-2. Properties (MV,WVC, PVA,WL) of Selected Petroleum Liquids, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

9.  EIIP Volume III, Chapter 12 - Marine Vessel Loading, Ballasting, and Transit, Table 12.4-1, Emission Points for Petroleum Vessel Traffic Classifications

10.  EIIP Volume III, Chapter 12 - Marine Vessel Loading, Ballasting, and Transit Table 12.4-1, Product Type Classifications for Common Petroleum Vessel Commodities

11.  Fax from Julie Tucker of Radian Corporation, Spatial Allocation of Waterways Shared by New York and New Jersey, November 13, 1992

12. Memorandum from Whitney Gadsby of the State of Delaware, Department of Natural Resources & Environmental Control, Division of Air & Waste Management, October 9, 1992

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Petroleum Transport, Marine Vessel-Jet Naphtha Transit

- SCC:  2505020150

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category: 1

1.  Surveying individual shipping companies

2.  Fuel transported analysis

The fuel transported analysis is selected because the input data elements are more readily available.

Emissions from marine vessel jet naphtha transit are calculated using tons of jet naphtha in transit at each port, allocated to the county level by the total fuel storage capacity of each docking facility and the mileage facing waterfront.

- Required Input Parameters:

- The following input data is required to calculate emissions for this source category.

1.  Jet naphtha imported & exported from each port, [FUEL in 103 tons] 2

2.  Water front length of a municipality downstream of advancing shipping in port [D in miles] 3 ,4,5

3.  Fuel oil storage capacity of individual docks within port, [DOCSTOR in barrels] 3,4

4.  Sum of the fuel oil storage capacity of all docks downstream of advancing shipping within port

[DOCSTOR in barrels] 3,4

5.  Total port storage capacity of fuel oil, [TOTSTOR  in barrels] 3,4

6.  Time of transit in each port, [TIME=.275*D  in hours] 6

7.  Emission factor, [EF in  lbs/week - 103 gallons] 7

- VOC=0.7

8.  Weekly activity, WAF=7 days/week

9.  Seasonal adjustment factor, SAF 7a

Summer Season Adjustment Factor		1.4

Fall Season Adjustment Factor			0.86

Winter Season Adjustment Factor		0.86

Spring Season Adjustment Factor			0.86

- Process:

The following equations are used to calculate the emissions without control  for this source category.

Epa    = ((FUEL*CF/CF2)*((TOTSTOR - DOCSTOR)/TOTSTOR)*(TIME/CF1)*(EF/CF)

Eps  = Epa/AADF*SAF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

CF1	= Conversion factor for units = 168 hrs/week

CF2        = Conversion factor for units = [6.4 lb/gal] 8

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1.  Ships only enter and leave ports for import/export purposes.

2.  All marine vessel traffic from port freight traffic sheets (see reference 2) constitutes marine vessel import/export  fuel transit activity. 9

3. Jet Naptha marine vessel loading includes naptha and solvents and petroleum products not elsewhere classified  activities from port freight traffic sheets included in reference 2.10

4.  Assume that the average travel time in hours for vessels using the port is computed by the equation: t = 0.275*d. 6

5.  Assume a valid allocation of  port emissions from fuel oil activities to a municipal basis includes  the subtraction of the fuel oil storage capacity of all docks downstream of advancing shipping (DOCSTOR) from the total fuel oil storage capacity within the port (TOTSTOR) followed by division by TOTSTOR.  After which this amount is multiplied by the equation that converts distance traveled to time where distance is represented by the length in miles a municipality  faces on the waterfront route taken by ships/barges in transit.

6. % emissions to NJ from shipping activity to Ports shared by NJ or New York are allocated as follows for each of these respective ports: 11

Upper Bay NY Harbor, NY & NJ                 :  54.3 % port emissions to NJ

Hudson River Channel, NY & NJ                 :  50.0 % port emissions to NJ

New York & New Jersey Channel,NY & NJ:  52.6 % port emissions to NJ

7.  % emissions to NJ from shipping activity to Ports shared by Pennsylannia and New Jersey, ie Delaware River (Phila to sea) and Delaware River (Phila to Trenton), Camden and Trenton is allocated by crediting all northbound shipping, ie upbound or inbound, to NJ12.  Assume emissions allocation to NJ achieved from dividing upbound port S & B by total port S & B shipping, ie (upbd S or upbdB)/(downbd & upbd S + downbd & upbd B). The following allocation of emissions has been achieved for ports on the Delaware river.

Delaware River (Phila to the Sea)=(1313+4361)/11318 = 0.501325

Delaware River (Phila to Trenton)=(122+473)/1178 = 0.5051

Trenton= (1+6)/12 = 0.58333

8. Distinct NJ municipalities located on opposite sides of a waterway receive 50 % of the total emissions determined for that portion of a waterway that each town shares with another town. 6   Specifically, the ports with distinct NJ towns located on opposite sides of their waterway include the ports of: 1) Newark except for the last mile which is entirely within the boundary of the City of Newark 2) Hackensack River 3)Raritan River except for the 1st mile which is entirely within the boundaries of the City of South Amboy 3) Portions of Mantua Creek 4) Portions of Intracoastal Waterway.

9. Transit from Delaware River (Phila to the Sea) includes the Port of Camden. 2  Therefore no adjustment is made to make allowance for a separate Camden calculation based on shipping data to Camden.  Camden is just included as one more dock station on the Delaware River Port (Phila to the Sea).

10. Transit from Delaware River (Phila to Trenton) includes through traffic to Trenton but not the port of Trenton.  The port of Trenton is only a single dock which is the last stop on the River.  Which means it does not receive any through shipping from any other port or upstream docks but only shipping directed to the  Port of Trenton.  For this reason the fuel oil storage capacity of Trenton docks have not been included in the Delaware River (Phila to Trenton) to allocate port emissions to municipality level (see item 5 above).  Moreover, the small amount of ships and barges traveling just to the single dock in the port of Trenton have not been deducted from the total number of shipping indicated for the Delaware River (Phila to Trenton) port which is used to determine NJ and Penn shipping proportions.  Whereas the small amount of shipping traveling to Trenton from the second to last port on the Delaware River is used only to apportion  NJ/Penn traffic from the second to last port in Burlington Twp to the last port on the river in Trenton.  (see item 7 above)

11. Assume that all shipping traveling to ports upstream of other designated ports must first travel through the waterways of the downstream port.  This requires that emissions caused by this through travel have to be included with the regular emissions associated with travel in the designated port.

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:    None

- Double Counting:

Emissions for this source category may have been reported in the point source inventory, however, based on the descriptions given by the point source facilities it cannot be determined.  Therefore, to be conservative, no adjustment of the area source emissions was conducted.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

- References:

1.  Procedures for the Preparation of Emission Inventories for Carbon Monoxide and Precursors of Ozone, Vol. I:  General Guidance for Stationary Sources,  United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-450/4-91-016, May 1991

2.  Waterborne Commerce of the United States, Part 1, Waterways and Harbors Atlantic Coast, Calendar Year 2001, US Army Corp of Engineers, Water  Resources Support Center

3. The Ports of New York, NY and NJ and Ports on Long Island, NY, Port Series No. 5,  US Army Corps of Engineers, Revised 1988

4. The Ports of Philadelphia, Pa, Camden, NJ, Wilmington, De, and Ports on Delaware River, Port Series No. 8, US Army Corps of Engineers, Revised 1984

5.  County Maps by Hagstrom or Alfred B. Patton.

6.  Memorandum from James Labanowski of Radian Corporation, Spatial Allocation of Commercial Marine Vessel Emissions for SIP Base Year Inventories, June 9, 1993

7.  EIIP Volume III, Chapter 12 - Marine Vessel Loading, Ballasting, and Transit, Table 12.4-5, Uncontrolled VOC Emission Factors for Petroleum Carrying Marine Vessels , Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

7a.  Based on monthly – temperature adjusted true vapor pressures obtained from USEPA Tanks 4.09b model for Newark, New Jersey.

8. Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, Section 5.2-14, Table 5.2-6. Total Organic Emission Factors for Petroleum Marine Vessel Sources, Section 7.1-83, Table 7.1-2. Properties (MV,WVC, PVA,WL) of Selected Petroleum Liquids, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

9.  EIIP Volume III, Chapter 12 - Marine Vessel Loading, Ballasting, and Transit, Table 12.4-1, Emission Points for Petroleum Vessel Traffic Classifications

10.  EIIP Volume III, Chapter 12 - Marine Vessel Loading, Ballasting, and Transit Table 12.4-1, Product Type Classifications for Common Petroleum Vessel Commodities

11.  Fax from Julie Tucker of Radian Corporation, Spatial Allocation of Waterways Shared by New York and New Jersey, November 13, 1992

12. Memorandum from Whitney Gadsby of the State of Delaware, Department of Natural Resources & Environmental Control, Division of Air & Waste Management, October 9, 1992

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Petroleum Transport, Marine Vessel-Kerosene Transit

- SCC:  2505020180

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category: 1

1.  Surveying individual shipping companies

2.  Fuel transported analysis

The fuel transported analysis is selected because the input data elements are more readily available.

Emissions from marine vessel kerosene transit are calculated using tons of kerosene in transit at each port, allocated to the county level by the total fuel storage capacity of each docking facility and the mileage facing waterfront.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  Kerosene imported & exported from each port, [FUEL  in 103 tons] 2

2.  Water front length of a municipality downstream of advancing shipping in port [D in miles] 3,4,5

3.  Fuel oil storage capacity of individual docks within port, [DOCSTOR in barrels] 3,4

4.  Sum of the fuel oil storage capacity of all docks downstream of advancing shipping within port

[DOCSTOR  in barrels] 3,4

5.  Total port storage capacity of fuel oil, [TOTSTOR  in barrels] 3,4

6.  Time of transit in each port, [TIME=.275*D  in hours] 6

7.  Emission factor, [EF in lbs/week-103 gallons] 5

- VOC= 0.005

8.  Weekly activity, WAF=7 days/week

9.  Seasonal adjustment factor, SAF 6a

Summer Season Adjustment Factor		1.54

Fall Season Adjustment Factor			0.82

Winter Season Adjustment Factor		0.82

Spring Season Adjustment Factor			0.82

- Process:

The following equations are used to calculate the emissions (without rule effectiveness) for this source category.

Epa  = ((FUEL*CF/CF2)*((TOTSTOR - DOCSTOR)/TOTSTOR)*(TIME/CF1)*(EF/CF)

Eps  = Epa/AADF*SAF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

CF1	= Conversion factor for units = 168 hrs/week

CF2       = Conversion factor for units = [7 lb/gal] 8

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1.  Ships only enter and leave ports for import/export purposes.

2.  All marine vessel traffic from port freight traffic sheets (see reference 2) constitutes marine vessel import/export  fuel transit activity. 9

3. Kerosene oil marine vessel loading includes kerosene fuel oil loading activities from port freight traffic sheets included in reference 2. 8

4.  Assume that the average travel time in hours for vessels using the port is computed by the equation: t = 0.275*d.6

5.  Assume that the average travel time in hours for vessels traveling outside the port is computed by the equation: t=0.125*d. 6

6.  Assume a valid allocation of  port emissions from fuel oil activities to a municipal basis includes  the subtraction of the fuel oil storage capacity of all docks downstream of advancing shipping (DOCSTOR) from the total fuel oil storage capacity within the port (TOTSTOR) followed by division by TOTSTOR.  After which this amount is multiplied by the equation that converts distance traveled to time where distance is represented by the length in miles a municipality  faces on the waterfront route taken by ships/barges in transit.

7. % emissions to NJ from shipping activity to Ports shared by NJ or New York are allocated as follows for each of these respective ports:11

Upper Bay NY Harbor, NY & NJ                 :  54.3 % port emissions to NJ

Hudson River Channel, NY & NJ                 :  50.0 % port emissions to NJ

New York & New Jersey Channel,NY & NJ:  52.6 % port emissions to NJ

8.  % emissions to NJ from shipping activity to Ports shared by Pennsylannia and New Jersey, ie Delaware River (Phila to sea) and Delaware River (Phila to Trenton), Camden and Trenton is allocated by crediting all northbound shipping, ie upbound or inbound, to NJ12.  Assume emissions allocation to NJ achieved from dividing upbound port S & B by total port S & B shipping, ie (upbd S or upbdB)/(downbd & upbd S + downbd & upbd B). The following allocation of emissions has been achieved for ports on the Delaware river.

Delaware River (Phila to the Sea)=(1313+4361)/11318 = 0.501325

Delaware River (Phila to Trenton)=(122+473)/1178 = 0.5051

Trenton= (1+6)/12 = 0.58333

9. Distinct NJ municipalities located on opposite sides of a waterway receive 50 % of the total emissions determined for that portion of a waterway that each town shares with another town. 6   Specifically, the ports with distinct NJ towns located on opposite sides of their waterway include the ports of: 1) Newark except for the last mile which is entirely within the boundary of the City of Newark 2) Hackensack River 3)Raritan River except for the 1st mile which is entirely within the boundaries of the City of South Amboy 3) Portions of Mantua Creek 4) Portions of Intracoastal Waterway.

10. Transit from Delaware River (Phila to the Sea) includes the Port of Camden. 2  Therefore no adjustment is made to make allowance for a separate Camden calculation based on shipping data to Camden.  Camden is just included as one more dock station on the Delaware River Port (Phila to the Sea).

11. Transit from Delaware River (Phila to Trenton) includes through traffic to Trenton but not the port of Trenton.  The port of Trenton is only a single dock which is the last stop on the River.  Which means it does not receive any through shipping from any other port or upstream docks but only shipping directed to the  Port of Trenton.  For this reason the fuel oil storage capacity of Trenton docks have not been included in the Delaware River (Phila to Trenton) to allocate port emissions to municipality level (see item 5 above).  Moreover, the small amount of ships and barges traveling just to the single dock in the port of Trenton have not been deducted from the total number of shipping indicated for the Delaware River (Phila to Trenton) port which is used to determine NJ and Penn shipping proportions.  Whereas the small amount of shipping traveling to Trenton from the second to last port on the Delaware River is used only to apportion  NJ/Penn traffic from the second to last port in Burlington Twp to the last port on the river in Trenton.  (see item 7 above)

12. Assume that all shipping traveling to ports upstream of other designated ports must first travel through the waterways of the downstream port.  This requires that emissions caused by this through travel have to be included with the regular emissions associated with travel in the designated port.

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:   None

- Double Counting:

Emissions for this source category may have been reported in the point source inventory, however, based on the descriptions given by the point source facilities it cannot be determined.  Therefore, to be conservative, no adjustment of the area source emissions was conducted.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

- References:

1.  Procedures for the Preparation of Emission Inventories for Carbon Monoxide and Precursors of Ozone, Vol. I:  General Guidance for Stationary Sources,  United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-450/4-91-016, May 1991

2.  Waterborne Commerce of the United States, Part 1, Waterways and Harbors Atlantic Coast, Calendar Year 2001, US Army Corp of Engineers, Water  Resources Support Center

3. The Ports of New York, NY and NJ and Ports on Long Island, NY, Port Series No. 5,  US Army Corps of Engineers, Revised 1988

4. The Ports of Philadelphia, Pa, Camden, NJ, Wilmington, De, and Ports on Delaware River, Port Series No. 8, US Army Corps of Engineers, Revised 1984

5.  County Maps by Hagstrom or Alfred B. Patton.

6.  Memorandum from James Labanowski of Radian Corporation, Spatial Allocation of Commercial Marine Vessel Emissions for SIP Base Year Inventories, June 9, 1993

6a.  Based on monthly – temperature adjusted true vapor pressures obtained from USEPA Tanks 4.09b model for Newark, New Jersey.

7.  EIIP Volume III, Chapter 12 - Marine Vessel Loading, Ballasting, and Transit, Table 12.4-5, Uncontrolled VOC Emission Factors for Petroleum Carrying Marine Vessels , Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

8. Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, Section 5.2-14, Table 5.2-6. Total Organic Emission Factors for Petroleum Marine Vessel Sources, Section 7.1-83, Table 7.1-2. Properties (MV,WVC, PVA,WL) of Selected Petroleum Liquids, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

9.  EIIP Volume III, Chapter 12 - Marine Vessel Loading, Ballasting, and Transit, Table 12.4-1, Emission Points for Petroleum Vessel Traffic Classifications

10.  EIIP Volume III, Chapter 12 - Marine Vessel Loading, Ballasting, and Transit Table 12.4-1, Product Type Classifications for Common Petroleum Vessel Commodities

11.  Fax from Julie Tucker of Radian Corporation, Spatial Allocation of Waterways Shared by New York and New Jersey, November 13, 1992

12. Memorandum from Whitney Gadsby of the State of Delaware, Department of Natural Resources & Environmental Control, Division of Air & Waste Management, October 9, 1992

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Petroleum Transport, Marine Vessel-Crude Oil Ballasting

- SCC:  2505020030

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category: 1

1.  Surveying individual shipping companies

2.  Fuel transported analysis

The fuel transported analysis is selected because the input data elements are more readily available.

Emissions from crude oil ballasting of marine vessels are calculated using tons of crude oil unloaded at each port, allocated to the county level by the total fuel storage capacity of each docking facility.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  Crude oil unloaded at each port (see item 5 below in Assumptions section), [FUEL in 103 tons] 2

2.  Fuel oil storage capacity of all docks within municipality within port, [MUNSTOR in barrels] 3,4

3.  Total port storage capacity of fuel oil (barrels), [TOTSTOR  in barrels] 3,4

4.  Ballast water - 40% of capacity, [BAL=40%] 5

5.  Emission factor, 	[EF in lbs/103 gallons of ballast water] 6

[PER = portion of EF represents VOC emissions] 7

- VOCS=EF*PER*BAL = 1.1*0.85*0.4 = 0.374

6.  Weekly activity, WAF=7 days/week

7.  Seasonal adjustment factor, SAF 7a

Summer Season Adjustment Factor		1.2

Fall Season Adjustment Factor			0.93

Winter Season Adjustment Factor		0.93

Spring Season Adjustment Factor			0.93

- Process:

The following equations are used to calculate the emissions without control for this source category.

Epa	= (FUEL*CF/CF1)*(MUNSTOR/TOTSTOR)*((BAL*EF*PER)/CF)

Eps	= Epa/AADF*SAF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

CF1       = Conversion factor for units = [7.1 lb/gal] 8

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1.  Only crude oil imported into the port is unloaded at the port

2.  All tankers carry crude oil at maximum capacity

3.  Barges are defined as non-self propelled vessels.

4.  Assume that total vessels are ballasted at a conservative 40 % of capacity. 5

5. Foreign and canadian imports and coastwise, lakewise and internal receipts and  internal inbound (Upbnd & Downbnd),  internal intra (Upbnd & Downbnd), internal intraport and intra-territory receipts from port freight traffic sheets  included in reference 2 constitutes fuel unloading or marine ballasting  activity.9

6. Crude oil marine vessel ballasting includes crude petroleum ballasting activities from port freight traffic sheets  included in reference 2. 10

7.  Assume that the number of ships (S) and barges (B) specified for all ports from trips and drafts of vessels  included in reference 2 includes both upbound and downbound shipping except as noted in item 10 below.

8.  Assume that the municipal fuel oil storage capacity of all docks within a municipality within a port (MUNSTOR) divided by the total fuel oil storage capacity within the port (TOTSTOR) allocates port emissions from fuel oil activities to a municipal basis.

9. % emissions to NJ from shipping activity to Ports shared by NJ or New York are allocated as follows for each of these respective ports 11 :

Upper Bay NY Harbor, NY & NJ                   :  54.3 % port emissions to NJ

Hudson River Channel, NY & NJ                   :  50.0 % port emissions to NJ

New York & New Jersey Channel,NY & NJ  : 52.6 % port emissions to NJ

10.  % emissions to NJ from shipping activity to Ports shared by Pennsylannia and New Jersey, ie Delaware River (Phila to sea), Delaware River (Phila to Trenton), Camden and Trenton is allocated by crediting all northbound (upbound or inbound) ships(S) and barges(B) to NJ.12  Assume this emission allocation is accomplished by including only upbound or inbound shipping in the top portion of the emission equation related to S & B loading but including both upbound and downbound shipping in its bottom portion for every port shared by Pennsylannia and New Jersey, ie upbd S/(downbd S & upbd S + downbd B & upbd B) or upbd B/(downbd S & upbd S + downbd B & upbd B).

11. Fuel and shipping (S & B) amounts for the Port of Camden are subtracted from those amounts obtained for the Port of Delaware River (Phila to the sea)  because this port includes Camden and loading data for Camden has been obtained separately without allocation from this larger port.

12. Assume a weekly activity factor of 7 days per week.

13. Assume a seasonal adjustment factor of 1.

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:   None

- Double Counting:

Emissions for this source category may have been reported in the point source inventory, however, based on the descriptions given by the point source facilities it cannot be determined.  Therefore, to be conservative, no adjustment of the area source emissions was conducted.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

- References:

1.  Procedures for the Preparation of Emission Inventories for Carbon Monoxide and Precursors of Ozone, Vol. I:  General Guidance for Stationary Sources,  United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-450/4-91-016, May 1991

2.  Waterborne Commerce of the United States, Part 1, Waterways and Harbors Atlantic Coast, Calendar Year 2001, US Army Corp of Engineers, Water  Resources Support Center

3. The Ports of New York, NY and NJ and Ports on Long Island, NY, Port Series No. 5,  US Army Corps of Engineers, Revised 1988

4. The Ports of Philadelphia, Pa, Camden, NJ, Wilmington, De, and Ports on Delaware River, Port Series No. 8, US Army Corps of Engineers, Revised 1984

5.  Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, Section  5.2.2.1.2, Ballasting Losses, page 5.2-9, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

6.  EIIP Volume III, Chapter 12 - Marine Vessel Loading, Ballasting, and Transit, Table 12.4-5, Uncontrolled VOC Emission Factors for Petroleum Carrying Marine Vessels , Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

7. Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, Section 5.2-14, Table 5.2-6. Total Organic Emission Factors for Petroleum Marine Vessel Sources, Footnote c

7a.  Based on monthly – temperature adjusted true vapor pressures obtained from USEPA Tanks 4.09b model for Newark, New Jersey.

8.  Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, Section 7.1-83, Table 7.1-2. Properties (MV,WVC, PVA,WL) of Selected Petroleum Liquids

9.  EIIP Volume III, Chapter 12 - Marine Vessel Loading, Ballasting, and Transit, Table 12.4-1, Emission Points for Petroleum Vessel Traffic Classifications

10.  EIIP Volume III, Chapter 12 - Marine Vessel Loading, Ballasting, and Transit Table 12.4-1, Product Type Classifications for Common Petroleum Vessel Commodities

11.  Fax from Julie Tucker of Radian Corporation, Spatial Allocation of Waterways Shared by New York and New Jersey, November 13, 1992

12. Memorandum from Whitney Gadsby of the State of Delaware, Department of Natural Resources & Environmental Control, Division of Air & Waste Management, October 9, 1992

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Petroleum Transport, Marine Vessel-Gasoline Ballasting

- SCC:  2505020120

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category: 1

1.  Surveying individual shipping companies

2.  Fuel transported analysis

The fuel transported analysis is selected because the input data elements are more readily available.

Emissions from gasoline ballasting of marine vessels are calculated using tons of gasoline unloaded at each port, allocated to the county level by the total fuel storage capacity of each docking facility.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  Gasoline unloaded at each port (see item 5 below in assumption section), [FUEL in 103 tons] 2

2.  Fuel oil storage capacity of all docks within municipality within port,[MUNSTOR in barrels] 3,4

3.  Total port storage capacity of fuel oil (barrels), [TOTSTOR in barrels] 3,4

4.  Ballast water - 40% of capacity, [BAL=40%] 5

5.  Emission factor, [EF in lbs/103 gallons of ballast water] 6

- VOC=EF*BAL = 0.8*0.4 = 0.32

6.  Weekly activity, WAF=7 days/week

7.  Seasonal adjustment factor, SAF 6a

Summer Season Adjustment Factor		0.82

Fall Season Adjustment Factor			1.06

Winter Season Adjustment Factor		1.06

Spring Season Adjustment Factor			1.06

- Process:

The following equations are used to calculate the emissions with  control  for this source category.

Epa	= (FUEL*CF/CF1)*(MUNSTOR/TOTSTOR)*BAL*EF*( 1 - (RP*RE*CE) )/CF

Eps	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

CF1  	= Conversion factor for units = [5.6 lb/gal] 7

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1.  Only gasoline imported into the port is unloaded at the port.

2.  All tankers carry gasoline at maximum capacity.

3.  Barges are defined as non-self propelled vessels.

4.  Assume that total vessels are ballasted at a conservative 40 % of capacity. 5

5. Foreign and canadian imports and coastwise, lakewise and internal receipts and  internal inbound (Upbnd & Downbnd),  internal intra (Upbnd & Downbnd), internal intraport and intra-territory receipts from port freight traffic sheets  included in reference 2 constitutes fuel unloading or marine ballasting  activity. 8

6. gasoline marine vessel ballasting includes gasoline and liquid natural gas ballasting activities from port freight traffic sheets  included in reference 2. 9

7.  Assume that the number of ships (S) and barges (B) specified for all ports from trips and drafts of vessels  included in reference 2 includes both upbound and downbound shipping except as noted in item 10 below.

8. Assume that the municipal fuel oil storage capacity of all docks within a municipality within a port (MUNSTOR) divided by the total fuel oil storage capacity within the port (TOTSTOR) allocates port emissions from fuel oil activities to a municipal basis.

9. % emissions to NJ from shipping activity to Ports shared by NJ or New York are allocated as follows for each of these respective ports 10 :

Upper Bay NY Harbor, NY & NJ                   :  54.3 % port emissions to NJ

Hudson River Channel, NY & NJ                   :  50.0 % port emissions to NJ

New York & New Jersey Channel,NY & NJ  : 52.6 % port emissions to NJ

10.  % emissions to NJ from shipping activity to Ports shared by Pennsylannia and New Jersey, ie Delaware River (Phila to sea), Delaware River (Phila to Trenton), Camden and Trenton is allocated by crediting all northbound (upbound or inbound) ships(S) and barges(B) to NJ.11  Assume this emission allocation is accomplished by including only upbound or inbound shipping in the top portion of the emission equation related to S & B loading but including both upbound and downbound shipping in its bottom portion for every port shared by Pennsylannia and New Jersey, ie upbd S/(downbd S & upbd S + downbd B & upbd B) or upbd B/(downbd S & upbd S + downbd B & upbd B).

11. Fuel and shipping (S & B) amounts for the Port of Camden are subtracted from those amounts obtained for the Port of Delaware River (Phila to the sea)  because this port includes Camden and loading data for Camden has been obtained separately without allocation from this larger port.

12.  Assume that every marine vessel gasoline loading and ballasting activity  installs and operates a control apparatus in accordance with N.J.A.C. 7:27-16.5, Marine tank vessel loading and ballasting operations.

13.  Assume a weekly activity factor of 7 days per week.

14.  Assume a seasonal adjustment factor of 1.

- Rule Effectiveness:

The emissions from this source category are regulated by the following rule:

- New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 16, 7:27-16.5, Marine tank

vessel loading and ballasting operations, June 20, 1994

- Double Counting:

Emissions for this source category may have been reported in the point source inventory, however, based on the descriptions given by the point source facilities it cannot be determined.  Therefore, to be conservative, no adjustment of the area source emissions was conducted.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

- References:

1.  Procedures for the Preparation of Emission Inventories for Carbon Monoxide and Precursors of Ozone, Vol. I:  General Guidance for Stationary Sources,  United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-450/4-91-016, May 1991

2.  Waterborne Commerce of the United States, Part 1, Waterways and Harbors Atlantic Coast, Calendar Year 2001, US Army Corp of Engineers, Water  Resources Support Center

3. The Ports of New York, NY and NJ and Ports on Long Island, NY, Port Series No. 5,  US Army Corps of Engineers, Revised 1988

4. The Ports of Philadelphia, Pa, Camden, NJ, Wilmington, De, and Ports on Delaware River, Port Series No. 8, US Army Corps of Engineers, Revised 1984

5.  Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, Section  5.2.2.1.2, Ballasting Losses, page 5.2-9, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

6.  EIIP Volume III, Chapter 12 - Marine Vessel Loading, Ballasting, and Transit, Table 12.4-5, Uncontrolled VOC Emission Factors for Petroleum Carrying Marine Vessels , Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

6a.  Based on monthly – temperature adjusted true vapor pressures obtained from USEPA Tanks 4.09b model for Newark, New Jersey.

7.  Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, Section 7.1-83, Table 7.1-2. Properties (MV,WVC, PVA,WL) of Selected Petroleum Liquids

8.  EIIP Volume III, Chapter 12 - Marine Vessel Loading, Ballasting, and Transit, Table 12.4-1, Emission Points for Petroleum Vessel Traffic Classifications

9.  EIIP Volume III, Chapter 12 - Marine Vessel Loading, Ballasting, and Transit Table 12.4-1, Product Type Classifications for Common Petroleum Vessel Commodities

10.  Fax from Julie Tucker of Radian Corporation, Spatial Allocation of Waterways Shared by New York and New Jersey, November 13, 1992

11. Memorandum from Whitney Gadsby of the State of Delaware, Department of Natural Resources & Environmental Control, Division of Air & Waste Management, October 9, 1992

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# On-Site Incineration, Industrial

- SCC:  2601000000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC), oxides of nitrogen (NOx), carbon monoxide (CO), sulfur dioxide (SO2), particulate matter less than or equal to 10 microns (PM10), particulate matter less than or equal to 2.5 microns (PM2.5) and lead (Pb).  The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category: 1

1.  Surveying individual facilities for incinerator data.

2.  Estimating solid waste throughput using permit data.

The solid waste estimation methodology using permit data is selected

Emissions from incineraters are calculated using equipment specific estimated waste throughput.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  1992 Area Source incinerator throughput data, WASTE (lbs/hour) 2, 2a

and active incinerator list March 19, 2004

Growth factors = 1, no growth is anticipated for this category 2b

2.   Annual operation in hours, ANNHRS  = 40 hrs/week*52 weeks =  2080 hrs/year

3.  Type of incinerator 2

4.  Emission factors, EF (lb/ton of waste burned) 3

##### Multi Chamber	Single Chamber

VOC:		        3			 15

NOx:	                     3			   2

CO:		      10			 20

SO2:		      2.5			2.5

PM10:		      4.7		            	5.7

PM2.5:		      4.7*26/38= 3.2 3a 	5.7*26/38=  3.9 3a

Lead:		     0 .001814		0.001814

5.  Weekly activity, WAF=7 days/week

6.  Seasonal adjustment factor, SAF=1 4

- Process:

The following equations are used to calculate the emissions  without control  for this source category.

Epa	= ((WASTE*ANNHRS)/CF)*EF/CF

Eps	= Epa*SAF/AADF

Epw	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

Epw	= (tons/day) for a typical winter day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1.  All industrial incinerators are single or multiple chamber design for industrial sources.

2.  Throughput is assumed to be 75% of permit allowables.

3.  All incinerators are assumed to be without controls.

4.  Assume that 1992 APEDS permit data approximates current incinerator activity given that any incinerator  known to have been closed has been deleted.

5. Assume incinerator operates for 40 hours over a 7 day week unless otherwise specified through the conductance of a survey of a specific incinerator.

6.  Assume seasonal adjustment factor of 1

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:

New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 11, N.J.A.C. 7:27-11, New Jersey State Department of Environmental Protection, April 15, 1991

- Double Counting:

Emissions for this source category may have also been reported in the point source inventory. 5  If so the incinerator was deleted from the area source inventory prior to calculation of area source emissions.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

NOx		tons/yr		tons/day		tons/day

CO		tons/yr		tons/day		tons/day

SO2         	tons/yr   	tons/day		tons/day

PM10		tons/yr   	tons/day		tons/day

PM2.5		tons/yr   	tons/day		tons/day

- References:

1. Procedures for Emission Inventory Preparation Vol III:  Area Sources,  September 1981, United States Environmental Protection Agency, Office of  Air Quality Planning and Standards, Research Triangle Park, NC,EPA-450/4-81-026c

2. 1992 APEDS, NJDEP Permit Data

2a.  Active incinerator list, Joann Held, NJDEP, March 19, 2004

2b. The State of New Jersey Department of Environmental Protection, State Implementation Plan (SIP) Revision for the Attainment and Maintenance of the Ozone National Ambient Air Quality Standard (NAAQS) New Jersey 1996 Actual Emission Inventory and Rate of Progress (ROP) Plans for 2002, 2005 and 2007, Appendix II, March 31, 2001

3.  Factor Information Retrieval (FIRE) system Version 6.23, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research  Triangle Park, NC, SCC 5-02-001-01 (Solid Waste, Industrial, Multiple Chamber), SCC 5-03-001-02 (Solid Waste, Industrial, Single Chamber)

3a.  Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, Appendix B.1, Section 2.1, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

4. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction,  Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c

5. NJ Point Source Emission Inventory for 2002, New Jersey Department of Environmental Protection

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# On-Site Incineration, Apartment-Solid Waste Incinerator

- SCC:  2601000000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC), oxides of nitrogen (NOx), carbon monoxide (CO), sulfur dioxide (SO2), particulate matter less than or equal to 10 microns (PM10), particulate matter less than or equal to 2.5 microns (PM2.5) and lead (Pb).  The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category: 1

1.  Surveying individual facilities for incinerator data.

2.  Estimating solid waste throughput using permit data.

The solid waste estimation methodology using permit data is selected

Emissions from incineraters are calculated using equipment specific estimated waste throughput.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  1992 Area Source incinerator throughput, WASTE (lbs/hr) 2

and active incinerator list March 19, 2004 2a

Growth factors = 1, no growth is anticipated for this category 2b

2.   Annual operation in hours, ANNHRS  = 40 hrs/week*52 weeks =  2080 hrs/year

3.  Type of incinerator 2

4.  Emission factors, EF (lb/ton of waste burned) 3

Solid Waste	Apartment Waste

VOC:		        3			 15

NOx:	                     3			   3

CO:		      10			 20

SO2:		      2.5			0.5

PM10:		      4.7 	            		11.5

PM2.5:		      4.7*26/38= 3.2 3a 	11.5*26/38=  7.8 3a

Pb:		    0.12 4                        -------

5.  Weekly activity, WAF=7 days/week

6.  Seasonal adjustment factor, SAF=1 4

- Process:

The following equations are used to calculate the emissions without control for this source category.

Epa	= ( EF*(WASTE*ANNHRS)/CF) )/CF

Eps	= Epa*SAF/AADF

Epw	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

Epw	= (tons/day) for a typical winter day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1.  All municipal solid waste incinerators represent  multiple chamber design for the burning of municipal solid waste.

2.  All apartment incinerators are flue fed without afterburners for the burning of residential waste.

3.  All incinerators are assumed to be without controls.

4.  Assume that 1992 APEDS permit data approximates current incinerator activity given that any incinerator  known to have been closed has been deleted.

5.   Assume incinerator operates for 40 hours over a 7 day week unless otherwise specified through the conductance of a survey of a specific incinerator.

6.  Throughput is assumed to be 75% of permit allowables.

7.  Municipal solid waste combustion is similar to commercial refuse combustion.

8.  Assume seasonal adjustment factor of 1

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:

New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 11, N.J.A.C. 7:27-11, New Jersey State Department of Environmental Protection, April 15, 1991

- Double Counting:

Emissions for this source category may have also been reported in the point source inventory.5  If so the incinerator was deleted from the area source inventory prior to calcualtion of area source emissions.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

NOx		tons/yr		tons/day		tons/day

CO		tons/yr		tons/day		tons/day

SO2         	tons/yr   	tons/day		tons/day

PM10		tons/yr   	tons/day		tons/day

PM2.5		tons/yr   	tons/day		tons/day

- References:

1. Procedures for Emission Inventory Preparation Vol III:  Area Sources,  September 1981, United States Environmental Protection Agency, Office of  Air Quality Planning and Standards, Research Triangle Park, NC,

EPA-450/4-81-026c

2. 1992  APEDS, NJDEP Permit Data

2a.  Active incinerator list, Joann Held, NJDEP, March 19, 2004

2b. The State of New Jersey Department of Environmental Protection, State Implementation Plan (SIP) Revision for the Attainment and Maintenance of the Ozone National Ambient Air Quality Standard (NAAQS) New Jersey 1996 Actual Emission Inventory and Rate of Progress (ROP) Plans for 2002, 2005 and 2007, Appendix II, March 31, 2001

3. Factor Information Retrieval (FIRE) system Version 6.23, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research   Triangle Park, NC, SCC 5-02-001-01 (Solid Waste, Commercial/Institutional, Multiple Chamber) SCC 5-02-003-01 (Solid Waste, Commercial/Institutional, Apartment Waste, Flue Fed)

3a.  Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, Appendix B.1, Section 2.1, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

4. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction,  Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c

5.  NJ Point Source Emission Inventory for 2002, New Jersey Department of Environmental Protection

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# On-Site Incineration, Pathological Incinerator

- SCC:  2601000000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC), oxides of nitrogen (NOx), carbon monoxide (CO), sulfur dioxide (SO2), particulate matter less than or equal to 10 microns (PM10), particulate matter less than or equal to 2.5 microns (PM2.5) and lead (Pb).  The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category: 1

1.  Surveying individual facilities for incinerator data.

2.  Estimating solid waste throughput using permit data.

The solid waste estimation methodology using permit data is selected

Emissions from incineraters are calculated using equipment specific estimated waste throughput.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  1992 Area Source incinerator throughput, WASTE (lbs/hr) 2

and active incinerator list March 19, 2004 3

Growth factors = 1, no growth is anticipated for this category 3a

2.   Annual operation in hours, ANNHRS  = 40 hrs/week*52 weeks =  2080 hrs/year

3.  Type of incinerator 2

4.  Emission factors, EF (lb/ton of waste burned)

- VOC:    	0.3 4

NOx:     	3.56 5

CO:      	2.95 5

SO2:    	2.17 5

PM-10:	3.04 6

PM-2.5:	2.02 6

Lead:      0.0728

5.  Weekly activity, WAF=7 days/week

6.  Seasonal adjustment factor, SAF=1 7

- Process:

The following equations are used to calculate the emissions without control for this source category.

Epa	= ( EF*(WASTE*ANNHRS)/CF) )/CF

Eps	= Epa*SAF/AADF

Epw	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

Epw	= (tons/day) for a typical winter day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1. All pathological incinerators represent crematory or pathological waste incinerators.

2. All incinerators are assumed to be without controls.

3. Throughput is assumed to be 75% of permit allowables.

4. Assume incinerator operates for 40 hours over a 7 day week unless otherwise specified through the conductance of a survey of a specific incinerator.

5. Assume that 1992 APEDS permit data approximates current incinerator activity given that any incinerator  known to have been closed has been deleted.

6. Assume seasonal adjustment factor of 1

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:

New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 11, N.J.A.C. 7:27-11, New Jersey State Department of Environmental Protection, April 15, 1991

- Double Counting:

Emissions for this source category may have also been reported in the point source inventory.8  If so the incinerator was deleted from the area source inventory prior to calculation of area source emissions.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

NOx		tons/yr		tons/day		tons/day

CO		tons/yr		tons/day		tons/day

SO2         	tons/yr   	tons/day		tons/day

PM10		tons/yr   	tons/day		tons/day

PM2.5		tons/yr   	tons/day		tons/day

- References:

1. Procedures for Emission Inventory Preparation Vol III:  Area Sources,  September 1981, United States Environmental Protection Agency, Office of  Air Quality Planning and Standards, Research Triangle Park, NC,

EPA-450/4-81-026c

2.  1992  APEDS, NJDEP Permit Data

3.  Active incinerator list, Joann Held, NJDEP, March 19, 2004

3a. The State of New Jersey Department of Environmental Protection, State Implementation Plan (SIP) Revision for the Attainment and Maintenance of the Ozone National Ambient Air Quality Standard (NAAQS) New Jersey 1996 Actual Emission Inventory and Rate of Progress (ROP) Plans for 2002, 2005 and 2007, Appendix II, March 31, 2001

4. Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC, Table 2.3-2, Uncontrolled Emission Factor

5. Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC, Table  2.3-1, Uncontrolled Emission Factor

6.  Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC, Table  2.3-2, Uncontrolled Emission Factor and TAble 2.3-15 Uncontrolled

7. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction,  Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c

8.  NJ Point Source Emission Inventory for 2002, New Jersey Department of Environmental Protection

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# On-Site Incineration, Sewage Sludge

- SCC:  2601000000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC), oxides of nitrogen (NOx), carbon monoxide (CO), sulfur dioxide (SO2), particulate matter less than or equal to 10 microns (PM10), particulate matter less than or equal to 2.5 microns (PM2.5) and lead (Pb).  The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category: 1

1.  Surveying individual facilities for incinerator data.

2.  Estimating solid waste throughput using permit data.

The solid waste estimation methodology using permit data is selected

Emissions from incineraters are calculated using equipment specific estimated waste throughput and permit allowable pollutant emissions.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  Incinerator throughput, WASTE (tons/day)

and permit allowable pollutant emissions 2, 3, 4, 5

Growth factors = 1, no growth is anticipated for this category 5a

2.  Type of incinerator 2

3.  Annual operation in hours, ANNHRS  =  4368  hrs/year 6

4.  Emission factors, EF (lb/ton of waste burned)

- VOC:    	1.7 7

NOx:     	5.0 7

CO:      	31 7

SO2:    	28 7

PM-10:	8.2 8

PM-2.5:	2.2 8

5.  Weekly activity, WAF=7 days/week

6.  Seasonal adjustment factor, SAF=1 9

- Process:

The following equations are used to calculate the emissions without control  for this source category.

Epa	=    ( EF*(WASTE*ANNHRS)/CF) )/CF

Eps	= Epa*SAF/AADF

Epw	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

Epw	= (tons/day) for a typical winter day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1.  All incinerators are assumed to be without controls.

2.  Throughput is assumed to be 75% of permit allowables.

3.  Assume weekly activity factor of 7 days per week

4.  Assume seasonal adjustment factor of 1

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:

New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 11, N.J.A.C. 7:27-11, New Jersey State Department of Environmental Protection, April 15, 1991

- Double Counting:

Emissions for this source category may have also been reported in the point source inventory.10  If so the incinerator was deleted from the area source inventory prior to calculation of area source emissions.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

NOx		tons/yr		tons/day		tons/day

CO		tons/yr		tons/day		tons/day

SO2         	tons/yr   	tons/day		tons/day

PM10		tons/yr   	tons/day		tons/day

PM2.5		tons/yr   	tons/day		tons/day

- References:

1.  Procedures for Emission Inventory Preparation Vol III:  Area Sources, September 1981, United States Environmental Protection Agency, Office of   Air Quality Planning and Standards, Research Triangle Park, NC, EPA-450/4-81-026c

2.  1992 APEDS, NJDEP Permit Data

3.  Active incinerator list, Joann Held, NJDEP, March 19, 2004

4.  1997 Permit Data, NJDEP

5.  2004 Permit Data, NJDEP

5a. The State of New Jersey Department of Environmental Protection, State Implementation Plan (SIP) Revision for the Attainment and Maintenance of the Ozone National Ambient Air Quality Standard (NAAQS) New Jersey 1996 Actual Emission Inventory and Rate of Progress (ROP) Plans for 2002, 2005 and 2007, Appendix II, March 31, 2001

6. Memo to File on telephone discussion with operating officer for Pequannock, Lincoln Park and Fairfield Sewage Sludge Incinerator,  October 4, 1999

7. Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC, Table 2.2-1, Uncontrolled Emission Factor

8.  Factor Information Retrieval (FIRE) system Version 6.23, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC, SCC 5-01-005-15

9. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction,  Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c

10.  NJ Point Source Emission Inventory for 2002, New Jersey Department of Environmental Protection

Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Open Burning

- SCC:  2610000100, 2610000400, 2610030000, 2610040400

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC), oxides of nitrogen (NOx), carbon monoxide (CO), sulfur dioxide (SO2), particulate matter less than or equal to 10 microns (PM10) and particulate matter less than or equal to 2.5 microns (PM2.5).  The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodology is used for this source category: 1

- 1.  MARAMA survey and calculations

Emissions from open burning are calculated using MARAMA's state specific survey and calculations.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  Emission calculated by Pechan for MARAMA 1

2.  Emission factors, EF (lb/ton burned) 1

Residential	Residential	Residential Municipal	Municpal Yard

Leaf		Brush		Solid Waste		Waste

VOC		28		19		19			8.56

NOx		6.2		5		5			6

CO		112		140		140			85

SO2		0.76		1.66		1.66			1

PM10		22		19.73		19.73			38

PM2.5		22		1.26		1.26			34.8

3.  Weekly activity, WAF=7 days/week

4.  Seasonal adjustment factors, SAF 1

Residential	Residential	Residential Municipal	Municpal Yard

Leaf		Brush		Solid Waste		Waste

Summer		0		0.24		1			0.24

Fall		4		1.12		1			1.12

Winter		0		0.80		1			0.80

Spring		0		1.84		1			1.84

- Process:

The following equations are used to calculate the emissions without control for this source category.

Epa	= MARAMA 1

Eps	= Epa*SAF/AADF

Epw	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

Epw	= (tons/day) for a typical winter day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:

New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 2, N.J.A.C. 7:27-2, New Jersey State Department of Environmental Protection, June 20, 1994

- Double Counting:

Emissions for this source category have not been reported in the point source inventory, therefore no adjustment of the area source emissions was required.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

NOx		tons/yr		tons/day		tons/day

CO		tons/yr		tons/day		tons/day

SO2         	tons/yr   	tons/day		tons/day

PM10		tons/yr   	tons/day		tons/day

PM2.5		tons/yr   	tons/day		tons/day

- References:

- 1.  MANE-VU Open Burning in Residential Areas, Emissions Inventory Development Report, E.H. Pechan & Associates, for Mid-Atlantic/Northeast Visibility Union (MANE-VU) organized by the Mid-Atlantic Regional Air Management Association (MARAMA)., January 31, 2004.

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Landfills

- SCC:  2620030000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category: 1, 12

1.  Required Reporting

2.  Guidelines for using assumptions with landfill data

3.  Regression model

4.  Population-based waste generation factor

The regression model is selected because it is considered more accurate than the population-based waste generation factor.  Emissions from landfills are calculated using landfill specific estimated emissions.

The USEPA Landfil2 and the updated Landgems 3.02 models were both used to calculate estimated emissions in tons/yr out to 2017.   Originally the USEPA Landfil2 model was applied to calculate emissions for 373 landfills. The Landgems 3.02 model was applied to the 17 major municipal solid waste landfills to determine their actual emissions with this updated model.  In addition, a sensitivity analysis was conducted to compare results between the two models. This sensitivity analysis determined that the updated Landgems 3.02 model increased emissions by approximately 8.95 percent.  Accordingly a scaled factor was applied to the balance of the landfill emissions by this amount to approximate application of the updated Landgems 3.02 model to the rest of the landfills.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category:

1.  Year the landfill was opened 2,3,4

2.  Year landfill was or will be closed 2,3,4

3.  The total capacity of the landfill 2,3,4,12

4.  Type of wastes in the landfill 2,3,4

5.  Nonmethane organic compounds concentration (NMOC) 5,6, 11

6.  Methane generation rate constant (k) 5,6,11

7.  Potential methane generation capacity (Lo) 5,6,11

8.  Discounted non-reactive VOC emissions from total NMHC landfill emissions 5,6

The following additional data is required to calculate emissions for this source category:

9.   Weekly activity, WAF=7 days/week

10. Seasonal adjustment factor, SAF=1 6a

- Process:

The following equations are used to calculate emissions for this source category:

Seasonal emissions in tons/day were calculated as follows:

Eps	= Epa*SAF/AADF

Epw	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

Epw	= (tons/day) for a typical winter day emission of pollutant

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1.  The 1993 annual operating tonnage (tons) is assumed to be representative of all previous years of landfill operations.

2.  The permit issuance date was assumed to be the date of opening.

3.  An average lifespan of 20 years will be used if no opening date data is available. 8,9

4.  NMOC is assumed to be 2,400 ppmv as hexane if the landfill received large amounts  of industrial wastes.  Otherwise NMOC is 600 ppmv as hexane. 5,6,11,12

5.  k is assumed to be 0.04/yr because New Jersey has normal to above average precipitation 5,6,111,12

6.  Lo is assumed to be 100 m3/ug 5,6, 11, 12

7.  Assume that active VOC emissions represent only 39 percent (%) of the total NMHC emissions from municipal waste landfills and 85 percent (%) of all NMHC emissions from industrial or hazardous waste landfills which accepted liquid solvents. 5,6

8.  Assume that landfill gas consists of 50 % by volume CO2 and 50 % by volume CH4.5,6, 11, 12

9. Assume that the landfills which accepted significant quantities of liquid solvents are included in the NJDEP document "Publicly Funded Cleanup Site Status Report" or indicated to have done so by phone survey conducted by NJDEP.  Otherwise all other landfills are considered to not have accepted significant quantities of liquid solvents.4,7

10.  Assume the  default density factors of 20,000 tons/acre or  1160 pounds  per cubic yards  if only landfill surface area can be determined. 8,9,12

11.  Assume that the average annual acceptance rate in megagrams per year represents the total capacity of the landfill divided by the number of years the landfill operated.

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:

New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 8, N.J.A.C. 7:27-8, Permits and Certificates for Minor Facilities, New Jersey State Department of Environmental Protection

- Double Counting:

Emissions for this source category have also been reported in the point source inventory. 10  If so the Landfill was deleted from the area source inventory prior to calculation of area source emissions.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

- References:

1. Procedures for the Preparation of Emission Inventories for Carbon Monoxideand Precursors of Ozone, Vol. I:  General Guidance for Stationary Sources, May 1991, United States Environmental Protection Agency, Office of Air  Quality Planning and Standards, Research Triangle Park, NC,   EPA-450/4-91-016

2. Annual Operating Statements for a Solid Waste Facility, Bureau of Registration, New Jersey Department of Environmental Protection

3.  Landfill Inventory, Bureau of Landfill Engineering, New Jersey Department of Environmental Protection and Energy

4.  Tabulation of  major solid waste landfill design parameters obtained from phone survey and  information from annual operating statements and Landfill inventory, 1997

5.  Landfill Air Emissions Estimation Model User's Manual, Control Technology  Center, Air and Energy Engineering Research Laboratory, Office of Research and Development, United States Environmental Protection Agency, Research Triangle Park, NC, 1997

6. Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition,, Section 2.4,  January 1997, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

6a. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction,  Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c

7. Revision to the New Jersey 15 Percent Rate of Progress Plan, Appendix II:Landfill Emission Inventory Updates, February 8, 1999, The State of New Jersey, Department of Environmental Protection

8. Memorandum from Titus Magnanao, Bureau of Landfill Engineering, New Jersey, Department of Environmental Protection and Energy, June 29, 1992

9. Update to Bureau of Landfill Engineering Memorandum

10.  NJ Point Source Emission Inventory for 2002, New Jersey Department of Environmental Protection

11.  Landgems 3.02 Model, USEPA, May 2005

12.  EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 15 - Landfills,  Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c, January 2001

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Industrial Treatment Works (ITWS)

- SCC:  2630010000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category: 1

1.  Surveying individual facilities

2.  Using the EPA Surface Impoundment Modeling System (SIMS)

3.  Using the EPA default emission factor

The SIMS model is selected because 1) the input parameters are readily available and 2) the SIMS model is considered more accurate than the EPA default emission factor.

Emissions from industrial treatment works are calculated using ITWS specific estimated emissions.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category using the SIMS model.

1.  Wastewater flowrates for each facility, FLOW (meters3/second) 2

2.  Type of industrial process at each facility 2,3

3.  ITW classification 2,4

4.  Surface area of the impoundment (see assumption 2 below)

5.  Water depth of the impoundment (see assumption 2 below)

6.  Industrial pollutants entering the ITW 2

The following additional data is required to calculate emissions for this source category:

7.   Weekly activity, WAF=7 days/week

8. Seasonal adjustment factor, SAF=1

9. Growth factors 4a

1996-1999	1999-2002

Growth factors, Atlantic City Air Quality Control Region		1.0377		1.055

Growth factors, Northern New Jersey Air Quality Control Region	1.027		1.0472

Growth factors, Southern New Jersey Air Quality Control Region	1.0236		1.0434

- Process:

The following equations are used to calculate emissions for this source category:

The USEPA SIMS model was used to calculate estimated 1996 emissions in tons/yr. The SIMS model uses Henry's Law in determining volatilization of the influent. 3   The emissions were then grown to 2002 as follows:

= (1996 estimated emissions) * (county growth factors 1996-1999)* (county growth factors 1999-2002)

Seasonal emissions in tons/day were calculated as follows:

Eps	= Epa*SAF/AADF

Epw	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

Epw	= (tons/day) for a typical winter day emission of pollutant

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1.  All ITWS are classified as either mechanically aerated with no  biodegradation or mechanically aerated, activated sludge.  This was dependent of the flowrate.  If the flowrate was greater than 5 million gallons per day, the ITW was classified as mechanically aerated and activated sludge.  If the flowrate was less than or equal to 5 million  gallons per day, the ITW was classified as mechanically aerated with no biodegradation. 5,6,7

2.  An educated guess was used to determine the initial surface area of the  impoundment.  This figure was then adjusted in order to obtain a reasonable depth of 2-5 meters.

3.  Assumed industrial wastewater factor of 100%

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:

New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 8, N.J.A.C. 7:27-8, Permits and Certificates for Minor Facilities, New Jersey State Department of Environmental Protection

- Double Counting:

Emissions for this source category have also been reported in the point source inventory. 8  If so the  ITW was deleted from the area source inventory prior to calculation of area source emissions.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

- References:

1.  Procedures for the Preparation of Emission Inventories for Carbon Monoxide and Precursors of Ozone, Vol. I:  General Guidance for Stationary Sources,  May 1991, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-450/4-91-016

2.  New Jersey Pollution Discharge Elimination System, Bureau of Water Resources, New Jersey Department of Environmental Protection and Energy, Trenton, NJ

3.  Background Document for the Surface Impoundment Modeling System (SIMS) Version 2.0", United States Environmental Protection Agency, Office of Air  Quality Planning and Standards, Research Triangle Park, NC, EPA-450/4-91-016

4.  Memorandum from RADIAN Corp to SIP Inventory Preparers and EPA regions concerning Inventory Guidance and Evaluation Section, April 24, 1992

4a. The State of New Jersey Department of Environmental Protection, State Implementation Plan (SIP) Revision for the Attainment and Maintenance of the Ozone National Ambient Air Quality Standard (NAAQS) New Jersey 1996 Actual Emission Inventory and Rate of Progress (ROP) Plans for 2002, 2005 and 2007, Appendix II, March 31, 2001

5.  Telephone conversation with Lucy Adams, Radian Corporation, Research Triangle Park, 1992

6.  Telephone conversation with Joe Wang, Water Technical Programs, NJDEP, 1992

7.   Memorandum to File concerning conversation with Chris   of Bureau of Construction Permits

8.  NJ Point Source Emission Inventory for 2002, New Jersey Department of Environmental Protection

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Public Owned Treatment Works (POTWS)

- SCC:  2630020000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category: 1

1.  Surveying individual facilities for inputing data into the EPA Surface  Impoundment Modeling System 	(SIMS)

2.  Using the EPA default emission factor

Surveying individual facilities and using the SIMS model is selected because 1) the input parameters are available and 2) the SIMS model is considered more accurate.

Emissions from public owned treatment works are calculated using POTW specific estimated emissions.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category us the USEPA SIMS model:

- Actual values for Facilities with activated sludge treatment systems

(West NY, Camden-Delaware,Two Bridges, Gloucester, Rockaway, Parsippany-Troy Hills (Hanover) POTW)

1.  93 Annual Wastewater flowrates for each facility, FLOW (meters3/second) 2,3

2.  Industrial wastewater factor IND (%) 2,3,4

3.  POTW classification 2,3,5

4.  Surface area of the impoundment (m2) 3

5.  Water depth of the impoundment (m) 3

6.  Industrial pollutants entering the POTW 2,3

7.  Residence time (sec) 3

8.  Biological Oxygen Content (BOD) 3

9.  Air rate (m3/sec) of dissolved oxygen/horsepower of impellers 3

Default values for facilities without activated sludge systems for which approximation of activated sludge systems will be made:  (Hamilton, Trenton, Morris POTW)

10.  93 Annual Wastewater flowrates for each facility, FLOW (meters3/second) 2,3

11.  Industrial wastewater factor IND (%) 2,3,4

12.  POTW classification 2,3,5

13.  Surface area of the impoundment (m2)= 8506*FLOW 6

14.  Water depth of the impoundment (m)= 3.81 meters 6

15.  Industrial pollutants entering the POTW 2,3

16.  Residence time (sec)= 6 hrs 6

17.  Biological Oxygen Content (BOD)=2500 6

18.  horsepower of impellers = sims default

The following additional data is required to calculate emissions for this source category:

19. Weekly activity, WAF=7 days/week

20. Seasonal adjustment factors, SAF 7

Summer Season Adjustment Factor		1.4

Fall Season Adjustment Factor			0.87

Winter Season Adjustment Factor		0.87

Spring Season Adjustment Factor			0.87

21. Growth factors 8

1996-1999	1999-2002

Growth factors, Northern New Jersey Air Quality Control Region	1.0247		1.0222

Growth factors, Southern New Jersey Air Quality Control Region	1.0219		1.0215

- Process:

The following equations are used to calculate emissions for this source category:

The USEPA SIMS model was used to calculate estimated 1996 emissions in tons/yr. The SIMS model uses Henry's Law in determining volatilization of the influent. 4   The emissions were then grown to 2002 as follows:

= (1996 estimated emissions) * (county growth factors 1996-1999)* (county growth factors 1999-2002)

Seasonal emissions in tons/day were calculated as follows:

Eps	= Epa*SAF/AADF

Epw	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

Epw	= (tons/day) for a typical winter day emission of pollutant

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1.  Emissions from facilities that use trickling filters, rotary biological contacters or treatment other than mechaniclly aerated/dissolved oxygen POTW will be estimated by classifying these facilities as a mechanically aerated, activated sludge with default factors for BOD, height, and surface area.

2.  An educated guess was used to determine default values for the initial surface area, height and BOD of those facilities for which an approximation of an activated sludge system was used to estimate emissions.

3.  Assume that the annual report and survey conducted for 1993 operations approximate operations conducted in 1996.

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:

New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 8, N.J.A.C. 7:27-8, Permits and Certificates for Minor Facilities, New Jersey State Department of Environmental Protection

- Double Counting:

Emissions for this source category have also been reported in the point source inventory. 9  If so the POTW was deleted from the area source inventory prior to calculation of area source emissions.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

- References:

1.  Procedures for the Preparation of Emission Inventories for Carbon Monoxide and Precursors of Ozone, Vol. I:  General Guidance for Stationary Sources, May 1991, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,   EPA-450/4-91-016

2.  1993 Facility Annual Report,  Bureau of Water  Resources, New Jersey Department of Environmental Protection and Energy,  Trenton, NJ

3.  Tabulation of POTW design parameters obtained from phone survey and flow information from annual reports, 1994

4.  Background Document for the Surface Impoundment Modeling System (SIMS) Version 2.0", United States Environmental Protection Agency, Office of Air  Quality Planning and Standards, Research Triangle Park, NC, EPA-450/4-91-016

5.   Memorandum from RADIAN Corp to SIP Inventory Preparers and EPA regions concerning Inventory Guidance and Evaluation Section, April 24, 1992

6.   Memorandum to File concerning conversation with Chris of Bureau of Construction Permits

7. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction,  Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c

8. The State of New Jersey Department of Environmental Protection, State Implementation Plan (SIP) Revision for the Attainment and Maintenance of the Ozone National Ambient Air Quality Standard (NAAQS) New Jersey 1996 Actual Emission Inventory and Rate of Progress (ROP) Plans for 2002, 2005 and 2007, Appendix II, March 31, 2001

9.  NJ Point Source Emission Inventory for 2002, New Jersey Department of Environmental Protection

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Leaking Underground Storage Tanks

- SCC:  2601000000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category: 1

1. Apply a default emission factor to the total number of underground storage tank remediations which involved soil removal.

2. Survey all the underground storage tank remediations which involved soil removal to determine the quantity of soil removed and the gasoline concentration in the soil.

Application of a default value to the total number of leaking underground storage tank remediations (LUST) which involved soil removal was used because that information is more readily available.

Emissions from LUSTs are calculated using estimated number of LUST remediations which involve soil, allocated to the county level using vehicle miles travelled.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

- 1.   Annual number of  LUST remediations which involve soil, USTOT 2

USTOT = 478

2.   Emission factors, EF (lbs/remediation-day) 1

- VOC = 28 lbs VOC/remediation-day

3.  Remediation-day, RD  = 1.5 2

4.  State and County Daily Vehicle Miles Travelled, SVMT, CVMT 3

5.  Weekly activity, WAF=7 days/week

6.  Seasonal adjustment factor, SAF=1

7. Growth factors = 1, no growth is anticipated for this category 4

- Process:

The following equations were used to calculate the 1996 emissions without control  for this source category:

Epa	= USTOT*EF*RD*((CVMT)/(SVMT))/CF

The emissions were then grown to 2002 as follows:

= (1996 estimated emissions) * (county growth factors 1996-1999)* (county growth factors 1999-2002)

Seasonal emissions in tons/day were calculated as follows:

Eps	= Epa*SAF/AADF

Epw	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	 = (tons/day) for a typical summer day emission of pollutant

Epw	 = (tons/day) for a typical winter day emission of pollutant

CF	 = Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1. The default emission rate represents events with 50 yd3 of soil removed and 10,000 ppmw of total petroleum       hydrocarbons (TPH) or 500 yd3 of soil removed and 1000 ppmw of TPH. 1

2. The total amount of VOC found in contaminated soil evolves during  excavation activities. 2

3. The average excavation activities will be completed within 1.5 days. 2

4. Assume weekly activity factor of  7 days per week

5. Assume seasonal adjustment factor of  1

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:   None

- Double Counting:

- Emissions for this source category have not been reported in the point source inventory.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

- References:

1. Memorandum from RADIAN Corp to SIP Inventory  Preparers and EPA regions concerning Inventory Guidance and Evaluation Section, May 6, 1992.

2. Memo to File  on telephone conversation with Joseph Miller of the Bureau of Leaking Underground Storage Tanks (LUST), May  28, 1999.

3. New Jersey Department of Environmental Protection 1996 Mobile Inventory

4. The State of New Jersey Department of Environmental Protection, State Implementation Plan (SIP) Revision for the Attainment and Maintenance of the Ozone National Ambient Air Quality Standard (NAAQS) New Jersey 1996 Actual Emission Inventory and Rate of Progress (ROP) Plans for 2002, 2005 and 2007, Appendix II, March 31, 2001

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Agricultural Field Burning, Land Clearing

- SCC:  2801500000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC),  carbon monoxide (CO), particulate matter less than or equal to 10 microns (PM10) and particulate matter less than or equal to 2.5 microns (PM2.5). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category: 1

1.  Contacting the Bureau of Forest Fire Management for information concerning agricultural field burning

This is the only recommended method and was employed in developing the inventory.

Emissions from agricultural field burning are calculated using the number of county and section specific permits.

- Required Input Parameters:

- The following input data was obtained to calculate emissions for this source category:

1. Total agricultural field burning land clearing permits issued by county for New Jersey Divisions A and B and

by section for New Jersey Division C,  CPER 2,3,4

2. Average Acreage per permit (average acres/permit):  ACRE 4

Division A,B,C:  7.5 acres per permit

3.   Fuel loading factors (ton matter burned/permit):  FL 5

Division A   :    20.0 tons per acre

Division B,C:   13.5 tons per acre

4.  Percentage of each municipality in each NJDEP Forestry Section 6

5.  Area of each municipality, square miles 7

6.  Emission Factors, EF, (lbs/ton burned) 8

VOC=Non Methane Hydrocarbons (NMHC):	19

CO:	140

PM10:	17

PM2.5:	17

7.  Weekly activity, WAF=7 days/week 9

8.  Seasonal adjustment factors, SAF 4

Summer Season Adjustment Factor		0.69

Fall Season Adjustment Factor			0.02

Winter Season Adjustment Factor		2.62

Spring Season Adjustment Factor			0.67

- Process:

- The following equations are used to calculate the emissions without control for this source category.

STEP 1:  For Division C calculate the number of  permits  issued at the municipal level based on the # permits in each section and the % of each municipality in each section.

STEP 2:  For Division C total the number of permits at the municipal level to obtain the number of permits at the county level.

STEP 3: For Divisions A, B and C, calculate the amount of agricultural land clearing material burned ALCTON (tons material burned) :

- ALCTON = CPER*ACRE*FL

- STEP 4: Calculate Emissions:

Epa        =  EF*ALCTON

Eps     =  Epa/AADF*SAF

Epw    =  Epa/AADF*SAF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

Epw	= (tons/day) for a typical winter day emission of pollutant

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1. Acreage of agricultural fields burned in each  county based on the number of agricultural field burning permits issued  in each county and represents the best available estimate of acreage burned per permit that can be provided by the NJ Forestry Service.

2. Assume that tons of matter burned per acreage is uniform across the specified division  and represents the best available estimate that can be provided by NJ Forestry Service.

3. Assume particulate matter from most agricultural refuse burning is within the submicrometer size range. 8

4. Assume that the EF for NMHC is equivalent to the EF for VOC.

5. North of the Raritan represents Division A and the Mullica River divides B and C division. 5,6

6. Assume that approximately 5/6 of Somerset County belongs to Division A and the remaining  1/6 belongs

to Division B. 2,3

7. Assume that 1/2 of Mercer County belongs to Division A and the other 1/2 belongs to Division B. 5,6

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:

New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 2, N.J.A.C. 7:27-2, New Jersey State Department of Environmental Protection, June 20, 1994

- Double Counting:

Emissions for this source category have not been reported in the point source inventory, therefore no adjustment of the area source emissions was required.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

CO		tons/yr		tons/day		tons/day

PM10		tons/yr   	tons/day		tons/day

PM2.5		tons/yr   	tons/day		tons/day

- References

1. Procedures for the Preparation of Emission Inventories for Carbon Monoxide and Precursors of Ozone, Vol. I:  General Guidance for Stationary Sources, May 1991, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-450/4-91-016

2.  NJDEP Division A Permit Log for 2002 by County, November 18, 2003

3.  NJDEP Division B Permit Log for 2002 by Township, November 18, 2003

4.  NJDEP Division C Permit Log for 2002 by Section, November 19, 2003

5. Memo to File on Phone Conversations with NJ Forestry Service

6. Administrative Map, State of New Jersey, NJDEP,  Forestry Service, November 1984

7. Estimate of 1996 Total Resident Population and Square Mile Area by Counties and Municipalities,   US Bureau of the Census, Population Data Division, 6/30/99

8. Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC, Table 2.5-5, Unspecified Forest Residue and Note b.

9. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction,  Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Agricultural Field Burning, Herbaceous

- SCC:  2801500000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC),  carbon monoxide (CO), particulate matter less than or equal to 10 microns (PM10) and particulate matter less than or equal to 2.5 microns (PM2.5). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category: 1

1.  Contacting the Bureau of Forest Fire Management for information concerning agricultural field burning

This is the only recommended method and was employed in developing the inventory.  Emissions from agricultural field burning are calculated using the number of county and section specific permits.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category:

1. Total agricultural field burning herbacous permits issued by county for New Jersey Divisions A and B and

by section for New Jersey Division C,  CPER 2,3,4

2. Average Acreage per permit (average acres/permit):  ACRE 4

Division A,B,C:  3 acres per permit

3.   Fuel loading factors (ton matter burned/permit):  FL 5

Division A, B, C:   1.0 tons per acre

4.  Percentage of each municipality in each NJDEP Forestry Section 6

5.  Area of each municipality, square miles 7

6.  Emission Factors, EF, (lbs/ton burned) 8

VOC=Non Methane Hydrocarbons (NMHC):	9

CO:	85

PM10:	15

PM2.5:	15

7.  Weekly activity, WAF=7 days/week 9

8.  Seasonal adjustment factors, SAF 10

Summer Season Adjustment Factor		0.55

Fall Season Adjustment Factor			0.78

Winter Season Adjustment Factor		1.16

Spring Season Adjustment Factor			1.51

- Process:

- The following equations are used to calculate the emissions without control for this source category.

STEP 1:  For Division C calculate the number of  permits  issued at the municipal level based on the # permits in each section and the % of each municipality in each section.

STEP 2:  For Division C total the number of permits at the municipal level to obtain the number of permits at the county level.

STEP 3: For Divisions A, B and C, calculate the amount of agricultural herbacous material burned HERBTON (tons material burned) :

- HERBTON = CPER*ACRE*FL

- STEP 4: Calculate Emissions:

Epa        =  EF*ALCTON

Eps     =  Epa/AADF*SAF

Epw    =  Epa/AADF*SAF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

Epw	= (tons/day) for a typical winter day emission of pollutant

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1. Acreage of agricultural fields burned in each  county based on the number of agricultural field burning permits issued  in each county and represents the best available estimate of acreage burned per permit that can be provided by the NJ Forestry Service.

2. Assume that tons of matter burned per acreage is uniform across the specified division  and represents the best available estimate that can be provided by NJ Forestry Service.

3. Assume particulate matter from most agricultural refuse burning is within the submicrometer size range. 8

4. Assume that the EF for NMHC is equivalent to the EF for VOC.

5. North of the Raritan represents Division A and the Mullica River divides B and C division. 5,6

6. Assume that approximately 5/6 of Somerset County belongs to Division A and the remaining  1/6 belongs

to Division B. 2,3

7. Assume that 1/2 of Mercer County belongs to Division A and the other 1/2 belongs to Division B. 5,6

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:

New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 2, N.J.A.C. 7:27-2, New Jersey State Department of Environmental Protection, June 20, 1994

- Double Counting:

Emissions for this source category have not been reported in the point source inventory, therefore no adjustment of the area source emissions was required.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

CO		tons/yr		tons/day		tons/day

PM10		tons/yr   	tons/day		tons/day

PM2.5		tons/yr   	tons/day		tons/day

- References

1. Procedures for the Preparation of Emission Inventories for Carbon Monoxide and Precursors of Ozone, Vol. I:  General Guidance for Stationary Sources, May 1991, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-450/4-91-016

2.  NJDEP Division A Permit Log for 2002 by County, November 18, 2003

3.  NJDEP Division B Permit Log for 2002 by Township, November 18, 2003

4.  NJDEP Division C Permit Log for 2002 by Section, November 19, 2003

5. Memo to File on Phone Conversations with NJ Forestry Service

6. Administrative Map, State of New Jersey, NJDEP,  Forestry Service, November 1984

7. Estimate of 1996 Total Resident Population and Square Mile Area by Counties and Municipalities,   US Bureau of the Census, Population Data Division, 6/30/99

8. Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC, Table 2.5-5, Unspecified Weeds and Note b.

9. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction,  Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c

10.  NJDEP Division A Fire Call in Log for 2002, November 18, 2003

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Agricultural Field Burning, Infested

- SCC:  2801500000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC),  carbon monoxide (CO), particulate matter less than or equal to 10 microns (PM10) and particulate matter less than or equal to 2.5 microns (PM2.5). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category: 1

1.  Contacting the Bureau of Forest Fire Management for information concerning agricultural field burning.

This is the only recommended method and was employed in developing the inventory.  Emissions from agricultural field burning are calculated using the number of county and section specific permits.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category:

1. Total agricultural field burning infested permits issued by county for New Jersey Divisions A and B and

by section for New Jersey Division C,  CPER 2,3,4

2. Average Acreage per permit (average acres/permit):  ACRE 4

Division A,B,C:  1.5 acres per permit

3.   Fuel loading factors (ton matter burned/permit):  FL 5

Division A, B, C:   2.0 tons per acre

4.  Percentage of each municipality in each NJDEP Forestry Section 6

5.  Area of each municipality, square miles 7

6.  Emission Factors, EF, (lbs/ton burned) 8

VOC=Non Methane Hydrocarbons (NMHC):	18

CO:	117

PM10:	21

PM2.5:	21

7.  Weekly activity, WAF=7 days/week 9

8.  Seasonal adjustment factors, SAF 10

Summer Season Adjustment Factor		0.55

Fall Season Adjustment Factor			0.78

Winter Season Adjustment Factor		1.16

Spring Season Adjustment Factor			1.51

- Process:

- The following equations are used to calculate the emissions without control for this source category.

STEP 1:  For Division C calculate the number of  permits  issued at the municipal level based on the # permits in each section and the % of each municipality in each section.

STEP 2:  For Division C total the number of permits at the municipal level to obtain the number of permits at the county level.

STEP 3: For Divisions A, B and C, calculate the amount of agricultural infested material burned INFTON (tons material burned) :

- INFTON = CPER*ACRE*FL

- STEP 4: Calculate Emissions:

Epa        =  EF*ALCTON

Eps     =  Epa/AADF*SAF

Epw    =  Epa/AADF*SAF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

Epw	= (tons/day) for a typical winter day emission of pollutant

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1. Acreage of agricultural fields burned in each  county based on the number of agricultural field burning permits issued  in each county and represents the best available estimate of acreage burned per permit that can be provided by the NJ Forestry Service.

2. Assume that tons of matter burned per acreage is uniform across the specified division  and represents the best available estimate that can be provided by NJ Forestry Service.

3. Assume particulate matter from most agricultural refuse burning is within the submicrometer size range. 8

4. Assume that the EF for NMHC is equivalent to the EF for VOC.

5. North of the Raritan represents Division A and the Mullica River divides B and C division. 5,6

6. Assume that approximately 5/6 of Somerset County belongs to Division A and the remaining  1/6 belongs

to Division B. 2,3

7. Assume that 1/2 of Mercer County belongs to Division A and the other 1/2 belongs to Division B. 5,6

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:

New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 2, N.J.A.C. 7:27-2, New Jersey State Department of Environmental Protection, June 20, 1994

- Double Counting:

Emissions for this source category have not been reported in the point source inventory, therefore no adjustment of the area source emissions was required.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

CO		tons/yr		tons/day		tons/day

PM10		tons/yr   	tons/day		tons/day

PM2.5		tons/yr   	tons/day		tons/day

- References

1. Procedures for the Preparation of Emission Inventories for Carbon Monoxide and Precursors of Ozone, Vol. I:  General Guidance for Stationary Sources, May 1991, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-450/4-91-016

2.  NJDEP Division A Permit Log for 2002 by County, November 18, 2003

3.  NJDEP Division B Permit Log for 2002 by Township, November 18, 2003

4.  NJDEP Division C Permit Log for 2002 by Section, November 19, 2003

5. Memo to File on Phone Conversations with NJ Forestry Service

6. Administrative Map, State of New Jersey, NJDEP,  Forestry Service, November 1984

7. Estimate of 1996 Total Resident Population and Square Mile Area by Counties and Municipalities,   US Bureau of the Census, Population Data Division, 6/30/99

8. Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC, Table 2.5-5, Unspecified Field Crops and Note b.

- 9. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction,  Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c

- 10.  NJDEP Division A Fire Call in Log for 2002, November 18, 2003

Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Agricultural Field Burning, Orchard

- SCC:  2801500000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC),  carbon monoxide (CO), particulate matter less than or equal to 10 microns (PM10) and particulate matter less than or equal to 2.5 microns (PM2.5). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category: 1

1.  Contacting the Bureau of Forest Fire Management for information concerning agricultural field burning

This is the only recommended method and was employed in developing the inventory.  Emissions from agricultural field burning are calculated using the number of county and section specific permits.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category:

1. Total agricultural field burning orchard permits issued by county for New Jersey Divisions A and B and

by section for New Jersey Division C,  CPER 2,3,4

2. Average Acreage per permit (average acres/permit):  ACRE 4

Division A,B,C:  7.5 acres per permit

3.   Fuel loading factors (ton matter burned/permit):  FL 5

Division A, B, C:   9.0 tons per acre

4.  Percentage of each municipality in each NJDEP Forestry Section 6

5.  Area of each municipality, square miles 7

6.  Emission Factors, EF, (lbs/ton burned) 8

VOC=Non Methane Hydrocarbons (NMHC):	8

CO:	52

PM10:	6

PM2.5:	6

7.  Weekly activity, WAF=7 days/week 9

8.  Seasonal adjustment factors, SAF 10

Summer Season Adjustment Factor		0.55

Fall Season Adjustment Factor			0.78

Winter Season Adjustment Factor		1.16

Spring Season Adjustment Factor			1.51

- Process:

- The following equations are used to calculate the emissions without control for this source category.

STEP 1:  For Division C calculate the number of  permits  issued at the municipal level based on the # permits in each section and the % of each municipality in each section.

STEP 2:  For Division C total the number of permits at the municipal level to obtain the number of permits at the county level.

STEP 3: For Divisions A, B and C, calculate the amount of agricultural orchard material burned ORCTON (tons material burned) :

- ORCTON = CPER*ACRE*FL

- STEP 4: Calculate Emissions:

Epa        =  EF*ALCTON

Eps     =  Epa/AADF*SAF

Epw    =  Epa/AADF*SAF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

Epw	= (tons/day) for a typical winter day emission of pollutant

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1. Acreage of agricultural fields burned in each  county based on the number of agricultural field burning permits issued  in each county and represents the best available estimate of acreage burned per permit that can be provided by the NJ Forestry Service.

2. Assume that tons of matter burned per acreage is uniform across the specified division  and represents the best available estimate that can be provided by NJ Forestry Service.

3. Assume particulate matter from most agricultural refuse burning is within the submicrometer size range. 8

4. Assume that the EF for NMHC is equivalent to the EF for VOC.

5. North of the Raritan represents Division A and the Mullica River divides B and C division. 5,6

6. Assume that approximately 5/6 of Somerset County belongs to Division A and the remaining  1/6 belongs

to Division B. 2,3

7. Assume that 1/2 of Mercer County belongs to Division A and the other 1/2 belongs to Division B. 5,6

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:

New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 2, N.J.A.C. 7:27-2, New Jersey State Department of Environmental Protection, June 20, 1994

- Double Counting:

Emissions for this source category have not been reported in the point source inventory, therefore no adjustment of the area source emissions was required.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

CO		tons/yr		tons/day		tons/day

PM10		tons/yr   	tons/day		tons/day

PM2.5		tons/yr   	tons/day		tons/day

- References

1. Procedures for the Preparation of Emission Inventories for Carbon Monoxide and Precursors of Ozone, Vol. I:  General Guidance for Stationary Sources, May 1991, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-450/4-91-016

2.  NJDEP Division A Permit Log for 2002 by County, November 18, 2003

3.  NJDEP Division B Permit Log for 2002 by Township, November 18, 2003

4.  NJDEP Division C Permit Log for 2002 by Section, November 19, 2003

5.  Memo to File on Phone Conversations with NJ Forestry Service

6. Administrative Map, State of New Jersey, NJDEP,  Forestry Service, November 1984

7. Estimate of 1996 Total Resident Population and Square Mile Area by Counties and Municipalities,   US Bureau of the Census, Population Data Division, 6/30/99

8. Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC, Table 2.5-5, Unspecified Orchard Crops and Note b.

9. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction,  Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c

10.  NJDEP Division A Fire Call in Log for 2002, November 18, 2003

Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Forest Wildfires

- SCC:  2810010000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC), oxides of nitrogen (NOx), carbon monoxide (CO), sulfur dioxide (SO2), particulate matter less than or equal to 10 microns (PM10) and particulate matter less than or equal to 2.5 microns (PM2.5).  The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category: 1

1.  Contacting the Bureau of Forest Fire Management for information

concerning wildfire burning

This is the only recommended method and was employed in developing the inventory.

Emissions from forest wildfires are calculated using county specific acres burned.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  Annual Acres burned by county and month, ACRES 2

2.  Fuel loading factor, FL 3

Division A, B, C = 11 tons/acre

3.  Emission Factors, EF, (lbs/ton burned)

VOC =  (296kg/hect*2.2046lb/kg)/(25Mgfuel/hect*1.1023tonfuel/Mg)=	23.7	4

NOx =   (49kg/hect*2.2046lb/kg)/(25Mgfuel/hect*1.1023tonfuel/Mg)=	4	4

CO =     (1730kg/hect*2.2046lb/kg)/(25Mgfuel/hect*1.1023tonfuel/Mg)=	140	4

PM10 = (15kg/Mg*2.2046lb/kg)/(1.1023tonfuel/Mg) =			30	5

PM2.5 = (13.5kg/Mg*2.2046lb/kg)/(1.1023tonfuel/Mg) =			27	5

4.  Weekly activity, WAF=7 days/week 6

5.   Seasonal adjustment factors, SAF 2

Summer Season Adjustment Factor		3.31

Fall Season Adjustment Factor			0.04

Winter Season Adjustment Factor		0.22

Spring Season Adjustment Factor			0.43

- Process:

The following equations are used to calculate the emissions without control  for this source category:

Epa        =  EF*ACRES*FL

Eps     =  Epa/AADF*SAF

Epw    =  Epa/AADF*SAF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

Epw	= (tons/day) for a typical winter day emission of pollutant

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1. The NJ Forest Service fuel loadings factors are assumed to be valid.

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:    None

- Double Counting:

Emissions for this source category have not been reported in the point source inventory, therefore no adjustment of the area source emissions was required.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

NOx		tons/yr		tons/day		tons/day

CO		tons/yr		tons/day		tons/day

SO2         	tons/yr   	tons/day		tons/day

PM10		tons/yr   	tons/day		tons/day

PM2.5		tons/yr   	tons/day		tons/day

- References:

1. Procedures for the Preparation of Emission Inventories for Carbon Monoxide and Precursors of Ozone, Vol. I:  General Guidance for Stationary Sources, May 1991, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-450/4-91-016

2. Wildfire Acres Burned in 2002,  December 2, 2003 fax from Pat Stockman, NJDEP, Forestry Service

3. Memo to File on Phone Conversations with NJ Forestry Service

4.  Compilation of Air Pollutant Emission Factors, AP-42, Fifth Edition, United States Environmental Protection Agency, Office of  Air Quality Planning and Standards, Research Triangle Park, NC, Table 13.1-2, Eastern Group

5.  Development of Emissions Inventory Methods for Wildland Fires, February 2002, Table 22, Average WIldfies

6. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction,  Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Managed Burning

- SCC:  2810015000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC), oxides of nitrogen (NOx), carbon monoxide (CO), sulfur dioxide (SO2), particulate matter less than or equal to 10 microns (PM10) and particulate matter less than or equal to 2.5 microns (PM2.5).  The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category: 1

1.  Contacting the Bureau of Forest Fire Management for information

concerning managed burning

This is the only recommended method and was employed in developing the inventory.

Emissions from managed burning are calculated using county specific acres burned.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  Annual Acres burned by county and month, ACRES 2

2.  Fuel loading factor, FL 3

Division A, B, C = 4 tons/acre

3.  Emission Factors, EF, (lbs/ton burned)

VOCA = (6.4 g/kg) * (907kg/ton)/(454 g/lb) = 	12.8	4  (Table 13.1-3 Hardwood Fire)

VOCBC = (3.5 g/kg) * (907kg/ton)/(454 g/lb) =  	7	4  (Table 13.1-3 Short Needle)

NOx =  (4 g/kg) * (907kg/ton)/(454 g/lb) =		8	4  (Section 13.1 page 13.1-6)

CO =  (143.8 g/kg) * (907kg/ton)/(454 g/lb) =	287.6	4  (Table 13.1-4 North and Centeral Eastern

Region, Average for the Region)

PM10 =  (14 g/kg) * (907kg/ton)/(454 g/lb) = 	28	4  (Table 13.1-4 North and Centeral Eastern

Region, Average for the Region)

PM2.5 = (12 g/kg) * (907kg/ton)/(454 g/lb) = 	24	4  (Table 13.1-3 Short Needle Conifer)

4.  Weekly activity, WAF=7 days/week 5

5.   Seasonal adjustment factors, SAF 2

Summer Season Adjustment Factor		0

Fall Season Adjustment Factor			0

Winter Season Adjustment Factor		2.85

Spring Season Adjustment Factor			1.15

- Process:

The following equations are used to calculate the emissions without control  for this source category:

Epa        =  EF*ACRES*FL

Eps     =  Epa/AADF*SAF

Epw    =  Epa/AADF*SAF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

Epw	= (tons/day) for a typical winter day emission of pollutant

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1. The NJ Forest Service fuel loadings factors are assumed to be valid.

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:

New Jersey Administrative Code, Title 7, Chapter 27, Subchapter 2, N.J.A.C. 7:27-2, New Jersey State Department of Environmental Protection, June 20, 1994

- Double Counting:

Emissions for this source category have not been reported in the point source inventory, therefore no adjustment of the area source emissions was required.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

NOx		tons/yr		tons/day		tons/day

CO		tons/yr		tons/day		tons/day

SO2         	tons/yr   	tons/day		tons/day

PM10		tons/yr   	tons/day		tons/day

PM2.5		tons/yr   	tons/day		tons/day

- References:

1. Procedures for the Preparation of Emission Inventories for Carbon Monoxide and Precursors of Ozone, Vol. I:  General Guidance for Stationary Sources, May 1991, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-450/4-91-016

2. Prescribed Fire in 2002,  November 20, 2003 email from Bert Plante, NJDEP, Forestry Service

3. Memo to File on Phone Conversations with NJ Forestry Service

4  Compilation of Air Pollutant Emission Factors, AP-42, Fifth Edition, United States Environmental Protection Agency, Office of  Air Quality Planning and Standards, Research Triangle Park, NC

5. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction,  Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Restaurant Operations

- SCC:  2302002200

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC), particulate matter less than or equal to 10 microns (PM10) and particulate matter less than or equal to 2.5 microns (PM2.5).  The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodology is used for this source category:

1. South Coast Air Quality Management District (SCAQMD)/Employment based analysis

Emissions from restaurant operations are calculated using the SCAQMD research and calculations and allocating emissions to the county level using population.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category:

1. The number of restaurants in the California South Coast Air Quality Management District (SCAQMD)

SCAQMDRES = 29,000 1

2. The number of restaurants in New Jersey estimated as follows:

NJRES = SCAQMDRES*( New Jersey population /SCAQMD population = 0.57) = 16,530

3. SCAQMD Emission Calculations as follows, where: 1

NJ emissions = SCAQMD emissions * ( New Jersey population /SCAQMD population = 0.57)

|  | Emission tpd | Emission tpd | Emission tpd | Emission tpd |
| --- | --- | --- | --- | --- |
|  | ###### SCAQMD | SCAQMD | NJ | NJ |
|  | PM | VOC | PM | VOC |
| Total Restaurant Operations (TRO) | 10.70 | 1.60 | 6.10 | 0.91 |
| Underfired Charbroiling | 8.99 | 1.14 | 5.12 | 0.65 |
| Chaindriven Charbroiling (Burger King) | 1.07 | 0.35 | 0.61 | 0.20 |
| Griddles | 0.54 | 0.06 | 0.30 | 0.04 |
| Deep Fat Fryers | 0.00 | 0.03 | 0.00 | 0.02 |
| Ovens | 0.00 | 0.00 | 0.00 | 0.00 |

4.  Emission Factor, EF, (lbs/ restaurant/day)

VOC = (TROVOC tpd * 2000 lb/ton)/NJRES = 0.11

PM10 = (TROPM tpd * 2000 lb/ton)/NJRES = 0.74

PM2.5 = (TROPM tpd * 2000 lb/ton)/NJRES = 0.74

5. County and state employment for NAICS 71111, 72211-32, 7224, CEMP, SEMP 2

6.  Weekly activity, WAF=7 days/week

7. Seasonal adjustment factor, SAF = 1

- Process:

The following equations are used to calculate the emissions without control for this source category.

Epa	= NJRES*(CEMP/SEMP)*EF*CF1/CF2

Eps	= Epa*SAF//ADDF

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

CF1	= Conversion factor for units = 365 days/year

CF2	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

- Rule Effectiveness:

The emissions from this source category are regulated by the following rule:   None

- Double Counting:

Emissions for this source category have not been reported in the point source inventory, therefore no adjustment of the area source emissions was required.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

PM10		tons/yr   	tons/day		tons/day

PM2.5		tons/yr   	tons/day		tons/day

- References:

1.  South Coast AIr Quality Management District (SCAQMD) Air Quality Management Plan 2003, Appendix IV-A District's Stationary and Mobile Source Control Measures, Emission Reductions From Restaurant Operations.

- 2.  Total 2002 employment by 6 digit NAICS code and by county, New Jersey  Department of Labor.

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Structural Fires

- SCC:  2810030000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC), oxides of nitrogen (NOx), carbon monoxide (CO), sulfur dioxide (SO2), particulate matter less than or equal to 10 microns (PM10) and particulate matter less than or equal to 2.5 microns (PM2.5).  The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category: 1

1.  Local fire department information

2.  Population based estimate of the number of structural fires

Local fire department information was used because the input data elements were readily available

Emissions from structural fires are calculated using fire department specific number of fires.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  Number of fires per Fire Department in 2002, FIRE 2

2.  New Jersey Fire Departments by county 3

3.  Fuel loading factor:  FL = 1.15 tons burned/fire 4

4.  Emission Factors, EF, (lbs/ton burned) 4

VOC = 11

NOx = 1.4

CO = 60

PM10 = 10.8

PM2.5 = 10.8

5.  Weekly activity, WAF=7 days/week 5

6.  Seasonal adjustment factor, SAF 5

Summer Season Adjustment Factor		0.8

Fall Season Adjustment Factor			0.94

Winter Season Adjustment Factor		1.32

Spring Season Adjustment Factor			0.94

- Process:

The following equations are used to calculate the emissions without control for this source category.

Epa= FIRE*FL*EF/CF

Eps	= Epa*SAF/AADF

Epw	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

Epw	= (tons/day) for a typical winter day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1. PM 2.5 and PM10 emission factors are conservatively assumed to be equal to PM.

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:    None

- Double Counting:

Emissions for this source category have not been reported in the point source inventory, therefore no adjustment of the area source emissions was required.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

NOx		tons/yr		tons/day		tons/day

CO		tons/yr		tons/day		tons/day

SO2         	tons/yr   	tons/day		tons/day

PM10		tons/yr   	tons/day		tons/day

PM2.5		tons/yr   	tons/day		tons/day

- References:

1. Procedures for the Preparation of Emission Inventories for Carbon Monoxideand Precursors of Ozone, Vol. I:  General Guidance for Stationary Sources, May 1991, United States Environmental Protection Agency, Office of Air  Quality Planning and Standards, Research Triangle Park, NC, EPA-450/4-91-016

2.  2002 Structural Fire Data, Number of Fires per Fire Department, Andy Fritz, NJ Division of Fire Safety, April 15, 2003.

3. Fire Department by County, NJ Division of Fire Safety website, April 15, 2003:

www.state.nj.us/dca/dfs/fdeptnum.htm

4. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 18 - Structure Fires, Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c, Table 18.4-1

5. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction,  Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Vehicle Fires

- SCC:  2810050000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC), oxides of nitrogen (NOx), carbon monoxide (CO), sulfur dioxide (SO2), particulate matter less than or equal to 10 microns (PM10) and particulate matter less than or equal to 2.5 microns (PM2.5).  The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category: 1

1.  Local fire department information

2.  Population based estimate of the number of structural fires

Local fire department information was used because the input data elements were readily available

Emissions from vehicle fires are calculated using fire department specific number of fires.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  Number of fires per Fire Department in 2002, FIRE 2

2.  New Jersey Fire Departments by county 3

3.  Fuel loading factor:  FL = 500 pounds burned/fire 4

4.  Emission Factors, EF, (lbs/ton burned) 4

VOC = 32

NOx = 4

CO = 125

PM10 = 100

PM2.5 = 100

5.  Weekly activity, WAF=7 days/week 5

6.  Seasonal adjustment factor, SAF = 1

- Process:

The following equations are used to calculate the emissions without control for this source category.

Epa= FIRE*FL*EF/CF

Eps	= Epa*SAF/AADF

Epw	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

Epw	= (tons/day) for a typical winter day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1. PM 2.5 and PM10 emission factors are conservatively assumed to be equal to PM.

2.  VOC emission factor is equal to nonmethane TOC

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:    None

- Double Counting:

Emissions for this source category have not been reported in the point source inventory, therefore no adjustment of the area source emissions was required.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

NOx		tons/yr		tons/day		tons/day

CO		tons/yr		tons/day		tons/day

SO2         	tons/yr   	tons/day		tons/day

PM10		tons/yr   	tons/day		tons/day

PM2.5		tons/yr   	tons/day		tons/day

- References:

1. Procedures for the Preparation of Emission Inventories for Carbon Monoxideand Precursors of Ozone, Vol. I:  General Guidance for Stationary Sources, May 1991, United States Environmental Protection Agency, Office of Air  Quality Planning and Standards, Research Triangle Park, NC, EPA-450/4-91-016

2.  2002 Vehicle Fire Data, Number of Fires per Fire Department, Andy Fritz, NJ Division of Fire Safety, April 15, 2003.

3. Fire Department by County, NJ Division of Fire Safety website, April 15, 2003:

www.state.nj.us/dca/dfs/fdeptnum.htm

4. EIIP Volume III, Area Sources Preferred and Alternative Methods, Area Souce Category Method Absract-Vehicle FIres, Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC,  EPA-454/R-97-004c, May 15, 2004

5. EIIP Volume III, Area Sources Preferred and Alternative Methods, Chapter 1 - Introduction,  Emission Inventory Improvement Program, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Cigarette Smoking

- SCC:  2810003000

The following describes the emission calculation methodology for this source category for the following pollutants: volatile organic compounds (VOC), oxides of nitrogen (NOx), carbon monoxide (CO), sulfur dioxide (SO2), particulate matter less than or equal to 10 microns (PM10) and particulate matter less than or equal to 2.5 microns (PM2.5).  The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation  methodology sheet.

- Calculation Methodology:

- The following calculation methodologies are recommended for this source category: 1

1.  Obtaining state specific data on cigarette smoking

This is the only recommended method and was used in developing the emission inventory.

Emissions from cigarette smoking are calculated using statewide packs of cigarettes sold, allocated to the county level using population.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category.

1.  Packs of cigarettes sold in New Jersey in 2002, PCIG 2

2.  Cigarettes per pack, #CIGPP = 20

3.  County and State population, CPOP, SPOP 3

4.  Emission Factors, EF, (mg/cigarette smoked) 1

VOC = 6.75

NOx = 0.35

CO = 19.0

PM10 = 22.5

PM2.5 = 22.5

5.  Weekly activity, WAF=7 days/week

6.  Seasonal adjustment factor, SAF=1

- Process:

The following equations are used to calculate the emissions without control for this source category.

Epa	= PCIG*#CIGPP*EF*(CPOP/SPOP)*CF1/CF2

Eps	= Epa*SAF/AADF

Epw	= Epa*SAF/AADF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

Epw	= (tons/day) for a typical winter day emission of pollutant

CF1    = Conversion factor for units = 2.205*106  pounds/miligrams

CF2	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

Assumptions:

1. Assume a weekly activity factor of 7 days per week

2. Assume a seasonal adjustment factor of 1

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:   None

- Double Counting:

Emissions for this source category have not been reported in the point source inventory, therefore no adjustment of the area source emissions was required.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant	Annual		Summer day		Winter day

VOC		tons/yr		tons/day		tons/day

NOx		tons/yr		tons/day		tons/day

CO		tons/yr		tons/day		tons/day

SO2         	tons/yr   	tons/day		tons/day

PM10		tons/yr   	tons/day		tons/day

PM2.5		tons/yr   	tons/day		tons/day

- References:

1. Procedures for the Preparation of Emission Inventories for Carbon Monoxideand Precursors of Ozone, Vol. I:  General Guidance for Stationary Sources,  May 1991, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC, EPA-450/4-91-016

2. Packs of cigarettes sold in New Jersey in 2002, email from Allison Gertel, Tobacco Control Unit, New Jersey Department of Health and Senior Services, citing data from Delnevo, Cristine, 2003, unpublished data, University of Medicine and Dentistry of NJ – School of Public Health, citing information from NJ Department of Treasury, Division of Revenue.

3. Estimate of  2002 Total Resident Population by County, NJDOT, July 17, 2003.

Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Paved Roads

- SCC: 2294000000

The following describes the emission calculation methodology for this source category for the following pollutants: fugitive dust particulate matter less than or equal to 10 microns (PM10) and fugitive dust particulate matter less than or equal to 2.5 microns (PM2.5). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation methodology sheet.

- Calculation Methodology:

- The following calculation methodology was used for this source category:

1.  USEPA Methodology/State specific VMT 1, 2

The paved road dust category includes emissions of fugitive dust particulate matter entrained by vehicular travel on paved roads.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category:

1.  Daily vehicle miles traveled by county, roadway type  (Freeway, Arterial, Local) and season, VMT 3

2.  Average vehicle weight by county, W

Calculated by taking average vehicle weight by vehicle class from EPA Mobile 6 model 4, 5 and averaging by vehicle class and county using average annual VMT 3

| County | Mean Vehicle Weight |
| --- | --- |
| Atlantic | 2.33 |
| Bergen | 2.49 |
| Burlington | 3.22 |
| Camden | 3.22 |
| Cape May | 2.31 |
| Cumberland | 2.60 |
| Essex | 2.53 |
| Gloucester | 3.22 |
| Hudson | 2.96 |
| Hunterdon | 3.32 |
| Mercer | 3.22 |
| Middlesex | 3.13 |
| Monmouth | 2.58 |
| Morris | 2.73 |
| Ocean | 2.49 |
| Passaic | 2.66 |
| Salem | 3.28 |
| Somerset | 2.76 |
| Sussex | 3.05 |
| Union | 2.67 |
| Warren | 4.12 |
| Statewide Average | 2.90 |

3.  Precipitation data, # of Days > 0.01inches precipitation per season, p 6

4.  Silt Loading, sL g/m2

Calculated by taking silt loading factors in EPA NEI guidance 1, 5  and averaging by county using urban and rural roadway mileage7

|  | Silt Loading Factors (g/m2) |  |  |  |
| --- | --- | --- | --- | --- |
| County | Freeway | Arterial | Local |
| Atlantic | 0.02 | 0.08 | 0.20 |
| Bergen | 0.02 | 0.05 | 0.20 |
| Burlington | 0.02 | 0.08 | 0.20 |
| Camden | 0.02 | 0.05 | 0.20 |
| Cape May | 0.02 | 0.11 | 0.20 |
| Cumberland | 0.02 | 0.09 | 0.20 |
| Essex | 0.02 | 0.05 | 0.20 |
| Gloucester | 0.02 | 0.08 | 0.20 |
| Hudson | 0.02 | 0.05 | 0.20 |
| Hunterdon | 0.02 | 0.13 | 0.20 |
| Mercer | 0.02 | 0.06 | 0.20 |
| Middlesex | 0.02 | 0.06 | 0.20 |
| Monmouth | 0.02 | 0.07 | 0.20 |
| Morris | 0.02 | 0.06 | 0.20 |
| Ocean | 0.02 | 0.08 | 0.20 |
| Passaic | 0.02 | 0.05 | 0.20 |
| Salem | 0.02 | 0.12 | 0.20 |
| Somerset | 0.02 | 0.07 | 0.20 |
| Sussex | 0.02 | 0.12 | 0.20 |
| Union | 0.02 | 0.05 | 0.20 |
| Warren | 0.02 | 0.11 | 0.20 |

| Constants 2 | PM 2.5 | PM 10 |  |
| --- | --- | --- | --- |
| 5. Base emission factor, empirical constant, k | 1.8 | 7.3 | g/mile |
| 6. c, emission factor for 1980's vehicle fleet exhaust, brake wear and tire wear | 0.1617 | 0.2119 | g/mile |

7.  Weekly activity, WAF=7 days/week

8.  Seasonal adjustment factors, SAF, calculated see below

Summer Season Adjustment Factor		1.09

Fall Season Adjustment Factor			0.99

Winter Season Adjustment Factor		0.94

Spring Season Adjustment Factor			0.99

- Process:

- The following equations are used to calculate the emissions without control for this source category. 1, 2

EF  = (((k * (sL/2)0.65 * (W/3)1.5 )-c) * (1-p/4N))/CF1

Epd	= (VMT* EF) /CF2) * DAF

Epsea	= (Epdfreeway + Epdfarterial + Epdlocal) * DAF

Epa	= ((Eps*N) + (Epw*N) + (Epsp*N) + (Epf*N)) * DAF

SAF 	= (Eps/Epa)*N

Where:

EF 	= emission factor by roadway type (freeway, arterial, local) and season(summer, winter, spring, fall) (pounds per mile)

CF1	= Conversion factor for units = 453.59237 grams/pound

Epd	= (tons/day) for a typical day emission of pollutant by roadway type and season

Epsea	= (tons/day) for a typical day emission of pollutant by season

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

Epw	= (tons/day) for a typical winter day emission of pollutant

Epsp	= (tons/day) for a typical spring day emission of pollutant

Epf	= (tons/day) for a typical fall day emission of pollutant

N 	= number of days in averaging period (summer=92, winter = 90, spring = 92, fall = 91)

CF2	= Conversion factor for units = 2000 lbs/ton

SAF	= Seasonal adjustment factor

DAF	= Dust adjustment factor, see "Fugitive Dust Inventory Discussion and Summary" in attachment to 2002 Periodic Emission Inventory.

Assumptions:

1.  Assume a weekly activity of 7 days

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:  None

- Double Counting:

Emissions for this source category have not been reported in the point source inventory, therefore no adjustment of the area source emissions was required.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant			Annual		Summer day		Winter day

Fugitive Dust PM10		tons/yr   	tons/day		tons/day

Fugitive Dust PM2.5		tons/yr   	tons/day		tons/day

- References

- 1.  Documentation for the 2002 Non-point Source National Emission Inventory for Criteria and Hazardous Air Pollutants (January 2004 Version), E.H. Pechan & Associates for EPA, March 2004.

2.  Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC, Section 13.2.1, Paved Roads, December 2003.

3.  New Jersey Department of Environmental Protection 2002 Mobile Inventory

- 4.  USEPA Mobile6 Model User's Guide,  Mobile6 Vehicle Classifications, March 13, 2002

- 5.  Email from Megan Schuster of MARAMA dated November 30, 2004 containing latest "Fugitive Dust From Paved Roads" Methodology Sheet from E.H. Pechan and Associates.

- 6. Local Climatological Data 2002, National Climatic Data Center, Newark and Atlantic City, NJ

7.  New Jersey's Roadway Mileage and Daily VMT by Functional Classification Distributed by County, 2002, New Jersey Department of Transportation, http://www.state.nj.us/transportation/refdata/roadway/vmt.shtm.

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Unpaved Roads

- SCC: 2296000000

The following describes the emission calculation methodology for this source category for the following pollutants: fugitive dust particulate matter less than or equal to 10 microns (PM10) and fugitive dust particulate matter less than or equal to 2.5 microns (PM2.5). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation methodology sheet.

- Calculation Methodology:

- The following calculation methodology was used for this source category:

1.  USEPA Methodology 1, 2

This source category provides estimates of the entrained geologic particulate matter emissions that result from vehicular travel over non-agricultural unpaved roads. The emissions result from the mechanical disturbance of the roadway and the vehicle generated air turbulence effects.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category:

1.  Unpaved Road Mileage, statewide, UPRM 3

2.  Average daily traffic volume, vehicles per day, statewide, ADTV 3

3.  1990 Rural population by county, CRPOP, SRPOP 4

4.  Precipitation data, # of Days > 0.01inches precipitation per season, p 5

5.  Mean vehicle speed, S = 33 mph 2

6.  Silt Content percent, s = 22 % 6

7.  Moisure content percent, M = 0.50 % 7

| Constants 2 | PM 2.5 | PM 10 |  |
| --- | --- | --- | --- |
| 8. Base emission factor, empirical constant, k | 0.27 | 1.80 | lbs/mile |
| 9. emipirical constant, a | 1.00 | 1.00 | lbs/mile |
| 10. emipirical constant, b |  |  |  |
| 11. emipirical constant, c | 0.20 | 0.20 | lbs/mile |
| 12. emipirical constant, d | 0.50 | 0.50 | lbs/mile |
| 13. c, emission factor for 1980's vehicle fleet exhaust, brake wear and tire wear | 0.00036 | 0.00047 | lbs/mile |

14.  Weekly activity, WAF=7 days/week

15.  Seasonal adjustment factors, SAF, calculated see below

Summer Season Adjustment Factor		1.11

Fall Season Adjustment Factor			0.95

Winter Season Adjustment Factor		1.05

Spring Season Adjustment Factor			0.89

- Process:

- The following equations are used to calculate the emissions without control for this source category.

Equations 1b and 2: 2

EF = (([k * (s/12)a * (S/30)d] / [(M/0.5)c])-c) * [(365 - p* 4)/365]

Epd	= [(UPRM*(CRPOP/SRPOP))*(ADTV*(CRPOP/SRPOP))* EF/CF] * DAF

Epa	= [(Eps*N) + (Epw*N) + (Epsp*N) + (Epf*N)] * DAF

SAF 	= (Epd/Epa)*N

Where:

EF 	= seasonal emission factor (pounds per mile)

Epd	= (tons/day) for a typical day emission of pollutant by season (summer, winter, spring, fall)

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

Epw	= (tons/day) for a typical winter day emission of pollutant

Epsp	= (tons/day) for a typical spring day emission of pollutant

Epf	= (tons/day) for a typical fall day emission of pollutant

N 	= number of days in averaging period (summer=92, winter = 90, spring = 92, fall = 91)

CF	= Conversion factor for units = 2000 lbs/ton

SAF	= Seasonal adjustment factor

DAF	= Dust adjustment factor, see "Fugitive Dust Inventory Discussion and Summary" in attachment to 2002 Periodic Emission Inventory.

Assumptions:

1.  Assume a weekly activity of 7 days

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:  None

- Double Counting:

Emissions for this source category have not been reported in the point source inventory, therefore no adjustment of the area source emissions was required.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant			Annual		Summer day		Winter day

Fugitive Dust PM10		tons/yr   	tons/day		tons/day

Fugitive Dust PM2.5		tons/yr   	tons/day		tons/day

- References

1.  U.S. Environmental Protection Agency, Office of Air Quality Planning and Standards.  Procedures Document for National Emission Inventory, Criteria Air Pollutants, 1985-1999.  EPA-454/R-01-006.  Section 4.8.1.7.2.2.  March 2001.

2.  Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC, Section 13.2.2, Unpaved Roads, December 2003.

- 3.  1996 Highway Statistics, U.S. Department of Transportation, Federal Highway Administration, Table HM-67

- 4.  US Census Bureau, Urban and Rural Population and Housing Units, New Jersey and Counties:  1990

- 5.  Local Climatological Data 2002, National Climatic Data Center, Newark and Atlantic City, NJ

- 6.  Evaluation of Potential Improvements in the Estimation of unpaved Road Fugitive Emission Inventories, William Barnard, Gary Stensland, Donald Gatz, Illinois State Water Survey, June 1987.

- 7.  Documentation for the 2002 Non-point Source National Emission Inventory for Criteria and Hazardous Air Pollutants (January 2004 Version), E.H. Pechan & Associates for EPA, March 2004.

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Construction -  Residential

- SCC: 2311010000

The following describes the emission calculation methodology for this source category for the following pollutants: fugitive dust particulate matter less than or equal to 10 microns (PM10) and fugitive dust particulate matter less than or equal to 2.5 microns (PM2.5). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation methodology sheet.

- Calculation Methodology:

- The following calculation methodology was used for this source category:

1.  USEPA Methodology 1

Residential building construction includes the construction of single-family units, two family units, and apartments buildings.  The emissions result predominantly from site preparation work which may include scraping, grading, loading, digging, compacting, light-duty vehicle travel, and other operations.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category:

1.  The county-level number of housing permits issued for single-family units, two-family units, and apartment buildings, Bp-single, Bp-two, Bp-apartment   2

2.  The regional-level percentage of new single-family units that do not have basements, %NOBM = 11% 3

3.  The regional-level percentage of new single-family units that have basements, %BM = 89% 3

4.  Building-to-acre conversion factor 1

- fsingle = 1/4 acre/building

- ftwo = 1/3 acre/building

- fapartment = 1/2 acre/building

5.  Emission factors, EF 1

- PM10single and two, no basement =  	0.032 	tons/acre-month

- PM10single and  two, basement =  	0.011 	tons/acre-month

- PM10apartment =  	0.11 	tons/acre-month

- PM2.5single and two, no basement = PM10*0.2= 	0.0064 	tons/acre-month

- PM2.5single and  two, basement = PM10*0.2= 	0.0022 	tons/acre-month

- PM2.5apartment = PM10*0.2= 	0.022 	tons/acre-month

- PM10dirt = 	0.058 tons/1,000 cubic yards dirt moved

- PM2.5dirt = PM10*0.2 = 	0.0118 tons/1,000 cubic yards dirt moved

6.  Duration of construction activity 1

- msingle and two = 6 months

- mapartment = 12 months

7.  Quantity of dirt moved during single-family unit basement construction, d = 652 cubic yards/building 1

8.  Precipitation-Evaporation Value (PE) = 124 4

4.  Silt Content percentage (s) = 18 % 5

6.  Weekly activity, WAF=7 days/week

7.  Seasonal adjustment factors, SAF

Summer Season Adjustment Factor		1.73

Fall Season Adjustment Factor			0.88

Winter Season Adjustment Factor		0.64

Spring Season Adjustment Factor			0.81

SAF based on activity and precipitation variations.  Activitiy variations for summer, fall, winter and spring are as follows: 1.53, 0.94, 0.60 and 0.94 6.  Precipitation variations based on number of days with at least 0.01 inches of precipitation and the precipitation control portion of Equation 2 of AP-42 Section 13.2.2 7.  This equation = ((total days - p) / total days), where the total days is equal to the number of days for each season and p is the number of days with at least 0.01 inches of precipitation. 8  The precipitation variations for summer, fall, winter and spring are as follows: 1.13, 0.94, 1.07 and 0.86.  The total SAF is the product of the activity SAF and the precipitation SAF (summer SAF = 1.53 * 1.13; fall SAF = 0.94 * 0.94; winter SAF = 0.60 * 1.07; and spring SAF = 0.94 * 0.86).

- Process:

- The following equations are used to calculate the emissions without control for this source category:

Single and two family, no basement, (tons/yr) for an annual emission of pollutant by county:

Epsingle and two, no basement =  [((Bp-single * f + Bp-two * f) * %NOBM) * EF * m  * (24/PE) * (s/9)] * DAF

Single and two family,  with basement, (tons/yr) for an annual emission of pollutant by county:

Epsingle and two, basement =

[(((Bp-single * f + Bp-two * f) * %BM) * EF * m  * (24/PE) * (s/9)) + ((Bp-single + Bp-two) * %BM) * EFdirt * d/1000)] * DAF

Apartment, (tons/yr) for an annual emission of pollutant by county:

Epapartment =  [((Bp-apartment * f) * EF * m  * (24/PE) * (s/9)] * DAF

Epa 	= [Epsingle and two, no basement  + Epsingle and two, basement + Epapartment] * DAF

Eps	= [Epa*SAF/AADF] * DAF

Epw	= [Epa*SAF/AADF] * DAF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

Epw	= (tons/day) for a typical winter day emission of pollutant

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

DAF	= Dust adjustment factor, see "Fugitive Dust Inventory Discussion and Summary" in attachment to 2002 Periodic Emission Inventory.

Assumptions:

1.  Assume a weekly activity of 7 days

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:  None

- Double Counting:

Emissions for this source category have not been reported in the point source inventory, therefore no adjustment of the area source emissions was required.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant			Annual		Summer day		Winter day

Fugitive Dust PM10		tons/yr   	tons/day		tons/day

Fugitive Dust PM2.5		tons/yr   	tons/day		tons/day

- References

1.  U.S. Environmental Protection Agency, Office of Air Quality Planning and Standards.  Procedures Document for National Emission Inventory, Criteria Air Pollutants, 1985-1999.  EPA-454/R-01-006.  Section 4.8.1.7.2.2.  March 2001.

2.  Bureau of the Census.  Building Permits website - http://censtats.census.gov/bldg/bldgprmt.shtml.  Annual data for New Jersey by county.

3.  Bureau of the Census.  Characteristics of New Housing website

- http://www.census.gov/const/www/charindex.html  Characteristics of New One-Family Houses Completed, Type of Foundation history table.  Annual data for the Northeast Region.

4.  Estimating Particulate Matter Emissions From Construction Operations, Final Report, Midwest Research Institute, for Eastern Research Group, for USEPA, September 30, 1999.

5.  Summary of Approaches Available for Fugitive Dust Sources Technical Memorandum, E.H. Pechan & Associates, for MARAMA, May 21, 2004.

6.  Seasonal and Monthly Activity Allocation Fractions for Nonroad Engine Emissions Modeling, EPA 420-P-02-010, Table 4, Mid-Atlantic Region.

7.  Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC, Section 13.2.2, Unpaved Roads, December 2003.

- 8.  Local Climatological Data 2002, National Climatic Data Center, Newark and Atlantic City, NJ

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Construction - Commercial

- SCC: 2311020000

The following describes the emission calculation methodology for this source category for the following pollutants: fugitive dust particulate matter less than or equal to 10 microns (PM10) and fugitive dust particulate matter less than or equal to 2.5 microns (PM2.5). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation methodology sheet.

- Calculation Methodology:

- The following calculation methodology was used for this source category:

1.  USEPA Methodology/State specific construction square footage 1

Commercial construction includes the construction of Assembly buildings, education buildings, factories, office buildings, storage facilities, hazardous uses, hotels, motels and guest houses, retail buildings, signs, fences.  The emissions result predominantly from site preparation work which may include scraping, grading, loading, digging, compacting, light-duty vehicle travel, and other operations.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category:

1.  Construction activity from permits, new and additions, by construction type and county (square feet) CON 2

Commercial construction includes the construction of Assembly buildings, education buildings, factories, office buildings, storage facilities, hazardous uses, hotels, motels and guest houses, retail buildings, signs, fences.

2.  Project Duration, m: 1

Signs, fences, misc	1	months

Retail			6	months

Other			12	months

3.  Precipitation-Evaporation Value (PE) = 124 3

4.  Silt Content percentage (s) = 18 % 4

5. Emission factors, EF (tons/acres/month) 1

PM10 =			0.19

PM2.5 =	PM10*0.2 = 	0.038

6.  Weekly activity, WAF=7 days/week

7.  Seasonal adjustment factors, SAF

Summer Season Adjustment Factor		1.73

Fall Season Adjustment Factor			0.88

Winter Season Adjustment Factor		0.64

Spring Season Adjustment Factor			0.81

SAF based on activity and precipitation variations.  Activitiy variations for summer, fall, winter and spring are as follows: 1.53, 0.94, 0.60 and 0.94 5.  Precipitation variations based on number of days with at least 0.01 inches of precipitation and the precipitation control portion of Equation 2 of AP-42 Section 13.2.2 6.  This equation = ((total days - p) / total days), where the total days is equal to the number of days for each season and p is the number of days with at least 0.01 inches of precipitation. 7  The precipitation variations for summer, fall, winter and spring are as follows: 1.13, 0.94, 1.07 and 0.86.  The total SAF is the product of the activity SAF and the precipitation SAF (summer SAF = 1.53 * 1.13; fall SAF = 0.94 * 0.94; winter SAF = 0.60 * 1.07; and spring SAF = 0.94 * 0.86).

- Process:

- The following equations are used to calculate the emissions without control for this source category:

Epa = [CON/CF * m * EF * (24/PE) *(s/9)] * DAF

Eps	= [Epa*SAF/AADF] * DAF

Epw	= [Epa*SAF/AADF] * DAF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

Epw	= (tons/day) for a typical winter day emission of pollutant

CF	= Conversion factor for units = 43,560 sf/acre

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

DAF	= Dust adjustment factor, see "Fugitive Dust Inventory Discussion and Summary" in attachment to 2002 Periodic Emission Inventory.

Assumptions:

1.  Assume a weekly activity of 7 days

2.  Assume a project duration of one month for signs and fences, 6 months for retail facilities and 12 months for all other.

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:  None

- Double Counting:

Emissions for this source category have not been reported in the point source inventory, therefore no adjustment of the area source emissions was required.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant			Annual		Summer day		Winter day

Fugitive Dust PM10		tons/yr   	tons/day		tons/day

Fugitive Dust PM2.5		tons/yr   	tons/day		tons/day

- References

1.  U.S. Environmental Protection Agency, Office of Air Quality Planning and Standards.  Procedures Document for National Emission Inventory, Criteria Air Pollutants, 1985-1999.  EPA-454/R-01-006.  Section 4.8.1.7.2.2.  March 2001.

2.  2002 Construction activity from permits, new and additions, by construction type and county (square feet), New Jersey Department of Community Affairs, John Lago, May 2003.

3.  Estimating Particulate Matter Emissions From Construction Operations, Final Report, Midwest Research Institute, for Eastern Research Group, for USEPA, September 30, 1999.

4.  Summary of Approaches Available for Fugitive Dust Sources Technical Memorandum, E.H. Pechan & Associates, for MARAMA, May 21, 2004.

5.  Seasonal and Monthly Activity Allocation Fractions for Nonroad Engine Emissions Modeling, EPA 420-P-02-010, Table 4, Mid-Atlantic Region.

6.  Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC, Section 13.2.2, Unpaved Roads, December 2003.

- 7.  Local Climatological Data 2002, National Climatic Data Center, Newark and Atlantic City, NJ

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Construction – Heavy Miscellaneous

- SCC: 2311020000

The following describes the emission calculation methodology for this source category for the following pollutants: fugitive dust particulate matter less than or equal to 10 microns (PM10) and fugitive dust particulate matter less than or equal to 2.5 microns (PM2.5). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation methodology sheet.

- Calculation Methodology:

- The following calculation methodology was used for this source category:

1.  USEPA Methodology 1

Heavy miscellaneous construction includes consturction of private driveways and parking areas, bridges, tunnels and elevated highways, sewers, water mains, pipelines other than sewer and water, power plants, sewage treatment plants, water treatement plants and other. The emissions result predominantly from site preparation work which may include scraping, grading, loading, digging, compacting, light-duty vehicle travel, and other operations.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category:

1.  Value of Construction Work in New Jersey, $CON: 2

|  | New $1000 | Additions, alterations or reconstruction $1000 | Total $ |
| --- | --- | --- | --- |
| private driveways and parking areas | 108,110 | 73,259 | 181,369,000 |
| bridges, tunnels and elevated highways | 163,628 | 98,249 | 261,877,000 |
| sewers | 177,612 | 51,171 | 228,783,000 |
| water mains | 173,687 | 38,170 | 211,857,000 |
| pipeline other than sewer and water | 124,313 | 18,662 | 142,975,000 |
| power plants | 107,933 | 39,942 | 147,875,000 |
| sewage treatment plants | 184,251 | 94,352 | 278,603,000 |
| water treatement plants | 138,390 | 43,157 | 181,547,000 |
| other | 393,995 | 134,143 | 528,138,000 |
| Total | 1,571,919 | 591,105 | 2,163,024,000 |

2.  County and state construction employment other than residential, commercial and roadway for NAICs 236210, 237110, 237120, 237130, 237310, 237990, CEMP, SEMP. 3

3.  Dollars-to-acres conversion factor, f = 1.6 acres/$1,000,000 4

4.  Duration of construction activity, m = 12 months/year 1

5.  Precipitation-Evaporation Value (PE) = 124 5

6.  Silt Content percentage (s) = 18 % 4

7.  Emission factors, EF(tons/acre-month) 1

PM-10 =			0.19

PM-2.5 = PM10*0.2=	0.038

8.  Weekly activity, WAF=7 days/week

9.  Seasonal adjustment factors, SAF

Summer Season Adjustment Factor		1.73

Fall Season Adjustment Factor			0.88

Winter Season Adjustment Factor		0.64

Spring Season Adjustment Factor			0.81

SAF based on activity and precipitation variations.  Activitiy variations for summer, fall, winter and spring are as follows: 1.53, 0.94, 0.60 and 0.94 6.  Precipitation variations based on number of days with at least 0.01 inches of precipitation and the precipitation control portion of Equation 2 of AP-42 Section 13.2.2 7.  This equation = ((total days - p) / total days), where the total days is equal to the number of days for each season and p is the number of days with at least 0.01 inches of precipitation. 8  The precipitation variations for summer, fall, winter and spring are as follows: 1.13, 0.94, 1.07 and 0.86.  The total SAF is the product of the activity SAF and the precipitation SAF (summer SAF = 1.53 * 1.13; fall SAF = 0.94 * 0.94; winter SAF = 0.60 * 1.07; and spring SAF = 0.94 * 0.86).

- Process:

- The following equations are used to calculate the emissions without control for this source category:

Epa =[CON/$1000000) *(CEMP/SEMP)  * f  * m * EF * (24/PE) *(s/9)] * DAF

Eps	= [Epa*SAF/AADF] * DAF

Epw	= [Epa*SAF/AADF] * DAF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

Epw	= (tons/day) for a typical winter day emission of pollutant

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

DAF	= Dust adjustment factor, see "Fugitive Dust Inventory Discussion and Summary" in attachment to 2002 Periodic Emission Inventory.

Assumptions:

1.  Assume a weekly activity of 7 days

2.  Assume a project duration of 12 months.

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:  None

- Double Counting:

Emissions for this source category have not been reported in the point source inventory, therefore no adjustment of the area source emissions was required.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant			Annual		Summer day		Winter day

Fugitive Dust PM10		tons/yr   	tons/day		tons/day

Fugitive Dust PM2.5		tons/yr   	tons/day		tons/day

- References

1.  U.S. Environmental Protection Agency, Office of Air Quality Planning and Standards.  Procedures Document for National Emission Inventory, Criteria Air Pollutants, 1985-1999.  EPA-454/R-01-006.  Section 4.8.1.7.2.2.  March 2001.

2.  New Jersey 1997 Economic Census, Construction, US Census Bureau, March 2000, Table 7

3. Total 2002 employment by 6 digit NAICS code and by county, New Jersey  Department of Labor.

4.  Summary of Approaches Available for Fugitive Dust Sources Technical Memorandum, E.H. Pechan & Associates, for MARAMA, May 21, 2004.

5.  Estimating Particulate Matter Emissions From Construction Operations, Final Report, Midwest Research Institute, for Eastern Research Group, for USEPA, September 30, 1999.

6.  Seasonal and Monthly Activity Allocation Fractions for Nonroad Engine Emissions Modeling, EPA 420-P-02-010, Table 4, Mid-Atlantic Region.

7.  Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC, Section 13.2.2, Unpaved Roads, December 2003.

- 8.  Local Climatological Data 2002, National Climatic Data Center, Newark and Atlantic City, NJ

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Construction -  Roadway

- SCC: 2311030000

The following describes the emission calculation methodology for this source category for the following pollutants: fugitive dust particulate matter less than or equal to 10 microns (PM10) and fugitive dust particulate matter less than or equal to 2.5 microns (PM2.5). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation methodology sheet.

- Calculation Methodology:

- The following calculation methodology was used for this source category:

1.  USEPA Methodology/State specific roadway mileage 1

The road construction dust source category provides estimates of the fugitive dust particulate matter due to construction activities while building roads. The emissions result from site preparation work which may include scraping, grading, loading, digging, compacting, light-duty vehicle travel, and other operations.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category:

1.  New Jersey roadway mileage 2001 by county and roadway type, RM2001 2

2.  New Jersey roadway mileage 2002 by county and roadway type, RM2002 2

3.  Conversion of miles to acres for each road type using estimates of acres disturbed per mile (f): 1

Interstate, urban and rural; Other arterial, urban, RC1 - 		15.2 acres/mile

Other arterial, rural, RC2 - 					12.7 acres/mile

Collectors, urban, RC3 - 					9.8 acres/mile

Collectors, rural, RC4 - 						7.9 acres/mile

4.  Emission factors, EF (tons/acres/month) 1

- PM-10 = 0.42

- PM-2.5 = 0.0840

5.  Duration of construction activity m , = 12 months 1

6.  Precipitation-Evaporation Value (PE) = 124 3

7.  Silt Content percentage (s) = 18 % 4

8.  Weekly activity, WAF=7 days/week

9.  Seasonal adjustment factors, SAF

Summer Season Adjustment Factor		1.73

Fall Season Adjustment Factor			0.88

Winter Season Adjustment Factor		0.64

Spring Season Adjustment Factor			0.81

SAF based on activity and precipitation variations.  Activitiy variations for summer, fall, winter and spring are as follows: 1.53, 0.94, 0.60 and 0.94 5.  Precipitation variations based on number of days with at least 0.01 inches of precipitation and the precipitation control portion of Equation 2 of AP-42 Section 13.2.2 6.  This equation = ((total days - p) / total days), where the total days is equal to the number of days for each season and p is the number of days with at least 0.01 inches of precipitation. 7  The precipitation variations for summer, fall, winter and spring are as follows: 1.13, 0.94, 1.07 and 0.86.  The total SAF is the product of the activity SAF and the precipitation SAF (summer SAF = 1.53 * 1.13; fall SAF = 0.94 * 0.94; winter SAF = 0.60 * 1.07; and spring SAF = 0.94 * 0.86).

- Process:

- The following equations are used to calculate the emissions without control for this source category:

EpRC 	=  [(RM2002-RM2001) * f * EF * m  * (24/PE) * (s/9)] * DAF

Epa	= [EpRC1 + EpRC2 + EpRC3 + EpRC4 ] * DAF

Eps	= [Epa*SAF/AADF] * DAF

Epw	= [Epa*SAF/AADF] * DAF

Where:

EpRC	= (tons/yr) for an annual emission of pollutant by roadway category and county

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

Epw	= (tons/day) for a typical winter day emission of pollutant

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

DAF	= Dust adjustment factor, see "Fugitive Dust Inventory Discussion and Summary" in attachment to 2002 Periodic Emission Inventory.

Assumptions:

1.  Assume a weekly activity of 7 days

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:  None

- Double Counting:

Emissions for this source category have not been reported in the point source inventory, therefore no adjustment of the area source emissions was required.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant			Annual		Summer day		Winter day

Fugitive Dust PM10		tons/yr   	tons/day		tons/day

Fugitive Dust PM2.5		tons/yr   	tons/day		tons/day

- References

1.  U.S. Environmental Protection Agency, Office of Air Quality Planning and Standards.  Procedures Document for National Emission Inventory, Criteria Air Pollutants, 1985-1999.  EPA-454/R-01-006.  Section 4.8.1.7.2.2.  March 2001.

2.  New Jersey's Roadway Mileage and Daily VMT by Functional Classification Distributed by County, 2002, New Jersey Department of Transportation, http://www.state.nj.us/transportation/refdata/roadway/vmt.shtm.

3.  Estimating Particulate Matter Emissions From Construction Operations, Final Report, Midwest Research Institute, for Eastern Research Group, for USEPA, September 30, 1999.

4.  Summary of Approaches Available for Fugitive Dust Sources Technical Memorandum, E.H. Pechan & Associates, for MARAMA, May 21, 2004.

5.  Seasonal and Monthly Activity Allocation Fractions for Nonroad Engine Emissions Modeling, EPA 420-P-02-010, Table 4, Mid-Atlantic Region.

6.  Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC, Section 13.2.2, Unpaved Roads, December 2003.

- 7.  Local Climatological Data 2002, National Climatic Data Center, Newark and Atlantic City, NJ

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# MIning and Quarrying – Sand and Gravel

- SCC: 2325020000

The following describes the emission calculation methodology for this source category for the following pollutants: fugitive dust particulate matter less than or equal to 10 microns (PM10) and fugitive dust particulate matter less than or equal to 2.5 microns (PM2.5). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation methodology sheet.

- Calculation Methodology:

- The following calculation methodology was used for this source category:

1.  USEPA Methodology 1, 2

Emissions for mining and quarrying for sand and gravel include: overburden removal, loading and unloading, and overburden replacement.  According to the USEPA, "Transfer and conveyance operations, crushing and screening operations, storage and travel on haul roads are not included because these activities are the most likely to have some type of control implemented." 1  In addition, NJDEP has determined that the majority of these types of emissions are included in the point source inventory.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category:

1.  New Jersey statewide sand and gravel production 2002, SAND&GRAVEL 3

= 19,590 thousand metric tons*1000*1.1023113 =  21,594,278 tons

2.  County and state sand and gravel mining employment for NAICS:  21232x, CEMP, SEMP. 4

3.  Moisture content %, M = 7.9 % 2

4.  Emissions Factors, EF lb/ton : 1, 2

|  | TSP lb/ton (AP-42 Table 11.9-4) | PM10 equation (AP-42 Table 11.9-1) | PM2.5 equation (AP-42 Table 11.9-1) | PM10 lb/ton | PM2.5 lb/ton |
| --- | --- | --- | --- | --- | --- |
| Overburden removal, OB | 0.058 | PM10 = TSP * (bulldozing overburden scaling factor, 0.75) | PM2.5 = TSP * (bulldozing overburden scaling factor, 0.105) | 0.0435 | 0.0061 |
| Loading, L |  | PM10 = (0.119/(M)^0.9*)*0.75 | PM2.5 = (1.16/(M^1.2*)*0.019 | 0.014 | 0.002 |
| Unloading end dump, ULED | 0.007 | PM10 = TSP * (bulldozing overburden scaling factor, 0.75) | PM2.5 = TSP * (bulldozing overburden scaling factor, 0.105) | 0.00525 | 0.0007 |
| Unloading bottom dump, ULBD | 0.066 | PM10 = TSP * (bulldozing overburden scaling factor, 0.75) | PM2.5 = TSP * (bulldozing overburden scaling factor, 0.105) | 0.0495 | 0.0069 |

5.  Weekly activity, WAF=7 days/week

6.  Seasonal adjustment factors, SAF

Summer Season Adjustment Factor		1.13

Fall Season Adjustment Factor			0.94

Winter Season Adjustment Factor		1.07

Spring Season Adjustment Factor			0.86

Seasonal production is assumed to be constant throughout the year.  Factor based on number of days with at least 0.01 inches of precipitation and the precipitation control portion of Equation 2 of AP-42 Section 13.2.2. 5  This equation = ((total days - p) / total days), where the total days is equal to the number of days during each season and p is the number of days with at least 0.01 inches of precipitation. 6 The precipitation variations for summer, fall, winter and spring are as follows: 1.13, 0.94, 1.07 and 0.86.

- Process:

- The following equations are used to calculate the emissions without control for this source category:

Equation 4.8-19: 1

Epa = [SAND&GRAVEL * (CEMP/SEMP) * (OB +  L + 0.5*ULED + ULBD)/CF] * DAF

Eps	= [Epa*SAF/AADF] * DAF

Epw	= [Epa*SAF/AADF] * DAF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

Epw	= (tons/day) for a typical winter day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

DAF	= Dust adjustment factor, see "Fugitive Dust Inventory Discussion and Summary" in attachment to 2002 Periodic Emission Inventory.

Assumptions:

1.  Assume a weekly activity of 7 days

2.  Blasting does not occur for sand and gravel mining based on conversations with William Layton of the New Jersey Concrete & Aggregate Association.

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:  None

- Double Counting:

Emissions for this source category regarding overburden removal, drilling and blasting, loading and unloading, and overburden replacement, have not been reported in the point source inventory, therefore no adjustment of the area source emissions was required.  Transfer and conveyance operations, crushing and screening operations, storage and travel on haul roads are included in the point source inventory.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant			Annual		Summer day		Winter day

Fugitive Dust PM10		tons/yr   	tons/day		tons/day

Fugitive Dust PM2.5		tons/yr   	tons/day		tons/day

- References

1.  U.S. Environmental Protection Agency, Office of Air Quality Planning and Standards.  Procedures Document for National Emission Inventory, Criteria Air Pollutants, 1985-1999.  EPA-454/R-01-006.  Section 4.8.1.7.2.2.  March 2001.

2.  Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC, Section 11.9, Western Surface Coal Mining October 1998.

3.  U.S. Geological Survey Minerals Yearbook, 2001, Table 1, Nonfuel Raw Mineral Production in New Jersey.

4. Total 2002 employment by 6 digit NAICS code and by county, New Jersey  Department of Labor.

5.  Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC, Section 13.2.2, Unpaved Roads, December 2003.

- 6.  Local Climatological Data 2002, National Climatic Data Center, Newark and Atlantic City, NJ

- Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# MIning and Quarrying - Stone

- SCC: 2325020000

The following describes the emission calculation methodology for this source category for the following pollutants: fugitive dust particulate matter less than or equal to 10 microns (PM10) and fugitive dust particulate matter less than or equal to 2.5 microns (PM2.5). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation methodology sheet.

- Calculation Methodology:

- The following calculation methodology was used for this source category:

1.  USEPA Methodology 1, 2

Emissions for mining and quarrying of stone include: overburden removal, drilling and blasting, loading and unloading, and overburden replacement.  According to the USEPA, "Transfer and conveyance operations, crushing and screening operations, storage and travel on haul roads are not included because these activities are the most likely to have some type of control implemented." 1  In addition, NJDEP has determined that the majority of these types of emissions are included in the point source inventory.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category:

1.  New Jersey statewide stone production 2002, STONE 3

= 31,000 thousand metric tons*1000*1.1023113 =  34,171,650 tons

2.  County and state stone mining employment for NAICS: 21231x, 213115, CEMP, SEMP. 4

3.  Moisture content %, M = 7.9 % 2

4.  Area of blast, AB = 16,000 sf 2

5.  Depth of blast. DB = 5 ft

6.  # blasts =  (STONE * 2000lb/ton) / AB * DB * 163lb/cf  = 4700 blasts

7.  Emissions Factors, EF lb/ton : 1, 2

|  | TSP lb/ton (AP-42 Table 11.9-4) | PM10 equation (AP-42 Table 11.9-1) | PM2.5 equation (AP-42 Table 11.9-1) | PM10 lb/ton | PM2.5 lb/ton |
| --- | --- | --- | --- | --- | --- |
| Overburden removal, OB | 0.058 | PM10 = TSP * (bulldozing overburden scaling factor, 0.75) | PM2.5 = TSP * (bulldozing overburden scaling factor, 0.105) | 0.0435 | 0.0061 |
| Blasting, B |  | PM10 = (0.000014 * (AB)^1.5) * 0.52 * #blasts | PM2.5 = (0.000014 * (AB)^1.5) * 0.03 * #blasts | 0.020 | 0.001 |
| Loading, L |  | PM10 = (0.119/(M)^0.9*)*0.75 | PM2.5 = (1.16/(M^1.2*)*0.019 | 0.014 | 0.002 |
| Unloading end dump, ULED | 0.007 | PM10 = TSP * (bulldozing overburden scaling factor, 0.75) | PM2.5 = TSP * (bulldozing overburden scaling factor, 0.105) | 0.00525 | 0.0007 |
| Unloading bottom dump, ULBD | 0.066 | PM10 = TSP * (bulldozing overburden scaling factor, 0.75) | PM2.5 = TSP * (bulldozing overburden scaling factor, 0.105) | 0.0495 | 0.0069 |

8.  Weekly activity, WAF=7 days/week

9.  Seasonal adjustment factors, SAF

Summer Season Adjustment Factor		1.13

Fall Season Adjustment Factor			0.94

Winter Season Adjustment Factor		1.07

Spring Season Adjustment Factor			0.86

Seasonal production is assumed to be constant throughout the year.  Factor based on number of days with at least 0.01 inches of precipitation and the precipitation control portion of Equation 2 of AP-42 Section 13.2.2. 5  This equation = ((total days - p) / total days), where the total days is equal to the number of days during each season and p is the number of days with at least 0.01 inches of precipitation. 6 The precipitation variations for summer, fall, winter and spring are as follows: 1.13, 0.94, 1.07 and 0.86.

- Process:

- The following equations are used to calculate the emissions without control for this source category:

Equation 4.8-19: 1

Epa = [STONE * (CEMP/SEMP) * (OB + B + L + 0.5*ULED + ULBD)/CF] * DAF

Eps	= [Epa*SAF/AADF] * DAF

Epw	= [Epa*SAF/AADF] * DAF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

Epw	= (tons/day) for a typical winter day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

DAF	= Dust adjustment factor, see "Fugitive Dust Inventory Discussion and Summary" in attachment to 2002 Periodic Emission Inventory.

Assumptions:

1.  Assume a weekly activity of 7 days

2.  Assume depth of blast equal to 5 feet based on conversations with William Layton of the New Jersey Concrete & Aggregate Association and an area of blast based on AP-42.  The size and # of blasts is conservatively overestimated.

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:  None

- Double Counting:

Emissions for this source category regarding overburden removal, drilling and blasting, loading and unloading, and overburden replacement, have not been reported in the point source inventory, therefore no adjustment of the area source emissions was required.  Transfer and conveyance operations, crushing and screening operations, storage and travel on haul roads are included in the point source inventory.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant			Annual		Summer day		Winter day

Fugitive Dust PM10		tons/yr   	tons/day		tons/day

Fugitive Dust PM2.5		tons/yr   	tons/day		tons/day

- References

1.  U.S. Environmental Protection Agency, Office of Air Quality Planning and Standards.  Procedures Document for National Emission Inventory, Criteria Air Pollutants, 1985-1999.  EPA-454/R-01-006.  Section 4.8.1.7.2.2.  March 2001.

2.  Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC, Section 11.9, Western Surface Coal Mining October 1998.

3.  U.S. Geological Survey Minerals Yearbook, 2001, Table 1, Nonfuel Raw Mineral Production in New Jersey.

4. Total 2002 employment by 6 digit NAICS code and by county, New Jersey  Department of Labor.

5.  Compilation of Air Pollutant Emission Factors, AP-42 Fifth Edition, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC, Section 13.2.2, Unpaved Roads, December 2003.

6.  Local Climatological Data 2002, National Climatic Data Center, Newark and Atlantic City, NJ

Area Sources Calculation Methodology Sheets for the 2002 Periodic Emission Inventory

# Agricultural Tilling

- SCC:  2801000003

The following describes the emission calculation methodology for this source category for the following pollutants: fugitive dust particulate matter less than or equal to 10 microns (PM10) and fugitive dust particulate matter less than or equal to 2.5 microns (PM2.5). The emissions are calculated on an annual basis and for a typical summer and winter day for each county and statewide.  Superscripts refer to references listed at the end of each calculation methodology sheet.

- Calculation Methodology:

- The following calculation methodology was used for this source category:

1.  USEPA Methodology 1

Fugitive dust emissions are produced by the mechanical disturbance of the soil during tilling operations.

- Required Input Parameters:

The following input data is required to calculate emissions for this source category:

1.  Acres of Land Tilled by county and crop type (“a”) 2

2.  Number of tillings per year by crop type (“p”) 2

Number of Tillings

Crop			Conservation Use		Conventional Use

Corn				2				6

Spring Wheat			1				4

Rice				5				5

Fall-Seeded Small Grain		3				5

Number of Tillings

Crop			Conservation Use		Conventional Use

Soybeans			1				6

Cotton				5				8

Sorghum				1				6

Forage				3				3

Permanent Pasture		1				1

Other Crops			3				3

Fallow				1				1

3.  Silt content fraction (“s”) = 0.18 3

4.  c	=	constant 4.8 lbs/acre-pass 1

5.  k	=	dimensionless particle size multiplier 1

PM10=0.21

PM2.5=0.042

7.  Weekly activity, WAF=7 days/week

8.  Seasonal adjustment factors, SAF 4

Summer Season Adjustment Factor		1.6

Fall Season Adjustment Factor			1.08

Winter Season Adjustment Factor		0.24

Spring Season Adjustment Factor			1.08

- Process:

- The following equations are used to calculate the emissions without control for this source category.

Epa = [c * k * s0.6 * p * a/CF] * DAF

Eps	= [Epa*SAF/AADF] * DAF

Epw	= [Epa*SAF/AADF] * DAF

Where:

Epa	= (tons/yr) for an annual emission of pollutant by county

Eps	= (tons/day) for a typical summer day emission of pollutant

Epw	= (tons/day) for a typical winter day emission of pollutant

CF	= Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WAF * 52 weeks/year)

SAF	= Seasonal adjustment factor

DAF	= Dust adjustment factor, see "Fugitive Dust Inventory Discussion and Summary" in attachment to 2002 Periodic Emission Inventory.

Assumptions:

1.  Assume a weekly activity of 7 days

- Rule Effectiveness:

- The emissions from this source category are regulated by the following rule:  None

- Double Counting:

Emissions for this source category have not been reported in the point source inventory, therefore no adjustment of the area source emissions was required.

- Output:

- The emissions will be calculated on a county and statewide basis for the following pollutants and seasons:

Pollutant			Annual		Summer day		Winter day

Fugitive Dust PM10		tons/yr   	tons/day		tons/day

Fugitive Dust PM2.5		tons/yr   	tons/day		tons/day

- References

1.  U.S. Environmental Protection Agency, Office of Air Quality Planning and Standards.  Procedures Document for National Emission Inventory, Criteria Air Pollutants, 1985-1999.  EPA-454/R-01-006.  Section 4.8.1.7.2.2.  March 2001.

2.  National Crop Residue Management Survey New Jersey 2000, Conservation Technology Information Center,

http://www.ctic.purdue.edu/CTIC/CRM.html

3. Summary of Approaches Available for Fugitive Dust Sources Technical Memorandum, E.H. Pechan & Associates, for MARAMA, May 21, 2004.

- 4. Seasonal and Monthly Activity Allocation Fractions for Nonroad Engine Emissions Modeling, EPA 420-P-02-010, Table 4, Mid-Atlantic Region.