The purpose of this document is to describe the calculation methodology for marine vessel-fuel combustion. The calculation methodology covers the following pollutants: volatile organic compounds (VOC), oxides of nitrogen  (NOx), carbon monoxide (CO), sulfur dioxide (SOx), particulate matter less then 10 microns (PM10) and particulate matter less then 2.5 microns (PM2.5).  All emissions are calculated on an annual basis.  In addition, VOC, NOx and CO are calculated for a typical summer day and CO, PM10 and PM2.5 are calculated for a winter day.  All emissions are calculated and presented by county.

I. Calculation Methodology Summary:

The United States Environmental Protection Agency (USEPA) recommends two calculation methodologies for the marine vessel-fuel combustion source category.  These recommended methodologies are the:

- Kilowatt power demand analysis; and,

- Fuel use analysis.

While both methodologies are recommended for use by the USEPA, the kilowatt power demand methodology provides a more accurate analysis, since it is based on horsepower demand within the designated port area.  In contrast, the fuel use methodology tends to overestimate commercial marine vessel (CMV) emissions because the fuel use data includes activity conducted outside the nonattainment area (NAA) port(s).  However, in order to utilize the kilowatt power demand methodology, one needs a comprehensive analysis of the CMV operations for the area in question.

The New Jersey Department of Environmental Protection (NJDEP) determined to use a kilowatt power demand analysis for the New York, Northern New Jersey, Long Island Nonattainment Area (hereafter referred to as the New York NAA) because a comprehensive analysis of CMV operations in the New York NAA had already been performed by Starcrest Consulting Group, LLC (hereafter referred to as Starcrest).1   However, since no comprehensive analysis of CMV operations in the Philadelphia, Wilmington, Trenton Nonattainment Area (hereafter referred to as the Philadelphia NAA) had been performed, the NJDEP had to use a fuel use analysis for this NAA.

A full explanation of the kilowatt power demand analysis methodology is included in the Starcrest report listed as Reference #1 in this attachment.  A brief summary of this methodology, as well as a detailed explanation of the fuel use methodology that NJDEP developed, is outlined below.

# II. Starcrest Kilowatt Power Demand Methodology:

Starcrest determined kilowatt power demand in its report by surveying the operations of all Ocean Going Vessels (OGV), towboats and harbor vessels that operate within the New York NAA.  This operational data included:

- CMVs, engines type and number;

- estimated mileage traversed in port channels and anchorages under open ocean;

- harbor transiting, maneuvering, anchorage and docking modes; and,

- fuel types and consumption rates.

The composite emission factors applied to the total kilowatt power demand determined for each CMV category are included in Table 1:

Table 1: CMV Emission Rates (grams pollutant/kW-hr)

| Vessel Type | VOC2 | NOx2 | CO2 | SO22 | PM102 | PM2.53 |
| --- | --- | --- | --- | --- | --- | --- |
| Bulk | 0.59 | 17.96 | 1.62 | 11.91 | 1.66 | 1.66 |
| Car carrier | 0.60 | 15.64 | 1.64 | 11.97 | 1.66 | 1.66 |
| Container | 0.58 | 18.91 | 1.59 | 11.79 | 1.64 | 1.64 |
| Cruise Ship | 0.59 | 13.43 | 1.63 | 12.85 | 1.71 | 1.71 |
| Miscellaneous | 0.62 | 14.27 | 1.69 | 12.24 | 1.71 | 1.71 |
| RORO* | 0.60 | 15.64 | 1.64 | 11.97 | 1.66 | 1.66 |
| Tanker | 0.49 | 15.65 | 1.34 | 12.44 | 1.49 | 1.49 |
| Towboat | 0.61 | 13.32 | 1.69 | 3.92 | 0.51 | 0.51 |

* Roll-on/Roll-off Vessels

The emission results from the application of these emission factors to horsepower demand generated CMV emission results by county4 were directly transferred onto the NJDEP’s inventory calculation sheets.

III. Fuel Use Methodology:

The NJDEP developed a fuel use methodology based on CMV fuel purchases for diesel and residual fuels and the number of trips of self propelled vessels within the Philadelphia NAA as indicated below:

- Required Input Parameters:

The following input data is required to calculate emissions for this source category:

a.	Total residual fuel sold in New Jersey for marine use [FUELR in 10E3 gallon]5

b.		Total distillate fuel sold in New Jersey for marine use [FUELD in 10E3 gallons]5

- Water front length of a municipality [D in miles]6, 7 and 8

- Trips of self propelled vessels => 18 ft and < 18 feet9

- Ni=>18 = self propelled vessels => 18 ft in port I

- Ns=>18 = self propelled vessels => 18 ft in state

- Ni<18    = self propelled vessels < 18 ft in port I

- Ns<18  = self propelled vessels < 18 ft in state

- Emission factor [EF in lbs/103 gallons]:

The emission factors required to calculate emissions for this source category are included in Table 2 below:

Table 2: CMV Emission Factors (lbs/103 gal fuel)

| Pollutant | Residual Fuel | Distillate Fuel |
| --- | --- | --- |
| VOC10 | 0.682 | 24 |
| NOx10 | 55.8 | 550 |
| CO10 | 3.45 | 61 |
| SOx3, 10 | 159 * 2.7% = 429.30 | 157 * 0.25% = 39.25 |
| PM1010 | 20 | 33 |
| PM2.53, 10 | PM10*0.92= 18.40 | PM10*0.92= 30.36 |

3.	Process:

The following equations were used to calculate the emissions (without rule effectiveness) for this source category.

- Fuel Allocation Step:

Fri  = (Ni=>18)/(Ns=>18)

Fdi = (Ni<18 + 2*Ni=>18)/(Ns<18 + 2*Ns=>18)

Where:

Fri  = the apportioning factor for residual fuel used in port I10

Fdi = the apportioning factor for distillate fuel used in port I10

- Municipal Allocation Step:

D = MI*(MAPI/mile)

MUN MILES = D*TRAV TIME*MUN PER

MUN ALLOC = MUN MILES/MUN MILES

Where:

MI 	= Measured inches of municipality port frontage on county map

MAPI 	= County map conversion factor of measured municipal inches per mile

TRAV TIME = percent travel time in port

MUN PER = Percent of Municipality on both banks of port (50 or 100%)

NOTE: For this equation, TRAV TIME equals 0.275 in the inner boundary of port10  and 0.125 in the outer boundary of port10

c.   Emission Calculation Step:

Epa=((EFd*FUELD*PERd*Fdi)+(EFr*FUELR*PERr*Fri))*MUN ALLOC/CF

Eps= Epa/AADF*SAFO

Where:

PERr  = Percent of the statewide residual oil fuel used in the port;

PERd = Percent of the statewide distillate oil fuel used in the port;

Efd     = Emission factor for residual fuel combustion

Efr      = Emission factor for distillate fuel combustion

Epa     = tons per year (tpy) for an annual emission of pollutant

Eps     = tons per day (tpd) for a typical summer day emission of pollutant

CF      = Conversion factor for units = 2000 lbs/ton

WA      = Weekly activity = 7 days/week

AADF  = Annual activity day factor (WA * 52 weeks/year)

SAFO   = Seasonal adjustment factor for the ozone season = 1

Rule Effectiveness:

The emissions from this source category are not regulated by a rule.

Double Counting:

Emissions for this source category have not been reported in the point source inventory, therefore no adjustment of the area source emissions was required.

Output:

The emissions will be calculated on a county basis for the following pollutants - season basis:

| Pollutant | Annual | Summer | Winter |
| --- | --- | --- | --- |
| VOC | Tons/year | Tons/day |  |
| NOx | Tons/year | Tons/day |  |
| CO | Tons/year | Tons/day | Tons/day |
| SOx | Tons/year |  |  |
| PM10 | Tons/year |  | Tons/day |
| PM2.5 | Tons/year |  | Tons/day |

Projection Procedure:

Marine transportation value added data is required to calculate projected emissions for this source category.

Assumptions:

These assumptions only apply to the Philadelphia NAA.  All assumptions applied to the New York NAA are included in the Starcrest report.1

- Using the length of a municipality along the waterfront route taken by self-propelled vessels in transit in the equation that converts distance traveled to time allocates port emissions from marine vessel fuel oil combustion activities to a municipal basis.

- All distillate oil is used by motorships and all residual fuel is used by steamships.10

- Steamship emissions within port area are represented by maneuver/cruise operating mode.

- Hydrocarbons (HC) emissions are equivalent to VOC emissions.

- Distinct New Jersey municipalities located on opposite sides of a waterway each receive 50 percent of the total emissions determined for that portion of a waterway.  Specifically, the ports with distinct New Jersey towns located on opposite sides of their waterway within the Philadelphia NAA include portions of Mantua Creek and the Intracoastal Waterway.

- Emissions on the Delaware River are split between Pennsylvania and New Jersey by assuming that all northbound emissions are in New Jersey and all southbound emissions are in Pennsylvania.  This allocation process was agreed to by the two states as part of the 1990 emission inventory submittal.11

- Transit from Delaware River from “ Philadelphia, PA to the Sea “ includes through traffic to the Port of Camden.  As such, no adjustment was made to make allowance for a separate Camden calculation based on shipping traffic to Camden.

- Transit from Delaware River from “ Philadelphia, PA to Trenton “ includes through traffic to the port of Trenton.  As such, no adjustment was made to make allowance for a separate Trenton calculation based on shipping traffic to Trenton.

- All ships traveling to ports upstream of other designated ports must first travel through the waterways of the downstream port.  This requires that emissions caused by this through travel be included with the regular emissions associated with travel within the designated port.

- The weekly activity factor of 7 days per week for marine vessel loading, ballasting and transit is the same for CMV fuel combustion.

- The seasonal adjustment factor of 1 for marine vessel loading, ballasting and transit is the same for CMV fuel combustion.

- PM10 emissions are equivalent to PM emissions.

- Total fuels sold in 2001 are equivalent to those sold in 2002.

- The total trips of self-propelled vessels in 2001 are equivalent to those determined for 2002.

References:

- The New York, Northern New Jersey, Long Island Non-attainment Area Commercial Marine Vessel Emissions Inventory, Volume 1, Appendix D, The Port Authority of New Jersey, Starcrest Consulting Group, LLC, April 2003.

- Ibid, Volume 2 – Appendices D, Tables D.12, 13, 14, 15 and 16.

- Documentation for Aircraft, Commercial Marine Vessel, Locomotive and Other Non-road components of the National Emissions Inventory, Volume 1 – Methodology, (USEPA, 2002).

- The New York, Northern New Jersey, Long Island Non-attainment Area Commercial Marine Vessel Emissions Inventory, Volume 3, Appendix F, Table F-7, pages F-38 to F-45 of the Starcrest report.

- The Fuel Oil and Kerosene Sales 2001 report, November 2002, Table 23. Adjusted Sales for Transportation Use: Distillate Fuel Oil and Residual Fuel Oil, 2001, page 45, Energy Information Administration, Office of Oil and Gas, US. Department of Energy, Washington DC.

- The Ports of New York, NY and NJ and Ports on Long Island, NY, Port Series No. 5, Revised 1988, US Army Corps of Engineers.

- The Ports of Philadelphia, PA, Camden, NJ, Wilmington, DE, and Ports on Delaware River, Port Series No. 8, Revised 1984, US Army Corps of Engineers.

- County Maps by Hagstrom or Alfred B. Patton.

- Waterborne Commerce of the United States, Calendar Year – 2001 Part 1, Waterways and Harbors, Atlantic Coast, US Army Corp of Engineers, Water Resources Support Center, July 2003.

- Radian Corporation Memorandum from James Labanowski to Wiley Barbour, Spatial Allocation of Commercial Marine Vessel Emissions for SIP Base Year Inventories, June 9, 1993.

11. State Implementation Plan (SIP) Revision for the Attainment and Maintenance of  the         Ozone National Ambient Air Quality Standards, Meeting the Requirements of the                Alternative Ozone Attainment Demonstration Policy, Appendix VI: 1990 Base Year              Emission Inventory, The State of New Jersey Department of Environmental Protection,       December 31, 1996.