The purpose of this document is to describe the calculation methodology for the following source categories: air carrier (commercial), air taxi, military and general aviation aircraft.  The calculation methodology covers the following pollutants: volatile organic compounds (VOC), oxides of nitrogen  (NOx), carbon monoxide (CO), sulfur dioxide (SOx), particulate matter less then 10 microns (PM10) and particulate matter less then 2.5 microns (PM2.5).  All emissions are calculated on an annual basis.  In addition, VOC, NOx and CO are calculated for a typical summer day and CO, PM10 and PM2.5 are calculated for a winter day.  All emissions are calculated and presented by county.

I. Calculation Methodology Summary:

The United States Environmental Protection Agency (USEPA) recommends two calculation methodologies for these aircraft source categories.1  These recommendations are to:

- Survey individual airports to obtain data on every aircraft landing and takeoff  (LTO) and Touch and Go (TGO) operation (i.e. fleet mix population) for model input.

- Calculate emissions based on total airport activity for commercial air carrier (AC), air taxi (AT), military (MI) and general aviation (GA) and apply the applicable aircraft emission factors for these categories.

The six major civilian airports (Atlantic City, Essex County, Morristown Municipal, Newark, Teterboro and Trenton Mercer) and Lakehurst Naval Air Engineering Station (hereafter referred to as Lakehurst) supplied data on every aircraft LTO and TGO operation. This information was input into the Federal Aviation Agency (FAA) Emissions and Dispersion Modeling System (EDMS)2 to determine VOC, NOx, CO and SOx emissions.

Specific Air Force emission factors were applied to LTO and TGO operations for every aircraft model included within McGuire Air Force Base (AFB) fleet mix population.

The USEPA default emission factors were applied to total AC, AT, MI and GA LTO operations conducted at every airport that did not provide NJDEP with fleet mix data.   Also, the USEPA default emission factors were used to determine PM10 and PM2.5 emissions for every airport because the EDMS model does not calculate particulate matter emissions.

II. Required Input Parameters:

The following input data is required to calculate emissions for these aircraft source categories:

1.	APPLICABLE AIRPORTS: Specifically, the required input data is a listing of all Federal Aviation Agency (FAA)-operated airports with their number of aircraft operations in New Jersey3.  This information is obtained by first going the to the website www.apo.data.faa.gov and selecting "TAF" (which stands for "Terminal Area Forecast") from the menu.  Once there, the next step is to perform a query for the State of New Jersey and for the year 2002.  This will generate the necessary list.  See the APO terminal Area forecast detail report included  in Sub-Attachment A for the presentation of raw FAA LTO data.  This raw data was formatted to conform include only LTOs conducted for itinerant and local operations as described below in Section II.2.f

It is also necessary to contact the New Jersey Department of Transportation (NJDOT) directly to confirm the continued operations of the smaller airports included in the 1996 inventory if they are no longer included in FAA listing or listed with no operations4, 5.  In addition, the NJDOT can provide a listing of all operating heliports in New Jersey 6, 7.

2.	FLEET MIX POPULATION FOR APPLICABLE AIRPORTS: This information is obtained by conducting a survey of 6 major FAA-operated airports within New Jersey, as well as McGuire AFB and Lakehurst.  The airports indicated below submitted the necessary information to develop their fleet mix population.   Their fleet mix population constitutes the number of LTO and/or TGO operations for every aircraft model type within each of the 4 major aircraft categories (AC, AT, MI and GA).

a.  	Newark8  and Teterboro9 airports provided fleet mix LTO data for the periodic emission inventory year 2002.  This LTO data could be input directly into EDMS.

b.  	Lakehurst10 provided monthly LTO fleet mix data that had to be compiled            to obtain an annual total.   See Table LH-1 in Sub-attachment A of this                  Attachment for fleet mix calculations.

c.   	Picatinny Arsenal11 provided annual operations. See Table  P-1 in 	Sub-attachment A.

d.	Atlantic City3, 11,12,13,14, Morristown Municipal3,15 and Trenton Mercer3, 11 & 16 airports provided LTO data developed from a baseline year other than 2002.  This data was scaled to correspond to the year 2002 based on total FAA operations of the four aircraft categories as indicated by the attached worksheets referenced below for each of these airports. See Tables AC-1, AC-2, AC-3 and AC-4 for fleet mix calculations for Atlantic City Airport (ACY), Table MMA-1 for the fleet mix calculations for Morristown Municipal Airport (MMU) and Table TMA-1 for fleet mix calculations for the Trenton Mercer Airport.  All of these tables are located in Sub-attachment A.

e.	Essex County Airport3 ,11 and McGuire Air Force Base (AFB)17.18,19 provided LTO data based on percentages of LTO operations for each aircraft type model included in its fleet mix population.  LTO operations were obtained by multiplying these LTO percentages for each specific aircraft type model by  the total LTO operations conducted at each respective airport.  See Table ECA-1 in Sub-attachment A of this Attachment for the fleet mix calculations for the Essex County Airport.  See Tables LTO-2, 3A and 3B in Sub-attachment B of this Attachment for the fleet mix calculations McGuire AFB.

f.	All remaining airports included in FAA aircraft list used the following methodology to obtain total LTO values for their AC, AT, MI and GA aircraft:

STEP 1: Compile all itinerant and local aircraft operations included in the FAA-operated airport listings3.   The necessary FAA information includes:

Itinerant operations: The departure or the arrival of an aircraft at the airport, i.e. scheduled routes where the aircraft will arrive or land at a different airport.  Includes all AC, AT,MI and GA operations.

Local operations: aircraft LTOs at the same airport, i.e. training exercises.  Includes only GA and MI operations.

STEP 2: Obtain total LTO operations for all aircraft categories by first adding itinerant and local GA and ML operations and then dividing all AC, AT, GA and Ml operations by 2.  See table 1 for an example of this step.

Table 1: FAA Aircraft Operations Compilation

| Itinerant Operations |  |  |  |  |  |  | Local Operations |  |  | Calculated Values |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Airport | FAA ID | AC | AT | GA | MI | ## Total | GA | Ml | Total | Total Ops | LTO | ACtot | ATtot | GAtot | MItot |
| TRINCA | 13N | 0 | 0 | 6984 | 0 | 6984 | 4702 | 0 | 4702 | 11686 | 5843 | 0 | 0 | 5843 | 0 |

In this table:

Total Operations

= Total Local Operations + Total Itinerant Operations

Landings and Take Offs (LTO)

= Total Operations/2

Total Air Carriers (ACtot)

= AC Itinerant operations (ACio) + AC Local operations (AClo )

Total Air Taxis (ATtot)

= AT Itinerant operations (ATio) + AT Local operations (ATlo )

Total General Aviation (GAtot)

= GA Itinerant operations (GAio) + GA Local operations (GAlo )

Total Military (MItot)

= MI Itinerant operations (MIio) + AC Local operations (MIlo )

In Sub-Attachment A refer to Table FAA-1 for the presentation of  the complete formatted compilation of  itinerant and local operational faa LTO data and to Table FAA-2 for the presentation of the final calculated total LTO values for AC, AT, MI and GA aircraft.

For those the airports included in the 2002 emission inventory that were not listed on the FAA operational listing but were listed as still operational by the NJDOT, the NJDEP used the same LTO as in 1996.5

All Heliports data was supplied by NJDOT.6, 7  An average of 20 LTOs was assumed for all heliports.  A Bell 206 helicopter was used to represent commercial heliports and a SK 60B was used represent hospital, military and state police heliports.  Helicopter emissions were calculated with EDMS and included in the GA aircraft category.

III. Emissions Calculation:

The FAA EDMS and USEPA default emission factor calculation methodologies are presented below.  EDMS is used to calculate emissions from the six major airports that supplied data on every aircraft LTO and TGO operation.  While the USEPA default emission factors are used for all the other airports that only provided total LTO counts to the FAA.  In addition, USEPA emission factors were used to calculate PM emissions from all airports because EDMS does not calculate PM emissions.

EDMS METHODOLOGY:

STEP A: Create an airport emission inventory in EDMS.  To do this, follow the instructions outlined below:

- Open the EDMS model.  Create a new study for the airport in question by clicking  “setup” under FILE.  Enter the parameters (FAA airport identification code and average yearly temperature20) that are specific to each airport included in the study. Utilized default values, including the default 3,000 feet value established for mixing height, unless the airport has provided specific values for use.  None of the airports investigated for 2002 had specific information.

- Input fleet mix LTO and TGO data and Time in Mode (TM) data.  Then go to EMISSIONS and click on “Available Aircraft Operations and Assignments”.  Once there, go to  “available aircraft/engines” and click on the default engine for the applicable aircraft (NOTE: the default engine will be in boldface type.  If no engine is in boldface type, then chose any reasonable engine for that aircraft based on available references21, 22).   Next, indicate the AC, AT, MI or GA category aircraft in the identification box.  (NOTE: the same aircraft/engine combinations operate in more then one category at the same airport).  Next, click “Operations” and input the total  “yearly LTO” and  “TGO” values for that specific aircraft under the AC, AT, MI or GA category designations.  Finally, click on “Times in mode” and select “total taxi and queue time” to input this value.  The total taxi and queue time for every aircraft is as follows:

Taxi Total Time = Taxi in + Taxi Out + Queue Time

The following are the Taxi Total Times by airport used in generating the inventory:

- Newark Airport (EWR) = Origin Airport all carriers all flights (Average scheduled departure to Take-off (minutes)) + destination airport all carriers all flights (Average Taxi-In (minutes)) =  27.01 + 8.43 =  35.3523

- Teterboro (TEB) = 19 minutes24

- Lakehurst, Atlantic City, Morris Municipal, Trenton Mercer, and Essex County airports = 9 minutes25

- To run the report, click on EMISSIONS and then on “Run Emissions Inventory”.

STEP B: Import the EDMS results from Step A above into NJDEP file to determine HC, NOx, CO and SOx emissions. To do this, follow the instructions outlined below:

- Import AIR_EMIS.dbf and AIR_POP.dbf into an excel worksheet.    A sample of this table is provided in Table 2 below:

Table 2: AIR_EMIS.dbf

| AIR_NAME | ENG_NAME | INST_NAME | MODE | CO_ TONS | HC_ TONS | NOX_ TONS | SOX_ TONS | PM10_ TONS | FUEL_ TONS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A310 | CF6-80A3 | AC | APCH | 0.836 | 0.134 | 3.224 | 0.299 |  | 298.560 |
| A310 | CF6-80A3 | AC | CLMB | 0.213 | 0.072 | 5.142 | 0.193 |  | 193.296 |
| A310 | CF6-80A3 | AC | TKOF | 0.284 | 0.085 | 8.408 | 0.284 |  | 284.045 |
| A310 | CF6-80A3 | AC | TAXI | 18.777 | 4.182 | 2.264 | 0.666 |  | 665.857 |
| A310 | CF6-80A3 | AC | APU | 0.224 | 0.023 | 0.516 | 0.054 | 0.000 |  |
| A310 | CF6-80A3 | AC | GSE | 50.545 | 2.036 | 2.985 | 0.295 | 0.138 |  |

- Sort file for emission inventory preparation.  To do this, perform both a mode and inst_name sort on AIR_EMIS.dbf.  This sort separates the file data into its mode components of APCH, CLMB, TKOF, TAXI, APU and GSE and its inst_name aircraft category components of AC, AT, MI and GA. This sort also allows for the deletion of GSE (Ground Support Equipment) mode data, which is deleted because airport GSE emissions are already included in a separate emission inventory category.   Next, copy and paste the applicable subsets of this sorted data into their respective AC, AT, MI or GA category emission spreadsheets.  This accounts for HC, NOx, CO and SOx emissions for each of these aircraft categories.

- Convert HC to VOC emissions by using the appropriate conversion factor in the following equation:

VOC = HCEDMS (TONS)*HC to VOC conversion factor

The VOC conversion factors are included in Table 3 below:

Table 3: HC to VOC Conversion Factors25

| Aircraft category | VOC Conversion factor |
| --- | --- |
| Air Carrier (AC) | 1.0947 |
| Air Taxi (AT) | 0.9914 |
| Military (MI) | 1.1046 |
| General Aviation (GA) | 0.9708 |

STEP C: Import total LTO and TGO counts from EDMS file into NJDEP file to determine PM10 emissions. To do this, follow the instructions outlined below:

- Import AIR_POP.dbf  into an excel worksheet.  A sample is provided in Table 4.

Table 4: AIR_POP.dbf

| AIR_NAME | ENG_NAME | MODE1 | MODE2 | MODE3 | MODE4 | ANNUAL_OPS | PEAKHR_OPS | TGO_OPS | INST_NAME | AA_TAXI/Q |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A310 | CF6-80A3 | 3.77 | 0.83 | 1.02 | 35.93 | 934 | 0.107 | 0 | AC | 35.35 |
| A319 | CFM56-5B6/P | 3.79 | 0.94 | 0.92 | 35.94 | 3312 | 0.378 | 0 | AC | 35.35 |
| A320 | V2527-A5 | 3.99 | 1.01 | 0.99 | 35.72 | 4427 | 0.505 | 0 | AC | 35.35 |
| A321 | CFM56-5B3/P | 3.73 | 1.07 | 1.03 | 35.96 | 247 | 0.028 | 0 | AC | 35.35 |
| A330-300 | PW4168 | 4.14 | 1.01 | 1.08 | 35.76 | 576 | 0.066 | 0 | AC | 35.35 |
| A340-200 | CFM56-5C2 | 3.83 | 1.59 | 1.60 | 36.11 | 6 | 0.001 | 0 | AC | 35.35 |

2. 	Sort this file for emission inventory preparation.   To do this, perform a inst_name sort on AIR_POP.dbf file to separate this data into its AC, AT, MI and GA category components.  Then copy and paste the applicable subsets of this sorted data into their respective AC, AT, MI or GA category emission spreadsheets.  Finally, use the applicable USEPA emission factor based on LTO activity determines PM10 emissions (see Table 5 below in III.2.A) as shown in the equation outline in Step D below.   PM2.5 emissions are assumed to represent 89 percent of PM10 emissions.26

STEP D: Final Emissions Calculation Step.  To do this, follow the equation below:

Epaedms = EDMS model AIR_EMIS.dbf  = VOC, NOx,CO & SOx (tons/year)

Epaedmspm = (total LTO+TGO sum from EDMS model POP_EMIS.dbf)/2 * PM_EF/CF = PM10 (tons/year)

Epaedms  = PM10 emissions * 0.89  = PM2.5 (tons/year)

Where:

Epaedms = Annual emission generated by EDMS for VOC, NOx, CO & SOx for aircraft categories AC, AT, MI and GA

Epaedmspm  = Annual emissions for PM based on total LTO number included in EDMS for aircraft categories AC, AT, MI and GA

CF = Conversion factor for units = 2000 lbs/ton

USEPA DEFAULT EMISSION FACTOR METHODOLOGY:

STEP A: Apply the USEPA Factor to the FAA Compilation.  To do this, follow the instructions outlined below:

- Multiply LTO operations by the USEPA Emission Factor (EF) (lbs of pollutant/LTO cycle) (see Table 5 below), as indicated by the equation below to calculate HC, NOx, CO, SOx, PM10 and PM2.5 emissions.

Epafaa   =  LTO*EF/CF

Where:  Epafaa = emissions from Airports included in FAA operational listing

Note that to calculate VOC emissions by applying the HC EF included in Table 5, one must first use the applicable VOC conversion factor as explained above in Step B of the EDMS Methodology (#3) by using the following equation:

VOC = HCEDMS (TONS)*HC to VOC conversion factor.

See table 3 above for the conversion factors.

Table 5: USEPA Emission Factors for LTO Emissions

| AIRCRAFT | HC25 | NOx25 | CO25 | SOx25 | PM1025 | PM2.526 |
| --- | --- | --- | --- | --- | --- | --- |
| Air Carrier | Not appl | Not appl | Not appl | Not appl | 0.6033 | 0.89*PM10 |
| Air Taxi | 1.2340 | 0.1580 | 28.1300 | 0.0150 | 0.6033 | 0.89*PM10 |
| Military | 1.2340 | 0.1580 | 28.1300 | 0.0150 | 0.6033 | 0.89*PM10 |
| General Aviation | 0.3940 | 0.0650 | 12.0140 | 0.0100 | 0.2367 | 0.89*PM10 |

- Calculate McGuire Air Force Base Emissions.  To do this, use the United States Air Force (USAF) methodology25, 26, 27, 28, 29, 30, 31, 32 for all military aircraft emissions except for particulate matter (PM) emissions for military aircraft C12 and C21A.  The USEPA default AT PM EF was applied to these aircraft operations in addition to civil AC operations conducted at McGuire AFB.  The EDMS model methodology outlined above was performed to determine VOC, NOx, CO and SOx emissions for all civil aircraft operations.  See Tables # LTO 1, 4, 5 and 6 in Subattachment B of this Attachment for a more detailed explanation of McGuire AFB emission calculation methodology.

The total aircraft emissions from McGuire AFB are expressed as follows:

Epamcg = McGuire AFB emissions (See table # LTO 8 in Subattachment B)

- Combine the emissions for each aircraft category.  The following equations are used to calculate the final emissions (without rule effectiveness) for the source categories of AC, AT, MI and GA.  The first equation combines emissions from the EDMS and USEPA Emission Factor methodologies for each individual aircraft category within their respective excel spreadsheets.

The reader should note that the EDMS emissions from each aircraft were sorted by their inst_name of AC, AT, MI and GA to allow for the import of these emissions into their respective worksheets as explained above in III.1.B.3.  The USEPA emission factor methodology did not require this sort because this methodology does not use a fleet mix population but instead uses total LTO counts of AC, AT, MI and GA for each airport as indicated above in III.1.D and III.2.A.

EMISSION CALCULATIONS

Epa  =

Epaedms or  Epaedmspm/CF or Epamcg/CF or + Epafaa/CF for each AC, AT and GA category

Eps  = Epa/AADF*SAFO

Where:

Epa  = (tons/year) for an annual emission of pollutant

Eps  = (tons/day) for a typical summer, fall, winter and spring season          day emission of pollutant

CF  = Conversion factor for units = 2000 lbs/ton

AADF	= Annual activity day factor (WA * 52 weeks/year)

Assumptions:

- Aircraft arrivals and departures included in FAA Air Traffic Activity for Fiscal Year 2002, after division by 2, corresponds to LTO AND TGO operations.

- The seasonal adjustment factor for all summer and winter seasons is 1.

- Weekly activity (WA) equals 7 days per /week except as specified for McGuire AFB.

- Aircraft containing the same designation of prop, jet, turbo and number of engines and similar size and trajectories will generate similar emissions in EDMS.  The reader should note that EDMS did not contain all aircraft types so similar aircraft were substituted for designated aircraft in fleet mix population from an available aircraft reference listing.21, 22

- Default aircraft engines selected in EDMS for aircraft types represent a typical engine for that aircraft.

- The time-in-modes for taxi in and out with queue time as stated above in III.1.A.2  are approximately an average value for all aircraft operations at each airport.

- The default mixing height of 3000 ft is an average value for all aircraft operations at each airport utilizing the EDMS model.

8.    USEPA default emission factors for air taxis can also be applied to military aircraft.

Rule Effectiveness:

The emissions from this source category are not regulated by a rule.

Double Counting:

Emissions for this source category have not been reported in the point source inventory, therefore no adjustment of the area source emissions was required.

Output:

The emissions will be calculated on a municipality basis for the following pollutants - season basis.

| Pollutant | Annual | Summer | Winter |
| --- | --- | --- | --- |
| VOC | tons/year | tons/day |  |
| NOx | tons/year | tons/day |  |
| CO | tons/year | tons/day | tons/day |
| SOx | tons/year |  |  |
| PM10 | tons/year |  | tons/day |
| PM2.5 | tons/year |  | tons/day |

Projection Procedure:

Projected LTO operations data from the FAA website  www.apo,data.faa.gov3  (obtained by querying TAF for the state of New Jersey and for the years 2002 to 2017) is required to calculate projected emissions for this source category

References:

- Procedures for Emission Inventory Preparation Vol IV: Mobile Sources, 1992, United States Environmental Protection Agency, Office of Air Quality Planning and Standards, Research Triangle Park, NC, EPA-450/4-81-026d Revised

- Emissions and Dispersion Modeling System (EDMS) version 4.12 for windows, developed by CSSI, inc. for the FAA office of Environment and Energy, October 2003, Washington D.C. (www.aee.faa.gov/emissions/edms).

3.	APO TERMINAL AREA FORECAST DETAIL REPORT, Federal Aviation Administration, Office of Aviation Policy, Plans and Management Analysis (www.apo.data.faa.gov).

4.	Proposed State Implementation Plan (SIP Revision) for the Attainment and         Maintenance of the Ozone National Ambient Air Quality Standard (NAAQS), New Jersey 1996 Actual Emission Inventory and Rate of Progress (ROP) Plan for       2002, 2005 and 2007, Appendix II: Inventory Projections for 1999, 2002, 2005     and 2007, Attachment II-R:Projection of New Jersey Aircraft Emissions: VOC,     NOx and Carbon Monoxide, December 31, 2000.

5.	Memo to file concerning Airports from 96 inventory still operating but not listed with operations by FAA, February 23, 2004.

- Memo to file concerning heliports operating in New Jersey, March 2, 2004

- March 9, 2004 email from John Kaiser of NJDOT (John.Kaiser@dot.state.nj.us , 609-530-2910) containing excel spreadsheet of 321 licensed helistop facilities in NJ.

- EWR Flight Movements CY2002 from May 27, 2003 email submittal from David Stearns of the Port Authority.

- Teterboro Airport Operations 2002 from May 28, 2003 email submittal from David Stearns of Port Authority.

- Air traffic Activity report for NAES Lakehurst by AC1 Leckbee from May 3, 2003 email submittal from Mike Figura (732-323-4857).

- May 28, 2003 Memo to file concerning aircraft activity at Mercer, Picatinny, Morristown, Atlantic City and other airports.

- Atlantic City International Airport Environmental Impact Statement, Air Quality Technical Report, EDMS 3.23 Study Information 2000 base, Orion International Technologies, Inc., January 2001.

- Ibid, Appendix A, Detailed Aircraft Input Assumptions, Baseline Assumptions.

- January 5,2000 Memo to file including misc military aircraft fleet mix spreadsheet developed from information provided by Patrick Ream of Atlantic City Airport.

- April 30, 2003 Fax from Kerry Ahearn of D.M. Airport Developers, Inc., Table 3: Modeled Aircraft for 2003.

- May 28, 2003 Fax from Nancy Neuman of Sandstone Environmental Associates, Inc. Sandstone Environmental Associates, Inc.

- April 2, 2004 email from Kim MacDonald of McGuire AFB concerning fleet mix at McGuire AFB.

- April 7, 2004 email from Kim MacDonald of McGuire AFB concerning further clarifications on fleet mix at McGuire AFB.

- April 8, 2004 memo to file concerning phone conversation with Kim MacDonald of McGuire AFB on commercial aircraft and general aviation aircraft activity at McGuire AFB.

- State Seasonal Averages for 2002 from data obtained from the National Oceanic and Atmospheric Administration, Local Climatological Data, Newark NJ 2002.

- Chapter 5.  Civil/Military Aircraft Type Designators, June 1, 1988.

- World Jet Inventory Year end 2002, Jet Information Services, Inc. March 2003 (http://www.jetinventory.com).

- U.S. Department of Transportation, Bureau of Transportation Statistics, Summary Statistics for Origin and Destination Airport for Newark International (EWR) from January 1, 2002 to December 31, 2002.  (http://www.bts.gov/xml/ontimesummarystatistics)

- December 12, 2003 Memo to File, Aircraft Taxi-out and In with Queue Time.

- Procedures for Emission Inventory Preparation, Volume IV:Mobile Sources, USEPA, 1992.

- Documentation for Aircraft, Commercial Marine Vessel, Locomotive, and Other Non-road Components of the National Emissions Inventory, Volume I – Methodology, Appendix A, (USEPA, 2002).

- June 5, 2003 Email from Kim MacDonald of McGuire AFB concerning Time-in-Mode values for McGuire military aircraft.

- Air Emissions Inventory Guidance Document for Mobile Sources at Air Force Installations, US Air Force IERA, January 2002.

- April 2, 2004 email from Kim MacDonald of McGuire AFB concerning updated LTO & TGO values for McGuire AFB military aircraft.

- April 7, 2004 email from Kim MacDonald of McGuire AFB concerning further clarifications on updated LTO & TGO values for McGuire AFB military aircraft.

- April 8, 2004 memo to file concerning phone conversation with Kim MacDonald of McGuire AFB on commercial aircraft and general aviation aircraft activity at McGuire AFB.

- Environmental Assessment of C-17 basing at McGuire AFB, New Jersey, Attachment C-3, page 3-39 concerning flight days for on base and transient aircraft, USAF, Air Mobility Command, March 2002.