The purpose of this document is to describe the calculation methodology for line haul locomotives classes 1 and 2 & 3, passenger trains (e.g., Amtrak), commuter lines and yard locomotives.  The calculation methodology covers the following pollutants: volatile organic compounds (VOC), oxides of nitrogen  (NOx), carbon monoxide (CO), sulfur dioxide (SOx), particulate matter less then 10 microns (PM10) and particulate matter less then 2.5 microns (PM2.5).  All emissions are calculated on an annual basis.  In addition, VOC, NOx and CO are calculated for a typical summer day and CO, PM10 and PM2.5 are calculated for a winter day.  All emissions are calculated and presented by county.

I.  Calculation Methodology Summary:

The United States Environmental Protection Agency (USEPA) recommends two calculation methodologies for these railroad source categories.  These recommendations are to:

- Survey individual railroad companies to obtain total fuel consumption on state level and gross ton miles traveled on rail tracks.

- Calculate emissions based on the estimated amount of fuel used by railroads operating in an area.

In generating the 2002 base year inventory, the NJDEP utilized both of the USEPA's recommendations.  All railroad companies submitted information in response to a survey conducted by NJDEP.  However, some of this information did not include total fuel consumption amounts. These amounts had to be calculated from the submission of other activity data by the rail lines.  Different activity data, allocation methods and emission factors have also been utilized to calculate emissions at the county level for various railroad categories as explained in Section II below.

Table 1 shows the six different railroad types that are classified under five USEPA Standard Classification Codes (SCC) categories.  These railroads either operate on their own rail lines or share a rail line with other trains.  Sufficient activity data exists to calculate emissions for all their railroad operations except for Class 1 category trains that operated on the Conrail Shared Asset (CSA) line.  Conrail (CR), however, submitted the necessary activity data for these trains in addition to their own trains for all activity conducted on these lines.  A step by step explanation of how railroad emissions are calculated for each railroad category or company and the CSA line is included in Section II below.

# Table 1: Railroad SCC Codes and Categories

| SCC code | Railroad Category | Railroads |
| --- | --- | --- |
| 2285002006 | Nationwide line-haul class R-1 railroad operations | Norfork Southern (NS) and CSX |
| 2285002007 | Regional line haul class R-2 and R-3 railroads | CSA, NY Susquehanna Western (NYSW) Freight railroad, Canadian Pacific (CP) and all the short-line railroads. |
| 2285002008 | Passenger Train | Amtrak |
| 2285002009 | Commuter Lines | NJ Transit |
| 2285002010 | Yard Locomotives | Yard rail operations at CR, CSX, NS and NYSW railroads. |
| 2285002010 | Yard Locomotives | Short-line switching railroads |

II. Required Input Parameters and Emission Calculations:

The following emission factors, activity data, adjustments and calculations are required to calculate emissions for each railroad category, company and CSA line:

## Emission Factors (EF):

## Table 2 summarizes fuel based emission factors (EF) for all railroad categories.  These EFs are applied to railroad fuel consumption.  Not all railroads could provide this information.   Some railroad companies could only provide activity data from which statewide fuel use could be determined and then allocated to the county level.  A step by step analysis of how this was accomplished for each Railroad Company and category is explained below in Section II.2 beginning with an examination of Class 1 railroads.

Table 2: Railroad Emission Factors (pounds of pollutant/gallons of fuel)1

| Pollutant | Line haul class 1 & 2-3, Passenger & Commuter Locomotives (EFf) | Yard switch Locomotives (EFy) |
| --- | --- | --- |
| VOC | 0.02205 | 0.04630 |
| NOx | 0.59525 | 0.79807 |
| CO | 0.05864 | 0.08400 |
| SOx | 0.03721 | 0.03721 |
| PM10 | 0.01477 | 0.02028 |
| PM2.5 | 0.01329 | 0.01825 |

- Nationwide Line-Haul Class 1 Railroad Operated on Company Designated and/or Owned and Operated Rail Lines (CSX and Norfork Southern Railroads):

These calculations only include CSX and NS railroad operations conducted on the rail lines that either they or New Jersey Transit own and/or operate.  CSX and NS railroads also operate on the CSA tracks.  A separate calculation based on input data provided by CR was performed for these railroads operations on CSA rail lines as discussed in Section II.4.

A. 	Activity Data Collection:

i.	Railroad activity data was obtained from the national and state level activity data for line haul operations.  This information is summarized in Table 3:

##### Table 3: National and State Level Activity Data for Line Haul Railroad Activity

| Activity Description | Activity Symbol | Activity Units | NS Class 1 Railroad2 | CSX Class 1 Railroad3 |
| --- | --- | --- | --- | --- |
| National diesel fuel use line haul freight operations | FUELHAUL_US | 10E3 Gallons | 433,678.71 | 514,107.57 |
| National gross ton Miles | GTM_US | 10E6 tons of freight & car * miles of rail | 373,281.20 | 469,392.73 |

- Additional state level activity data specific to each rail line that NS and CSX own and/or operate in the state is included in Table 4.  This does not include any rail activity on the CSA line.  CSA line activity is handled separately and discussed in Section 3 below.  Note that Table 4, however, does include references to the CSA Excel spreadsheet in addition to communications with CSX and NS.  This is because although neither of these railroads provided complete activity data concerning their own operations on their designated rail lines, Conrail did provide records for all freight gross tonnage transported on their CSA lines.  Thus it was necessary to refer to the CSA Excel spreadsheet to fill in these gaps.

# Table 4: Class 1 Railroad Line Haul Activity on Company Designated and/or Owned and Operated Rail Lines

| Railroad Line | Class 1 Railroad | Freight Gross Tons Transported over Railroad Line (GT_LINE) 10E6 | Total Rail Line Miles (MILES_LINE) |
| --- | --- | --- | --- |
| Lehigh-14 | NS | 19.20 | 43.35 |
| Nat Dock-North 2-Southern Tier4 | NS | 8.20 | 30.45 |
| Morristown- Boonton5 | NS | 0.36 | 28.20 |
| Morristown-Morristown5 | NS | 0.36 | 28.00 |
| High Bridge to Bound Brook5 | NS | 0.07 | 21.90 |
| River line CP2 to Stateline CP18.86 | CSX | 59.16 | 16.80 |
| West Trenton line6 | CSX | 33.10 | 24.80 |
| Northern Line6 | CSX | 33.10 | 21.00 |

The rail line miles included in Table 4 represent the mileage traveled by the designated freight gross tonnage between the rail line points of origin and departure and/or termination within the State of New Jersey.  This does not include any operations on the CSA rail lines.  This measurement was determined using a tape measure on each rail lines located on the scaled New Jersey geographic atlas7 and Hagstrom county maps8.  Miles of rail within each county (MILES_CTY) were also determined in this manner, as this procedure was used on every railroad line in New Jersey.

Note the atlas and county maps do not designate either the name of the line or even the company that operates the line, even though they include all the rail lines in New Jersey and designate some of the yard depots and passenger terminals.  Additional sources of information were used to determine which rail lines on the atlas and maps represent CSX, NS and the other railroads.  A summary of these sources, and the information provided by these sources, is as follows:

- Not to scale Northern New Jersey Local Freight Service (LFS) map9 includes a color scheme indicator of all Class 1 tracks that conduct freight operations in New Jersey.  This map was sufficient to identify different railroad company operations on the unnamed rail lines located on a scaled NJ atlas or county maps.

- Not to scale NJ Transit Passenger Rail System map10 includes a color scheme indicator of all tracks that conduct commuter service operations on rail lines on which Class 1 railroads also operate.

- A CSA excel spreadsheet identified the CSX and NS rail line junctures with the CSA rail lines.11

The activity data included in Tables 3 and 4 was applied to allocate national fuel use from the national to the county level for all rail operations conducted by CSX and NS.  This allocation process is discussed in further detail in Section B.

- Allocation Process:

- Allocation of Class 1 railroad line haul activity from a national to a county level.

Step 1:	Develop Fuel Consumption Index (FCI) from national activity data to convert county level Gross Ton Miles (GTM) to county level fuel use.

FCI  =  GTM_US/FUELHAUL_US

Where:

GTM_US 	      = National gross ton miles

FUELHAUL_US = National diesel fuel use

for line haul freight operations (gals)

Step 2: 	Allocate gross tonnage transported by each railroad line running through several counties to individual county level by determination of  the rail line mileage within an individual county: (MILES_CTY).

MILES_CTY  =  MI*MAPI

Where:

MI	  = Measured inches of track within each individual                                                county on county map (in).

MAPI  = County map conversion factor for inches to miles                        				      (in/mile).

Note that MILES_CTY must equal total MILES_LINE for each rail line included in Table 4 above.

Step 3:	Determine Fuel Use at the County Level (FUEL_CTY)

FUEL_CTY = MILES_CTY*GT_LINE/FCI

Where:

MILES_CTY   =  Rail line mileage within

individual county (Miles)

GT_LINE	 =  Freight gross tons (GT) transported over

railroad line

FCI 		 =  Fuel Consumption Index = Fuel/GTM (gal/GTM)

- Emission Calculation:

Epa = EFf * FUEL_CTY/CF

Eps = Epa/AADF * SAFO

Where:

Epa     = Annual emission of pollutant (tons/yr)

Eps     = Typical summer day emission of pollutant (tons/day)

AADF = Annual activity day factor (WA*52 weeks/year)

CF      = units conversion factor (2000lbs/ton)

Weekly activity (WA) =7 days/week

Seasonal adjustment factor (ozone season), SAFO = 1

Refer to Tables CSX-1 & 2 and NS-1 & 2 in Sub-Attachment C for the complete calculations performed for these Class 1 Railroads.

- Nationwide Yard Switching Operations at Class 1 Railroad Company Designated and/or Owned and Operated Yard Depots (CSX and NS Railroads):

These calculations only include railroad operations conducted at yard depots designated as owned and/or operated by CSX and NS.  These railroads also conduct switching yard operations on CSA yard depots.   A separate calculation based on input data provided by CR was performed for these operations on CSA yard depots as explained below in Section II.4.

### A.	Activity Data Collection:

- Activity data was obtained from the national activity data for yard switching operations included in Table 5:

##### Table 5: National and State Level Activity Data for Yard Railroad Activity

| Activity Description | Activity Symbol | Activity Units | NS Class 1 Railroad3 | CSX Class 1 Railroad4 |
| --- | --- | --- | --- | --- |
| National diesel fuel use Yard switch operations | FUELYARD_US | 10E3 gals | 38,810,939 | 56,172,596 |

- Additional county level activity data specific to each rail depot that NS and CSX operate in the state was also obtained as indicated in Tables 6A and B:

1.		NS owned and/or operated Switch Yards:

Table 6A: Class 1 NS Railroad Switch Yard Activity

| Yard Depot | County Location | Railroad | Activity Description | ###### Activity5 |
| --- | --- | --- | --- | --- |
| Croxton Yard (Kearny & North Bergen) | Hudson | NS | Trains per week | 23 |
| Erail-Elizabeth Port | Union | NS | Trains per week | 17 |
| Manville Yard | Somerset | NS | Trains per week | 5 |
| TOTAL |  | NS | Trains per week | 45 |

2.	CSX owned and/or operated Switch Yards:

Table 6B: Class 1 CSX Railroad Switch Yard Activity

| Yard Depot | County Location | Activity Description | GT_Line6 &15 | ###### MILES_LINE6,7,8,9,10&11 |
| --- | --- | --- | --- | --- |
| Little Ferry Yard adjacent to the NYS & W Yard (River Line) | Bergen | ½ freight Gross Tons transported over rail lines (GT_LINE) & total railroad line mileage (MILES_LINE) included in Table # 4 above for River Line Rail Line. | 29.5800 | 16.80 |
| North Bergen Yard (River Line) | Bergen | ½ freight Gross Tons transported over rail lines (GT_LINE) & total railroad line mileage (MILES_LINE) included in Table # 4 above for River Line Rail Line. | 29.5800 | 16.80 |
| Kearny Yard (Northern Runner) | Hudson | 1/3 freight Gross Tons transported over rail lines (GT_LINE) & total railroad line mileage (MILES_LINE) included in Table # 4 above for the N runner rail line. | 11.0333 | 24.80 |
| South Kearny Yard (Northern Runner) | Hudson | 1/3 freight Gross Tons transported over rail lines (GT_LINE) & total railroad line mileage (MILES_LINE) included in Table # 4 above for the N runner rail line. | 11.0333 | 24.80 |
| Elizabethport (Trumbell Street) | Union | 1/3 freight Gross Tons transported over rail lines (GT_LINE) & total railroad line mileage (MILES_LINE) included in Table # 4 above for the N runner rail line. | 11.0333 | 24.80 |
| Manville Yard (West Trenton Line) | Somerset | Total of freight Gross Tons transported over railroad (GT_LINE) and total railroad line mileage (MILES_LINE) included in Table # 4 above for W. Trenton Rail Line. | 33.1000 | 21.00 |

Note that the activity data presented above is the same activity used for railroad line-haul operations (refer to Table 4) except that the GT_LINE amounts for the River and Northern Runner lines were divided into equal portions for the several yard depots that service each of those lines.  This relates individual rail line activity to those yard depots that serve as their switching and refueling stations.  For example, both the Little Ferry and North Bergen Yards service the River Line Rail Line. This means that the GT_LINE amount for the line must be proportioned equally between these two yard depots to properly correlate line haul activity conducted on the line with yard switching activity conducted at these two depots that service the line haul activities.

All of the activity data included in Tables 6A & B were applied to allocate this state level activity to the county level for all yard switching operations conducted by the NS and CSX railroads.  This allocation process is discussed in detail in Section II.2.B.

- Allocation Process:

1.	The NJDEP determine fuel use activity for switch yard operations from ratio of fuel use at the national level for switch yards included in Table 5 and the line haul operations included in Table 3: (YARD_LH_RATIO).  The following equation shows the equation used for this determination:

(YARD_LH_RATIO) = FUELYARD_US/FUELHAUL_US

Where:

FUELYARD_US    =  National diesel fuel use for yard

switch operations (gals)

FUELHAUL_US    =  National diesel fuel use for

line-haul operations (gals)

Application of this ratio to fuel use for line haul operations allowed the NJDEP to convert this activity to fuel use for switch yard operations at NS and CSX railroads.  This is discussed in greater detail in Sections II.3.B.1.i and ii:

- NS owned and/or operated switch yards: Steps 1 - 4 describe the procedures used to determine fuel use activity for switch yard operations conducted by NS railroads:

Step 1:	Determine total fuel use on the state level  (FUELHAUL_STATE) by totaling the fuel use on the county level (FUELHAUL_CTY) for NS line haul operations:

FUELHAUL_STATE = FUELHAUL_CTY

Step 2: 	Convert total line haul fuel use into switch yard fuel use (FUELYARD_STATE)  by applying the yard and line haul fuel use ratio (YARD_LH_RATIO)  developed above in Step 1:

FUELYARD_STATE = FUELHAUL_STATE * YARD_LH_RATIO

Step 3:	Proportion train activity data included in Table 6A to allocate fuel use for switching yard operations from the state to the county level (CTY_ALLOC):

CTY_ALLOC = TRAIN_NUM/TRAIN_NUM

Where:

TRAIN_NUM   = 	Number of train trips to an individual switch yard depot on individual NS rail lines every week.

TRAIN_NUM = 	The summation of train trips to all switching yards.

Step 4: 	Allocate fuel use for switching yard operations from the state to the county level using the following equation:

FUELYARD_CTY = FUELYARD_STATE*CTY_ALLOC

Where:

FUELYARD_CTY      = Fuel Use for yard operations at county level (gal)

FUELYARD_STATE  = Fuel Use for yard operations at state level (gal)

CTY_ALLOC              = State to County fuel use allocation factor

- NS owned and/or operated Switch Yards: Steps 1 and 2 describe the procedures used to determine fuel use activity for switch yard operations conducted by NS railroads:

Step 1: Calculate fuel use for the line haul operations conducted on each individual              CSX line included above in Table # 6B to determine the switch yard fuel                    use.

FUELHAUL_CTY   = GT_LINE *MILES_LINE*FCI

Where:

FUELHAUL_CTY  = Fuel use for line haul

operations at county level (gal)

GT_LINE               = Gross Tons transported on line (Tons)

MILES_LINE         =  Rail line mileage (miles)

FCI			= Fuel Consumption Index (gal)

Step 2: 	Convert total line haul fuel use into the switch yard fuel use from application of the fuel use ratio developed above in Step One:

FUELYARD_CTY = FUELHAUL_CTY * YARD-LH_RATIO

Where:

FUELYARD_CTY     = Fuel use for yard switching

operations at county level (gal)

FUELHAUL_CTY     = Fuel use for line haul operations

at county level (gal)

YARD-LH_RATIO     = Ratio of fuel use for switch yards

and line haul operations

- Emission Calculation:

Epa = EFy*FUELYARD_CTY/CF

Eps = Epa/AADF*SAFO

Where:

Epa     = Annual emission of pollutant (tons/yr)

Eps     = Typical summer day emission of pollutant (tons/day)

AADF = Annual activity day factor (WA*52 weeks/year)

CF      = units conversion factor (2000lbs/ton)

Weekly activity (WA) = 7 days/week

Seasonal adjustment factor (ozone season), SAFO = 1

Refer to Tables CSX-3 and NS-3 in Sub-Attachment C to review the complete calculations for these Class 1 Railroads.

CSX and NS railroads also operate on the Conrail CSA track.  A separate calculation based on input data provided by Conrail was performed for these two Class 1 railroads, as well as all railroads that operate on the CSA line.  This calculation is discussed in Section II.4.B.

4.	Conrail Shared Assets (CSA) Line Haul Operations:

The CSA line represents the remaining lines under Conrail control.  The CSX and NS Class 1 railroads, Canadian Pacific Class 2 railroad and other railroads use these lines to either pass through onto their final destination, for switching their freight to another line or for their final destination to a rail yard.  Conrail mainly conducts line-haul switching operations with these railroads and operates several yard terminals.  They may also conduct some short distance freight hauling for local industries.  Conrail no longer conducts Class 1 long distance line haul operations.  They do, however, records the Class 1 and all other railroad activity conducted on its tracks.   This information was provided to the NJDEP in Excel spreadsheet format.

- Activity Data Collection:

The Conrail CSA submittal included seven CSA rail line Excel spreadsheets4 entitled:

- Lehigh 1

- Nat Dock-North 2

- Chem Coast 3

- Amboy-4

- NEC-S

- SJERSEY-N

- SJERSEY-S

The information included on these spreadsheets include:

- Gross Tons (GT) of freight transported over line segments.

- Rail Miles for each railroad line segment.

- Gross Ton Miles (GTM) that results from the multiplication of GT with rail miles.

The above information is presented at the county level for every juncture or switching station that makes up an individual rail line segment.  As an example, Table 7 shows the type of information included on one of the CSA spreadsheet (the example is for the Lehigh Line):

Table 7: Line Segment Sample from Conrail CSA Excel Spreadsheet Submittal for Lehigh Line4

| Loc-#1 | MP-#1 | LOC-#2 | MP-#2 | Rail | County | Miles | Tons | Gross Ton-miles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bound brook | 33.5 | County line | 32.9 | ALL | Somerset | 0.6 | 39.3 | 23,580,000 |
|  |  |  |  | CR |  | 0.6 | 0.6 | 360,000 |
|  |  |  |  | CP |  | 0.6 | 0.4 | 240,000 |
|  |  |  |  | CSX |  | 0.6 | 20.1 | 12,060,000 |
|  |  |  |  | NS |  | 0.6 | 18.2 | 10,920,000 |
| County line | 32.9 | South Plainfield | 26.5 | ALL | Middlesex | 6.4 | 39.3 | 251,520,000 |
|  |  |  |  | CR |  | 6.4 | 0.6 | 3,840,000 |
|  |  |  |  | CP |  | 6.4 | 0.4 | 2,560,000 |
|  |  |  |  | CSX |  | 6.4 | 20.1 | 128,640,000 |
|  |  |  |  | NS |  | 6.4 | 18.2 | 116,480,000 |

The example table indicates that CR, CP, CSX and NS railroads transport their various gross tonnage of freight on 0.6 miles of the Lehigh line segment located between the Bound Brook and County Line locations within Somerset County.  The next adjoining line segment follows from the county line to the South Plainfield location for transport of GT over 6.4 miles of track in Middlesex County.  Note that the GT between these two segments are the same because no switching or yard activity has occurred to unload freight from these trains at this point.  The GTM value is obtained by multiplying GT by the miles over which the GT were transported.

Using the pivot table function in Excel on each of the seven CSA rail line spreadsheets, the NJDEP summed all the GTM conducted for each individual railroad at the county level. The results from this pivot table compilation were then presented in a single Excel spreadsheet.  A sample of this compilation for the Lehigh Line is presented in Table 8.

###### Table 8: Sample Pivot Cell Compilation for Lehigh Line

| County | Sum of ton-miles for all lines | CP GTM | CR GTM | CSX GTM | NJT GTM | NS GTM |
| --- | --- | --- | --- | --- | --- | --- |
| Essex | 239,071,000 | 760,000 | 10,191,000 | 143,760,000 | 9,500,000 | 74,860,000 |
| Hudson | 57,992,000 | 0 | 532,000 | 42,260,000 | 0 | 15,200,000 |
| Middlesex | 418,369,400 | 4,200,000 | 5,619,400 | 211,050,000 | 0 | 197,500,000 |
| Somerset | 118,582,700 | 1,160,000 | 3,309,450 | 58,290,000 | 0 | 55,823,250 |
| Union | 432,370,150 | 4,000,000 | 2,620,150 | 201,000,000 | 42,750,000 | 182,000,000 |

- Allocation Process:

The compilation sample presented in Table 8 shows that five railroads operate on the Lehigh line.  However, only GTM from the CSX, NS and CR railroads were converted to fuel use as indicated in Step 1 and 2 below.  No conversions were conducted for the New Jersey Transit (NJT) and Canadian Pacific (CP) railroads from the information provided by the CSA Excel spreadsheet.  This is because all their emission calculations were performed on other Excel spreadsheets from information that these railroads supplied in separate disclosures as discussion in greater detail in Sections II.6 through 11.

Step 1: 	Apply the same Fuel Consumption Index (FCI) factor used in Section II.2.B.1 for NS and CSX railroads (FCI_NS and FCI_CSX) to their respective county level GTM (GTM_CTY) to obtain their fuel use at county level (FUEL_CTY):

FUEL_CTY = GTM_CTY/FCI_NS and FCI_CSX

Step 2:	Use GTM for the CR railroad (GTM_CTY) to allocate line haul fuel use on the northern and southern New Jersey regional level (FUELHAUL_NORTH or SOUTH) to the county level:

FUEL_CTY = (GTM_CTY/GTM_CTY)*FUELHAUL_NORTH

And

FUEL_CTY = (GTM_CTY/GTM_CTY)*FUELHAUL_SOUTH

Where GTM_CTY represents the summation of all the GTM_CTY from either the North or South NJ regions.

*Note that the FUELHAUL_NORTH or SOUTH for THE CR railroad was not directly available because CR only provided total fuel use that combined line haul and freight operations.  As such, additional calculations were performed by the NJDEP to obtain line haul fuel use from total fuel use as explained in Section II.5.

The equation in Section II.4.C was used to calculate county level emissions for the NS, CSX and CR railroads.

C.	Emission Calculation:

Epa = EFf*(FUEL_CTY)/CF

Eps = Epa/AADF*SAFO

Where:

FUEL_CTY = Fuel use at the county level

Epa     = Annual emission of pollutant (tons/yr)

Eps     = Typical summer day emission of pollutant (tons/day)

AADF = Annual activity day factor (WA*52 weeks/year)

Refer to Tables CSA-1 and 2 and CR-4 in Sub-Attachment C for the complete calculations performed for the CSA and CR Railroads.

###### 5.  Conrail (CR) Yard Operations:

Conrail operates the following Yard depots on the CSA lines:12

| Northern New Jersey District | Southern New Jersey District |
| --- | --- |
| Bayonne Yard | Paulsboro Yard |
| South Kearny Yard | Burlington Yard |
| South Amboy Yard | Milville Yard |
| North Bergen Yard | Pavonia Yard |
| Linden Yard |  |
| Oak Island Yard |  |
| Metuchen Yard |  |
| Port Newark Yard |  |
| Port Reading Yard |  |
| Manville Yard |  |

A.	Activity Data Collection:

CR submitted two Excel spreadsheets that included local switch and yard switch hours of operation for all yard depot designations.12  The local switch hours constitute line haul switching operations.  The yard switch operations constitute both line haul and yard switching operations, and are performed at the yard designations in the northern or southern New Jersey districts.

###### B.	Allocation Process:

Table 9 is an example of how the line haul operations are presented on the CR local switch hours spreadsheet for a typical yard designation.  The example is for the South Amboy Yard.

Table 9: Local Switch hours for Conrail CSA Line Haul Operations12

| Location | Symbol (Northern New Jersey) | 2002 Switch Hours | Primary Duties |
| --- | --- | --- | --- |
| Northern New Jersey – South Amboy Yard | WPSA-31 | 2228.7 | Switch customers: Cranberry / Freehold / Tennent |
|  | WPSA-33 | 2946.74 | Switch customers: Dayton / Jamesburg |
|  | WPSA-35 | 2414.88 | Switch customers: Lakewood / Toms River / Farmingdale |
|  | WPSA-91 | 29.13 | Works any location listed above as needed |
| Total For South Amboy Yard |  | 7619.45 | Total line haul hours of operation |

Table 9 shows that the total local switch hours for the South Amboy Yard designation is 7619.5 hours of line haul operations.  These hours represent the time that CR spent switching freight cars from one customer to another or switching freight trains to another rail line within the area defined by the South Amboy Yard designation.  This procedure was conducted for every CR yard designation to obtain total local switch hours of line haul operations for both northern and southern New Jersey as presented in Table CR-1 of Sub-Attachment C.

Switch hours for line haul operations in addition to yard operations were obtained from the CR yard switch hours Excel spreadsheet. Table 10 is an example of how the line haul and yard switch operations are presented in this Excel spreadsheet for a typical yard designation. The example is for the South Amboy Yard.

Table 10: Conrail Yard Switch hours (Includes Line Haul & Yard Operations)12

| Location | Symbol | Total hours | Line Haul Hours | Yard switch hours | % Yard or LH hours | LH or Yard Designation | Operation Description |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Northern New Jersey – South Amboy Yard | YPSA-02 | 2339.46 | 2339.46 |  |  | Line haul | Switch customers: Phoenix / Parlin / E Brunswick |
|  | YPSA-03 | 2414.62 |  | 2173.16 | 90% | Yard | Yard Switching: Browns Yd (Old Bridge) |
|  |  |  | 241.46 |  | 10% | Line haul | Switch customers: Phoenix |
|  | YPSA-22 | 2435.42 | 1461.25 |  | 60% | Line haul | Switch customers: Runyon / Spotswood / Phoenix |
|  |  |  |  | 974.17 | 40% | Yard | Yard Switching:Browns Yd (Old Bridge) |
|  | YPSA-60 | 3010.72 | 3010.72 |  |  | Yard | Transfer: Browns yard to Newark (Oak Island) & return |
|  | YPSA-91 | 691.56 | 478.18 |  | 69.14% | Line Haul | Works any location listed above as needed |
|  | YPSA-91 |  |  | 213.38 | 30.86% | Yard | Works any location listed above as needed |
| Totals For South Amboy Yard |  | 10891.78 | 7531.07 | 3360.71 | 100% |  |  |

This example shows the designation of total hours as attributable to either yard or line haul operations or a percentage of both for each operation description.  The percentages of yard or switch hours for mixed operations were provided directly by CR.  For the operation description “works any location,” the NJDEP performed the following two steps to determine percentage of yard and line haul operations for this segment within the South Amboy yard designation:

Step 1: % line haul = (total line haul hours)/(total line haul + yard hours)

Step 2:  % yard      = 1 - % line haul

The NJDEP calculated the sum of yard switch hours for every yard designation to obtain total yard switch hours of line haul and yard operations for both northern and southern New Jersey.  The results of these calculations are included in Table CR-2 of Sub-Attachment C.

The total yard switch hours and the total local switch hours are summed and presented in Table 11:

Table 11: Total Local and Yard Switch Hours for Line Haul Operations

| Region | Total Local Switch for Line Haul Ops | Total Yard Switch for Line Haul Ops | Total Switch Hours for Line Haul Ops | Total Switch Hours for Yard Ops | Total Hours |
| --- | --- | --- | --- | --- | --- |
| Northern NJ | 21,163 | 88,562 | 109,725 | 44,887 | 154,612 |
| Southern NJ | 33,736 | 10,708 | 44,444 | 13,360 | 57,804 |
| Total State | 54,899 | 99,270 | 154,169 | 58,247 | 212,416 |

The total switch hours for line haul and yard operations were used to determine the percentage of fuel used by CR railroads on the CSA line using the same proportional relationships shown above in Steps 1 and 2.  These percentages were than applied to Conrail's total rail fuel use for both the northern and southern New Jersey regions to determine fuel use for line haul and yard operations within these regions.  The results are presented in Table 12.

Table 12: Total Percent Hours for Line Haul and Yard Operations and

Designated Fuel Use

| Region | % LH | % Yard | Total Fuel13 | Line-Haul fuel (FUEL_LH) | Yard fuel (FUEL_YARD) |
| --- | --- | --- | --- | --- | --- |
| Northern NJ | 0.71 | 0.29 | 2158620 | 1531932 | 626688 |
| Southern NJ | 0.77 | 0.23 | 587308 | 451563 | 135745 |
| Total State | 0.73 | 0.27 | 2745928 | 1992961 | 752967 |

The fuel use for the CR line haul operations was applied to the GTM allocation factor referenced in Step 2 of Section II.4.B.  The fuel use for yard operations was applied directly to the yard locomotive emission factor as shown in the emission calculations included in Section II.5.C.  Note that the county level allocation step used for allocating CR line haul emissions to the county level in Section II.4.B is not needed for yard switching operations, because each individual yard is always located within one county.

###### C.	Emission Calculations:

Epa = EFy*FUEL_YARD/CF

Eps = Epa/AADF*SAFO

Where:

FUEL_YARD = Fuel use for yard operations

Epa     = Annual emission of pollutant (tons/yr)

Eps     = Typical summer day emission of pollutant (tons/day)

AADF = Annual activity day factor (WA*52 weeks/year)

Refer to Tables CR-3, 4 and 5 in Sub-Attachment C for the complete calculations performed to determine line haul and yard fuel use and yard emissions.

- Canadian Pacific:

The Canadian Pacific railroad line runs from Phillipsburg to Pattenburg to 3 Bridges-Nestolanic to Flagtown to Port Reading Junction and onward toward the Lehigh Valley line to Oak Island.

- Activity Data:

Table 13: Canadian Pacific Activity Data14

| Description | Symbol | Value | Units |
| --- | --- | --- | --- |
| Diesel Fuel Use Rate | FUEL_RATE | 2.5 | Gallon/engine/mile |
| Total Miles in system |  CTY MILES | 68 | Miles |
| Number Engines | ENG_NUM | 2 |  |
| Annual trips | TRIPS | 312 |  |

B.	Allocation Process:

County Allocation Step (This line was mapped out at the county level7,8,9,10  & 14):

CTY MILES =  MI*MAPI

CTY ALLOC = CTY MILES/  CTY MILES

Where:

MI                 = Measured inches of county track on county map (in)

MAPI            = County map conversion factor for inches to miles (in/mile)

CTY MILES               =  Rail miles at county level

CTY ALLOC              =  Ratio of county track mileage to sum of all county                                                       track mileage.

C.	Fuel Use Calculation:

FUEL_NJ = FUEL_RATE* CTY MILES*ENG_NUM*TRIPS

Where:

FUEL_NJ = Total Annual NJ Fuel Use

D.	Emission Calculation:

Epa = EFf*CTY ALLOC*FUEL_NJ/CF

Eps = Epa/AADF*SAFO

Where:

Epa     = Annual emission of pollutant (tons/yr)

Eps     = Typical summer day emission of pollutant (tons/day)

AADF = Annual activity day factor (WA*52 weeks/year)

CF      = units conversion factor (2000lbs/ton)

Weekly activity, WA=7 days/week

Seasonal adjustment factor (ozone season), SAFO = 1

7.   New York Susquehanna Western Railroad Freight Line:

The New York Susquehanna Western railroad freight line runs from Vernon to Jersey City.

A.	Activity Data:

The total New Jersey diesel fuel use for freight and switching operations:

FUEL (10E3 gallons)14 = 623,000 gallons

- Allocation Process:

The New York Susquehanna Western railroad freight line was mapped out at the municipal level for the 1996 inventory.7,8,9,10 &14  This municipal level mapping was maintained for the 2002 inventory, since the rail line remains the same.  By introducing the corresponding county FIP code column for each municipality on the Excel calculation sheet, the NJDEP could determine all emissions from the municipal to the county level.

MUN_MILES = MI*MAPI

MUN_ALLOC = MUN_MILES/  MUN_MILES

Where:

MI      = Measured inches of municipal track on county map (in)

MAPI = County map conversion factor for inches to miles  (in/mile)

MUN_MILES (miles)    = Total miles of track in municipality

 MUN MILES (miles) = Total miles in New Jersey

MUN_ALLOC               = Ratio of municipal track mileage to sum of all                                                                         municipal track mileage.

###### Fuel Allocation Process:

The NJDEP performed the following steps to calculate total state level fuel use for line haul (FUEL_LH) and yard operations at Ridgefield Park (FUEL_YD):

Step 1: FUEL_LH =  FUEL*2/314

Step 2: FUEL_YD = FUEL*1/314

Where:

FUEL = State fuel use  for both line-haul and yard switching operations

D.	Emission Calculation Step:

Epa = EFf*MUN_ALLOC*FUEL_LH/CF + FUEL_YD*EFy

Eps = Epa/AADF*SAFO

Where:

Epa  = Annual emission of pollutant (tons/yr)

Eps  = Typical summer day emission of pollutant (tons/day)

AADF  = Annual activity day factor (WA*52 weeks/year)

CF = units conversion factor (2000lbs/ton)

8.  Shortline Freight/Commuter Railroads operating in more than one municipality:

- Activity Data:

The following activity data was collected for each shortline railroad:

1.  Total systemwide diesel fuel use for freight: FUEL (10E3 gallons)14

2.  Number of trips made by each line: TRIP14

3.  Number of cars pulled by each line locomotive: NUM_CAR14

4.  Weight of typical car pulled by each line locomotive: CAR_TON (tons)14

5.  Weight of typical freight placed on each car: FREIGHT_TON (tons)14

- Municipal Allocation Step:

Note that most of these shortlines are mapped out at the municipal level7,8,9,10 & 14.  However by introducing the corresponding county FIP code column for each municipality on the Excel calculation sheet, the NJDEP could determine all emissions from the municipal to the county level.

MUN_MILES = MI*MAPI

MUN_GTM    = MUN MILES*(TRIP*NUM_CAR)*(CAR_TON+FREIGHT_TON)

Where:

MI               =  Measured inches of municipality track for a specific

shortline railroad line on county map (in).

MAPI          = County map conversion factor for inches to miles (in/mile)

MUN_MILES = Rail miles at county level

MUN_GTM = Ratio of municipal track mileage of a specific short line                                               to sum of all municipal track mileage for that specific line.

C.	Fuel Allocation Step: (Systemwide to Municipal Use):

FCI = MUN_GTM/FUEL

MUN FUEL = MUN_GTM/FCI

Where:

FCI              = Fuel consumption index (GTM/gallons)

MUN_FUEL = municipal fuel use (gallons)

###### Emission Calculation Step:

Epa = EFf*MUN_FUEL/CF

Eps = Epa/AADF*SAFO

Where:

Epa      = Annual emission of pollutant (tons/yr)

Eps      = Typical summer day emission of pollutant (tons/day)

AADF  = Annual activity day factor (WA*52 weeks/year)

CF       = Units conversion factor (2000lbs/ton)

9.	Shortline Railroads located in one municipality:

A.	Activity Data:

Total systemwide diesel fuel use for freight: FUEL (10E3 gallons)14

B.       Emission Calculation Step:

Epa = EFf*FUEL/CF

Eps = Epa/AADF*SAFO

Where:

Epa  = Annual emission of pollutant (tons/yr)

Eps  = Typical summer day emission of pollutant (tons/day)

AADF  = Annual activity day factor (WA*52 weeks/year)

CF = units conversion factor (2000lbs/ton)

Refer to Table SHL-1 in Sub-Attachment C for the complete calculations performed to determine short-line railroad fuel use and emissions.

- Passenger Railroad (AMTRAK):

This railroad line follows the Northeast Corridor from New York City to Trenton.

- Activity Data:

Railroad line follows the Northeast line from New York City, New York straight down to Trenton, New Jersey with Fuel Use of:  82,500 gallons14

B.      County Allocation Step:

CTY_MILES  =  MI*MAPI

CTY_ALLOC = CTY_MILES/  CTY_MILES

Where:

MI      = Measured inches of county track on county map (in)

MAPI  = County map conversion factor for inches to miles (in/mile)

CTY_ALLOC = Ratio of county track mileage to sum of all county                                                  track mileage.

###### Emission Calculation Step:

Epa = EFf*CTY_ALLOC*FUEL/CF

Eps = Epa/AADF*SAFO

Where:

Epa      = Annual emission of pollutant (tons/yr)

Eps      = Typical summer day emission of pollutant                                                                         (tons/day)

AADF  = Annual activity day factor (WA*52 weeks/year)

CF      = units conversion factor (2000lbs/ton)

11.       Statewide Public Railroad (New Jersey Transit):

###### Activity Data Collection and Adjustment:

The following activity data was collected and adjustments were made as follows:

1.  	Switch & Line Haul Fuel Use:

- The NJDEP obtained daily fuel use for all New Jersey Transit (NJT) engine designations for the six NJT rail yards listed in Table 14.  This information was provided in excel spreadsheet format15. The yards listed in Table 14 act as fuel depots for NJT commuter and yard switching and calibration equipment diesel fuel operations.

Table 14: NJT Yard Designations

| ###### Yard Symbol | Yard Name |
| --- | --- |
| AC | Atlantic City |
| BH | Bayhead |
| HO | Hoboken |
| KE | Kearny |
| PM | Port Morris |
| RA | Raritan |

A sample of the daily fuel consumption activity data for the Atlantic City

Depot is presented in Table 15.

Table 15: NJT Yard Fuel Consumption15

| Location | Engine Designation | Date | Gallons |
| --- | --- | --- | --- |
| AC | 4206 | 01-Jan-02 | 667 |
| AC | 4207 | 01-Jan-02 | 687 |
| AC | 4202 | 01-Jan-02 | 793 |
| AC | 4212 | 01-Jan-02 | 575 |

- The NJDEP then used Excel's pivot table function to obtain annual summations of daily fuel use data for each engine designation on each of the 6 NJT rail yard Excel spreadsheets.  Table 16 is an example of the summation of the daily fuel use data for the AC yard designation:

Table 16: Annual Fuel Use for AC Yard Depot

| Engine | Gallons |
| --- | --- |
| 4145 | 799 |
| 4147 | 6111 |
| 4149 | 16534 |
| 4201 | 45261 |
| 4202 | 41156 |
| 4203 | 50168 |
| 4204 | 40408 |
| 4205 | 41624 |
| 4206 | 88679 |
| 4207 | 59533 |
| 4208 | 75479 |
| 4209 | 28009 |
| 4210 | 32835 |
| 4211 | 62788 |
| 4212 | 16613 |
| 4213 | 28828 |
| 4214 | 39019 |
| 4215 | 39092 |
| 4216 | 87446 |
| 4217 | 104851 |
| 4218 | 118779 |
| 4219 | 34389 |
| 4300 | 480 |
| 4302 | 760 |
| 4303 | 1234 |
| CALI | 1197 |
| Switch | 2474 |
| Line Haul | 1058401 |
| Calibration | 1197 |
| TOTAL | 1062072 |

Note that the engine designations 4300, 4302 and 4303 and CALI in Table 16 represent respective yard switching and equipment calibration operations.  The NJDEP separates these operations from line haul fuel use and then sums each category of fuel use at the bottom of each of the 6 NJT pivot tables as explained in Section B.11.1.iii.

- Table 17 summarizes the line-haul from yard switching and calibration operations by reference to their corresponding engine designation ranges:

Table 17: Engine Designation for Railroad Operations16

| Railroad Operation | ###### Engine Designation |
| --- | --- |
| Calibration | CALI |
| Switching yard | 0500, 0501, 0502, 0503, 4300, 4301, 4302 and 4303 |
| Line haul | All numbered engine designations other those indicated for switching yard engines. |

- The NJDEP then conducted separate summations of line-haul, switching and yard engine designation fuel use for all yard depots, and calculated a grand total for fuel use for all line-haul and switching operations except the specific amount designated for Atlantic City.  This information is summarized in Table 18.   This data represents fuel use for all line-haul and switching operations conducted in North and Central New Jersey.  Note that all of the trains that run on the Atlantic City Line use fuel transferred from the Atlantic City depot.

Table 18: Yard Switch & Line Haul Annual Fuel Use at NJ Transit Yard Depots for the Year 2002

| YARD FUEL DEPOT | YARD SWITCH (Gallons) | LINE HAUL (Gallons) |
| --- | --- | --- |
| BAYHEAD* | 20,950 | 1,061,496 |
| HOBOKEN | 55,940 | 4,251,723 |
| KEARNY | 244,808 | 492,231 |
| PORT MORRIS* | 31,996 | 1,456,500 |
| RARITAN | 10,563 | 1,925,924 |
| TOTAL (not including Atlantic City) | 364,257 | 9,187,875 |
| ATLANTIC CITY | 2,474 | 1,058,401 |

Note that approximately half of the fuel designated for Bayhead is used for yard switching operations at Long Branch.  Similarly, approximately half the fuel designated for Port Morris is used for switching operations at Dover.16

- Annual Car Mile Allocation for North Jersey Train Lines:

The NJDEP obtained monthly car-mile data and summed this data to obtain the annual total for each rail line.  This information is summarized in Table 19:

Table 19: Annual Car Miles and Allocation for North Jersey Train Lines for 199617

| ###### Train | Annual car miles | Car Mile Allocations for North & Central NJ lines |
| --- | --- | --- |
| Pascack Valley | 767359.8 | 0.0472 |
| Boonton | 1427809.6 | 0.0879 |
| Main line | 2271442.1 | 0.1399 |
| Bergen County line | 1763879 | 0.1086 |
| Port Jervis | 2605801.6 | 0.1604 |
| M & E – Gladstone | 21386.6 | 0.0013 |
| M & E – Dover | 638767 | 0.0393 |
| Midtown | 0 | 0 |
| Montclair Line | 91200 | 0.0056 |
| Raritan valley line | 3486304.2 | 0.2147 |
| NJCL – Long branch | 0 | 0 |
| NJCL – Bayhead | 2248025.2 | 0.1384 |
| Northeast corridor | 918953.4 | 0.056583 |
| Princeton shuttle | 0 | 0 |
| Total railroad lines (not including Atlantic City line) | 16240928.5 | 1.0000 |
| Atlantic city line | 1799454.1 | 1.0000 |

Note that the above table only contains 1996 data since the 2002 data was not available.  Also, the last column of this table represents the percent allocation applied to the fuel use for North and Central New Jersey and Atlantic City as discussed in Section II.11.C.

- Systemwide to Railroad Line Fuel Use Allocation Step:

The NJDEP used car miles as indicated in Step 1 to allocate fuel use for all NJT rail lines except Atlantic City Line:

Step 1: CAR_MILE_ALLOC = CAR_MILES/CAR_MILES

Where:

CAR MILE     = Car miles indicated in Table 9 above for any of the                                     fourteen (14) individual North and Central Jersey lines.

CAR MILES = Car miles summation from the fourteen (14) lines                                        indicated above.

Note that the Atlantic City Line is a total diesel fuel operation serviced by its own yard depot.  A car mile allocation step is not conducted for this rail line because all the fuel indicated above in Table 18 for this depot is used only for this line and its yard operations, i.e. CAR MILE ALLOC = 1.  Therefore, for the Atlantic City Line, only a county level allocation as discussed in Section II.11.D needs to be performed.

###### C.	Railroad Line to County Level Allocation Step:

The NJDEP allocated individual rail line fuel use to the county level from measured mileage of rail line using the following procedure:7,8,9 &10

MILES_CTY = MI*MAPI

ALLOC_CTY= MILES_CTY/MILES_CTY

Where:

MI                  =  Measured inches of county track for a                                                          specific NJ Transit line on county map (in).

MAPI             =  County map conversion factor for                                                                                      inches to miles (in/mile).

ALLOC_CTY =  Ratio of municipal track mileage to sum of all                                  			       municipal track mileage for a given railroad line.

The MILES_CTY should closely approximate LINE MILES.  Also note that the measurement follows the same procedure used previously for measuring all rail lines that travel through more then one county.

###### D.        Emission Calculation:

Epa = (EFf*FUEL_HAUL*CAR_MILE_ALLOC*ALLOC_CTY)/CF +                                  EFy*FUEL_YARD/CF

Eps = Epa/AADF*SAFO

Where:

Epa      = Annual emission of pollutant (tons/year)

Eps      = Typical summer day emission of pollutant (tons/day)

AADF  = Annual activity day factor (WA*52 weeks/year)

CF       = Units conversion factor (2000lbs/ton)

Refer to Tables NJT-1 & 2 in Sub-Attachment C for the calculations performed to determine NJT passenger and yard switch emissions.

Rule Effectiveness:

The emissions from this source category are regulated by the new locomotive emission standards (40 C.F.R. Part 92).

Double Counting:

Emissions from this source category have not been reported in the point source inventory, therefore no adjustment of the area source emissions was required.

Output:

The emissions were calculated on a county basis for the following pollutants - season basis.

| Pollutant | Annual | Summer | Winter |
| --- | --- | --- | --- |
| VOC | Tons/year | Tons/day |  |
| NOx | Tons/year | Tons/day |  |
| CO | Tons/year | Tons/day | Tons/day |
| SOx | Tons/year |  |  |
| PM10 | Tons/year |  | Tons/day |
| PM2.5 | Tons/year |  | Tons/day |

Projection Procedure:

Train transportation value added data is required to calculate projected emissions for this source category.

Assumptions:

- The national Fuel Use Index (FCI) for CSX and NS railroads corresponds to the FCI for the State of NJ.

- The Conrail CSA line GTM amounts represent the GTM amounts for those CSX and NS lines connected to the CSA line.

- The percent of yard and line-haul hours of operation relate to the correct proportion of fuel use for these operations when applied to total Conrail CSA fuel use.

- One-third (1/3) of the total fuel for NY Susquehanna Western railroad represents yard activity.

- The car miles by location from July 1995 to June 1996 submitted by NJ transit apply to this activity conducted in 2002.

- Approximately ½ of the fuel designated for Bayhead is used for yard switching operations at Long Branch and similar circumstances exist in regard to Port Morris and the NJT switching yard operated at Dover.

References:

- Documentation for Aircraft, Commercial Marine Vessel, Locomotive, and Other NonRoad Components of the National Inventory, Volume I-Methodology, Appendix C, locomotive Emission Estimation Methodology, Table C-1, Criteria Pollutant Emission Factors, Emission Factor and Inventory Group, USEPA, Research Triangle Park, NC

- Class I Railroad Annual Report for the Year Ending December 31, 2002, Norfork Southern Combined Railroad Subsidiaries

- Class I Railroad Annual Report for the Year Ending December 27, 2002, CSX Transportation, Inc.

- Excel spreadsheet email submittal from Joseph Seamen of Conrail, Conrail_1d.xls, Seven (7) CSA Rail Line GTM Excel Spreadsheets, September 29, 2003

- Norfork Southern Operations and Diesel Fuel Use in New Jersey, email submission from Richard Crawford, June 10, 2003

- Email submittal from John Cassillini of CSX, September 29, 2003

- New Jersey Delorme Atlas & Gazetteer, 2000

- County Maps by Hagstrom or Alfred B. Patton.

- Northern New Jersey Local Freight Service (LFS) Map

- NJ Transit Passenger Rail System Map

- Excel spreadsheet email submittal from Joseph Seamen of Conrail, Color coded not to scale Map of CSA lines on excel spreadsheet, Conrail_1e.xls, September 29, 2003

- Excel spreadsheet email submittal from Joseph Seamen of Conrail, local switch and yard hours, Conrail_1a.xls, August 19, 2003

- Excel spreadsheet email submittal from Joseph Seamen of Conrail, Fuel Usage 2002, Conrail_1c.xls, August 19, 2003

- Survey of Shortline, NY Susquehanna, Amtrak and Canadian Paciifc Railroad activities for 2002, New Jersey Department of Environmental Protection, August 27, 2004

- Diesel Fuel Consumption by Location for 2002, August S. Andolino of NJ  Transit Rail Operations, August 6, 2003

- Memorandum concerning NJ Transit Yard depot operations, July 21, 2004

- Monthly Car Miles by Location from July 1995 to June 1996, NJ Transit Rail Operations, August 29, 1997